IEEE P802.11
Wireless LANs

| 802.11 TGmb LB160 Power-saving buffered units |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-02-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

Comment 2216 in LB160 relates to inconsistency in the use of the terms “MSDU, A-MSDU and/or MMPDU”.

This submission proposes one possible solution.

R1: updated following 2010-02-12 telecon.  Corrected two errors as noted in the minutes 11-10/0214r0.

# Introduction

Comment 2216 reads:

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 2216 | 658.01 | 11.6.2 | P | In Clause 11, There's a lot of variability in use of "or" and "and" in the phrase "MSDUs, A-MSDUs, or MPDUs". Generally it should be an "or" when the singular is used (where selection of one is implied) and an "and" when the plural is used (where inclusion of all is implied). | Review Clause 11 and correct use of "and" and "or" as described in the comment. | AGREE IN PRINCIPLE (EDITOR: 2010-01-29 13:24:09Z) - As indicated and expand also MSDU/MPDU to include an "and" or an "or" as appropriate. |

The resolution shown above (not yet approved by TGmb) shows a “conservative” response to this comment.  This has been speculatively edited in D2.02.

However, while performing this editing, the author noticed lots of places in Clause 11 (specifically 11.2 on power saving) where one or more of “MSDU, A-MSDU or MMPDU” were missing.

This begs the question of whether a more radical response is appropriate.

Furthermore,  the following comment resolution was approved in January 2010 (and edited in D2.03):

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 2070 | 621.5 | 11.2.1.0a | P | I disagree with the change of frame types from "management frames of the following subtypes: Action, Disassocation, Deauthentication, or Probe Response (when sent in response to a unicast Probe Request)" to "an Action frame", and want this unchanged in REVmb. | Restore D1.0 frame types for power save action. | AGREE IN PRINCIPLE (MAC: 2010-01-19 23:35:27Z).Insert the following paragraph within clause 11.2.1.0a: "A bufferable management frame is a unicast management frame, addressed to an associated STA, of the following subtypes: Action, Disassociation, Deauthentication, or Probe Response (when sent in response to a unicast probe request); or a group-addressed management frame of the following subtypes: Action, Disassociation, or Deauthentication. A bufferable MMPDU is an MMPDU that will be transmitted using a bufferable management frame." Wherever "Action frame" is used in clause 11.2.1, replace with "bufferable management frame". Wherever "MMPDU" is used in clause 11.2.1, replace with "bufferable MMPDU." Make changes for consistency or to remove and do away with redundancy following these changes. |

This modifes many of these locations to address specifically the issue of what types of management frame are bufferable.

The proposed resolution builds on this concept by introducing defined terms to replace what now becomes (after incorporation of CID 2070) “MSDU(s), A-MSDU(s) and/or [bufferable] MMPDU(s)”.

# Proposed Resolution

## Concept

Insert the following in 11.2.1.0a:

A bufferable unit(BU) is an item that is buffered to operate the power-saving protocol.

A BU is an MSDU, A-MSDU or bufferable MMPDU.

An individually-addressed BU is an individually-addressed MSDU, individually-addressed A-MSDU or individually-addressed bufferable MMPDU.

An group-addressed BU is an group-addressed MSDU or group-addressed bufferable MMPDU.

Throughout 11.2, replace any use of the term “MSDU, A-MSDU and/or bufferable MMPDU” with BU, adjusting for the plural form as necessary.  Likewise replace the term when qualified with individually-addressed or group-addressed with individually-addressed or group-addressed BU, as appropriate.

## Detail

Change 11.2 as follows.   Note, any edits that introduce technical changes are highlighted.

Changes are shown based on D2.03.

- Power management

- Power management in an infrastructure network

- General(#28)

A bufferable management frame is a unicast management frame, of the following subtypes: Action, Disassociation, Deauthentication, or Probe Response (when sent in response to a unicast probe request); or a group-addressed management frame of the following subtypes: Action, Disassociation, or Deauthentication.  A bufferable MMPDU is an MMPDU that will be transmitted using a bufferable management frame. A non-bufferable MMPDU is an MMPDU that is not a bufferable MMPDU.

A bufferable unit (BU) is an item that is buffered to operate the power-saving protocol. A BU is an MSDU, A-MSDU or bufferable MMPDU.

An individually-addressed BU is an individually-addressed MSDU, individually-addressed A-MSDU or individually-addressed bufferable MMPDU.

A group-addressed BU is an group-addressed MSDU or group-addressed bufferable MMPDU.

(#2211)STAs changing Power Management mode shall inform the AP of this fact using the Power Management bits within the Frame Control field of transmitted frames. A STA shall remain in its current Power Management mode until it informs the AP of a Power Management mode change via a frame exchange that includes an acknowledgment from the AP. Power Management mode shall not change during any single frame exchange sequence, as described in Annex S.(#2176)

The AP shall buffer individually addressed BUs addressed(Ed) to STAs operating in a PS mode. These buffered BUs shall be transmitted only at designated times.(#61)

If any STA in its BSS is in PS mode, the AP shall buffer all group addressed BUs and deliver them to all STAs immediately following the next Beacon frame containing a DTIM transmission.

The STAs that currently have buffered BUs within the AP are identified in a TIM, which shall be included as an element within all Beacon frames generated by the AP. A STA shall determine that  BU is buffered for it by receiving and interpreting a TIM.(#61)

STAs operating in PS modes shall periodically listen for Beacon frames, as determined by the STA's ListenInterval and the ReceiveDTIMs parameter in the MLME-POWERMGT.request primitive.

In a BSS operating under the DCF, or during the CP of a BSS using the PCF, upon determining that an BU is currently buffered in the AP, a STA operating in the PS mode  shall transmit a short PS-Poll frame to the AP, which shall respond with the corresponding buffered BU immediately, or acknowledge the PS-Poll and respond with the corresponding BU at a later time. If the TIM indicating the buffered BU is sent during a CFP, a CF-Pollable STA operating in the PS mode does not send a PS-Poll frame, but remains active until the buffered BUis received (or the CFP ends). (#61)(#2211)

- “with another STA” below is redundant because only a STA can set up DLS, and it cannot do that with itself as target.

- As there are implicitly two QoS STA associated with any DLS link, references to “the QoS STA” are ambiguous and need to be qualified.

A non-AP(#2224) QoS STA may be in PS mode before the setup of DLS or Block Ack. Once DLS is set up with another (#1112)STA, the (#1112) QoS(11n) STA suspends the PS mode and shall(#1478) be awake. When a STA enters normal (non-APSD) PS mode, any downlink Block Ack agreement without an associated schedule is suspended for the duration of this PS mode. BUs for a(Ed) TID without a schedule are sent in a frame using Normal Ack following a PS-poll as described in rest of this clause. Uplink Block Ack, Block Acks for any TID with a schedule, and any Block Acks to APSD STA continue to operate normally.

- STA Power Management modes

A STA may be in one of two different power states:

- Awake: STA is fully powered.

- Doze: STA is not able to transmit or receive and consumes very low power.

The manner in which a STA transitions between these two power states shall be determined by the STA’s Power Management mode and reflected in (#1706,Ed)dot11PowerManagementMode(#1005). These modes are summarized in Power Management modes.

| - Power Management modes |  |
| --- | --- |
| Active mode or AM | STA may receive frames at any time. In Active mode, a STA shall be in the Awake state. A STA on the polling list of a PCF shall be in Active mode for the duration of the CFP. |
| PS | STA listens to selected Beacon frames (based upon the ListenInterval parameter of the MLME-ASSOCIATE.request primitive) and sends PS-Poll frames to the AP if the TIM element(Ed) in the most recent Beacon frame indicates an individually-addressed BU is(#61) buffered for that STA. The AP shall transmit buffered individually-addressed BUs to a PS STA only in response to a PS-Poll from that STA, or during the CFP in the case of a CF-Pollable PS STA. In PS mode, a STA shall be in the Doze state and shall enter the Awake state to receive selected Beacon frames, to receive group addressed(#1359) transmissions following certain received Beacon frames, to transmit, and to await responses to transmitted PS-Poll frames or (for CF-Pollable STAs) to receive CF transmissions of buffered BUs.(#61) |

The Power Management mode of a STA is selected by the PowerManagementMode parameter of the MLME-POWERMGT.request primitive(#2172). Once the STA updates its Power Management mode, the MLME shall issue an MLME-POWERMGT.confirm primitive(#2172) indicating the success of the operation.

To change Power Management modes, a STA shall inform the AP through a successful frame exchange as described in Annex S(Ed) initiated by the (#1112)STA and that includes an ACK from the AP. The Power Management bit in the Frame Control field of the frame sent by the STA in this exchange indicates the Power Management mode that the STA shall adopt upon successful completion of the entire frame exchange. The Power Management bit shall be ignored in frame exchanges initiated by the AP. A non-AP STA shall not change power management mode(#2091) using a frame exchange that does not receive an ACK.(#1362) The Power Management bit shall be set to zero in all management frames that are not bufferable(#2222) management frames.(#61)

A STA that is changing from Doze to Awake in order to transmit shall perform CCA until a frame sequence is detected by which it can correctly set its NAV, or until a period of time equal to the ProbeDelay has transpired.

- AP TIM transmissions

The TIM shall identify the STAs for which traffic is pending and buffered in the AP. This information is coded in a partial virtual bitmap, as described in 7.3.2.6 (TIM). In addition, the TIM contains an indication whether group addressed(#1359) traffic is pending. Every STA is assigned an AID by the AP as part of the association process. AID 0 (zero) is reserved to indicate the presence of buffered group addressed(#1359) BUs. The AP shall identify those STAs for which it is prepared to deliver buffered BUs by setting bits in the TIM’s partial virtual bitmap that correspond to the appropriate AIDs.

- TIM types

Two different TIM types are distinguished: TIM and DTIM. After a DTIM, the AP shall transmit(#1485) buffered group addressed(#1359) BUs(#1485), before transmitting any individually addressed(#1359) frames.

The AP shall transmit a TIM with every Beacon frame. Every dot11DTIMPeriod(#1005), a TIM of type DTIM is transmitted within a Beacon frame, rather than an ordinary TIM.

Error: Reference source not found illustrates the AP and STA activity under the assumptions that no PCF is operating and that a DTIM is transmitted once every three TIMs. The top line in Error: Reference source not found represents the time axis, with the beacon interval shown together with a DTIM Interval of three beacon intervals. The second line depicts AP activity. The AP schedules Beacon frames for transmission every beacon interval, but the Beacon frames may be delayed if there is traffic at the TBTT. This is indicated as “busy medium” on the second line. For the purposes of this figure, the important fact about Beacon frames is that they contain TIMs, some of which are DTIMs. Note that the second STA with ReceiveDTIMs set to false does not power-on its receiver for all DTIMs.

The third and fourth lines in Error: Reference source not found depict the activity of two STAs operating with different power management requirements. Both STAs power-on their receivers when they need to listen for a TIM. This is indicated as a ramp-up of the receiver power prior to the TBTT. The first STA, for example, powers up its receiver and receives a TIM in the first Beacon frame; that TIM indicates the presence of a bufferedUBU) for the receiving STA. The receiving STA then generates a PS-Poll frame, which elicits the transmission of the buffered B) from the AP. Group addressed(#1359) BUs are sent by the AP subsequent to the transmission of a Beacon frame containing a DTIM. The DTIM is indicated by the DTIM count field of the TIM element having a value of 0.

- Power management with APSD

QoS APs capable of supporting automatic power save delivery (APSD) shall signal this capability through the use of the APSD subfield in the Capability Information field in Beacon, Probe Response, and (Re)Association Response management frames.

(#1112)QoS STAs use the Power Management field in the Frame Control field of a frame to indicate whether it is in active or PS mode. As APSD is a mechanism for the delivery of downlink data and bufferable management frames to power-saving STAs, the frames transmitted by(Ed) a (#1112)STA using APSD shall have the Power Management bit in the Frame Control field set to 1 for buffering to take place at the AP.

APSD defines two delivery mechanisms, namely unscheduled APSD (U-APSD) and scheduled APSD (SAPSD). (#1112)STAs may use U-APSD to have some or all of their BUs delivered during unscheduled SPs. (#1112)STAs may use S-APSD to schedule delivery of some or all of their BUs during scheduled SPs.

If there is no unscheduled SP in progress, the unscheduled SP begins when the AP receives a trigger frame from a (#1112)STA, which is a QoS data or QoS Null frame using(#1146) an AC the STA has configured to be trigger-enabled. An A-MPDU that contains one or more trigger frames acts as a trigger frame.(11n) An unscheduled SP ends after the AP has attempted to transmit at least one BU using(#1146) a delivery-enabled AC and destined for the (#1112)STA, but no more than the number indicated in the Max SP Length field if the field has a nonzero value.

In order to configure an AP to deliver BUs during an unscheduled SP, a (Ed)(#1112)STA designates one or more of its ACs to be delivery-enabled and one or more of its AC to be trigger-enabled. A (#1112)STA may configure an AP to use U-APSD using two methods. First, the (#1112)STA may set individual U-APSD Flag bits in the QoS Info subfield of the QoS Capability element carried in (Re)Association Request frames. When a U-APSD Flag bit is 1(#1627), it indicates that the corresponding AC is both delivery- and trigger-enabled. When all four U-APSD Flag subfields are set to 1 in (Re)Association Request frames, all the ACs associated with the (#1112)STA are trigger- and delivery-enabled during (re)association. When all four U-APSD Flag subfields are set to 0 in (Re)Association Request frames, none of the ACs associated with the (#1112)STA is trigger- or delivery-enabled during (re)association.

NOTE—The delivery of bufferable MMPDUs is controlled by AC_VO.

Alternatively, the (#1112)STA may designate one or more AC as trigger-enabled and one or more AC as delivery-enabled by sending an ADDTS Request frame per AC to the AP with the APSD subfield set to 1 and the Schedule subfield set to 0 in the TS Info field in the TSPEC element. APSD settings in a TSPEC request take precedence over the static U-APSD settings carried in the QoS Capability element. In other words, a TSPEC request overwrites any previous U-APSD setting of an AC. The request may be sent for ACs for which the ACM subfield is(#1217) 0.

A (#1112)STA may set an AC to be trigger- or delivery-enabled for its own use by setting up TSPECs with the APSD subfield set to 1 and the Schedule subfield set to 0 in the uplink or downlink direction, respectively. An uplink TSPEC plus a downlink TSPEC, or a bidirectional(Ed) TSPEC with the APSD subfield set to 1 and the Schedule subfield set to 0, makes an AC both trigger- and delivery-enabled. An uplink TSPEC plus a downlink TSPEC, or a bidirectional(Ed) TSPEC with the APSD and the Schedule subfields both set to 0, makes an AC neither trigger- nor delivery-enabled.

A scheduled SP starts at fixed intervals of time specified in the Service Interval field. In order to use a scheduled SP for a TS when the access policy is controlled channel access, a (#1112)STA shall send an ADDTS Request frame to the AP with the APSD subfield of the TS Info field in the TSPEC element set to 1. To use a scheduled SP for a TS for a AC when the access policy is contention-based channel access, a (#1112)STA shall send an ADDTS Request frame to the AP with the APSD and Schedule subfields of the TS Info field in the TSPEC element both set to 1. If the APSD mechanism is supported by the AP and the AP accepts the corresponding ADDTS Request frame from the (#1112)STA, the AP shall respond to the ADDTS Request frame with a response containing the Schedule element indicating that the requested service can be accommodated by the AP. The first scheduled SP starts when the lower order 4 octets of the TSF timer equals the value specified in the Service Start Time field. A (#1112)STA using scheduled SP shall first wake up to receive downlink individually addressed(#1359) BUs  buffered and/or polls from the AP/HC. The STA shall wake up subsequently at a fixed time interval equal to the SI. The AP may modify the service start time by indicating so in the Schedule element in a successful(#1127)(#1486) ADDTS Response frame (which is sent in response to an ADDTS Request frame)(#1486) and in Schedule frames (which are sent at other times)(#1486).

A scheduled SP begins at the scheduled wakeup time that corresponds to the SI and the service start time indicated in the Schedule element sent in response to a TSPEC. The STA shall wake up at a subsequent time when

(TSF – service start time) mod minimum SI = 0.

If scheduled services periods are supported in a BSS, a STA may use both unscheduled and scheduled APSD on different ACs at the same time. When a (#1112)STA establishes scheduled delivery for an AC, that AC shall be considered delivery-enabled. However, the AP shall not transmit BUs using(#1146) that AC during an SP that is initiated by a trigger frame, and it shall not treat BUs using(#1146) the AC that are received from the STA as trigger frames. The AP shall decline any ADDTS Request frame that indicates the use of both scheduled and unscheduled APSD to be used on the same AC at the same time.

APSD shall be used only to deliver individually addressed(#1359) BUs. Group addressed(#1359) BU delivery shall follow the frame delivery rules defined for group addressed(#1359) BUs as defined in AP operation during the CFP.

- AP operation during the CP

APs shall maintain a Power Management status for each currently associated STA that indicates in which Power Management mode the STA is currently operating. APs that implement and signal their support of APSD shall maintain an APSD and an access policy status for each currently associated (#1112)STA that indicates whether the (#1112)STA is presently using APSD and shall maintain the schedule (if any) for the (#1112)STA. An AP shall, depending on the Power Management mode of the STA, temporarily buffer BUs destined to the STA. An AP implementing APSD shall, if a (#1112)STA is using APSD and is in PS mode, temporarily buffer BUs destined to that (#1112)STA. BUs addressed directly to STAs operating in the Active mode shall be buffered for power management reasons.

- BUs destined for PS STAs, shall be temporarily buffered in the AP. (#1128)The algorithm to manage this buffering is beyond the scope of this standard, with the exception that if the AP is QoS-enabled, it shall preserve the order of arrival of frames on a per-TID, per-STA basis.

- Non-bufferable MMPDUs and BUs destined for STAs in the Active mode, shall be directly transmitted to those STAs.

- At every beacon interval, the AP shall assemble the partial virtual bitmap containing the buffer status per destination for STAs in the PS mode and shall send this out in the TIM field of the Beacon frame. At every beacon interval, the APSD-capable AP shall assemble the partial virtual bitmap containing the buffer status of nondelivery-enabled ACs (if there exists at least one nondelivery-enabled AC) per destination for (#1112)STAs in PS mode and shall send this out in the TIM field of the Beacon frame. When all ACs are delivery-enabled, the APSD-capable AP shall assemble the partial virtual bitmap containing the buffer status for all ACs per destination(#1112).

- If a (#1112)STA has set up a scheduled SP, it shall automatically wake up at each SP. Therefore, the APSD-capable AP shall transmit frames associated with admitted traffic with the APSD subfield set to 1 in the TSPECs buffered for the (#1112)STA during a scheduled SP. If the (#1112)STA has set up to use unscheduled SPs, the AP shall buffer BUs(#1488) using(#1146) delivery-enabled ACs until it has received a trigger frame using(#1146) a trigger-enabled AC from the nonAP STA, which indicates the start of an unscheduled SP. A trigger frame received by the AP from a (#1112)STA that already has an unscheduled SP underway shall not trigger the start of a new unscheduled SP. The AP transmits BUs destined for the (#1112)STA and using(#1146) delivery-enabled ACs during an unscheduled SP. The bit for AID 0 (zero) in the bit map control field of the TIM (#1684)element(#31) shall be set to 1 when  traffic is buffered, according to 7.3.2.6 (TIM).

- If any associated STAs are in PS mode, all group addressed(#1359) BUs in which the Order bit in the Frame Control field is 0(#1627) shall be buffered.(Ed)

- (#2211)Immediately after every DTIM, the AP shall transmit all buffered group addressed(#1359) BUs. The More Data field of each group addressed(#1359) frame shall be set to indicate the presence of further buffered group addressed(#1359) BUs. If the AP is unable to transmit all of the buffered group addressed(#1359) BUs before the primary or secondary(11n) TBTT following the DTIM, the AP shall indicate that it will continue to deliver the group addressed(#1359) BUs by setting the bit for AID 0 (zero) in the Bit Map Control(11n) field of the TIM element of every Beacon frame, until all buffered group addressed(#1359) BUs have been transmitted. When the AP transmits an STBC DTIM or TIM Beacon frame, the AP shall retransmit all group-addressed BUs that were transmitted following the non-STBC DTIM or TIM Beacon frame except that they are transmitted using the basic STBC MCS. It may be the case that a complete set of buffered group-addressed BUs is sent over a period of time during which non-STBC and STBC transmissions are interleaved, but the transition from non-STBC group-addressed transmissions to STBC group-addressed transmissions shall be preceded by the transmission of an STBC Beacon frame and the transition from STBC group-addressed transmissions to non-STBC group-addressed transmissions shall be preceded by the transmission of a non-STBC Beacon frame.

- A single buffered BU for a STA in the PS mode shall be forwarded to the STA after a PS-Poll has been received from that STA. For a (#1112)STA(#1489) using U-APSD, the AP transmits one frame destined for the (#1112)STA from any AC that is not delivery-enabled in response to PS-Poll from the (#1112)STA. When all ACs associated with the (#1112)STA are delivery-enabled, AP transmits one BU from the highest priority AC. The AP can respond with either an immediate data or management frame or with an ACK, while delaying the responding data or management(11n) frame.

For a STA in PS mode and not using U-APSD, the More Data field of the response data(11n) or management frame shall be set to indicate the presence of further buffered BUs for the polling STA. For a (#1112)STA using U-APSD, the More Data field shall be set to indicate the presence of further buffered BUs that do not use(#1146) delivery-enabled ACs. When all ACs associated with the (#1112)STA are delivery-enabled, the More Data field shall be set to indicate the presence of further buffered BUs using(#1146) delivery-enabled ACs. If there are buffered BUs to transmit to the STA, the AP may set the More Data bit in a QoS +CF-Ack frame to 1, in response to a QoS data frame to indicate that it has one or more pending BUs buffered for the PS STA identified by the RA (11n)in the QoS +CF-Ack frame. An AP may also set the More Data bit in an ACK frame to 1(#1627) in response to a QoS data frame to indicate that it has one or more pending BUs buffered for the PS STA identified by the RA (11n)in the ACK frame, if that PS STA has set the More Data Ack subfield in the QoS Capability (#1684)element to 1.

Further PS-Poll frames from the same STA shall be acknowledged and ignored until the BU has either been successfully delivered or presumed failed due to maximum retries being exceeded. This prevents a retried PS-Poll from being treated as a new request to deliver a buffered BU.

- At each scheduled APSD SP for a (#1112)STA, the APSD-capable AP shall attempt to transmit at least one BU, using(#1146) admitted TSPECs with the APSD and Schedule subfields both set to 1, that are destined for the (#1112)STA. At each unscheduled SP for a (#1112)STA, the AP shall attempt to transmit at least one BU, but no more than the value specified in the Max SP Length field in the QoS Capability element from delivery-enabled ACs, that are destined for the (#1112)STA.

The More Data bit of the directed data or bufferable management(#2222) frame using(#1146) delivery-enabled ACs and destined for that (#1112)STA indicates that more BUs are buffered for the delivery-enabled ACs. The More Data bit equal to 1(#1627) in data or bufferable management(#2222)(#1487) frames using(#1146) nondelivery-enabled ACs and destined for that (#1112)STA indicates that more BUs are buffered for the nondelivery-enabled ACs. For all frames except for the final frame of the SP, the EOSP subfield of the QoS Control field of the QoS data frame shall be set to 0 to indicate the continuation of the SP. An AP may also set the More Data bit to 1 in a QoS +CF-Ack frame in response to a QoS data frame to indicate that it has one or more pending BUs buffered for the target STA identified by the RA (11n)in the QoS +CF-Ack frame. If the QoS data frame is using(#1146) a delivery-enabled AC, the More Data bit in the QoS +CF-Ack frame indicates more BUs for  delivery-enabled ACs. If the QoS data frame is not using(#1146) a delivery-enabled AC, the More Data bit in the QoS +CF-Ack frame indicates more BUs for ACs that are not delivery-enabled.

The AP considers an(Ed) APSD STA to be in Awake state after it has sent a QoS +CF-Ack frame, with the EOSP subfield in the QoS Control field set to 0, to the APSD STA. If necessary, the AP may generate an extra QoS Null frame, with the EOSP set to 1. When the AP has transmitted a directed frame to the (#1112)STA with the EOSP subfield set to 1 during the SP except for retransmissions of that frame, the AP shall not transmit any more frames to that STA(11n) using this mechanism until the next SP. The AP shall set the EOSP subfield to 1 to indicate the end of the SP in APSD.

- If the AP does not receive an acknowledgment to a directed data(#1487)(#1494) or bufferable management(#2222) frame sent to a (#1112)STA in PS mode following receipt of a PS-Poll from that (#1112)STA, it may retransmit the frame for at most the lesser of the maximum retry limit and (#1706)dot11QAPMissingAckRetryLimit times before the next Beacon frame, but it shall retransmit that frame at least once before the next Beacon frame, time permitting and subject to its appropriate lifetime limit. If an acknowledgment to the retransmission is not received, it may wait until after the next Beacon frame to further retransmit that frame subject to its appropriate lifetime limit.

- If the AP does not receive an acknowledgment to a directed data frame(#1494) containing all or part of an MSDU or A-MSDU(11n) sent with the EOSP subfield set to 1, it shall retransmit that frame at least once within the same SP, subject to applicable retry or lifetime limit. The maximum number of retransmissions within the same SP is the lesser of the maximum retry limit and (#1706)dot11QAPMissingAckRetryLimit. If an acknowledgment to the retransmission of this last frame in the same SP is not received, it may wait until the next SP to further retransmit that frame, subject to its applicable retry or lifetime limit.

- An AP can delete buffered BUs for implementation-dependent reasons, including the use of an aging function and availability of buffers. The AP may base the aging function on the listen interval specified by the (#1112)STA in the (Re)Association Request frame.

- When an AP is informed that a STA changes to the Active mode, then the AP shall send buffered BUs (if any exist) to that STA without waiting for a PS-Poll. When an AP is informed that an APSD-capable (#1112)STA is not using APSD, then the AP shall send buffered BUs (if any exist) to that (#1112)STA according to the rules corresponding to the current PS mode of the (#1112)STA.

- AP operation during the CFP

APs shall maintain a Power Management status for each currently associated CF-Pollable STA that indicates in which Power Management mode the STA is currently operating. An AP shall, for STAs in PS mode, temporarily buffer BUs addressed(Ed) to the STA.(#61)

- BUs destined for PS STAs shall be temporarily buffered in the AP. The algorithm to manage this buffering is beyond the scope of this standard.(#61)

- Non-bufferable MMPDUs(#2222) and all BUs destined to STAs in the Active mode shall be transmitted as defined in Clause 9 (MAC sublayer functional description).(#61)

- Prior to every CFP, and at each beacon interval within the CFP, the AP shall assemble the partial virtual bitmap containing the buffer status per destination for STAs in the PS mode, set to 1(#1627) the bits in the partial virtual bitmap for STAs the PC is intending to poll during this CFP, and shall send this out in the TIM field of the DTIM. The bit for AID 0 (zero) in the Bit Map Control field of the TIM (#1684)element(#31) shall be set to 1(#1627) when  group-addressed(11n) traffic is buffered, according to 7.3.2.6 (TIM).

- All group addressed(#1359) MSDUs in which the Order bit in the Frame Control field is 0(#1627) shall be buffered if any associated STAs are in the PS mode, regardless of(11n) whether those STAs are CF-Pollable.

- Frames are not the unit of buffering. The first sentence is incorrect. This also fails to take into account buffered MMPDUs.

- Immediately after every DTIM (Beacon frame with DTIM Count field of the TIM element equal to zero), the AP shall transmit all buffered group addressed(#1359) frames. The More Data field shall be set to 1(#1627) in the headers of all but the final such frame to indicate the presence of further buffered group addressed(#1359) BUs. If the AP is unable to transmit all of the buffered group addressed(#1359) BUs before the non-STBC or STBC(11n) TBTT following the DTIM, the AP shall indicate that it will continue to deliver the group addressed(#1359) BUs by setting the bit for AID 0 (zero) in the Bit Map Control(11n) field of the TIM element of every Beacon frame, until all buffered group addressed(#1359) frames have been transmitted. When the AP transmits an STBC DTIM or TIM Beacon frame, the AP shall retransmit all group-addressed BUs(Ed) that were transmitted following the non-STBC DTIM or TIM Beacon frame except that they are transmitted using the basic STBC MCS. It may be the case that a complete set of buffered group-addressed BUs(Ed) is sent over a period of time during which non-STBC and STBC transmissions are interleaved, but the transition from non-STBC group-addressed transmissions to STBC group-addressed transmissions shall be preceded by the transmission of a STBC Beacon frame and the transition from STBC group-addressed transmissions to non-STBC group-addressed transmissions shall be preceded by the transmission of a non-STBC Beacon frame.

- Buffered BUs for STAs in the PS mode shall be forwarded to the CF-Pollable STAs under control of the PC. Transmission of these buffered BUs) as well as CF-Polls to STAs in the PS mode that were indicated in the DTIM in accordance with Prior to every CFP, and at each beacon interval within the CFP, the AP shall assemble the partial vi of this subclause shall begin immediately after transmission of buffered group addressed(#1359) frames (if any), and shall occur in order by increasing AID of CF-Pollable STAs. A CF-Pollable STA for which the TIM element of the most recent Beacon frame indicated buffered BUs  shall be in the Awake state at least until the receipt of a directed frame from the AP in which the Frame Control field does not indicate the existence of more buffered BU). After acknowledging the last of the buffered BU), the CF-Pollable STA operating in the PS mode may enter the Doze state until the next DTIM is expected.

- An AP shall have an aging function to delete pending traffic buffered for an excessive time period. The exact specification of the aging function is beyond the scope of this standard.

- When an AP detects that a CF-Pollable STA has changed from the PS mode to the Active mode, then the AP shall queue any buffered frames addressed to that STA for transmission to that CF-Pollable STA as directed by the AP’s PC.

- Receive operation for STAs in PS mode during the CP

A STA(Ed) in PS mode shall operate as follows to receive a BU from the AP when no PC is operating and during the CP when a PC is operating.

- The STA(Ed) shall wake up early enough to be able to receive the first Beacon frame scheduled for transmission after the time corresponding to the last TBTT plus the STA’s current ListenInterval.

- When the(Ed) STA detects that the bit corresponding to its AID is 1(#1627) in the TIM, the STA shall issue a PS-Poll to retrieve the buffered BU. The PS-Poll shall be transmitted after a random delay uniformly distributed between zero and aCWmin slots following a DIFS.

- The STA shall remain in the Awake state until it receives the BU in response to its poll or it receives another Beacon frame whose TIM indicates that the AP does not have any BUs buffered for this STA. If the bit corresponding to the STA’s AID is 1(#1627) in the subsequent TIM, the STA shall issue another PS-Poll to retrieve the buffered BU. When a (#1112)STA that is using U-APSD and has all ACs delivery-enabled detects that the bit corresponding to its AID is 1(#1627) in the TIM, the (#1112)STA shall issue a trigger frame or a PS-Poll frame to retrieve the buffered BU.

- (#2211)If the More Data field in the received data or bufferable management(#2222)(#1487) frame indicates that more traffic for that STA is buffered, the STA, at its convenience, shall Poll until no more BUs are buffered for that STA.

- When ReceiveDTIMs is true, the STA shall wake up early enough to be able to receive either every non-STBC DTIM or every STBC DTIM(11n) sent by the AP of the BSS. A STA that stays awake to receive group addressed(#1359) BUs shall elect to receive all group-addressed non-STBC transmissions or all group-addressed STBC transmissions and(11n) remain awake until the More Data field of the appropriate type (non-STBC or STBC) of(11n) group addressed(#1359) BUs indicates there are no further buffered group addressed(#1359) BUs of that type(11n) or until a TIM is received indicating there are no more buffered group addressed(#1359) BUs of that type(11n). If a (#1112)STA receives a QoS +CF-Ack frame from its AP with the More Data bit set to 1, then the STA shall operate exactly as if it received a TIM with its AID bit equal to 1(#1627). If a (#1112)STA has set the More Data Ack subfield in QoS Capability (#1684)element to 1, then if it receives an ACK frame from its AP with the More Data bit set to 1, the STA shall operate exactly as if it received a TIM with its AID bit equal to 1(#1627). For example, a STA that is using the PS-Poll delivery method shall issue a PS-Poll frame to retrieve a buffered BU. See also 9.2.7 (Group addressed(#1359) MPDU transfer procedure).

- Receive operation for STAs in PS mode during the CFP

A STA in PS mode that is(Ed) associated as CF-Pollable shall operate as follows in a BSS with an active PC to receive BUs from the AP during the CFP:

- The STA shall enter the Awake state so as to receive the Beacon frame (which contains a DTIM) at the start of each CFP.

- To receive group addressed(#1359) BUs, the STA shall wake up early enough to be able to receive either every non-STBC DTIM or every STBC DTIM(11n) that may be sent during the CFP. A STA receiving group addressed(#1359) BUs shall elect to receive all group-addressed non-STBC transmissions or all group-addressed STBC transmissions and(11n) remain awake until the More Data field of the group addressed(#1359) BUs indicates there are no further buffered group addressed(#1359) BUs of that type(11n), or until a TIM is received indicating there are no more group addressed(#1359) BUs of that type(11n) buffered. See also 9.2.7 (Group addressed(#1359) MPDU transfer procedure).

- When the(Ed) STA detects that the bit corresponding to its AID is 1(#1627) in the DTIM at the start of the CFP (or in a subsequent TIM during the CFP), the STA shall remain in the Awake state for at least that portion of the CFP through the time that the STA receives a directed BU from the AP carried in a frame with the More Data field in the Frame Control field indicating that no further traffic is buffered.

- (#2211)If the More Data field in the Frame Control field of the last data(#1494) or bufferable management(#2222)(#1487) frame received from the AP indicates that more traffic for the STA is buffered, then, when the CFP ends, the STA may remain in the Awake state and transmit PS-Poll frames during the CP to request the delivery of additional buffered BUs, or may enter the Doze state during the CP (except at TBTTs for DTIMs expected during the CP), awaiting the start of the next CFP.

- Receive operation using APSD(#1112)

A (#1112)STA using APSD shall operate as follows to receive  a BU from the AP:

- If a scheduled SP has been set up, the (#1112)STA wakes up at its scheduled start time. (The (#1112)STA shall wake up early enough to receive transmissions at the scheduled SP.)

- If the (#1112)STA is initiating an unscheduled SP, the (#1112)STA wakes up and transmits a trigger frame to the AP. When one or more ACs are not delivery-enabled, the (#1112)STA may retrieve BUs using(#1146) those ACs by sending PS-Poll frames to the AP.

- The (#1112)STA shall remain awake until it receives a QoS data frame or bufferable management(#2222) frame(#1499) addressed to it, with the EOSP subfield in the QoS Control field set to 1.

- The (#1112)STA may send additional PS-Poll frames if the More Data subfield is(#1217) 1 in downlink individually addressed(#1359) data or bufferable management(#2222)(#1487) frames that do not use(#1146) any (#1500)delivery-enabled ACs. The (#1112)STA may send additional trigger frames if the More Data subfield is(#1217) 1 in downlink individually addressed(#1359) data or bufferable management(#2222)(#1487) frames that use(#1146) delivery-enabled ACs.

- STAs operating in the Active mode

A STA operating in this mode shall have its receiver activated continuously; such STAs do not need to interpret the TIM (#1684)elements in Beacon frames.

- AP aging function

Any AP aging function shall not cause the buffered BUs to be discarded after any period that is shorter than the ListenInterval of the STA for which the BUs are buffered. The exact specification of the aging function is beyond the scope of this standard.

- PSMP power management(11n)

An AP transmits a PSMP frame containing a schedule only for STAs that are awake.

NOTE—A STA in power save mode is awake as defined in Power management with APSD(U-APSD, S-APSD), in AP operation during the CP (PS-poll), or during a DTIM period.

The AP may signal the end of the SP for all awake associated PSMP-capable STAs by setting the More PSMP field to 0 or by sending CF-End frame instead of the next PSMP frame.

NOTE 1—The AP can also signal the end of an SP on a per-STA basis using the EOSP field set to 1 in the QoS Control field, as defined in 7.1.3.5.2 (EOSP (end of service period) subfield) and AP operation during the CP. This field remains set to 1 for any retransmissions of the same frame, and no more new frames are sent to this particular STA in the current SP.

NOTE 2—If a STA is awake at the start of a scheduled PSMP session, the operation of the More Data field in the Frame Control field and the TIM element are defined by the S-APSD rules in Power management with APSD, AP operation during the CP, and Receive operation using APSD.

A STA shall wake up at the start of the next PSMP frame if the More PSMP field is set to 1 in the current PSMP frame, unless the STA has been permitted to return to sleep through the reception of a frame addressed to it with the EOSP field set to 1 or the maximum SP interval has elapsed.

A PPDU containing MPDUs addressed to a STA shall not start after expiry of the STA’s PSMP-DTT. A STA completes the reception of any PPDU that starts before the end of the PSMP-DTT. If no frames addressed to a STA begin within a PSMP-DTT, it can assume that no frame addressed to it will arrive during this PSMP sequence.

The STA shall be awake to receive at the start of the PSMP-DTT determined from a STA_INFO field that has the STA_INFO Type subfield set to 2 and the AID field matching the STA’s AID where the PSMP-DTT Duration subfield is not set to 0.

- Power management in an IBSS

- Introduction(#28)

Subclause Power management in an IBSS specifies the power management mechanism for use within an IBSS.

- Basic approach

The basic approach is similar to the infrastructure case in that the STAs are synchronized, and group addressed(#1359) BUs and those BUs that are to be transmitted to a power-conserving STA are first announced during a period when all STAs are awake. The announcement is done via an (#1138)ATIM sent in an ATIM Window. A STA in the PS mode shall listen for these announcements to determine if it needs to remain in the Awake state. The presence of the ATIM window in the IBSS indicates if the STA may use PS mode(11n). To maintain correct information on the power save state of other STAs in an IBSS, a STA needs to remain awake during the ATIM window. At other times the STA may enter the Doze state except as indicated in the following procedures.(#1130)

- The following para “during which only Beacon or ATIM frames shall be transmitted“ does not allow Null Data during the ATIM window, which conflicts with the resolution of CID 1131

When an BU is to be transmitted to a destination STA that is in a PS mode, the transmitting STA first transmits an ATIM frame during the ATIM Window, in which all the STAs including those operating in a PS mode are awake. The ATIM Window is defined as a specific period of time, defined by the value of the ATIM Window parameter in the IBSS Parameter Set supplied to the MLME-START.request primitive, following a TBTT, during which only Beacon or ATIM frames shall be transmitted. ATIM transmission times are randomized, after a Beacon frame is either transmitted or received by the STA, using the backoff procedure with the CW equal to aCWmin. Directed ATIMs shall be acknowledged. If a STA transmitting a directed ATIM does not receive an acknowledgment, the STA shall execute the backoff procedure for retransmission of the ATIM. Group addressed(#1359) ATIMs shall not be acknowledged.

If a STA receives a directed ATIM frame during the ATIM Window, it shall acknowledge the directed ATIM and stay awake for the entire beacon interval to receive(#1131) the announced BU. If a STA does not receive an ATIM, it may enter the Doze state at the end of the ATIM Window. Transmissions of BUs announced by ATIMs are randomized after the ATIM Window, using the backoff procedure described in Clause 9 (MAC sublayer functional description).

It is possible that an ATIM may be received from more than one STA, and that a STA that receives an ATIM may receive more than a single BU from the transmitting STA. ATIM frames are only addressed to the destination STA of the BU.

An ATIM for a  BU shall have a destination address identical to that of the BU.

After the ATIM interval, only those individually-addressed BUs that have been successfully announced with an acknowledged ATIM, and group-addressed(#1359) BUs that have been announced with an ATIM, shall be transmitted to STAs in the PS mode. These frames shall be transmitted using the DCF (for non-QoS STAs)(#1502) or EDCAF(#1485) (for QoS STAs)(#1502).

Error: Reference source not found illustrates the basic PS operation.

The estimated power-saving state of another STA may be based on the power management information transmitted by that STA and on additional information available locally, such as a history of failed transmission attempts. The use of RTS/CTS in an IBSS may reduce the number of transmissions to a STA that is in PS mode. If an RTS is sent and a CTS is not received, the transmitting STA may assume that the destination STA is in PS mode. The method of estimating the power management state of other STAs in the IBSS is outside the scope of this standard.

- Initialization of power management within an IBSS

The following procedure shall be used to initialize power management within a new IBSS, or to learn about the power management being used within an existing IBSS.

- A STA joining an existing IBSS by the procedure in Error: Reference source not found(#1598) shall update its ATIM Window with the value contained in the ATIM Window field of the IBSS Parameter Set element within the Beacon or Probe Response management frame received during the scan procedure.

- A STA creating a new IBSS by the procedure in Error: Reference source not found(#1598) shall set the value of the ATIM Window field of the IBSS Parameter Set element within the Beacon management frames transmitted to the value of its ATIM Window.

- The start of the ATIM Window shall be the TBTT, defined in Error: Reference source not found(#1598). The end of the ATIM Window shall be defined as

TSF timer MOD dot11BeaconInterval = ATIMWindow, where ATIMWindow is the value of the ATIM Window parameter of the IBSS Parameter Set from the MLME-Start.request or MLME-JOIN.request primitives.

- The ATIM Window period shall be static during the lifetime of the IBSS.

- An ATIM Window value of zero shall indicate that power management is not usable within the IBSS.

- STA power state transitions

A STA may enter PS mode if the value of the ATIM window in use within the IBSS is greater than zero. A STA shall not enter PS mode if the value of the ATIM window in use within the IBSS is equal to zero.(#20) A STA shall set the Power Management subfield in the Frame Control field of data frames(Ed) containing all or part of a BU that it transmits using the rules in 7.1.3.1.6 (Power Management field).

A STA in PS mode shall transition between Awake and Doze states according to the following rules:

- If a STA is operating in PS mode, it shall enter the Awake state prior to each TBTT.

- If a STA receives a (#1131)group addressed(#1359) ATIM management frame during the ATIM Window, the STA(#1131) shall remain in the Awake state until either the STA receives a group addressed frame from the source STA with the More Data field set to 0 or(#1131) the end of the next ATIM Window, whichever occurs first(#1131).

- If a STA receives at least one directed ATIM management frame containing the STA’s individual address in the RA field during the ATIM Window then the STA shall remain in the Awake state at least until the earlier of the completion of the successful transmission to and reception from the source STA of each received ATIM, a frame with EOSP field set to one, and the end of the next ATIM Window(#1131).

- If a STA transmits at least one directed ATIM management frame containing a STA’s individual address in the RA field during the ATIM Window then the STA shall remain in the Awake state at least until the earlier of the completion of the successful transmission to and reception from the destination STA of each transmitted ATIM, a frame with EOSP field set to one, and the end of the next ATIM Window(#1131).

- If a STA transmits a Beacon(#1131), the STA(#1131) shall remain in the Awake state until the end of the next ATIM Window(#1131).

- If the STA has not transmitted an ATIM and does not receive either a directed ATIM management frame containing the STA’s(#1131) individual address, or a group addressed(#1359) ATIM management frame during the ATIM Window, the STA(#1131) may return to the Doze state following the end of the current ATIM Window.

- ATIM and frame transmission

If power management is in use within an IBSS, all STAs shall buffer individually-addressed BUs for STAs that are known to be in PS mode. BUs may be sent to STAs in Active mode at any valid time.

- Following the reception or transmission of the Beacon frame, during the ATIM Window, the STA shall transmit a directed ATIM management frame to each STA for which it has one or more buffered individually addressed BUs. If the STA has one or more buffered group addressed(#1359) MSDUs, with the Strictly Ordered bit equal to 0(#1627), or has one or more buffered group-addressed MMPDUs, it shall transmit an appropriately addressed group addressed(#1359) ATIM frame.(#1131)

- All STAs shall use the backoff procedure defined in 9.2.5.2 (Backoff procedure for DCF) for transmission of the first ATIM following the Beacon frame. All remaining ATIMs shall be transmitted using the conventional DCF access procedure.

- ATIM management frames shall be transmitted only(#1503) during the ATIM Window.

- A STA shall transmit no frame types other than RTS, CTS, and ACK Control frames, (#1131)Beacon and ATIM management frames and (#2146)(QoS)Null data frames(#1131) during the ATIM Window.

- Directed ATIM management frames shall be acknowledged. If no acknowledgment is received, the ATIM shall be retransmitted using either the DCF (for non-QoS STAs) or the EDCAF (for QoS STAs)(#1502). Group addressed(#1359) ATIM management frames shall not be acknowledged.

- If a STA is unable to transmit an ATIM during the ATIM Window, for example due to contention with other STAs, the STA should(#1504) retain the buffered BUs and attempt to transmit the ATIM during the next ATIM Window.

- Immediately following the ATIM Window, a STA shall begin transmission of any(#1505) buffered group addressed(#1359) frames for which an ATIM was previously transmitted. Following the transmission of any group addressed(#1359) frames, any BUs addressed to STAs for which an acknowledgment for a previously transmitted ATIM frame was received shall be transmitted. All STAs shall use the backoff procedure defined in 9.2.5.2 (Backoff procedure for DCF) for transmission of the first frame following the ATIM Window. All remaining frames shall be transmitted using either the DCF (for non-QoS STAs) or the EDCAF (for QoS STAs)(#1502).

- If a buffered BU is transmitted using fragmentation and the BU has been partially transmitted when the next Beacon frame is sent, the STA shall retain the buffered BU and announce the remaining fragments by transmitting an ATIM during the next ATIM Window.

- If a STA is unable to transmit a buffered BU during the beacon interval in which it was announced, for example due to contention with other STAs, the STA shall retain the buffered BU and announce the BU again by transmitting an ATIM during the next ATIM Window.

- Following the transmission of all buffered BUs, a STA may transmit BUs without announcement to STAs that are known to be in the Awake state for the current beacon interval(#1131).

- A STA may discard BUs buffered for later transmission to power-saving STAs if the STA determines that the BU has been buffered for an excessive amount of time or if other conditions internal to the STA implementation make it desirable to discard buffered BUs (e.g., buffer starvation). A BU should not(#1506) be discarded that has been buffered for less than dot11BeaconPeriod. The algorithm to manage this buffering is beyond the scope of this standard.

- A STA may transmit individually addressed or group addressed(#1359) Null data frames within the ATIM window to indicate the STA’s intent to change power management modes. The STA may transition into PS mode after acknowledgements have been successfully received for all individually addressed Null data frames or after the STA has transnmitted group addressed(#1359) Null data frames at least dot11BSSBroadcastNullCount times(#1131).

- SM power save(11n)

A STA consumes power on all active receive chains, even though they are not necessarily required for the actual frame exchange. The SM Power Save feature allows a STA to operate with only one active receive chain for a significant portion of time.

The STA controls which receive chains are active through the PHY-RXCONFIG.request primitive specifying a PHYCONFIG_VECTOR parameter ACTIVE_RXCHAIN_SET that indicates which of its receive chains should be active.

In dynamic SM power save mode, a STA enables its multiple receive chains when it receives the start of a frame sequence addressed to it. Such a frame sequence shall start with a single-spatial stream individually addressed frame that requires an immediate response and that is addressed to the STA in dynamic SM power save mode. An RTS/CTS sequence may be used for this purpose. The receiver shall be capable of receiving a PPDU that is sent using an MCS that indicates more than one spatial stream a SIFS after the end of its response frame transmission. The receiver switches to the multiple receive chain mode when it receives the RTS addressed to it and switches back immediately when the frame sequence ends.

NOTE—A STA in dynamic SM power save mode cannot distinguish between an RTS/CTS sequence that precedes a MIMO transmission and any other RTS/CTS and, therefore, always enables its multiple receive chains when it receives an RTS addressed to itself.

The receiver can determine the end of the frame sequence through any of the following:

- It receives an individually addressed frame addressed to another STA.

- It receives a frame with a TA that differs from the TA of the frame that started the TXOP.

- The CS mechanism (see 9.2.0b.2 (CS mechanism(#1606))) indicates that the medium is idle at the TxPIFS slot boundary (defined in 9.2.10 (DCF timing relations)).

A STA in static SM power save mode maintains only a single receive chain active.

An HT STA may use the SM Power Save Action frame to communicate its SM Power Save state. A non-AP HT STA may also use SM Power Save bits in the HT Capabilities element of its Association Request to achieve the same purpose. The latter allows the STA to use only a single receive chain immediately after association.

A STA that has one or more DLS links shall notify all STAs with which it has a DLS link of any change in SM power save mode before operating in that mode.

Changes to the number of active receive chains are made only after the SM Power Save Mode indication has been successfully delivered (i.e., by acknowledgment of a frame carrying the HT Capabilities element or by acknowledgment of a SM Power Save frame). The SM Power Save Mode indication shall be transmitted using a individually addressed frame.