IEEE P802.11
Wireless LANs

| LB160 CID 2113 proposed resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-02-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., MS SJ-14-4, San Jose, Ca 95134-1706 | +1-408-527-0815 | petere@cisco.com |

Abstract

LB160 CID 2113 (MAC) proposed resolution, based on P802.11m Draft 2.04. Remove radio performance specifications that come from regulation, leaving those that come from the standard. 802.11mb approved Revision 6 for incorporation in Draft 2.05. Revision 7 adds approved editing instructions from 11-09-1111-02 removing regulatory tables from clauses 15 and 18, so that it can be applied in amendments that complete before REVmb is published. Revision 8 has changes suggested by 802.11mb LB162.

Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGx Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGx Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

Discussion:

In every country, equipment that conforms to IEEE 802.11 must meet regulatory requirements in order to be sold and to be operated. The knowledge of the regulatory requirements is shared among all the approved devices – masters must know rules for master devices, clients must know rules for client devices. There is not need to restate those rules in an IEEE 802.11 standard. Offering an Informative table I-1 should be enough to guide readers to regulatory references.

Regulatory Classes were first defined in 802.11j-2004, and went to great lengths to describe the existing situation in Japan’e 4.9 GHz and 5.0 GHz bands, well before the final rules were issued in May, 2005. There were relaxed emissions masks for rural areas, and Clear Channel Assessment requirements depend on whether the master station was fixed or not fixed.

In hindsight, we should have removed the Annex I Japanese emissions masks and adjacent channel limits when the rules became final, and changed clause 17.3.9.1 Transmit power levels to state (like clause 15.4.7.1) “The maximum allowable output power is measured in accordance with practices specified by the appropriate regulatory bodies.”

In 2005 802.11 REVma added the US 4.9 Public Safety Band (FCC 47CFR Part 90.1201-90.1217) to Annex I, yet Annex I was informative in 802.11-2007.

802.11y-2008 added I.2.4 CCA-ED threshold, which is not defined in law, and changed Annex I from informative to normative. We did this to keep country and band-specific behavior out of clause 17. In 802.11mb Draft 1.0 comment resolution, the resolutions to CIDs 1054 and 1055 removed the country tables from 18.4.6.2 and 15.4.6.2, so there is no regulatory-specific information in the Draft 2.0 PHY clauses. The regulatory information in Annex I.2 is in normative clauses, and duplicates the rules for operation in several regulatory domains.

Proposed REVmb Radio performance specification changes:

We propose to change Transmit power limits in 17.3.9.1 and 20.1.23.3 to say “The maximum allowable output power is measured in accordance with practices specified by the appropriate regulatory bodies.” We propose to remove from Annex I I.2.1 Transmit and receive in-band and out-of-band spurious emissions, I.2.2 Transmit power levels, and I.2.3 Transmit spectrum mask text that reference law. We propose to change Annex J-1 to remove columns labeled Transmit Power Limit and Emissions Limit Set, as laws of regulatory domains are the final authority.

Comment xxxx asks for a more scaleable way to convey Country Information and Regulatory Classes, and we accept doing this. Our proposed resolution is to remove any normative references to regulation from normative text, and instead designate Operating Class to communicate the shared knowledge of normative requirements. We replace “Regulatory Class” with “Operating Class” everywhere in the standard, both singular and plural, with and without the space character after the word “regulatory” and change normative text to describe operating requirements, not regulatory ones.

With respect to the Behavior limits set, behaviors 5, 7, 8, 9, 11 and 12 are the result of regulation, and need not be normatively bound in the standard. We propose to add semantic labels to the behaviors in Annex I and the new table J.4.

With respect to the 802.11af TV white spaces and other sub-GHz amendments, we reserve 22 Operating classes above 802.11n-2009 Table J-3 (57 is the highest class defined), before assigning the global operating classes. We reserve 61 classes above 5.85 GHz to be defined in the future.

Proposed Resolution:

Accept comment xxxx based on this discussion and editorial instructions in 10/0210r8.

## Editing instructions:

In existing text, maintaining capitalization, globally change “Regulatory Classes” to “Operating Classes”

In existing text, maintaining capitalization, globally change “Regulatory Class” to “Operating Class”

In existing text, maintaining capitalization, globally change “RegulatoryClasses” to “OperatingClasses”

In existing text, maintaining capitalization and italics, globally change “RegulatoryClass” to “OperatingClass”

In Annex A existing text, maintaining capitalization, globally change “Regulatory and Coverage Class” to “Operating and Coverage Class”

In existing text, maintaining capitalization, globally change “ a Operating” to “ an Operating”

In Figure 7-46, change “Regulatory Extension Identifier” to “Operating Extension Identifier” and change “Regulatory Class” to “Operating Class”

In Figures 7-62a, 7-62c, 7-62e and 7-62g, change “Regulatory Class” to “Operating Class”

In Figures 7-68a, 7-68b, 7-68c and 7-68e, change “Regulatory Class” to “Operating Class”

In Figures 7-95a, 7-95b, 7-95o11, 7-95o12, 7-95o13 and 7-95o25, change “Regulatory Class” to “Operating Class”

In Figures 7-101g, 7-101h4, 7-101h5, 7-101h6 and 7-101h7, change “Regulatory Class” to “Operating Class”

In 3, insert text as shown:

3.xx operating class: An Annex J index into a set of values for radio operation in a regulatory domain.

In 7.3.2.9 second paragraph, change “Regulatory Extension Identifier” to “Operating Extension Identifier” and change “regulatory triplet” to “operating triplet” as shown:

true and the First Channel Number/OperatingRegulatory Extension Identifier octet has a positive integer value of 201 or greater, then that triplet comprises the OperatingRegulatory Extension Identifier, OperatingRegulatory Class, and Coverage Class fields. Together they are referred to as an operatingregulatory triplet.

In 7.3.2.9 fourth paragraph, change “Regulatory Extension Identifier” to “Operating Extension Identifier” as shown:

The First Channel Number/OperatingRegulatory Extension Identifier field shall be 1 octet in length.

In 7.3.2.9 fifth paragraph, change “regulatory triplet” to “operating triplet” as shown:

Where dot11OperatingRegulatoryClassesRequired is true, consecutive subband triplets following an operatingregulatory triplet have monotonically increasing First Channel Number fields.

In 7.3.2.9 eighth paragraph, change as shown:

An operatingregulatory class is an index into a set of values for radio equipment operationsets of rules. The OperatingRegulatory Class field shall be 1 octet in length.

In 7.3.2.9 tenth paragraph, change beginning of first sentence as shown:

The Coverage Class field of the operatingregulatory triplet specifies

In 11.10.15 second paragraph, change as shown:

The contents of the AP Channel Report elements may be compiled from the list of unique operatingregulatory/

channel pairs found in the Neighbor Report. The contents of the AP channel report may be configured or obtained by other means beyond the scope of this standard.

In 11.11.3 fourth paragraph, delete first sentence.

Change title in 15.4.6.2 description as shown:

15.4.6.2 Channel nNumbering of operating channels

Change text in 15.4.6.2 description as shown:

When dot11OperatingClassesRequired is true, channel numbering shall be as specified in 17.3.8.3.2 and channelization shall be as specified in 17.3.8.3.3. When dot11OperatingClassesDefined is false or not defined, tThe channel center frequencies and CHNL_ID numbers shall be as shown in Table 15-7 (DSSS PHY

frequency channel plan). See the applicable regulations for the countries in which the implementation will

operate. For each supported regulatory domain, all channels in Table 15-7 (DSSS PHY frequency channel

plan) marked with “X” shall be supported .

Delete columns titled Regulatory domains in Table 15-7 DSSS PHY frequency channel plan.

In 17.3.9.1, change the paragraph as shown:

The maximum allowable transmit power by regulatory domain is defined in Annex I output power is measured in accordance with practices specified by the appropriate regulatory bodies.

Change title in 18.4.6.2 description as shown:

18.4.6.2 Channel nNumbering of operating channels

Change text in 18.4.6.2 description as shown:

When dot11OperatingClassesRequired is true, channel numbering shall be as specified in 17.3.8.3.2 and channelization shall be as specified in 17.3.8.3.3. When dot11OperatingClassesDefined is false or not defined, tThe channel center frequencies and CHNL_ID numbers shall be as shown in Table 18-9 (High Rate PHY

frequency channel plan). See the applicable regulations for the countries in which the implementation will

operate. For each supported regulatory domain, all channels in Table 18-9 (High Rate PHY frequency

channel plan) marked with “X” shall be supported.

Delete columns titled Regulatory domains in Table 18-9 High Rate PHY frequency channel plan.

In 20.3.21.3, change the paragraph as shown:

The maximum allowable transmit power by regulatory domain is defined in Annex I output power is measured in accordance with practices specified by the appropriate regulatory bodies.

Change text in Annex D dot11CountryString description as shown:

The third octet shall be one of the following:

1. an ASCII space character, if the regulations under

which the station is operating encompass all

environments for the current frequency band in the country,

2. an ASCII 'O' character, if the regulations under

which the station is operating are for an Outdoor

environment only, or

3. an ASCII 'I' character, if the regulations under

which the station is operating are for an Indoor

environment only.

4. an ASCII 'X' character, if the station is operating

under a non-country entity. The first two octets of the

non-country entity shall be two ASCII 'XX' characters.

5. the binary representation of the Regulatory Class table number currently in use, from the set of tables defined in Annex J, e.g., Table J-1 is represented as x’01’."

Change title of Annex I as shown:

RegulatoryOperating referencesclasses

Change Annex I-1 fourth paragraph as shown:

BehaviorEmissions limits sets are listed in Table I-2 (BehaviorEmissions limits sets), and behavior limits sets are specified in Table I-23 (Behavior limits sets). The external regulatory references in Table I-2 (Emissions limits sets) and Table I-3 (Behavior limits sets) are informative, as they do not specify technical parameters.

Delete existing Table I-2 entirely

Delete Table I-3 columns “United States”, “Europe” and “Japan”

Change Behavior limits set as shown and insert columns “Encoding” and “Descriptions” as shown:

Table I-2—Behavior limits sets

| Encoding | Behavior limits set | Description |
| --- | --- | --- |
| 0 | 0 | Not specified |
| 1 | 1 NomadicBehavioruse | The location of the station may change but is stationary while in use. The nomadic use EIRP power limits apply if the country allows more than one transmit power limit in the band. This behaviour is only specified in bands where behavior 10 License Exempt bands is also allowed. |
| 2 | 2 IindoorOonlyBehavior use | The location of operation must be ‘indoors’ as defined in the regulatory domain. Globally, this currently applies only to the 5.15-5.25 GHz band. |
| 3 | 3 TtransmitP powerC controlBehaviora | Adaptive transmit power control as defined in the regulatory domain, or clause 11.8 if not defined for the regulatory domain. |
| 4 | 4 DdynamicF frequency SselectionBehaviora | Adaptive dynamic frequency selection as defined in the regulatory domain, or clause 11.9 if not defined for the regulatory domain. |
| 5 | 5 IBSSRrestrictionsBehavior | Operation where an IBSS is required to communicate with a licensed operator or agent designated by regulatory authorities. |
| 6 | 6 CS4 msBehavior CSab | No station can transmit for more than 4 ms without carrier sensing, whether transmitting fragments or frames, unless it is controlled by another STA. |
| 7 | 7 LlicensedB base STABehavior | A station operating at a fixed point under the control of a licensed operator. |
| 8 | 8 Mmobile STABehavior | A station intended to be used while in motion or during halts at unspecified points. |
| 9 | 9 PpublicS safetyBehavior | Generally this covers all government entities, private companies sponsored by a government entity (such as private ambulance services) and any organization with critical infrastructure (power companies, pipelines, etc.). |
| 10 | 10 License ExemptB bandsBehavior | Frequency bands where some fixed stations can be operated without a license at a higher radiated transmit power than permitted for nomadic use. This behaviour is only specified in bands where behaviour 1 nomadic use is also allowed. |
| 11 | 11 Rregistered STABehaviorc | A station operating at a location registered with regulatory authorities or their designated agent. |
| 12 | 12 Ddependent STABehaviorc | A station operating under the control of a licensed operator or registered station. |
| 13 | 13 20/40 MHz BSS primary channel with secondary channel above the primary channel or 20 MHz BSS primary channel operated by an FC HT AP and also 20 MHz operational channel for a non-AP STA when the non-AP STA is associated with an FC HT AP.d See NOTE. PrimaryChannelLowerBehaviord | 20/40 MHz BSS primary channel with secondary channel above the primary channel or 20 MHz BSS primary channel operated by an FC HT AP and also 20 MHz operational channel for a non-AP STA when the non-AP STA is associated with an FC HT AP.d See NOTE. |
| 14 | 14 20/40 MHz BSS primary channel with secondary channel below the primary channel or 20 MHz BSS primary channel operated by an FC HT AP and also 20 MHz operational channel for a non-AP STA when the non-AP STA is associated with an FC HT AP.d See NOTE. PrimaryChannelUpperBehaviord | 20/40 MHz BSS primary channel with secondary channel below the primary channel or 20 MHz BSS primary channel operated by an FC HT AP and also 20 MHz operational channel for a non-AP STA when the non-AP STA is associated with an FC HT AP.d See NOTE. |
| 15 | 15 CCA-EDBehaviora | CCA shall also detect a medium busy condition when CCA-EnergyDetect detects a channel busy condition. |
| 16 | 16 DFS_50_100_Behavior with 50-100 µs pulses | A station operating in a band where radiolocation radar is primary, and station operation has in-service monitoring requirements for 50-100 µs radar pulses. |
| 17-255 | Reserved |  |

Change I.2.1 as shown:
Spurious transmissions from compliant devices shall conform to national regulations. For operation in the

United States, refer to FCC CFR47 [B8], Section 15.407. For operation in Europe, refer to

ETSI EN 301 893-1. For operation in Japan, refer to MIC EO Article 49.20 and Article 49.21, Section

1.

Delete clause I.2.2 text and insert one sentence as follows:

The maximum allowable output power is measured in accordance with practices specified by the appropriate regulatory bodies.

Delete clause 1.2.3 text and insert one sentence as follows:

Transmit spectrum masks defined in regulation are subject to change or revision at any time.

In Annex J-1 second paragraph, change “Regulatory Extension Identifier” to “Operating Extension Identifier” and change “regulatory triplet” to “operating triplet” as shown:

The Country element (see 7.3.2.9) allows a STA to configure its PHY and MAC for operation when the operatingregulatory triplet of OperatingRegulatory Extension Identifier, OperatingRegulatory Class, and Coverage Class fields is present. The operatingregulatory triplet indicates both PHY and MAC configuration characteristics and operational characteristics.

Change third paragraph in Annex J-1 as shown:

The operatingregulatory class is an index into a set of values for radio operation in a regulatory domain. The

operatingregulatory class tables also contain pointers to behaviors and signal detection limits in Annex Iregulatory documents for each regulatory domain where further operational requirements may be found.

Delete seventh paragraph (Emissions limits) text in Annex J-1

Change eighth paragraph (Behavior limits) text in Annex J-1 as shown:

A behavior limits set is an enumerated list, each element of which points to a row in Table I-23 (Behavior

limits sets) containing behavior limits and references to regulations where behavior limits in various

regulatory domains can be found.

Delete Columns labelled “Transmit power limit” and “Emissions limits set” in Annex J-1 Tables J-1, J-2 and J-3

Insert a Column to right of Operating class column in Table J-1 with values as shown and change Behavior limits set as shown:

| Operating class | Global operating class (see Table J-4) | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Behavior limits set |
| --- | --- | --- | --- | --- | --- |
| 1 | 115 | 5 | 20 | 36, 40, 44, 48 | 1, 2NomadicBehavior, IndoorOnlyBehavior |
| 2 | 118 | 5 | 20 | 52, 56, 60, 64 | 1, 4, 16NomadicBehavior, DynamicFrequencySelectionBehavior, DFS_50_100_Behavior |
| 3 | 124 | 5 | 20 | 149, 153, 157, 161 | 1NomadicBehavior |
| 4 | 121 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 1, 4, 16NomadicBehavior, DynamicFrequencySelectionBehavior, DFS_50_100_Behavior |
| 5 | 125 | 5 | 20 | 149, 153, 157, 161, 165 | 10LicenseExemptBehavior |
| 6 | 101 | 4.9375 | 5 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10 | 9PublicSafetyBehavior |
| 7 | 101 | 4.9375 | 5 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10 | 9PublicSafetyBehavior |
| 8 | 102 | 4.89 | 10 | 11, 13, 15, 17, 19 | 9PublicSafetyBehavior |
| 9 | 102 | 4.89 | 10 | 11, 13, 15, 17, 19 | 9PublicSafetyBehavior |
| 10 | 103 | 4.85 | 20 | 21, 25 | 9PublicSafetyBehavior |
| 11 | 103 | 4.85 | 20 | 21, 25 | 9PublicSafetyBehavior |
| 12 | 81 | 2.407 | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11 | 10LicenseExemptBehavior |
| 13 | 94 | 3.000 | 20 | 133, 137 | 3, 4, 6, 11, 15TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, CS4msBehavior, RegisteredSTABehavior, CCA-EDBehavior |
| 13 | 94 | 3.000 | 20 | 133, 137 | 3, 4, 5, 6, 12, 15TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, DependentSTABehavior, CCA-EDBehavior |
| 14 | 95 | 3.000 | 10 | 132, 134, 136, 138 | 3, 4, 6, 11, 15TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, CS4msBehavior, RegisteredSTABehavior, CCA-EDBehavior |
| 14 | 95 | 3.000 | 10 | 132, 134, 136, 138 | 3, 4, 5, 6, 12, 15TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, DependentSTABehavior, CCA-EDBehavior |
| 15 | 96 | 3.0025 | 5 | 131. 132. 133. 134. 135. 136. 137, 138 | 3, 4, 6, 1, 15TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, CS4msBehavior, RegisteredSTABehavior, CCA-EDBehavior |
| 15 | 96 | 3.0025 | 5 | 131. 132. 133. 134. 135. 136. 137, 138 | 3, 4, 5, 6, 12, 15TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, DependentSTABehavior, CCA-EDBehavior |
| 16-21 | Reserved | Reserved | Reserved | Reserved | Reserved |
| 22 | 116 | 5 | 40 | 36, 44 | 1, 2, 13NomadicBehavior, IndoorOnlyBehavior, PrimaryChannelLowerBehavior |
| 23 | 119 | 5 | 40 | 52, 60 | 1, 4, 13NomadicBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelLowerBehavior |
| 24 | 122 | 5 | 40 | 100, 108, 116, 124, 132 | 1, 4, 13, 16NomadicBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelLowerBehavior, DFS_50_100_Behavior |
| 25 | 126 | 5 | 40 | 149, 157 | 1, 13NomadicBehavior, PrimaryChannelLowerBehavior |
| 26 | 126 | 5 | 40 | 149, 157 | 10, 13LicenseExemptBehavior, PrimaryChannelLowerBehavior |
| 27 | 117 | 5 | 40 | 40, 48 | 1, 2, 14NomadicBehavior, IndoorOnlyBehavior, PrimaryChannelUpperBehavior |
| 28 | 120 | 5 | 40 | 56, 64 | 1, 4, 14NomadicBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelUpperBehavior |
| 29 | 123 | 5 | 40 | 104, 112, 120, 128, 136 | 1, 4, 14, 16NomadicBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelUpperBehavior, DFS_50_100_Behavior |
| 30 | 127 | 5 | 40 | 153, 161 | 1, 14NomadicBehavior, PrimaryChannelUpperBehavior |
| 31 | 127 | 5 | 40 | 153, 161 | 10, 14LicenseExemptBehavior, PrimaryChannelUpperBehavior |
| 32 | 83 | 2.407 | 40 | 1, 2, 3, 4, 5, 6, 7 | 10, 13LicenseExemptBehavior, PrimaryChannelLowerBehavior |
| 33 | 84 | 2.407 | 40 | 5, 6, 7, 8, 9, 10, 11 | 10, 14LicenseExemptBehavior, PrimaryChannelUpperBehavior |
| 34-255 | Reserved | Reserved | Reserved | Reserved | Reserved |

Insert Editor’s Note before heading of Table J-2:

Editor’s Note: Any <ANA> flags in the following tables will be replaced by values assigned by the 802.11

ANA, once those values have been assigned, and reserved values will be updated as necessary.

Insert a Column to right of Operating class column in Table J-2 with values as shown and change Behavior limits set as shown:

| Operating class | Global operating class (see Table J-4) | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Behavior limits set |
| --- | --- | --- | --- | --- | --- |
| 1 | 115 | 5 | 20 | 36, 40, 44, 48 | 2, 3 IndoorOnlyBehavior, TransmitPowerControlBehavior |
| 2 | 118 | 5 | 20 | 52, 56, 60, 64 | 1, 3, 4NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior |
| 3 | 121 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 1, 3, 4NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior |
| 4 | 81 | 2.407 | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13(#1302) | 10LicenseExemptBehavior |
| 5 | 116 | 5 | 40 | 36, 44 | 2, 3, 13IndoorOnlyBehavior, TransmitPowerControlBehavior, PrimaryChannelLowerBehavior |
| 6 | 119 | 5 | 40 | 52, 60 | 1, 3, 4, 13NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelLowerBehavior |
| 7 | 122 | 5 | 40 | 100, 108, 116, 124, 132 | 1, 3, 4, 13NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelLowerBehavior |
| 8 | 117 | 5 | 40 | 40, 48 | 2, 3, 14IndoorOnlyBehavior, TransmitPowerControlBehavior, PrimaryChannelUpperBehavior |
| 9 | 120 | 5 | 40 | 56, 64 | 1, 3, 4, 14NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelUpperBehavior |
| 10 | 123 | 5 | 40 | 104, 112, 120, 128, 136 | 1, 3, 4, 14NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelUpperBehavior |
| 11 | 83 | 2.407 | 40 | 1, 2, 3, 4, 5, 6, 7, 8, 9 | 10, 13LicenseExemptBehavior, PrimaryChannelLowerBehavior |
| 12 | 84 | 2.407 | 40 | 5, 6, 7, 8, 9, 10, 11, 12, 13 | 10, 14LicenseExemptBehavior, PrimaryChannelUpperBehavior |
| <ANA> | 125 | 5 | 20 | 149, 153, 157, 161, 165, 169 | 4 DynamicFrequencySelectionBehavior |
| 13-255 | Reserved | Reserved | Reserved | Reserved | Reserved |

Insert a Column to right of Operating class column in Table J-3 with values as shown and change Behavior limits set as shown:

| Operating class | Global operating class (see Table J-4) | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Behavior limits set |
| --- | --- | --- | --- | --- | --- |
| 1 | 115 | 5 | 20 | 36, 40, 44, 48 | 1, 2, 6NomadicBehavior, IndoorOnlyBehavior, CS4msBehavior |
| 2 | 112 | 5 | 20 | 8, 12, 16 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 3 | 112 | 5 | 20 | 8, 12, 16 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 4 | 112 | 5 | 20 | 8, 12, 16 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 5 | 112 | 5 | 20 | 8, 12, 16 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 6 | 112 | 5 | 20 | 8, 12, 16 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 7 | 109 | 4 | 20 | 184, 188, 192, 196 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 8 | 109 | 4 | 20 | 182, 188, 192, 196 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 9 | 109 | 4 | 20 | 184, 188, 192, 196 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 10 | 109 | 4 | 20 | 184, 188, 192, 196 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 11 | 109 | 4 | 20 | 184, 188, 192, 196 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 12 | 113 | 5 | 10 | 7, 8, 9, 11 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 13 | 113 | 5 | 10 | 7, 8, 9, 11 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 14 | 113 | 5 | 10 | 7, 8, 9, 11 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 15 | 113 | 5 | 10 | 7, 8, 9, 11 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 16 | 110 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 17 | 110 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 18 | 110 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 19 | 110 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 20 | 110 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 21 | 114 | 5.0025 | 5 | 6, 7, 8, 9, 10, 11 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 22 | 114 | 5.0025 | 5 | 6, 7, 8, 9, 10, 11 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 23 | 114 | 5.0025 | 5 | 6, 7, 8, 9, 10, 11 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 24 | 114 | 5.0025 | 5 | 6, 7, 8, 9, 10, 11 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 25 | 111 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 26 | 111 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 27 | 111 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 5, 6, 7 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior |
| 28 | 111 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 29 | 111 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 5, 6, 8 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior |
| 30 | 81 | 2.407 | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13(#1303) | 6, 10 CS4msBehavior, LicenseExemptBehavior |
| 31 | 82 | 2.414 | 25 | 14 | 6, 10 CS4msBehavior, LicenseExemptBehavior |
| 32 | 118 | 5 | 20 | 52, 56, 60, 64 | 1, 2, 3, 4, 5, 6NomadicBehavior, IndoorOnlyBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior |
| 33 | 118 | 5 | 20 | 52, 56, 60, 64 | 1, 2, 4, 5, 6NomadicBehavior, IndoorOnlyBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior |
| 34 | 121 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 1, 3, 4, 5, 6, 16NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, DFS_50_100_Behavior |
| 35 | 121 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 1, 4, 5, 6, 16NomadicBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, DFS_50_100_Behavior |
| 36 | 116 | 5 | 40 | 36, 44 | 1, 2, 6, 13NomadicBehavior, IndoorOnlyBehavior, CS4msBehavior, PrimaryChannelLowerBehavior |
| 37 | 119 | 5 | 40 | 52, 60 | 1, 2, 3, 4, 5, 6, 13NomadicBehavior, IndoorOnlyBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, PrimaryChannelLowerBehavior |
| 38 | 119 | 5 | 40 | 52, 60 | 1, 2, 4, 5, 6, 13NomadicBehavior, IndoorOnlyBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, PrimaryChannelLowerBehavior |
| 39 | 122 | 5 | 40 | 100, 108, 116, 124, 132 | 1, 3, 4, 5, 6, 13, 16NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, PrimaryChannelLowerBehavior, DFS_50_100_Behavior |
| 40 | 122 | 5 | 40 | 100, 108, 116, 124, 132 | 1, 4, 5, 6, 13, 16NomadicBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, PrimaryChannelLowerBehavior, DFS_50_100_Behavior |
| 41 | 117 | 5 | 40 | 40, 48 | 1, 2, 6, 14NomadicBehavior, IndoorOnlyBehavior, CS4msBehavior, PrimaryChannelUpperBehavior |
| 42 | 120 | 5 | 40 | 56, 64 | 1, 2, 3, 4, 5, 6, 14NomadicBehavior, IndoorOnlyBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, PrimaryChannelUpperBehavior |
| 43 | 120 | 5 | 40 | 56, 64 | 1, 2, 4, 5, 6, 14NomadicBehavior, IndoorOnlyBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, PrimaryChannelUpperBehavior |
| 44 | 123 | 5 | 40 | 104, 112, 120, 128, 136 | 1, 3, 4, 5, 6, 14, 16NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, PrimaryChannelUpperBehavior, DFS_50_100_Behavior |
| 45 | 123 | 5 | 40 | 104, 112, 120, 128, 136 | 1, 4, 5, 6, 14, 16NomadicBehavior, DynamicFrequencySelectionBehavior, IBSSRestrictionsBehavior, CS4msBehavior, PrimaryChannelUpperBehavior, DFS_50_100_Behavior |
| 46 | 104 | 4 | 40 | 184, 192 | 5, 6, 7, 13 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior, PrimaryChannelLowerBehavior |
| 47 | 104 | 4 | 40 | 184, 192 | 5, 6, 8, 13 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior, PrimaryChannelLowerBehavior |
| 48 | 104 | 4 | 40 | 184, 192 | 5, 6, 7, 13 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior, PrimaryChannelLowerBehavior |
| 49 | 104 | 4 | 40 | 184, 192 | 5, 6, 8, 13 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior, PrimaryChannelLowerBehavior |
| 50 | 104 | 4 | 40 | 184, 192 | 5, 6, 8, 13 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior, PrimaryChannelLowerBehavior |
| 51 | 105 | 4 | 40 | 188, 196 | 5, 6, 7, 14 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior, PrimaryChannelUpperBehavior |
| 52 | 105 | 4 | 40 | 188, 196 | 5, 6, 8, 14 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior, PrimaryChannelUpperBehavior |
| 53 | 105 | 4 | 40 | 188, 196 | 5, 6, 7, 14 IBSSRestrictionsBehavior, CS4msBehavior, LicensedBaseSTABehavior, PrimaryChannelUpperBehavior |
| 54 | 105 | 4 | 40 | 188, 196 | 5, 6, 8, 14 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior, PrimaryChannelUpperBehavior |
| 55 | 105 | 4 | 40 | 188, 196 | 5, 6, 8, 14 IBSSRestrictionsBehavior, CS4msBehavior, MobileSTABehavior, PrimaryChannelUpperBehavior |
| 56 | 83 | 2.407 | 40 | 1, 2, 3, 4, 5, 6, 7, 8, 9 | 10, 13LicenseExemptBehavior, PrimaryChannelLowerBehavior |
| 57 | 84 | 2.407 | 40 | 5, 6, 7, 8, 9, 10, 11, 12, 13 | 10, 14LicenseExemptBehavior, PrimaryChannelUpperBehavior |
| <ANA> | 121 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 1, 3, 4, 6, 10NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, CS4msBehavior, LicenseExemptBehavior |
| 58-255 | Reserved | Reserved | Reserved | Reserved | Reserved |

Insert text before Annex J-2 as follows:

Operating classes for operation anywhere in the world are enumerated in table J-4 (Global operating classes), and are used in addition to the operating classes enumerated in tables J-1, J-2 and J-3 (see 7.3.2.54 Supported Operating Classes). Where a BSS includes STAs that do not support global operating classes, then all requests and action frames to those STAs that convey elements containing operating classes shall use non-global operating class values.

# Table J-4—Global operating classes

| Operating class | Non-global operating class(es) | Channel starting Frequency (GHz) | Channel spacing (MHz) | Channel set | Behavior limits set |
| --- | --- | --- | --- | --- | --- |
| 1-80 |  | Reserved | Reserved | Reserved | Reserved |
| 81 | J-1-12, J-2-4, J-3-30 | 2.407 | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13 |  |
| 82 | J-3-31 | 2.414 | 25 | 14 |  |
| 83 | J-1-32, J-2-11, J-3-56 | 2.407 | 40 | 1, 2, 3, 4, 5, 6, 7, 8, 9 | PrimaryChannelLowerBehavior |
| 84 | J-1-33, J-2-12, J-3-46,47,48,49,50, J-3-57 | 2.407 | 40 | 5, 6, 7, 8, 9, 10, 11, 12, 13 | PrimaryChannelUpperBehavior |
| 85-93 |  | Reserved | Reserved | Reserved | Reserved |
| 94 | J-1-13 | 3 | 20 | 133, 137 | TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, CS4msBehavior, CCA-EDBehavior |
| 95 | J-1-14 | 3 | 10 | 132, 134, 136, 138 | TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, CS4msBehavior, CCA-EDBehavior |
| 96 | J-1-15 | 3.0025 | 5 | 131, 132, 133, 134, 135, 136, 137, 138 | TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, CS4msBehavior, CCA-EDBehavior |
| 97-100 |  | Reserved | Reserved | Reserved | Reserved |
| 101 | J-1-6,7 | 4.85 | 20 | 21, 25 |  |
| 102 | J-1-8,9 | 4.89 | 10 | 11, 13, 15, 17, 19 |  |
| 103 | J-1-10,11 | 4.9375 | 5 | 1, 2, 3, 4, 5,k 6, 7, 8, 9, 10 |  |
| 104 | J-3-46,47,48,49,50 | 4 | 40 | 184, 192 | PrimaryChannelLowerBehavior |
| 105 | J-3-51,52,53,54,55 | 4 | 40 | 188, 196 | PrimaryChannelUpperBehavior |
| 106 | - | 4 | 20 | 191, 195 |  |
| 107 | - | 4 | 10 | 189, 191, 193, 195, 197 |  |
| 108 | - | 4.0025 | 5 | 188, 189, 190, 191, 192, 193, 194, 195, 196, 197 |  |
| 109 | J-3-7,8,9,10,11 | 4 | 20 | 184, 188, 192, 196 |  |
| 110 | J-3-16,17,18,19,20 | 4 | 10 | 183, 184, 185, 186, 187, 188, 189 |  |
| 111 | J-3-25,26,27,28,29 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 |  |
| 112 | J-3-2,3,4,5,6 | 5 | 20 | 8, 12, 16 |  |
| 113 | J-3-12,13,14,15 | 5 | 10 | 7, 8, 9, 10, 11 |  |
| 114 | J-3-21,22,23,24 | 5.0025 | 5 | 6, 7, 8, 9, 10, 11 |  |
| 115 | J-1-1, J-2-1, J-3-1 | 5 | 20 | 36, 40, 44, 48 | IndoorOnlyBehavior |
| 116 | J-1-22, J-2-5, J-3-36 | 5 | 40 | 36, 44 | IndoorOnlyBehavior, PrimaryChannelLowerBehavior |
| 117 | J-1-27, J-2-8, J-3-41 | 5 | 40 | 40, 48 | IndoorOnlyBehavior, PrimaryChannelUpperBehavior |
| 118 | J-1-2, J-2-2, J-3-32,33 | 5 | 20 | 52, 56, 60, 64 | TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, DFS_50_100_Behavior |
| 119 | J-1-23, J-2-6, J-3-37,38 | 5 | 40 | 52, 60 | TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelLowerBehavior, DFS_50_100_Behavior |
| 120 | J-1-28, J-2-9, J-3-42,43 | 5 | 40 | 56, 64 | TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelUpperBehavior, DFS_50_100_Behavior |
| 121 | J-1-4, J-2-3, J-3-34,35, <ANA> | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, DFS_50_100_Behavior |
| 122 | J-1-24, J-2-7, J-3-39,40 | 5 | 40 | 100, 108, 116, 124, 132 | TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelLowerBehavior, DFS_50_100_Behavior |
| 123 | J-1-29, J-2-10, J-3-44,45 | 5 | 40 | 104, 112, 120, 128, 136 | TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior, PrimaryChannelUpperBehavior, DFS_50_100_Behavior |
| 124 | J-1-3 | 5 | 20 | 149, 153, 157, 161 | NomadicBehavior |
| 125 | J-1-5, J-2-<ANA> | 5 | 20 | 149, 153, 157, 161, 165, 169 | LicenseExemptBehavior |
| 126 | J-1-25,26 | 5 | 40 | 149, 157 | PrimaryChannelLowerBehavior |
| 127 | J-1-30,31 | 5 | 40 | 153, 161 | PrimaryChannelUpperBehavior |
| 128-191 | - | Reserved | Reserved | Reserved | Reserved |
| 192-254 | - | Vendor specific | Vendor specific | Vendor specific | Vendor specific |
| 255 | - | Reserved | Reserved | Reserved | Reserved |

Non-global operating classes refer to the operating classes enumerated in the leftmost column of tables J-1, J-2 and J-3 (see 7.3.2.54 Supported Operating Classes).

Vendor Specific operating classes are used to carry information not defined in this standard within a single defined format, so that reserved operating classes are not usurped for nonstandard purposes and so that interoperability is more easily achieved in the presence of nonstandard information.

NOTE—This example Country element (see Figure 7-46) describes USA operation (‘55’, ’53’) using both J-1 class 12 and J-4 class 81 for 2.4 GHz band, 11 channels at 100 mW limit (in hexadecimal): ‘07’, ’0F’, ’55’, ’53’, ’04’, ‘C9’, ’0C’, ’0’, ‘01’, ‘0B’, ‘64’, ‘C9’, ’51’, ’0’, ‘01’, ‘0B’, ‘64’. "