IEEE P802.11
Wireless LANs

| 802.11 TGmb LB160 comment 2214 “Compile the MIB” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009- |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

Comment 2214 on WG letter ballot 160 says “Compile the MIB”.

This submission shows the changes necessary to compile the MIB.

# Introduction

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2214 |  | General | A | Compile the MIB and resolve any compilation errors | As in comment | AGREE (EDITOR: 2010-01-20 14:54:57Z) Also change terminology in D2.01 "Enabled" -> "Activated" when at the end of a MIB variable name. | EDITOR |

The resolution shown above is an unapproved resolution written by the editor and speculatively edited in D2.02, which shows about 66 changes tagged by for this CID.

Since D2.02, the editor has worked with an IETF MIB expert, Richard Young.  The result of this work is:

- The discovery of the correct tools to use to compile the mib.  The recommended tools are “smitools” from the Institute of Operating Systems and Computer Networks at the Technical University of Braunschweig, Germany, which may be accessed using the following URL:

  - http://www.ibr.cs.tu-bs.de/bin/smitools.cgi

- About 500 changes are necessary to fix compilation errors, address the more important compilation warnings, and to adapt to IETF requirements on MIB style.

- There are numerous remaining compilation warnings.  The most important class of these relate to MIB variables that are not included in any compliance group – i.e., for which there is no requirement that they be present.   Since addressing these errors would involve making decisions about which variable need to be present, it was decided that these decisions were outside the scope of this comment, and will need to be addressed in a future letter ballot comment.

This document contains the changes to the MIB to address this comment, including both changes present in D2.02,  and those additional changes discovered by Richard.

The REVmb technical editor would like to sincerely thank Richard Young for his work on this topic.

Proposed updated resolution for CID 2214:

Agree in principle. Make the changes as shown in document 11-10/0216r0.

Furthermore,  approved comment resolution for comment 2039:

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2039 | 1389.02 | D | A | dot11xxxIfIndex are Integer32 almost everywhere, but should not be negative. In SNMP (RFC-2587) terms, ifIndex could be "SYNTAX InterfaceIndex", which could be unsigned 32 but not negative. "It is recommended that values are assigned contiguously " starting from 1. | Change Annex D IfIndex SYNTAX to be one value throughout. | AGREE (GEN: 2010-01-20 08:12:11Z) Accept. Change SYNTAX for dot11LCIDSEIndex to Integer32 | EDITOR |

Is non-responsive and wrong.  The comment says “don’t use Integer32” and “provide consistency”.  So the resolution does the exact opposite,  it changes one value to Integer32.

The changes shown below also address this comment by changing the syntax of all dot11xxxIfIndex objects to InterfaceIndex.

A new resolution is necessary for 2039,  as shown below.

Proposed updated resolution for CID 2039:

Agree in principle. Make the changes as shown in document 11-10/0216r0.

(This is an identical resolution to comment 2214).

# Remaining Compilation Warnings

The compilation of this module generates the warnings shown below.  These warnings have not been addressed for reasons described by comments below.

mibs/20100212r0P802.11REVmb_mib.txt:106: [5] {hyphen-in-object} warning: object identifier name `member-body' should not include hyphens in SMIv2 MIB

mibs/20100212r0P802.11REVmb_mib.txt:4242: [5] {node-implicit} warning: implicit node definition

mibs/20100212r0P802.11REVmb_mib.txt:28216: [3] {group-member-invalid} node `dot11EDCATable' is an invalid member of group `dot11Qosadditions'

mibs/20100212r0P802.11REVmb_mib.txt:28216: [3] {group-member-invalid} node `dot11QAPEDCATable' is an invalid member of group `dot11Qosadditions'

mibs/20100212r0P802.11REVmb_mib.txt:28216: [3] {group-member-invalid} node `dot11QosCountersTable' is an invalid member of group `dot11Qosadditions'

mibs/20100212r0P802.11REVmb_mib.txt:29411: [4] {group-object-status} warning: current group `dot11PhyOFDMComplianceGroup3' includes deprecated object `dot11FrequencyBandsImplemented'

mibs/20100212r0P802.11REVmb_mib.txt:30096: [4] {compliance-group-status} warning: current compliance statement `dot11Compliance' includes deprecated group `dot11PhyOperationComplianceGroup'

mibs/20100212r0P802.11REVmb_mib.txt:30221: [4] {compliance-group-status} warning: current compliance statement `dot11Compliance' includes deprecated group `dot11PhyAntennaComplianceGroup'

mibs/20100212r0P802.11REVmb_mib.txt:30235: [4] {compliance-group-status} warning: current compliance statement `dot11Compliance' includes deprecated group `dot11PhyRegDomainsSupportGroup'

mibs/20100212r0P802.11REVmb_mib.txt:1399: [4] {group-membership} warning: node `dot11RSNAPreauthenticationImplemented' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:3169: [4] {group-membership} warning: node `dot11RSNAProtectedManagementFramesActivated' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:3196: [4] {group-membership} warning: node `dot11RSNAUnprotectedManagementFramesAllowed' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:3228: [4] {group-membership} warning: node `dot11AssociationSAQueryMaximumTimeout' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:3258: [4] {group-membership} warning: node `dot11AssociationSAQueryRetryTimeout' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:3356: [4] {group-membership} warning: node `dot11RejectUnadmittedTraffic' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:3386: [4] {group-membership} warning: node `dot11BSSBroadcastNullCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:4297: [4] {group-membership} warning: notification `dot11Associate' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:4317: [4] {group-membership} warning: notification `dot11AssociateFailed' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:4339: [4] {group-membership} warning: notification `dot11Reassociate' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:4359: [4] {group-membership} warning: notification `dot11ReassociateFailed' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:4661: [4] {group-membership} warning: node `dot11MitigationRequirement' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:4686: [4] {group-membership} warning: node `dot11ChannelSwitchTime' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:4717: [4] {group-membership} warning: node `dot11PowerCapabilityMaxImplemented' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:4740: [4] {group-membership} warning: node `dot11PowerCapabilityMinImplemented' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:5536: [4] {group-membership} warning: node `dot11RSNATKIPCounterMeasuresInvoked' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:5563: [4] {group-membership} warning: node `dot11RSNA4WayHandshakeFailures' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:5696: [4] {group-membership} warning: node `dot11RSNAConfigSMKUpdateCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:6480: [4] {group-membership} warning: node `dot11RSNAStatsCMACICVErrors' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:6504: [4] {group-membership} warning: node `dot11RSNAStatsCMACReplays' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:6530: [4] {group-membership} warning: node `dot11RSNAStatsRobustMgmtCCMPReplays' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:6554: [4] {group-membership} warning: node `dot11RSNABIPMICErrors' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:6953: [4] {group-membership} warning: node `dot11RMRequestNextIndex' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:15362: [4] {group-membership} warning: node `dot11RMNeighborReportMobilityDomain' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:16172: [4] {group-membership} warning: node `dot11RMNeighborReportHTExplicitCompressedBeamformingFeedbackCap' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:16943: [4] {group-membership} warning: node `dot11FTMobilityDomainID' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:16982: [4] {group-membership} warning: node `dot11FTOverDSActivated' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17008: [4] {group-membership} warning: node `dot11FTResourceRequestSupported' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17035: [4] {group-membership} warning: node `dot11FTR0KeyHolderID' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17068: [4] {group-membership} warning: node `dot11FTR0KeyLifetime' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17097: [4] {group-membership} warning: node `dot11FTR1KeyHolderID' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17123: [4] {group-membership} warning: node `dot11FTReassociationDeadline' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17286: [4] {group-membership} warning: node `dot11LCIDSEIfIndex' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17306: [4] {group-membership} warning: node `dot11LCIDSECurrentRegulatoryClass' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17333: [4] {group-membership} warning: node `dot11LCIDSELatitudeResolution' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17357: [4] {group-membership} warning: node `dot11LCIDSELatitudeInteger' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17382: [4] {group-membership} warning: node `dot11LCIDSELatitudeFraction' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17407: [4] {group-membership} warning: node `dot11LCIDSELongitudeResolution' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17431: [4] {group-membership} warning: node `dot11LCIDSELongitudeInteger' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17456: [4] {group-membership} warning: node `dot11LCIDSELongitudeFraction' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17481: [4] {group-membership} warning: node `dot11LCIDSEAltitudeType' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17526: [4] {group-membership} warning: node `dot11LCIDSEAltitudeResolution' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17549: [4] {group-membership} warning: node `dot11LCIDSEAltitudeInteger' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17575: [4] {group-membership} warning: node `dot11LCIDSEAltitudeFraction' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17600: [4] {group-membership} warning: node `dot11LCIDSEDatum' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17634: [4] {group-membership} warning: node `dot11RegLocAgreement' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17663: [4] {group-membership} warning: node `dot11RegLocDSE' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17690: [4] {group-membership} warning: node `dot11DependentSTA' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17718: [4] {group-membership} warning: node `dot11DependentEnablementIdentifier' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17748: [4] {group-membership} warning: node `dot11DSEEnablementTimeLimit' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17776: [4] {group-membership} warning: node `dot11DSEEnablementFailHoldTime' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17804: [4] {group-membership} warning: node `dot11DSERenewalTime' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:17832: [4] {group-membership} warning: node `dot11DSETransmitDivisor' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:18261: [4] {group-membership} warning: node `dot11SPPAMSDUCapable' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:18281: [4] {group-membership} warning: node `dot11SPPAMSDURequired' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:18301: [4] {group-membership} warning: node `dot11FortyMHzOptionImplemented' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:19461: [4] {group-membership} warning: node `dot11FortyMHzIntolerant' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:19480: [4] {group-membership} warning: node `dot11BSSWidthTriggerScanInterval' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:19504: [4] {group-membership} warning: node `dot11BSSWidthChannelTransitionDelayFactor' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:19529: [4] {group-membership} warning: node `dot11OBSSScanPassiveDwell' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:19553: [4] {group-membership} warning: node `dot11OBSSScanActiveDwell' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:19577: [4] {group-membership} warning: node `dot11OBSSScanPassiveTotalPerChannel' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:19601: [4] {group-membership} warning: node `dot11OBSSScanActiveTotalPerChannel' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:19625: [4] {group-membership} warning: node `dot112040BSSCoexistenceManagementSupport' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:19645: [4] {group-membership} warning: node `dot11OBSSScanActivityThreshold' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:20985: [4] {group-membership} warning: node `dot11PBACErrors' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21278: [4] {group-membership} warning: node `dot11EDCATableCWmin' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21317: [4] {group-membership} warning: node `dot11EDCATableCWmax' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21356: [4] {group-membership} warning: node `dot11EDCATableAIFSN' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21394: [4] {group-membership} warning: node `dot11EDCATableTXOPLimit' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21442: [4] {group-membership} warning: node `dot11EDCATableMSDULifetime' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21475: [4] {group-membership} warning: node `dot11EDCATableMandatory' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21641: [4] {group-membership} warning: node `dot11QAPEDCATableCWmin' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21679: [4] {group-membership} warning: node `dot11QAPEDCATableCWmax' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21721: [4] {group-membership} warning: node `dot11QAPEDCATableAIFSN' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21758: [4] {group-membership} warning: node `dot11QAPEDCATableTXOPLimit' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21805: [4] {group-membership} warning: node `dot11QAPEDCATableMSDULifetime' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21837: [4] {group-membership} warning: node `dot11QAPEDCATableMandatory' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:21981: [4] {group-membership} warning: node `dot11QosTransmittedFragmentCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22007: [4] {group-membership} warning: node `dot11QosFailedCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22034: [4] {group-membership} warning: node `dot11QosRetryCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22059: [4] {group-membership} warning: node `dot11QosMultipleRetryCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22084: [4] {group-membership} warning: node `dot11QosFrameDuplicateCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22109: [4] {group-membership} warning: node `dot11QosRTSSuccessCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22135: [4] {group-membership} warning: node `dot11QosRTSFailureCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22161: [4] {group-membership} warning: node `dot11QosACKFailureCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22186: [4] {group-membership} warning: node `dot11QosReceivedFragmentCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22210: [4] {group-membership} warning: node `dot11QosTransmittedFrameCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22234: [4] {group-membership} warning: node `dot11QosDiscardedFrameCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22258: [4] {group-membership} warning: node `dot11QosMPDUsReceivedCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22281: [4] {group-membership} warning: node `dot11QosRetriesReceivedCount' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:22985: [4] {group-membership} warning: node `dot11TransmitSoundingPPDUOptionImplemented' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:23004: [4] {group-membership} warning: node `dot11NumberOfActiveRxAntennas' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:25833: [4] {group-membership} warning: node `dot11ERPBCCOptionActivated' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:26168: [4] {group-membership} warning: node `dot11NumberOfSpatialStreamsImplemented' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:26191: [4] {group-membership} warning: node `dot11NumberOfSpatialStreamsActivated' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:26481: [4] {group-membership} warning: node `dot11HighestSupportedDataRate' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:26500: [4] {group-membership} warning: node `dot11TxMCSSetDefined' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:26519: [4] {group-membership} warning: node `dot11TxRxMCSSetNotEqual' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:26538: [4] {group-membership} warning: node `dot11TxMaximumNumberSpatialStreamsSupported' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:26561: [4] {group-membership} warning: node `dot11TxUnequalModulationSupported' must be contained in at least one conformance group

mibs/20100212r0P802.11REVmb_mib.txt:27250: [5] {group-unref} warning: deprecated group `dot11SMTbase' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:27317: [5] {group-unref} warning: deprecated group `dot11MACbase' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:27454: [5] {group-unref} warning: deprecated group `dot11PhyFHSSComplianceGroup' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:27558: [5] {group-unref} warning: deprecated group `dot11CountersGroup' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:27587: [5] {group-unref} warning: current group `dot11NotificationGroup' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:27605: [5] {group-unref} warning: deprecated group `dot11SMTbase2' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:27655: [5] {group-unref} warning: deprecated group `dot11PhyOFDMComplianceGroup' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:27677: [5] {group-unref} warning: deprecated group `dot11SMTbase3' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:27858: [5] {group-unref} warning: deprecated group `dot11SMTbase4' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:28025: [5] {group-unref} warning: current group `dot11RSNPMKcachingGroup' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:28046: [5] {group-unref} warning: current group `dot11RSNSMKcachingGroup' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:28069: [5] {group-unref} warning: current group `dot11SMTbase5' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:28136: [5] {group-unref} warning: deprecated group `dot11MACbase2' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:28177: [5] {group-unref} warning: deprecated group `dot11CountersGroup2' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:28233: [5] {group-unref} warning: deprecated group `dot11SMTbase6' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:28321: [5] {group-unref} warning: deprecated group `dot11PhyOFDMComplianceGroup2' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:28351: [5] {group-unref} warning: deprecated group `dot11SMTbase7' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:29234: [5] {group-unref} warning: deprecated group `dot11SMTbase8' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:29443: [5] {group-unref} warning: deprecated group `dot11SMTbase9' is not referenced in this module

mibs/20100212r0P802.11REVmb_mib.txt:292: [5] {type-without-format} warning: type `WEPKeytype' has no format specification

mibs/20100212r0P802.11REVmb_mib.txt:304: [5] {type-without-format} warning: type `TSFType' has no format specification

# Changes

The changes required in Annex D are shown as a bulleted list of changes (largely matching those made in D2.02),  plus tracked changes below (those that will be made in D2.03).

The reason for the change is shown in a comment.

Change Annex D as listed below:

- change terminology "Enabled" -> "Activated" when at the end of a MIB variable name.

- initial lower-case enumeration values in dot11HTProtection

- Change smart quotes to straight quotes throughout

- In dot11CalibrationOptionImplemented, change “STATS” to “STATUS”

- Add "Superseded by dot11MACbase” to dot11MACbase2

- Plus make any other changes to naming to provide consistency between definition and use

- Add a revision statement to the module

Change Annex D as follows:

-- Editor's Note: The <year> tag, which occurs throughout this Clause, will be

-- replaced by the year of

---publication during publication by the IEEE-SA editorial staff.

-- **********************************************************************

-- * IEEE 802.11 MIB

-- **********************************************************************

IEEE802dot11-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, NOTIFICATION-TYPE,

Integer32, Counter32, Counter64, Unsigned32,

TimeTicks  FROM SNMPv2-SMI

DisplayString , MacAddress, RowStatus, TEXTUAL-CONVENTION,

TruthValue	FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP,

NOTIFICATION-GROUP	FROM SNMPv2-CONF

ifIndex, InterfaceIndex

FROM IF-MIB;

-- **********************************************************************

-- *  MODULE IDENTITY

-- **********************************************************************

ieee802dot11 MODULE-IDENTITY

LAST-UPDATED "200908030000Z"

ORGANIZATION "IEEE 802.11"

CONTACT-INFO

"    Chair: Bruce Kraemer

Postal: 517 La Costa Court

Melbourne, FL 32940, USA

Tel: +1 321 427 4098

Fax: +1 321 751 3988

E-mail: bkraemer@ieee.org

Editor: Terry L. Cole

E-mail: terry.cole+ieee@gmail.com"

DESCRIPTION

"The MIB module for IEEE 802.11 entities.

iso(1).member-body(2).us(840).ieee802dot11(10036)"

::= { us 10036 }

-- **********************************************************************

-- *  Tree Definition

-- **********************************************************************

member-body     OBJECT IDENTIFIER ::= { iso 2 }

us              OBJECT IDENTIFIER ::= { member-body 840 }

-- **********************************************************************

-- *  Major sections

-- **********************************************************************

--  Station ManagemenT (SMT) Attributes

--  DEFINED AS "The SMT object class provides the necessary support

--  at the station to manage the processes in the station such that

--  the station may work cooperatively as a part of an IEEE 802.11

--  network."

dot11smt OBJECT IDENTIFIER ::= { ieee802dot11 1 }

--  dot11smt GROUPS

--  dot11StationConfigTable										::= { dot11smt 1 }

--  dot11AuthenticationAlgorithmsTable										::= { dot11smt 2 }

--  dot11WEPDefaultKeysTable										::= { dot11smt 3 }

--  dot11WEPKeyMappingsTable										::= { dot11smt 4 }

--  dot11PrivacyTable										::= { dot11smt 5 }

--  dot11SMTnotification										::= { dot11smt 6 }

--  dot11MultiDomainCapabilityTable										::= { dot11smt 7 }

--  dot11SpectrumManagementTable										::= { dot11smt 8 }

--  dot11RSNAConfigTable							::= { dot11smt 9 }

--  dot11RSNAConfigPairwiseCiphersTable							::= { dot11smt 10 }

--  dot11RSNAConfigAuthenticationSuitesTable							::= { dot11smt 11 }

--  dot11RSNAStatsTable							::= { dot11smt 12 }

--  dot11RegulatoryClassesTable							::= { dot11smt 13 }

--  dot11RadioResourceMeasurement						 	::= { dot11smt 14 }

--  dot11FastBSSTransitionConfigTable							::= { dot11smt 15 }

--  dot11LCIDSETable       							::= { dot11smt 16 }

--  dot11HTStationConfigTable 							::= { dot11smt 17 }

--  MAC Attributes

--  DEFINED AS "The MAC object class provides the necessary support

--  for the access control, generation, and verification of frame

--  check sequences (FCSs), and proper delivery of valid data to

--  upper layers."

dot11mac OBJECT IDENTIFIER ::= { ieee802dot11 2 }

--  MAC GROUPS

--  dot11OperationTable							::= { dot11mac 1 }

--  dot11CountersTable							::= { dot11mac 2 }

--  dot11GroupAddressesTable							::= { dot11mac 3 }

--  dot11EDCATable							::= { dot11mac 4 }

--  dot11QAPEDCATable							::= { dot11mac 5 }

--  dot11QosCountersTable							::= { dot11mac 6 }

--  dot11ResourceInfoTable 							::= { dot11mac 7 }

--  Resource Type ID

dot11res 			  OBJECT IDENTIFIER 	::= { ieee802dot11 3 }

dot11resAttribute 	OBJECT IDENTIFIER 	::= { dot11res 1 }

--  PHY Attributes

--  DEFINED AS "The PHY object class provides the necessary support

--  for required PHY operational information that may vary from PHY

--  to PHY and from STA to STA to be communicated to upper layers."

dot11phy OBJECT IDENTIFIER ::= { ieee802dot11 4 }

--  PHY GROUPS

--  dot11PhyOperationTable 	 						 ::= { dot11phy 1 }

--  dot11PhyAntennaTable   							 ::= { dot11phy 2 }

--  dot11PhyTxPowerTable    	 						 ::= { dot11phy 3 }

--  dot11PhyFHSSTable        	 						 ::= { dot11phy 4 }

--  dot11PhyDSSSTable         							 ::= { dot11phy 5 }

--  dot11PhyIRTable           							 ::= { dot11phy 6 }

--  dot11RegDomainsSupportedTable							 ::= { dot11phy 7 }

--  dot11AntennasListTable							 ::= { dot11phy 8 }

--  dot11SupportedDataRatesTxTable							 ::= { dot11phy 9 }

--  dot11SupportedDataRatesRxTable							 ::= { dot11phy 10 }

--  dot11PhyOFDMTable							 ::= { dot11phy 11 }

--  dot11PhyHRDSSSTable							 ::= { dot11phy 12 }

--  dot11HoppingPatternTable							 ::= { dot11phy 13 }

--  dot11PhyERPTable							 ::= { dot11phy 14 }

--  dot11PhyHTTable 							 ::= { dot11phy 15 }

--  dot11SupportedMCSTxTable 							 ::= { dot11phy 16 }

--  dot11SupportedMCSRxTable 							 ::= { dot11phy 17 }

--  dot11TransmitBeamformingConfigTable 							 ::= { dot11phy 18 }

-- **********************************************************************

-- *  Textual conventions from 802 definitions

-- **********************************************************************

WEPKeytype ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Represents the type of WEP key."

SYNTAX      OCTET STRING (SIZE (5))

TSFType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Represents the type of TSF."

SYNTAX      OCTET STRING (SIZE (8))

-- **********************************************************************

-- *  MIB attribute OBJECT-TYPE definitions follow

-- **********************************************************************

-- **********************************************************************

-- *  dotStationConfig  TABLE

-- **********************************************************************

dot11StationConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11StationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Station Configuration attributes.  In tabular form to allow for

multiple instances on an agent."

::= { dot11smt 1 }

dot11StationConfigEntry OBJECT-TYPE

SYNTAX Dot11StationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11StationConfigTable.  It is possible for

there to be multiple IEEE 802.11 interfaces on one agent, each

with its unique MAC address. The relationship between an IEEE

802.11 interface and an interface in the context of the Inter-

net-standard MIB is one-to-one.  As such, the value of an ifIn-

dex object instance can be directly used to identify

corresponding instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11StationConfigTable 1 }

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID	MacAddress,

dot11MediumOccupancyLimit	Unsigned32,

dot11CFPollable	TruthValue,

dot11CFPPeriod	Unsigned32,

dot11CFPMaxDuration	Unsigned32,

dot11AuthenticationResponseTimeOut	Unsigned32,

dot11PrivacyOptionImplemented	TruthValue,

dot11PowerManagementMode	INTEGER,

dot11DesiredSSID	OCTET STRING,

dot11DesiredBSSType	INTEGER,

dot11OperationalRateSet	OCTET STRING,

dot11BeaconPeriod	Unsigned32,

dot11DTIMPeriod	Unsigned32,

dot11AssociationResponseTimeOut	Unsigned32,

dot11DisassociateReason	Unsigned32,

dot11DisassociateStation	MacAddress,

dot11DeauthenticateReason	Unsigned32,

dot11DeauthenticateStation	MacAddress,

dot11AuthenticateFailStatus	Unsigned32,

dot11AuthenticateFailStation	MacAddress,

dot11MultiDomainCapabilityImplemented	TruthValue,

dot11MultiDomainCapabilityActivated	TruthValue,

dot11CountryString	OCTET STRING,

dot11SpectrumManagementImplemented	TruthValue,

dot11SpectrumManagementRequired	TruthValue,

dot11RSNAOptionImplemented	TruthValue,

dot11RSNAPreauthenticationImplemented	TruthValue,

dot11RegulatoryClassesImplemented	TruthValue,

dot11RegulatoryClassesRequired	TruthValue,

dot11QosOptionImplemented	TruthValue,

dot11ImmediateBlockAckOptionImplemented	TruthValue,

dot11DelayedBlockAckOptionImplemented	TruthValue,

dot11DirectOptionImplemented	TruthValue,

dot11APSDOptionImplemented	TruthValue,

dot11QAckOptionImplemented	TruthValue,

dot11QBSSLoadOptionImplemented	TruthValue,

dot11QueueRequestOptionImplemented	TruthValue,

dot11TXOPRequestOptionImplemented	TruthValue,

dot11MoreDataAckOptionImplemented	TruthValue,

dot11AssociateInNQBSS	TruthValue,

dot11DLSAllowedInQBSS	TruthValue,

dot11DLSAllowed	TruthValue,

dot11AssociateStation	MacAddress,

dot11AssociateID 	Unsigned32,

dot11AssociateFailStation 	MacAddress,

dot11AssociateFailStatus 	Unsigned32,

dot11ReassociateStation 	MacAddress,

dot11ReassociateID 	Unsigned32,

]

dot11ReassociateFailStation 	MacAddress,

dot11ReassociateFailStatus 	Unsigned32,

dot11RadioMeasurementImplemented 	TruthValue,

dot11RadioMeasurementActivated 	TruthValue,

dot11RMMeasurementProbeDelay 	Unsigned32,

dot11RMMeasurementPilotPeriod 	Unsigned32,

dot11RMLinkMeasurementActivated 	TruthValue,

dot11RMNeighborReportActivated 	TruthValue,

dot11RMParallelMeasurementsActivated 	TruthValue,

dot11RMRepeatedMeasurementsActivated 	TruthValue,

dot11RMBeaconPassiveMeasurementActivated 	TruthValue,

dot11RMBeaconActiveMeasurementActivated 	TruthValue,

dot11RMBeaconTableMeasurementActivated 	TruthValue,

dot11RMBeaconMeasurementReportingConditionsActivated 	TruthValue,

dot11RMFrameMeasurementActivated 	TruthValue,

dot11RMChannelLoadMeasurementActivated 	TruthValue,

dot11RMNoiseHistogramMeasurementActivated 	TruthValue,

dot11RMStatisticsMeasurementActivated 	TruthValue,

dot11RMLCIMeasurementActivated 	TruthValue,

dot11RMLCIAzimuthActivated 	TruthValue,

dot11RMTransmitStreamCategoryMeasurementActivated 	TruthValue,

dot11RMTriggeredTransmitStreamCategoryMeasurementActivated

TruthValue,

dot11RMAPChannelReportActivated 	TruthValue,

dot11RMMIBActivated 	TruthValue,

dot11RMMaxMeasurementDuration 	Unsigned32,

dot11RMNonOperatingChannelMaxMeasurementDuration 	Unsigned32,

dot11RMMeasurementPilotTransmissionInformationActivated	TruthValue,

dot11RMMeasurementPilotActivated 	Unsigned32,

dot11RMNeighborReportTSFOffsetActivated 	TruthValue,

dot11RMRCPIMeasurementActivated 	TruthValue,

dot11RMRSNIMeasurementActivated 	TruthValue,

dot11RMBSSAverageAccessDelayActivated 	TruthValue,

dot11RMBSSAvailableAdmissionCapacityActivated 	TruthValue,

dot11RMAntennaInformationActivated 	TruthValue,

dot11FastBSSTransitionImplemented 	TruthValue,

dot11LCIDSEImplemented 	TruthValue,

dot11LCIDSERequired  	TruthValue,

dot11DSERequired 	TruthValue,

dot11ExtendedChannelSwitchActivated 	TruthValue,

dot11RSNAProtectedManagementFramesActivated	TruthValue,

dot11RSNAUnprotectedManagementFramesAllowed	TruthValue,

dot11AssociationSAQueryMaximumTimeout	Unsigned32,

dot11AssociationSAQueryRetryTimeout	Unsigned32,

dot11HighThroughputOptionImplemented	TruthValue,

dot11RSNAPBACRequired	TruthValue,

dot11PSMPOptionImplemented	TruthValue,

dot11RejectUnadmittedTraffic	TruthValue,

dot11BSSBroadcastNullCount	Unsigned32 }

dot11StationID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive.

The purpose of dot11StationID is to allow a manager to identify

a station for its own purposes.  This attribute provides for

that eventuality while keeping the true MAC address independent.

Its syntax is MAC address, and the default value is the sta-

tion's assigned, unique MAC address."

::= { dot11StationConfigEntry 1 }

dot11MediumOccupancyLimit OBJECT-TYPE

SYNTAX Unsigned32 (0..1000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the maximum amount of time, in TU, that

a point coordinator (PC) may control the usage of the wireless

medium (WM) without relinquishing control for long enough to

allow at least one instance of DCF access to the medium.  The

maximum value of this variable is 1000."

DEFVAL { 100 }

::= { dot11StationConfigEntry 2 }

dot11CFPollable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

When this attribute is true, it indicates that the STA is able

to respond to a CF-Poll with a data frame within a SIFS time.

This attribute is false if the STA is not able to respond to a

CF-Poll with a data frame within a SIFS time."

::= { dot11StationConfigEntry 3 }

dot11CFPPeriod OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

In an AP, it is written by the MAC when it receives an MLME-

START.request primitive.

In a non-AP STA, it is written by the MAC when it receives an

updated CF Parameter Set in a Beacon frame.

The attribute describes the number of DTIM intervals between the

start of CFPs.  It is modified by MLME-START.request primitive."

::= { dot11StationConfigEntry 4 }

dot11CFPMaxDuration OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

In an AP, it is written by the MAC when it receives an MLME-

START.request primitive.

In a non-AP STA, it is written by the MAC when it receives an

updated CF Parameter Set in a Beacon frame.

The attribute describes the maximum duration of the CFP in TU

that may be generated by the PCF. It is modified by MLME-

START.request primitive."

::= { dot11StationConfigEntry 5 }

dot11AuthenticationResponseTimeOut OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next Authentication frame.

This attribute specifies the number of time units (TUs) that a

responding STA should wait for the next frame in the authentica-

tion sequence."

::= { dot11StationConfigEntry 6 }

dot11PrivacyOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the IEEE

802.11 WEP option is implemented. "

DEFVAL { false }

::= { dot11StationConfigEntry 7 }

dot11PowerManagementMode OBJECT-TYPE

SYNTAX INTEGER { active(1), powersave(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when an MLME-POWERMGT.request primitive

is received.

This attribute specifies the power management mode of the STA.

When set to active, it indicates that the station is not in

power-save (PS) mode. When set to powersave, it indicates that

the station is in power-save mode. The power management mode is

transmitted in all frames according to the rules in  7.1.3.1.7

(More Data field)."

::= { dot11StationConfigEntry 8 }

dot11DesiredSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-SCAN.request primitive.

This attribute reflects the Service Set ID (SSID) used in the

DesiredSSID parameter of the most recent MLME-SCAN.request prim-

itive.  This value may be modified by an external management

entity and used by the local SME to make decisions about the

Scanning process."

::= { dot11StationConfigEntry 9 }

dot11DesiredBSSType OBJECT-TYPE

SYNTAX INTEGER { infrastructure(1), independent(2), any(3) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-SCAN.request primitive.

This attribute specifies the type of BSS the station uses when

scanning for a BSS with which to synchronize. This value is used

to filter Probe Response frames and Beacon frames. When set to

infrastructure, the station synchronizes only with a BSS in

which the Capability Information field has the ESS subfield set

to 1. When set to independent, the station synchronizes only

with a BSS whose Capability Information field has the IBSS sub-

field set to 1. When set to any, the station may synchronize to

either type of BSS."

::= { dot11StationConfigEntry 10 }

dot11OperationalRateSet OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..126))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when joining or establishing a BSS.

This attribute specifies the set of non-HT data rates at which

the station may transmit data. The attribute that specifies the

set of HT data rates is dot11HTOperationalMCSSet.  Each octet

contains a value representing a rate.  Each rate is within the

range from 2 to 127, corresponding to data rates in increments

of 500 kbit/s from 1 Mb/s to 63.5 Mb/s, and is supported (as

indicated in the supported rates table) for receiving data. This

value is reported in transmitted Beacon, Probe Request, Probe

Response, Association Request, Association Response, Reassocia-

tion Request, and Reassociation Response frames, and is used to

determine whether a BSS with which the station desires to syn-

chronize is suitable. It is also used when starting a BSS, as

specified in  10.3.3 (Synchronization)."

::= { dot11StationConfigEntry 11 }

dot11BeaconPeriod OBJECT-TYPE

SYNTAX Unsigned32 (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive.

This attribute specifies the number of TUs that a station uses

for scheduling Beacon transmissions. This value is transmitted

in Beacon and Probe Response frames."

::= { dot11StationConfigEntry 12 }

dot11DTIMPeriod OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive.

This attribute specifies the number of beacon intervals that

elapse between transmission of Beacon frames containing a TIM

element whose DTIM Count field is 0. This value is transmitted

in the DTIM Period field of Beacon frames."

::= { dot11StationConfigEntry 13 }

dot11AssociationResponseTimeOut OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of TU that a requesting STA

should wait for a response to a transmitted association-request

MMPDU."

::= { dot11StationConfigEntry 14 }

dot11DisassociateReason OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA disassociates.

This attribute holds the most recently transmitted Reason Code

in a Disassociation frame.  If no Disassociation frame has been

transmitted, the value of this attribute is 0."

REFERENCE "IEEE Std 802.11-<year>, 7.3.1.7 (Reason Code field)"

::= { dot11StationConfigEntry 15 }

dot11DisassociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA disassociates.

This attribute holds the MAC address from the Address 1 field of

the most recently transmitted Disassociation frame.  If no Dis-

association frame has been transmitted, the value of this

attribute is 0."

::= { dot11StationConfigEntry 16 }

dot11DeauthenticateReason OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA deauthenticates.

This attribute holds the most recently transmitted Reason Code

in a Deauthentication frame.  If no Deauthentication frame has

been transmitted, the value of this attribute is 0."

REFERENCE "IEEE Std 802.11-<year>, 7.3.1.7 (Reason Code field)"

::= { dot11StationConfigEntry 17 }

dot11DeauthenticateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA deauthenticates.

This attribute holds the MAC address from the Address 1 field of

the most recently transmitted Deauthentication frame.  If no

Deauthentication frame has been transmitted, the value of this

attribute is 0."

::= { dot11StationConfigEntry 18 }

dot11AuthenticateFailStatus OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA fails to authenticate.

This attribute holds the most recently transmitted Status Code

in a failed Authentication frame.  If no failed Authentication

frame has been transmitted, the value of this attribute is 0."

REFERENCE "IEEE Std 802.11-<year>, 7.3.1.9 (Status Code field)"

::= { dot11StationConfigEntry 19 }

dot11AuthenticateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA fails to authenticate.

This attribute holds the MAC address from the Address 1 field of

the most recently transmitted failed Authentication frame.  If

no failed Authentication frame has been transmitted, the value

of this attribute is 0."

::= { dot11StationConfigEntry 20 }

dot11MultiDomainCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of supporting multiple regulatory domains.

The capability is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 21 }

dot11MultiDomainCapabilityActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive.

This attribute, when true, indicates that the capability of the

station to operate in multiple regulatory domains is enabled.

The capability is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 22 }

dot11CountryString OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive.

This attribute identifies the country or non-country entity in

which the station is operating. If it is a country, the first

two octets of this string is the two character country code as

described in document ISO/IEC 3166-1.  The third octet is one of

the following:

1. an ASCII space character, if the regulations under which the

station is operating encompass all environments for the current

frequency band in the country,

2. an ASCII 'O' character, if the regulations under which the

station is operating are for an Outdoor environment only, or

3. an ASCII 'I' character, if the regulations under which the

station is operating are for an Indoor environment only.

4. an ASCII 'X' character, if the station is operating under a

non-country entity. The first two octets of the non-country

entity is two ASCII 'XX' characters.

5. the binary representation of the Regulatory Class table num-

ber currently in use, from the set of tables defined in Annex J,

e.g. Table J.1 is represented as x '01'."

::= { dot11StationConfigEntry 23 }

dot11SpectrumManagementImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of supporting spectrum management. The capa-

bility is disabled otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 24 }

dot11SpectrumManagementRequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive.

A STA will use the defined TPC and DFS procedures if this

attribute is true; otherwise it will not use the defined TPC and

DFS procedures."

DEFVAL { false }

::= { dot11StationConfigEntry 25 }

dot11RSNAOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This variable indicates whether the entity is RSNA-capable."

::= { dot11StationConfigEntry 26 }

dot11RSNAPreauthenticationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This variable indicates whether the entity supports RSNA

preauthentication. This cannot be true unless

dot11RSNAOptionImplemented is true."

::= { dot11StationConfigEntry 27 }

dot11RegulatoryClassesImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of supporting regulatory classes. The capabil-

ity is disabled otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 28 }

dot11RegulatoryClassesRequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive.

A STA will use the defined regulatory classes procedures if this

attribute is true; otherwise it will not use the defined regula-

tory classes' procedures."

DEFVAL { false }

::= { dot11StationConfigEntry 29 }

dot11QosOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of supporting QoS. The capability is disabled,

otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 30 }

dot11ImmediateBlockAckOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of supporting Immediate Block Ack. The capa-

bility is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 31 }

dot11DelayedBlockAckOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of supporting Delayed Block Ack. The capabil-

ity is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 32 }

dot11DirectOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of sending and receiving frames from a non-AP

STA in the BSS. The capability is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 33 }

dot11APSDOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute is available only at an AP. When true, this

attribute indicates that the AP implementation is capable of

delivering data and polls to stations using APSD."

::= { dot11StationConfigEntry 34 }

dot11QAckOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of interpreting the CF-Ack bit in a received

frame of type data even if the frame is not directed to the QoS

station. The capability is disabled, otherwise. A STA is capable

of interpreting the CF-Ack bit in a received data frame if that

station is the recipient of the data frame, regardless of the

value of this MIB variable."

DEFVAL { false }

::= { dot11StationConfigEntry 35 }

dot11QBSSLoadOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute is available only at an AP. This attribute, when

true, indicates that the AP implementation is capable of gener-

ating and transmitting the BSS load element in the Beacon and

Probe Response frames. The capability is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 36 }

dot11QueueRequestOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute is available only at an AP. This attribute, when

true, indicates that the AP is capable of processing the Queue

Size field in QoS Control field of QoS Data type frames. The

capability is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 37 }

dot11TXOPRequestOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute is available only at an AP. This attribute, when

true, indicates that the AP is capable of processing the TXOP

Duration requested field in QoS Control field of QoS Data type

frames. The capability is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 38 }

dot11MoreDataAckOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of interpreting the More Data bit in the ACK

frames. The capability is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 39 }

dot11AssociateInNQBSS OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that the station may asso-

ciate in a non-QoS BSS. When false, this attribute indicates

that the station will not associate in a non-QoS BSS."

DEFVAL { true }

::= { dot11StationConfigEntry 40 }

dot11DLSAllowedInQBSS OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute available at the AP, when true, indicates that

STAs may set up DLS and communicate using DLS with other STAs in

the BSS. When false, this attribute indicates that the stations

will not set up DLS nor communicate with other STAs using DLS in

the BSS."

DEFVAL { true }

::= { dot11StationConfigEntry 41 }

dot11DLSAllowed OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute available at non-AP STAs, when true, indicates

that the STA may set up DLS or accept DLS Requests, and communi-

cate with other STAs in the BSS using DLS. When false, this

attribute indicates that the STA will not set up DLS nor accept

DLS, nor communicate with other STAs in the BSS using DLS."

DEFVAL { true }

::= { dot11StationConfigEntry 42 }

dot11AssociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA associates.

This attribute indicates the MAC address from the Address 1

field of the most recently transmitted association response

frame. If no association response frame has been transmitted,

the value of this attribute is 0."

::= { dot11StationConfigEntry 43 }

dot11AssociateID OBJECT-TYPE

SYNTAX Unsigned32 (0..2007)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA associates.

This attribute indicates the Association ID from the most

recently transmitted association response frame. If no associa-

tion response frame has been transmitted, the value of this

attribute is 0."

::= { dot11StationConfigEntry 44 }

dot11AssociateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA fails to associate.

This attribute indicates the MAC address from the Address 1

field of the most recently transmitted failed association

response frame. If no failed association response frame has been

transmitted, the value of this attribute is 0."

::= { dot11StationConfigEntry 45 }

dot11AssociateFailStatus OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA fails to associate.

This attribute indicates the most recently transmitted Status

Code in a failed association response frame. If no failed asso-

ciation response frame has been transmitted, the value of this

attribute is 0."

::= { dot11StationConfigEntry 46 }

dot11ReassociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA reassociates.

This attribute indicates the MAC address from the Address 1

field of the most recently transmitted reassociation response

frame. If no reassociation response frame has been transmitted,

the value of this attribute is 0."

::= { dot11StationConfigEntry 47 }

dot11ReassociateID OBJECT-TYPE

SYNTAX Unsigned32 (0..2007)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA reassociates.

This attribute indicates the Association ID from the most

recently transmitted reassociation response frame. If no

reassociation response frame has been transmitted, the value of

this attribute is 0."

::= { dot11StationConfigEntry 48 }

dot11ReassociateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA fails to reassociate.

This attribute indicates the MAC address from the Address 1

field of the most recently transmitted failed reassociation

response frame. If no failed reassociation response frame has

been transmitted, the value of this attribute is 0."

::= { dot11StationConfigEntry 49 }

dot11ReassociateFailStatus OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA fails to reassociate.

This attribute indicates the most recently transmitted Status

Code in a failed reassociation response frame. If no failed

reassociation response frame has been transmitted, the value of

this attribute is 0."

::= { dot11StationConfigEntry 50 }

dot11RadioMeasurementImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of supporting Radio Measurement. Otherwise it

is not capable of performing Radio Measurement."

DEFVAL { false }

::= { dot11StationConfigEntry 51 }

dot11RadioMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when any of the

MIB variables dot11RMxxxxActivated is set to true.

Changes take effect with the next MLME-START.request primitive

or MLME-JOIN.request primitive.

This attribute, when true, indicates that one or more of the

Radio Measurement Activated Capabilities MIB attributes

(dot11RMxxxActivated), listed in 7.3.2.45 ((#1704)RM Enabled

Capabilities element(11k)(#1720)), are set to true. A STA may

use the defined Radio Measurement procedures if this attribute

is true."

DEFVAL { false }

::= { dot11StationConfigEntry 52 }

dot11RMMeasurementProbeDelay OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The value of ProbeDelay to be used when making a beacon type

measurement with measurement mode active when

dot11RMActiveBeaconMeasurementActivated is true."

::= { dot11StationConfigEntry 53 }

dot11RMMeasurementPilotPeriod OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of TUs that a station uses

for scheduling Measurement Pilot transmissions. This value is

transmitted in Measurement Pilot frames. The default period is

25% of dot11BeaconPeriod."

::= { dot11StationConfigEntry 54 }

dot11RMLinkMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Link Measurement is enabled. A value of false

indicates the station has no Link Measurement capability or that

the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 55 }

dot11RMNeighborReportActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Neighbor Report is enabled. false indicates the

station has no Neighbor Report capability or that the capability

is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 56 }

dot11RMParallelMeasurementsActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Parallel Measurements is enabled. false indicates

the station has no Parallel Measurements capability or that the

capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 57 }

dot11RMRepeatedMeasurementsActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Repeated Measurments is enabled. false indicates

the station has no Repeated Measurments capability or that the

capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 58 }

dot11RMBeaconPassiveMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Beacon Passive Measurement is enabled. false

indicates the station has no Beacon Passive Measurement capabil-

ity or that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 59 }

dot11RMBeaconActiveMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Beacon Active Measurement is enabled. false indi-

cates the station has no Beacon Active Measurement capability or

that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 60 }

dot11RMBeaconTableMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Beacon Table Measurement is enabled. false indi-

cates the station has no Beacon Table Measurement capability or

that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 61 }

dot11RMBeaconMeasurementReportingConditionsActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Beacon Measurement Reporting Conditions is

enabled. false indicates the station has no Beacon Measurement

Reporting Conditions capability or that the capability is

present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 62 }

dot11RMFrameMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Frame Measurement is enabled. false indicates the

station has no Frame Measurement capability or that the capabil-

ity is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 63 }

dot11RMChannelLoadMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Channel Load Measurement is enabled. false indi-

cates the station has no Channel Load Measurement capability or

that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 64 }

dot11RMNoiseHistogramMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Noise Histogram Measurement is enabled. false

indicates the station has no Noise Histogram Measurement capa-

bility or that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 65 }

dot11RMStatisticsMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Statistics Measurement is enabled. false indi-

cates the station has no Statistics Measurement capability or

that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 66 }

dot11RMLCIMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for LCI Measurement is enabled. false indicates the

station has no LCI Measurement capability or that the capability

is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 67 }

dot11RMLCIAzimuthActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for LCI Azimuth Measurement is enabled. false indi-

cates the station has no LCI Azimuth Measurement capability or

that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 68 }

dot11RMTransmitStreamCategoryMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Transmit Stream/Category Measurement is enabled.

false indicates the station has no Transmit Stream/Category Mea-

surement capability or that the capability is present but is

disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 69 }

dot11RMTriggeredTransmitStreamCategoryMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Triggered Transmit Stream/Category Measurement is

enabled. false indicates the station has no Triggered Transmit

Stream/Category Measurement capability or that the capability is

present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 70 }

dot11RMAPChannelReportActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for AP Channel Report is enabled. false indicates the

station has no AP Channel Report capability or that the capabil-

ity is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 71 }

dot11RMMIBActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for RM MIB is enabled. false indicates the station

has no RM MIB capability or that the capability is present but

is disabled. See RM MIB details."

DEFVAL { false }

::= { dot11StationConfigEntry 72 }

dot11RMMaxMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32 (0 .. 7)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the maximum measurement duration for

operating channel measurements, where

Max Measurement Duration in TUs = 2<superscript>

(dot11RMMaxMeasurementDuration - 4)</superscript> * BeaconInter-

val

Further details are provided in 11.10.3 (Measurement Dura-

tion(11k))"

::= { dot11StationConfigEntry 73 }

dot11RMNonOperatingChannelMaxMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32 (0 .. 7)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute indicates the maximum measurement duration for

non-operating channel measurements, where

Non-OpMax Measurement Duration in TUs = 2<superscript>

(dot11RMNonOperatingChannelMaxMeasurementDuration - 4)

</superscript> * BeaconInterval

Further details are provided in 11.10.3 (Measurement Dura-

tion(11k))"

::= { dot11StationConfigEntry 74 }

dot11RMMeasurementPilotTransmissionInformationActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Measurement Pilot Transmission information is

enabled. false indicates the station has no Measurement Pilot

Transmission information capability or that the capability is

present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 75 }

dot11RMMeasurementPilotActivated OBJECT-TYPE

SYNTAX Unsigned32 (0 .. 7)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the station capability for Measurement

Pilot. 0 indicates the station has no Measurement Pilot capabil-

ity or that the capability is present but is disabled. Activated

values 1-7 are defined in Table 11-11 (Measurement Pilot Acti-

vated(#1005) definition)."

DEFVAL { 0 }

::= { dot11StationConfigEntry 76 }

dot11RMNeighborReportTSFOffsetActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Neighbor Report TSF Offset is enabled. false

indicates the station has no Neighbor Report TSF Offset capabil-

ity or that the capability is present but is disabled. The "

DEFVAL { false }

::= { dot11StationConfigEntry 77 }

dot11RMRCPIMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for RCPI Measurement is enabled. false indicates the

station has no RCPI Measurement capability or that the capabil-

ity is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 78 }

dot11RMRSNIMeasurementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for RSNI Measurement is enabled. false indicates the

station has no RSNI Measurement capability or that the capabil-

ity is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 79 }

dot11RMBSSAverageAccessDelayActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for BSS Average Access Delay is enabled. false indi-

cates the station has no BSS Average Access Delay capability or

that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 80 }

dot11RMBSSAvailableAdmissionCapacityActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for BSS Available Admission Capacity is enabled.

false indicates the station has no BSS Available Admission

Capacity capability or that the capability is present but is

disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 81 }

dot11RMAntennaInformationActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that

dot11RadioMeasurementActivated is true and that the station

capability for Antenna information is enabled. false indicates

the station has no Antenna information capability or that the

capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 82 }

dot11FastBSSTransitionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This object indicates if the entity is fast BSS transition capa-

ble."

::= { dot11StationConfigEntry 83 }

dot11LCIDSEImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of supporting LCI DSE. The capability is dis-

abled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 84 }

dot11LCIDSERequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

A STA will use the Dependent Station Enablement procedures if

this attribute is true."

DEFVAL { false }

::= { dot11StationConfigEntry 85 }

dot11DSERequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

This attribute, when true, indicates that the station operation

is dependent on enablement from enabling STAs. This attribute,

when false, indicates that the station operation does not depend

on enablement from STAs."

DEFVAL { false }

::= { dot11StationConfigEntry 86 }

dot11ExtendedChannelSwitchActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

This attribute, when true, indicates that the station implemen-

tation is capable of supporting Extended Channel Switch

Announcement. The capability is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 87 }

--**********************************************************

--* Management Frame Protection MIBs

--**********************************************************

dot11RSNAProtectedManagementFramesActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This variable indicates whether or not this STA enables Manage-

ment Frame Protection."

DEFVAL { false }

::= { dot11StationConfigEntry 88 }

dot11RSNAUnprotectedManagementFramesAllowed OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This variable indicates whether or not this STA supports RSNA

STAs which do not provide Robust Management frames protection."

DEFVAL { true }

::= { dot11StationConfigEntry 89 }

--**********************************************************

--* SA Query Procedure MIBs

--**********************************************************

dot11AssociationSAQueryMaximumTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of time units (TUs) that an

AP can wait, from the scheduling of the first SA Query Request

to allow association process to be started without starting

additional SA Query procedure if a successful SA Query Response

is not received."

DEFVAL { 1000 }

::= { dot11StationConfigEntry 90 }

dot11AssociationSAQueryRetryTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of time units (TUs) that an

AP waits between issuing two subsequent MLME-SAQuery.request

primitive primitives."

DEFVAL { 201 }

::= { dot11StationConfigEntry 91 }

dot11HighThroughputOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether the entity is HT Capable."

::= { dot11StationConfigEntry 92 }

dot11RSNAPBACRequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This variable indicates whether this STA requires the Protec-

tion of Block Ack agreements."

DEFVAL { false }

::= { dot11StationConfigEntry 93 }

dot11PSMPOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable of supporting PSMP."

DEFVAL { false }

::= { dot11StationConfigEntry 94 }

--Editor's Note: **********************************************************

--Editor's Note: * Mib variables introduced by REVmb

--Editor's Note: These are given a temporary starting offset of 1000.

--Editor's Note: Once all approved amendments have been incorporated,  these numbers

--Editor's Note: will be adjusted to remove any holes in the numbering.

--Editor's Note: **********************************************************

dot11RejectUnadmittedTraffic OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect at the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute when true indicates the STA's MA-UNITDATA primi-

tive rejects frames whose requested priority corresponds to an

AC for which admission control is mandatory and for which there

is not an admitted TSPEC."

DEFVAL { false }

::= { dot11StationConfigEntry 1000 }

dot11BSSBroadcastNullCount OBJECT-TYPE

SYNTAX Unsigned32 (1..64)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of group addressed Null data

frames a STA may transmit before it changes power management

modes."

DEFVAL { 3 }

::= {dot11StationConfigEntry 1001 }

-- **********************************************************************

-- *    End of dot11StationConfig  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11AuthenticationAlgorithms  TABLE

-- **********************************************************************

dot11AuthenticationAlgorithmsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11AuthenticationAlgorithmsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This (conceptual) table of attributes is a set of all the

authentication algorithms supported by the stations.  The fol-

lowing are the default values and the associated algorithm:

Value = 1: Open System

Value = 2: Shared Key

Value = 3: Fast BSS Transition (FT)"

REFERENCE "IEEE Std 802.11-<year>, 7.3.1.1 (Authentication Algorithm Number

field)"

::= { dot11smt 2 }

dot11AuthenticationAlgorithmsEntry OBJECT-TYPE

SYNTAX Dot11AuthenticationAlgorithmsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the Authentication Algorithms

Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11AuthenticationAlgorithmsIndex }

::= { dot11AuthenticationAlgorithmsTable  1 }

Dot11AuthenticationAlgorithmsEntry ::=

SEQUENCE { dot11AuthenticationAlgorithmsIndex										Unsigned32,

dot11AuthenticationAlgorithm      										INTEGER,

dot11AuthenticationAlgorithmsActivated										TruthValue }

dot11AuthenticationAlgorithmsIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the Authentication Algorithms Table."

::= { dot11AuthenticationAlgorithmsEntry 1 }

dot11AuthenticationAlgorithm OBJECT-TYPE

SYNTAX INTEGER { openSystem(1), sharedKey(2), fastBSSTransition(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute is the authentication algorithm described by this

entry in the table. The following values can be used here

Value = 1: Open System

Value = 2: Shared Key

Value = 3: Fast BSS Transition (FT)"

::= { dot11AuthenticationAlgorithmsEntry 2 }

dot11AuthenticationAlgorithmsActivated  OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true at a station, enables the acceptance

of the authentication algorithm described in the corresponding

table entry in authentication frames received by the station

that have odd authentication sequence numbers.  The default

value of this attribute is true when the value of

dot11AuthenticationAlgorithm is Open System and false other-

wise."

::= { dot11AuthenticationAlgorithmsEntry 3 }

-- **********************************************************************

-- *    End of dot11AuthenticationAlgorithms  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11WEPDefaultKeys  TABLE

-- **********************************************************************

dot11WEPDefaultKeysTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WEPDefaultKeysEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for WEP default keys.  This table contains the

four WEP default secret key values corresponding to the four

possible KeyID values.  The WEP default secret keys are logi-

cally WRITE-ONLY.  Attempts to read the entries in this table

return unsuccessful status and values of null or zero.  The

default value of each WEP default key is null."

REFERENCE "IEEE Std 802.11-<year>, 8.2.1 (Wired equivalent privacy (WEP))"

::= { dot11smt 3 }

dot11WEPDefaultKeysEntry OBJECT-TYPE

SYNTAX Dot11WEPDefaultKeysEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the WEP Default Keys Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11WEPDefaultKeyIndex }

::= { dot11WEPDefaultKeysTable  1 }

Dot11WEPDefaultKeysEntry ::=

SEQUENCE { dot11WEPDefaultKeyIndex     Unsigned32,

dot11WEPDefaultKeyValue     WEPKeytype }

dot11WEPDefaultKeyIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the WEP Default Keys Table. The value of this

variable is equal to the WEPDefaultKeyID + 1"

::= { dot11WEPDefaultKeysEntry 1 }

dot11WEPDefaultKeyValue OBJECT-TYPE

SYNTAX WEPKeytype

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

A WEP default secret key value."

::= { dot11WEPDefaultKeysEntry 2 }

-- **********************************************************************

-- *    End of dot11WEPDefaultKeys  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11WEPKeyMappings  TABLE

-- **********************************************************************

dot11WEPKeyMappingsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WEPKeyMappingsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for WEP Key Mappings.  The MIB supports the

ability to share a separate WEP key for each RA/TA pair.  The Key

Mappings Table contains zero or one entry for each MAC address

and contains two fields for each entry: WEPOn and the corre-

sponding WEP key.  The WEP key mappings are logically WRITE-

ONLY.  Attempts to read the entries in this table return unsuc-

cessful status and values of null or zero.  The default value

for all WEPOn fields is false."

REFERENCE "IEEE Std 802.11-<year>, 8.2.1 (Wired equivalent privacy (WEP))"

::= { dot11smt 4 }

dot11WEPKeyMappingsEntry OBJECT-TYPE

SYNTAX Dot11WEPKeyMappingsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the WEP Key Mappings Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11WEPKeyMappingIndex }

::= { dot11WEPKeyMappingsTable  1 }

Dot11WEPKeyMappingsEntry ::=

SEQUENCE { dot11WEPKeyMappingIndex								Unsigned32,

dot11WEPKeyMappingAddress								MacAddress,

dot11WEPKeyMappingWEPOn								TruthValue,

dot11WEPKeyMappingValue								WEPKeytype,

dot11WEPKeyMappingStatus								RowStatus }

dot11WEPKeyMappingIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the WEP Key Mappings Table."

::= { dot11WEPKeyMappingsEntry 1 }

dot11WEPKeyMappingAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The MAC address of the STA for which the values from this key

mapping entry are to be used."

::= { dot11WEPKeyMappingsEntry 2 }

dot11WEPKeyMappingWEPOn OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Boolean as to whether WEP is to be used when communicating with

the dot11WEPKeyMappingAddress STA."

::= { dot11WEPKeyMappingsEntry 3 }

dot11WEPKeyMappingValue OBJECT-TYPE

SYNTAX WEPKeytype

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"A WEP secret key value."

::= { dot11WEPKeyMappingsEntry 4 }

dot11WEPKeyMappingStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the WEP key mapping Table."

DEFVAL { active }

::= { dot11WEPKeyMappingsEntry 5 }

-- **********************************************************************

-- *    End of dot11WEPKeyMappings  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11Privacy TABLE

-- **********************************************************************

dot11PrivacyTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PrivacyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes concerned with IEEE 802.11 Privacy.

Created as a table to allow multiple instantiations on an

agent."

::= { dot11smt 5 }

dot11PrivacyEntry OBJECT-TYPE

SYNTAX Dot11PrivacyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PrivacyTable Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11PrivacyTable 1 }

Dot11PrivacyEntry ::=

SEQUENCE { dot11PrivacyInvoked										TruthValue,

dot11WEPDefaultKeyID										Unsigned32,

dot11WEPKeyMappingLengthImplemented										Unsigned32,

dot11ExcludeUnencrypted										TruthValue,

dot11WEPICVErrorCount										Counter32,

dot11WEPExcludedCount										Counter32,

dot11RSNAActivated								TruthValue,

dot11RSNAPreauthenticationActivated								TruthValue }

dot11PrivacyInvoked OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

When this attribute is true, it indicates that some level of

security is invoked for transmitting data frames. For WEP-only

clients, the security mechanism used is WEP.

For RSNA-capable clients, an additional variable

dot11RSNAActivated indicates whether RSNA is enabled. If

dot11RSNAActivated is false or the MIB variable does not exist,

the security mechanism invoked is WEP; if dot11RSNAActivated is

true, RSNA security mechanisms invoked are configured in the

dot11RSNAConfigTable."

DEFVAL { false }

::= { dot11PrivacyEntry 1 }

dot11WEPDefaultKeyID  OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the use of the first, second, third, or

fourth element of the WEPDefaultKeys array when set to values of

zero, one, two, or three."

REFERENCE "IEEE Std 802.11-<year>, 8.2.1 (Wired equivalent privacy (WEP))"

DEFVAL { 0 }

::= { dot11PrivacyEntry 2 }

dot11WEPKeyMappingLengthImplemented  OBJECT-TYPE

SYNTAX Unsigned32 (10..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The maximum number of tuples that dot11WEPKeyMappings can hold."

REFERENCE "IEEE Std 802.11-<year>, 8.2.1 (Wired equivalent privacy (WEP))"

::= { dot11PrivacyEntry 3 }

dot11ExcludeUnencrypted  OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

When this attribute is true, the STA does not indicate at the

MAC service interface received MSDUs that have the Protected

Frame subfield of the Frame Control field equal to zero. When

this attribute is false, the STA may accept MSDUs that have the

Protected Frame subfield of the Frame Control field equal to

zero."

DEFVAL { false }

::= { dot11PrivacyEntry 4 }

dot11WEPICVErrorCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when an ICV error is detected.

This counter increments when a frame is received with the

Protected Frame subfield of the Frame Control field set to one

and the value of the ICV as received in the frame does not match

the ICV value that is calculated for the contents of the

received frame. ICV errors for TKIP are not counted in this

variable but in dot11RSNAStatsTKIPICVErrors."

::= { dot11PrivacyEntry 5 }

dot11WEPExcludedCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a bad frame is received.

This counter increments when a frame is received with the

Protected Frame subfield of the Frame Control field set to zero

and the value of dot11ExcludeUnencrypted causes that frame to be

discarded."

::= { dot11PrivacyEntry 6 }

dot11RSNAActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

When this object is true, this indicates that RSNA is enabled on

this entity. The entity will advertise the RSN  element in its

Beacon and Probe Response frames. Configuration variables for

RSNA operation are found in the dot11RSNAConfigTable.

This object requires that dot11PrivacyInvoked also be set to

true."

::= { dot11PrivacyEntry 7 }

dot11RSNAPreauthenticationActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

When this object is true, this indicates that RSNA preauthenti-

cation is enabled on this entity.

This object requires that dot11RSNAActivated also be set to

true."

::= { dot11PrivacyEntry 8 }

-- **********************************************************************

-- *    End of dot11Privacy  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11SMTnotification Objects

-- **********************************************************************

dot11SMTnotification OBJECT IDENTIFIER ::= { dot11smt 6 }

dot11Disassociate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11DisassociateReason, dot11DisassociateStation }

STATUS current

DESCRIPTION

"The disassociate notification is sent when the STA sends a Dis-

association frame. The value of the notification includes the

MAC address of the MAC to which the Disassociation frame was

sent and the reason for the disassociation.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

::= { dot11SMTnotification 0 1 }

dot11Deauthenticate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11DeauthenticateReason,

dot11DeauthenticateStation }

STATUS current

DESCRIPTION

"The deauthenticate notification is sent when the STA sends a

Deauthentication frame. The value of the notification includes

the MAC address of the MAC to which the Deauthentication frame

was sent and the reason for the deauthentication.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

::= { dot11SMTnotification 0 2 }

dot11AuthenticateFail NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AuthenticateFailStatus,

dot11AuthenticateFailStation }

STATUS current

DESCRIPTION

"The authenticate failure notification is sent when the STA

sends an Authentication frame with a status code other than

'successful'. The value of the notification includes the MAC

address of the MAC to which the Authentication frame was sent

and the reason for the authentication failure.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

::= { dot11SMTnotification 0 3 }

dot11Associate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AssociateStation, dot11AssociateID }

STATUS current

DESCRIPTION

"The associate notification is sent when the STA sends an Asso-

ciation Response frame with a status code equal to 'Successful.'

The value of the notification includes the MAC address of the

MAC to which the Association Response frame was sent and the

Association ID. ifIndex - Each 802.11 interface is represented

by an ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 4 }

dot11AssociateFailed NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AssociateFailStatus,

dot11AssociateFailStation }

STATUS current

DESCRIPTION

"The associate failed notification is sent when the STA sends an

Association Response frame with a status code other than 'Suc-

cessful.' The value of the notification includes the MAC address

of the MAC to which the Association Response frame was sent and

the reason for the association failure. ifIndex - Each 802.11

interface is represented by an ifEntry. Interface tables in this

MIB module are indexed by ifIndex."

::= { dot11SMTnotification 0 5 }

dot11Reassociate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11ReassociateStation, dot11ReassociateID}

STATUS current

DESCRIPTION

"The reassociate notification is sent when the STA sends an

Reassociation Response frame with a status code equal to 'Suc-

cessful.' The value of the notification includes the MAC address

of the MAC to which the Reassociation Response frame was sent

and the Reassociation ID. ifIndex - Each 802.11 interface is

represented by an ifEntry. Interface tables in this MIB module

are indexed by ifIndex."

::= { dot11SMTnotification 0 6 }

dot11ReassociateFailed NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11ReassociateFailStatus,

dot11ReassociateStation }

STATUS current

DESCRIPTION

"The reassociate failed notification is sent when the STA sends

an Reassociation Response frame with a status code other than

'Successful.' The value of the notification includes the MAC

address of the MAC to which the Reassociation Response frame was

sent and the reason for the reassociation failure. ifIndex -

Each 802.11 interface is represented by an ifEntry. Interface

tables in this MIB module are indexed by ifIndex."

::= { dot11SMTnotification 0 7 }

-- **********************************************************************

-- *    End of dot11SMTnotification Objects

-- **********************************************************************

-- **********************************************************************

-- * dot11MultiDomainCapability TABLE

-- **********************************************************************

dot11MultiDomainCapabilityTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MultiDomainCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This (conceptual) table of attributes for cross-domain mobil-

ity."

::= { dot11smt 7 }

dot11MultiDomainCapabilityEntry OBJECT-TYPE

SYNTAX Dot11MultiDomainCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Multiple Domain Capability

Table.

IfIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB are indexed by ifIndex."

INDEX { ifIndex,

dot11MultiDomainCapabilityIndex }

::= { dot11MultiDomainCapabilityTable 1 }

Dot11MultiDomainCapabilityEntry ::=

SEQUENCE { dot11MultiDomainCapabilityIndex									Unsigned32,

dot11FirstChannelNumber									Unsigned32,

dot11NumberofChannels									Unsigned32,

dot11MaximumTransmitPowerLevel									Integer32 }

dot11MultiDomainCapabilityIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the Multi Domain Capability Table."

::= { dot11MultiDomainCapabilityEntry 1 }

dot11FirstChannelNumber OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the value of the lowest channel number

in the subband for the associated domain country string."

DEFVAL { 0 }

::= { dot11MultiDomainCapabilityEntry 2 }

dot11NumberofChannels OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the value of the total number of chan-

nels allowed in the subband for the associated domain country

string."

DEFVAL { 0 }

::= { dot11MultiDomainCapabilityEntry 3 }

dot11MaximumTransmitPowerLevel OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the maximum transmit power, in dBm,

allowed in the subband for the associated domain country

string."

DEFVAL { 0 }

::= { dot11MultiDomainCapabilityEntry 4 }

-- ********************************************************************

-- * End of dot11MultiDomainCapability TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11SpectrumManagement TABLE

-- ********************************************************************

dot11SpectrumManagementTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SpectrumManagementEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"(Conceptual) table of attributes for spectrum management"

::= {dot11smt 8}

dot11SpectrumManagementEntry OBJECT-TYPE

SYNTAX Dot11SpectrumManagementEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Spectrum Management Table.

IfIndex - Each 802.11 interface is represented by an ifEntry.

Interface tables in this MIB are indexed by ifIndex."

INDEX {ifIndex, dot11SpectrumManagementIndex}

::= { dot11SpectrumManagementTable 1 }

Dot11SpectrumManagementEntry ::=

SEQUENCE {

dot11SpectrumManagementIndex								Unsigned32,

dot11MitigationRequirement								Integer32,

dot11ChannelSwitchTime								Unsigned32,

dot11PowerCapabilityMaxImplemented								Integer32,

dot11PowerCapabilityMinImplemented								Integer32 }

dot11SpectrumManagementIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the Spectrum Management Table."

::= { dot11SpectrumManagementEntry 1 }

dot11MitigationRequirement OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the mitigation requirement in dB

required."

DEFVAL { 3 }

::= { dot11SpectrumManagementEntry 2 }

dot11ChannelSwitchTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates assumed channel switch time, measured

in TUs. The unit of this attribute is TUs. The minimum value of

this attribute is 1 TU."

DEFVAL { 2 }

::= { dot11SpectrumManagementEntry 3 }

dot11PowerCapabilityMaxImplemented OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute indicates the maximum transmit Power Capability

of this station. The unit of this attribute is dBm."

DEFVAL { 0 }

::= { dot11SpectrumManagementEntry 4 }

dot11PowerCapabilityMinImplemented OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute indicates the minimum transmit Power Capability

of this station. The unit of this attribute is dBm."

DEFVAL { -100 }

::= { dot11SpectrumManagementEntry 5 }

-- ******************************************************************

-- * End of dot11SpectrumManagement TABLE

-- ******************************************************************

-- ********************************************************************

-- * dot11RSNAConfig TABLE (RSNA and TSN)

-- ********************************************************************

dot11RSNAConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RSNAConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The table containing RSNA configuration objects."

::= { dot11smt 9 }

dot11RSNAConfigEntry OBJECT-TYPE

SYNTAX Dot11RSNAConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RSNAConfigTable."

INDEX { ifIndex }

::= { dot11RSNAConfigTable 1 }

Dot11RSNAConfigEntry ::=

SEQUENCE {

dot11RSNAConfigVersion				 			Unsigned32,

dot11RSNAConfigPairwiseKeysImplemented						 	Unsigned32,

dot11RSNAConfigGroupCipher				 			OCTET STRING,

dot11RSNAConfigGroupRekeyMethod					 		INTEGER,

dot11RSNAConfigGroupRekeyTime					 		Unsigned32,

dot11RSNAConfigGroupRekeyPackets					 		Unsigned32,

dot11RSNAConfigGroupRekeyStrict				 			TruthValue,

dot11RSNAConfigPSKValue				 			OCTET STRING,

dot11RSNAConfigPSKPassPhrase					 		DisplayString,

dot11RSNAConfigGroupUpdateCount					 		Unsigned32,

dot11RSNAConfigPairwiseUpdateCount						 	Unsigned32,

dot11RSNAConfigGroupCipherSize					 		Unsigned32,

dot11RSNAConfigPMKLifetime				 			Unsigned32,

dot11RSNAConfigPMKReauthThreshold					 		Unsigned32,

dot11RSNAConfigNumberOfPTKSAReplayCountersImplemented						 	Unsigned32,

dot11RSNAConfigSATimeout				 			Unsigned32,

dot11RSNAAuthenticationSuiteSelected						 	OCTET STRING,

dot11RSNAPairwiseCipherSelected				 			OCTET STRING,

dot11RSNAGroupCipherSelected					 		OCTET STRING,

dot11RSNAPMKIDUsed			 				OCTET STRING,

dot11RSNAAuthenticationSuiteRequested						 	OCTET STRING,

dot11RSNAPairwiseCipherRequested					 		OCTET STRING,

dot11RSNAGroupCipherRequested					 		OCTET STRING,

dot11RSNATKIPCounterMeasuresInvoked				 			Unsigned32,

dot11RSNA4WayHandshakeFailures				 			Unsigned32,

dot11RSNAConfigNumberOfGTKSAReplayCountersImplemented						 	Unsigned32,

dot11RSNAConfigSTKKeysImplemented						Unsigned32,

dot11RSNAConfigSTKCipher 							OCTET STRING,

dot11RSNAConfigSTKRekeyTime 							Unsigned32,

dot11RSNAConfigSMKUpdateCount 							Unsigned32,

dot11RSNAConfigSTKCipherSize 							Unsigned32,

dot11RSNAConfigSMKLifetime 							Unsigned32,

dot11RSNAConfigSMKReauthThreshold 						Unsigned32,

dot11RSNAConfigNumberOfSTKSAReplayCountersImplemented 							                          Unsigned32,

dot11RSNAPairwiseSTKSelected 							OCTET STRING,

dot11RSNASMKHandshakeFailures 							Unsigned32 }

-- dot11RSNAConfigEntry 1 has been deprecated.

dot11RSNAConfigVersion OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The highest RSNA version this entity supports. See 7.3.2.9

(Country (#1684)element)."

::= { dot11RSNAConfigEntry 2 }

dot11RSNAConfigPairwiseKeysImplemented OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This object indicates how many pairwise keys the entity supports

for RSNA."

::= { dot11RSNAConfigEntry 3 }

dot11RSNAConfigGroupCipher OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object indicates the group cipher suite selector the entity

must use. The group cipher suite in the RSN  element takes its

value from this variable. It consists of an OUI (the first 3

octets) and a cipher suite identifier (the last octet)."

::= { dot11RSNAConfigEntry 4 }

dot11RSNAConfigGroupRekeyMethod OBJECT-TYPE

SYNTAX INTEGER { disabled(1), timeBased(2), packetBased(3),

timepacketBased(4) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object selects a mechanism for rekeying the RSNA GTK. The

default is time-based, once per day. Rekeying the GTK is only

applicable to an entity acting in the Authenticator role (an AP

in an ESS)."

DEFVAL { timeBased }

::= { dot11RSNAConfigEntry 5 }

dot11RSNAConfigGroupRekeyTime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The time in seconds after which the RSNA GTK is refreshed. The

timer starts at the moment the GTK was set using the MLME-SET-

KEYS.request primitive."

DEFVAL { 86400 } -- once per day

::= { dot11RSNAConfigEntry 6 }

dot11RSNAConfigGroupRekeyPackets OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "1000 packets"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

A packet count (in 1000s of packets) after which the RSNA GTK is

refreshed. The packet counter starts at the moment the GTK was

set using the MLME-SETKEYS.request primitive and it counts all

packets encrypted using the current GTK."

::= { dot11RSNAConfigEntry 7 }

dot11RSNAConfigGroupRekeyStrict OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object signals that the GTK be refreshed whenever a STA

leaves the BSS that possesses the GTK."

::= { dot11RSNAConfigEntry 8 }

dot11RSNAConfigPSKValue OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The PSK for when RSNA in PSK mode is the selected AKM suite. In

that case, the PMK will obtain its value from this object.

This object is logically write-only. Reading this variable

returns unsuccessful status or null or zero."

::= { dot11RSNAConfigEntry 9 }

dot11RSNAConfigPSKPassPhrase OBJECT-TYPE

SYNTAX DisplayString

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The PSK, for when RSNA in PSK mode is the selected AKM suite, is

configured by dot11RSNAConfigPSKValue.

An alternative manner of setting the PSK uses the password-to-

key algorithm defined in H.4. This variable provides a means to

enter a pass-phrase. When this object is written, the RSNA

entity uses the password-to-key algorithm specified in H.4 to

derive a preshared and populate dot11RSNAConfigPSKValue with

this key.

This object is logically write-only. Reading this variable

returns unsuccessful status or null or zero."

::= { dot11RSNAConfigEntry 10 }

-- dot11RSNAConfigEntry 11 and dot11RSNAConfigEntry 12 have been

-- deprecated.

dot11RSNAConfigGroupUpdateCount OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The number of times Message 1 in the RSNA Group Key Handshake

will be retried per GTK Handshake attempt."

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 13 }

dot11RSNAConfigPairwiseUpdateCount OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The number of times Message 1 and Message 3 in the RSNA 4-Way

Handshake will be retried per 4-Way Handshake attempt."

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 14 }

dot11RSNAConfigGroupCipherSize OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object indicates the length in bits of the group cipher

key."

::= { dot11RSNAConfigEntry 15 }

dot11RSNAConfigPMKLifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS	read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The maximum lifetime of a PMK in the PMK cache."

DEFVAL { 43200 }

::= { dot11RSNAConfigEntry 16 }

dot11RSNAConfigPMKReauthThreshold OBJECT-TYPE

SYNTAX Unsigned32 (1..100)

UNITS "percentage"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The percentage of the PMK lifetime that should expire before an

IEEE 802.1X reauthentication occurs."

DEFVAL { 70 }

::= { dot11RSNAConfigEntry 17 }

dot11RSNAConfigNumberOfPTKSAReplayCountersImplemented OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Specifies the number of PTKSA replay counters per association:

0 -> 1 replay counter,

1 -> 2 replay counters,

2 -> 4 replay counters,

3 -> 16 replay counters"

::= { dot11RSNAConfigEntry 18 }

dot11RSNAConfigSATimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The maximum time a security association takes to set up."

DEFVAL { 60 }

::= { dot11RSNAConfigEntry 19 }

dot11RSNAAuthenticationSuiteSelected OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when a secu-

rity association is established.

The selector of the last AKM suite negotiated."

::= { dot11RSNAConfigEntry 20 }

dot11RSNAPairwiseCipherSelected OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when a secu-

rity association is established.

The selector of the last pairwise cipher negotiated."

::= { dot11RSNAConfigEntry 21 }

dot11RSNAGroupCipherSelected OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when a secu-

rity association is established.

The selector of the last group cipher negotiated."

::= { dot11RSNAConfigEntry 22 }

dot11RSNAPMKIDUsed OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(16))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when a secu-

rity association is established.

The selector of the last PMKID used in the last 4-Way Hand-

shake."

::= { dot11RSNAConfigEntry 23 }

dot11RSNAAuthenticationSuiteRequested OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when a secu-

rity association is established.

The selector of the last AKM suite requested."

::= { dot11RSNAConfigEntry 24 }

dot11RSNAPairwiseCipherRequested OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when a secu-

rity association is established.

The selector of the last pairwise cipher requested."

::= { dot11RSNAConfigEntry 25 }

dot11RSNAGroupCipherRequested OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when a secu-

rity association is established.

The selector of the last group cipher requested."

::= { dot11RSNAConfigEntry 26 }

dot11RSNATKIPCounterMeasuresInvoked OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

Counts the number of times that a TKIP MIC failure occurred two

times within 60 s and TKIP countermeasures were invoked. This

attribute counts both local and remote MIC failure events

reported to this STA. It increments every time TKIP countermea-

sures are invoked"

::= { dot11RSNAConfigEntry 27 }

dot11RSNA4WayHandshakeFailures OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when the con-

dition described below occurs.

Counts the number of 4-Way Handshake failures."

::= { dot11RSNAConfigEntry 28 }

dot11RSNAConfigNumberOfGTKSAReplayCountersImplemented OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Specifies the number of GTKSA replay counters per association:

0 -> 1 replay counter,

1 -> 2 replay counters,

2 -> 4 replay counters,

3 -> 16 replay counters"

::= { dot11RSNAConfigEntry 29 }

dot11RSNAConfigSTKKeysImplemented OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This object indicates how many STK keys the entity supports for

RSNA."

::= { dot11RSNAConfigEntry 30 }

dot11RSNAConfigSTKCipher OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object specifies the ciphersuite used by the STK for a DLS

link."

::= { dot11RSNAConfigEntry 31 }

dot11RSNAConfigSTKRekeyTime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The time in seconds after which an RSNA STK is refreshed. The

timer starts at the moment the STK was set using the MLMESET-

KEYS. request primitive."

DEFVAL { 86400 } -- once per day

::= { dot11RSNAConfigEntry 32 }

dot11RSNAConfigSMKUpdateCount OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The number of times Message 1 in the RSNA SMK Handshake will be

retried per SMK Handshake attempt."

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 33 }

dot11RSNAConfigSTKCipherSize OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object indicates the length in bits of the STK cipher key."

::= { dot11RSNAConfigEntry 34 }

dot11RSNAConfigSMKLifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The maximum lifetime of an SMK in the SMK cache."

DEFVAL { 43200 }

::= { dot11RSNAConfigEntry 35 }

dot11RSNAConfigSMKReauthThreshold OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the number of seconds for which an SMK

authentication is valid. A new SMK authentication must be com-

pleted successfully before the number of seconds indicated by

this attribute elapse, from the prior authentication, before the

STAs become unauthenticated."

::= { dot11RSNAConfigEntry 36 }

dot11RSNAConfigNumberOfSTKSAReplayCountersImplemented OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Specifies the number of STKSA replay counters per association:

0 -> 1 replay counter,

1 -> 2 replay counters,

2 -> 4 replay counters,

3 -> 16 replay counters"

::= { dot11RSNAConfigEntry 37 }

dot11RSNAPairwiseSTKSelected OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when a secu-

rity association is established.

The selector of the last STK cipher negotiated."

::= { dot11RSNAConfigEntry 38 }

dot11RSNASMKHandshakeFailures OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME when the con-

dition described below occurs.

Counts the number of SMK Handshake failures."

::= { dot11RSNAConfigEntry 39 }

-- ********************************************************************

-- * End of dot11RSNAConfig TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RSNAConfigPairwiseCiphers TABLE

-- ********************************************************************

dot11RSNAConfigPairwiseCiphersTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RSNAConfigPairwiseCiphersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table lists the pairwise ciphers supported by this entity.

It allows enabling and disabling of each pairwise cipher by net-

work management. The pairwise cipher suite list in the RSN ele-

ment is formed using the information in this table."

::= { dot11smt 10 }

dot11RSNAConfigPairwiseCiphersEntry OBJECT-TYPE

SYNTAX Dot11RSNAConfigPairwiseCiphersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The table entry, indexed by the interface index (or all inter-

faces) and the pairwise cipher."

INDEX { ifIndex, dot11RSNAConfigPairwiseCipherIndex }

::= { dot11RSNAConfigPairwiseCiphersTable 1 }

Dot11RSNAConfigPairwiseCiphersEntry ::=

SEQUENCE {

dot11RSNAConfigPairwiseCipherIndex						Unsigned32,

dot11RSNAConfigPairwiseCipherImplemented						OCTET STRING,

dot11RSNAConfigPairwiseCipherActivated						TruthValue,

dot11RSNAConfigPairwiseCipherSizeImplemented						Unsigned32 }

dot11RSNAConfigPairwiseCipherIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary index into the

dot11RSNAConfigPairwiseCiphersTable."

::= { dot11RSNAConfigPairwiseCiphersEntry 1 }

dot11RSNAConfigPairwiseCipherImplemented OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The selector of a supported pairwise cipher. It consists of an

OUI (the first 3 octets) and a cipher suite identifier (the last

octet)."

::= { dot11RSNAConfigPairwiseCiphersEntry 2 }

dot11RSNAConfigPairwiseCipherActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object enables or disables the pairwise cipher."

::= { dot11RSNAConfigPairwiseCiphersEntry 3 }

dot11RSNAConfigPairwiseCipherSizeImplemented OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This object indicates the length in bits of the pairwise cipher

key. This should be 256 for TKIP and 128 for CCMP."

::= { dot11RSNAConfigPairwiseCiphersEntry 4 }

-- ********************************************************************

-- * End of dot11RSNAConfigPairwiseCiphers TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RSNAConfigAuthenticationSuites TABLE

-- ********************************************************************

dot11RSNAConfigAuthenticationSuitesTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RSNAConfigAuthenticationSuitesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table lists the AKM suites supported by this entity. Each

AKM suite can be individually enabled and disabled. The AKM

suite list in the RSN element is formed using the information in

this table."

::= { dot11smt 11 }

dot11RSNAConfigAuthenticationSuitesEntry OBJECT-TYPE

SYNTAX Dot11RSNAConfigAuthenticationSuitesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (row) in the

dot11RSNAConfigAuthenticationSuitesTable."

INDEX { dot11RSNAConfigAuthenticationSuiteIndex }

::= { dot11RSNAConfigAuthenticationSuitesTable 1 }

Dot11RSNAConfigAuthenticationSuitesEntry ::=

SEQUENCE {

dot11RSNAConfigAuthenticationSuiteIndex							Unsigned32,

dot11RSNAConfigAuthenticationSuiteImplemented							OCTET STRING,

dot11RSNAConfigAuthenticationSuiteActivated							TruthValue }

dot11RSNAConfigAuthenticationSuiteIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used as an index into the

dot11RSNAConfigAuthenticationSuitesTable."

::= { dot11RSNAConfigAuthenticationSuitesEntry 1 }

dot11RSNAConfigAuthenticationSuiteImplemented OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The selector of an AKM suite. It consists of an OUI (the first 3

octets) and a cipher suite identifier (the last octet)."

::= { dot11RSNAConfigAuthenticationSuitesEntry 2 }

dot11RSNAConfigAuthenticationSuiteActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This variable indicates whether the corresponding AKM suite is

enabled/disabled."

::= { dot11RSNAConfigAuthenticationSuitesEntry 3 }

-- ********************************************************************

-- * End of dot11RSNAConfigAuthenticationSuites TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RSNAStats TABLE

-- ********************************************************************

dot11RSNAStatsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RSNAStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table maintains per-STA statistics in an RSN. The entry

with dot11RSNAStatsSTAAddress set to FF-FF-FF-FF-FF-FF contains

statistics for group addressed traffic."

::= { dot11smt 12 }

dot11RSNAStatsEntry OBJECT-TYPE

SYNTAX Dot11RSNAStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RSNAStatsTable."

INDEX { ifIndex, dot11RSNAStatsIndex }

::= { dot11RSNAStatsTable 1 }

Dot11RSNAStatsEntry ::=

SEQUENCE {

dot11RSNAStatsIndex						Unsigned32,

dot11RSNAStatsSTAAddress						MacAddress,

dot11RSNAStatsVersion						Unsigned32,

dot11RSNAStatsSelectedPairwiseCipher						OCTET STRING,

dot11RSNAStatsTKIPICVErrors						Counter32,

dot11RSNAStatsTKIPLocalMICFailures						Counter32,

dot11RSNAStatsTKIPRemoteMICFailures						Counter32,

dot11RSNAStatsCCMPReplays						Counter32,

dot11RSNAStatsCCMPDecryptErrors						Counter32,

dot11RSNAStatsTKIPReplays						Counter32,

dot11RSNAStatsCMACICVErrors 		 				Counter32,

dot11RSNAStatsCMACReplays 						Counter32,

dot11RSNAStatsRobustMgmtCCMPReplays 						Counter32,

dot11RSNABIPMICErrors 						Counter32  }

dot11RSNAStatsIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An auxiliary index into the dot11RSNAStatsTable."

::= { dot11RSNAStatsEntry 1 }

dot11RSNAStatsSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME.

The MAC address of the STA to which the statistics in this

conceptual row belong."

::= { dot11RSNAStatsEntry 2 }

dot11RSNAStatsVersion OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME.

The RSNA version with which the STA associated."

::= { dot11RSNAStatsEntry 3 }

dot11RSNAStatsSelectedPairwiseCipher OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the RSNA Key Management in the SME.

The pairwise cipher suite Selector (as defined in 7.3.2.25.1

(Cipher suites)) used during association, in transmission

order."

::= { dot11RSNAStatsEntry 4 }

dot11RSNAStatsTKIPICVErrors OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

Counts the number of TKIP ICV errors encountered when decrypting

packets for the STA."

::= { dot11RSNAStatsEntry 5 }

dot11RSNAStatsTKIPLocalMICFailures OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

Counts the number of MIC failures encountered when checking the

integrity of packets received from the STA at this entity."

::= { dot11RSNAStatsEntry 6 }

dot11RSNAStatsTKIPRemoteMICFailures OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

Counts the number of MIC failures encountered by the STA identi-

fied by dot11StatsSTAAddress and reported back to this entity."

::= { dot11RSNAStatsEntry 7 }

dot11RSNAStatsCCMPReplays OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

The number of received CCMP MPDUs discarded by the replay

mechanism."

::= { dot11RSNAStatsEntry 8 }

dot11RSNAStatsCCMPDecryptErrors OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

The number of received MPDUs discarded by the CCMP decryption

algorithm."

::= { dot11RSNAStatsEntry 9 }

dot11RSNAStatsTKIPReplays OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

Counts the number of TKIP replay errors detected."

::= { dot11RSNAStatsEntry 10 }

dot11RSNAStatsCMACICVErrors OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the the condition described below

occurs.

The number of received MPDUs discarded by the CMAC integrity

check algorithm."

::= { dot11RSNAStatsEntry 11 }

dot11RSNAStatsCMACReplays OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the the condition described below

occurs.

The number of received MPDUs discarded by the CMAC replay

errors."

::= { dot11RSNAStatsEntry 12 }

dot11RSNAStatsRobustMgmtCCMPReplays OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the the condition described below

occurs.

The number of received Robust Management frame MPDUs discarded

due to CCMP replay errors"

::= {dot11RSNAStatsEntry 13 }

dot11RSNABIPMICErrors OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the the condition described below

occurs.

The number of received MMPDUs discarded due to BIP MIC errors"

::= {dot11RSNAStatsEntry 14 }

-- ********************************************************************

-- * End of dot11RSNAStats TABLE

-- ********************************************************************

-- **********************************************************************

-- * dot11RegulatoryClasses TABLE

-- **********************************************************************

dot11RegulatoryClassesTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RegulatoryClassesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"(Conceptual) table of attributes for regulatory classes"

::= {dot11smt 13 }

dot11RegulatoryClassesEntry OBJECT-TYPE

SYNTAX Dot11RegulatoryClassesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Regulatory Classes Table.

IfIndex - Each 802.11 interface is represented by an ifEntry.

Interface tables in this MIB are indexed by ifIndex."

INDEX {ifIndex, dot11RegulatoryClassesIndex }

::= { dot11RegulatoryClassesTable 1 }

Dot11RegulatoryClassesEntry ::=

SEQUENCE {

dot11RegulatoryClassesIndex 						Unsigned32,

dot11RegulatoryClass 						Unsigned32,

dot11CoverageClass 						Unsigned32 }

dot11RegulatoryClassesIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the Regulatory Classes Table."

::= { dot11RegulatoryClassesEntry 1 }

dot11RegulatoryClass OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME or external management entity when the

device is initialized.

This attribute indicates the regulatory class to be used."

DEFVAL { 0 }

::= { dot11RegulatoryClassesEntry 2 }

dot11CoverageClass OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME or external management entity when the

device is initialized.

This attribute indicates the coverage class to be used."

DEFVAL { 0 }

::= { dot11RegulatoryClassesEntry 3 }

-- ********************************************************************

-- * End of dot11RegulatoryClasses TABLE

-- ********************************************************************

-- **********************************************************************

-- * IEEE 802.11 RM MIB

-- **********************************************************************

-- * The primary interface to the Radio Measurements is meant to be

-- * real-time information obtained through the request/response mechanisms of

-- * RM.  A secondary interface to the measurements is through retention of

-- * information in the MIB.  The information, meant to be retained for later

-- * access, includes these MIB entries. SNMP requests for

-- * information are obtained via SNMP SETs and GETs (see [B16a]).

-- ********************************************************************

-- * Radio Measurement

-- ********************************************************************

dot11RadioResourceMeasurement OBJECT IDENTIFIER ::= { dot11smt 14 }

-- **********************************************************************

-- *  dot11RMRequest and dot11RMReport Usage

-- *

-- * The dot11RMRequest and dot11RMReport portions of the RM MIB

-- * provide access to the Radio Measurement service. By performing

-- * SET operations on the various dot11RMRequest MIB objects,

-- * radio measurements may be initiated directly on the local STA or

-- * on any peer station within the same BSS. Subsequently, by

-- * performing GET operations on the various dot11RMReport MIB

-- * objects the results of the requested measurements may be

-- * retrieved.

-- *

-- * In the diagram below, a radio measurement could be initiated

-- * for STA x by performing a MIB.set operation on the RM MIB of

-- * STA x and specifying the MAC address of STA x in

-- * dot11RMRqstTargetAdd. Additionally, it is possible to have STA x

-- * request a measurement from STA y by performing a MIB.set operation

-- * on the SME MIB of STA x and specifying the MAC address of STA y in

-- * dot11RMRqstTargetAdd. In both cases the result of the measurements

-- * can be retrieved by performing a MIB.get operation on the RM MIB

-- * of STA x upon completion of the measurement.

-- *

-- *

-- *          MIB.Set                                        MIB.Set

-- *             or                                             or

-- *          MIB.Get                                        MIB.Get

-- *     _  _  _  _  _  _ _                               _  _  _  _  _  _ _

-- *    | SME    |         |                             | SME    |         |

-- *    |       \ /        |                             |       \ /        |

-- *    |   +---------+    |                             |   +---------+    |

-- *    |   | RM MIB  |    |                             |   | RM MIB  |    |

-- *    |   |         |    |                             |   |         |    |

-- *    |   |         |    |                             |   |         |    |

-- *    |   +---------+    |                             |   +---------+    |

-- *    |                  |                             |                  |

-- *    |   / \            |                             |   / \            |

-- *    |    |   MREQUEST  |                             |    |   MREQUEST  |

-- *    |_  _  _  _  _  _ _|                             |_  _  _  _  _  _ _|

-- *    |    |   MREPORT   |                             |    |   MREPORT   |

-- *    |   \ /  MEASURE   |     Action Frames           |   \ /  MEASURE   |

-- *    |                  | <== Measurement Request ==> |                  |

-- *    |                  | <== Measurement Report ==>  |                  |

-- *    | MLME             |                             | MLME             |

--      | _  _  _  _  _  _ |                             | _  _  _  _  _  _ |

-- *           STA x                                          STA y

-- *

-- * Each STA maintains a single dot11RMRequestTable in the SME MIB

-- * used to initiate RM Measurement Requests. Each dot11RMRequestEntry

-- * in the table represents an individual Measurement Request that

-- * makes up a complete Measurement Request Action frame.

-- * Multiple Measurement Requests may be concatenated into a single

-- * Measurement Request Action frame by setting the same

-- * dot11RMRqstToken value into multiple dot11RMRequestEntrys.

-- *

-- * Each row, dot11RMRequestEntry, of the dot11RMRequestTable

-- * provides read-create access for the initiation of a measurement

-- * request. The dot11RMRequestNextIndex object can be used to

-- * determine which is the next available row. Each row corresponding to

-- * one measurement in the sequence is created with a dot11RMRqstRowStatus

-- * set to notInService. Once the dot11RMRequestEntry(s) have been

-- * created for a desired measurement sequence the corresponding

-- * dot11RMRqstRowStatus(s) objects are set to active to indicate that

-- * the SME can trigger the appropriate MLME primitives. Upon processing

-- * the request, the SME returns the corresponding dot11RMRqstRowStatus(s)

-- * object to notInService and are now available for additional

-- * measurement requests.

-- *

-- * After a radio measurement is complete the RM populates the RMReport

-- * objects with the results of the measurement. Each STA maintains a set

-- * of RMReport tables, one corresponding to each measurement type. The

-- * results of the entire measurement sequence are spread across the tables

-- * based on the type of measurements requested. Each xxxReportEntry

-- * within a xxxReportTable contains a xxxRprtRqstToken that corresponds

-- * to the original dot11RMRqstToken in the measurement request. So the

-- * results of the measurement can be collected by searching the appropriate

-- * xxxReportTables and retrieve any reports with the matching request

-- * token.

-- **********************************************************************

-- ********************************************************************

-- * Radio Measurement Requests

-- ********************************************************************

dot11RMRequest OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 1 }

-- ********************************************************************

-- * dot11RMRequest TABLE

-- ********************************************************************

dot11RMRequestNextIndex OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when able to accept a new request.

Identifies a hint for the next value of dot11RMRqstIndex to be

used in a row creation attempt for dot11RMRequestTable. If no

new rows can be created for some reason, such as memory, pro-

cessing requirements, etc., the SME sets this attribute to 0. It

updates this attribute to a proper value other than 0 as soon as

it is capable of receiving new measurement requests. The nextIn-

dex is not necessarily sequential nor monotonically increasing."

::= { dot11RMRequest 1 }

dot11RMRequestTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This group contains the current list of requests for RM reports

to be issued and have been issued until removed. A network man-

ager adds a RM request by creating a row with createAndWait row

status and then filling in the request parameters/attributes.

The request becomes active to be issued when the row status is

set to Active. The columnar objects or attributes other than the

rowStatus are not written if the rowStatus is Active. The

request rows can be deleted, if commanded by a network manager

via changing the value of dot11RMRqstRowStatus to Destroy. This

may leave orphaned rows if a manager crashes and forgets which

rows are being used by it. One recommended way to manage

orphaned or finished rows is to delete rows if their

dot11RMRqstRowStatus remains other than Active for longer than a

period (recommend at least 5 minutes, RFC 2579). Or another rec-

ommended way is to delete older rows as needed based on their

dot11RMRqstTimeStamp values. This can be done by the agent as

well as the manager. "

::= { dot11RMRequest 2 }

dot11RMRequestEntry OBJECT-TYPE

SYNTAX Dot11RMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RMRequestTable Indexed by

dot11RMRqstIndex."

INDEX { dot11RMRqstIndex }

::= { dot11RMRequestTable 1 }

Dot11RMRequestEntry ::=

SEQUENCE {

dot11RMRqstIndex	Unsigned32,

dot11RMRqstRowStatus	RowStatus,

dot11RMRqstToken	OCTET STRING,

dot11RMRqstRepetitions	Unsigned32,

dot11RMRqstIfIndex	InterfaceIndex,

dot11RMRqstType	INTEGER,

dot11RMRqstTargetAdd	MacAddress,

dot11RMRqstTimeStamp	TimeTicks,

dot11RMRqstChanNumber	Unsigned32,

dot11RMRqstRegulatoryClass	Unsigned32,

dot11RMRqstRndInterval	Unsigned32,

dot11RMRqstDuration	Unsigned32,

dot11RMRqstParallel	TruthValue,

dot11RMRqstEnable	TruthValue,

dot11RMRqstRequest	TruthValue,

dot11RMRqstReport	TruthValue,

dot11RMRqstDurationMandatory	TruthValue,

dot11RMRqstBeaconRqstMode	INTEGER,

dot11RMRqstBeaconRqstDetail	INTEGER,

dot11RMRqstFrameRqstType	INTEGER,

dot11RMRqstBssid	MacAddress,

dot11RMRqstSSID	OCTET STRING,

dot11RMRqstBeaconReportingCondition	INTEGER,

dot11RMRqstBeaconThresholdOffset	Integer32,

dot11RMRqstSTAStatRqstGroupID	INTEGER,

dot11RMRqstLCIRqstSubject	INTEGER,

dot11RMRqstLCILatitudeResolution	Unsigned32,

dot11RMRqstLCILongitudeResolution	Unsigned32,

dot11RMRqstLCIAltitudeResolution	Unsigned32,

dot11RMRqstLCIAzimuthType	INTEGER,

dot11RMRqstLCIAzimuthResolution	Unsigned32,

dot11RMRqstPauseTime	Unsigned32,

dot11RMRqstTransmitStreamPeerQSTAAddress	MacAddress,

dot11RMRqstTransmitStreamTrafficIdentifier	Unsigned32,

dot11RMRqstTransmitStreamBin0Range	Unsigned32,

dot11RMRqstTrigdQoSAverageCondition	TruthValue,

dot11RMRqstTrigdQoSConsecutiveCondition	TruthValue,

dot11RMRqstTrigdQoSDelayCondition	TruthValue,

dot11RMRqstTrigdQoSAverageThreshold	Unsigned32,

dot11RMRqstTrigdQoSConsecutiveThreshold	Unsigned32,

dot11RMRqstTrigdQoSDelayThresholdRange	Unsigned32,

dot11RMRqstTrigdQoSDelayThreshold	Unsigned32,

dot11RMRqstTrigdQoSMeasurementCount	Unsigned32,

dot11RMRqstTrigdQoSTimeout	Unsigned32,

dot11RMRqstChannelLoadReportingCondition	INTEGER,

dot11RMRqstChannelLoadReference	Unsigned32,

dot11RMRqstNoiseHistogramReportingCondition	INTEGER,

dot11RMRqstAnpiReference	Unsigned32,

dot11RMRqstAPChannelReport	OCTET STRING,

dot11RMRqstSTAStatPeerSTAAddress	MacAddress,

dot11RMRqstFrameTransmitterAddress	MacAddress,

dot11RMRqstVendorSpecific	OCTET STRING }

dot11RMRqstIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for RM Request elements in dot11RMRequestTable, greater

than 0."

::= { dot11RMRequestEntry 1 }

dot11RMRqstRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

The Row Status column of the current row, used for tracking sta-

tus of an individual request. When this attribute is set to

Active, AND a measurement request can be unambiguously created

based on the parameters in the row, then the MLME may proceed to

issue the request to its intended targets when appropriate. If

not, this attribute may be set to Not-ready immediately to indi-

cate parametric errors. However, it is the network managers

responsibility to correct the error. If the request is success-

fully issued to the target STA, then the rowStatus is set to

notInService."

REFERENCE

"Subclause 7.3.2.21 (Measurement Request element)"

::= { dot11RMRequestEntry 2 }

dot11RMRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates a unique string to identify a group of

rows to be issued as parallel or sequential measurements. To

guarantee the uniqueness of this token across multiple network

managers, it is recommended that this token be prefixed with the

IP address of the network manager creating this row. This token

is not necessarily equivalent to the measurement tokens in RM

request frames. If this attribute is an empty string, then this

row of request is independent from other requests."

DEFVAL { "" }

::= { dot11RMRequestEntry 3 }

dot11RMRqstRepetitions OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the requested number of repetitions for

all the measurement request elements in this frame.  A value of

zero in the Number of Repetitions field indicates measurement

request elements are executed once without repetition."

::= { dot11RMRequestEntry 4 }

dot11RMRqstIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

The ifIndex on which this row of the RM Request is to be issued."

::= { dot11RMRequestEntry 5 }

dot11RMRqstType OBJECT-TYPE

SYNTAX INTEGER {

channelLoad(3),

noiseHistogram(4),

beacon(5),

frame(6),

staStatistics(7),

lci(8),

transmitStream(9),

pause(255)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the measurement type of this RM request

row."

::= { dot11RMRequestEntry 6 }

dot11RMRqstTargetAdd OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

The MAC address of STA for this row of RM Request is to be issued

to. If this attribute matches the MAC address of the

dot11RMRqstIfIndex, then measurement request is for this STA

itself to carry out."

::= { dot11RMRequestEntry 7 }

dot11RMRqstTimeStamp OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the sysUpTime Value the last time when

the dot11RMRqstRowStatus is set to active or when this row is

created the first time. This attribute is set by this STA or AP

automatically, not by an SNMP manager."

::= { dot11RMRequestEntry 8 }

dot11RMRqstChanNumber OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

The target STA channel number on which to perform the measure-

ments indicated in this request. The Channel Number is only

defined within the indicated Regulatory Class for this measure-

ment request. This attribute is ignored if dot11RMRqstType = STA

statistics Request, LCI Request, Transmit Stream/Category Mea-

surement, or Measurement Pause. However, even in that case, the

manager should set this attribute to the current channel for

this interface, so that the row can be set to active when ready

with all attributes indicated."

::= { dot11RMRequestEntry 9 }

dot11RMRqstRegulatoryClass OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the channel set for this measurement

request. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

request. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11RMRequestEntry 10 }

dot11RMRqstRndInterval OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the upper bound of the random delay to

be used prior to making the measurement, expressed in units of

TUs. See 11.10.2 (Measurement start time(11k)). This attribute

is ignored if dot11RMRqstType = STA statistics Request, LCI

Request, Transmit Stream/Category Measurement or Measurement

Pause."

DEFVAL { 0 }

::= { dot11RMRequestEntry 11 }

dot11RMRqstDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the preferred or mandatory measurement

duration for this Measurement Request. This attribute is ignored

if dot11RMRqstType = LCI Request or Measurement Pause."

DEFVAL { 0 }

::= { dot11RMRequestEntry 12 }

dot11RMRqstParallel OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the parallel bit for this Measurement

Request element. This attribute, when false, indicates that the

measurement is performed in sequence. This attribute, when true,

indicates that this measurement should start at the same time as

the measurement described by the next Measurement Request ele-

ment in the next row if the next row indicates the same value for

dot11RMRqstToken."

DEFVAL { false }

::= { dot11RMRequestEntry 13 }

dot11RMRqstEnable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the enable bit for this Measurement

Request element."

DEFVAL { false }

::= { dot11RMRequestEntry 14 }

dot11RMRqstRequest OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the request bit for this Measurement

Request element. This attribute, when true, indicates that this

STA accepts measurement requests from the target STA."

DEFVAL { false }

::= { dot11RMRequestEntry 15 }

dot11RMRqstReport OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the report bit for this Measurement

Request element. This attribute, when true, indicates that the

target STA may enable autonomous measurement reports to the

requesting STA."

DEFVAL { false }

::= { dot11RMRequestEntry 16 }

dot11RMRqstDurationMandatory OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the duration mandatory bit for this

Measurement Request element. This attribute, when true, indi-

cates that the indicated Measurement Duration is a mandatory

duration for this measurement. This attribute, when false, indi-

cates that the indicated Measurement Duration is a maximum dura-

tion for this measurement."

DEFVAL { false }

::= { dot11RMRequestEntry 17 }

dot11RMRqstBeaconRqstMode OBJECT-TYPE

SYNTAX INTEGER {

passive(0),

active(1),

beaconTable(2)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the Measurement Mode for this Beacon

Request element. This attribute is only valid if the

dot11RMRqstType is 5, indicating a beacon request, and is

ignored otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 18 }

dot11RMRqstBeaconRqstDetail OBJECT-TYPE

SYNTAX INTEGER {

noBody(0),

fixedFieldsAndRequestedElements(1),

allBody(2)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

dot11RMRqstBeaconRqstDetail indicates the Reporting Detail for

Beacon Request element. This attribute is only valid if the

dot11RMRqstType is 5, indicating a beacon request, and is

ignored otherwise."

DEFVAL { 2 }

::= { dot11RMRequestEntry 19 }

dot11RMRqstFrameRqstType OBJECT-TYPE

SYNTAX INTEGER {

frameCountRep(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

dot11RMRqstFrameRqstType indicates the Frame Request Type for

Frame Request element. This attribute is only valid if the

dot11RMRqstType is 6, indicating a frame request, and is ignored

otherwise."

DEFVAL { 1 }

::= { dot11RMRequestEntry 20 }

dot11RMRqstBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

BSSID indicates the BSSID of the particular AP for which this

measurement

is requested. The BSSID is set to the wildcard BSSID when the

measurement is to be performed on any AP(s) on the indicated

channel. This attribute is only valid if the dot11RMRqstType is

5, indicating a beacon request, and is ignored otherwise."

DEFVAL { 'FFFFFFFFFFFF'H }

::= { dot11RMRequestEntry 21 }

dot11RMRqstSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the SSID for the measurement. Zero

length MIB element for SSID indicates the wildcard SSID. The

SSID is set to the wildcard SSID when the measurement is to be

performed on all ESSs/IBSSs on the indicated channel. This

attribute is only valid if the dot11RMRqstType is 5, indicating

a beacon request, and is ignored otherwise."

DEFVAL { ''H }

::= { dot11RMRequestEntry 22 }

dot11RMRqstBeaconReportingCondition OBJECT-TYPE

SYNTAX INTEGER {

afterEveryMeasurement(0),

rcpiAboveAbsoluteThreshold(1),

rcpiBelowAbsoluteThreshold(2),

rsniAboveAbsoluteThreshold(3),

rsniBelowAbsoluteThreshold(4),

rcpiAboveOffsetThreshold(5),

rcpiBelowOffsetThreshold(6),

rsniAboveOffsetThreshold(7),

rsniBelowOffsetThreshold(8),

rcpiInBound(9),

rsniInBound(10)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates when the Beacon Measurement results are

to be reported to the requesting STA. This attribute is only

valid if the dot11RMRqstType is 5, indicating a beacon request,

and is ignored otherwise."

REFERENCE

"IEEE 802.11, Table 7-29g (Reporting Condition for Beacon

Report)-Reporting Condition values for Beacon Request element"

DEFVAL {0}

::= { dot11RMRequestEntry 23 }

dot11RMRqstBeaconThresholdOffset OBJECT-TYPE

SYNTAX Integer32

UNITS "0.5 dB"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

Threshold/Offset provides either the threshold value or the off-

set value to be used for conditional reporting. For indicated

Reporting Conditions 1-4, the integer range is (0..255). For

indicated Reporting Conditions 5-10, the integer range is  (-

127..+127). This attribute is only valid if the dot11RMRqstType

is 5, indicating a beacon request, and is ignored otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 24 }

dot11RMRqstSTAStatRqstGroupID OBJECT-TYPE

SYNTAX INTEGER {

dot11CountersTable(0),

dot11MacStatistics(1),

dot11QosCountersTableforUP0(2),

dot11QosCountersTableforUP1(3),

dot11QosCountersTableforUP2(4),

dot11QosCountersTableforUP3(5),

dot11QosCountersTableforUP4(6),

dot11QosCountersTableforUP5(7),

dot11QosCountersTableforUP6(8),

dot11QosCountersTableforUP7(9),

bSSAverageAccessDelays(10)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

The attribute indicates the group identity for this Measurement

Request element. This attribute is only valid if the

dot11RMRqstType is 7, indicating a statistics request, and is

ignored otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 25 }

dot11RMRqstLCIRqstSubject OBJECT-TYPE

SYNTAX INTEGER {

local(0),

remote(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

The attribute indicates the subject of the LCI measurement

request. This attribute is only valid if the dot11RMRqstType is

8, indicating an LCI request, and is ignored otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 26 }

dot11RMRqstLCILatitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute is 6 bits indicating the number of valid

bits in the fixed-point value of Latitude of the LCI measurement

request. This attribute is only valid if the dot11RMRqstType is

8, indicating an LCI request, and is ignored otherwise."

::= { dot11RMRequestEntry 27 }

dot11RMRqstLCILongitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute is 6 bits indicating the number of valid bits in

the fixed-point value of Longitude of the LCI measurement

request. This attribute is only valid if the dot11RMRqstType is

8, indicating an LCI request, and is ignored otherwise."

::= { dot11RMRequestEntry 28 }

dot11RMRqstLCIAltitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute is 6 bits indicating the number of valid

bits in the fixed-point value of Altitude of the LCI measurement

request. This attribute is only valid if the dot11RMRqstType is

8, indicating an LCI request, and is ignored otherwise."

::= { dot11RMRequestEntry 29 }

dot11RMRqstLCIAzimuthType OBJECT-TYPE

SYNTAX INTEGER {

frontSurfaceofSta(0),

radioBeam(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the azimuth reference for the LCI Azi-

muth measurement request. This attribute is only valid if the

dot11RMRqstType is 8, indicating an LCI request, and is ignored

otherwise."

DEFVAL{ 0 }

::= { dot11RMRequestEntry 30 }

dot11RMRqstLCIAzimuthResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..15)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute is 4 bits indicating the number of valid

bits in the fixed-point value of Azimuth of the LCI Azimuth

measurement request. This attribute is only valid if the

dot11RMRqstType is 8, indicating an LCI request, and is ignored

otherwise."

::= { dot11RMRequestEntry 31 }

dot11RMRqstPauseTime OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

UNITS "10 TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute is a 16 bit unsigned integer number

representing the time period for which measurements are

suspended or paused. Measurement Pause Requests are used to

provide time delays between the execution times of measurement

request elements in a Measurement Request Frame. This attribute

is only valid if the dot11RMRqstType is 255, indicating an pause

request, and is ignored otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 32 }

dot11RMRqstTransmitStreamPeerQSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the peer STA address to be measured for

a Transmit Stream/Category Measurement measurement. This

attribute is only valid if the dot11RMRqstType is 9, indicating

a transmit stream/category request, and is ignored otherwise."

::= { dot11RMRequestEntry 33 }

dot11RMRqstTransmitStreamTrafficIdentifier OBJECT-TYPE

SYNTAX Unsigned32 (0..16)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the TC, or TS to be measured for a

Transmit Stream/Category Measurement measurement. This attribute

is only valid if the dot11RMRqstType is 9, indicating a transmit

stream/category request, and is ignored otherwise."

::= { dot11RMRequestEntry 34 }

dot11RMRqstTransmitStreamBin0Range OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the delay range for bin 0 of the trans-

mit delay histogram. This attribute is only valid if the

dot11RMRqstType is 9, indicating a transmit stream/category

request, and is ignored otherwise."

::= { dot11RMRequestEntry 35 }

dot11RMRqstTrigdQoSAverageCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute, when true, indicates a request for triggered

reporting with trigger based on the number of discarded MSDUs

reaching the  dot11RMRqstTrigdQoSAverageThreshold when averaged

over dot11RMRqstTrigdQoSMEasurementCount consecutive MSDUs. This

attribute is only valid if the dot11RMRqstType is 9, indicating

a transmit stream/category request, and is ignored otherwise."

DEFVAL { false }

::= { dot11RMRequestEntry 36 }

dot11RMRqstTrigdQoSConsecutiveCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute, when true, indicates a request for triggered

reporting with trigger based on the consecutive number of MSDUs

discarded reaching dot11RMRqstTrigdQoSConsecutiveThreshold. This

attribute is only valid if the dot11RMRqstType is 9, indicating

a transmit stream/category request, and is ignored otherwise."

DEFVAL { false }

::= { dot11RMRequestEntry 37 }

dot11RMRqstTrigdQoSDelayCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute, when true, indicates a request for triggered

reporting with trigger based on the consecutive number of MSDUs

that experience a transmit delay greater than

dot11RMRqstTrigdQoSDelayThresholdRange reaching

dot11RMRqstTrigdQoSDelayThreshold. This attribute is only valid

if the dot11RMRqstType is 9, indicating a transmit stream/cate-

gory request, and is ignored otherwise."

DEFVAL { false }

::= { dot11RMRequestEntry 38 }

dot11RMRqstTrigdQoSAverageThreshold OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the trigger threshold for triggered

Transmit Stream/Category Measurement based on average MSDUs dis-

carded. Trigger occurs if the number of MSDUs discarded over the

moving average number of transmitted MSDUs in

dot11RMRqstTrigdQoSMeasurementCount reaches this threshold. This

attribute is only valid if the dot11RMRqstType is 9, indicating

a transmit stream/category request, and is ignored otherwise."

DEFVAL { 10 }

::= { dot11RMRequestEntry 39 }

dot11RMRqstTrigdQoSConsecutiveThreshold OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the trigger threshold for triggered

Transmit Stream/Category Measurement based on consecutive MSDUs

discarded. Trigger occurs if the consecutive number of MSDUs

discarded reaches this threshold. This attribute is only valid

if the dot11RMRqstType is 9, indicating a transmit stream/cate-

gory request, and is ignored otherwise."

DEFVAL { 5 }

::= { dot11RMRequestEntry 40 }

dot11RMRqstTrigdQoSDelayThresholdRange OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the minimum transmit delay for delayed

MSDU counts. Trigger occurs if the a consecutive number of MSDUs

experience a transmit delay greater than or equal to the lower

bound of the bin of the Transmit Delay Histogram given by the

value of this attribute + 2, e.g. if this attribute is 1 the

lower bound of bin 3. This attribute is only valid if the

dot11RMRqstType is 9, indicating a transmit stream/category

request, and is ignored otherwise."

DEFVAL { 1 }

::= { dot11RMRequestEntry 41 }

dot11RMRqstTrigdQoSDelayThreshold OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the number of consecutive delayed MSDUs

needed for trigger. Trigger occurs if the consecutive number of

MSDUs that experience a transmit delay greater than

dot11RMRqstQoSDelayThresholdRange reaches this value. This

attribute is only valid if the dot11RMRqstType is 9, indicating

a transmit stream/category request, and is ignored otherwise."

DEFVAL { 20 }

::= { dot11RMRequestEntry 42 }

dot11RMRqstTrigdQoSMeasurementCount OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the number of MSDUs to be used as a

moving average count in the average error threshold and in

determining the scope of the reported Transmit Stream/Category

measurement in a triggered measurement report. This attribute is

only valid if the dot11RMRqstType is 9, indicating a transmit

stream/category request, and is ignored otherwise."

DEFVAL { 100 }

::= { dot11RMRequestEntry 43 }

dot11RMRqstTrigdQoSTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

UNITS "100 TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the timeout interval during which a

measuring STA will not generate further triggered Transmit

Stream/Category measurement reports after a trigger condition

has been met and a report generated. This attribute is only

valid if the dot11RMRqstType is 9, indicating a transmit stream/

category request, and is ignored otherwise."

DEFVAL { 20 }

::= { dot11RMRequestEntry 44 }

dot11RMRqstChannelLoadReportingCondition OBJECT-TYPE

SYNTAX INTEGER {

afterEveryMeasurement(0),

chanLoadAboveReference(1),

chanLoadBelowReference(2)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates when the Channel Load Measurement

results are to be reported to the requesting STA. This attribute

is only valid if the dot11RMRqstType is 3, indicating a channel

load request, and is ignored otherwise."

REFERENCE

"IEEE 802.11, Table Table 7-29b (Reporting Condition for Channel

Load Report)-Reporting Condition values for Channel Load Request

element"

DEFVAL {0}

::= { dot11RMRequestEntry 45 }

dot11RMRqstChannelLoadReference OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

UNITS "1/255"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the channel load reporting condition

reference value. The measured Channel Load is compared to this

reference value and a report is issued if the reporting condi-

tion is satisfied. The reference value is in the same units as

Channel Load and represents the fractional time of the measure-

ment duration during which the STA determined the channel to be

busy. This attribute is only valid if the dot11RMRqstType is 3,

indicating a channel load request, and is ignored otherwise."

DEFVAL { 5 }

::= { dot11RMRequestEntry 46 }

dot11RMRqstNoiseHistogramReportingCondition OBJECT-TYPE

SYNTAX INTEGER {

afterEveryMeasurement(0),

aNPIAboveReference(1),

aNPIBelowReference(2)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates when the Noise Histogram Measurement

results are to be reported to the requesting STA. This attribute

is only valid if the dot11RMRqstType is 4, indicating a noise

histogram request, and is ignored otherwise."

REFERENCE

"IEEE 802.11, Table Table 7-29d (Reporting Condition for Noise

Histogram Report)-Reporting Condition for Noise Histogram

Report"

DEFVAL {0}

::= { dot11RMRequestEntry 47 }

dot11RMRqstAnpiReference OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

UNITS "0.5 dbm"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the noise histogram reporting condition

ANPI reference value. The measured ANPI is compared to this ref-

erence value and a report is issued if the indicated reporting

condition is satisfied.

ANPIval = Int[(ANPIpower in dBm + 110)*2], for ANPI in the range

-110 dBm to 0 dBm. ANPIval = 220 for ANPI > 0 dBm. ANPIval = 255

when ANPI is not available. This attribute is only valid if the

dot11RMRqstType is 4, indicating a noise histogram request, and

is ignored otherwise."

DEFVAL { 5 }

::= { dot11RMRequestEntry 48 }

dot11RMRqstAPChannelReport OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the specific channels to be used for

the requested beacon measurements. Zero length is the null

default for this attribute. Each octet indicates a different

channel within the indicated Regulatory Class. This list of

channels is the Channel List in the AP Channel Report element

described in 7.3.2.36 (AP Channel Report element(11k)). This

attribute is only valid if the dot11RMRqstType is 5, indicating

a beacon request, and is ignored otherwise."

DEFVAL { ''H }

::= { dot11RMRequestEntry 49 }

dot11RMRqstSTAStatPeerSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the peer STA address to be measured for

a statistics request. This attribute is only valid if the

dot11RMRqstType is 7, indicating a statistics request, and is

ignored otherwise."

::= { dot11RMRequestEntry 50 }

dot11RMRqstFrameTransmitterAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute indicates the Transmitter Address (TA) of the

frames to be counted in this frame request. This attribute is

only valid if the dot11RMRqstType is 6, indicating a frame

request, and is ignored otherwise."

::= { dot11RMRequestEntry 51 }

dot11RMRqstVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity when requesting a

measurement.

Changes take effect when dot11RMRqstRowStatus is set to Active.

This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement request

element. Zero length is the null default for this attribute.

This attribute is valid for all requests."

DEFVAL { ''H }

::= { dot11RMRequestEntry 52 }

-- **********************************************************************

-- * End of dot11RMRequest TABLE

-- **********************************************************************

-- ********************************************************************

-- * Radio Measurement Reports

-- * Report tables contain measurement reports received by this STA or

-- * results of measurements performed by this STA.

-- ********************************************************************

dot11RMReport OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 2 }

-- ********************************************************************

-- * dot11ChannelLoadReport TABLE

-- ********************************************************************

dot11ChannelLoadReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Channel Load reports

that have been received by the MLME. The report tables are main-

tained as a FIFO to preserve freshness, thus the rows in this

table can be deleted for memory constraints or other implementa-

tion constraints determined by the vendor. New rows have differ-

ent RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically

increase RprtIndex for new reports being written in the table."

::= { dot11RMReport 1 }

dot11ChannelLoadReportEntry OBJECT-TYPE

SYNTAX Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11ChannelLoadReportTable Indexed by

dot11ChannelLoadRprtIndex."

INDEX { dot11ChannelLoadRprtIndex }

::= { dot11ChannelLoadReportTable 1 }

Dot11ChannelLoadReportEntry ::=

SEQUENCE {

dot11ChannelLoadRprtIndex	Unsigned32,

dot11ChannelLoadRprtRqstToken	OCTET STRING,

dot11ChannelLoadRprtIfIndex	InterfaceIndex,

dot11ChannelLoadMeasuringSTAAddr	MacAddress,

dot11ChannelLoadRprtChanNumber	Unsigned32,

dot11ChannelLoadRprtRegulatoryClass	Unsigned32,

dot11ChannelLoadRprtActualStartTime	TSFType,

dot11ChannelLoadRprtMeasurementDuration	Unsigned32,

dot11ChannelLoadRprtChannelLoad	Unsigned32,

dot11ChannelLoadRprtVendorSpecific	OCTET STRING,

dot11ChannelLoadRprtMeasurementMode	INTEGER }

dot11ChannelLoadRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Channel Load Report elements in

dot11ChannelLoadReportTable, greater than 0."

::= { dot11ChannelLoadReportEntry 1 }

dot11ChannelLoadRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the request token that was indicated in

the Measurement request that generated this measurement report.

This should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be  multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11ChannelLoadReportEntry 2 }

dot11ChannelLoadRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The ifIndex of the interface over which this ChannelLoad Report

was received."

::= { dot11ChannelLoadReportEntry 3 }

dot11ChannelLoadMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The MAC address of the measuring STA for this row of the Channel

Load report."

::= { dot11ChannelLoadReportEntry 4 }

dot11ChannelLoadRprtChanNumber OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel number used for this Chan-

nel Load Report. The Channel Number is only defined within the

indicated Regulatory Class for this measurement report."

::= { dot11ChannelLoadReportEntry 5 }

dot11ChannelLoadRprtRegulatoryClass OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel set for this measurement

report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11ChannelLoadReportEntry 6 }

dot11ChannelLoadRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the TSF value at the time when the

measurement started."

::= { dot11ChannelLoadReportEntry 7 }

dot11ChannelLoadRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the duration over which the ChannelLoad

Report was measured."

::= { dot11ChannelLoadReportEntry 8 }

dot11ChannelLoadRprtChannelLoad OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

UNITS "1/255"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

Channel Load contains the fractional duration over which the

measuring STA determined the channel to be busy during the mea-

surement duration."

REFERENCE

"Subclause 7.3.2.22.4 (Channel Load Report(11k))"

::= { dot11ChannelLoadReportEntry 9 }

dot11ChannelLoadRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11ChannelLoadReportEntry 10 }

dot11ChannelLoadRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is

refusing to generate the report, 0 indicates the STA success-

fully carried out the measurement request."

DEFVAL { 0 }

::= { dot11ChannelLoadReportEntry 11 }

-- ********************************************************************

-- * End of dot11ChannelLoadReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11NoiseHistogramReport TABLE

-- ********************************************************************

dot11NoiseHistogramReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Noise Histogram reports

that have been received by the MLME. The report tables are main-

tained as a FIFO to preserve freshness, thus the rows in this

table can be deleted for memory constraints or other implementa-

tion constraints determined by the vendor. New rows have differ-

ent RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically

increase RprtIndex for new reports being written in the table."

::= { dot11RMReport 2 }

dot11NoiseHistogramReportEntry OBJECT-TYPE

SYNTAX Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11NoiseHistogramReportTable Indexed by

dot11NoiseHistogramRprtIndex."

INDEX { dot11NoiseHistogramRprtIndex }

::= { dot11NoiseHistogramReportTable 1 }

Dot11NoiseHistogramReportEntry ::=

SEQUENCE {

dot11NoiseHistogramRprtIndex	Unsigned32,

dot11NoiseHistogramRprtRqstToken	OCTET STRING,

dot11NoiseHistogramRprtIfIndex	InterfaceIndex,

dot11NoiseHistogramMeasuringSTAAddr	MacAddress,

dot11NoiseHistogramRprtChanNumber	Unsigned32,

dot11NoiseHistogramRprtRegulatoryClass	Unsigned32,

dot11NoiseHistogramRprtActualStartTime	TSFType,

dot11NoiseHistogramRprtMeasurementDuration	Unsigned32,

dot11NoiseHistogramRprtAntennaID	Unsigned32,

dot11NoiseHistogramRprtANPI	Unsigned32,

dot11NoiseHistogramRprtIPIDensity0	Integer32,

dot11NoiseHistogramRprtIPIDensity1	Integer32,

dot11NoiseHistogramRprtIPIDensity2	Integer32,

dot11NoiseHistogramRprtIPIDensity3	Integer32,

dot11NoiseHistogramRprtIPIDensity4	Integer32,

dot11NoiseHistogramRprtIPIDensity5	Integer32,

dot11NoiseHistogramRprtIPIDensity6	Integer32,

dot11NoiseHistogramRprtIPIDensity7	Integer32,

dot11NoiseHistogramRprtIPIDensity8	Integer32,

dot11NoiseHistogramRprtIPIDensity9	Integer32,

dot11NoiseHistogramRprtIPIDensity10	Integer32,

dot11NoiseHistogramRprtVendorSpecific	OCTET STRING,

dot11NoiseHistogramRprtMeasurementMode	INTEGER}

dot11NoiseHistogramRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Noise Histogram elements in

dot11NoiseHistogramReportTable, greater than 0."

::= { dot11NoiseHistogramReportEntry 1 }

dot11NoiseHistogramRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the request token that was indicated in

the measurement request that generated this measurement report.

This should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11NoiseHistogramReportEntry 2 }

dot11NoiseHistogramRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The ifIndex of the interface over which this Noise Histogram

Report was received. "

::= { dot11NoiseHistogramReportEntry 3 }

dot11NoiseHistogramMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The MAC address of the measuring STA for this row of Noise His-

togram report."

::= { dot11NoiseHistogramReportEntry 4 }

dot11NoiseHistogramRprtChanNumber OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel number used for this Noise

Histogram Report. The Channel Number is only defined within the

indicated Regulatory Class for this measurement report."

::= { dot11NoiseHistogramReportEntry 5 }

dot11NoiseHistogramRprtRegulatoryClass OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel set for this measurement

report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11NoiseHistogramReportEntry 6 }

dot11NoiseHistogramRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the TSF value at the time when the

measurement started."

::= { dot11NoiseHistogramReportEntry 7 }

dot11NoiseHistogramRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the duration over which the Noise His-

togram Report was measured."

::= { dot11NoiseHistogramReportEntry 8 }

dot11NoiseHistogramRprtAntennaID OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the identifying number for the antenna

used for this measurement. The value 0 indicates that the

antenna identifier is unknown. The value 255 indicates that the

measurement was made with multiple antennas or that the antenna

ID is unknown. The value 1 is used for a STA with only one

antenna.  STAs with more than one antenna assign Antenna IDs to

each antenna as consecutive, ascending numbers. Each  Antenna ID

number represents a unique antenna characterized by a fixed rel-

ative position, a fixed relative direction and a peak gain for

that position and direction."

::= { dot11NoiseHistogramReportEntry 9 }

dot11NoiseHistogramRprtANPI OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

UNITS "0.5 dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the Average Noise Power Indicator

(ANPI) for this Noise Histogram measurement. The ANPI value rep-

resents the average noise plus interference power on the mea-

sured channel at the antenna connector during the measurement

duration To calculate ANPI, the STA measures and uses IPI in the

indicated channel when NAV is equal to 0 (when virtual CS mecha-

nism indicates idle channel) except during frame transmission or

reception."

::= { dot11NoiseHistogramReportEntry 10 }

dot11NoiseHistogramRprtIPIDensity0 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: Power <= -

92dBm."

::= { dot11NoiseHistogramReportEntry 11 }

dot11NoiseHistogramRprtIPIDensity1 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -92dBm <

Power <= -89dBm."

::= { dot11NoiseHistogramReportEntry 12 }

dot11NoiseHistogramRprtIPIDensity2 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -89dBm <

Power <= -86dBm."

::= { dot11NoiseHistogramReportEntry 13 }

dot11NoiseHistogramRprtIPIDensity3 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -86dBm <

Power <= -83dBm."

::= { dot11NoiseHistogramReportEntry 14 }

dot11NoiseHistogramRprtIPIDensity4 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -83dBm <

Power <= -80dBm."

::= { dot11NoiseHistogramReportEntry 15 }

dot11NoiseHistogramRprtIPIDensity5 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -80dBm <

Power <= -75dBm."

::= { dot11NoiseHistogramReportEntry 16 }

dot11NoiseHistogramRprtIPIDensity6 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition:

-75dBm < Power <= -70dBm."

::= { dot11NoiseHistogramReportEntry 17 }

dot11NoiseHistogramRprtIPIDensity7 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition:

-70dBm < Power <= -65dBm."

::= { dot11NoiseHistogramReportEntry 18 }

dot11NoiseHistogramRprtIPIDensity8 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition:

-65dBm < Power <= -60dBm."

::= { dot11NoiseHistogramReportEntry 19 }

dot11NoiseHistogramRprtIPIDensity9 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition:

-60dBm < Power <= -55dBm."

::= { dot11NoiseHistogramReportEntry 20 }

dot11NoiseHistogramRprtIPIDensity10 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition:

-55dBm < Power."

::= { dot11NoiseHistogramReportEntry 21 }

dot11NoiseHistogramRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11NoiseHistogramReportEntry 22 }

dot11NoiseHistogramRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is

refusing to generate the report, 0 indicates the STA success-

fully carried out the measurement request."

DEFVAL { 0 }

::= { dot11NoiseHistogramReportEntry 23 }

-- ********************************************************************

-- * End of dot11NoiseHistogramReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11BeaconReport TABLE

-- ********************************************************************

dot11BeaconReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Beacon reports that

have been received by the MLME. The report tables are maintained

as FIFO to preserve freshness, thus the rows in this table can

be deleted for memory constraints or other implementation con-

straints determined by the vendor. New rows have different

RprtIndex values than those deleted within the range limitation

of the index. One easy way is to monotonically increase RprtIn-

dex for new reports being written in the table."

::= { dot11RMReport 3 }

dot11BeaconReportEntry OBJECT-TYPE

SYNTAX Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11BeaconReportTable Indexed by

dot11BeaconRprtIndex."

INDEX { dot11BeaconRprtIndex }

::= { dot11BeaconReportTable 1 }

Dot11BeaconReportEntry ::=

SEQUENCE {

dot11BeaconRprtIndex	Unsigned32,

dot11BeaconRprtRqstToken	OCTET STRING,

dot11BeaconRprtIfIndex	InterfaceIndex,

dot11BeaconMeasuringSTAAddr	MacAddress,

dot11BeaconRprtChanNumber	Unsigned32,

dot11BeaconRprtRegulatoryClass	Unsigned32,

dot11BeaconRprtActualStartTime	TSFType,

dot11BeaconRprtMeasurementDuration	Unsigned32,

dot11BeaconRprtPhyType	INTEGER,

dot11BeaconRprtReportedFrameType	INTEGER,

dot11BeaconRprtRCPI	Unsigned32,

dot11BeaconRprtRSNI	Unsigned32,

dot11BeaconRprtBSSID	MacAddress,

dot11BeaconRprtAntennaID	Unsigned32,

dot11BeaconRprtParentTSF	TSFType,

dot11BeaconRprtReportedFrameBody	OCTET STRING,

dot11BeaconRprtVendorSpecific	OCTET STRING,

dot11BeaconRprtMeasurementMode	INTEGER }

dot11BeaconRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Report elements in dot11BeaconReportTable,

greater than 0."

::= { dot11BeaconReportEntry 1 }

dot11BeaconRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the request token that was indicated in

the measurement request that generated this measurement report.

This should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11BeaconReportEntry 2 }

dot11BeaconRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The ifIndex of the interface over which this Beacon Report was

received."

::= { dot11BeaconReportEntry 3 }

dot11BeaconMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The MAC address of the measuring STA for this row of Beacon

report."

::= { dot11BeaconReportEntry 4 }

dot11BeaconRprtChanNumber OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel number used for this Beacon

Report. The Channel Number is only defined within the indicated

Regulatory Class for this measurement report."

::= { dot11BeaconReportEntry 5 }

dot11BeaconRprtRegulatoryClass OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel set for this measurement

report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11BeaconReportEntry 6 }

dot11BeaconRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the TSF value at the time when the mea-

surement started."

::= { dot11BeaconReportEntry 7 }

dot11BeaconRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the duration over which the Beacon

Report was measured."

::= { dot11BeaconReportEntry 8 }

dot11BeaconRprtPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

UNITS "dot11PHYType"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the PHY Type for this row of Beacon

Report."

::= { dot11BeaconReportEntry 9 }

dot11BeaconRprtReportedFrameType OBJECT-TYPE

SYNTAX INTEGER {

beaconOrProbeResponse(0),

measurementPilot(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the frame type reported in

dot11BeaconRprtReportedFrameBody"

::= { dot11BeaconReportEntry 10 }

dot11BeaconRprtRCPI OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

UNITS "0.5 dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the received channel power of the bea-

con or probe response frame in dBm, as defined in the RCPI mea-

surement subclause for the indicated PHY Type.

RCPIval = Int[(RCPIpower in dBm + 110)*2], for RCPI in the range

-110 dBm to 0 dBm. RCPIval = 220 for RCPI > 0 dBm. RCPIval = 255

when RCPI is not available."

::= { dot11BeaconReportEntry 11 }

dot11BeaconRprtRSNI OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

UNITS "0.5 dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the received signal to noise ratio of

the beacon or probe response frame in dB. RSNI is the received

signal to noise plus interference ratio derived from the mea-

sured RCPI for the received frame and from the measured ANPI for

the channel used to receive the frame. RSNI is calculated by the

ratio of  the received signal power (RCPI - ANPI) over the noise

plus interference power (ANPI) where

RSNI = [(ratio(dB) + 10) * 2], for ratios in the range -10dB to

+118dB."

::= { dot11BeaconReportEntry 12 }

dot11BeaconRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the BSSID of the beacon for this row of

Beacon Report."

::= { dot11BeaconReportEntry 13 }

dot11BeaconRprtAntennaID OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the identifying number for the antenna

used for this measurement. The value 0 indicates that the

antenna identifier is unknown.  The value 255 indicates that

this measurement was made with multiple antennas. The value 1 is

used for a STA with only one antenna.  STAs with more than one

antenna assign Antenna IDs to each antenna as consecutive,

ascending numbers. Each Antenna ID number represents a unique

antenna characterized by a fixed relative position, a fixed rel-

ative direction and a peak gain for that position and direc-

tion."

::= { dot11BeaconReportEntry 14 }

dot11BeaconRprtParentTSF OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the TSF value of the serving measuring

STA's TSF value at the time the measuring STA received the bea-

con or probe response frame."

::= { dot11BeaconReportEntry 15 }

dot11BeaconRprtReportedFrameBody OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..100))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the fixed fields and elements from the

frame body of the Beacon, Measurement Pilot or Probe Response

frame being received. All reported TIM elements are truncated to

4 octets."

::= { dot11BeaconReportEntry 16 }

dot11BeaconRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11BeaconReportEntry 17 }

dot11BeaconRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is

refusing to generate the report, 0 indicates the STA success-

fully carried out the measurement request."

DEFVAL { 0 }

::= { dot11BeaconReportEntry 18 }

-- ********************************************************************

-- * End of dot11BeaconReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FrameReport TABLE

-- ********************************************************************

dot11FrameReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Frame reports that have

been received by the MLME. The report tables are maintained as a

FIFO to preserve freshness, thus the rows in this table can be

deleted for memory constraints or other implementation con-

straints determined by the vendor. New rows have different

RprtIndex values than those deleted within the range limitation

of the index. One easy way is to monotonically increase RprtIn-

dex for new reports being written in the table."

::= { dot11RMReport 4 }

dot11FrameReportEntry OBJECT-TYPE

SYNTAX Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FrameReportTable Indexed by

dot11FrameRprtIndex."

INDEX { dot11FrameRprtIndex }

::= { dot11FrameReportTable 1 }

Dot11FrameReportEntry ::=

SEQUENCE {

dot11FrameRprtIndex	Unsigned32,

dot11FrameRprtIfIndex	InterfaceIndex,

dot11FrameRprtRqstToken	Unsigned32,

dot11FrameRprtChanNumber	Unsigned32,

dot11FrameRprtRegulatoryClass	Unsigned32,

dot11FrameRprtActualStartTime	TSFType,

dot11FrameRprtMeasurementDuration	Unsigned32,

dot11FrameRprtTransmitSTAAddress	MacAddress,

dot11FrameRprtBSSID	MacAddress,

dot11FrameRprtPhyType	INTEGER,

dot11FrameRprtAvgRCPI	Unsigned32,

dot11FrameRprtLastRSNI	Unsigned32,

dot11FrameRprtLastRCPI	Unsigned32,

dot11FrameRprtAntennaID	Unsigned32,

dot11FrameRprtNumberFrames	Unsigned32,

dot11FrameRprtVendorSpecific	OCTET STRING,

dot11FrameRptMeasurementMode	INTEGER }

dot11FrameRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Frame Report elements in dot11FrameReportTable,

greater than 0."

::= { dot11FrameReportEntry 1 }

dot11FrameRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The ifIndex of the interface over which this Frame Report was

received."

::= { dot11FrameReportEntry 2 }

dot11FrameRprtRqstToken OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

Index for Frame Request elements in dot11FrameRequestTable that

corresponds to this row of frame report. Since a single frame

request can generate multiple rows in the frame report table,

one per BSSID, this dot11FrameRprtRqstToken indicates which

request this particular row indicates. If this row of report is

received without a particular request, this attribute should be

0"

::= { dot11FrameReportEntry 3 }

dot11FrameRprtChanNumber OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel number used for this Frame

Report. The Channel Number is only defined within the indicated

Regulatory Class for this measurement report."

::= { dot11FrameReportEntry 4 }

dot11FrameRprtRegulatoryClass OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel set for this measurement

report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11FrameReportEntry 5 }

dot11FrameRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the TSF value at the time when measure-

ment started."

::= { dot11FrameReportEntry 6 }

dot11FrameRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the duration over which the Frame

Report was measured, expressed in units of TUs."

::= { dot11FrameReportEntry 7 }

dot11FrameRprtTransmitSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The MAC address of STA for this row of Frame report that it has

been received from."

::= { dot11FrameReportEntry 8 }

dot11FrameRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the BSSID of the STA that transmitted

this frame."

::= { dot11FrameReportEntry 9 }

dot11FrameRprtPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

UNITS "dot11PHYType"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the PHY used for frame reception in

this row of the frame report."

::= { dot11FrameReportEntry 10 }

dot11FrameRprtAvgRCPI OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

UNITS "0.5 dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the average value for the received

channel power of all the frames received and counted in this

Frame Report Entry, in dBm, as defined in the RCPI measurement

subclause for the indicated PHY Type.

RCPIval = Int[(RCPIpower in dBm + 110)*2], for RCPI in the range

-110 dBm to 0 dBm. RCPIval = 220 for RCPI > 0 dBm. RCPIval = 255

when RCPI is not available."

::= { dot11FrameReportEntry 11 }

dot11FrameRprtLastRSNI OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

UNITS "0.5 dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the received signal to noise ratio of

the received frame in dBm. RSNI is the received signal to noise

plus interference ratio derived from the RCPI for the received

frame and from the most recent ANPI value measured on the chan-

nel used to receive the frame. RSNI may be calculated by the

ratio of  the received signal power (RCPI - ANPI) over the noise

plus interference power (ANPI) where RSNI = [(ratio(dB) + 10) *

2],  for  ratios in the range -10dB to +118dB.  Other measurement

techniques are allowed."

::= { dot11FrameReportEntry 12 }

dot11FrameRprtLastRCPI OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the received channel power of the most

recently measured frame in this Frame Report entry, in dBm, as

defined in the RCPI measurement subclause for the indicated PHY

Type.

RCPIval = Int[(RCPIpower in dBm + 110)*2], for RCPI in the range

-110 dBm to 0 dBm. RCPIval = 220 for RCPI > 0 dBm. RCPIval = 255

when RCPI is not available."

::= { dot11FrameReportEntry 13 }

dot11FrameRprtAntennaID OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the identifying number for the antenna

used for this measurement. The value 0 indicates that the

antenna identifier is unknown.  The value 255 indicates that

this measurement was made with multiple antennas. The value 1 is

used for a STA with only one antenna.  STAs with more than one

antenna assign Antenna IDs to each antenna as consecutive,

ascending numbers. Each  Antenna ID number represents a unique

antenna characterized by a fixed relative position, a fixed rel-

ative direction and a peak gain for that position and direc-

tion."

::= { dot11FrameReportEntry 14 }

dot11FrameRprtNumberFrames  OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the number of received frames in the

measurement Report Frame for this row of the Frame Report."

::= { dot11FrameReportEntry 15 }

dot11FrameRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11FrameReportEntry 16 }

dot11FrameRptMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is

refusing to generate the report, 0 indicates the STA success-

fully carried out the measurement request."

DEFVAL { 0 }

::= { dot11FrameReportEntry 17 }

-- ********************************************************************

-- * End of dot11FrameReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11STAStatisticsReport TABLE

-- ********************************************************************

dot11STAStatisticsReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of STA Statistics reports

that have been received by the MLME. The report tables are main-

tained as a FIFO to preserve freshness, thus the rows in this

table can be deleted for memory constraints or other implementa-

tion constraints determined by the vendor. New rows have differ-

ent RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically

increase RprtIndex for new reports being written in the table."

::= { dot11RMReport 5 }

dot11STAStatisticsReportEntry OBJECT-TYPE

SYNTAX Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11STAStatisticsReportTable Indexed by

dot11STAStatisticsReportIndex."

INDEX { dot11STAStatisticsReportIndex }

::= { dot11STAStatisticsReportTable 1 }

Dot11STAStatisticsReportEntry ::=

SEQUENCE {

dot11STAStatisticsReportIndex	Unsigned32,

dot11STAStatisticsReportToken	OCTET STRING,

dot11STAStatisticsIfIndex	 InterfaceIndex,

dot11STAStatisticsSTAAddress	MacAddress,

dot11STAStatisticsMeasurementDuration	Unsigned32,

dot11STAStatisticsGroupID	INTEGER,

dot11STAStatisticsTransmittedFragmentCount	Counter32,

dot11STAStatisticsGroupTransmittedFrameCount	Counter32,

dot11STAStatisticsFailedCount	Counter32,

dot11STAStatisticsRetryCount	Counter32,

dot11STAStatisticsMultipleRetryCount	Counter32,

dot11STAStatisticsFrameDuplicateCount	Counter32,

dot11STAStatisticsRTSSuccessCount	Counter32,

dot11STAStatisticsRTSFailureCount	Counter32,

dot11STAStatisticsACKFailureCount	Counter32,

dot11STAStatisticsQosTransmittedFragmentCount	Counter32,

dot11STAStatisticsQosFailedCount	Counter32,

dot11STAStatisticsQosRetryCount	Counter32,

dot11STAStatisticsQosMultipleRetryCount	Counter32,

dot11STAStatisticsQosFrameDuplicateCount	Counter32,

dot11STAStatisticsQosRTSSuccessCount	Counter32,

dot11STAStatisticsQosRTSFailureCount	Counter32,

dot11STAStatisticsQosACKFailureCount	Counter32,

dot11STAStatisticsQosReceivedFragmentCount	Counter32,

dot11STAStatisticsQosTransmittedFrameCount	Counter32,

dot11STAStatisticsQosDiscardedFrameCount	Counter32,

dot11STAStatisticsQosMPDUsReceivedCount	Counter32,

dot11STAStatisticsQosRetriesReceivedCount	Counter32,

dot11STAStatisticsReceivedFragmentCount	Counter32,

dot11STAStatisticsGroupReceivedFrameCount	Counter32,

dot11STAStatisticsFCSErrorCount	Counter32,

dot11STAStatisticsTransmittedFrameCount	Counter32,

dot11STAStatisticsAPAverageAccessDelay	Unsigned32,

dot11STAStatisticsAverageAccessDelayBestEffort	Unsigned32,

dot11STAStatisticsAverageAccessDelayBackground	Unsigned32,

dot11STAStatisticsAverageAccessDelayVideo	Unsigned32,

dot11STAStatisticsAverageAccessDelayVoice	Unsigned32,

dot11STAStatisticsStationCount	Unsigned32,

dot11STAStatisticsChannelUtilization	Unsigned32,

dot11STAStatisticsVendorSpecific	OCTET STRING,

dot11STAStatisticsRprtMeasurementMode	INTEGER }

dot11STAStatisticsReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for STA Statistics Report elements in

dot11STAStatisticsReportTable, greater than 0."

::= { dot11STAStatisticsReportEntry 1 }

dot11STAStatisticsReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the token that was indicated in the

measurement request that generated this measurement report. This

should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11STAStatisticsReportEntry 2 }

dot11STAStatisticsIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

Identifies the Interface that this row of STA Statistics Report

has been received on"

::= { dot11STAStatisticsReportEntry 3 }

dot11STAStatisticsSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The MAC address of the STA that returned this STA Statistics

Report."

::= { dot11STAStatisticsReportEntry 4 }

dot11STAStatisticsMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the duration over which the STA Statis-

tics was measured.  A zero value for this attribute indicates

that the reported statistics are a current snapshot of the sta-

tistics variables.  A non-zero value for this attribute indi-

cates that the reported statistics contain the difference in the

corresponding statistics variables over the indicated duration."

::= { dot11STAStatisticsReportEntry 5 }

dot11STAStatisticsGroupID OBJECT-TYPE

SYNTAX INTEGER {

dot11CountersTable(0),

dot11MacStatistics(1),

dot11QosCountersTableforUP0(2),

dot11QosCountersTableforUP1(3),

dot11QosCountersTableforUP2(4),

dot11QosCountersTableforUP3(5),

dot11QosCountersTableforUP4(6),

dot11QosCountersTableforUP5(7),

dot11QosCountersTableforUP6(8),

dot11QosCountersTableforUP7(9),

bSSAverageAccessDelays(10)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the value of

dot11RMRqstSTAStatRqstGroupID returned from the STA in this STA

Statistics Report."

DEFVAL { 0 }

::= { dot11STAStatisticsReportEntry 6 }

dot11STAStatisticsTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11TransmittedFragmentCount returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 0, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 7 }

dot11STAStatisticsGroupTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11GroupTransmittedFrameCount returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 0, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 8 }

dot11STAStatisticsFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11FailedCount returned from the STA in

this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 0, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 9 }

dot11STAStatisticsRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11RetryCount returned from the STA in

this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 1, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 10 }

dot11STAStatisticsMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11MultipleRetryCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 1, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 11 }

dot11STAStatisticsFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11FrameDuplicateCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 1, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 12 }

dot11STAStatisticsRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11RTSSuccessCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 1, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 13 }

dot11STAStatisticsRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11RTSFailureCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 1, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 14 }

dot11STAStatisticsACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11ACKFailureCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 1, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 15 }

dot11STAStatisticsQosTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosTransmittedFragmentCount returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 16 }

dot11STAStatisticsQosFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosFailedCount returned from the STA

in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 17 }

dot11STAStatisticsQosRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosRetryCount returned from the STA

in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 18 }

dot11STAStatisticsQosMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosMultipleRetryCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 19 }

dot11STAStatisticsQosFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosFrameDuplicateCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 20 }

dot11STAStatisticsQosRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosRTSSuccessCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 21 }

dot11STAStatisticsQosRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosRTSFailureCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 22 }

dot11STAStatisticsQosACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosACKFailureCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 23 }

dot11STAStatisticsQosReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosReceivedFragmentCount returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 24 }

dot11STAStatisticsQosTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosTransmittedFrameCount returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 25 }

dot11STAStatisticsQosDiscardedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosDiscardedFrameCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 26 }

dot11STAStatisticsQosMPDUsReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosMPDUsReceivedCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 27 }

dot11STAStatisticsQosRetriesReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosRetriesReceivedCount returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 2-9, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 28 }

dot11STAStatisticsReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11ReceivedFragmentCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 0, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 29 }

dot11STAStatisticsGroupReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11GroupReceivedFrameCount returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 0, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 30 }

dot11STAStatisticsFCSErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11FCSErrorCount returned from the STA

in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 0, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 31 }

dot11STAStatisticsTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11TransmittedFrameCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 0, and is ignored

otherwise."

::= { dot11STAStatisticsReportEntry 32 }

dot11STAStatisticsAPAverageAccessDelay OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the AP Average Access Delay (AAD)

returned from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

access delay value over the indicated duration. This attribute

is only valid if the dot11STAStatisticsGroupID is 10, and is

ignored otherwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.39 (BSS Average Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 33 }

dot11STAStatisticsAverageAccessDelayBestEffort OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Average Access Delay (AAD) for the

Best Effort Access Category returned from the STA in this STA

Statistics Report. If dot11STAStatisticsMeasurementDuration

indicates a non-zero value, this attribute indicates the differ-

ence in the referenced access delay value over the indicated

duration. This attribute is only valid if the

dot11STAStatisticsGroupID is 10, and is ignored otherwise."

REFERENCE

"IEEE 802.11  Subclause 7.3.2.44 (BSS AC Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 34 }

dot11STAStatisticsAverageAccessDelayBackground OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Average Access Delay (AAD) for the

Background Access Category returned from the STA in this STA

Statistics Report. If dot11STAStatisticsMeasurementDuration

indicates a non-zero value, this attribute indicates the differ-

ence in the referenced access delay value over the indicated

duration. This attribute is only valid if the

dot11STAStatisticsGroupID is 10, and is ignored otherwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.44 (BSS AC Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 35 }

dot11STAStatisticsAverageAccessDelayVideo OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Average Access Delay (AAD) for the

Video Access Category returned from the STA in this STA Statis-

tics Report. If dot11STAStatisticsMeasurementDuration indicates

a non-zero value, this attribute indicates the difference in the

referenced access delay value over the indicated duration. This

attribute is only valid if the dot11STAStatisticsGroupID is 10,

and is ignored otherwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.44 (BSS AC Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 36 }

dot11STAStatisticsAverageAccessDelayVoice OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Average Access Delay (AAD) for the

Voice Access Category returned from the STA in this STA Statis-

tics Report. If dot11STAStatisticsMeasurementDuration indicates

a non-zero value, this attribute indicates the difference in the

referenced access delay value over the indicated duration. This

attribute is only valid if the dot11STAStatisticsGroupID is 10,

and is ignored otherwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.44 (BSS AC Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 37 }

dot11STAStatisticsStationCount OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11AssociatedStationCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the referenced

dot11 variable over the indicated duration. This attribute is

only valid if the dot11STAStatisticsGroupID is 10, and is

ignored otherwise."

::= { dot11STAStatisticsReportEntry 38 }

dot11STAStatisticsChannelUtilization OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

UNITS "1/255"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Channel Utilization returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero

value, this attribute indicates the difference in the Channel

Utilization value over the indicated duration. The Channel Uti-

lization is the time fraction during which the AP sensed the

channel busy. This attribute is only valid if the

dot11STAStatisticsGroupID is 10, and is ignored otherwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.28 (BSS Load element)"

::= { dot11STAStatisticsReportEntry 39 }

dot11STAStatisticsVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11STAStatisticsReportEntry 40 }

dot11STAStatisticsRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is

refusing to generate the report, 0 indicates the STA success-

fully carried out the measurement request."

DEFVAL { 0 }

::= { dot11STAStatisticsReportEntry 41 }

-- ********************************************************************

-- * End of dot11STAStatisticsReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11LCIReport TABLE

-- ********************************************************************

dot11LCIReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of LCI reports that have

been received by the MLME. The report tables are maintained as a

FIFO to preserve freshness, thus the rows in this table can be

deleted for memory constraints or other implementation con-

straints determined by the vendor. New rows have different

RprtIndex values than those deleted within the range limitation

of the index. One easy way is to monotonically increase RprtIn-

dex for new reports being written in the table."

::= { dot11RMReport 6 }

dot11LCIReportEntry OBJECT-TYPE

SYNTAX Dot11LCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11LCIReportTable Indexed by

dot11LCIReportIndex."

INDEX { dot11LCIReportIndex }

::= { dot11LCIReportTable 1 }

Dot11LCIReportEntry ::=

SEQUENCE {

dot11LCIReportIndex	Unsigned32,

dot11LCIReportToken	OCTET STRING,

dot11LCIIfIndex	InterfaceIndex,

dot11LCISTAAddress	MacAddress,

dot11LCILatitudeResolution	Unsigned32,

dot11LCILatitudeInteger	Integer32,

dot11LCILatitudeFraction	Integer32,

dot11LCILongitudeResolution	Unsigned32,

dot11LCILongitudeInteger	INTEGER,

dot11LCILongitudeFraction	INTEGER,

dot11LCIAltitudeType	INTEGER,

dot11LCIAltitudeResolution	Unsigned32,

dot11LCIAltitudeInteger	Integer32,

dot11LCIAltitudeFraction	Integer32,

dot11LCIDatum	Unsigned32,

dot11LCIAzimuthType	INTEGER,

dot11LCIAzimuthResolution	Unsigned32,

dot11LCIAzimuth	Integer32,

dot11LCIVendorSpecific	OCTET STRING,

dot11LCIRprtMeasurementMode	INTEGER}

dot11LCIReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for LCI Report elements in dot11LCIReportTable, greater

than 0."

::= { dot11LCIReportEntry 1 }

dot11LCIReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the token that was indicated in the

measurement request that generated this measurement report. This

should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11LCIReportEntry 2 }

dot11LCIIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

Identifies the Interface that this row of LCI Report has been

received on"

::= { dot11LCIReportEntry 3 }

dot11LCISTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The MAC address of the STA that returned this LCI Report."

::= { dot11LCIReportEntry 4 }

dot11LCILatitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the latitude resolution as 6 bits indi-

cating the number of valid bits in the fixed-point value of Lat-

itude. This field is derived from IETF RFC-3825, and is accessed

big-endian."

::= { dot11LCIReportEntry 5 }

dot11LCILatitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the latitude as a 34 bit fixed point

value consisting of 9 bits of integer and 25 bits of fraction.

This field contains the 9 bits of integer portion of Latitude.

This field is derived from IETF RFC-3825, and is accessed big-

endian."

::= { dot11LCIReportEntry 6 }

dot11LCILatitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the latitude as a 34 bit fixed point

value consisting of 9 bits of integer and 25 bits of fraction.

This field contains the 25 bits of fraction portion of Latitude.

This field is derived from IETF RFC-3825, and is accessed big-

endian."

::= { dot11LCIReportEntry 7 }

dot11LCILongitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the longitude resolution as 6 bits

indicating the number of valid bits in the fixed-point value of

Longitude. This field is derived from IETF RFC-3825, and is

accessed big-endian."

::= { dot11LCIReportEntry 8 }

dot11LCILongitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the longitude as a 34 bit fixed point

value consisting of 9 bits of integer and 25 bits of fraction.

This field contains the 9 bits of integer portion of Longitude.

This field is derived from IETF RFC-3825, and is accessed big-

endian."

::= { dot11LCIReportEntry 9 }

dot11LCILongitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the longitude as a 34 bit fixed point

value consisting of 9 bits of integer and 25 bits of fraction.

This field contains the 25 bits of fraction portion of Longi-

tude. This field is derived from IETF RFC-3825, and is accessed

big-endian."

::= { dot11LCIReportEntry 10 }

dot11LCIAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the altitude Type as four bits encoding

the type of altitude. Codes defined are:meters : in 2s-comple-

ment fixed-point 22-bit integer part with 8-bit fraction floors

: in 2s-complement fixed-point 22-bit integer part with 8-bit

fraction. This field is derived from IETF RFC-3825, and is

accessed big-endian."

::= { dot11LCIReportEntry 11 }

dot11LCIAltitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the altitude resolution as 6 bits indi-

cating the number of valid bits in the altitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIReportEntry 12 }

dot11LCIAltitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-2097151..2097151)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the altitude as a 30 bit value defined

by the Altitude type field. The field is encoded as a 2s-comple-

ment fixed-point 22-bit integer Part with 8-bit fraction.  This

field contains the fixed-point Part of Altitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIReportEntry 13 }

dot11LCIAltitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-127..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the altitude as a 30 bit value defined

by the Altitude type field. The field is encoded as a 2s-comple-

ment fixed-point 22-bit integer Part with 8-bit fraction.  This

field contains the fraction part of Altitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIReportEntry 14 }

dot11LCIDatum OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the datum as an eight-bit value encod-

ing the horizontal and vertical references used for the coordi-

nates given in this LCI."

::= { dot11LCIReportEntry 15 }

dot11LCIAzimuthType OBJECT-TYPE

SYNTAX INTEGER {

frontSurfaceOfSTA(0),

radioBeam(1) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the azimuth Type as a one bit attribute

encoding the type of Azimuth. Codes defined are: front surface

of STA : in 2s-complement fixed-point 9-bit integer; and radio

beam : in 2s-complement fixed-point 9-bit integer"

::= { dot11LCIReportEntry 16 }

dot11LCIAzimuthResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..15)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the azimuth Resolution as 4 bits indi-

cating the number of valid bits in the azimuth."

::= { dot11LCIReportEntry 17 }

dot11LCIAzimuth OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the azimuth as a 9 bit value defined by

the Azimuth Type field.The field is encoded as a 2s-complement

fixed-point 9-bit integer horizontal angle in degrees from True

North."

::= { dot11LCIReportEntry 18 }

dot11LCIVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11LCIReportEntry 19 }

dot11LCIRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is

refusing to generate the report, 0 indicates the STA success-

fully carried out the measurement request."

DEFVAL { 0 }

::= { dot11LCIReportEntry 20 }

-- ********************************************************************

-- * End of dot11LCIReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11TransmitStreamReport TABLE

-- ********************************************************************

dot11TransmitStreamReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11TransmitStreamReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Transmit Delay Metrics

reports that have been received by the MLME. The report tables

are maintained as a FIFO to preserve freshness, thus the rows in

this table can be deleted for memory constraints or other imple-

mentation constraints determined by the vendor. New rows have

different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically

increase RprtIndex for new reports being written in the table."

::= { dot11RMReport 7 }

dot11TransmitStreamReportEntry OBJECT-TYPE

SYNTAX Dot11TransmitStreamReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11TransmitStreamReportTable Indexed by

dot11TransmitStreamRprtIndex."

INDEX { dot11TransmitStreamRprtIndex }

::= { dot11TransmitStreamReportTable 1 }

Dot11TransmitStreamReportEntry ::=

SEQUENCE {

dot11TransmitStreamRprtIndex	Unsigned32,

dot11TransmitStreamRprtRqstToken	OCTET STRING,

dot11TransmitStreamRprtIfIndex	InterfaceIndex,

dot11TransmitStreamMeasuringSTAAddr	MacAddress,

dot11TransmitStreamRprtActualStartTime	TSFType,

dot11TransmitStreamRprtMeasurementDuration	Unsigned32,

dot11TransmitStreamRprtPeerSTAAddress	MacAddress,

dot11TransmitStreamRprtTID	Unsigned32,

dot11TransmitStreamRprtAverageQueueDelay	Unsigned32,

dot11TransmitStreamRprtAverageTransmitDelay	Unsigned32,

dot11TransmitStreamRprtTransmittedMSDUCount	Unsigned32,

dot11TransmitStreamRprtMSDUDiscardedCount	Unsigned32,

dot11TransmitStreamRprtMSDUFailedCount	Unsigned32,

dot11TransmitStreamRprtMultipleRetryCount	Unsigned32,

dot11TransmitStreamRprtCFPollsLostCount	Unsigned32,

dot11TransmitStreamRprtBin0Range	Unsigned32,

dot11TransmitStreamRprtDelayHistogram	OCTET STRING,

dot11TransmitStreamRprtReason	INTEGER,

dot11TransmitStreamRprtVendorSpecific	OCTET STRING,

dot11TransmitStreamRprtMeasurementMode	INTEGER}

dot11TransmitStreamRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Transmit Delay Metrics Report elements in

dot11TransmitStreamReportTable, greater than 0."

::= { dot11TransmitStreamReportEntry 1 }

dot11TransmitStreamRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the request token that was indicated in

the measurement request that generated this measurement report.

This should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11TransmitStreamReportEntry 2 }

dot11TransmitStreamRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The ifIndex of the interface on which this TransmitStream Report

was received."

::= { dot11TransmitStreamReportEntry 3 }

dot11TransmitStreamMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The MAC address of the measuring STA for this row of Transmit

Delay Metrics report."

::= { dot11TransmitStreamReportEntry 4 }

dot11TransmitStreamRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the TSF value at the time when the mea-

surement started or for a triggered Transmit Stream/Category

Measurement report the TSF value at the reporting QoS STA when

the trigger condition was met."

::= { dot11TransmitStreamReportEntry 5 }

dot11TransmitStreamRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the duration over which the Transmit

Delay Metrics Report was measured. For a triggered Transmit

Stream/Category Measurement Report, metrics are reported over a

number of transmitted MSDUs rather than a duration, hence Mea-

surement Duration is set to 0."

::= { dot11TransmitStreamReportEntry 6 }

dot11TransmitStreamRprtPeerSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The MAC address present in the Address 1 field of the measured

data frames for this row of Transmit Stream/Category Measurement

report."

::= { dot11TransmitStreamReportEntry 7 }

dot11TransmitStreamRprtTID OBJECT-TYPE

SYNTAX Unsigned32 (0..16)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the TC or TS for which traffic is to be

measured. Values 0 to 15 are defined. Values 16-255 are

reserved."

::= { dot11TransmitStreamReportEntry 8 }

dot11TransmitStreamRprtAverageQueueDelay OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the average delay of the frames (MSDUs)

that are passed to the MAC during the measurement duration for

the indicated destination and the indicated Traffic Identifier.

Queue Delay is measured from the time the MSDU is passed to the

MAC until the transmission starts and is expressed in units of

TUs."

::= { dot11TransmitStreamReportEntry 9 }

dot11TransmitStreamRprtAverageTransmitDelay OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the average delay of the frames (MSDUs)

that are successfully transmitted during the measurement dura-

tion for the indicated destination and the indicated Traffic

Identifier. Delay is measured from the time the MSDU is passed

to the MAC until ACK is received from the intermediate destina-

tion."

::= { dot11TransmitStreamReportEntry 10}

dot11TransmitStreamRprtTransmittedMSDUCount OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the number of MSDUs to the peer STA for

the TC, or TS given by the Traffic Identifier successfully

transmitted in the measurement duration"

::= {dot11TransmitStreamReportEntry 11}

dot11TransmitStreamRprtMSDUDiscardedCount OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the number of MSDUs to the peer STA for

the TC, or TS given by the Traffic Identifier discarded due

either to the number of transmit attempts exceeding

dot11ShortRetryLimit or dot11LongRetryLimit as appropriate, or

due to the MSDU lifetime having been reached"

::= {dot11TransmitStreamReportEntry 12}

dot11TransmitStreamRprtMSDUFailedCount OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the number of MSDUs to the peer STA for

the TC, or TS given by the Traffic Identifier discarded during

the measurement duration due to the number of transmit attempts

exceeding dot11ShortRetryLimit or dot11LongRetryLimit as appro-

priate."

::= {dot11TransmitStreamReportEntry 13}

dot11TransmitStreamRprtMultipleRetryCount OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the number of MSDUs for the TC, or TS

given by the Traffic Identifier that are successfully transmit-

ted after more than one retransmission attempt."

::= {dot11TransmitStreamReportEntry 14}

dot11TransmitStreamRprtCFPollsLostCount OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the number of QoS (+)CF-Poll frames

transmitted to the peer STA where there was no response from the

QoS STA."

::= {dot11TransmitStreamReportEntry 15}

dot11TransmitStreamRprtBin0Range OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the delay range for Bin 0 of the delay

histogram."

::= { dot11TransmitStreamReportEntry 16 }

dot11TransmitStreamRprtDelayHistogram OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (6))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the histogram of delay of the frames

(MSDUs) that are successfully transmitted during the measurement

duration for the indicated Traffic Identifier and the indicated

destination. Delay is measured from the time the MSDU is passed

to the MAC until the ACK is received from the intermediate des-

tination and is expressed in units of TUs. "

::= { dot11TransmitStreamReportEntry 17 }

dot11TransmitStreamRprtReason OBJECT-TYPE

SYNTAX INTEGER {

averageTrigger(0),

consecutiveTrigger(1),

delayTrigger(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the Reason field indicating the reason

that the measuring QoS STA sent the Transmit Stream/Category

measurement report."

DEFVAL { 0 }

::= { dot11TransmitStreamReportEntry 18 }

dot11TransmitStreamRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11TransmitStreamReportEntry 19 }

dot11TransmitStreamRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is

refusing to generate the report, 0 indicates the STA success-

fully carried out the measurement request."

DEFVAL { 0 }

::= { dot11TransmitStreamReportEntry 20 }

-- ********************************************************************

-- * End of dot11TransmitStreamReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * Radio Measurement Configuration Information

-- ********************************************************************

dot11RMConfig OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 3 }

-- ********************************************************************

-- * dot11APChannelReport TABLE

-- ********************************************************************

dot11APChannelReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"AP Channel Report information, in tabular form."

::= { dot11RMConfig 1 }

dot11APChannelReportEntry OBJECT-TYPE

SYNTAX Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11APChannelReportTable. Each entry in the

table is indexed by dot11APChannelReportIndex."

INDEX { dot11APChannelReportIndex }

::= { dot11APChannelReportTable 1 }

Dot11APChannelReportEntry ::=

SEQUENCE {

dot11APChannelReportIndex	Unsigned32,

dot11APChannelReportIfIndex	InterfaceIndex,

dot11APChannelReportRegulatoryClass	Unsigned32,

dot11APChannelReportChannelList	OCTET STRING}

dot11APChannelReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for AP channel report entry in dot11APChannelReportTable,

greater than 0."

::= { dot11APChannelReportEntry 1 }

dot11APChannelReportIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The ifIndex for this row of the AP channel report."

::= { dot11APChannelReportEntry 2 }

dot11APChannelReportRegulatoryClass OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel set for this AP Channel

Report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11APChannelReportEntry 3 }

dot11APChannelReportChannelList OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute lists the specific channels in this AP Channel

Report. Zero length is the null default for this attribute. Each

octet indicates a different channel within the indicated Regula-

tory Class. This list of channels is the Channel List in the AP

Channel Report element described in 7.3.2.36 (AP Channel Report

element(11k)). "

DEFVAL { ''H }

::= { dot11APChannelReportEntry 4 }

-- ********************************************************************

-- * End of dot11APChannelReportTable TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RMNeighborReport TABLE

-- ********************************************************************

dot11RMNeighborReportNextIndex OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Identifies the next available index for managing the Neighbor

Report table. If this attribute is 0, it indicates that the

Neighbor Report feature is not configurable via SNMP, or the

table is full and new rows cannot be accepted."

::= { dot11RMConfig 2 }

dot11RMNeighborReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RMNeighborReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains pertinent information on a collection of

BSSID's  that are candidates to which STA's  can roam. The rows

are created using createAndWait method and fill in the

attributes. When the rowStatus is set to active, the row can be

included in Neighbor Report IEs. If there is an error, the row-

Status is set to notReady by SME. Since this table contains all

Neighbor Report element entries for all interfaces enabled with

the Neighbor Report feature, it is possible to have too many

entries for one interface, while still remaining under the

MaxTableSize. In that situation, SME includes Neighbor Report

entries only with lower dot11RMNeighborReportIFIndex up to the

maximum possible number of entries for a particular interface

identified by ifIndex. SME sets the rowStatus to notInService

for those rows that cannot be included in the Neighbor Report

element for that interface."

::= { dot11RMConfig 3 }

dot11RMNeighborReportEntry OBJECT-TYPE

SYNTAX Dot11RMNeighborReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RMNeighborReportTable"

INDEX { dot11RMNeighborReportIndex }

::= { dot11RMNeighborReportTable 1 }

Dot11RMNeighborReportEntry ::=

SEQUENCE {

dot11RMNeighborReportIndex	Unsigned32,

dot11RMNeighborReportIfIndex	InterfaceIndex,

dot11RMNeighborReportBSSID	MacAddress,

dot11RMNeighborReportAPReachability	INTEGER,

dot11RMNeighborReportSecurity	TruthValue,

dot11RMNeighborReportCapSpectrumMgmt	TruthValue,

dot11RMNeighborReportCapQoS	TruthValue,

dot11RMNeighborReportCapAPSD	TruthValue,

dot11RMNeighborReportCapRM	TruthValue,

dot11RMNeighborReportCapDelayBlockAck	TruthValue,

dot11RMNeighborReportCapImmediateBlockAck	TruthValue,

dot11RMNeighborReportKeyScope	TruthValue,

dot11RMNeighborReportRegulatoryClass	Unsigned32,

dot11RMNeighborReportChannelNumber	Unsigned32,

dot11RMNeighborReportPhyType	INTEGER,

dot11RMNeighborReportNeighborTSFInfo	OCTET STRING,

dot11RMNeighborReportPilotInterval	Unsigned32,

dot11RMNeighborReportPilotMultipleBSSID 	OCTET STRING,

dot11RMNeighborReportRMEnabledCapabilities	OCTET STRING,

dot11RMNeighborReportVendorSpecific	OCTET STRING,

dot11RMNeighborReportRowStatus	RowStatus,

dot11RMNeighborReportMobilityDomain	TruthValue,

dot11RMNeighborReportCapHT	TruthValue,

dot11RMNeighborReportHTLDPCCodingCap	TruthValue,

dot11RMNeighborReportHTSupportedChannelWidthSet	TruthValue,

dot11RMNeighborReportHTSMPowerSave	Unsigned32,

dot11RMNeighborReportHTGreenfield	TruthValue,

dot11RMNeighborReportHTShortGIfor20MHz	TruthValue,

dot11RMNeighborReportHTShortGIfor40MHz	TruthValue,

dot11RMNeighborReportHTTxSTBC	TruthValue,

dot11RMNeighborReportHTRxSTBC	Unsigned32,

dot11RMNeighborReportHTDelayedBlockAck	TruthValue,

dot11RMNeighborReportHTMaxAMSDULength	TruthValue,

dot11RMNeighborReportHTDSSCCKModein40MHz	TruthValue,

dot11RMNeighborReportHTFortyMHzIntolerant	TruthValue,

dot11RMNeighborReportHTLSIGTXOPProtectionSupport	TruthValue,

dot11RMNeighborReportHTMaxAMPDULengthExponent	Unsigned32,

dot11RMNeighborReportHTMinMPDUStartSpacing	Unsigned32,

dot11RMNeighborReportHTRxMCSBitMask	Unsigned32,

dot11RMNeighborReportHTRxHighestSupportedDataRate	Unsigned32,

dot11RMNeighborReportHTTxMCSSetDefined	TruthValue,

dot11RMNeighborReportHTTxRxMCSSetNotEqual	TruthValue,

dot11RMNeighborReportHTTxMaxNumberSpatialStreamsSupported

Unsigned32,

dot11RMNeighborReportHTTxUnequalModulationSupported	TruthValue,

dot11RMNeighborReportHTPCO	TruthValue,

dot11RMNeighborReportHTPCOTransitionTime	Unsigned32,

dot11RMNeighborReportHTMCSFeedback	Unsigned32,

dot11RMNeighborReportHTCSupport	TruthValue,

dot11RMNeighborReportHTRDResponder	TruthValue,

dot11RMNeighborReportHTImplictTransmitBeamformingReceivingCap

TruthValue,

dot11RMNeighborReportHTReceiveStaggeredSoundingCap	TruthValue,

dot11RMNeighborReportHTTransmitStaggeredSoundingCap	TruthValue,

dot11RMNeighborReportHTReceiveNDPCap	TruthValue,

dot11RMNeighborReportHTTransmitNDPCap	TruthValue,

dot11RMNeighborReportHTImplicitTransmitBeamformingCap	TruthValue,

dot11RMNeighborReportHTTransmitBeamformingCalibration	Unsigned32,

dot11RMNeighborReportHTExplicitCSITransmitBeamformingCap

TruthValue,

dot11RMNeighborReportHTExplicitNonCompressedSteeringCap

TruthValue,

dot11RMNeighborReportHTExplicitCompressedSteeringCap	TruthValue,

dot11RMNeighborReportHTExplicitTransmitBeamformingFeedback

Unsigned32,

dot11RMNbRprtHTExplicitNonCompressedBeamformingFeedbackCap

Unsigned32,

dot11RMNeighborReportHTExplicitCompressedBeamformingFeedbackCap

Unsigned32,

dot11RMNeighborReportHTTransmitBeamformingMinimalGrouping

Unsigned32,

dot11RMNbRprtHTCSINumberofTxBeamformingAntennasSuppt

Unsigned32,

dot11RMNbRprtHTNonCompressedSteeringNumofTxBmfmingAntennasSuppt

Unsigned32,

dot11RMNbRprtHTCompressedSteeringNumberofTxBmfmingAntennasSuppt

Unsigned32,

dot11RMNbRprtHTCSIMaxNumberofRowsTxBeamformingSuppt

Unsigned32,

dot11RMNeighborReportHTTransmitBeamformingChannelEstimationCap

Unsigned32,

dot11RMNeighborReportHTAntSelectionCap	TruthValue,

dot11RMNeighborReportHTExplicitCSIFeedbackBasedTxASELCap

TruthValue,

dot11RMNeighborReportHTAntIndicesFeedbackBasedTxASELCap

TruthValue,

dot11RMNeighborReportHTExplicitCSIFeedbackBasedCap	TruthValue,

dot11RMNeighborReportHTAntIndicesFeedbackCap	TruthValue,

dot11RMNeighborReportHTRxASELCap	TruthValue,

dot11RMNeighborReportHTTxSoundingPPDUsCap	TruthValue,

dot11RMNeighborReportHTInfoPrimaryChannel	Unsigned32,

dot11RMNeighborReportHTInfoSecChannelOffset	Unsigned32,

dot11RMNeighborReportHTInfoSTAChannelWidth	TruthValue,

dot11RMNeighborReportHTInfoRIFSMode	TruthValue,

dot11RMNeighborReportHTInfoProtection	Unsigned32,

dot11RMNeighborReportHTInfoNonGreenfieldHTSTAsPresent	TruthValue,

dot11RMNeighborReportHTInfoOBSSNonHTSTAsPresent	TruthValue,

dot11RMNeighborReportHTInfoDualBeacon	TruthValue,

dot11RMNeighborReportHTInfoDualCTSProtection	TruthValue,

dot11RMNeighborReportHTInfoSTBCBeacon	TruthValue,

dot11RMNeighborReportHTInfoLSIGTXOPProtectionSup	TruthValue,

dot11RMNeighborReportHTInfoPCOActive	TruthValue,

dot11RMNeighborReportHTInfoPCOPhase	TruthValue,

dot11RMNeighborReportHTInfoBasicMCSSet	Unsigned32,

dot11RMNeighborReportHTSecChannelOffset	Unsigned32,

dot11RMNeighborReportExtCapPSMPSupport	TruthValue,

dot11RMNeighborReportExtCapSPSMPSup	TruthValue,

dot11RMNeighborReportExtCapServiceIntervalGranularity	Unsigned32

}

dot11RMNeighborReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Neighbor Report configuration table in

dot11RMNeighborReportTable, greater than 0."

::= { dot11RMNeighborReportEntry 1 }

dot11RMNeighborReportIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

The ifIndex for this row of the Neighbor Report."

::= { dot11RMNeighborReportEntry 2 }

dot11RMNeighborReportBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the BSSID of the AP described by this

row of Neighbor Report."

::= { dot11RMNeighborReportEntry 3 }

dot11RMNeighborReportAPReachability OBJECT-TYPE

SYNTAX INTEGER {

notReachable(1),

unknown(2),

reachable(3)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the reachability of the AP represented

by the dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 4 }

dot11RMNeighborReportSecurity OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute, when true, indicates that the neighbor AP iden-

tified by this BSSID supports the same security provisioning as

used by the AP which  provided this neighbor report. This

attribute, when false, indicates either that the neighbor AP

identified by this BSSID does not support the same security pro-

visioning or that the security information for this neighbor AP

is not available at this time."

::= { dot11RMNeighborReportEntry 5 }

dot11RMNeighborReportCapSpectrumMgmt OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the spectrum management capability of

the AP represented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 6 }

dot11RMNeighborReportCapQoS OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the QoS capability of the AP repre-

sented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 7 }

dot11RMNeighborReportCapAPSD OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the APSD capability of the AP repre-

sented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 8 }

dot11RMNeighborReportCapRM OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the RM capability of the AP represented

by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 9 }

dot11RMNeighborReportCapDelayBlockAck OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the Delayed BlockAck capability of the

AP represented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 10 }

dot11RMNeighborReportCapImmediateBlockAck OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the Immediate BlockAck capability of

the AP represented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 11 }

dot11RMNeighborReportKeyScope OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute, when true, indicates the neighbor AP identified

by this BSSID has the same authenticator as the AP which pro-

vided this neighbor report. This attribute, when false, indi-

cates that the neighbor AP identified by this BSSID has a

different authenticator or that authenticator information is not

available."

::= { dot11RMNeighborReportEntry 12 }

dot11RMNeighborReportRegulatoryClass OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the channel set for this Neighbor

Report entry. Country, Regulatory Class and Channel Number

together specify the channel frequency and spacing for this mea-

surement report. Valid values of Regulatory Class are shown in

Annex J."

REFERENCE

"Annex J"

::= { dot11RMNeighborReportEntry 13 }

dot11RMNeighborReportChannelNumber OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the current operating channel of the AP

represented by the dot11NeighborReportBSSID. The Channel Number

is only defined within the indicated Regulatory Class for this

Neighbor Report entry."

::= { dot11RMNeighborReportEntry 14 }

dot11RMNeighborReportPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

UNITS "dot11PHYType"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the PHY Type of the neighbor AP identi-

fied by this BSSID."

::= { dot11RMNeighborReportEntry 15 }

dot11RMNeighborReportNeighborTSFInfo OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (6))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates TSF timing information for the neighbor

AP identified by this BSSID. The TSF timing information includes

the TSF Offset and the Beacon Interval, as defined in subclause

7.3.2.37 (Neighbor Report element(11k))."

::= { dot11RMNeighborReportEntry 16 }

dot11RMNeighborReportPilotInterval OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates Measurement Pilot Interval for the

neighbor AP identified by this BSSID, as defined in subclause

7.3.1.18 (Measurement Pilot Interval field(11k))."

::= { dot11RMNeighborReportEntry 17 }

dot11RMNeighborReportPilotMultipleBSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1))

UNITS "BSSID LSBs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates n, where 2n is the maximum number of

BSSIDs in the multiple BSSID set, as described in subclause

11.10.11 (Multiple BSSID Set(11k))."

::= { dot11RMNeighborReportEntry 18 }

dot11RMNeighborReportRMEnabledCapabilities OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(7))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute indicates the detailed enabled capabilities of

the AP represented by the dot11NeighborReportBSSID, as defined

in subclause 7.3.2.45 ((#1704)RM Enabled Capabilities ele-

ment(11k)(#1720))."

REFERENCE

"IEEE 802.11 -Subclause 7.3.2.45 ((#1704)RM Enabled Capabili-

ties element(11k)(#1720))"

::= { dot11RMNeighborReportEntry 19 }

dot11RMNeighborReportVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11RMNeighborReportEntry 20 }

dot11RMNeighborReportRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

Contains the row status of the Neighbor Report, essentially used

for indicating whether the row has all valid attributes filled

in. Then set to active to be used in Neighbor Report elements.

If any parameter is invalid, the SME sets this attribute back to

notReady. It is the responsibility of the manager to correct the

parameters."

::= { dot11RMNeighborReportEntry 21 }

dot11RMNeighborReportMobilityDomain OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when a measurement report is completed.

Indicates a common mobility domain identifier (MDID) and an

identical value of the FT Capability and Policy value."

::= { dot11RMNeighborReportEntry 22 }

dot11RMNeighborReportCapHT OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The High Throughput Bit when set to 1 indicates that the AP

represented by this BSSID is an HT AP including the HT Capabili-

ties element in its Beacons and that the contents of that HT

Capabilities element are identical to the HT Capabilities ele-

ment advertised by the AP sending the report. See 7.3.2.37

(Neighbor Report element(11k))"

::= { dot11RMNeighborReportEntry 23 }

dot11RMNeighborReportHTLDPCCodingCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT LDPC coding capability indicates support for receiving

LDPC coded packets, set to false if not supported, set to true

if supported. See 7.3.2.56.2 (HT Capabilities Info field)"

::= { dot11RMNeighborReportEntry 24 }

dot11RMNeighborReportHTSupportedChannelWidthSet OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Supported channel width set indicates which channel

widths the STA supports, Set to false if only 20 MHz operation

is supported, Set to true if both 20 MHz and 40 MHz operation is

supported. See 7.3.2.56.2 (HT Capabilities Info field)"

::= { dot11RMNeighborReportEntry 25 }

dot11RMNeighborReportHTSMPowerSave OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT SM Power Save indicates the SM power save mode, set to 0

for static SM power save mode, set to 1 for dynamic SM power save

mode, set to 3 for SM power save disabled, the value 2 is

reserved, See 7.3.2.56.2 (HT Capabilities Info field)"

::= { dot11RMNeighborReportEntry 26 }

dot11RMNeighborReportHTGreenfield OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT-greenfield indicates support for the reception of PPDUs

with HT-greenfield format, set to false if not supported, set to

true if supported. See 7.3.2.56.2 (HT Capabilities Info field)"

::= { dot11RMNeighborReportEntry 27 }

dot11RMNeighborReportHTShortGIfor20MHz OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Short GI for 20 MHZ indicates short GI support for the

reception of 20 MHz packets, set to false if not supported, set

to true if supported See 7.3.2.56.2 (HT Capabilities Info

field)"

::= { dot11RMNeighborReportEntry 28 }

dot11RMNeighborReportHTShortGIfor40MHz OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Short GI for 40 MHz indicates short GI support for the

reception of 40 MHz packets, set to false if not supported, set

to true if supported. See 7.3.2.56.2 (HT Capabilities Info

field)"

::= { dot11RMNeighborReportEntry 29}

dot11RMNeighborReportHTTxSTBC OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Tx STBC indicates support for the transmission of PPDUs

using STBC, set to false if not supported, set to true if sup-

ported. See 7.3.2.56.2 (HT Capabilities Info field)"

::= { dot11RMNeighborReportEntry 30}

dot11RMNeighborReportHTRxSTBC OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Rx STBC indicates support for the reception of PPDUs

using STBC, set to 0 for no support, set to 1 for support of one

spatial stream, set to 2 for support of one and two spatial

streams, set to 3 for support of one, two, and three spatial

streams. See 7.3.2.56.2 (HT Capabilities Info field)"

::= { dot11RMNeighborReportEntry 31}

dot11RMNeighborReportHTDelayedBlockAck OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT-delayed Block ACK indicates support for HT-delayed Block

ACK operation, set to false if not supported, set to true if

supported. Support indicates that the STA is able to accept an

ADDBA request for HT-delayed Block ACK. See 7.3.2.56.2 (HT Capa-

bilities Info field)"

::= { dot11RMNeighborReportEntry 32 }

dot11RMNeighborReportHTMaxAMSDULength OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Maximum A-MSDU length indicates maximum A-MSDU length,

set to false for 3839 octets, set to true for 7935 octets. See

7.3.2.56.2 (HT Capabilities Info field)"

::= { dot11RMNeighborReportEntry 33 }

dot11RMNeighborReportHTDSSCCKModein40MHz OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT DSSS/CCK Mode in 40 MHz indicates use of DSSS/CCK mode

in a 40 MHz capable BSS operating in 20/40 MHz mode, set to false

if DSSS/CCK in 40 MHz is not allowed, set to true if the DSSS/CCK

in 40 MHz is allowed. See 7.3.2.56.2 (HT Capabilities Info

field)"

::= { dot11RMNeighborReportEntry 34 }

dot11RMNeighborReportHTFortyMHzIntolerant OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Forty MHz Intolerant indicates whether other BSSs

receiving this information are required to prohibit 40 MHz

transmissions, set to true to prohibit 20/40 MHz BSS operation,

otherwise set to false. See 7.3.2.56.2 (HT Capabilities Info

field)"

::= { dot11RMNeighborReportEntry 35 }

dot11RMNeighborReportHTLSIGTXOPProtectionSupport OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT L-SIG TXOP protection support indicates support for the

LSIG TXOP protection mechanism, set to false if not supported,

set to true if supported. See 7.3.2.56.2 (HT Capabilities Info

field)"

::= { dot11RMNeighborReportEntry 36 }

dot11RMNeighborReportHTMaxAMPDULengthExponent OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Maximum A-MPDU Length Exponent indicates the maximum

length of A-MPDU that the STA can receive. This field is an

integer in the range 0 to 3. The length defined by this field is

equal to 2(13 + Maximum A-MPDU Length) - 1 octets. See

7.3.2.56.3 (A-MPDU Parameters field)"

::= { dot11RMNeighborReportEntry 37 }

dot11RMNeighborReportHTMinMPDUStartSpacing OBJECT-TYPE

SYNTAX Unsigned32 (0..7)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Minimum MPDU Start Spacing determines the minimum time

between the start of adjacent MPDUs within an AMPDU, measured at

the PHY-SAP, set to 0 for no restriction, set to 1 for 1/4 microsecond,

set to 2 for 1/2 microsecond, set to 3 for 1 microsecond, set to 4 for 2 microsecond, set to

5 for 4 microsecond, set to 6 for 8 microsecond, set to 7 for 16 microsecond. See 7.3.2.56.3

(A-MPDU Parameters field)"

::= { dot11RMNeighborReportEntry 38 }

dot11RMNeighborReportHTRxMCSBitMask OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Rx MCS Bitmask is a 77 bit subfield that defines a set of

MCS values, where bit B0 corresponds to MCS 0 and bit B76 corre-

sponds to MCS 76, set to 0 when the MSC is not supported, set to

1 when the MSC is supported. See 7.3.2.56.4 (Supported MCS Set

field)"

::= { dot11RMNeighborReportEntry 39}

dot11RMNeighborReportHTRxHighestSupportedDataRate OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Highest Supported Data Rate is a 10 bit subfield that

defines the highest data rate that the STA is able to receive,

in units of 1 Mb/s, where 1 represents 1 Mb/s, and incrementing

by 1 Mb/s steps to the value 1023, which represents 1023 Mb/s.

See 7.3.2.56.4 (Supported MCS Set field)"

::= { dot11RMNeighborReportEntry 40 }

dot11RMNeighborReportHTTxMCSSetDefined OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Tx MCS Set Defined indicates if the Tx MCS set is

defined, set to false if no Tx MCS set is defined, set to true if

Tx MSC set is defined. See 7.3.2.56.4 (Supported MCS Set field)"

::= { dot11RMNeighborReportEntry 41 }

dot11RMNeighborReportHTTxRxMCSSetNotEqual OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Tx RX MCS set not equal indicates if the Tx MCS set is

defined to be equal to the Rx MCS set, set to false where no Tx

MCS set is defined or where the Tx MCS Set is defined to be equal

to the RX MCS Set, set to true where the TX MCS set may differ

from the Rx MCS set. See 7.3.2.56.4 (Supported MCS Set field)"

::= { dot11RMNeighborReportEntry 42 }

dot11RMNeighborReportHTTxMaxNumberSpatialStreamsSupported OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Tx maximum number spatial streams supported indicates

maximum number of spatial streams supported when the Tx MCS Set

may differ from the Rx MCS set, set to 0 where no TX MCS set is

defined or where the Tx MCS set is defined to be equal to the RX

MCS set or where the maximum number of spatial streams supported

when transmitting is 1 spatial stream and the Tx MCS set may

differ from the Rx MCS set, set to 1 where the maximum number of

spatial streams supported when transmitting is 2 spatial streams

and the Tx MCS set may differ from the Rx MCS set, set to 2 where

the maximum number of spatial streams supported when transmit-

ting is 3 spatial streams and the Tx MCS set may differ from the

Rx MCS set, set to 3 where the maximum number of spatial streams

supported when transmitting is 4 spatial streams and the Tx MCS

set may differ from the Rx MCS set. See 7.3.2.56.4 (Supported

MCS Set field)"

::= { dot11RMNeighborReportEntry 43 }

dot11RMNeighborReportHTTxUnequalModulationSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

" The HT Tx UEQM supported indicates whether transmit UEQM is supported

when the Tx MCS set may differ from the Rx MCS set, set to false where no

TX MCS set is defined or where the Tx MCS set is defined to be equal to the

RX MCS set or when UEQM is not supported and the Tx MCS set may differ from

the Rx MCS set, set to true when UEQM is supported and the Tx MCS set may

differ from the Rx MCS set. See 7.3.2.56.4 (Supported MCS Set field)"

::= { dot11RMNeighborReportEntry 44 }

dot11RMNeighborReportHTPCO OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT PCO indicates support for PCO, set to false if not sup-

ported, set to true if supported. See 7.3.2.56.5 (HT Extended

Capabilities field)"

::= { dot11RMNeighborReportEntry 45 }

dot11RMNeighborReportHTPCOTransitionTime OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT PCO transition time indicates that the STA can switch

between 20 MHz channel width and 40 MHz channel width within the

indicated time, set to 0 for no transition, set to 1 for 400 microsecond,

set to 2 for 1.5 ms, set to 3 for 5 ms. For the no transition

case (set to 0) the PCO active STA does not change its operation

channel width and is able to receive 40 MHz PPDUs during the 20

MHz phase. See 7.3.2.56.5 (HT Extended Capabilities field)"

::= { dot11RMNeighborReportEntry 46 }

dot11RMNeighborReportHTMCSFeedback OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT MFB indicates the capability of the STA to provide MFB,

set to 0 if the STA does not provide MFB, set to 2 if the STA

provide only unsolicited MFB, set to 3 if the STA can provide

MFB in response to MRQ as well as unsolicited MFB. Note the

value 1 is reserved. See 7.3.2.56.5 (HT Extended Capabilities

field)"

::= { dot11RMNeighborReportEntry 47 }

dot11RMNeighborReportHTCSupport OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT +HTC support indicates support of the HT Control field,

set to false if not supported, set to true if supported. See

7.3.2.56.5 (HT Extended Capabilities field)"

::= { dot11RMNeighborReportEntry 48 }

dot11RMNeighborReportHTRDResponder OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT RD responder indicates support for acting as a revere

direction responder, set to false if not supported, set to true

if supported. See 7.3.2.56.5 (HT Extended Capabilities field)"

::= { dot11RMNeighborReportEntry 49}

dot11RMNeighborReportHTImplictTransmitBeamformingReceivingCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT implicit Transmit Beamforming receiving capable indi-

cates whether this STA can receive Transmit Beamforming steered

frames using implicit feedback, set to false if not supported,

set to true if supported. See 7.3.2.56.6 (Transmit Beamforming

Capabilities)"

::= { dot11RMNeighborReportEntry 50 }

dot11RMNeighborReportHTReceiveStaggeredSoundingCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT receive staggered sounding capable indicates whether

this STA can receive staggered sounding frames, set to false if

not supported, set to true if supported. See 7.3.2.56.6 (Trans-

mit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 51 }

dot11RMNeighborReportHTTransmitStaggeredSoundingCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT transmit staggered sounding capable indicates whether

this STA can transmit staggered sounding frames, set to false if

not supported, set to true if supported. See 7.3.2.56.6 (Trans-

mit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 52 }

dot11RMNeighborReportHTReceiveNDPCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Receive NDP capable indicates whether this receiver can

interpret NDPs as sounding frames, set to false if not sup-

ported, set to true if supported. See 7.3.2.56.6 (Transmit Beam-

forming Capabilities)"

::= { dot11RMNeighborReportEntry 53 }

dot11RMNeighborReportHTTransmitNDPCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Transmit NDP capable indicates whether this STA can

transmit NDPs as sounding frames, set to false if not supported,

set to true if supported. See 7.3.2.56.6 (Transmit Beamforming

Capabilities)"

::= { dot11RMNeighborReportEntry 54 }

dot11RMNeighborReportHTImplicitTransmitBeamformingCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Implicit Transmit Beamforming capable indicates whether

this STA can apply implicit transmit beamforming, set to false

if not supported, set to true if supported. See 7.3.2.56.6

(Transmit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 55 }

dot11RMNeighborReportHTTransmitBeamformingCalibration OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Beamforming Calibration indicates that the STA can par-

ticipate in a calibration procedure initiated by another STA

that is capable of generating an immediate response Sounding

PPDU and can provide a CSI report in response to the receipt of

a Sounding PPDU, set to 0 if not supported, set to 1 is the STA

can respond to a calibration request using the CSI report but

cannot initiate calibration, set to 3 if the STA can both ini-

tiate and respond to a calibration request. See 7.3.2.56.6

(Transmit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 56 }

dot11RMNeighborReportHTExplicitCSITransmitBeamformingCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT explicit CSI Transmit Beamforming capable indicates

whether this STA can apply transmit beamforming using SCI

explicit feedback in its transmission, set to false if not sup-

ported, set to true if supported. See 7.3.2.56.6 (Transmit Beam-

forming Capabilities)"

::= { dot11RMNeighborReportEntry 57 }

dot11RMNeighborReportHTExplicitNonCompressedSteeringCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT explicit noncompressed steering capable indicates

whether this STA can apply transmit beamforming using noncom-

pressed beamforming feedback matrix explicit feedback in its

transmission, set to false if not supported, set to true if sup-

ported. See 7.3.2.56.6 (Transmit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 58 }

dot11RMNeighborReportHTExplicitCompressedSteeringCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT explicit compressed steering capable indicates whether

this STA can apply transmit beamforming using compressed beam-

forming feedback matrix explicit feedback in its transmission,

set to false if not supported, set to true if supported. See

7.3.2.56.6 (Transmit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 59 }

dot11RMNeighborReportHTExplicitTransmitBeamformingFeedback OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT explicit Transmit Beamforming CSI feedback indicates

whether this receiver can return CSI explicit feedback, set to 0

if not supported, set to 1 for delayed feedback, set to 2 for

immediate feedback, set to 3 for delayed and immediate feedback.

See 7.3.2.56.6 (Transmit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 60 }

dot11RMNbRprtHTExplicitNonCompressedBeamformingFeedbackCap OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Explicit noncompressed beamforming feedback capable

indicates whether this receiver can return noncompressed beam-

forming feedback matrix explicit feedback, set to 0 if not sup-

ported, set to 1 for delayed feedback, set to 2 for immediate

feedback, set to 3 for delayed and immediate feedback. See

7.3.2.56.6 (Transmit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 61 }

dot11RMNeighborReportHTExplicitCompressedBeamformingFeedbackCap OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT explicit compressed beamforming feedback capable indi-

cates whether this receiver can return compressed beamforming

feedback matrix explicit feedback, set to 0 if not supported,

set to 1 for delayed feedback, set to 2 for immediate feedback,

set to 3 for delayed and immediate feedback. See 7.3.2.56.6

(Transmit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 62 }

dot11RMNeighborReportHTTransmitBeamformingMinimalGrouping OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Transmit Beamforming minimal grouping indicates the min-

imal grouping used for explicit feedback reports, set to 0 if

the STA supports groups of 1 (no grouping), set to 1 to indicate

groups of 1, 2, set to 2 to indicate groups of 1, 4, set to 3 to

indicate groups of 1, 2, 4. See 7.3.2.56.6 (Transmit Beamforming

Capabilities)"

::= { dot11RMNeighborReportEntry 63 }

dot11RMNbRprtHTCSINumberofTxBeamformingAntennasSuppt OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT CSI number of beamformer antennas supported indicates

the maximum number of beamformer antennas the beamformee can

support when CSI feedback is required, set to 0 for single Tx

antenna sounding, set to 1 for 2 Tx antenna sounding, set to 2

for 3 Tx antenna sounding, set to 3 for 4 Tx antenna sounding.

See 7.3.2.56.6 (Transmit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 64 }

dot11RMNbRprtHTNonCompressedSteeringNumofTxBmfmingAntennasSuppt OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT noncompressed steering number of beamformer antennas

supported indicates the maximum number of beamformer antennas

the beamformee can support when noncompressed beamforming feed-

back matrix is required, set to 0 for single Tx antenna sound-

ing, set to 1 for 2 Tx antenna sounding, set to 2 for 3 Tx

antenna sounding, set to 3 for 4 Tx antenna sounding. See

7.3.2.56.6 (Transmit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 65 }

dot11RMNbRprtHTCompressedSteeringNumberofTxBmfmingAntennasSuppt OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT compressed steering number of beamformer antennas sup-

ported indicates the maximum number of beamformer antennas the

beamformee can support when compressed beamforming feedback

matrix is required, set to 0 for single Tx antenna sounding, set

to 1 for 2 Tx antenna sounding, set to 2 for 3 Tx antenna sound-

ing, set to 3 for 4 Tx antenna sounding. See 7.3.2.56.6 (Trans-

mit Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 66 }

dot11RMNbRprtHTCSIMaxNumberofRowsTxBeamformingSuppt OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT CSI max number of rows beamformer supported indicates

the maximum number of rows of CSI explicit feedback from the

beamformee or calibration responder or transmit ASEL responder

that a beamformer or calibration initiator or transmit ASEL ini-

tiator can support when SCI feedback is required, set to 0 for a

single row of CSI, set to 1 for 2 rows of CSI, set to 2 for 3

rows of CSI, set to 3 for 4 rows of CSI. See 7.3.2.56.6 (Transmit

Beamforming Capabilities)"

::= { dot11RMNeighborReportEntry 67 }

dot11RMNeighborReportHTTransmitBeamformingChannelEstimationCap OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT channel estimation capability indicates the maximum num-

ber of space-time streams for which channel dimensions can be

simultaneously estimated when receiving an NDP sounding PPDU or

the extension portion of the HT-LTFs in a staggered sounding

PPDU. Set to 0 for 1 space-time stream, set to 1 for 2 space-time

streams, set to 2 for 3 space-time streams, set to 3 for 4 space-

time streams. See 7.3.2.56.6 (Transmit Beamforming Capabili-

ties)"

::= { dot11RMNeighborReportEntry 68 }

dot11RMNeighborReportHTAntSelectionCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT ASEL capable indicates whether this STA supports ASEL,

set to false if not supported, set to true if supported. See

7.3.2.56.7 (ASEL Capability field)"

::= { dot11RMNeighborReportEntry 69 }

dot11RMNeighborReportHTExplicitCSIFeedbackBasedTxASELCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT explicit CSI feedback based transmit ASEL capable indi-

cates whether this STA has transmit ASEL capability based on

explicit CSI feedback, set to false if not supported, set to

true if supported. See 7.3.2.56.7 (ASEL Capability field)"

::= { dot11RMNeighborReportEntry 70 }

dot11RMNeighborReportHTAntIndicesFeedbackBasedTxASELCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT antenna indices feedback based transmit ASEL capable

indicates whether this STA has transmit ASEL capability based on

antenna indices feedback, set to false if not supported, set to

true if supported. See 7.3.2.56.7 (ASEL Capability field)"

::= { dot11RMNeighborReportEntry 71 }

dot11RMNeighborReportHTExplicitCSIFeedbackBasedCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The explicit CSI feedback capable indicates whether this STA

can compute CSI and feedback in support of ASEL, set to false if

not supported, set to true is supported. HT See 7.3.2.56.7 (ASEL

Capability field)"

::= { dot11RMNeighborReportEntry 72 }

dot11RMNeighborReportHTAntIndicesFeedbackCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT antenna indices feedback capable indicates whether this

STA has Rx ASEL capability, set to false if not supported, set

to true if supported. See 7.3.2.56.7 (ASEL Capability field)"

::= { dot11RMNeighborReportEntry 73 }

dot11RMNeighborReportHTRxASELCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT receive ASEL capable indicates whether this STA has Rx

ASEL capability, set to false if not supported, set to true if

supported. See 7.3.2.56.7 (ASEL Capability field)"

::= { dot11RMNeighborReportEntry 74 }

dot11RMNeighborReportHTTxSoundingPPDUsCap OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT transmit sounding PPDUs capable indicates whether this

STA can transmit sounding PPDUs for ASEL training per request,

set to false if not supported, set to true if supported. See

7.3.2.56.7 (ASEL Capability field)"

::= { dot11RMNeighborReportEntry 75 }

dot11RMNeighborReportHTInfoPrimaryChannel OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info primary channel indicates the channel number of the

primary channel, encoding: channel number of the primary chan-

nel. See 7.3.2.57 (HT Operation element)"

::= { dot11RMNeighborReportEntry 76 }

dot11RMNeighborReportHTInfoSecChannelOffset OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info secondary channel offset indicates the offset of

the secondary channel relative to the primary channel, set to 1

if the secondary channel is above the primary channel, set to 3

if the secondary channel is below the primary channel, set to 0

if no secondary channel is present. The value 2 is reserved. See

7.3.2.57 (HT Operation element)"

::= { dot11RMNeighborReportEntry 77 }

dot11RMNeighborReportHTInfoSTAChannelWidth OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info STA channel width defines the channel widths that

may be used to transmit to the STA, set to false for a 20 MHz

channel width, set to true allows use of any channel width in

the supported channel width set. See 7.3.2.57 (HT Operation ele-

ment)"

::= { dot11RMNeighborReportEntry 78 }

dot11RMNeighborReportHTInfoRIFSMode OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info RIFS mode indicates whether use of RIFS is permit-

ted within the BSS, set to false if use of RIFS is prohibited,

set to true if use of RIFS is permitted. See 7.3.2.57 (HT Opera-

tion element)"

::= { dot11RMNeighborReportEntry 79 }

dot11RMNeighborReportHTInfoProtection OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info protection indicates protection requirements of HT

transmissions. Set to 0 if all STAs detected in the primary or

the secondary channel or that are a member of this BSS are HT

STAs and either all STAs that are known by the transmitting STA

to be a member of this BSS are 20/40 MHz HT in a 20/40 MHz BSS or

this BSS is a 20 MHz BSS. Set to 1 (nonmember protection mode) if

there is at least one non-HT STA detected in either the primary

or the secondary channel or in both the primary and secondary

channels and that is not known by the transmitting STA to be a

member of this BSS and all STAs that are known by the transmit-

ting STA to be a member of this BSS are HT STAs. Set to 2 if all

STAs detected in the primary or the secondary channel or that

are known by the transmitting STA to be a member of this BSS are

HT STAs and this BSS is a 20/40 MHz BSS and there is at least one

20 MHz HT STA associated with this BSS. Set to 3 (non-HT mixed

mode) otherwise. See 7.3.2.56.2 (HT Capabilities Info field)"

::= { dot11RMNeighborReportEntry 80 }

dot11RMNeighborReportHTInfoNonGreenfieldHTSTAsPresent OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info nongreenfield HT STAs present indicates if any HT

STAs that are not HT-greenfield capable have associated. Deter-

mines when a non-AP STA should use HT-greenfield protection.

Present in Beacon and Probe Response frames transmitted by an

AP. Set to false if all HT STAs that are associated are HT-

greenfield capable, set to true if one or more HT STAs that are

not HT-greenfield capable are associated. See 7.3.2.57 (HT Oper-

ation element)"

::= { dot11RMNeighborReportEntry 81 }

dot11RMNeighborReportHTInfoOBSSNonHTSTAsPresent OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info OBSS non-HT STAs present indicates if the use of

protection for non-HT STAs by OBSSs is determined to be desir-

able. Present in Beacon and Probe Response frames transmitted by

an AP, set to true if the use of protection for non-HT STAs by

OBSSs is determined to be desirable, set to false otherwise. See

7.3.2.57 (HT Operation element)"

::= { dot11RMNeighborReportEntry 82 }

dot11RMNeighborReportHTInfoDualBeacon OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info dual beacon indicates whether the AP transmits an

STBC beacon, set to false if no STBC beacon is transmitted, set

to true if an STBC beacon is transmitted. See 7.3.2.57 (HT Oper-

ation element)"

::= { dot11RMNeighborReportEntry 83 }

dot11RMNeighborReportHTInfoDualCTSProtection OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info dual CTS protection is used by the AP to set a NAV

at STAs that do not support STBC and at STAs that can associate

solely through the secondary beacon, set to false if dual CTS

protection is not required, set to true if dual CTS protection

is required. See 7.3.2.57 (HT Operation element)"

::= { dot11RMNeighborReportEntry 84 }

dot11RMNeighborReportHTInfoSTBCBeacon OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info STBC beacon indicates whether the beacon containing

this element is a primary or a STBC beacon, set to false in a

primary beacon, set to true in a STBC beacon. See 7.3.2.57 (HT

Operation element)"

::= { dot11RMNeighborReportEntry 85 }

dot11RMNeighborReportHTInfoLSIGTXOPProtectionSup OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info L-SIG TXOP protection full support indicates

whether all HT STA in the BSS support L-SIG TXOP protection, set

to false if one or more HT STA in the BSS do not support L-SIG

TXOP protection, set to true if all HT STA in the BSS support L-

SIG TXOP protection. See 7.3.2.57 (HT Operation element)"

::= { dot11RMNeighborReportEntry 86 }

dot11RMNeighborReportHTInfoPCOActive OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info PCO active indicates whether PCO is active in the

BSS, set to false if PCO is not active in the BSS, set to true if

PCO is active in the BSS. See 7.3.2.57 (HT Operation element)"

::= { dot11RMNeighborReportEntry 87}

dot11RMNeighborReportHTInfoPCOPhase OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info PCO phase indicates the PCO phase of operation, set

to false indicates a switch to or continued 20 MHz phase, set to

true indicates a switch to or continuation of 40 MHz phase. See

7.3.2.57 (HT Operation element)"

::= { dot11RMNeighborReportEntry 88 }

dot11RMNeighborReportHTInfoBasicMCSSet OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info Basic MCS Set indicates values that are supported

by all HT STAs in the BSS. The Basic MSC Set is a bitmap of size

128 bits. Bit 0 corresponds to MCS 0. A bit is set to 1 to indi-

cate support for that MCS, set to 0 otherwise. See 7.3.2.57 (HT

Operation element)"

::= { dot11RMNeighborReportEntry 89 }

dot11RMNeighborReportHTSecChannelOffset OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT secondary channel offset indicates the position of the

secondary channel relative to the primary channel, set to 1 to

indicate that the secondary channel is above the primary chan-

nel, set to 3 to indicate the secondary channel is below the

primary channel, set to 0 to indicate that no secondary channel

is present. The value 2 is reserved. See 7.3.2.20a (Secondary

Channel Offset element(11n))"

::= { dot11RMNeighborReportEntry 90 }

dot11RMNeighborReportExtCapPSMPSupport OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Extended Capabilities PSMP support indicates support for

PSMP operation, set to false if PSMP is not supported, set to

true if PSMP operation is supported. See 7.3.2.27 (Extended

Capabilities (#1684)element)"

::= { dot11RMNeighborReportEntry 91 }

dot11RMNeighborReportExtCapSPSMPSup OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The HT Info S-PSMP support indicates support for scheduled

PSMP, set to false when PSMP is supported is set to false and

when PSMP support is set to 1 if the STA does not support S-PSMP,

set to true when PSMP support is set to 1 if the STA supports S-

PSMP. See 7.3.2.27 (Extended Capabilities (#1684)element)"

::= { dot11RMNeighborReportEntry 92 }

dot11RMNeighborReportExtCapServiceIntervalGranularity OBJECT-TYPE

SYNTAX Unsigned32 (0..7)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Extended Capabilities SI granularity indicates the duration

of the shortest SI, set to 0 for 5 ms, set to 1 for 10 ms, set to

2 for 15 ms, set to 3 for 20 ms, set to 4 for 25 ms, set to 5 for

30 ms, set to 6 for 35 ms, set to 7 for 40 ms. See 7.3.2.27

(Extended Capabilities (#1684)element)"

::= { dot11RMNeighborReportEntry 93 }

-- ********************************************************************

-- * End of dot11RMNeighborReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FastBSSTransitionConfig TABLE

-- ********************************************************************

dot11FastBSSTransitionConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FastBSSTransitionConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The table containing fast BSS transition configuration

objects."

::= { dot11smt 15 }

dot11FastBSSTransitionConfigEntry OBJECT-TYPE

SYNTAX Dot11FastBSSTransitionConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FastBSSTransitionConfigTable."

INDEX { ifIndex }

::= { dot11FastBSSTransitionConfigTable 1 }

Dot11FastBSSTransitionConfigEntry ::=

SEQUENCE {

dot11FastBSSTransitionActivated	TruthValue,

dot11FTMobilityDomainID	OCTET STRING,

dot11FTOverDSActivated	TruthValue,

dot11FTResourceRequestSupported	TruthValue,

dot11FTR0KeyHolderID	OCTET STRING,

dot11FTR0KeyLifetime	Unsigned32,

dot11FTR1KeyHolderID	OCTET STRING,

dot11FTReassociationDeadline	Unsigned32 }

dot11FastBSSTransitionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

When this object is true, this indicates that fast BSS transi-

tion (FT) is enabled on this entity. The entity will advertise

the FT-related elements in its Beacon and Probe Response frames.

This object requires that dot11FastBSSTransitionImplemented also

be set to true."

::= { dot11FastBSSTransitionConfigEntry 1 }

dot11FTMobilityDomainID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(2))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the Mobility Domain identifier (MDID)

of this entity.

The MDID is used to indicate a group of APs, within an ESS,

between which a STA can use fast BSS transition services. Fast

BSS transitions are allowed only between APs that have the same

MDID and are within the same ESS. They are not allowed between

APs with different MDIDs or in different ESSs.

Since fast BSS transition services are defined only within the

scope of an ESS, there is no requirement that MDIDs be unique

across ESSs."

::= { dot11FastBSSTransitionConfigEntry 2 }

dot11FTOverDSActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

When this object is true, this indicates that fast BSS transi-

tion via the over-the-DS protocol as described in Clause 11A

(Fast BSS transition) is enabled on this AP entity."

::= { dot11FastBSSTransitionConfigEntry 3 }

dot11FTResourceRequestSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

When this object is true, this indicates that the fast BSS tran-

sition (FT) resource request procedures of 11A.10 (Remote

request broker (RRB) communication) are supported on this AP

entity."

::= { dot11FastBSSTransitionConfigEntry 4 }

dot11FTR0KeyHolderID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..48))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the PMK-R0 key holder identifier (R0KH-

ID) of the Authenticator of this AP.

NOTE: Backend protocol may allow longer NAS Client identifiers

(e.g., RADIUS allows up to 253-octet NAS-Identifier), but when

used with fast BSS transition, the maximum length is limited to

48 octets. The same value must be used for the NAS Client iden-

tifier and dot11FTR0KeyHolderID."

::= { dot11FastBSSTransitionConfigEntry 5 }

dot11FTR0KeyLifetime OBJECT-TYPE

SYNTAX Unsigned32 (60..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the default lifetime of the PMK-R0, in

seconds, when a Session-Timeout attribute is not provided during

the EAP authentication. This attribute also applies when the

PMK-R0 is derived from a PSK."

DEFVAL { 1209600 }

::= { dot11FastBSSTransitionConfigEntry 6 }

dot11FTR1KeyHolderID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(6))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the PMK-R1 key holder identifier (R1KH-

ID) of the Authenticator of this AP. It is set to a MAC address

of the entity holding the PMK-R1 in the Authenticator."

::= { dot11FastBSSTransitionConfigEntry 7 }

dot11FTReassociationDeadline OBJECT-TYPE

SYNTAX Unsigned32 (1000..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of time units (TUs) that

this target AP entity retains a PTKSA and reserve any specified

resources for a STA while waiting for a reassociation from that

STA. It is assumed that this value will be administered consis-

tently across the mobility domain"

DEFVAL { 1000 }

::= { dot11FastBSSTransitionConfigEntry 8 }

-- *********************************************************************

-- * End of dot11FastBSSTransitionConfig TABLE

-- *********************************************************************

-- ********************************************************************

-- * dot11LCIDSE TABLE

-- ********************************************************************

dot11LCIDSETable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LCIDSEEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains conceptual table of attributes for Dependent

Station Enablement."

::= { dot11smt 16 }

dot11LCIDSEEntry OBJECT-TYPE

SYNTAX Dot11LCIDSEEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11LCIDSETable Indexed by dot11LCIDSEIndex."

INDEX { dot11LCIDSEIndex }

::= { dot11LCIDSETable 1 }

Dot11LCIDSEEntry ::=

SEQUENCE {

dot11LCIDSEIndex                         Unsigned32,

dot11LCIDSEIfIndex                       InterfaceIndex,

dot11LCIDSECurrentRegulatoryClass        Unsigned32,

dot11LCIDSELatitudeResolution            Unsigned32,

dot11LCIDSELatitudeInteger               Integer32,

dot11LCIDSELatitudeFraction              Integer32,

dot11LCIDSELongitudeResolution           Unsigned32,

dot11LCIDSELongitudeInteger              Integer32,

dot11LCIDSELongitudeFraction             Integer32,

dot11LCIDSEAltitudeType                  INTEGER,

dot11LCIDSEAltitudeResolution            Unsigned32,

dot11LCIDSEAltitudeInteger               Integer32,

dot11LCIDSEAltitudeFraction              Integer32,

dot11LCIDSEDatum                         Unsigned32,

dot11RegLocAgreement                     TruthValue,

dot11RegLocDSE                           TruthValue,

dot11DependentSTA                        TruthValue,

dot11DependentEnablementIdentifier       Unsigned32,

dot11DSEEnablementTimeLimit              Unsigned32,

dot11DSEEnablementFailHoldTime           Unsigned32,

dot11DSERenewalTime                      Unsigned32,

dot11DSETransmitDivisor                  Unsigned32 }

dot11LCIDSEIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for LCI DSE elements in dot11LCIDSETable, greater than

0."

::= { dot11LCIDSEEntry 1 }

dot11LCIDSEIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Each IEEE 802.11 interface is represented by an ifEntry. Inter-

face Tables in this MIB are indexed by ifIndex."

::= { dot11LCIDSEEntry 2 }

dot11LCIDSECurrentRegulatoryClass OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Current Regulatory Class is 8 bits indicating the particular

Regulatory Class in use by the radio."

::= { dot11LCIDSEEntry 3 }

dot11LCIDSELatitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Latitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Latitude. This field is derived

from IETF RFC-3825."

::= { dot11LCIDSEEntry 4 }

dot11LCIDSELatitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Latitude is a twos-complement 34-bit fixed point value consist-

ing of 9 bits of integer and 25 bits of fraction. This field con-

tains the 9 bits of integer portion of Latitude. This field is

derived from IETF RFC-3825."

::= { dot11LCIDSEEntry 5 }

dot11LCIDSELatitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Latitude is a twos-complement 34-bit fixed point value consist-

ing of 9 bits of integer and 25 bits of fraction. This field con-

tains the 25 bits of fraction portion of Latitude. This field is

derived from IETF RFC-3825."

::= { dot11LCIDSEEntry 6 }

dot11LCIDSELongitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Longitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Longitude. This field is

derived from IETF RFC-3825."

::= { dot11LCIDSEEntry 7 }

dot11LCIDSELongitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Longitude is a twos-complement 34-bit fixed point value consist-

ing of 9 bits of integer and 25 bits of fraction. This field con-

tains the 9 bits of integer portion of Longitude. This field is

derived from IETF RFC-3825."

::= { dot11LCIDSEEntry 8 }

dot11LCIDSELongitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Longitude is a twos-complement 34-bit fixed point value consist-

ing of 9 bits of integer and 25 bits of fraction. This field con-

tains the 25 bits of fraction portion of Longitude. This field

is derived from IETF RFC-3825."

::= { dot11LCIDSEEntry 9 }

dot11LCIDSEAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2),

hagm(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Altitude Type is 4 bits encoding the type of altitude.

Codes defined are:

meters : in 2s-complement fixed-point 22-bit integer part with

8-bit fraction

floors : in 2s-complement fixed-point 22-bit integer part with

8-bit fraction

hagm : Height Above Ground in meters, in 2s-complement fixed-

point 22-bit integer part with 8-bit fraction.

This field is derived from IETF RFC-3825."

DEFVAL { 3 }

::= { dot11LCIDSEEntry 10 }

dot11LCIDSEAltitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Altitude resolution is 6 bits indicating the number of valid

bits in the altitude. This field is derived from IETF RFC-3825."

::= { dot11LCIDSEEntry 11 }

dot11LCIDSEAltitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-2097151..2097151)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Altitude is a 30-bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit inte-

ger Part with 8-bit fraction. This field contains the fixed-

point Part of Altitude. This field is derived from IETF RFC-

3825."

::= { dot11LCIDSEEntry 12 }

dot11LCIDSEAltitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-127..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Altitude is a 30-bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit inte-

ger Part with 8-bit fraction. This field contains the fraction

part of Altitude. This field is derived from IETF RFC-3825."

::= { dot11LCIDSEEntry 13 }

dot11LCIDSEDatum OBJECT-TYPE

SYNTAX Unsigned32 (1..3)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

Datum is an 8-bit value encoding the horizontal and vertical

references used for the coordinates given in this LCI. IETF RFC-

3825 defines the values of Datum. Type 1 is WGS-84, the coordi-

nate system used by GPS. Type 2 is NAD83 with NAVD88 vertical

reference. Type 3 is NAD83 with Mean Lower Low Water vertical

datum. All other types are reserved. This field is derived from

IETF RFC-3825."

DEFVAL { 1 }

::= { dot11LCIDSEEntry 14 }

dot11RegLocAgreement OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a DSE Enablement Action frame is

received.

RegLocAgreement reports the Enabling STA's Agreement status.

False indicates it is operating away from national borders and

outside national policy zones. True indicates it is operating by

agreement near national borders or inside national policy

zones."

DEFVAL { false }

::= { dot11LCIDSEEntry 15 }

dot11RegLocDSE OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a DSE Enablement Action frame is

received.

RegLocDSE reports the Enabling STA's DSE status. False indicates

Dependent STAs are not enabled. True indicates Dependent STA

operation is enabled."

DEFVAL { false }

::= { dot11LCIDSEEntry 16 }

dot11DependentSTA OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a DSE Enablement Action frame is

received.

This attribute reports the Dependent STA status of the STA that

sent the beacon or Probe Response with this information. False

indicates that STA is not operating as a Dependent STA. True

indicates that STA is operating as a Dependent STA."

DEFVAL { true }

::= { dot11LCIDSEEntry 17 }

dot11DependentEnablementIdentifier OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a DSE Enablement Action frame is

received.

This attribute reports the Dependent STA identifier assigned by

the enabling STA to the dependent station."

DEFVAL { 0 }

::= { dot11LCIDSEEntry 18 }

dot11DSEEnablementTimeLimit OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

dot11DSEAssociateTimeLimit indicates the maximum number of sec-

onds that a dependent STA may transmit in a DSE frequency band

while attaining enablement with an enabling STA. Unless another

value is mandated by regulatory authorities, the value is 32

seconds."

DEFVAL { 32 }

::= { dot11LCIDSEEntry 19 }

dot11DSEEnablementFailHoldTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

dot11DSEAssociateFailHoldTime indicates the number of seconds

that a dependent STA must not transmit in a DSE frequency band

when it fails to attain enablement with an enabling STA within

dot11DSEEnablementTimeLimit seconds. Unless another value is

mandated by regulatory authorities, the value is 512 seconds."

DEFVAL { 512 }

::= { dot11LCIDSEEntry 20}

dot11DSERenewalTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

dot11DSERenewalTime indicates the maximum number of seconds that

a dependent STA may operate in a DSE frequency band without

receiving and decoding an enabling signal from its enabling STA.

Unless another value is mandated by regulatory authorities, the

value is 60 seconds."

DEFVAL { 60 }

::= { dot11LCIDSEEntry 21}

dot11DSETransmitDivisor OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

dot11DSETransmitDivisor indicates the value used by a dependent

STA when operating in a DSE frequency band and transmitting. The

dependent STA sends an Action frame when the sum of

dot11TransmittedFragmentCount, dot11GroupTransmittedFrameCount

and dot11ReceivedFragmentCount modulo dot11DSETransmitDivisor

equals zero. Unless another value is mandated by regulatory

authorities, the default value is 256."

DEFVAL { 256 }

::= { dot11LCIDSEEntry 22 }

-- ********************************************************************

-- * End of dot11LCIDSE TABLE

-- ********************************************************************

-- **********************************************************************

-- * dot11HTStationConfig TABLE

-- **********************************************************************

dot11HTStationConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11HTStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Station Configuration attributes. In tabular form to allow for

multiple instances on an agent."

::= { dot11smt 17 }

dot11HTStationConfigEntry OBJECT-TYPE

SYNTAX Dot11HTStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the dot11HTStationConfig Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11HTStationConfigTable 1 }

Dot11HTStationConfigEntry ::=

SEQUENCE {

dot11HTOperationalMCSSet	OCTET STRING,

dot11MIMOPowerSave	INTEGER,

dot11NDelayedBlockAckOptionImplemented	TruthValue,

dot11MaxAMSDULength	INTEGER,

dot11STBCControlFrameOptionImplemented	TruthValue,

dot11LsigTxopProtectionOptionImplemented	TruthValue,

dot11MaxRxAMPDUFactor	Unsigned32,

dot11MinimumMPDUStartSpacing	Unsigned32,

dot11PCOOptionImplemented	TruthValue,

dot11TransitionTime	Unsigned32,

dot11MCSFeedbackOptionImplemented	INTEGER,

dot11HTControlFieldSupported	TruthValue,

dot11RDResponderOptionImplemented	TruthValue,

dot11SPPAMSDUCapable	TruthValue,

dot11SPPAMSDURequired	TruthValue,

dot11FortyMHzOptionImplemented	TruthValue }

dot11HTOperationalMCSSet OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..127))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the set of MCS at which the sta-

tion may transmit data. Each octet contains a value representing

a rate. Each MCS shall be within the range from 1 to 127, and

shall be supported for receiving data. This value is reported in

transmitted Beacon, Probe Request, Probe Response, Association

Request, Association Response, Reassociation Request, and Reas-

sociation Response frames, and is used to determine whether a

BSS with which the station desires to synchronize is suitable.

It is also used when starting a BSS, as specified in 10.3."

::= { dot11HTStationConfigEntry 1 }

dot11MIMOPowerSave OBJECT-TYPE

SYNTAX INTEGER { static(1), dynamic(2), mimo(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an 8-bit integer value that identifies the configured

power save state of MIMO."

::= { dot11HTStationConfigEntry 2 }

dot11NDelayedBlockAckOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable of supporting the No Ack option of the Delayed

Block Ack."

DEFVAL { false }

::= { dot11HTStationConfigEntry 3 }

dot11MaxAMSDULength OBJECT-TYPE

SYNTAX INTEGER { short(3839), long(7935) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the supported maximum size of A-MSDU."

DEFVAL { short }

::= { dot11HTStationConfigEntry 4 }

dot11STBCControlFrameOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable of processing the received control frames that

are STBC frames."

DEFVAL { false }

::= { dot11HTStationConfigEntry 5 }

dot11LsigTxopProtectionOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable of supporting L-SIG TXOP protection option."

DEFVAL { false }

::= { dot11HTStationConfigEntry 6 }

dot11MaxRxAMPDUFactor OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the maximum length of A-MPDU that the

STA can receive. The Maximum Rx A-MPDU defined by this field is

equal to 213+dot11MaxRxAMPDUFactor -1 octets."

DEFVAL { 0 }

::= { dot11HTStationConfigEntry 7 }

dot11MinimumMPDUStartSpacing OBJECT-TYPE

SYNTAX Unsigned32 (0..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the minimum time between the start of

adjacent MPDUs within an A-MPDU. This time is measured at the

PHY-SAP; the number of octets between the start of two consecu-

tive MPDUs in A-MPDU shall be equal or greater than

(dot11MinimumMPDUStartSpacing*PHY-bit-rate)/8. The encoding of

the minimum time to this attribute is:

0 	no restriction

1	1/4 microsecond

2 	1/2 microsecond

3	1 microsecond

4	2 microsecond

5 	4 microsecond

6	8 microsecond

7	16 microsecond "

DEFVAL { 0 }

::= { dot11HTStationConfigEntry 8 }

dot11PCOOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable of supporting PCO."

DEFVAL { false }

::= { dot11HTStationConfigEntry 9 }

dot11TransitionTime OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates that the maximum transition time

within which the STA can switch between 20 MHz channel width and

40 MHz channel width with a high probability. The encoding of

the transition time to this attribute is:

0	no transition

1	400 microsecond

2	1500 microsecond

3	5000 microsecond "

DEFVAL { 2 }

::= { dot11HTStationConfigEntry 10 }

dot11MCSFeedbackOptionImplemented OBJECT-TYPE

SYNTAX INTEGER { none(0), unsolicited (2), both (3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the MCS feed back capability supported

by the station implementation."

DEFVAL { 0 }

::= { dot11HTStationConfigEntry 11 }

dot11HTControlFieldSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable of receiving HT Control field."

DEFVAL { false }

::= { dot11HTStationConfigEntry 12 }

dot11RDResponderOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable operating as an RD responder."

DEFVAL { false }

::= { dot11HTStationConfigEntry 13 }

dot11SPPAMSDUCapable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the STA implementa-

tion is capable of protecting the A-MSDU bit (Bit 7) in the QoS

Control field when dot11RSNAActivated is true."

DEFVAL { false }

::= { dot11HTStationConfigEntry 14 }

dot11SPPAMSDURequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the STA is configured

to disallow (not to send or receive) of PP A-MSDUs when

dot11RSNAActivated is true."

DEFVAL { false }

::= { dot11HTStationConfigEntry 15 }

dot11FortyMHzOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the STA is capable of

transmitting and receiving on a 40 MHz channel using a 40 MHz

mask."

DEFVAL { false }

::= { dot11HTStationConfigEntry 16 }

-- **********************************************************************

-- * End of dot11HTStationConfig TABLE

-- **********************************************************************

-- **********************************************************************

-- * MAC Attribute Templates

-- **********************************************************************

-- **********************************************************************

-- * dot11Operation TABLE

-- **********************************************************************

dot11OperationTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11OperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains MAC attributes pertaining to the operation of

the MAC.  This has been implemented as a table in order to allow

for multiple instantiations on an agent."

::= { dot11mac 1 }

dot11OperationEntry OBJECT-TYPE

SYNTAX Dot11OperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11OperationEntry Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11OperationTable 1 }

Dot11OperationEntry ::=

SEQUENCE {

dot11MACAddress	MacAddress,

dot11RTSThreshold	Unsigned32,

dot11ShortRetryLimit	Unsigned32,

dot11LongRetryLimit	Unsigned32,

dot11FragmentationThreshold	Unsigned32,

dot11MaxTransmitMSDULifetime	Unsigned32,

dot11MaxReceiveLifetime	Unsigned32,

dot11ManufacturerID	DisplayString,

dot11ProductID	DisplayString,

dot11CAPLimit	Unsigned32,

dot11HCCWmin	Unsigned32,

dot11HCCWmax	Unsigned32,

dot11HCCAIFSN	Unsigned32,

dot11ADDBAResponseTimeout	Unsigned32,

dot11ADDTSResponseTimeout	Unsigned32,

dot11ChannelUtilizationBeaconInterval	Unsigned32,

dot11ScheduleTimeout	Unsigned32,

dot11DLSResponseTimeout	Unsigned32,

dot11QAPMissingAckRetryLimit	Unsigned32,

dot11EDCAAveragingPeriod	Unsigned32,

dot11HTProtection	 INTEGER,

dot11RIFSMode	TruthValue,

dot11PSMPControlledAccess	TruthValue,

dot11ServiceIntervalGranularity	Unsigned32,

dot11DualCTSProtection	TruthValue,

dot11LSIGTXOPFullProtectionActivated	TruthValue,

dot11NonGFEntitiesPresent	TruthValue,

dot11PCOActivated	TruthValue,

dot11PCOFortyMaxDuration	Unsigned32,

dot11PCOTwentyMaxDuration	Unsigned32,

dot11PCOFortyMinDuration	Unsigned32,

dot11PCOTwentyMinDuration	Unsigned32,

dot11FortyMHzIntolerant	TruthValue,

dot11BSSWidthTriggerScanInterval	Unsigned32,

dot11BSSWidthChannelTransitionDelayFactor	Unsigned32,

dot11OBSSScanPassiveDwell	Unsigned32,

dot11OBSSScanActiveDwell	Unsigned32,

dot11OBSSScanPassiveTotalPerChannel	Unsigned32,

dot11OBSSScanActiveTotalPerChannel	Unsigned32,

dot112040BSSCoexistenceManagementSupport	TruthValue,

dot11OBSSScanActivityThreshold	Unsigned32 }

dot11MACAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Unique MAC Address assigned to the STA."

::= { dot11OperationEntry 1 }

dot11RTSThreshold OBJECT-TYPE

SYNTAX Unsigned32 (0..65536)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the number of octets in a PSDU, below

which an RTS/CTS handshake will not be performed, except as RTS/

CTS is used as a cross modulation protection mechanism as

defined in 9.13 (Protection mechanisms(11n)). An RTS/CTS hand-

shake will be performed at the beginning of any frame exchange

sequence where the  PSDU is of type Data or Management, the PSDU

has an individual address in the Address1 field, and the length

of the PSDU is greater than this threshold. Setting this

attribute to be larger than the maximum PSDU size has the effect

of turning off the RTS/CTS handshake for frames of Data or Man-

agement type transmitted by this STA. Setting this attribute to

zero has the effect of turning on the RTS/CTS handshake for all

frames of Data or Management type transmitted by this STA."

DEFVAL { 65536 }

::= { dot11OperationEntry 2 }

dot11ShortRetryLimit OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the maximum number of transmission

attempts of a frame, the length of which is less than or equal to

dot11RTSThreshold, that is made before a failure condition is

indicated."

DEFVAL { 7 }

::= { dot11OperationEntry 3 }

dot11LongRetryLimit OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the maximum number of transmission

attempts of a frame, the length of which is greater than

dot11RTSThreshold, that is made before a failure condition is

indicated."

DEFVAL { 4 }

::= { dot11OperationEntry 4 }

dot11FragmentationThreshold OBJECT-TYPE

SYNTAX Unsigned32 (256..3000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the current maximum size, in octets, of

the MPDU that may be delivered to the security encapsulation. An

MSDU is broken into fragments if its size exceeds the value of

this attribute after adding MAC headers and trailers. Fields

added to the frame by security encapsulation are not counted

against the limit specified by this attribute. An MSDU or MMPDU

is fragmented when the resulting frame has an individual address

in the Address1 field, and the length of the frame is larger

than this threshold, excluding security encapsulation fields.

The default value for this attribute is the lesser of 3000 or

the aMPDUMaxLength of the attached PHY and never exceeds the

lesser of 3000 or the aMPDUMaxLength of the attached PHY."

::= { dot11OperationEntry 5 }

dot11MaxTransmitMSDULifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The MaxTransmitMSDULifetime is the elapsed time in TU, after the

initial transmission of an MSDU, after which further attempts to

transmit the MSDU are terminated."

DEFVAL { 512 }

::= { dot11OperationEntry 6 }

dot11MaxReceiveLifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The MaxReceiveLifetime is the elapsed time in TU, after the ini-

tial reception of a fragmented MMPDU or MSDU, after which fur-

ther attempts to reassemble the MMPDU or MSDU are terminated."

DEFVAL { 512 }

::= { dot11OperationEntry 7 }

dot11ManufacturerID OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The ManufacturerID includes, at a minimum, the name of the manu-

facturer.  It may include additional information at the manufac-

turer's discretion.  The default value of this attribute is

null."

::= { dot11OperationEntry 8 }

dot11ProductID OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The ProductID includes, at a minimum, an identifier that is

unique to the manufacturer.  It may include additional informa-

tion at the manufacturer's discretion. The default value of this

attribute is null."

::= { dot11OperationEntry 9 }

dot11CAPLimit OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the maximum number of TUs a Controlled

access phase(CAP) can last."

::= { dot11OperationEntry 10 }

dot11HCCWmin OBJECT-TYPE

SYNTAX Unsigned32 -- (0..aCWmin)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the value of the minimum size of the

window that is used by the HC for generating a random number for

the backoff. The value of this attribute is such that it could

always be expressed in the form of 2^X - 1, where X is an inte-

ger."

DEFVAL { 0 }

::= { dot11OperationEntry 11 }

dot11HCCWmax OBJECT-TYPE

SYNTAX Unsigned32 -- (0..aCWmax)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the value of the maximum size of the

window that is used by the HC for generating a random number for

the backoff. The value of this attribute is such that it could

always be expressed in the form of 2^X - 1, where X is an inte-

ger."

DEFVAL { 0 }

::= { dot11OperationEntry 12 }

dot11HCCAIFSN OBJECT-TYPE

SYNTAX Unsigned32 (1..15)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of slots, after a SIFS dura-

tion, that the HC senses the medium idle either before transmit-

ting or executing a backoff."

DEFVAL { 1 }

::= { dot11OperationEntry 13 }

dot11ADDBAResponseTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the maximum number of seconds a BA is

to be responded."

DEFVAL { 1 }

::= { dot11OperationEntry 14 }

dot11ADDTSResponseTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the maximum number of seconds an ADDTS

request is to be responded."

DEFVAL { 1 }

::= { dot11OperationEntry 15 }

dot11ChannelUtilizationBeaconInterval OBJECT-TYPE

SYNTAX Unsigned32 (1..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the number of beacon intervals over

which the channel busy time should be averaged."

DEFVAL { 50 }

::= { dot11OperationEntry 16 }

dot11ScheduleTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the duration in TUs after which a STA

could go into power-save mode."

DEFVAL { 10 }

::= { dot11OperationEntry 17 }

dot11DLSResponseTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the maximum number of seconds a direct

link request is to be responded."

DEFVAL { 10 }

::= { dot11OperationEntry 18 }

dot11QAPMissingAckRetryLimit OBJECT-TYPE

SYNTAX Unsigned32 (1..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the number of times the AP may retry a

frame for which it does not receive an ACK for a STA in power-

save mode after receiving a PS-Poll and sending a directed

response or after the AP does not receive an ACK to a directed

MPDU sent with the EOSP set to 1."

::= { dot11OperationEntry 19 }

dot11EDCAAveragingPeriod OBJECT-TYPE

SYNTAX Unsigned32 (1..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the number of seconds over which the

admitted_time and used_time are computed."

DEFVAL { 5 }

::= { dot11OperationEntry 20 }

dot11HTProtection OBJECT-TYPE

SYNTAX INTEGER { htNoProtection (0), htNonmemberProtection(1),

ht20MHzProtection(2), htNonHTmixed(3) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the level of protection that needs to

be provided to the transmissions in an IEEE 802.11 network with

HT STAs."

DEFVAL { htNoProtection }

::= { dot11OperationEntry 21 }

dot11RIFSMode OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that use of RIFS is

allowed in the BSS."

DEFVAL { false }

::= { dot11OperationEntry 22 }

dot11PSMPControlledAccess OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true indicates that the AP accepts associ-

ations only from STAs for which dot11PSMPOptionImplemented is

true."

DEFVAL { false }

::= { dot11OperationEntry 23 }

dot11ServiceIntervalGranularity OBJECT-TYPE

SYNTAX Unsigned32 (0..7)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the SI granularity to be used for

scheduled PSMP. The value of the granularity is given by

(dot11ServiceIntervalGranularity+1)*5 ms."

DEFVAL { 0 }

::= { dot11OperationEntry 24 }

dot11DualCTSProtection OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true indicates that the AP uses dual CTS

protection to protect the non-STBC frame and STBC frame trans-

missions."

DEFVAL { false }

::= { dot11OperationEntry 25 }

dot11LSIGTXOPFullProtectionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the LSIG TXOP protec-

tion may be used by STAs that have the attribute

dot11LSigTxopProtectionOptionImplemented set to true."

DEFVAL { false }

::= { dot11OperationEntry 26 }

dot11NonGFEntitiesPresent OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that STA that are not HT-

greenfield Capable are present in the BSS."

DEFVAL { false }

::= { dot11OperationEntry 27 }

dot11PCOActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the PCO is acti-

vated."

DEFVAL { false }

::= { dot11OperationEntry 28 }

dot11PCOFortyMaxDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..200)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute indicates the maximum duration of 40 MHz phase in

TU under PCO operation. The value of this attribute shall be

equal to or larger than dot11PCOFortyMinDuration."

DEFVAL { 30 }

::= { dot11OperationEntry 29 }

dot11PCOTwentyMaxDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..200)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute indicates the maximum duration of 20 MHz phase in

TU under PCO operation. The value of this attribute shall be

equal to or larger than dot11PCOTwentyMinDuration."

DEFVAL { 30 }

::= { dot11OperationEntry 30 }

dot11PCOFortyMinDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..200)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute indicates the minimum duration of 40 MHz phase in

TU under PCO operation."

DEFVAL { 20 }

::= { dot11OperationEntry 31 }

dot11PCOTwentyMinDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..200)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute indicates the minimum duration of 20 MHz phase in

TU under PCO operation."

DEFVAL { 20 }

::= { dot11OperationEntry 32 }

dot11FortyMHzIntolerant OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the STA requests that

40 MHz mask PPDUs are not transmitted within range of the STA."

DEFVAL { false }

::= { dot11OperationEntry 33 }

dot11BSSWidthTriggerScanInterval OBJECT-TYPE

SYNTAX Unsigned32 (10..900)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum interval in seconds

between scan operations to be performed to detect BSS channel

width trigger events."

DEFVAL { 300 }

::= { dot11OperationEntry 34 }

dot11BSSWidthChannelTransitionDelayFactor OBJECT-TYPE

SYNTAX Unsigned32 (5..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the minimum ratio between the delay

time in performing a switch from 20 MHz BSS operation to 20/40

MHz BSS operation and the maximum interval between OBSS scan

operations."

DEFVAL { 5 }

::= { dot11OperationEntry 35 }

dot11OBSSScanPassiveDwell OBJECT-TYPE

SYNTAX Unsigned32 (5..1000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the minimum amount of time in TU that

the STA continuously scans each channel when performing a pas-

sive OBSS scan operation."

DEFVAL { 20 }

::= { dot11OperationEntry 36 }

dot11OBSSScanActiveDwell OBJECT-TYPE

SYNTAX Unsigned32 (10..1000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the minimum amount of time in TU that

the STA continuously scans each channel when performing an

active OBSS scan operation."

DEFVAL { 10 }

::= { dot11OperationEntry 37 }

dot11OBSSScanPassiveTotalPerChannel OBJECT-TYPE

SYNTAX Unsigned32 (200..10000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the minimum total amount of time in TU

that the STA scans each channel when performing a passive OBSS

scan operation."

DEFVAL { 200 }

::= { dot11OperationEntry 38 }

dot11OBSSScanActiveTotalPerChannel OBJECT-TYPE

SYNTAX Unsigned32 (20..10000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the minimum total amount of time in TU

that the STA scans each channel when performing an active OBSS

scan operation."

DEFVAL { 20 }

::= { dot11OperationEntry 39 }

dot112040BSSCoexistenceManagementSupport OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the STA supports the

transmission and reception of the 20/40 BSS Coexistence Manage-

ment frame."

DEFVAL { false }

::= { dot11OperationEntry 40 }

dot11OBSSScanActivityThreshold OBJECT-TYPE

SYNTAX Unsigned32 (0..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates in hundredths of percent, the maximum

total time that a STA may be active on the medium during a period

of dot11BSSWidthChannelTransitionDelayFactor *

dot11BSSWidthTriggerScanInterval seconds without being obligated

to perform OBSS Scan operations. The default value of this

attribute is 25, which equates to 0.25%."

DEFVAL { 25 }

::= { dot11OperationEntry 41}

-- **********************************************************************

-- *    End of dot11Operation TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11Counters TABLE

-- **********************************************************************

dot11CountersTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11CountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes that are MAC counters. Implemented

as a table to allow for multiple instantiations on an agent."

::= { dot11mac 2 }

dot11CountersEntry OBJECT-TYPE

SYNTAX Dot11CountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11CountersEntry Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11CountersTable 1 }

Dot11CountersEntry ::=

SEQUENCE { dot11TransmittedFragmentCount	Counter32,

dot11GroupTransmittedFrameCount	Counter32,

dot11FailedCount	Counter32,

dot11RetryCount	Counter32,

dot11MultipleRetryCount	Counter32,

dot11FrameDuplicateCount	Counter32,

dot11RTSSuccessCount	Counter32,

dot11RTSFailureCount	Counter32,

dot11ACKFailureCount	Counter32,

dot11ReceivedFragmentCount	Counter32,

dot11GroupReceivedFrameCount	Counter32,

dot11FCSErrorCount	Counter32,

dot11TransmittedFrameCount	Counter32,

dot11WEPUndecryptableCount	Counter32,

dot11QosDiscardedFragmentCount	Counter32,

dot11AssociatedStationCount	Counter32,

dot11QosCFPollsReceivedCount	Counter32,

dot11QosCFPollsUnusedCount	Counter32,

dot11QosCFPollsUnusableCount	Counter32,

dot11QoSCFPollsLostCount	Counter32,

dot11TransmittedAMSDUCount	Counter32,

dot11FailedAMSDUCount	Counter32,

dot11RetryAMSDUCount	Counter32,

dot11MultipleRetryAMSDUCount	Counter32,

dot11TransmittedOctetsInAMSDUCount	Counter64,

dot11AMSDUAckFailureCount	Counter32,

dot11ReceivedAMSDUCount	Counter32,

dot11ReceivedOctetsInAMSDUCount	Counter64,

dot11TransmittedAMPDUCount	Counter32,

dot11TransmittedMPDUsInAMPDUCount	Counter32,

dot11TransmittedOctetsInAMPDUCount	Counter64,

dot11AMPDUReceivedCount	Counter32,

dot11MPDUInReceivedAMPDUCount	Counter32,

dot11ReceivedOctetsInAMPDUCount	Counter64,

dot11AMPDUDelimiterCRCErrorCount	Counter32,

dot11ImplicitBARFailureCount	Counter32,

dot11ExplicitBARFailureCount	Counter32,

dot11ChannelWidthSwitchCount	Counter32,

dot11TwentyMHzFrameTransmittedCount	Counter32,

dot11FortyMHzFrameTransmittedCount	Counter32,

dot11TwentyMHzFrameReceivedCount	Counter32,

dot11FortyMHzFrameReceivedCount	Counter32,

dot11PSMPUTTGrantDuration	Counter32,

dot11PSMPUTTUsedDuration	Counter32,

dot11GrantedRDGUsedCount	Counter32,

dot11GrantedRDGUnusedCount	Counter32,

dot11TransmittedFramesInGrantedRDGCount	Counter32,

dot11TransmittedOctetsInGrantedRDGCount	Counter64,

dot11BeamformingFrameCount	Counter32,

dot11DualCTSSuccessCount	Counter32,

dot11DualCTSFailureCount	Counter32,

dot11STBCCTSSuccessCount	Counter32,

dot11STBCCTSFailureCount	Counter32,

dot11nonSTBCCTSSuccessCount	Counter32,

dot11nonSTBCCTSFailureCount	Counter32,

dot11RTSLSIGSuccessCount	Counter32,

dot11RTSLSIGFailureCount	Counter32,

dot11PBACErrors	Counter32 }

dot11TransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a fragment is successfully trans-

mitted.

This counter is incremented for an acknowledged MPDU with an

individual address in the address 1 field or an MPDU with a

group address in the address 1 field of type Data or Manage-

ment."

::= { dot11CountersEntry 1 }

dot11GroupTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a group addressed frame is trans-

mitted.

This counter is incremented only when the group bit is set in

the destination MAC address of a successfully transmitted MSDU.

When operating as a STA in an ESS, where these frames are

directed to the AP, this implies having received an acknowledg-

ment to all associated MPDUs."

::= { dot11CountersEntry 2 }

dot11FailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when an MSDU is not transmitted success-

fully due to the number of transmit attempts exceeding either

the  dot11ShortRetryLimit or dot11LongRetryLimit."

::= { dot11CountersEntry 3 }

dot11RetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when an MSDU is successfully transmitted

after one or more retransmissions."

::= { dot11CountersEntry 4 }

dot11MultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when an MSDU is successfully transmitted

after more than one retransmission."

::= { dot11CountersEntry 5 }

dot11FrameDuplicateCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when a frame is received that the

Sequence Control field indicates is a duplicate."

::= { dot11CountersEntry 6 }

dot11RTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a CTS is received in response to an

RTS.

This counter increments when a CTS is received in response to an

RTS."

::= {  dot11CountersEntry 7 }

dot11RTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when a CTS is not received in response

to an RTS."

::= { dot11CountersEntry 8 }

dot11ACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when an ACK is not received when

expected."

::= {  dot11CountersEntry 9 }

dot11ReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a fragment is successfully

received.

This counter is incremented for each successfully received MPDU

of type Data or Management."

::= { dot11CountersEntry 10 }

dot11GroupReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a group addressed frame is

received.

This counter increments when a MSDU is received with the group

bit set in the destination MAC address."

::= { dot11CountersEntry 11 }

dot11FCSErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when an FCS error is detected in a

received MPDU."

::= { dot11CountersEntry 12 }

dot11TransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a frame is successfully transmit-

ted.

This counter increments for each successfully transmitted MSDU."

::= { dot11CountersEntry 13 }

dot11WEPUndecryptableCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when a frame is received with the

Protected Frame subfield of the Frame Control field set to one

and the WEPOn value for the key mapped to the transmitter's MAC

address indicates that the frame should not have been encrypted

or that frame is discarded due to the receiving STA not imple-

menting the privacy option."

::= { dot11CountersEntry 14 }

dot11QosDiscardedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments for each QoS Data MPDU that has been

discarded."

::= { dot11CountersEntry 15 }

dot11AssociatedStationCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a STA associates or disassociates.

This counter, only available at AP, increments when a station

associates or reassociates. This counter decrements when a sta-

tion disassociates."

::= { dot11CountersEntry 16 }

dot11QosCFPollsReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a CF-Poll is received.

This counter increments for each QoS (+)CF-Poll that has been

received."

::= { dot11CountersEntry 17 }

dot11QosCFPollsUnusedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a CF-Poll is unused.

This counter increments for each QoS (+)CF-Poll that has been

received but not used."

::= { dot11CountersEntry 18 }

dot11QosCFPollsUnusableCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments for each QoS (+)CF-Poll that has been

received but could not be used due to the TXOP size being

smaller than the time that is required for one frame exchange

sequence."

::= { dot11CountersEntry 19 }

dot11QoSCFPollsLostCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments for each QoS (+)CF-Poll that has been

issued where there was no response from the QoS STA."

::= { dot11CountersEntry 20 }

dot11TransmittedAMSDUCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for an acknowledged A-MSDU

frame with an individual address in the address 1 field or an A-

MSDU frame with a group  address in the address 1 field."

::= { dot11CountersEntry 21 }

dot11FailedAMSDUCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when an A-MSDU is not trans-

mitted successfully due to the number of transmit attempts

exceeding either the dot11ShortRetryLimit or

dot11LongRetryLimit."

::= { dot11CountersEntry 22 }

dot11RetryAMSDUCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when an A-MSDU is success-

fully transmitted after one or more retransmissions."

::= { dot11CountersEntry 23 }

dot11MultipleRetryAMSDUCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when an A-MSDU is success-

fully transmitted after more than one retransmission."

::= { dot11CountersEntry 24 }

dot11TransmittedOctetsInAMSDUCount OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented by the number of octets in

the framebody of an A-MSDU frame when an A-MSDU frame is suc-

cessfully transmitted."

::= { dot11CountersEntry 25 }

dot11AMSDUAckFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when an acknowledgment to an

A-MSDU is not received when expected. This acknowledgment can be

in an ACK or the BlockAck frame."

::= { dot11CountersEntry 26 }

dot11ReceivedAMSDUCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for a received A-MSDU frame

with the station's  MAC address in the address 1 field or an A-

MSDU frame with a group address in the address 1 field."

::= { dot11CountersEntry 27 }

dot11ReceivedOctetsInAMSDUCount OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented by the number of octets in

the framebody of an A-MSDU frame when an A-MSDU frame is

received."

::= { dot11CountersEntry 28 }

dot11TransmittedAMPDUCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when an A-MPDU is transmit-

ted."

::= { dot11CountersEntry 29 }

dot11TransmittedMPDUsInAMPDUCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment by the number of MPDUs in the A-

MPDU when an A-MPDU is transmitted."

::= { dot11CountersEntry 30 }

dot11TransmittedOctetsInAMPDUCount OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented by the number of octets in

the A-MPDU frame when an A-MPDU frame is transmitted."

::= { dot11CountersEntry 31 }

dot11AMPDUReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when the MAC receives an A-

MPDU from the PHY."

::= { dot11CountersEntry 32 }

dot11MPDUInReceivedAMPDUCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented by the number of MPDUs

received in the A-MPDU when an A-MPDU is received."

::= { dot11CountersEntry 33 }

dot11ReceivedOctetsInAMPDUCount OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented by the number of octets in

the A-MPDU frame when an A-MPDU frame is received."

::= { dot11CountersEntry 34 }

dot11AMPDUDelimiterCRCErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when an MPDU delimiter has a

CRC error when this is the first CRC error in the received A-

MPDU or when the previous delimiter has been decoded correctly."

::= { dot11CountersEntry 35 }

dot11ImplicitBARFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when the expected BlockAck is

not received in response to an Implicit BlockAckReq frame."

::= { dot11CountersEntry 36 }

dot11ExplicitBARFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when the expected BlockAck is

not received in response to an Explicit BlockAckReq."

::= { dot11CountersEntry 37 }

dot11ChannelWidthSwitchCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be increment when the bandwidth used is

switched from 20 to 40 or vice-versa."

::= { dot11CountersEntry 38 }

dot11TwentyMHzFrameTransmittedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when a Frame is transmitted

only on the primary channel."

::= { dot11CountersEntry 39 }

dot11FortyMHzFrameTransmittedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when a Frame is transmitted

on both control and secondary channels."

::= { dot11CountersEntry 40 }

dot11TwentyMHzFrameReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when a Frame is received only

on the primary channel."

::= { dot11CountersEntry 41 }

dot11FortyMHzFrameReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when a Frame is received on

both the control and secondary channels."

::= { dot11CountersEntry 42 }

dot11PSMPUTTGrantDuration OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter contains the cumulative duration of PSMP-UTT

granted to the STA, in units of 4 microsecond."

::= { dot11CountersEntry 43 }

dot11PSMPUTTUsedDuration OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter contains the cumulative duration of transmission

by the STA during its allocated PSMP-UTT, in units of 4 microsecond"

::= { dot11CountersEntry 44 }

dot11GrantedRDGUsedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter at the RD initiator shall be incremented when an

allocated RDG is used by the station, apart from transmitting a

response frame such as ACK or BlockAck frames."

::= { dot11CountersEntry 45 }

dot11GrantedRDGUnusedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter at the initiator shall be incremented when an

allocated RDG is not used by the station, apart from transmit-

ting a response frame such as ACK or BlockAck frames."

::= { dot11CountersEntry 46 }

dot11TransmittedFramesInGrantedRDGCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter at the initiator shall be incremented for every

frame, other than response frames such as ACK or BlockAck

frames, transmitted by the station during a granted RDG."

::= { dot11CountersEntry 47 }

dot11TransmittedOctetsInGrantedRDGCount OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter at the initiator shall be incremented by the num-

ber of octets in the framebody of a frame, other than response

frames such as ACK or BlockAck frames, transmitted by the sta-

tion during a granted RDG."

::= { dot11CountersEntry 48 }

dot11BeamformingFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when the transmitter sends a

frame with new/updated beamforming parameters."

::= { dot11CountersEntry 49 }

dot11DualCTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when AP sends a dual CTS in

response to a STA initiating TXOP in extended range."

::= { dot11CountersEntry 50 }

dot11DualCTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when AP fails to send a dual

CTS in response to a STA initiating TXOP in extended range."

::= { dot11CountersEntry 51 }

dot11STBCCTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when AP does not detect a

collision PIFS after sending a CTS to self STBC frame in

extended range."

::= { dot11CountersEntry 52 }

dot11STBCCTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when AP detects a collision

PIFS after sending a CTS to self STBC frame in extended range."

::= { dot11CountersEntry 53 }

dot11nonSTBCCTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when AP does not detect a

collision PIFS after sending a CTS to self that is an non-STBC

frame in extended range."

::= { dot11CountersEntry 54 }

dot11nonSTBCCTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when AP detects a collision

PIFS after sending a CTS to self that is an non-STBC frame in

extended range."

::= { dot11CountersEntry 55 }

dot11RTSLSIGSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when the duration/ID field is

set according to the rules of EPP in the received CTS following

a transmission of RTS in EPP mode."

::= { dot11CountersEntry 56 }

dot11RTSLSIGFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented when the duration/ID field is

not set according to the rules of EPP in the received CTS fol-

lowing a transmission of RTS in EPP mode."

::= { dot11CountersEntry 57 }

dot11PBACErrors OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"this variable indicates the number of errors encountered in the

PBAC procedures."

::= { dot11CountersEntry 58}

-- **********************************************************************

-- * End of dot11Counters TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11GroupAddresses  TABLE

-- **********************************************************************

dot11GroupAddressesTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11GroupAddressesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A conceptual table containing a set of MAC addresses identify-

ing the multicast-group addresses for which this STA will

receive frames.  The default value of this attribute is null."

::= { dot11mac 3 }

dot11GroupAddressesEntry OBJECT-TYPE

SYNTAX Dot11GroupAddressesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the Group Addresses Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11GroupAddressesIndex }

::= { dot11GroupAddressesTable  1 }

Dot11GroupAddressesEntry ::=

SEQUENCE { dot11GroupAddressesIndex    Unsigned32,

dot11Address                MacAddress,

dot11GroupAddressesStatus   RowStatus }

dot11GroupAddressesIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the Group Addresses Table."

::= { dot11GroupAddressesEntry 1 }

dot11Address OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

MAC address identifying multicast-group addresses from which

this STA will receive frames."

::= { dot11GroupAddressesEntry 2 }

dot11GroupAddressesStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the Group Addresses Table."

DEFVAL { active }

::= { dot11GroupAddressesEntry 3 }

-- **********************************************************************

-- *    End of dot11GroupAddresses TABLE

-- **********************************************************************

-- **********************************************************************

-- *    SMT EDCA Config TABLE

-- **********************************************************************

dot11EDCATable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11EDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for EDCA default parameter values at a non-AP

STA. This table contains the four entries of the EDCA parameters

corresponding to four possible ACs. Index 1 corresponds to

AC_BK, index 2 to AC_BE, index 3 to AC_VI, and index 4 to AC_VO."

REFERENCE

"IEEE 802.11-<year>, 9.1.3.1 (HCF contention-based channel access

(EDCA))"

::= { dot11mac 4 }

dot11EDCAEntry OBJECT-TYPE

SYNTAX Dot11EDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the EDCA Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11EDCATableIndex }

::= { dot11EDCATable  1 }

Dot11EDCAEntry ::=

SEQUENCE { dot11EDCATableIndex    								Unsigned32,

dot11EDCATableCWmin								Unsigned32,

dot11EDCATableCWmax								Unsigned32,

dot11EDCATableAIFSN								Unsigned32,

dot11EDCATableTXOPLimit								Unsigned32,

dot11EDCATableMSDULifetime								Unsigned32,

dot11EDCATableMandatory								TruthValue }

dot11EDCATableIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the EDCA Table. The value of this variable is

1, if the value of the AC is AC_BK.

2, if the value of the AC is AC_BE.

3, if the value of the AC is AC_VI.

4, if the value of the AC is AC_VO."

::= { dot11EDCAEntry 1 }

dot11EDCATableCWmin OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the MAC upon receiving an EDCA Parameter Set in

a Beacon frame.

Changes take effect as soon as practical in the implementation.

This attribute specifies the value of the minimum size of the

window that is used by a STA for a particular AC for generating

a random number for the backoff. The value of this attribute is

such that it could always be expressed in the form of 2^X - 1,

where X is an integer. The default value for this attribute is

aCWmin, if dot11EDCATableIndex is 1 or 2.

(aCWmin+1)/2 - 1, if dot11EDCATableIndex is 3.

(aCWmin+1)/4 - 1, if dot11EDCATableIndex is 4."

::= { dot11EDCAEntry 2 }

dot11EDCATableCWmax OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the MAC upon receiving an EDCA Parameter Set in

a Beacon frame.

Changes take effect as soon as practical in the implementation.

This attribute specifies the value of the maximum size of the

window that is used by a STA for a particular AC for generating

a random number for the backoff. The value of this attribute is

such that it could always be expressed in the form of 2^X - 1,

where X is an integer. The default value for this attribute is

aCWmax, if dot11EDCATableIndex is 1 or 2.

aCWmin, if dot11EDCATableIndex is 3.

(aCWmin+1)/2 - 1, if dot11EDCATableIndex is 4."

::= { dot11EDCAEntry 3 }

dot11EDCATableAIFSN OBJECT-TYPE

SYNTAX Unsigned32 (2..15)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the MAC upon receiving an EDCA Parameter Set in

a Beacon frame.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of slots, after a SIFS dura-

tion, that the STA, for a particular AC, senses the medium idle

either before transmitting or executing a backoff. The default

value for this attribute is

7, if dot11EDCATableIndex is 1,

3, if dot11EDCATableIndex is 2

2, otherwise."

::= { dot11EDCAEntry 4 }

dot11EDCATableTXOPLimit OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the MAC upon receiving an EDCA Parameter Set in

a Beacon frame.

Changes take effect as soon as practical in the implementation.

This attribute specifies the maximum number of microseconds of

an EDCA TXOP for a given AC. The default value for this

attribute is

1)  0 for all PHYs, if dot11EDCATableIndex is 1 or 2; this

implies that the sender can send one MSDU in an EDCA TXOP,

2)  3008 microseconds for Clause 17 (Orthogonal frequency divi-

sion multiplexing (OFDM) PHY specification (11y)) and Clause 19

(Extended Rate PHY (ERP) specification(#1468)(#1729)) PHY and

6016 microseconds for Clause 18 (High Rate direct sequence

spread spectrum (HR/DSSS) PHY specification) PHY, if

dot11EDCATableIndex is 3,

3)  1504 microseconds for Clause 17 (Orthogonal frequency divi-

sion multiplexing (OFDM) PHY specification (11y)) and Clause 19

(Extended Rate PHY (ERP) specification(#1468)(#1729)) PHY and

3264 microseconds for Clause 18 (High Rate direct sequence

spread spectrum (HR/DSSS) PHY specification) PHY, if

dot11EDCATableIndex is 4."

::= { dot11EDCAEntry 5 }

dot11EDCATableMSDULifetime OBJECT-TYPE

SYNTAX Unsigned32 (0..500)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the MAC upon receiving an EDCA Parameter Set in

a Beacon frame.

Changes take effect as soon as practical in the implementation.

This attribute specifies (in TUs) the maximum duration an MSDU,

for a given AC, would be retained by the MAC before it is dis-

carded."

DEFVAL { 500 }

::= { dot11EDCAEntry 6 }

dot11EDCATableMandatory OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the MAC upon receiving an EDCA Parameter Set in

a Beacon frame.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that admission control is

mandatory for the given AC. When false, this attribute indicates

that the admission control is not mandatory for the given AC."

DEFVAL { false }

::= { dot11EDCAEntry 7 }

-- **********************************************************************

-- *    End of SMT EDCA Config TABLE

-- **********************************************************************

-- **********************************************************************

-- *    SMT AP EDCA Config TABLE

-- **********************************************************************

dot11QAPEDCATable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11QAPEDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for EDCA default parameter values at the AP.

This table contains the four entries of the EDCA parameters cor-

responding to four possible ACs. Index 1 corresponds to AC_BK,

index 2 to AC_BE, index 3 to AC_VI, and index 4 to AC_VO."

REFERENCE

"IEEE 802.11-<year>, 9.9.1 (HCF contention-based channel access

(EDCA))"

::= { dot11mac 5 }

dot11QAPEDCAEntry OBJECT-TYPE

SYNTAX Dot11QAPEDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the EDCA Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11QAPEDCATableIndex }

::= { dot11QAPEDCATable  1 }

Dot11QAPEDCAEntry ::=

SEQUENCE { dot11QAPEDCATableIndex   		 								Unsigned32,

dot11QAPEDCATableCWmin										Unsigned32,

dot11QAPEDCATableCWmax										Unsigned32,

dot11QAPEDCATableAIFSN										Unsigned32,

dot11QAPEDCATableTXOPLimit										Unsigned32,

dot11QAPEDCATableMSDULifetime										Unsigned32,

dot11QAPEDCATableMandatory								TruthValue }

dot11QAPEDCATableIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the EDCA Table. The value of this variable is

1, if the value of the AC is AC_BK.

2, if the value of the AC is AC_BE.

3, if the value of the AC is AC_VI.

4, if the value of the AC is AC_VO."

::= { dot11QAPEDCAEntry 1 }

dot11QAPEDCATableCWmin OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the value of the minimum size of the

window that is used by an AP for a particular AC for generating

a random number for the backoff. The value of this attribute is

such that it could always be expressed in the form of 2^X - 1,

where X is an integer. The default value for this attribute is

aCWmin, if dot11QAPEDCATableIndex is 1 or 2.

(aCWmin+1)/2 - 1, if dot11QAPEDCATableIndex is 3.

(aCWmin+1)/4 - 1, if dot11QAPEDCATableIndex is 4."

::= { dot11QAPEDCAEntry 2 }

dot11QAPEDCATableCWmax OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the value of the maximum size of the

window that is used by an AP for a particular AC for generating

a random number for the backoff. The value of this attribute is

such that it could always be expressed in the form of 2^X - 1,

where X is an integer. The default value for this attribute is

aCWmax, if dot11QAPEDCATableIndex is 1.

4*(aCWmin+1) - 1, if dot11QAPEDCATableIndex is 2.

aCWmin, if dot11QAPEDCATableIndex is 3.

(aCWmin+1)/2 - 1, if dot11QAPEDCATableIndex is 4."

::= { dot11QAPEDCAEntry 3 }

dot11QAPEDCATableAIFSN OBJECT-TYPE

SYNTAX Unsigned32 (1..15)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of slots, after a SIFS dura-

tion, that the AP, for a particular AC, senses the medium idle

either before transmitting or executing a backoff. The default

value for this attribute is

7, if dot11QAPEDCATableIndex is 1,

3, if dot11QAPEDCATableIndex is 2

1, otherwise."

::= { dot11QAPEDCAEntry 4 }

dot11QAPEDCATableTXOPLimit OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the maximum number of microseconds of

an EDCA TXOP for a given AC at the AP. The default value for this

attribute is

1)  0 for all PHYs, if dot11QAPEDCATableIndex is 1 or 2; this

implies that the sender can send one MSDU in an EDCA TXOP,

2)  3008 microseconds for Clause 17 (Orthogonal frequency divi-

sion multiplexing (OFDM) PHY specification (11y)) and Clause 19

(Extended Rate PHY (ERP) specification(#1468)(#1729)) PHY and

6016 microseconds for Clause 18 (High Rate direct sequence

spread spectrum (HR/DSSS) PHY specification) PHY, if

dot11QAPEDCATableIndex is 3,

3)  1504 microseconds for Clause 17 (Orthogonal frequency divi-

sion multiplexing (OFDM) PHY specification (11y)) and Clause 19

(Extended Rate PHY (ERP) specification(#1468)(#1729)) PHY and

3264 microseconds for Clause 18 (High Rate direct sequence

spread spectrum (HR/DSSS) PHY specification) PHY, if

dot11QAPEDCATableIndex is 4."

::= { dot11QAPEDCAEntry 5 }

dot11QAPEDCATableMSDULifetime OBJECT-TYPE

SYNTAX Unsigned32 (0..500)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies (in TUs) the maximum duration an MSDU,

for a given AC, would be retained by the MAC at the AP before it

is discarded."

DEFVAL { 500 }

::= { dot11QAPEDCAEntry 6 }

dot11QAPEDCATableMandatory OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that admission control is

mandatory for the given AC. When false, this attribute indicates

that the admission control is not mandatory for the given AC.

The default value for this parameter is false."

::= { dot11QAPEDCAEntry 7 }

-- **********************************************************************

-- *    End of SMT AP EDCA Config TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11QosCounters TABLE

-- **********************************************************************

dot11QosCountersTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11QosCountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes that are MAC counters implemented

as a table to allow for multiple instantiations on an agent."

::= { dot11mac 6 }

dot11QosCountersEntry OBJECT-TYPE

SYNTAX Dot11QosCountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the EDCA Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11QosCountersIndex }

::= { dot11QosCountersTable 1 }

Dot11QosCountersEntry ::=

SEQUENCE { dot11QosCountersIndex    											Unsigned32,

dot11QosTransmittedFragmentCount											Counter32,

dot11QosFailedCount											Counter32,

dot11QosRetryCount											Counter32,

dot11QosMultipleRetryCount											Counter32,

dot11QosFrameDuplicateCount											Counter32,

dot11QosRTSSuccessCount											Counter32,

dot11QosRTSFailureCount											Counter32,

dot11QosACKFailureCount											Counter32,

dot11QosReceivedFragmentCount											Counter32,

dot11QosTransmittedFrameCount											Counter32,

dot11QosDiscardedFrameCount											Counter32,

dot11QosMPDUsReceivedCount											Counter32,

dot11QosRetriesReceivedCount											Counter32}

dot11QosCountersIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..16)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the QoSCounter Table. The value of this variable

is equal to TID + 1."

::= { dot11QosCountersEntry 1 }

dot11QosTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a QoS fragment is transmitted.

This counter is incremented for an acknowledged MPDU, for a par-

ticular UP, with an individual address in the address 1 field or

an MPDU with a group address in the address 1 field, either

belonging to a particular TID. This counter has relevance only

for TIDs between 0 and 7."

::= { dot11QosCountersEntry 2 }

dot11QosFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when an MSDU, for a particular UP, is

not transmitted successfully due to the number of transmit

attempts exceeding either the dot11ShortRetryLimit or

dot11LongRetryLimit. This counter has relevance only for TIDs

between 0 and 7."

::= { dot11QosCountersEntry 3 }

dot11QosRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when an MSDU, of a particular UP, is

successfully transmitted after one or more retransmissions. This

counter has relevance only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 4 }

dot11QosMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when an MSDU, of a particular UP, is

successfully transmitted after more than one retransmissions.

This counter has relevance only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 5 }

dot11QosFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when a frame, of a particular UP, is

received that the Sequence Control field indicates is a dupli-

cate. This counter has relevance only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 6 }

dot11QosRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a CTS is received in response to an

RTS.

This counter increments when a CTS is received in response to an

RTS that has been sent for the transmission of an MPDU of a par-

ticular UP. This counter has relevance only for TIDs between 0

and 7."

::= { dot11QosCountersEntry 7 }

dot11QosRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when a CTS is not received in response

to an RTS that has been sent for the transmission of an MPDU of

a particular UP. This counter has relevance only for TIDs

between 0 and 7."

::= { dot11QosCountersEntry 8 }

dot11QosACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments when an ACK is not received in response

to an MPDU of a particular UP. This counter has relevance only

for TIDs between 0 and 7."

::= { dot11QosCountersEntry 9 }

dot11QosReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a QoS fragment is received.

This counter is incremented for each successfully received MPDU

of type Data of a particular UP. This counter has relevance only

for TIDs between 0 and 7."

::= { dot11QosCountersEntry 10 }

dot11QosTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a QoS frame is transmitted.

This counter increments for each successfully transmitted MSDU

of a particular UP. This counter has relevance only for TIDs

between 0 and 7."

::= { dot11QosCountersEntry 11 }

dot11QosDiscardedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments for each Discarded MSDU of a particular

UP. This counter has relevance only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 12 }

dot11QosMPDUsReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when a QoS MPDU is received.

This counter increments for each received MPDU of a particular

TID."

::= { dot11QosCountersEntry 13 }

dot11QosRetriesReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when the condition described below

occurs.

This counter increments for each received MPDU of a particular

TID with the retry bit set to 1."

::= { dot11QosCountersEntry 14 }

-- **********************************************************************

-- *    End of dot11QosCounters TABLE

-- **********************************************************************

-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

dot11ResourceTypeIDName OBJECT-TYPE

SYNTAX DisplayString (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Contains the name of the Resource Type ID managed object. The

attribute is read-only and always contains the value RTID.  This

attribute value is not used as a naming attribute for any other

managed object class."

REFERENCE "IEEE Std 802.1F-1993,  A.7"

DEFVAL { "RTID" }

::= { dot11resAttribute 1 }

-- **********************************************************************

-- *    dot11ResourceInfo  TABLE

-- **********************************************************************

dot11ResourceInfoTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11ResourceInfoEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Provides a means of indicating, in data readable from a managed

object, information that identifies the source of the implemen-

tation."

REFERENCE "IEEE Std 802.1F-1993,  A.7"

::= { dot11resAttribute 2 }

dot11ResourceInfoEntry OBJECT-TYPE

SYNTAX Dot11ResourceInfoEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11ResourceInfo Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11ResourceInfoTable 1 }

Dot11ResourceInfoEntry ::=

SEQUENCE { dot11manufacturerOUI             OCTET STRING,

dot11manufacturerName            DisplayString,

dot11manufacturerProductName     DisplayString,

dot11manufacturerProductVersion  DisplayString }

dot11manufacturerOUI OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Takes the value of an organizationally unique identifier."

::= { dot11ResourceInfoEntry 1 }

dot11manufacturerName OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

A printable string used to identify the manufacturer of the

resource.  Maximum string length is 128 octets."

::= { dot11ResourceInfoEntry 2 }

dot11manufacturerProductName OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

A printable string used to identify the manufacturer's product

name of the resource.  Maximum string length is 128 octets."

::= { dot11ResourceInfoEntry 3 }

dot11manufacturerProductVersion OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Printable string used to identify the manufacturer's product

version of the resource.  Maximum string length is 128 octets."

::= { dot11ResourceInfoEntry 4 }

-- **********************************************************************

-- * End of dot11ResourceInfo  TABLE

-- **********************************************************************

-- **********************************************************************

-- * PHY Attribute Templates

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyOperation  TABLE

-- **********************************************************************

dot11PhyOperationTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"PHY level attributes concerned with operation.  Implemented as

a table indexed on ifIndex to allow for multiple instantiations

on an Agent."

::= { dot11phy 1 }

dot11PhyOperationEntry OBJECT-TYPE

SYNTAX Dot11PhyOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyOperation Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11PhyOperationTable 1 }

Dot11PhyOperationEntry ::=

SEQUENCE { dot11PHYType            INTEGER,

dot11CurrentRegDomain   Unsigned32,

dot11TempType           INTEGER }

dot11PHYType OBJECT-TYPE

SYNTAX INTEGER { fhss(1), dsss(2), irbaseband(3), ofdm(4),

hrdsss(5), erp(6), ht (7)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the PHY.

This is an 8-bit integer value that identifies the PHY type sup-

ported by the attached PLCP and PMD. Currently defined values

and their corresponding PHY types are:

FHSS 2.4 GHz = 01, DSSS 2.4 GHz = 02, IR Baseband = 03,

OFDM = 04, HRDSSS = 05, ERP = 06, HT = 07"

::= { dot11PhyOperationEntry 1 }

dot11CurrentRegDomain OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

The current regulatory domain this instance of the PMD is sup-

porting.  This object corresponds to one of the RegDomains

listed in dot11RegDomainsSupported."

::= { dot11PhyOperationEntry 2 }

dot11TempType OBJECT-TYPE

SYNTAX INTEGER { tempType1(1), tempType2(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the PHY.

There are different operating temperature requirements dependent

on the anticipated environmental conditions. This attribute

describes the current PHY's operating temperature range capabil-

ity. Currently defined values and their corresponding tempera-

ture ranges are:

Type 1 = X'01'-Commercial range of 0 to 40 degrees C,

Type 2 = X'02'-Industrial range of -30 to 70 degrees C."

::= { dot11PhyOperationEntry 3 }

-- **********************************************************************

-- * End of dot11PhyOperation  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyAntenna  TABLE

-- **********************************************************************

dot11PhyAntennaTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyAntennaEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for PhyAntenna.  Implemented as a table

indexed on ifIndex to allow for multiple instances on an agent."

::= { dot11phy 2}

dot11PhyAntennaEntry OBJECT-TYPE

SYNTAX Dot11PhyAntennaEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyAntenna Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11PhyAntennaTable 1 }

Dot11PhyAntennaEntry ::=

SEQUENCE {

dot11CurrentTxAntenna	Unsigned32,

dot11DiversitySupportImplemented	INTEGER,

dot11CurrentRxAntenna  	Unsigned32,

dot11AntennaSelectionOptionImplemented 	TruthValue,

dot11TransmitExplicitCSIFeedbackASOptionImplemented	TruthValue,

dot11TransmitIndicesFeedbackASOptionImplemented	TruthValue,

dot11ExplicitCSIFeedbackASOptionImplemented	TruthValue,

dot11TransmitIndicesComputationASOptionImplemented	TruthValue,

dot11ReceiveAntennaSelectionOptionImplemented	TruthValue,

dot11TransmitSoundingPPDUOptionImplemented	TruthValue,

dot11NumberOfActiveRxAntennas	Unsigned32 }

dot11CurrentTxAntenna OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The current antenna being used to transmit.  This value is one

of the values appearing in dot11TxAntennaImplemented. This may

be used by a management agent to control which antenna is used

for transmission. "

::= { dot11PhyAntennaEntry 1 }

dot11DiversitySupportImplemented OBJECT-TYPE

SYNTAX INTEGER { fixedlist(1), notsupported(2), dynamic(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This implementation's support for diversity, encoded as:

X'01'-diversity is available and is performed over the fixed

list of antennas defined in

dot11DiversitySelectionRxImplemented.

X'02'-diversity is not supported.

X'03'-diversity is supported and control of diversity is also

available, in which case the attribute

dot11DiversitySelectionRxImplemented can be dynamically modified

by the LME."

::= { dot11PhyAntennaEntry 2 }

dot11CurrentRxAntenna OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the PHY.

The current antenna being used to receive, if the dot11 Diversi-

tySupport indicates that diversity is not supported.  The

selected antenna is one of the antennae marked for receive in

the dot11AntennasListTable."

::= { dot11PhyAntennaEntry 3 }

dot11AntennaSelectionOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that ASEL is supported by

the station implementation. "

DEFVAL { false }

::= { dot11PhyAntennaEntry 4 }

dot11TransmitExplicitCSIFeedbackASOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the transmit ASEL

based on explicit CSI feedback is supported by the station

implementation. "

DEFVAL { false }

::= { dot11PhyAntennaEntry 5 }

dot11TransmitIndicesFeedbackASOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the transmit ASEL

based on antenna indices feedback is supported by the station

implementation."

DEFVAL { false }

::= { dot11PhyAntennaEntry 6 }

dot11ExplicitCSIFeedbackASOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the computation of

CSI and feedback the results to support the peer to do ASEL is

supported by the station implementation."

DEFVAL { false }

::= { dot11PhyAntennaEntry 7 }

dot11TransmitIndicesComputationASOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the transmit ASEL

based on antenna indices selection computation and feedback the

results to support the peer to do ASEL is supported by the sta-

tion implementation."

DEFVAL { false }

::= { dot11PhyAntennaEntry 8 }

dot11ReceiveAntennaSelectionOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the receive ASEL is

supported by the station implementation. "

DEFVAL { false }

::= { dot11PhyAntennaEntry 9 }

dot11TransmitSoundingPPDUOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the transmission of

sounding PPDUs is supported by the station implementation. "

DEFVAL { false }

::= { dot11PhyAntennaEntry 10 }

dot11NumberOfActiveRxAntennas OBJECT-TYPE

SYNTAX Unsigned32 (1..4)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"his attribute indicates the number of current active antennas

being used to receive."

::= { dot11PhyAntennaEntry 11 }

-- **********************************************************************

-- * End of dot11PhyAntenna  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyTxPower  TABLE

-- **********************************************************************

dot11PhyTxPowerTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyTxPowerEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyTxPowerTable.  Implemented as a

table indexed on STA ID to allow for multiple instances on an

Agent."

::= { dot11phy 3}

dot11PhyTxPowerEntry OBJECT-TYPE

SYNTAX Dot11PhyTxPowerEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyTxPower Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11PhyTxPowerTable 1 }

Dot11PhyTxPowerEntry ::=

SEQUENCE { dot11NumberSupportedPowerLevelsImplemented  Unsigned32,

dot11TxPowerLevel1               Unsigned32,

dot11TxPowerLevel2               Unsigned32,

dot11TxPowerLevel3               Unsigned32,

dot11TxPowerLevel4               Unsigned32,

dot11TxPowerLevel5               Unsigned32,

dot11TxPowerLevel6               Unsigned32,

dot11TxPowerLevel7               Unsigned32,

dot11TxPowerLevel8               Unsigned32,

dot11CurrentTxPowerLevel         Unsigned32 }

dot11NumberSupportedPowerLevelsImplemented OBJECT-TYPE

SYNTAX Unsigned32 (1..8)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The number of power levels supported by the PMD. This attribute

can have a value of 1 to 8."

::= { dot11PhyTxPowerEntry 1 }

dot11TxPowerLevel1 OBJECT-TYPE

SYNTAX Unsigned32 (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The transmit output power for LEVEL1 in mW. This is also the

default power level."

::= { dot11PhyTxPowerEntry 2 }

dot11TxPowerLevel2 OBJECT-TYPE

SYNTAX Unsigned32 (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The transmit output power for LEVEL2 in mW."

::= { dot11PhyTxPowerEntry 3 }

dot11TxPowerLevel3 OBJECT-TYPE

SYNTAX Unsigned32 (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The transmit output power for LEVEL3 in mW."

::= { dot11PhyTxPowerEntry 4 }

dot11TxPowerLevel4 OBJECT-TYPE

SYNTAX Unsigned32 (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The transmit output power for LEVEL4 in mW."

::= { dot11PhyTxPowerEntry 5 }

dot11TxPowerLevel5 OBJECT-TYPE

SYNTAX Unsigned32 (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The transmit output power for LEVEL5 in mW."

::= { dot11PhyTxPowerEntry 6 }

dot11TxPowerLevel6 OBJECT-TYPE

SYNTAX Unsigned32 (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The transmit output power for LEVEL6 in mW."

::= { dot11PhyTxPowerEntry 7 }

dot11TxPowerLevel7 OBJECT-TYPE

SYNTAX Unsigned32 (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The transmit output power for LEVEL7 in mW."

::= { dot11PhyTxPowerEntry 8 }

dot11TxPowerLevel8 OBJECT-TYPE

SYNTAX Unsigned32 (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The transmit output power for LEVEL8 in mW."

::= { dot11PhyTxPowerEntry 9 }

dot11CurrentTxPowerLevel OBJECT-TYPE

SYNTAX Unsigned32 (1..8)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the PHY.

The TxPowerLevel N currently being used to transmit data. Some

PHYs also use this value to determine the receiver sensitivity

requirements for CCA."

::= { dot11PhyTxPowerEntry 10 }

-- **********************************************************************

-- * End of dot11PhyTxPower  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyFHSS  TABLE

-- **********************************************************************

dot11PhyFHSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyFHSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyFHSSTable.  Implemented as a

table indexed on STA ID to allow for multiple instances on an

Agent."

::= { dot11phy 4 }

dot11PhyFHSSEntry OBJECT-TYPE

SYNTAX Dot11PhyFHSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyFHSS Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11PhyFHSSTable 1 }

Dot11PhyFHSSEntry ::=

SEQUENCE { dot11HopTime											Unsigned32,

dot11CurrentChannelNumber											Unsigned32,

dot11MaxDwellTime											Unsigned32,

dot11CurrentDwellTime											Unsigned32,

dot11CurrentSet											Unsigned32,

dot11CurrentPattern											Unsigned32,

dot11CurrentIndex											Unsigned32,

dot11EHCCPrimeRadix											Integer32,

dot11EHCCNumberofChannelsFamilyIndex											Integer32,

dot11EHCCCapabilityImplemented											TruthValue,

dot11EHCCCapabilityActivated											TruthValue,

dot11HopAlgorithmAdopted											INTEGER,

dot11RandomTableFlag											TruthValue,

dot11NumberofHoppingSets											Unsigned32,

dot11HopModulus											Unsigned32,

dot11HopOffset											Unsigned32 }

dot11HopTime OBJECT-TYPE

SYNTAX Unsigned32 (224)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The time in microseconds for the PMD to change from channel 2 to

channel 80."

::= { dot11PhyFHSSEntry 1 }

dot11CurrentChannelNumber OBJECT-TYPE

SYNTAX Unsigned32 (0..200)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the PHY.

The current channel number of the frequency output by the RF

synthesizer."

::= { dot11PhyFHSSEntry 2 }

dot11MaxDwellTime OBJECT-TYPE

SYNTAX Unsigned32 (1..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

The maximum time in TU that the transmitter is permitted to

operate on a single channel."

::= { dot11PhyFHSSEntry 3 }

dot11CurrentDwellTime OBJECT-TYPE

SYNTAX Unsigned32 (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

In a non-AP STA, it is written by the MAC when it receives an FH

Parameter Set element, and changes take effect as soon as prac-

tical in the implementation.

In an AP, it is written by an external management entity, and

changes take effect for the next Beacon.

The current time in TU that the transmitter operates on a single

channel, as set by the MAC."

DEFVAL { 19 }

::= { dot11PhyFHSSEntry 4 }

dot11CurrentSet OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The current set of patterns the PLME is using to determine the

hopping sequence. "

::= { dot11PhyFHSSEntry 5 }

dot11CurrentPattern OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The current pattern the PLME is using to determine the hop

sequence."

::= { dot11PhyFHSSEntry 6 }

dot11CurrentIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the PHY.

The current index value the PLME is using to determine the Cur-

rentChannelNumber."

::= { dot11PhyFHSSEntry 7 }

dot11EHCCPrimeRadix OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the value to be used as the prime radix

(N) in the HCC and EHCC algorithms."

::= { dot11PhyFHSSEntry 8 }

dot11EHCCNumberofChannelsFamilyIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute indicates the value to be used as the maximum for

the family index (a) in the HCC and EHCC algorithms. The value

of this field is not less than the prime radix minus 3 (N - 3).

The valid range of allowed values is (N - 1), (N - 2), and (N -

3)."

::= { dot11PhyFHSSEntry 9 }

dot11EHCCCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implemen-

tation is capable of generating the HCC or EHCC algorithms for

determining Hopping patterns.  The capability is disabled, oth-

erwise."

DEFVAL { false }

::= { dot11PhyFHSSEntry 10 }

dot11EHCCCapabilityActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that the capability of the

station to operate using the HCC or EHCC algorithms for deter-

mining Hopping Patterns is enabled.  The capability is disabled,

otherwise."

DEFVAL { false }

::= { dot11PhyFHSSEntry 11 }

dot11HopAlgorithmAdopted OBJECT-TYPE

SYNTAX INTEGER { crnt(1), hopindex(2), hcc(3) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, indicates which of the algorithms will be used

to generate the Hopping Patterns.

Valid values are:

1 - hopping patterns as defined in Clause 14 (Frequency-Hopping

spread spectrum (FHSS) PHY specification for the 2.4 GHz indus-

trial, scientific, and medical (ISM) band)

2 - hop index method (with or without table)

3 - HCC/EHCC method"

::= { dot11PhyFHSSEntry 12 }

dot11RandomTableFlag OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, indicates that a Random Table is present when

the value is true. When the value is false it indicates that a

Random Table is not present and that the hop index method is to

be used to determine the hopping sequence."

DEFVAL { true }

::= { dot11PhyFHSSEntry 13 }

dot11NumberofHoppingSets OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

The Number of Sets field indicates the total number of sets

within the hopping patterns."

::= { dot11PhyFHSSEntry 14 }

dot11HopModulus OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

The number of allowed channels for the hopping set. This is

defined by the governing regulatory agency for the country code

of the country in which this device is operating."

::= { dot11PhyFHSSEntry 15 }

dot11HopOffset OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the PHY.

The next position in the hopping set."

::= { dot11PhyFHSSEntry 16 }

-- **********************************************************************

-- * End of dot11PhyFHSS  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyDSSSEntry  TABLE

-- **********************************************************************

dot11PhyDSSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyDSSSEntry.  Implemented as a

table indexed on ifIndex to allow for multiple instances on an

Agent."

::= { dot11phy 5 }

dot11PhyDSSSEntry OBJECT-TYPE

SYNTAX Dot11PhyDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyDSSSEntry Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11PhyDSSSTable 1 }

Dot11PhyDSSSEntry ::=

SEQUENCE { dot11CurrentChannel    Unsigned32,

dot11CCAModeSupported  Unsigned32,

dot11CurrentCCAMode    INTEGER,

dot11EDThreshold       Integer32 }

dot11CurrentChannel OBJECT-TYPE

SYNTAX Unsigned32 (1..14)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the PHY.

The current operating frequency channel of the DSSS PHY. Valid

channel numbers are as defined in 15.4.6.2 (Channel Number-

ing(#1055) of operating channels)"

::= { dot11PhyDSSSEntry 1 }

dot11CCAModeSupported OBJECT-TYPE

SYNTAX Unsigned32 (1..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

dot11CCAModeSupported is a bit-significant value, representing

all of the CCA modes supported by the PHY. Valid values are:

energy detect only (ED_ONLY) = 01,

carrier sense only (CS_ONLY) = 02,

carrier sense and energy detect (ED_and_CS)= 04

This attribute is not used to indicate the CCA modes supported

by a higher rate extension PHY.  Rather, the

dot11HRCCAModeImplemented attribute is used to indicate the CCA

modes of the higher rate extension PHY."

::= { dot11PhyDSSSEntry 2 }

dot11CurrentCCAMode OBJECT-TYPE

SYNTAX INTEGER { edonly(1), csonly(2), edandcs(4), cswithtimer(8),

hrcsanded(16) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The current CCA method in operation.   Valid values are:

energy detect only (edonly) = 01,

carrier sense only (csonly) = 02,

carrier sense and energy detect (edandcs)= 04

carrier sense with timer (cswithtimer)= 08

high rate carrier sense and energy detect (hrcsanded)=16."

::= { dot11PhyDSSSEntry 3 }

dot11EDThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The current Energy Detect Threshold being used by the DSSS PHY."

::= { dot11PhyDSSSEntry 4 }

-- **********************************************************************

-- * End of dot11PhyDSSSEntry  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyIR  TABLE

-- **********************************************************************

dot11PhyIRTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyIREntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyIRTable.  Implemented as a

table indexed on ifIndex to allow for multiple instances on an

Agent."

::= { dot11phy 6 }

dot11PhyIREntry OBJECT-TYPE

SYNTAX Dot11PhyIREntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyIR Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11PhyIRTable 1 }

Dot11PhyIREntry ::=

SEQUENCE { dot11CCAWatchdogTimerMax       Unsigned32,

dot11CCAWatchdogCountMax       Unsigned32,

dot11CCAWatchdogTimerMin       Unsigned32,

dot11CCAWatchdogCountMin       Unsigned32 }

dot11CCAWatchdogTimerMax OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This parameter, together with CCAWatchdogCountMax, determines

when energy detected in the channel can be ignored."

::= { dot11PhyIREntry 1 }

dot11CCAWatchdogCountMax OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This parameter, together with CCAWatchdogTimerMax, determines

when energy detected in the channel can be ignored."

::= { dot11PhyIREntry 2 }

dot11CCAWatchdogTimerMin OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The minimum value to which CCAWatchdogTimerMax can be set."

::= { dot11PhyIREntry 3 }

dot11CCAWatchdogCountMin OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The minimum value to which CCAWatchdogCount can be set."

::= { dot11PhyIREntry 4 }

-- **********************************************************************

-- * End of dot11PhyIR  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11RegDomainsSupported  TABLE

-- **********************************************************************

dot11RegDomainsSupportedTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RegDomainsSupportedEntry

MAX-ACCESS not-accessible

STATUS deprecated

DESCRIPTION

"There are different operational requirements dependent on the

regulatory domain.  This attribute list describes the regulatory

domains the PLCP and PMD support in this implementation.  Cur-

rently defined values and their corresponding Regulatory Domains

are:

FCC (USA) = X'10', DOC (Canada) = X'20', ETSI (most of Europe) =

X'30', Spain = X'31', France = X'32', Japan = X'40', China =

X'50', Other = X'00' "

::= { dot11phy 7}

dot11RegDomainsSupportedEntry OBJECT-TYPE

SYNTAX Dot11RegDomainsSupportedEntry

MAX-ACCESS not-accessible

STATUS deprecated

DESCRIPTION

"An entry in the dot11RegDomainsSupportedTable.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11RegDomainsSupportedIndex }

::= { dot11RegDomainsSupportedTable 1 }

Dot11RegDomainsSupportedEntry ::=

SEQUENCE { dot11RegDomainsSupportedIndex    Unsigned32,

dot11RegDomainsImplementedValue    INTEGER }

dot11RegDomainsSupportedIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS deprecated

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the RegDomainsSupport Table."

::= { dot11RegDomainsSupportedEntry 1 }

dot11RegDomainsImplementedValue OBJECT-TYPE

SYNTAX INTEGER { other (0), fcc(16), doc(32), etsi(48), spain (49),

france(50), japan (64)}

MAX-ACCESS read-only

STATUS deprecated

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

There are different operational requirements dependent on the

regulatory domain.  This attribute list describes the regulatory

domains the PLCP and PMD support in this implementation.  Cur-

rently defined values and their corresponding Regulatory Domains

are:

FCC (USA) = X'10', DOC (Canada) = X'20', ETSI (most of Europe) =

X'30', Spain = X'31', France = X'32', Japan = X'40', China =

X'50' "

::= { dot11RegDomainsSupportedEntry 2 }

-- **********************************************************************

-- * End of dot11RegDomainsSupported  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11AntennasList  TABLE

-- **********************************************************************

dot11AntennasListTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11AntennasListEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table represents the list of antennae.  An antenna can be

marked to be capable of transmitting, receiving, and/or for par-

ticipation in receive diversity.  Each entry in this table rep-

resents a single antenna with its properties.  The maximum

number of antennae that can be contained in this table is 255."

::= { dot11phy 8 }

dot11AntennasListEntry OBJECT-TYPE

SYNTAX Dot11AntennasListEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11AntennasListTable, representing the prop-

erties of a single antenna.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11AntennaListIndex }

::= { dot11AntennasListTable 1 }

Dot11AntennasListEntry ::=

SEQUENCE { dot11AntennaListIndex     Unsigned32,

dot11TxAntennaImplemented   TruthValue,

dot11RxAntennaImplemented   TruthValue,

dot11DiversitySelectionRxImplemented TruthValue }

dot11AntennaListIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The unique index of an antenna which is used to identify the

columnar objects in the dot11AntennasList Table."

::= { dot11AntennasListEntry 1 }

dot11TxAntennaImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

When true, this object indicates that the antenna represented by

dot11AntennaIndex can be used as a transmit antenna."

::= { dot11AntennasListEntry 2 }

dot11RxAntennaImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

When true, this object indicates that the antenna represented by

the dot11AntennaIndex can be used as a receive antenna."

::= { dot11AntennasListEntry 3 }

dot11DiversitySelectionRxImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

When true, this object indicates that the antenna represented by

dot11AntennaIndex can be used for receive diversity.  This

object is true only if the antenna can be used as a receive

antenna, as indicated by dot11RxAntennaImplemented."

::= { dot11AntennasListEntry 4 }

-- **********************************************************************

-- * End of dot11AntennasList  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11SupportedDataRatesTx  TABLE

-- **********************************************************************

dot11SupportedDataRatesTxTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SupportedDataRatesTxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The Transmit bit rates supported by the PLCP and PMD, repre-

sented by a count from X'02-X'7f, corresponding to data rates in

increments of 500kbit/s from 1 Mb/s to 63.5 Mb/s subject to lim-

itations of each individual PHY."

::= { dot11phy 9 }

dot11SupportedDataRatesTxEntry OBJECT-TYPE

SYNTAX Dot11SupportedDataRatesTxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the dot11SupportedDataRatesTx

Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11SupportedDataRatesTxIndex }

::= { dot11SupportedDataRatesTxTable  1 }

Dot11SupportedDataRatesTxEntry ::=

SEQUENCE { dot11SupportedDataRatesTxIndex  Unsigned32,

dot11ImplementedDataRatesTxValue  Unsigned32 }

dot11SupportedDataRatesTxIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index object that identifies which data rate to access. Range

is 1..255."

::= { dot11SupportedDataRatesTxEntry 1 }

dot11ImplementedDataRatesTxValue OBJECT-TYPE

SYNTAX Unsigned32 (2..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The Transmit bit rates supported by the PLCP and PMD, repre-

sented by a count from X'02-X'7f, corresponding to data rates in

increments of 500kbit/s from 1 Mb/s to 63.5 Mb/s subject to lim-

itations of each individual PHY."

::= { dot11SupportedDataRatesTxEntry 2 }

-- **********************************************************************

-- * End of dot11SupportedDataRatesTx  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11SupportedDataRatesRx  TABLE

-- **********************************************************************

dot11SupportedDataRatesRxTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SupportedDataRatesRxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The receive bit rates supported by the PLCP and PMD, repre-

sented by a count from X'002-X'7f, corresponding to data rates

in increments of 500kbit/s from 1 Mb/s to 63.5 Mb/s."

::= { dot11phy 10 }

dot11SupportedDataRatesRxEntry OBJECT-TYPE

SYNTAX Dot11SupportedDataRatesRxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the dot11SupportedDataRatesRx

Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try.  Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex,

dot11SupportedDataRatesRxIndex }

::= { dot11SupportedDataRatesRxTable  1 }

Dot11SupportedDataRatesRxEntry ::=

SEQUENCE { dot11SupportedDataRatesRxIndex  Unsigned32,

dot11ImplementedDataRatesRxValue  Unsigned32 }

dot11SupportedDataRatesRxIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index object that identifies which data rate to access. Range

is 1..255."

::= { dot11SupportedDataRatesRxEntry 1 }

dot11ImplementedDataRatesRxValue OBJECT-TYPE

SYNTAX Unsigned32 (2..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The receive bit rates supported by the PLCP and PMD, represented

by a count from X'02-X'7f, corresponding to data rates in incre-

ments of 500kbit/s from 1 Mb/s to 63.5 Mb/s."

::= { dot11SupportedDataRatesRxEntry 2 }

-- **********************************************************************

-- * End of dot11SupportedDataRatesRx  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyOFDM TABLE

-- **********************************************************************

dot11PhyOFDMTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyOFDMEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyOFDMTable. Implemented as a

table indexed on ifindex to allow for multiple instances on an

Agent."

::= { dot11phy 11 }

dot11PhyOFDMEntry OBJECT-TYPE

SYNTAX Dot11PhyOFDMEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyOFDM Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11PhyOFDMTable 1 }

Dot11PhyOFDMEntry ::=

SEQUENCE { dot11CurrentFrequency        											Unsigned32,

dot11TIThreshold             											Integer32,

dot11FrequencyBandsImplemented 											Unsigned32,

dot11ChannelStartingFactor 											Unsigned32,

dot11FiveMHzOperationImplemented 											TruthValue,

dot11TenMHzOperationImplemented 											TruthValue,

dot11TwentyMHzOperationImplemented											TruthValue,

dot11PhyOFDMChannelWidth 											INTEGER,

dot11OFDMCCAEDImplemented 											TruthValue,

dot11OFDMCCAEDRequired 											TruthValue,

dot11OFDMEDThreshold 											Unsigned32 }

dot11CurrentFrequency OBJECT-TYPE

SYNTAX Unsigned32 (0..200)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME.

Changes take effect as soon as practical in the implementation.

The number of the current operating frequency channel of the

OFDM PHY."

::= { dot11PhyOFDMEntry 1 }

dot11TIThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS deprecated

DESCRIPTION

"Superseded by PHY-specific or regulatory CCA energy detect lim-

its.

The Threshold being used to detect a busy medium (frequency).

CCA reports a busy medium upon detecting the RSSI above this

threshold."

::= { dot11PhyOFDMEntry 2 }

dot11FrequencyBandsImplemented OBJECT-TYPE

SYNTAX Unsigned32 (1..127)

MAX-ACCESS read-only

STATUS deprecated

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

The capability of the OFDM PHY implementation to operate in the

4.9 GHz and 5 GHz bands. Coded as an integer value with bit 0 LSB

as follows:

bit 0 .. capable of operating in the 5.15-5.25 GHz band

bit 1 .. capable of operating in the 5.25-5.35 GHz band

bit 2 .. capable of operating in the 5.725-5.825 GHz band

bit 3 .. capable of operating in the 5.47-5.725 GHz band

bit 4 .. capable of operating in the lower Japanese (5.15-5.25

GHz) band

bit 5 .. capable of operating in the 5.03-5.091 GHz band

bit 6 .. capable of operating in the 4.94-4.99 GHz band

For example, for an implementation capable of operating in the

5.15-5.35 GHz bands this attribute would take the value 3."

::= { dot11PhyOFDMEntry 3 }

dot11ChannelStartingFactor OBJECT-TYPE

SYNTAX Unsigned32 (8000..10000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The base factor from which channel center frequencies are calcu-

lated. This number is multiplied by 500 kHz to form the base

frequency to be added to the channel number x 5 MHz."

DEFVAL { 10000 }

::= { dot11PhyOFDMEntry 4 }

dot11FiveMHzOperationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the 5 MHz Operation is

implemented."

DEFVAL { false }

::= { dot11PhyOFDMEntry 5 }

dot11TenMHzOperationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the 10 MHz Operation

is implemented."

DEFVAL { false }

::= { dot11PhyOFDMEntry 6 }

dot11TwentyMHzOperationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the 20 MHz Operation

is implemented."

DEFVAL { true }

::= { dot11PhyOFDMEntry 7 }

dot11PhyOFDMChannelWidth OBJECT-TYPE

SYNTAX INTEGER { width5MHz(1), width10MHz(2), width20MHz(3)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME.

Changes take effect as soon as practical in the implementation.

This is an 8-bit integer value that identifies the OFDM PHY

channel width. Currently defined values and their corresponding

Channel widths are:

5MHz = 01, 10MHz = 02, 20MHz = 03"

::= { dot11PhyOFDMEntry 8 }

dot11OFDMCCAEDImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute indicates that the OFDM PHY is capable of CCA-

Energy Detect."

::= { dot11PhyOFDMEntry 9 }

dot11OFDMCCAEDRequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

This attribute indicates that the PHY CCA-Energy Detect func-

tionality is enabled."

::= { dot11PhyOFDMEntry 10 }

dot11OFDMEDThreshold OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

The current Energy Detect Threshold being used by the OFDM PHY."

::= { dot11PhyOFDMEntry 11 }

-- **********************************************************************

-- * End of dot11PhyOFDM TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyHRDSSSEntry TABLE

-- **********************************************************************

dot11PhyHRDSSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyHRDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyHRDSSSEntry. Implemented as a

table indexed on ifIndex to allow for multiple instances on an

Agent."

::= { dot11phy 12 }

dot11PhyHRDSSSEntry OBJECT-TYPE

SYNTAX Dot11PhyHRDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyHRDSSSEntry Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex }

::= { dot11PhyHRDSSSTable 1 }

Dot11PhyHRDSSSEntry ::=

SEQUENCE { dot11ShortPreambleOptionImplemented												TruthValue,

dot11PBCCOptionImplemented												TruthValue,

dot11ChannelAgilityPresent												TruthValue,

dot11ChannelAgilityActivated												TruthValue,

dot11HRCCAModeImplemented												Unsigned32 }

dot11ShortPreambleOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the short preamble

option as defined in 18.2.2.2 (Short PPDU format) is imple-

mented."

DEFVAL { false }

::= { dot11PhyHRDSSSEntry 1 }

dot11PBCCOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the PBCC modulation

option as defined in  18.4.6.6 (DSSS/PBCC data modulation and

modulation rate (optional)) is implemented."

DEFVAL { false }

::= { dot11PhyHRDSSSEntry 2 }

dot11ChannelAgilityPresent OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute indicates that the PHY is capable of channel

agility."

::= { dot11PhyHRDSSSEntry 3 }

dot11ChannelAgilityActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute indicates that the PHY channel agility function-

ality is enabled."

::= { dot11PhyHRDSSSEntry 4 }

dot11HRCCAModeImplemented OBJECT-TYPE

SYNTAX Unsigned32 (1..31)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

dot11HRCCAModeImplemented is a bit-significant value, represent-

ing all of the CCA modes supported by the PHY. Valid values are:

energy detect only (ED_ONLY) = 01,

carrier sense only (CS_ONLY) = 02,

carrier sense and energy detect (ED_and_CS)= 04,

carrier sense with timer (CS_and_Timer)= 08,

high rate carrier sense and energy detect (HRCS_and_ED)= 16

or the logical sum of any of these values.  In the high rate

extension PHY, this attribute is used in preference to the

dot11CCAModeSupported attribute."

::= { dot11PhyHRDSSSEntry 5 }

-- **********************************************************************

-- * End of dot11PhyHRDSSSEntry TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11HoppingPattern TABLE

-- **********************************************************************

dot11HoppingPatternTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11HoppingPatternEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The (conceptual) table of attributes necessary for a frequency

hopping implementation to be able to create the hopping

sequences necessary to operate in the subband for the associated

domain country string."

::= { dot11phy 13 }

dot11HoppingPatternEntry OBJECT-TYPE

SYNTAX Dot11HoppingPatternEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Hopping Pattern Table that

indicates the random hopping sequence to be followed.

IfIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB are indexed by ifIndex."

INDEX { ifIndex,

dot11HoppingPatternIndex }

::= { dot11HoppingPatternTable 1 }

Dot11HoppingPatternEntry ::=

SEQUENCE { dot11HoppingPatternIndex											Unsigned32,

dot11RandomTableFieldNumber											Unsigned32 }

dot11HoppingPatternIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the colum-

nar objects in the Hopping Pattern Table."

::= { dot11HoppingPatternEntry 1 }

dot11RandomTableFieldNumber OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

This attribute indicates the value of the starting channel num-

ber in the hopping sequence of the subband for the associated

domain country string."

DEFVAL { 0 }

::= { dot11HoppingPatternEntry 2 }

-- **********************************************************************

-- * End of dot11HoppingPattern TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyERP TABLE

-- **********************************************************************

dot11PhyERPTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyERPEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyERPEntry. Implemented as a

table indexed on ifIndex to allow for multiple instances on an

Agent."

::= { dot11phy 14 }

dot11PhyERPEntry OBJECT-TYPE

SYNTAX Dot11PhyERPEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyERPEntry Table.

ifIndex - Each 802.11 interface is represented by an ifEntry.

Interface tables in this MIB module are indexed by ifIndex."

INDEX {ifIndex}

::= { dot11PhyERPTable 1 }

Dot11PhyERPEntry ::= SEQUENCE {

dot11ERPPBCCOptionImplemented								TruthValue,

dot11ERPBCCOptionActivated								TruthValue,

dot11DSSSOFDMOptionImplemented								TruthValue,

dot11DSSSOFDMOptionActivated								TruthValue,

dot11ShortSlotTimeOptionImplemented								TruthValue,

dot11ShortSlotTimeOptionActivated								TruthValue }

dot11ERPPBCCOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the ERP-PBCC modula-

tion option as defined in 19.6 (ERP-PBCC operation specifica-

tions) is implemented."

DEFVAL { false }

::= { dot11PhyERPEntry 1 }

dot11ERPBCCOptionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized.

This attribute, when true, indicates that the ERP-PBCC option as

defined in 19.6 (ERP-PBCC operation specifications) is enabled."

DEFVAL { false }

::= { dot11PhyERPEntry 2 }

dot11DSSSOFDMOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the DSSS-OFDM option

as defined in 19.7 (DSSS-OFDM operation specifications) is

implemented."

DEFVAL { false }

::= { dot11PhyERPEntry 3 }

dot11DSSSOFDMOptionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request primitive or

MLME-JOIN.request primitive.

This attribute, when true, indicates that the DSSS-OFDM option

as defined in 19.7 (DSSS-OFDM operation specifications) is

enabled."

DEFVAL { false }

::= { dot11PhyERPEntry 4 }

dot11ShortSlotTimeOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the Short Slot Time

option as defined in 7.3.1.4 (Capability Information field) is

implemented."

DEFVAL { false }

::= { dot11PhyERPEntry 5}

dot11ShortSlotTimeOptionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that the Short Slot Time

option as defined in 7.3.1.4 (Capability Information field) is

enabled."

DEFVAL { false }

::= { dot11PhyERPEntry 6 }

-- **********************************************************************

-- * dot11 Phy HT TABLE

-- **********************************************************************

dot11PhyHTTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyHTEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyHTTable. Implemented as a table

indexed on ifIndex to allow for multiple instances on an Agent."

::= { dot11phy 15 }

dot11PhyHTEntry OBJECT-TYPE

SYNTAX Dot11PhyHTEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyHTEntry Table. ifIndex - Each IEEE

802.11 interface is represented by an ifEntry. Interface tables

in this MIB module are indexed by ifIndex."

INDEX {ifIndex}

::= { dot11PhyHTTable 1 }

Dot11PhyHTEntry ::= SEQUENCE {

dot11FortyMHzOperationImplemented	TruthValue,

dot11FortyMHzOperationActivated	TruthValue,

dot11CurrentPrimaryChannel	Unsigned32,

dot11CurrentSecondaryChannel	Unsigned32,

dot11NumberOfSpatialStreamsImplemented	Unsigned32,

dot11NumberOfSpatialStreamsActivated	Unsigned32,

dot11HTGreenfieldOptionImplemented	TruthValue,

dot11HTGreenfieldOptionActivated	TruthValue,

dot11ShortGIOptionInTwentyImplemented	TruthValue,

dot11ShortGIOptionInTwentyActivated	TruthValue,

dot11ShortGIOptionInFortyImplemented	TruthValue,

dot11ShortGIOptionInFortyActivated	TruthValue,

dot11LDPCCodingOptionImplemented	TruthValue,

dot11LDPCCodingOptionActivated	TruthValue,

dot11TxSTBCOptionImplemented	TruthValue,

dot11TxSTBCOptionActivated	TruthValue,

dot11RxSTBCOptionImplemented	TruthValue,

dot11RxSTBCOptionActivated	TruthValue,

dot11BeamFormingOptionImplemented	TruthValue,

dot11BeamFormingOptionActivated	TruthValue,

dot11HighestSupportedDataRate	Unsigned32,

dot11TxMCSSetDefined	TruthValue,

dot11TxRxMCSSetNotEqual	TruthValue,

dot11TxMaximumNumberSpatialStreamsSupported	Unsigned32,

dot11TxUnequalModulationSupported 	TruthValue }

dot11FortyMHzOperationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the 40 MHz Operation

is implemented."

DEFVAL { false }

::= { dot11PhyHTEntry 1 }

dot11FortyMHzOperationActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the 40 MHz Operation

is enabled."

DEFVAL { false }

::= { dot11PhyHTEntry 2 }

dot11CurrentPrimaryChannel OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the operating channel. If 20/40 MHz

BSS is currently in use then this attribute indicates the pri-

mary channel."

::= { dot11PhyHTEntry 3 }

dot11CurrentSecondaryChannel OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the channel number of the secondary

channel. If 20/40 MHz BSS is not currently in use, this

attribute value shall be 0."

::= { dot11PhyHTEntry 4 }

dot11NumberOfSpatialStreamsImplemented OBJECT-TYPE

SYNTAX Unsigned32 (1..4)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the maximum number of spatial streams

implemented."

DEFVAL { 2 }

::= { dot11PhyHTEntry 5 }

dot11NumberOfSpatialStreamsActivated OBJECT-TYPE

SYNTAX Unsigned32 (1..4)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the maximum number of spatial streams

enabled."

DEFVAL { 2 }

::= { dot11PhyHTEntry 6 }

dot11HTGreenfieldOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the HT-greenfield

option is implemented."

DEFVAL { false }

::= { dot11PhyHTEntry 7 }

dot11HTGreenfieldOptionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the HT-greenfield

option is enabled."

DEFVAL { false }

::= { dot11PhyHTEntry 8 }

dot11ShortGIOptionInTwentyImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the Short Guard

option is implemented for 20 MHz operation."

DEFVAL { false }

::= { dot11PhyHTEntry 9 }

dot11ShortGIOptionInTwentyActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the Short Guard

option is enabled for 20 MHz operation."

DEFVAL { false }

::= { dot11PhyHTEntry 10 }

dot11ShortGIOptionInFortyImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the Short Guard

option is implemented for 40 MHz operation."

DEFVAL { false }

::= { dot11PhyHTEntry 11 }

dot11ShortGIOptionInFortyActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the Short Guard

option is enabled for 40 MHz operation."

DEFVAL { false }

::= { dot11PhyHTEntry 12 }

dot11LDPCCodingOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the LDPC coding

option is implemented."

DEFVAL { false }

::= { dot11PhyHTEntry 13 }

dot11LDPCCodingOptionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the LDPC coding

option is enabled."

DEFVAL { false }

::= { dot11PhyHTEntry 14 }

dot11TxSTBCOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the entity is capable

of transmitting frames using STBC option."

DEFVAL { false }

::= { dot11PhyHTEntry 15 }

dot11TxSTBCOptionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the entity's  capabil-

ity of transmitting frames using STBC option is enabled."

DEFVAL { false }

::= { dot11PhyHTEntry 16 }

dot11RxSTBCOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the entity is capable

of receiving frames that are sent using the STBC."

DEFVAL { false }

::= { dot11PhyHTEntry 17 }

dot11RxSTBCOptionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the entity's  capabil-

ity of receiving frames that are sent using the STBC is

enabled."

DEFVAL { false }

::= { dot11PhyHTEntry 18 }

dot11BeamFormingOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the beamforming

option is implemented."

DEFVAL { false }

::= { dot11PhyHTEntry 19 }

dot11BeamFormingOptionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the beamforming

option is enabled."

DEFVAL { false }

::= { dot11PhyHTEntry 20 }

dot11HighestSupportedDataRate OBJECT-TYPE

SYNTAX Unsigned32 (0..600)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the Highest Data Rate in Mb/s at

which the station may receive data."

DEFVAL { 0 }

::= { dot11PhyHTEntry 21 }

dot11TxMCSSetDefined OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the Tx MCS set is

defined."

DEFVAL { false }

::= { dot11PhyHTEntry 22 }

dot11TxRxMCSSetNotEqual OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the supported Tx and

Rx MCS sets are not equal."

DEFVAL { false }

::= { dot11PhyHTEntry 23 }

dot11TxMaximumNumberSpatialStreamsSupported OBJECT-TYPE

SYNTAX Unsigned32 (0..3)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the Tx maximum number of spatial

streams supported."

DEFVAL { 0 }

::= { dot11PhyHTEntry 24 }

dot11TxUnequalModulationSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that Tx UEQM is sup-

ported."

DEFVAL { false }

::= { dot11PhyHTEntry 25 }

-- **********************************************************************

-- * End of dot11 PHY HT TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11 Supported MCS Tx TABLE

-- **********************************************************************

dot11SupportedMCSTxTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SupportedMCSTxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"he Transmit MCS supported by the PLCP and PMD, represented by a

count from 1 to 127, subject to limitations of each individual

PHY."

::= { dot11phy 16 }

dot11SupportedMCSTxEntry OBJECT-TYPE

SYNTAX Dot11SupportedMCSTxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the dot11SupportedMCSTx Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed by

ifIndex."

INDEX { ifIndex, dot11SupportedMCSTxIndex }

::= { dot11SupportedMCSTxTable 1 }

Dot11SupportedMCSTxEntry ::=

SEQUENCE {

dot11SupportedMCSTxIndex 	Unsigned32,

dot11SupportedMCSTxValue 	Unsigned32 }

dot11SupportedMCSTxIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index object that identifies which MCS to access. Range is

1..255."

::= { dot11SupportedMCSTxEntry 1 }

dot11SupportedMCSTxValue OBJECT-TYPE

SYNTAX Unsigned32 (1..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Transmit MCS supported by the PLCP and PMD, represented by

a count from 1 to 127, subject to limitations of each individual

PHY."

::= { dot11SupportedMCSTxEntry 2 }

-- **********************************************************************

-- * End of dot11 Supported MCS Tx TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11 Supported MCS Rx TABLE

-- **********************************************************************

dot11SupportedMCSRxTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SupportedMCSRxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The receive MCS supported by the PLCP and PMD, represented by a

count from 1 to 127, subject to limitations of each individual

PHY."

::= { dot11phy 17 }

dot11SupportedMCSRxEntry OBJECT-TYPE

SYNTAX Dot11SupportedMCSRxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the dot11SupportedMCSRx Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX { ifIndex, dot11SupportedMCSRxIndex }

::= { dot11SupportedMCSRxTable 1 }

Dot11SupportedMCSRxEntry ::=

SEQUENCE {

dot11SupportedMCSRxIndex 	Unsigned32,

dot11SupportedMCSRxValue 	Unsigned32 }

dot11SupportedMCSRxIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index object that identifies which MCS to access. Range is

1..255."

::= { dot11SupportedMCSRxEntry 1 }

dot11SupportedMCSRxValue OBJECT-TYPE

SYNTAX Unsigned32 (1..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The receive MCS supported by the PLCP and PMD, represented by a

count from 1 to 127, subject to limitations of each individual

PHY."

::= { dot11SupportedMCSRxEntry 2 }

-- **********************************************************************

-- * End of dot11 Supported MCS Rx TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11 Transmit Beamforming Config TABLE

-- **********************************************************************

dot11TransmitBeamformingConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11TransmitBeamformingConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11TransmitBeamformingConfigTable.

Implemented as a table indexed on ifIndex to allow for multiple

instances on an Agent."

::= { dot11phy 18 }

dot11TransmitBeamformingConfigEntry OBJECT-TYPE

SYNTAX Dot11TransmitBeamformingConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11TransmitBeamformingConfig Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEn-

try. Interface tables in this MIB module are indexed by ifIn-

dex."

INDEX {ifIndex}

::= { dot11TransmitBeamformingConfigTable 1 }

Dot11TransmitBeamformingConfigEntry ::=

SEQUENCE {

dot11ReceiveStaggerSoundingOptionImplemented	TruthValue,

dot11TransmitStaggerSoundingOptionImplemented	TruthValue,

dot11ReceiveNDPOptionImplemented	TruthValue,

dot11TransmitNDPOptionImplemented	TruthValue,

dot11ImplicitTransmitBeamformingOptionImplemented	TruthValue,

dot11CalibrationOptionImplemented 	INTEGER,

dot11ExplicitCSITransmitBeamformingOptionImplemented	TruthValue,

dot11ExplicitNonCompressedBeamformingMatrixOptionImplemented

TruthValue,

dot11ExplicitTransmitBeamformingCSIFeedbackOptionImplemented

INTEGER,

dot11ExplicitNonCompressedBeamformingFeedbackOptionImplemented

INTEGER,

dot11ExplicitCompressedBeamformingFeedbackOptionImplemented

INTEGER,

dot11NumberBeamFormingCSISupportAntenna 	Unsigned32,

dot11NumberNonCompressedBeamformingMatrixSupportAntenna

dot11NumberCompressedBeamformingMatrixSupportAntenna	Unsigned32

}

dot11ReceiveStaggerSoundingOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the STA implementa-

tion supports the receiving of staggered sounding frames."

DEFVAL { false }

::= { dot11TransmitBeamformingConfigEntry 1 }

dot11TransmitStaggerSoundingOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the STA implementa-

tion supports the transmission of staggered sounding frames."

DEFVAL { false }

::= { dot11TransmitBeamformingConfigEntry 2 }

dot11ReceiveNDPOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the STA implementa-

tion is capable of receiving NDP as sounding frames."

DEFVAL { false }

::= { dot11TransmitBeamformingConfigEntry 3 }

dot11TransmitNDPOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the STA implementa-

tion is capable of transmitting NDP as sounding frames."

DEFVAL { false }

::= { dot11TransmitBeamformingConfigEntry 4 }

dot11ImplicitTransmitBeamformingOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that STA implementation is

capable of applying implicit transmit beamforming."

DEFVAL { false }

::= { dot11TransmitBeamformingConfigEntry 5 }

dot11CalibrationOptionImplemented OBJECT-TYPE

SYNTAX INTEGER { inCapable (0), unableToInitiate (1),

ableToInitiate (2), fullyCapable (3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the level of calibration supported by

the STA implementation."

DEFVAL { inCapable }

::= { dot11TransmitBeamformingConfigEntry 6 }

dot11ExplicitCSITransmitBeamformingOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that STA implementation is

capable of applying transmit beamforming using CSI explicit

feedback in its transmission."

DEFVAL { false }

::= { dot11TransmitBeamformingConfigEntry 7 }

dot11ExplicitNonCompressedBeamformingMatrixOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that STA implementation is

capable of applying transmit beamforming using noncompressed

beamforming feedback matrix explicit feedback in its transmis-

sion."

DEFVAL { false }

::= { dot11TransmitBeamformingConfigEntry 8 }

dot11ExplicitTransmitBeamformingCSIFeedbackOptionImplemented OBJECT-TYPE

SYNTAX INTEGER { inCapable (0), delayed (1), immediate (2),

unsolicitedImmediate (3), aggregated (4),

delayedAggregated (5), immediateAggregated(6),

unsolicitedImmediateAggregated (7) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the level of CSI explicit feedback

returned by the STA implementation."

DEFVAL { inCapable }

::= { dot11TransmitBeamformingConfigEntry 9 }

dot11ExplicitNonCompressedBeamformingFeedbackOptionImplemented OBJECT-TYPE

SYNTAX INTEGER { inCapable (0), delayed (1), immediate (2),

unsolicitedImmediate (3), aggregated (4),

delayedAggregated (5), immediateAggregated(6),

unsolicitedImmediateAggregated (7) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the level of noncompressed beamforming

feedback matrix explicit feedback returned by the STA implemen-

tation."

DEFVAL { inCapable }

::= { dot11TransmitBeamformingConfigEntry 10 }

dot11ExplicitCompressedBeamformingFeedbackOptionImplemented OBJECT-TYPE

SYNTAX INTEGER { inCapable (0), delayed (1), immediate (2),

unsolicitedImmediate (3), aggregated (4),

delayedAggregated (5), immediateAggregated(6),

unsolicitedImmediateAggregated (7) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the level of noncompressed beamforming

feedback matrix explicit feedback returned by the STA implemen-

tation."

DEFVAL { inCapable }

::= { dot11TransmitBeamformingConfigEntry 11 }

dot11NumberBeamFormingCSISupportAntenna OBJECT-TYPE

SYNTAX Unsigned32 (1..4)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the maximum number of beamform anten-

nas the beamformee can support when CSI feedback is required."

::= { dot11TransmitBeamformingConfigEntry 12 }

dot11NumberNonCompressedBeamformingMatrixSupportAntenna OBJECT-TYPE

SYNTAX Unsigned32 (1..4)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the maximum number of beamform anten-

nas the beamformee can support when noncompressed beamforming

feedback matrix feedback is required."

::= { dot11TransmitBeamformingConfigEntry 13 }

dot11NumberCompressedBeamformingMatrixSupportAntenna OBJECT-TYPE

SYNTAX Unsigned32 (1..4)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the maximum number of beamform anten-

nas the beamformee can support when compressed beamforming feed-

back matrix feedback is required."

::= { dot11TransmitBeamformingConfigEntry 14 }

-- **********************************************************************

-- * End of dot11 Transmit Beamforming Config TABLE

-- **********************************************************************

-- **********************************************************************

-- * End of dot11PhyERP TABLE

-- **********************************************************************

-- **********************************************************************

-- * Conformance Information

-- **********************************************************************

dot11Conformance				OBJECT IDENTIFIER ::= { ieee802dot11 5 }

dot11Groups 				OBJECT IDENTIFIER ::= { dot11Conformance 1 }

dot11Compliances				OBJECT IDENTIFIER ::= { dot11Conformance 2 }

-- **********************************************************************

-- *   Groups - units of conformance

-- **********************************************************************

dot11SMTbase OBJECT-GROUP

OBJECTS { dot11StationID, dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut }

STATUS deprecated

DESCRIPTION

"Superseded by dot11SMTbase2.

The SMT object class provides the necessary support at the STA

to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 1 }

dot11SMTprivacy OBJECT-GROUP

OBJECTS { dot11PrivacyInvoked,

dot11WEPKeyMappingLengthImplemented, dot11ExcludeUnencrypted,

dot11WEPICVErrorCount , dot11WEPExcludedCount ,

dot11WEPDefaultKeyID,

dot11WEPDefaultKeyValue,

dot11WEPKeyMappingWEPOn,

dot11WEPKeyMappingValue , dot11WEPKeyMappingAddress,

dot11WEPKeyMappingStatus }

STATUS current

DESCRIPTION

"The SMTPrivacy package is a set of attributes that are present

if WEP is implemented in the STA."

::= { dot11Groups 2 }

dot11MACbase OBJECT-GROUP

OBJECTS { dot11MACAddress, dot11Address,

dot11GroupAddressesStatus,

dot11RTSThreshold, dot11ShortRetryLimit,

dot11LongRetryLimit, dot11FragmentationThreshold,

dot11MaxTransmitMSDULifetime,

dot11MaxReceiveLifetime, dot11ManufacturerID,

dot11ProductID }

STATUS deprecated

DESCRIPTION

"Superseded by dot11MACbase2.

The MAC object class provides the necessary support for the

access control, generation, and verification of frame check

sequences (FCSs), and proper delivery of valid data to upper

layers."

::= { dot11Groups 3 }

dot11MACStatistics OBJECT-GROUP

OBJECTS { dot11RetryCount, dot11MultipleRetryCount,

dot11RTSSuccessCount, dot11RTSFailureCount,

dot11ACKFailureCount, dot11FrameDuplicateCount }

STATUS current

DESCRIPTION

"The MACStatistics package provides extended statistical infor-

mation on the operation of the MAC.  This package is completely

optional."

::= { dot11Groups 4 }

dot11ResourceTypeID OBJECT-GROUP

OBJECTS { dot11ResourceTypeIDName, dot11manufacturerOUI,

dot11manufacturerName, dot11manufacturerProductName,

dot11manufacturerProductVersion }

STATUS current

DESCRIPTION

"Attributes used to identify a STA, its manufacturer, and vari-

ous product names and versions."

::= { dot11Groups 5 }

dot11SmtAuthenticationAlgorithms OBJECT-GROUP

OBJECTS { dot11AuthenticationAlgorithm,

dot11AuthenticationAlgorithmsActivated }

STATUS current

DESCRIPTION

"Authentication Algorithm Table."

::= { dot11Groups 6 }

dot11PhyOperationComplianceGroup OBJECT-GROUP

OBJECTS { dot11PHYType, dot11CurrentRegDomain, dot11TempType }

STATUS deprecated

DESCRIPTION

"Superseded by dot11PhyOperationComplianceGroup2.

PHY layer operations attributes."

::= { dot11Groups 7 }

dot11PhyAntennaComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentTxAntenna, dot11DiversitySupportImplemented,

dot11CurrentRxAntenna }

STATUS deprecated

DESCRIPTION

"Attributes for Data Rates for IEEE 802.11."

::= { dot11Groups 8 }

dot11PhyTxPowerComplianceGroup OBJECT-GROUP

OBJECTS { dot11NumberSupportedPowerLevelsImplemented, dot11TxPowerLevel1,

dot11TxPowerLevel2, dot11TxPowerLevel3, dot11TxPowerLevel4,

dot11TxPowerLevel5, dot11TxPowerLevel6, dot11TxPowerLevel7,

dot11TxPowerLevel8, dot11CurrentTxPowerLevel }

STATUS current

DESCRIPTION

"Attributes for Control and Management of transmit power."

::= { dot11Groups 9 }

dot11PhyFHSSComplianceGroup OBJECT-GROUP

OBJECTS { dot11HopTime, dot11CurrentChannelNumber,

dot11MaxDwellTime, dot11CurrentDwellTime, dot11CurrentSet,

dot11CurrentPattern, dot11CurrentIndex }

STATUS deprecated

DESCRIPTION

"Superseded by dot11PhyFHSSComplianceGroup2.

Attributes that configure the Frequency Hopping for IEEE

802.11."

::= { dot11Groups 10 }

dot11PhyDSSSComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentChannel, dot11CCAModeSupported,

dot11CurrentCCAMode, dot11EDThreshold }

STATUS current

DESCRIPTION

"Attributes that configure the DSSS for IEEE 802.11."

::= { dot11Groups 11 }

dot11PhyIRComplianceGroup OBJECT-GROUP

OBJECTS { dot11CCAWatchdogTimerMax, dot11CCAWatchdogCountMax,

dot11CCAWatchdogTimerMin, dot11CCAWatchdogCountMin }

STATUS current

DESCRIPTION

"Attributes that configure the baseband IR for IEEE 802.11."

::= { dot11Groups 12 }

dot11PhyRegDomainsSupportGroup OBJECT-GROUP

OBJECTS { dot11RegDomainsImplementedValue }

STATUS deprecated

DESCRIPTION

"Attributes that specify the supported Regulation Domains."

::= { dot11Groups 13 }

dot11PhyAntennasListGroup OBJECT-GROUP

OBJECTS { dot11TxAntennaImplemented,

dot11RxAntennaImplemented, dot11DiversitySelectionRxImplemented

}

STATUS current

DESCRIPTION

"Attributes that specify the supported Regulation Domains."

::= { dot11Groups 14 }

dot11PhyRateGroup OBJECT-GROUP

OBJECTS { dot11ImplementedDataRatesTxValue,

dot11ImplementedDataRatesRxValue }

STATUS current

DESCRIPTION

"Attributes for Data Rates for IEEE 802.11."

::= { dot11Groups 15 }

dot11CountersGroup OBJECT-GROUP

OBJECTS { dot11TransmittedFragmentCount,

dot11GroupTransmittedFrameCount,

dot11FailedCount, dot11ReceivedFragmentCount,

dot11GroupReceivedFrameCount,

dot11FCSErrorCount,

dot11WEPUndecryptableCount,

dot11TransmittedFrameCount }

STATUS deprecated

DESCRIPTION

"Superseded by dot11CountersGroup2.

Attributes from the dot11CountersGroup that are not described in

the dot11MACStatistics group.  These objects are mandatory."

::= { dot11Groups 16 }

dot11NotificationGroup NOTIFICATION-GROUP

NOTIFICATIONS { dot11Disassociate,

dot11Deauthenticate,

dot11AuthenticateFail }

STATUS current

DESCRIPTION

"IEEE 802.11 notifications"

::= { dot11Groups 17 }

dot11SMTbase2 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation }

STATUS deprecated

DESCRIPTION

"Superseded by dot11SMTbase3.

The SMTbase2 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 18 }

dot11PhyOFDMComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentFrequency,

dot11TIThreshold,

dot11FrequencyBandsImplemented,

dot11ChannelStartingFactor }

STATUS deprecated

DESCRIPTION

"Superseded by dot11PhyOFDMComplianceGroup2.

Attributes that configure the OFDM for IEEE 802.11."

::= { dot11Groups 19 }

dot11SMTbase3 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityActivated,

dot11CountryString }

STATUS deprecated

DESCRIPTION

"Superseded by dot11SMTbase4.

The SMTbase3 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 network, when the STA

is capable of multi-domain operation. This object group should

be implemented when the multi-domain capability option is imple-

mented."

::= { dot11Groups 20 }

dot11MultiDomainCapabilityGroup OBJECT-GROUP

OBJECTS { dot11FirstChannelNumber,

dot11NumberofChannels,

dot11MaximumTransmitPowerLevel }

STATUS current

DESCRIPTION

"The dot11MultiDomainCapabilityGroup object class provides the

objects necessary to manage the channels usable by a STA, when

the multi-domain capability option is implemented."

::= { dot11Groups 21 }

dot11PhyFHSSComplianceGroup2 OBJECT-GROUP

OBJECTS { dot11HopTime, dot11CurrentChannelNumber,

dot11MaxDwellTime, dot11CurrentDwellTime, dot11CurrentSet,

dot11CurrentPattern, dot11CurrentIndex, dot11EHCCPrimeRadix,

dot11EHCCNumberofChannelsFamilyIndex,

dot11EHCCCapabilityImplemented, dot11EHCCCapabilityActivated,

dot11HopAlgorithmAdopted, dot11RandomTableFlag,

dot11NumberofHoppingSets, dot11HopModulus,

dot11HopOffset, dot11RandomTableFieldNumber }

STATUS current

DESCRIPTION

"Attributes that configure the Frequency Hopping for IEEE 802.11

when multi-domain capability option is implemented."

::= { dot11Groups 22 }

dot11PhyHRDSSSComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentChannel, dot11CCAModeSupported,

dot11CurrentCCAMode, dot11EDThreshold,

dot11ShortPreambleOptionImplemented,

dot11PBCCOptionImplemented, dot11ChannelAgilityPresent,

dot11ChannelAgilityActivated, dot11HRCCAModeImplemented }

STATUS current

DESCRIPTION

"Attributes that configure the HRDSSS for IEEE 802.11."

::= { dot11Groups 23 }

dot11PhyERPComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentChannel,

dot11ShortPreambleOptionImplemented,

dot11ChannelAgilityPresent,

dot11ChannelAgilityActivated,

dot11DSSSOFDMOptionImplemented,

dot11DSSSOFDMOptionActivated,

dot11PBCCOptionImplemented,

dot11ERPPBCCOptionImplemented,

dot11ShortSlotTimeOptionImplemented,

dot11ShortSlotTimeOptionActivated }

STATUS current

DESCRIPTION

"Attributes that configure the ERP."

::= { dot11Groups 24 }

dot11RSNAadditions OBJECT-GROUP

OBJECTS { dot11RSNAActivated,

dot11RSNAPreauthenticationActivated }

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11 MIB

required to manage RSNA functionality. Note that additional

objects for managing this functionality are located in the IEEE

802.11 RSN MIB."

::= { dot11Groups 25 }

dot11SMTbase4 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityActivated,

dot11CountryString,

dot11RSNAOptionImplemented }

STATUS deprecated

DESCRIPTION

"Superseded by dot11SMTbase5.

The SMTbase4 object class provides the necessary support at the

IEEE STA to manage the processes in the STA so that the STA may

work cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 26 }

-- ********************************************************************

-- * Groups - units of conformance - RSN

-- ********************************************************************

dot11RSNBase OBJECT-GROUP

OBJECTS {

dot11RSNAConfigVersion,

dot11RSNAConfigPairwiseKeysImplemented,

dot11RSNAConfigGroupCipher,

dot11RSNAConfigGroupRekeyMethod,

dot11RSNAConfigGroupRekeyTime,

dot11RSNAConfigGroupRekeyPackets,

dot11RSNAConfigGroupRekeyStrict,

dot11RSNAConfigPSKValue,

dot11RSNAConfigPSKPassPhrase,

dot11RSNAConfigGroupUpdateCount,

dot11RSNAConfigPairwiseUpdateCount,

dot11RSNAConfigGroupCipherSize,

dot11RSNAConfigPairwiseCipherImplemented,

dot11RSNAConfigPairwiseCipherActivated,

dot11RSNAConfigPairwiseCipherSizeImplemented,

dot11RSNAConfigAuthenticationSuiteImplemented,

dot11RSNAConfigAuthenticationSuiteActivated,

dot11RSNAConfigNumberOfPTKSAReplayCountersImplemented,

dot11RSNAConfigSATimeout,

dot11RSNAConfigNumberOfGTKSAReplayCountersImplemented,

dot11RSNAAuthenticationSuiteSelected,

dot11RSNAPairwiseCipherSelected,

dot11RSNAGroupCipherSelected,

dot11RSNAPMKIDUsed,

dot11RSNAAuthenticationSuiteRequested,

dot11RSNAPairwiseCipherRequested,

dot11RSNAGroupCipherRequested,

dot11RSNAStatsSTAAddress,

dot11RSNAStatsVersion,

dot11RSNAStatsSelectedPairwiseCipher,

dot11RSNAStatsTKIPICVErrors,

dot11RSNAStatsTKIPLocalMICFailures,

dot11RSNAStatsTKIPRemoteMICFailures,

dot11RSNAStatsCCMPReplays,

dot11RSNAStatsCCMPDecryptErrors,

dot11RSNAStatsTKIPReplays,

dot11RSNAConfigSTKKeysImplemented,

dot11RSNAConfigSTKCipher,

dot11RSNAConfigSTKRekeyTime,

dot11RSNAConfigSTKCipherSize,

dot11RSNAConfigNumberOfSTKSAReplayCountersImplemented,

dot11RSNAPairwiseSTKSelected,

dot11RSNASMKHandshakeFailures }

STATUS current

DESCRIPTION

"The dot11RSNBase object class provides the necessary support

for managing RSNA functionality in the STA."

::= { dot11Groups 27 }

dot11RSNPMKcachingGroup OBJECT-GROUP

OBJECTS {

dot11RSNAConfigPMKLifetime,

dot11RSNAConfigPMKReauthThreshold

}

STATUS current

DESCRIPTION

"The dot11RSNPMKcachingGroup object class provides the necessary

support for managing PMK caching functionality in the STA"

::= { dot11Groups 28 }

dot11RSNSMKcachingGroup OBJECT-GROUP

OBJECTS {

dot11RSNAConfigSMKLifetime,

dot11RSNAConfigSMKReauthThreshold

}

STATUS current

DESCRIPTION

"The dot11RSNSMKcachingGroup object class provides the necessary

support for managing SMK caching functionality in the STA."

::= { dot11Groups 29 }

dot11SMTbase5 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityActivated,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired }

STATUS current

DESCRIPTION

"The SMTbase5 object class provides the necessary support at the

STA to manage the processes in the STA so that the STA may work

cooperatively as a part of an IEEE 802.11 network, when the STA

is capable of multidomain operation. This object group should be

implemented when the multidomain capability option is imple-

mented."

::= { dot11Groups 30 }

dot11MACbase2 OBJECT-GROUP

OBJECTS { dot11MACAddress, dot11Address,

dot11GroupAddressesStatus,

dot11RTSThreshold, dot11ShortRetryLimit,

dot11LongRetryLimit, dot11FragmentationThreshold,

dot11MaxTransmitMSDULifetime,

dot11MaxReceiveLifetime, dot11ManufacturerID,

dot11ProductID, dot11CAPLimit, dot11HCCWmin,

dot11HCCWmax, dot11HCCAIFSN,

dot11ADDBAResponseTimeout, dot11ADDTSResponseTimeout,

dot11ChannelUtilizationBeaconInterval, dot11ScheduleTimeout,

dot11DLSResponseTimeout, dot11QAPMissingAckRetryLimit,

dot11EDCAAveragingPeriod }

STATUS deprecated

DESCRIPTION

"Superseded by dot11MACbase3.

The MAC object class provides the necessary support for the

access control, generation, and verification of frame check

sequences (FCSs), and proper delivery of valid data to upper

layers."

::= { dot11Groups 31 }

dot11CountersGroup2 OBJECT-GROUP

OBJECTS { dot11TransmittedFragmentCount,

dot11GroupTransmittedFrameCount,

dot11FailedCount, dot11ReceivedFragmentCount,

dot11GroupReceivedFrameCount,

dot11FCSErrorCount,

dot11WEPUndecryptableCount,

dot11TransmittedFrameCount,

dot11QosDiscardedFragmentCount,

dot11AssociatedStationCount,

dot11QosCFPollsReceivedCount,

dot11QosCFPollsUnusedCount,

dot11QosCFPollsUnusableCount }

STATUS deprecated

DESCRIPTION

"Superseded by dot11CountersGroup3.

Attributes from the dot11CountersGroup that are not described in

the dot11MACStatistics group. These objects are mandatory."

::= { dot11Groups 32 }

dot11Qosadditions OBJECT-GROUP

OBJECTS { dot11EDCATable, dot11QAPEDCATable,

dot11QosCountersTable }

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11 MIB

required to manage QoS functionality."

::= { dot11Groups 33 }

dot11SMTbase6 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityActivated,

dot11CountryString,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateInNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed }

STATUS deprecated

DESCRIPTION

"Superseded by dot11SMTbase7.

The SMTbase6 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 34 }

dot11PhyOFDMComplianceGroup2 OBJECT-GROUP

OBJECTS { dot11CurrentFrequency,

dot11TIThreshold,

dot11FrequencyBandsImplemented,

dot11ChannelStartingFactor,

dot11FiveMHzOperationImplemented,

dot11TenMHzOperationImplemented,

dot11TwentyMHzOperationImplemented,

dot11PhyOFDMChannelWidth }

STATUS deprecated

DESCRIPTION

"Superseded by dot11PhyOFDMComplianceGroup3.

Attributes that configure the OFDM for IEEE 802.11."

::= { dot11Groups 35}

dot11SMTbase7 OBJECT-GROUP

OBJECTS{ dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID,

dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod,

dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityActivated,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateInNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed,

dot11AssociateStation,

dot11AssociateID,

dot11AssociateFailStation,

dot11AssociateFailStatus,

dot11ReassociateStation,

dot11ReassociateID,

dot11ReassociateFailStation,

dot11ReassociateFailStatus,

dot11RadioMeasurementImplemented,

dot11RadioMeasurementActivated,

dot11RMMeasurementProbeDelay,

dot11RMMeasurementPilotPeriod,

dot11RMLinkMeasurementActivated,

dot11RMNeighborReportActivated,

dot11RMParallelMeasurementsActivated,

dot11RMRepeatedMeasurementsActivated,

dot11RMBeaconPassiveMeasurementActivated,

dot11RMBeaconActiveMeasurementActivated,

dot11RMBeaconTableMeasurementActivated,

dot11RMBeaconMeasurementReportingConditionsActivated,

dot11RMFrameMeasurementActivated,

dot11RMChannelLoadMeasurementActivated,

dot11RMNoiseHistogramMeasurementActivated,

dot11RMStatisticsMeasurementActivated,

dot11RMLCIMeasurementActivated,

dot11RMLCIAzimuthActivated,

dot11RMTransmitStreamCategoryMeasurementActivated,

dot11RMTriggeredTransmitStreamCategoryMeasurementActivated,

dot11RMAPChannelReportActivated,

dot11RMMIBActivated,

dot11RMMaxMeasurementDuration,

dot11RMNonOperatingChannelMaxMeasurementDuration,

dot11RMMeasurementPilotTransmissionInformationActivated,

dot11RMMeasurementPilotActivated,

dot11RMNeighborReportTSFOffsetActivated,

dot11RMRCPIMeasurementActivated,

dot11RMRSNIMeasurementActivated,

dot11RMBSSAverageAccessDelayActivated,

dot11RMBSSAvailableAdmissionCapacityActivated,

dot11RMAntennaInformationActivated}

STATUS deprecated

DESCRIPTION

"Superseded by dot11SMTbase8.

The SMTbase7 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 net-work, when the STA

is capable of multi-domain operation. This object group should

be implemented when the multi-domain capability option is imple-

mented."

::= { dot11Groups 36 }

dot11SMTRMRequest OBJECT-GROUP

OBJECTS {

dot11RMRqstRowStatus,

dot11RMRqstToken,

dot11RMRqstRepetitions,

dot11RMRqstIfIndex,

dot11RMRqstType,

dot11RMRqstTargetAdd,

dot11RMRqstTimeStamp,

dot11RMRqstChanNumber,

dot11RMRqstRegulatoryClass,

dot11RMRqstRndInterval,

dot11RMRqstDuration,

dot11RMRqstParallel,

dot11RMRqstEnable,

dot11RMRqstRequest,

dot11RMRqstReport,

dot11RMRqstDurationMandatory,

dot11RMRqstBeaconRqstMode,

dot11RMRqstBeaconRqstDetail,

dot11RMRqstFrameRqstType,

dot11RMRqstBssid,

dot11RMRqstSSID,

dot11RMRqstBeaconReportingCondition,

dot11RMRqstBeaconThresholdOffset,

dot11RMRqstSTAStatRqstGroupID,

dot11RMRqstLCIRqstSubject,

dot11RMRqstLCILatitudeResolution,

dot11RMRqstLCILongitudeResolution,

dot11RMRqstLCIAltitudeResolution,

dot11RMRqstLCIAzimuthType,

dot11RMRqstLCIAzimuthResolution,

dot11RMRqstPauseTime,

dot11RMRqstTransmitStreamPeerQSTAAddress,

dot11RMRqstTransmitStreamTrafficIdentifier,

dot11RMRqstTransmitStreamBin0Range,

dot11RMRqstTrigdQoSAverageCondition,

dot11RMRqstTrigdQoSConsecutiveCondition,

dot11RMRqstTrigdQoSDelayCondition,

dot11RMRqstTrigdQoSAverageThreshold,

dot11RMRqstTrigdQoSConsecutiveThreshold,

dot11RMRqstTrigdQoSDelayThresholdRange,

dot11RMRqstTrigdQoSDelayThreshold,

dot11RMRqstTrigdQoSMeasurementCount,

dot11RMRqstTrigdQoSTimeout,

dot11RMRqstChannelLoadReportingCondition,

dot11RMRqstChannelLoadReference,

dot11RMRqstNoiseHistogramReportingCondition,

dot11RMRqstAnpiReference,

dot11RMRqstAPChannelReport,

dot11RMRqstSTAStatPeerSTAAddress,

dot11RMRqstFrameTransmitterAddress,

dot11RMRqstVendorSpecific }

STATUS current

DESCRIPTION

"The SMTRMRequest package is a set of attributes that are

present if the STA supports the Radio Measurement service."

::= { dot11Groups 37 }

dot11SMTRMReport OBJECT-GROUP

OBJECTS {

dot11ChannelLoadRprtRqstToken,

dot11ChannelLoadRprtIfIndex,

dot11ChannelLoadMeasuringSTAAddr,

dot11ChannelLoadRprtChanNumber,

dot11ChannelLoadRprtRegulatoryClass,

dot11ChannelLoadRprtActualStartTime,

dot11ChannelLoadRprtMeasurementDuration,

dot11ChannelLoadRprtChannelLoad,

dot11ChannelLoadRprtVendorSpecific,

dot11ChannelLoadRprtMeasurementMode,

dot11NoiseHistogramRprtRqstToken,

dot11NoiseHistogramRprtIfIndex,

dot11NoiseHistogramMeasuringSTAAddr,

dot11NoiseHistogramRprtChanNumber,

dot11NoiseHistogramRprtRegulatoryClass,

dot11NoiseHistogramRprtActualStartTime,

dot11NoiseHistogramRprtMeasurementDuration,

dot11NoiseHistogramRprtAntennaID,

dot11NoiseHistogramRprtANPI,

dot11NoiseHistogramRprtIPIDensity0,

dot11NoiseHistogramRprtIPIDensity1,

dot11NoiseHistogramRprtIPIDensity2,

dot11NoiseHistogramRprtIPIDensity3,

dot11NoiseHistogramRprtIPIDensity4,

dot11NoiseHistogramRprtIPIDensity5,

dot11NoiseHistogramRprtIPIDensity6,

dot11NoiseHistogramRprtIPIDensity7,

dot11NoiseHistogramRprtIPIDensity8,

dot11NoiseHistogramRprtIPIDensity9,

dot11NoiseHistogramRprtIPIDensity10,

dot11NoiseHistogramRprtVendorSpecific,

dot11NoiseHistogramRprtMeasurementMode,

dot11BeaconRprtRqstToken,

dot11BeaconRprtIfIndex,

dot11BeaconMeasuringSTAAddr,

dot11BeaconRprtChanNumber,

dot11BeaconRprtRegulatoryClass,

dot11BeaconRprtActualStartTime,

dot11BeaconRprtMeasurementDuration,

dot11BeaconRprtPhyType,

dot11BeaconRprtReportedFrameType,

dot11BeaconRprtRCPI,

dot11BeaconRprtRSNI,

dot11BeaconRprtBSSID,

dot11BeaconRprtAntennaID,

dot11BeaconRprtParentTSF,

dot11BeaconRprtReportedFrameBody,

dot11BeaconRprtVendorSpecific,

dot11BeaconRprtMeasurementMode,

dot11FrameRprtIfIndex,

dot11FrameRprtRqstToken,

dot11FrameRprtChanNumber,

dot11FrameRprtRegulatoryClass,

dot11FrameRprtActualStartTime,

dot11FrameRprtMeasurementDuration,

dot11FrameRprtTransmitSTAAddress,

dot11FrameRprtBSSID,

dot11FrameRprtPhyType,

dot11FrameRprtAvgRCPI,

dot11FrameRprtLastRSNI,

dot11FrameRprtLastRCPI,

dot11FrameRprtAntennaID,

dot11FrameRprtNumberFrames,

dot11FrameRprtVendorSpecific,

dot11FrameRptMeasurementMode,

dot11STAStatisticsReportToken,

dot11STAStatisticsIfIndex,

dot11STAStatisticsSTAAddress,

dot11STAStatisticsMeasurementDuration,

dot11STAStatisticsGroupID,

dot11STAStatisticsTransmittedFragmentCount,

dot11STAStatisticsGroupTransmittedFrameCount,

dot11STAStatisticsFailedCount,

dot11STAStatisticsRetryCount,

dot11STAStatisticsMultipleRetryCount,

dot11STAStatisticsFrameDuplicateCount,

dot11STAStatisticsRTSSuccessCount,

dot11STAStatisticsRTSFailureCount,

dot11STAStatisticsACKFailureCount,

dot11STAStatisticsQosTransmittedFragmentCount,

dot11STAStatisticsQosFailedCount,

dot11STAStatisticsQosRetryCount,

dot11STAStatisticsQosMultipleRetryCount,

dot11STAStatisticsQosFrameDuplicateCount,

dot11STAStatisticsQosRTSSuccessCount,

dot11STAStatisticsQosRTSFailureCount,

dot11STAStatisticsQosACKFailureCount,

dot11STAStatisticsQosReceivedFragmentCount,

dot11STAStatisticsQosTransmittedFrameCount,

dot11STAStatisticsQosDiscardedFrameCount,

dot11STAStatisticsQosMPDUsReceivedCount,

dot11STAStatisticsQosRetriesReceivedCount,

dot11STAStatisticsReceivedFragmentCount,

dot11STAStatisticsGroupReceivedFrameCount,

dot11STAStatisticsFCSErrorCount,

dot11STAStatisticsTransmittedFrameCount,

dot11STAStatisticsAPAverageAccessDelay,

dot11STAStatisticsAverageAccessDelayBestEffort,

dot11STAStatisticsAverageAccessDelayBackground,

dot11STAStatisticsAverageAccessDelayVideo,

dot11STAStatisticsAverageAccessDelayVoice,

dot11STAStatisticsStationCount,

dot11STAStatisticsChannelUtilization,

dot11STAStatisticsVendorSpecific,

dot11STAStatisticsRprtMeasurementMode,

dot11LCIReportToken,

dot11LCIIfIndex,

dot11LCISTAAddress,

dot11LCILatitudeResolution,

dot11LCILatitudeInteger,

dot11LCILatitudeFraction,

dot11LCILongitudeResolution,

dot11LCILongitudeInteger,

dot11LCILongitudeFraction,

dot11LCIAltitudeType,

dot11LCIAltitudeResolution,

dot11LCIAltitudeInteger,

dot11LCIAltitudeFraction,

dot11LCIDatum,

dot11LCIAzimuthType,

dot11LCIAzimuthResolution,

dot11LCIAzimuth,

dot11LCIVendorSpecific,

dot11LCIRprtMeasurementMode,

dot11TransmitStreamRprtRqstToken,

dot11TransmitStreamRprtIfIndex,

dot11TransmitStreamMeasuringSTAAddr,

dot11TransmitStreamRprtActualStartTime,

dot11TransmitStreamRprtMeasurementDuration,

dot11TransmitStreamRprtPeerSTAAddress,

dot11TransmitStreamRprtTID,

dot11TransmitStreamRprtAverageQueueDelay,

dot11TransmitStreamRprtAverageTransmitDelay,

dot11TransmitStreamRprtTransmittedMSDUCount,

dot11TransmitStreamRprtMSDUDiscardedCount,

dot11TransmitStreamRprtMSDUFailedCount,

dot11TransmitStreamRprtMultipleRetryCount,

dot11TransmitStreamRprtCFPollsLostCount,

dot11TransmitStreamRprtBin0Range,

dot11TransmitStreamRprtDelayHistogram,

dot11TransmitStreamRprtReason,

dot11TransmitStreamRprtVendorSpecific,

dot11TransmitStreamRprtMeasurementMode

}

STATUS current

DESCRIPTION

"The SMTRMReport package is a set of attributes that are present

if the STA supports the Radio Measurement service."

::= { dot11Groups 38 }

dot11SMTRMConfig OBJECT-GROUP

OBJECTS {

dot11APChannelReportIfIndex,

dot11APChannelReportRegulatoryClass,

dot11APChannelReportChannelList,

dot11RMNeighborReportIfIndex,

dot11RMNeighborReportBSSID,

dot11RMNeighborReportAPReachability,

dot11RMNeighborReportSecurity,

dot11RMNeighborReportCapSpectrumMgmt,

dot11RMNeighborReportCapQoS,

dot11RMNeighborReportCapAPSD,

dot11RMNeighborReportCapRM,

dot11RMNeighborReportCapDelayBlockAck,

dot11RMNeighborReportCapImmediateBlockAck,

dot11RMNeighborReportKeyScope,

dot11RMNeighborReportChannelNumber,

dot11RMNeighborReportRegulatoryClass,

dot11RMNeighborReportPhyType,

dot11RMNeighborReportNeighborTSFInfo,

dot11RMNeighborReportPilotInterval,

dot11RMNeighborReportPilotMultipleBSSID,

dot11RMNeighborReportRMEnabledCapabilities,

dot11RMNeighborReportVendorSpecific,

dot11RMNeighborReportRowStatus,

dot11RMNeighborReportCapHT,

dot11RMNeighborReportHTLDPCCodingCap,

dot11RMNeighborReportHTSupportedChannelWidthSet,

dot11RMNeighborReportHTSMPowerSave,

dot11RMNeighborReportHTGreenfield,

dot11RMNeighborReportHTShortGIfor20MHz,

dot11RMNeighborReportHTShortGIfor40MHz,

dot11RMNeighborReportHTTxSTBC,

dot11RMNeighborReportHTRxSTBC,

dot11RMNeighborReportHTDelayedBlockAck,

dot11RMNeighborReportHTMaxAMSDULength,

dot11RMNeighborReportHTDSSCCKModein40MHz,

dot11RMNeighborReportHTFortyMHzIntolerant,

dot11RMNeighborReportHTLSIGTXOPProtectionSupport,

dot11RMNeighborReportHTMaxAMPDULengthExponent,

dot11RMNeighborReportHTMinMPDUStartSpacing,

dot11RMNeighborReportHTRxMCSBitMask,

dot11RMNeighborReportHTRxHighestSupportedDataRate,

dot11RMNeighborReportHTTxMCSSetDefined,

dot11RMNeighborReportHTTxRxMCSSetNotEqual,

dot11RMNeighborReportHTTxMaxNumberSpatialStreamsSupported,

dot11RMNeighborReportHTTxUnequalModulationSupported,

dot11RMNeighborReportHTPCO,

dot11RMNeighborReportHTPCOTransitionTime,

dot11RMNeighborReportHTMCSFeedback,

dot11RMNeighborReportHTCSupport,

dot11RMNeighborReportHTRDResponder,

dot11RMNeighborReportHTImplictTransmitBeamformingReceivingCap,

dot11RMNeighborReportHTReceiveStaggeredSoundingCap,

dot11RMNeighborReportHTTransmitStaggeredSoundingCap,

dot11RMNeighborReportHTReceiveNDPCap,

dot11RMNeighborReportHTTransmitNDPCap,

dot11RMNeighborReportHTImplicitTransmitBeamformingCap,

dot11RMNeighborReportHTTransmitBeamformingCalibration,

dot11RMNeighborReportHTExplicitCSITransmitBeamformingCap,

dot11RMNeighborReportHTExplicitNonCompressedSteeringCap,

dot11RMNeighborReportHTExplicitCompressedSteeringCap,

dot11RMNeighborReportHTExplicitTransmitBeamformingFeedback,

dot11RMNbRprtHTExplicitNonCompressedBeamformingFeedbackCap,

dot11RMNbRprtHTExplicitNonCompressedBeamformingFeedbackCap,

dot11RMNeighborReportHTTransmitBeamformingMinimalGrouping,

dot11RMNbRprtHTCSINumberofTxBeamformingAntennasSuppt,

dot11RMNbRprtHTCSINumberofTxBeamformingAntennasSuppt]

dot11RMNbRprtHTNonCompressedSteeringNumofTxBmfmingAntennasSuppt,

dot11RMNbRprtHTCompressedSteeringNumberofTxBmfmingAntennasSuppt,

dot11RMNbRprtHTCSIMaxNumberofRowsTxBeamformingSuppt,

dot11RMNeighborReportHTTransmitBeamformingChannelEstimationCap,

dot11RMNeighborReportHTAntSelectionCap,

dot11RMNeighborReportHTExplicitCSIFeedbackBasedTxASELCap,

dot11RMNeighborReportHTAntIndicesFeedbackBasedTxASELCap,

dot11RMNeighborReportHTExplicitCSIFeedbackBasedCap,

dot11RMNeighborReportHTAntIndicesFeedbackCap,

dot11RMNeighborReportHTRxASELCap,

dot11RMNeighborReportHTTxSoundingPPDUsCap,

dot11RMNeighborReportHTInfoPrimaryChannel,

dot11RMNeighborReportHTInfoSecChannelOffset,

dot11RMNeighborReportHTInfoSTAChannelWidth,

dot11RMNeighborReportHTInfoRIFSMode,

dot11RMNeighborReportHTInfoProtection,

dot11RMNeighborReportHTInfoNonGreenfieldHTSTAsPresent,

dot11RMNeighborReportHTInfoOBSSNonHTSTAsPresent,

dot11RMNeighborReportHTInfoDualBeacon,

dot11RMNeighborReportHTInfoDualCTSProtection,

dot11RMNeighborReportHTInfoSTBCBeacon,

dot11RMNeighborReportHTInfoLSIGTXOPProtectionSup,

dot11RMNeighborReportHTInfoPCOActive,

dot11RMNeighborReportHTInfoPCOPhase,

dot11RMNeighborReportHTInfoBasicMCSSet,

dot11RMNeighborReportHTSecChannelOffset,

dot11RMNeighborReportExtCapPSMPSupport,

dot11RMNeighborReportExtCapSPSMPSup,

dot11RMNeighborReportExtCapServiceIntervalGranularity

}

STATUS current

DESCRIPTION

"The SMTRMConfig package is a set of attributes that are

present if the STA supports the Radio Measurement service."

::= { dot11Groups 39 }

dot11FTComplianceGroup OBJECT-GROUP

OBJECTS { dot11FastBSSTransitionActivated }

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11 MIB

required to manage fast BSS transition functionality. Note that

additional objects for managing this functionality are located

in the dot11FastBSS TransitionConfigTable."

::= { dot11Groups 40}

dot11SMTbase8 OBJECT-GROUP

OBJECTS {	dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityActivated,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateInNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed,

dot11AssociateStation,

dot11AssociateID,

dot11AssociateFailStation,

dot11AssociateFailStatus,

dot11ReassociateStation,

dot11ReassociateID,

dot11ReassociateFailStation,

dot11ReassociateFailStatus,

dot11RadioMeasurementImplemented,

dot11RadioMeasurementActivated,

dot11RMMeasurementProbeDelay,

dot11RMMeasurementPilotPeriod,

dot11RMLinkMeasurementActivated,

dot11RMNeighborReportActivated,

dot11RMParallelMeasurementsActivated,

dot11RMRepeatedMeasurementsActivated,

dot11RMBeaconPassiveMeasurementActivated,

dot11RMBeaconActiveMeasurementActivated,

dot11RMBeaconTableMeasurementActivated,

dot11RMBeaconMeasurementReportingConditionsActivated,

dot11RMFrameMeasurementActivated,

dot11RMChannelLoadMeasurementActivated,

dot11RMNoiseHistogramMeasurementActivated,

dot11RMStatisticsMeasurementActivated,

dot11RMLCIMeasurementActivated,

dot11RMLCIAzimuthActivated,

dot11RMTransmitStreamCategoryMeasurementActivated,

dot11RMTriggeredTransmitStreamCategoryMeasurementActivated,

dot11RMAPChannelReportActivated,

dot11RMMIBActivated,

dot11RMMaxMeasurementDuration,

dot11RMNonOperatingChannelMaxMeasurementDuration,

dot11RMMeasurementPilotTransmissionInformationActivated,

dot11RMMeasurementPilotActivated,

dot11RMNeighborReportTSFOffsetActivated,

dot11RMRCPIMeasurementActivated,

dot11RMRSNIMeasurementActivated,

dot11RMBSSAverageAccessDelayActivated,

dot11RMBSSAvailableAdmissionCapacityActivated,

dot11RMAntennaInformationActivated,

dot11FastBSSTransitionImplemented }

STATUS deprecated

DESCRIPTION

"Superseded by dot11SMTbase9.

The SMTbase8 object class provides the necessary support at the

STA to manage the processes in the STA so that the STA may work

cooperatively as a part of an IEEE 802.11 network, when the STA

is capable of multidomain operation. This object group should be

implemented when the multidomain capability option is imple-

mented."

::= { dot11Groups 41 }

dot11PhyOFDMComplianceGroup3 OBJECT-GROUP

OBJECTS { dot11CurrentFrequency,

dot11FrequencyBandsImplemented,

dot11ChannelStartingFactor,

dot11FiveMHzOperationImplemented,

dot11TenMHzOperationImplemented,

dot11TwentyMHzOperationImplemented,

dot11PhyOFDMChannelWidth,

dot11OFDMCCAEDImplemented,

dot11OFDMCCAEDRequired,

dot11OFDMEDThreshold }

STATUS current

DESCRIPTION

"Attributes that configure the OFDM for IEEE 802.11."

::= { dot11Groups 42}

dot11SMTbase9 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityActivated,

dot11CountryString,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateInNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed,

dot11RadioMeasurementImplemented,

dot11RadioMeasurementActivated,

dot11FastBSSTransitionImplemented,

dot11LCIDSEImplemented,

dot11LCIDSERequired,

dot11DSERequired,

dot11ExtendedChannelSwitchActivated }

STATUS deprecated

DESCRIPTION

"Superseded by dot11SMTbase10.

The SMTbase9 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 network, when the STA

is capable of multi-domain operation. This object group should

be implemented when the multi-domain capability option is imple-

mented."

::= { dot11Groups 43 }

dot11PhyAntennaComplianceGroup2 OBJECT-GROUP

OBJECTS {

dot11CurrentTxAntenna,

dot11DiversitySupportImplemented,

dot11CurrentRxAntenna,

dot11AntennaSelectionOptionImplemented,

dot11TransmitExplicitCSIFeedbackASOptionImplemented,

dot11TransmitIndicesFeedbackASOptionImplemented,

dot11ExplicitCSIFeedbackASOptionImplemented,

dot11TransmitIndicesComputationASOptionImplemented,

dot11ReceiveAntennaSelectionOptionImplemented }

STATUS current

DESCRIPTION

"Attributes for Data Rates for IEEE 802.11."

::= { dot11Groups 44 }

dot11MACbase3 OBJECT-GROUP

OBJECTS {

dot11MACAddress,

dot11Address,

dot11GroupAddressesStatus,

dot11RTSThreshold,

dot11ShortRetryLimit,

dot11LongRetryLimit,

dot11FragmentationThreshold,

dot11MaxTransmitMSDULifetime,

dot11MaxReceiveLifetime,

dot11ManufacturerID,

dot11ProductID,

dot11CAPLimit,

dot11HCCWmin,

dot11HCCWmax,

dot11HCCAIFSN,

dot11ADDBAResponseTimeout,

dot11ADDTSResponseTimeout,

dot11ChannelUtilizationBeaconInterval,

dot11ScheduleTimeout,

dot11DLSResponseTimeout,

dot11QAPMissingAckRetryLimit,

dot11EDCAAveragingPeriod,

dot11HTProtection,

dot11RIFSMode,

dot11PSMPControlledAccess,

dot11ServiceIntervalGranularity,

dot11DualCTSProtection,

dot11LSIGTXOPFullProtectionActivated,

dot11NonGFEntitiesPresent, dot11PCOActivated,

dot11PCOFortyMaxDuration,

dot11PCOTwentyMaxDuration,

dot11PCOFortyMinDuration,

dot11PCOTwentyMinDuration }

STATUS current

DESCRIPTION

"The MAC object class provides the necessary support for the

access control, generation, and verification of frame check

sequences (FCSs), and proper delivery of valid data to upper

layers."

::= { dot11Groups 45 }

dot11CountersGroup3 OBJECT-GROUP

OBJECTS {

dot11TransmittedFragmentCount,

dot11TransmittedFrameCount,

dot11FailedCount,

dot11ReceivedFragmentCount,

dot11GroupReceivedFrameCount,

dot11FCSErrorCount,

dot11WEPUndecryptableCount,

dot11TransmittedFrameCount,

dot11QosDiscardedFragmentCount,

dot11AssociatedStationCount,

dot11QosCFPollsReceivedCount,

dot11QosCFPollsUnusedCount,

dot11QosCFPollsUnusableCount,

dot11QoSCFPollsLostCount,

dot11TransmittedAMSDUCount,

dot11FailedAMSDUCount,

dot11RetryAMSDUCount,

dot11MultipleRetryAMSDUCount,

dot11TransmittedOctetsInAMSDUCount,

dot11AMSDUAckFailureCount,

dot11ReceivedAMSDUCount,

dot11ReceivedOctetsInAMSDUCount,

dot11TransmittedAMPDUCount,

dot11TransmittedMPDUsInAMPDUCount,

dot11TransmittedOctetsInAMPDUCount,

dot11AMPDUReceivedCount,

dot11MPDUInReceivedAMPDUCount,

dot11ReceivedOctetsInAMPDUCount,

dot11AMPDUDelimiterCRCErrorCount,

dot11ImplicitBARFailureCount,

dot11ExplicitBARFailureCount,

dot11ChannelWidthSwitchCount,

dot11TwentyMHzFrameTransmittedCount,

dot11FortyMHzFrameTransmittedCount,

dot11TwentyMHzFrameReceivedCount,

dot11FortyMHzFrameReceivedCount,

dot11PSMPUTTGrantDuration,

dot11PSMPUTTUsedDuration,

dot11GrantedRDGUsedCount,

dot11GrantedRDGUnusedCount,

dot11TransmittedFramesInGrantedRDGCount,

dot11TransmittedOctetsInGrantedRDGCount,

dot11BeamformingFrameCount,

dot11DualCTSSuccessCount,

dot11DualCTSFailureCount,

dot11STBCCTSSuccessCount,

dot11STBCCTSFailureCount,

dot11nonSTBCCTSSuccessCount,

dot11nonSTBCCTSFailureCount,

dot11RTSLSIGSuccessCount,

dot11RTSLSIGFailureCount }

STATUS current

DESCRIPTION

"Attributes from the dot11CountersGroup that are not described

in the dot11MACStatistics group. These objects are mandatory."

::= { dot11Groups 46 }

dot11SMTbase10 OBJECT-GROUP

OBJECTS {

dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID,

dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod,

dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityActivated,

dot11CountryString,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateInNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed,

dot11AssociateStation,

dot11AssociateID,

dot11AssociateFailStation,

dot11AssociateFailStatus,

dot11ReassociateStation,

dot11ReassociateID,

dot11ReassociateFailStation,

dot11ReassociateFailStatus,

dot11RadioMeasurementImplemented,

dot11RadioMeasurementActivated,

dot11FastBSSTransitionImplemented,

dot11LCIDSEImplemented,

dot11LCIDSERequired,

dot11DSERequired,

dot11ExtendedChannelSwitchActivated,

dot11HighThroughputOptionImplemented,

dot11RSNAPBACRequired,

dot11PSMPOptionImplemented }

STATUS current

DESCRIPTION

"The SMTbase10 object class provides the necessary support at

the STA to manage the processes in the STA such that the STA may

work cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 47 }

dot11PhyMCSGroup OBJECT-GROUP

OBJECTS {

dot11SupportedMCSTxValue,

dot11SupportedMCSRxValue }

STATUS current

DESCRIPTION

"Attributes for MCS for IEEE 802.11 HT."

::= { dot11Groups 48 }

dot11PhyHTComplianceGroup OBJECT-GROUP

OBJECTS {

dot11HighThroughputOptionImplemented,

dot11FortyMHzOperationImplemented,

dot11FortyMHzOperationActivated,

dot11CurrentPrimaryChannel,

dot11CurrentSecondaryChannel,

dot11HTGreenfieldOptionImplemented,

dot11HTGreenfieldOptionActivated,

dot11ShortGIOptionInTwentyImplemented,

dot11ShortGIOptionInTwentyActivated,

dot11ShortGIOptionInFortyImplemented,

dot11ShortGIOptionInFortyActivated,

dot11LDPCCodingOptionImplemented,

dot11LDPCCodingOptionActivated,

dot11TxSTBCOptionImplemented,

dot11TxSTBCOptionActivated,

dot11RxSTBCOptionImplemented,

dot11RxSTBCOptionActivated,

dot11BeamFormingOptionImplemented,

dot11BeamFormingOptionActivated }

STATUS current

DESCRIPTION

"Attributes that configure the HT for IEEE 802.11."

::= { dot11Groups 49 }

dot11HTMACAdditions OBJECT-GROUP

OBJECTS {

dot11HTOperationalMCSSet,

dot11MIMOPowerSave,

dot11NDelayedBlockAckOptionImplemented,

dot11MaxAMSDULength,

dot11STBCControlFrameOptionImplemented,

dot11LsigTxopProtectionOptionImplemented,

dot11MaxRxAMPDUFactor,

dot11MinimumMPDUStartSpacing,

dot11PCOOptionImplemented,

dot11TransitionTime,

dot11MCSFeedbackOptionImplemented,

dot11HTControlFieldSupported,

dot11RDResponderOptionImplemented }

STATUS current

DESCRIPTION

"Attributes that configure the HT for IEEE 802.11."

::= { dot11Groups 50 }

dot11TransmitBeamformingGroup OBJECT-GROUP

OBJECTS {

dot11ReceiveStaggerSoundingOptionImplemented,

dot11TransmitStaggerSoundingOptionImplemented,

dot11ReceiveNDPOptionImplemented,

dot11TransmitNDPOptionImplemented,

dot11ImplicitTransmitBeamformingOptionImplemented,

dot11CalibrationOptionImplemented,

dot11ExplicitCSITransmitBeamformingOptionImplemented,

dot11ExplicitNonCompressedBeamformingMatrixOptionImplemented,

dot11ExplicitTransmitBeamformingCSIFeedbackOptionImplemented,

dot11ExplicitNonCompressedBeamformingFeedbackOptionImplemented,

dot11ExplicitCompressedBeamformingFeedbackOptionImplemented,

dot11NumberBeamFormingCSISupportAntenna,

dot11NumberNonCompressedBeamformingMatrixSupportAntenna,

dot11NumberCompressedBeamformingMatrixSupportAntenna }

STATUS current

DESCRIPTION

"Attributes that configure the Beamforming for IEEE 802.11 HT."

::= { dot11Groups 51 }

dot11RegulatoryClassesGroup OBJECT-GROUP

OBJECTS { dot11RegulatoryClass,

dot11CoverageClass }

STATUS current

DESCRIPTION

"Attributes that configure the OFDM for IEEE 802.11 in many reg-

ulatory domains."

::= { dot11Groups 52 }

-- Editor's Note: dot11RegulatoryClassesGroup was originally assigned to dot11Groups 29,

-- Editor's Note: but this collides with dot11RSNSMKcachingGroup.  This offset selected on the basis

-- Editor's Note: that it is the best estimate of the next available number once .11p, .11u,

-- Editor's Note:  .11v and .11z have been incorporated.

-- **********************************************************************

-- * Compliance Statements

-- **********************************************************************

dot11Compliance MODULE-COMPLIANCE

STATUS  current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the

IEEE 802.11 MIB."

MODULE  -- this module

MANDATORY-GROUPS {

dot11SMTbase10,

dot11MACbase3, dot11CountersGroup3,

dot11SmtAuthenticationAlgorithms,

dot11ResourceTypeID, dot11PhyOperationComplianceGroup }

GROUP dot11PhyDSSSComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is dsss.  This group is mutually exclusive with the

groups dot11PhyIRComplianceGroup, dot11PhyFHSSComplianceGroup2,

dot11PhyOFDMComplianceGroup3and dot11PhyHRDSSSComplianceGroup."

GROUP dot11PhyIRComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is irbaseband.  This group is mutually exclusive

with the groups dot11PhyDSSSComplianceGroup,

dot11PhyFHSSComplianceGroup2, dot11PhyOFDMComplianceGroup3and

dot11PhyHRDSSSComplianceGroup."

GROUP dot11PhyFHSSComplianceGroup2

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is fhss.  This group is mutually exclusive with the

groups dot11PhyDSSSComplianceGroup, dot11PhyIRComplianceGroup,

dot11PhyOFDMComplianceGroup3and dot11PhyHRDSSSComplianceGroup."

GROUP dot11PhyOFDMComplianceGroup3

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is ofdm. This group is mutually exclusive with the

groups dot11PhyDSSSComplianceGroup, dot11PhyIRComplianceGroup,

dot11PhyFHSSComplianceGroup2 and

dot11PhyHRDSSSComplianceGroup."

GROUP dot11PhyHRDSSSComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is hrdsss. This group is mutually exclusive with

the groups dot11PhyDSSSComplianceGroup,

dot11PhyIRComplianceGroup, dot11PhyFHSSComplianceGroup2 and

dot11PhyOFDMComplianceGroup3."

GROUP dot11PhyERPComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is ERP. This group is mutually exclusive with the

groups dot11PhyIRComplianceGroup and

dot11PhyFHSSComplianceGroup2."

GROUP dot11PhyHTComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType has the value of ht. This group is mutually exclu-

sive with the groups dot11PhyIRComplianceGroup and

dot11PhyFHSSComplianceGroup"

GROUP dot11HTMACAdditions

DESCRIPTION

"The dot11HTMACAdditions group is optional."

GROUP dot11SMTprivacy

DESCRIPTION

"The dot11SMTprivacy group is optional."

GROUP dot11MACStatistics

DESCRIPTION

"The dot11MACStatistics group is optional."

GROUP dot11PhyAntennaComplianceGroup

DESCRIPTION

"The dot11PhyAntennaComplianceGroup group is optional."

GROUP dot11PhyTxPowerComplianceGroup

DESCRIPTION

"The dot11PhyTxPowerComplianceGroup group is optional."

GROUP dot11PhyRegDomainsSupportGroup

DESCRIPTION

"The dot11PhyRegDomainsSupportGroup group is optional."

GROUP dot11PhyAntennasListGroup

DESCRIPTION

"The dot11PhyAntennasListGroup group is optional."

GROUP dot11PhyRateGroup

DESCRIPTION

"The dot11PhyRateGroup group is optional."

GROUP dot11MultiDomainCapabilityGroup

DESCRIPTION

"The dot11MultiDomainCapabilityGroup group is optional."

GROUP dot11RSNAadditions

DESCRIPTION

"The dot11RSNAadditions group is optional."

GROUP dot11RegulatoryClassesGroup

DESCRIPTION

"The dot11RegulatoryClassesGroup group is optional."

GROUP dot11Qosadditions

DESCRIPTION

"The dot11Qosadditions group is optional."

GROUP dot11FTComplianceGroup

DESCRIPTION

"The dot11FTComplianceGroup group is optional."

GROUP dot11PhyAntennaComplianceGroup2

DESCRIPTION

"The dot11PhyAntennaComplianceGroup2 group is optional."

GROUP dot11PhyMCSGroup

DESCRIPTION

"The dot11PhyMCSGroup group is optional."

GROUP dot11TransmitBeamformingGroup

DESCRIPTION

"The dot11TransmitBeamformingGroup group is optional."

-- OPTIONAL-GROUPS { dot11SMTprivacy, dot11MACStatistics,

--    dot11PhyAntennaComplianceGroup, dot11PhyTxPowerComplianceGroup,

--    dot11PhyRegDomainsSupportGroup,

--    dot11PhyAntennasListGroup, dot11PhyRateGroup,

--    dot11MultiDomainCapabilityGroup,

--    dot11PhyFHSSComplianceGroup2, dot11RSNAadditions,

--    dot11RegulatoryClassesGroup, dot11Qosadditions,

--    dot11RMCompliance, dot11FTComplianceGroup

--	  dot11PhyAntennaComplianceGroup2,

--	   dot11HTMACadditions,

--	  dot11PhyMCSGroup,

--	  dot11TransmitBeamformingGroup }

--

::= { dot11Compliances 1 }

-- ********************************************************************

-- * Compliance Statements - RSN

-- ********************************************************************

dot11RSNCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the

IEEE 802.11 RSN MIB."

MODULE -- this module

MANDATORY-GROUPS {

dot11RSNBase }

-- OPTIONAL-GROUPS {dot11RSNPMKcachingGroup }

::= { dot11Compliances 2 }

-- ********************************************************************

-- * Compliance Statements - RM

-- ********************************************************************

dot11RMCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the

IEEE 802.11 MIB for Measurement Services."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTRMRequest,

dot11SMTRMReport,

dot11SMTRMConfig

}

-- OPTIONAL-GROUPS { }

::= { dot11Compliances 3 }

-- **********************************************************************

-- *   End of 802.11 MIB

-- **********************************************************************

END