IEEE P802.11
Wireless LANs

| 11af draft amendment proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-02-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., MS SJ-14-4, San Jose, CA 95134-1706 | +1-408-527-0815 | petere@cisco.com |

Abstract

Submission for candidate P802.11af draft text, revised in response to questions during 802.11 TGaf teleconference March 9th, and Padam Kafle’s emailed comments.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

This amendment’s baseline is IEEE Std 802.11™–2007, as amended by

- Amendment 1 802.11k-2008

- Amendment 2 802.11r-2008

- Amendment 3 802.11y-2008

- Amendment 4 802.11w-2009

- Amendment 5 802.11n-2009

- Draft P802.11p_D10.0

- Draft P802.11s_D4.01

- Draft P802.11u_D8.03

- Draft P802.11v_D9.0

- Draft P802.11z_D7.0

- Draft P802.11aa_D0.04

3. Definitions

EDITORIAL NOTE—The subclause numbering of definitions is of the form “3.af<x>” where <x> is an increasing number. The 802.11 technical editor will assign numbers when merging this list into the baseline document.

Insert new definition 3.af1 retaining the alphabetic ordering:

3.af1 TVWS: Television White Space - TV channels where regulatory authorities permit operation by 802.11 STAs.

4. Abbreviations and acronyms

Insert the following new acronym into Clause 4, while maintaining alphabetic ordering:

TVWS			TV White Space

17. Orthogonal frequency division multiplexing (OFDM) PHY specification

17.3 OFDM PLCP sublayer

17.3.8 PMD operating specifications (general)

17.3.8.3 Operating channel frequencies

17.3.8.3.2 Channel numbering

Change the first sentence of 17.3.8.3.2 as shown:

Channel numbering in TVWS bands is given in regulation, otherwise channel center frequencies are defined at every integral multiple of 5 MHz above Channel starting frequency.

18. High Rate direct sequence spread spectrum (HR/DSSS) PHY

Specification

18.4 High Rate PMD sublayer

18.4.6 PMD operating specifications, general

18.4.6.2 Numbering of operating channels

Change the first sentence of 18.4.6.2 as shown:

The channel numbering in TVWS bands is given in regulation, otherwise channel center frequencies and CHNL_ID numbers shall be as shown in Table 18-9.

20. High Throughput (HT) PHY specification

20.3 HT PLCP sublayer

20.3.15 Channel numbering and channelization

Change the first paragraph as shown:

The STA may operate in the 5 GHz band and/or 2.4 GHz band. When using 20 MHz channels, it uses

channels defined in 17.3.8.3 (5 GHz band) or 18.4.6 (2.4 GHz band). When using 40 MHz channels, it can operate in the channels defined in 20.3.15.1 (Channel allocation in the 2.4 GHz Band), and 20.3.15.2

(Channel allocation in the 5 GHz band) and 20.3.15.2a (Channel allocation in TVWS bands).

Insert new clause 20.3.15.2a as shown:

20.3.15.2a Channel allocation in TVWS bands

Channel center frequencies are defined in regulations that define channel numbers for operation in TVWS bands.

Annex A (normative) Protocol Implementation Conformance Statement (PICS) proforma

A.4 PICS proforma—IEEE Std 802.11-<year>

A.4.3 IUT configuration

Insert the following row in the appropriate place in A.4.3:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFaf | TVWS Operation | Annex I, Annex J | CF6&CF8&C F10&CF11&CF15:O | Yes ℃ No ℃ N/A ℃ |

Insert the following table in the appropriate place in A.4:

### A.4.af TVWS functions

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *WS1 | Fixed STA TVWS Operation | 11.11.3, Annex I, Annex J.2.4 | CFaf:O | Yes ℃ No ℃ N/A ℃ |
| *WS2 | Master STA TVWS Operation | 11.11.3, 11.11.4, Annex I, Annex J.2.4 | CFaf:O | Yes ℃ No ℃ N/A ℃ |
| *WS3 | Client STA TVWS Operation | 11.11.5, Annex I, Annex J.2.4 | CFaf:O | Yes ℃ No ℃ N/A ℃ |
| WS3.1 | Dependent STA TVWS Behavior | 11.11.5, Annex I, Annex J.2.4 | WS3:M | Yes ℃ No ℃ N/A ℃ |

Annex I (normative) Regulatory Classes

I.1 External regulatory references

Change the text in the United States documents field as shown:

FCC CFR47 [B8], Part 15, Sections 15.205, 15.209, and 15.247; and Subpart E, Sections 15.401–

15.407,  Section 15.701-716,  Section 90.210, Section 90.371-383 Section 90.1201–90.1217,

Sections 90.1301–90.1337, Section 95.639, Section 95.1501-1511

Insert the row for Behavior limits set <ANA I.3>, and change the Reserved row in Table I.3 as follows (note that the entire table is not shown here):

EDITORIAL NOTE—The <ANA I.3> will be replaced with a number assigned by the 802.11 Assigned Numbers Authority once that assignment has been made.

Table I.3—Behavior limits sets

| Behavior limits set | United States | Europe | Japan |
| --- | --- | --- | --- |
| <ANA I.3> | A station operating in a TVWS band where broadcast TV operation is primary, and station operation has geolocation database and in-service monitoring requirements. When operating in TVWS, channel numbers are as assigned in regulation. | A station operating in a TVWS band where broadcast TV operation is primary, and station operation has geolocation database and in-service monitoring requirements. When operating in TVWS, channel numbers are as assigned in regulation. | A station operating in a TVWS band where broadcast TV operation is primary, and station operation has geolocation database and in-service monitoring requirements. When operating in TVWS, channel numbers are as assigned in regulation. |
| <ANA I.3> +1 — 255 | Reserved | Reserved | Reserved |

Annex J (normative) Country elements and regulatory classes

J.1 Country information and regulatory classes

Insert rows for Regulatory classes <ANA> through <ANA+9>, and change the Reserved row in Table J.1 as follows (note that the entire table is not shown here):

EDITORIAL NOTE—<ANA> will be replaced with a number assigned by the 802.11 Assigned Numbers Authority once that assignment has been made.

Table J.1—Regulatory classes in the United States

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- |
| <ANA> | Center on center | 40 | — | — | 3, 4, 5, 10, 11, 12, 13, <ANA I.3> |
| <ANA+1> | Center on center | 40 | — | — | 3, 4, 5, 10, 11, 12, 14, <ANA I.3> |
| <ANA+2> | Center on center | 20 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+3> | Center on center | 10 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+4> | Center on center | 5 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+5> | Center on lower bound | 40 | — | — | 3, 4, 5, 10, 11, 12, 13, <ANA I.3> |
| <ANA+6> | Center on lower bound | 40 | — | — | 3, 4, 5, 10, 11, 12, 14, <ANA I.3> |
| <ANA+7> | Center on lower bound | 20 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+8> | Center on lower bound | 10 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+9> | Center on lower bound | 5 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |

NOTE—Channel starting frequency “Center on lower bound” means the center frequency of the 802.11 primary modulation is on the lowest frequency of the TV channel, and “Center on center” means the center frequency of the 802.11 primary modulation is on the center frequency of the TV channel.

J.2 Band-specific operating requirements

Insert the following new section, J.2.4, at the end of Annex J:

J.2.4 TVWS band in the United States (54 MHz to 688 MHz)

STAs operating under the behavior limits set <ANA I.3> in Table I.3 are required to be operated under the control of a TV bands database.

NOTE— There are significant requirements in FCC CFR47 [B8], Sections 15.701-717 that are subjects of petitions for reconsideration:

https://mentor.ieee.org/802.11/dcn/09/11-09-0423-01-0wng-fcc-08-260-petitions-for-reconsideration.xls ;

IEEE 802 Petition for Reconsideration http://fjallfoss.fcc.gov/ecfs/document/view?id=6520201311 . In particular, Sections 15.713-5(TV bands database) and 15.717(TVBDs that rely on spectrum sensing) will not become effective until approval has been given by the Office of Management and Budget.

The registration authority is the FCC’s TV bands database administrator (FCC CFR47 [B8], Sections 15.713-715).

NOTE— IEEE 802.11 terms differ from FCC CFR47 [B8], Section 15.703 Definitions in many particulars, but generally an enabling STA is a Master Mode station, and each dependent STA has only one enabling STA at any time. Generally, dependent STAs are Client stations. Enabling STAs shall be “registered” with a TV bands database and operate based on the information received from the TV bands database. Dependent STA operation with DSE (11.11.5) procedures do not require that DSE enablement Public Action frames be transmitted only in a particular band – allowing other unlicensed bands to be used for DSE enablement and control. DSE procedures do not require that the enabling signal be received in a TVWS band, only that it be received over the air – allowing cellular bands, public safety bands and unlicensed bands to be used for transmitting the enabling signal. An enabling STA may act as a dependent STA in a BSS that operates under the control of another enabling STA –permitting an IT department to operate as the TV bands database interface for all the Master Mode stations IT manages.

Regulations specify the following:

— A registered STA “must” not operate as an enabling STA until the database provides TVWS channels to attempt operation on. After receiving a list of TVWS channels from the database, a registered STA may act as a “Master” STA, and control the operation of dependent STAs.

— Dependent non-AP STAs and dependent APs are “Client” STAs.

— “Client” STAs are not registered, and they “must” operate under the control of an enabling STA.

STAs shall use the following:

— CCA-ED

— CS/CCA

— TPC

— DFS

No STA shall use channel switch announcement.

STAs shall have the following elements set to true:

— dot11LCIDSERequired

— dot11RegulatoryClassesRequired

— dot11SpectrumManagementRequired

— dot11MultiDomainCapabilityEnabled

— dot11ExtendedChannelSwitchEnabled

For dependent STA operation in TVWS, the enabling signal (see 11.11.4) is received directly from a Master Mode station or a dependent AP, and is a frame containing an information element indicating that enablement in TVWS is possible.

STAs shall set the value of dot11DSETransmitDivisor to 512 and the other dot11DSE timer values as shown in Table J.5 (TVWS DSE timer limits).

Table J.5—TVWS DSE timer limits

| Parameter | Seconds |
| --- | --- |
| dot11DSEEnablementTimeLimit | 32 |
| dot11DSEEnablementFailHoldTime | 512 |
| dot11DSERenewalTime | 60 |