IEEE P802.11
Wireless LANs

| TGu Single GAS Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Necati Canpolat | Intel Corp. | JF3-206 2111 NE 25 th Ave, Hillsboro, OR 97124 | 503-264-8014 | necati.canpolat@intel.com |
| Vivek Gupta | Intel Corp | JF3-206 2111 NE 25 th Ave, Hillsboro, OR 97124 | 408-765-7766 | vivek.g.gupta@intel.com |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | 858-525-3693 | gabor.bajko@nokia.com |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA | 630-363-1389 | dstanley@arubanetworks.com |
| Kaberi Banerjee | Self | 5521 Bigoak Drive, San Jose, CA 95129 | 408-973-1278 | kaberi@pacbell.net |
| Stephen McCann | Research in Motion UK Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-905-629-4746 | mmontemurro@rim.com |

Abstract

This submission addresses the following comments from the P802.11u Initial Sponsor Ballot: 313, 329, 332, and 334.  TGu has currently two separate ways of query transport protocols i.e., Native GAS and Non-Native GAS Protocol. The Native-GAS is used for local queries and the Non-Native GAS is used to query the information remotely located at an external server.  This dual query scheme requires the non-AP STA to have, at a minimum, knowledge as to what may be categorized as local information, prior to the enquiry. The Non-AP STA need not know where the information is located. The non-AP STA should not be forced to know if the information is local or remotely located. It unnecessarily makes the information query implementation and behaviour complicated. The same information could be locally or remotely stored and retrieved. TGu seems to have made an implicit assumption about the location of the information, which has led to creation of two methods for query transport. Non-Native GAS is indeed a super set of the two GAS transport mechanisms. This submission combines the Native and Non-Native GAS into a single protocol, GAS.

This document is based on IEEE 802.11u-d8.02.

3. Definitions

Change the following text as shown:

3.273 Information Query Protocol: The query protocol for information retrieval.

Change the following text as shown:

4. Abbreviations and acronyms

Change the abbreviation as following:

IQP 	information query protocol

Change the text in the following sub-clauses as shown:

7.3.2.27 Extended Capabilities information element

Insert the following element and change the Reserved row in Table 7-35a as shown:

| Table 7-35a—Capabilities field |  |  |
| --- | --- | --- |
| Bit(s) | Information | Notes |
| 28 | Interworking | When dot11InterworkingServiceEnabled is true, the Interworking field is set to 1 to indicate the STA supports Interworking Service as described in 11.23. When dot11InterworkingServiceEnabled is false, the Interworking field is set to 0 to indicate the STA does not support this capability. |
| 29 | QoS Map | When dot11QosMapEnabled is true, the QoS Map field is set to 1 to indicate the STA supports QoS Map service as described in 11.23.8. When dot11QosMapEnabled is false, the QoS Map field is set to 0 to indicate the STA does not support this capability. |
| 31 | EBR | When dot11EBREnabled is true, the EBR field is set to 1 to indicate the STA supports EBR as described in 7.3.2.91. When dot11EBREnabled is false, the EBR field is set to 0 to indicate the STA does not support this capability. |
| 32 | SSPN Interface | When dot11SSPNInterfaceEnabled is true, the SSPN Interface field is set to 1 to indicate the AP supports SSPN Interface service as described in 11.23.5. When dot11SSPNInterfaceEnabled is false, the SSPN Interface is set to 0 to indicate the AP does not support this capability. |
| 33 | EAS | When dot11EASEnabled is true, the EAS field is set to 1 to indicate the STA supports the EAS mechanism as described in 11.23.6. When dot11EASEnabled is false, the EAS field is set to 0 to indicate the STA does not support this capability |
| 34 | MSGCF Capability | When dot11MSGCFEnabled is true, the MSGCF Capability field is set to 1 to indicate the non-AP STA supports the MSGCF in 11B. When dot11MSGCFEnabled is false, the MSGCF Capability is set to 0 to indicate the non-AP STA does not support this capability. |
| 35 28 - n | Reserved | All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

7.3.2.90 Advertisement Protocol element

Change the Advertisement Protocol Ids in Table 7-43be.

| Table 7-43be—Advertisement Protocol ID definitions |  |
| --- | --- |
| Name | Value |
| Information Query Protocol | 0 |
| MIH Information Service | 1 |
| MIH Command and Event Services Capability Discovery | 2 |
| Emergency Alert System (EAS) | 3 |
| Location-to-Service Translation Protocol | 4 |
| Reserved | 5-220 |
| Vendor Specific | 221 |
| Reserved | 222-255 |

—	The Information Query Protocol (IQP) supports information retrieval from an information repository on a STA or external network.  IQP is a protocol used by a requesting STA to query another STA (i.e., the receiving STA can respond to queries with or without proxying the query to a server in an external network). See 11.23.3.1.1 for information on IQP procedures.

—	MIH Information Service is a service defined in IEEE 802.21 (see IEEE P802.21-2008) to support information retrieval from an information repository in an external network.

—	MIH Command and Event Services capability discovery is a mechanism defined in IEEE 802.21 (see IEEE P802.21-2008) to support discovering capabilities of command service and event service entities in an external network.

Change the following sub clauses as shown:

7.3.4 Information Query Protocol elements

Information Query Protocol (IQP) elements are defined to have a common format consisting of a 2-octet Info ID field, a 2-octet length field, and a variable-length element-specific information field. Each element is assigned a unique Info ID as defined in this standard. The IQP element format is shown in Figure 7-95o130. See Annex W.1 for informative text on IQP usage.

|  | Info ID | Length | Information |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |
| Figure 7-95o130—Information Query Protocol element format |  |  |  |

Each IQP element in 7.3.4 is assigned a unique 2-octet Info ID. The set of valid Info IDs are defined in Table 7-43bn. The 2-octet Info ID is encoded following the conventions given in 7.1.1.

The Length field specifies the number of octets in the Information field and is encoded following the conventions given in 7.1.1.

The  IQP elements that may be configured are shown in Table 7-43bn.

| Table 7-43bn—Information Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | IQP Info Element (clause) |
| Reserved | 0-255 | n/a |
| IQP Capability List | 256 | 7.3.4.1 |
| Venue Name information | 257 | 7.3.4.2 |
| Emergency Call Number information | 258 | 7.3.4.3 |
| Network Authentication Type information | 259 | 7.3.4.4 |
| Roaming Consortium List | 260 | 7.3.4.5 |
| IP Address Type Availability information | 261 | 7.3.4.7 |
| NAI Realm List | 262 | 7.3.4.8 |
| Secondary NAI Realm List | 263 | 7.3.4.9 |
| 3GPP Cellular Network information | 264 | 7.3.4.10 |
| AP Geo Location | 265 | 7.3.4.11 |
| AP Civic Location | 266 | 7.3.4.12 |
| AP Location Public Identifier URI | 267 | 7.3.4.13 |
| Domain Name List | 268 | 7.3.4.14 |
| Emergency Alert Identifier URI | 269 | 7.3.4.15 |
| TDLS Discovery | 270 | 7.3.4.16 |
| Reserved | 271– 56796 | n/a |
| Information Query Protocol vendor-specific list | 56797 | 7.3.4.6 |
| Reserved | 56798 – 65535 | n/a |

7.3.4.1 Interworking Capability List

The IQP Capability List provides a list of information/capabilities that has been configured on a STA or a remote information repository. The IQP Capability List element is returned in response to a GAS Query Request.

The format of the IQP Capability List is provided in Figure 7-95o131

.

|  | Info ID | Length | Info ID #1 | Info ID #2 (optional) |  | Info ID #n (optional) | IQP Vendor-Specific List #1 (optional) |  | IQP Vendor-Specific List #n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 | 0 or 2 |  | 0 or 2 | variable |  | variable |
| Figure 7-95o131—IQP Capability List format |  |  |  |  |  |  |  |  |  |

The first Info ID is a 2-octet field whose value is drawn from Table 7-43bn corresponding to the IQP Capability List.

The Length is a 2-octet field whose value is equal to 2 times the number of Info IDs following the length, excluding those in IQP Vendor Specific Lists.

Each subsequent Info ID included in the IQP Capability List declares that a subsequent query for that Info ID will return the requested IQP element. Each Info ID returned is one of the Info IDs in Table 7-43bn. The Info ID for IQP Capability List is always included in the IQP Capability List returned in a Native-GAS Query Response. The list does not include any duplicate Info IDs, except possibly the Info ID for the IQP  Vendor-specific list. The Info IDs returned in the IQP Capability List are ordered by increasing Info ID value except for IQP Vendor-specific lists which are always ordered to be at the end.

The IQP Vendor-specific list is defined in 7.3.4.6. The IQP Capability List and IQP Vendor-specific list are structured such that the first 2 octets of IQP Vendor-specific list are interpreted in the IQP Capability List as the Info ID corresponding to the IQP Vendor-specific list. When a IQP Vendor-specific list is present in the IQP Capability List, the IQP Capability List element contains the capabilities of that vendor-specific query protocol.

7.3.4.5 Roaming Consortium List

The Roaming Consortium List provides a list of information about the Roaming Consortium and/or SSPs whose networks are accessible via this AP. This list may be returned in response to a GAS Query Request see 11.23.3.1.1.1. The format of the Roaming Consortium List is provided in Figure 7-95o138.

.

7.3.4.6 Information Query Protocol vendor specific list

The Information Query Protocol vendor-specific list is used to query for information not defined in this standard within a single defined format, so that reserved Info IDs are not usurped for nonstandard purposes and inter operability is more easily achieved in the presence of nonstandard information. The element is in the format shown in Figure 7-95o140.

|  | Info ID | Length | OI | Vendor Specific Content |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable | variable |
| Figure 7-95o140—Information Query Protocol vendor specific query format |  |  |  |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the Information Query Protocol vendor specific list.

7.3.4.7 IP Address Type Availability Information

The IP Address Type Availability information provides non-AP STA with the information about the availability of IP address version and type which could be allocated to the non-AP STA after successful association. This information may be returned in response to a GAS Query Request. The format of the IP Address Type Availability information is shown in Figure 7-95o141.

7.3.4.8 NAI Realm List

The Secondary NAI Realm Pages is a 1-octet field which specifies the total number of non-null Secondary NAI Realm Lists which can be returned in response to a IQP query for Secondary NAI Realm Lists. If the value of NAI Realm Pages is 1, then only 1 non-null Secondary NAI Realm List can be returned and its Info ID value is the first value in Table 7-43bn for Secondary NAI Realm List. If the value of NAI Realm Pages is 2, then only 2 non-null Secondary NAI Realm Lists can be returned and their Info ID values are the first two values in Table 7-43bn for Secondary NAI Realm List, and so on.

7.3.4.9 Secondary NAI Realm List

The NAI Realm List provides a list of NAI realms corresponding to SSPs or other entities whose networks or services are accessible via this AP; optionally included for each NAI realm is a list of one or more EAP Method sub-fields, which that NAI realm uses for authentication. The NAI Realm List may be returned in response to a GAS Query Request. The format of the Secondary NAI Realm List is provided in Figure 7-95o148.

7.3.4.11 AP Geo Location

The AP Geo Location element provides the AP’s location in LCI format, see 7.3.2.22.9. This list element may be returned in response to a GAS Query Request. The format of the AP Geo Location element is provided in Figure 7-95o150.

7.3.4.12 AP Civic Location

The AP Civic Location element provides the AP’s location in Civic format. This list element may be returned in response to a GAS Query Request. The format of the AP Civic Location element is provided in Figure 7-95o151.

7.3.4.13 AP Location Public Identifier URI

The AP Location Public Identifier URI element provides an indirect reference to where the location information for the AP can be retrieved. This list element may be returned in response to a GAS Query Request. The format of the AP Location Public Identifier URI element is provided in Figure 7-95o152.

7.3.4.14 Domain Name List

The Domain Name List element provides a list of domain names corresponding to SSPs whose networks are accessible via the AP. Domain Names in this element are taken from dot11DomainNameTable. This list element may be returned in response to a GAS Query Request. The format of the Domain Name List element is provided in Figure 7-95o153.

7.3.4.16 TDLS Discovery information

The TDLS Discovery information contains information that may be used by STAs to discover TDLS capabilities of a peer STA.The format of the TDLS Discovery information is provided in Figure 7-95o156.

|  | Info ID | Length | OUI | TDLS Discovery Information |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 3 | 0 - 252 |
| Figure 7-95o156—TDLS Discovery information format |  |  |  |  |

## 11.23 WLAN Interworking with External Networks Procedures

Change the following subclauses as shown:

11.23.3 Interworking Procedures: Generic Advertisement Services

This subclause describes the actions and procedures that are used to invoke Generic Advertisement Services (GAS). GAS may be used to enable network selection for STAs where dot11InterworkingServiceEnabled is true. GAS provides transport mechanisms for advertisement services while STAs are in un-associated state as well as the associated state, as defined in 11.3. This is accomplished via the use of Public Action management frames which are class 1 frames. When Management Frame Protection is negotiated, stations shall use individually addressed Protected Dual of Public Action frames instead of individually addressed Public Action frames.

A GAS message exchange may take place between two STAs; one STA transmits a GAS Query and the receiving STA transmits the GAS Query Response as described in 11.23.3.1.

GAS uses GAS Public Action frames for transport of a query request and response. using one of the query protocols in Table 7-43be.GAS shall be supported by a STA when dot11InterworkingServiceEnabled is true and one or more dot11GASAdvertisementID are not null. Non-AP STAs shall not use GAS  Advertisement Protocols unless the Advertisement Protocol ID is included in the Advertisement Protocol element in a beacon or Probe Response frame. The Advertisement Protocol element specifies the Advertisement Protocols that a non-AP STA may use to communicate with advertisement servers, which may be located in a STA or an external network. The Advertisement Protocol identifies the query language used by the advertisement server.

The GAS protocol, which is used to transport Queries and Query Responses, is transparent to the Advertisement Protocol. GAS information delivery is supported only using individually addressed action frames.

11.23.3.1  Information Query Protocol

11.23.3.1.1 Information Query Protocol procedures

Information Query Protocol (IQP) shall be supported by a STA whenever dot11InterworkingServiceEnabled is true.  When Advertisement Prodocol ID is set to IQP in Beacon and Probe Response frames, a non-AP STA may use IQP to retrieve network related information and discover supported services.

A STA accomplishes this by transmitting one or more IQP Info IDs or IQP vendor-specific query elements in the Query Request field in a GAS Initial Request frame. The receiving STA responds to the query using a GAS Initial Response or GAS Comeback Response frames.

IQP frame usage for Infrastructure BSSs and IBSSs shall be in accordance with Table 11-3. Frame usage defines the entities permitted to transmit and receive particular IQP elements. STAs where both dot11InterworkingServiceEnabled is true which are capable of operating in an IBSS shall be capable of requesting a IQP Capability List and returning the IQP Capability List in a GAS message exchange; STA support for all other IQP elements is optional. STAs where  dot11InterworkingServiceEnabled is true shall be capable of returning the IQP Capability List in an IQP GAS message exchange;  STA’s support for all other IQP elements is optional.

| Table 11-3—Information Query Protocol usage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | BSS |  | IBSS |
| Info Name | Info Element (clause) | AP | Non-AP STA | STA |
| IQP Capability List | 7.3.4.1 | T, R | T, R | T, R |
| Venue Name information | 7.3.4.2 | T | R | ---- |
| Emergency Call Number information | 7.3.4.3 | T | R | ---- |
| Network Authentication Type information | 7.3.4.4 | T | R | ---- |
| Roaming Consortium List | 7.3.4.5 | T | R | ---- |
| Information Query Protocol vendor-specific list | 7.3.4.6 | T, R | T, R | T, R |
| IP Address Type Availability information | 7.3.4.7 | T, R | T, R | T, R |
| NAI Realm List | 7.3.4.8 | T | R | T, R |
| Secondary NAI Realm List | 7.3.4.9 | T | R | T, R |
| 3GPP Cellular Network information | 7.3.4.10 | T | R | ---- |
| AP Geo Location | 7.3.4.11 | T | R | T, R |
| AP Civic Location | 7.3.4.12 | T | R | T, R |
| Domain Name List | 7.3.4.14 | T | R | ---- |
| Emergency Alert Identifier URI | 7.3.4.15 | T | R | T, R |
| TDLS Discovery | 7.3.4.16 | T | R | T, R |
| Symbols T IQP element may be transmitted by MAC entity R IQP element may be received by MAC entity --- IQP element is neither transmitted nor received by MAC entity |  |  |  |  |

A STA that encounters an unknown or reserved IQP Info ID value in a GAS frame (see Table 7-57aj) received without error shall ignore that IQP Info ID and shall parse any remaining IQP Info IDs.

A STA that encounters an unknown vendor-specific OI field or subfield in a GAS frame (see Table 7-57aj) received without error shall ignore that field or subfield respectively, and shall parse any remaining fields or subfields for additional information with recognizable field or subfield values.

11.23.3.1.1.1 Roaming Consortium List procedures

The Roaming Consortium List, which contains a set of OIs, can be retrieved from an AP by a non-AP STA using the GAS procedures defined in 11.23.3.2. The list of OIs included in the Roaming Consortium List shall be those OIs in the dot11RoamingConsortiumTable. An AP shall only include an OI in the dot11RoamingConsortiumTable, if in conjunction with an AS, it is capable of successfully authenticating a non-AP STA having valid security credentials for the SSPN identified by that OI. Methods used by the AP to authenticate the non-AP STA include, but are not limited to, RSNA algorithms and Open System authentication.

11.23.3.1.1.2 AP Geo Location procedures

A STA where dot11InterworkingServiceEnabled is true may retrieve an AP's Geo location using GAS procedures in 11.23.3.1. A STA in the associated state should retrieve Geo location information from the AP using the procedures in 11.10.8.

11.23.3.1.1.3 AP Civic Location procedures

A STA where dot11InterworkingServiceEnabled is true may retrieve an AP's Civic location using GAS procedures in 11.23.3.1. A STA in the associated state should retrieve Civic location information from the AP using the procedures in 11.10.8.

11.23.3.1.1.4 AP Procedures for advertising EAP Method associated with an NAI Realm

When dot11RSNAEnabled is true, NAI realms along with their supported authentication methods may be advertised using the NAI Realm List (see 7.3.4.8) and Secondary NAI Realm List(s) (see 7.3.4.9). Each realm may be optionally associated with a set of EAP methods. Each EAP method may be optionally associated with a set of Authentication Parameters. The NAI realm information provides a hint on the methods a STA can establish an association in an RSN IEEE 802.1X environment. If the non-AP STA recognizes the NAI realm, it may attempt authentication even if it believes the EAP methods are incorrect.

A non-AP STA where dot11InterworkingServiceEnabled is true may process the NAI realm list. The selection of the NAI realm the non-AP STA uses for authentication is out of scope of this standard. A non-AP STA requests the NAI Realm list using GAS procedures defined in 11.23.3.2 If the Secondary NAI Realm Pages field in the NAI Realm list returned in response to a GAS query is non-zero, the non-AP STA should post one or more subsequent queries to obtain the Secondary NAI Realm lists.

A non-AP STA having dot11InterworkingServiceEnabled set to true may optionally process the EAP Method list as follows:

—	The EAP Method list provided by the AP shall be in priority order (the most preferred EAP Method is listed first).

—	The credential types help the STA to determine what credentials to use for authentication.

—	The STA should confirm the GAS advertisement after an RSNA is established by performing a GAS query for the NAI Realm List using Protected Dual of Public Action frames.

The policy which determines whether or not a non-AP STA should attempt authentication and/or association with any particular IEEE 802.11 Access Network is outside the scope of this standard.

11.23.3.1.2 Information Query Protocol Procedures at the Requesting  Non-AP STA

The STA sends a GAS query by transmitting a GAS Initial Request frame containing a Dialog Token, an Advertisement Protocol element containing an Advertisement Protocol ID set to IQP, zero or more Information Query Info ID values drawn from Table 7-43bg that are not equal to the vendor-specific value, followed by one or more IQP vendor-specific query elements in the Query Request field.

Upon receipt of an MLME-GAS.request primitive with Advertisement Protocol ID set to IQP, the requesting STA shall engage in a GAS message exchanges as described in 11.23.3.2.1.

| Table 11-4—GAS MLME Primitive’s Encoding of Result Code to Status Code field |  |
| --- | --- |
| StatusCode | ResultCode |
| 59 | ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED |
| 60 | NO_REQUEST_OUTSTANDING |
| 61 | QUERY_RESPONSE_OUTSTANDING |
| 62 | TIMEOUT |
| 63 | QUERY_RESPONSE_TOO_LARGE |
| 64 | PARTIAL_QUERY_RESPONSE_CONFIG |
| 65 | SERVER_UNREACHABLE |
| 66 | REQUEST_INFO_NOT_CONFIGURED |
| 67 | TRANSMISSION_FAILURE |
| 68 | PARTIAL_QUERY_RESPONSE_SIZE |

11.23.3.1.3  Information Query Protocol  Procedures at the Responding STA

Upon receipt of a GAS Initial Request frame with Advertisement Protocol ID set to IQP, an MLME-GAS.indication primitive shall be issued to the STA’s SME. Upon receipt of an MLME-GAS.response primitive, the STA shall transmit a GAS Initial Response frame to the requesting STA according to the  procedures described in 11.23.3.2.2.

- If the query request corresponds to information immediately available on the STA, the STA shall transmit a GAS Initial Response frame containing one or more IQP Info elements corresponding to the query (Table 7-43bg). If the query request Info ID contained a value equal to the Information Query protocol Vendor Specific List value, then one or more Information Query protocol Vendor Specific List elements (see 7.3.4.6) shall be returned in the Query Response field.

- If the query request corresponds to information that is not immediately available on the STA, the STA shall transmit a GAS Initial Response frame with a GAS Comeback Delay set to the value in dot11GASComebackDelay for this Advertisement Protocol and a Query Response Length set to 0. The query result shall be provided in the GAS Comeback.

c)	If all of the InfoIDs in the query requests corresponds to information that is not available on the STA or external server server, the STA shall transmit a directed GAS Initial Response or GAS Comeback Response frame  a Status Code equal to “Request Info Not Configured”, an Advertisement Protocol element containing an Advertisement Protocol ID set to IQP and a Query Response Length set to 0.

d)	If one or more of the Info IDs in the query request corresponds to information that has been configured on the STA or external server and one or more of the Info IDs in the query request corresponds to information that is not available on the STA, the STA shall transmit a directed GAS Initial Response or GAS Comeback Response frame a Status Code equal to “Partial Query Response Config”, an Advertisement Protocol element containing an Advertisement Protocol ID set to IQP and a Query Response containing the available IQP response elements.

NOTE—this behavior facilitates a requesting STA to combine a query for the IQP Capabilities List with a query for other IQP elements into a single query request before it’s known to the requesting STA whether or not they can be provided in a query response. This may lead to a more efficient query and response message exchange.

e)	If the query response is larger than the MMPDU maximum payload size, the STA shall transmit the fragmented query result as described in 11.23.3.2.3. 	.

11.23.3.2 GAS Protocol

A non-AP STA obtains GAS advertisement capability information from Beacon or Probe Response frames. The Advertisement Protocol element indicates the Advertisement Protocol IDs supported in the BSS or IBSS. A non-AP STA transmits a GAS query using GAS Initial Request frames and the receiving STAprovides information on how to receive the query response via a GAS Initial Response frame. The flow of GAS messages is shown in Figure 11-17d.

Figure 11-17d Flow of GAS messages.

11.23.3.2.1 Non-AP STA Procedures to Transmit a GAS Query

Upon receipt of an MLME-GAS.request primitive, the non-AP STA shall engage in the following procedure to transmit a query:

a)	The non-AP STA sends aGAS query by transmitting a GAS Initial Request frame containing a Dialog Token, an Advertisement Protocol element containing an Advertisement Protocol ID and the Query Request field.

b)	Upon transmission of the GAS Initial Request frame, the STA shall set a timer, referred to as the dot11GASResponseTimer, equal to the dot11GASResponseTimeout MIB object or the QueryFailureTimeout parameter provided in the MLME-GAS.request primitive. If both values are present, the timer shall be set to the lesser of the two values.

c)	If the non-AP STA is not in the associated state, it shall remain in active mode until the receipt of a GAS Initial Response frame with the same Dialog Token as in the GAS Initial Request frame or until the expiry of the timer, whichever occurs first. If the non-AP STA is in the associated state, it may go into power save state until the GAS Initial Response frame is available for receipt or the timer expiry, whichever occurs first.

d) 	If a GAS Initial Response frame is received with a status value of “successful”, and the GAS Comeback Delay is set to 0, then the query was successful and the query result is provided in the GAS Initial Response frame. The MLME shall issue an MLME-GAS.confirm primitive indicating successful completion of the query along with the query response.

e)	If a GAS Initial Response frame is received with a status value equal to “successful”, and the GAS Comeback Delay is not zero, then the GAS query was successfully sent, the query result was not immediately available and the non-AP STA shall use the procedures outlined in 11.23.3.2.3 to retrieve the query response. Upon reception of the GAS Initial Response frame with a status value equal to “successful”, the non-AP STA shall reset the dot11GASResponseTimer to the value in the dot11GASResponseTimeout MIB object.

f)	If a GAS Initial Response frame is received with a status value equal to “Advertisement Protocol Not Supported”, the GAS query was not successful and the MLME shall issue an MLME-GAS.confirm primitive with status so indicating and shall set the Query Response Length field to 0.

g)	If a GAS Initial Response frame is received with a status value equal to “Server unreachable”, the GAS query was not successful and the MLME shall issue an MLME-GAS.confirm primitive with status so indicating and shall set the Query Response Length field to 0.

h)	If the dot11GASResponseTimer expires before a GAS Initial Response frame is received, the  GAS query was not successful and the MLME shall issue an MLME-GAS.confirm primitive indicating “timeout” and shall set the Query Response Length field to 0.

11.23.3.2.2 STA Procedures to Respond to a GAS Initial Request query

Upon receipt of a GAS Initial Request frame, an MLME-GAS.indication primitive shall be issued to the STA’s SME. Upon receipt of an MLME-GAS.response primitive, the STA shall transmit a GAS Initial Response frame to the requesting non-AP STA according to the following procedures. If the requesting non-AP STA is in the associated state and in the power-save mode, the STA shall buffer the frame for transmission according to the procedures in 11.2.1; otherwise the STA shall queue the frame for transmission.

a)	If the Advertisement Protocol ID in the Advertisement Protocol element does not equal the value contained in any dot11GASAdvertisementID MIB object, then the STA shall not post the query to an internal or external advertisement server. The STA shall transmit a directed GAS Initial Response frame to the requesting non-AP STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code equal to “Advertisement Protocol Not Supported” (see Table 11-4), an Advertisement Protocol element corresponding to the Advertisement Protocol ID contained in the GAS Initial Request frame and a Comeback Delay and Query Response Length both set to 0.

b)	If the query request corresponds to an advertisement protocol whose server is currently unreachable, the STA shall transmit a directed GAS Initial Response frame to the requesting non-AP STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code equal to “Server Unreachable”, an Advertisement Protocol element containing an Advertisement Protocol ID corresponding to the Advertisement Protocol ID contained in the GAS Initial Request frame and a Comeback Delay and Query Response Length both set to 0. The method used by the AP to determine the server is unreachable is out of scope of this specification. A non-AP STA receiving a status code indicating the server is unreachable should wait at least 1 minute before transmitting any further queries using the same Advertisement Protocol ID to any AP in the homogeneous ESS.

c)	If the Advertisement Protocol ID in the Advertisement Protocol element equals the value contained in any dot11GASAdvertisementID MIB object, then the AP shall post the query to the local or remote advertisement server in an external network. The methods and protocols the AP uses to post the query are outside the scope of this specification.

d)	If the query result is immediately available and its length is equal to or less than the maximum MMPDU size, the STA shall transmit an individually addressed GAS Initial Response frame to the requesting non-AP STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to “success”, an Advertisement Protocol element corresponding to the Advertisement Protocol ID contained in the GAS Initial Request frame, a GAS Comeback Delay set to 0 indicating the query is completed and the query result is contained in the GAS Initial Response frame and a Query Response Length set to the query response length.

e)	If the query result is immediately available but its length is larger than the maximum MMPDU, the STA shall transmit an individually addressed GAS Initial Response frame to the requesting non-AP STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to “success”, an Advertisement Protocol element corresponding to the Advertisement Protocol ID contained in the GAS Initial Request frame, a GAS Comeback Delay set to the value in dot11GASComebackDelay for this Advertisement Protocol and a Query Response Length set to 0, indicating the query resulted in fragmentation and the responding STA shall queue and transmit the query result as described in 11.23.2.3. The requesting STA shall retrieve the query result as described in 11.23.2.4.

f)	If the query result is not immediately available,  the STA initializes a timer, referred to as the PostReplyTimer, to the value in dot11GASResponseTimeout MIB object and transmits an individually addressed GAS Initial Response frame to the requesting non-AP STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to “success”, an Advertisement Protocol element corresponding to the Advertisement Protocol ID contained in the GAS Initial Request frame, a GAS Comeback Delay set to the value in dot11GASComebackDelay for this Advertisement Protocol and a Query Response Length set to 0.

11.23.3.2.3 STA Procedures for delivering a GAS Query Response

If the query result is immediately available at the STA but its length is larger than maximum MMPDU, the STA shall buffer the query response for a minimum of dot11GASResponseBufferingTime and process it as if the query result is not available as described below.

If the query result is not immedieately avaliable at the STA, after receiving a query response from a server the STA shall buffer the query response for a minimum of dot11GASResponseBufferingTime after the expiry of the GAS Comeback Delay. If the STA does not receive a GAS Comeback Request frame whose source MAC address and Dialog Token match the source MAC address and Dialog Token respectively of the corresponding GAS Initial Response frame within this time, it may drop the query response.

If the query result is larger than the configured Query Response Length Limit, it shall discard the response and instead return a status code of “GAS Query Response larger than permitted per configured STA policy” in the GAS Comeback Response frame. This behavior helps to prevent abuses of the medium which may be caused by overly general queries, which evoke a very large query response.

The GAS protocol supports Query Responses whose length is greater than the 802.11 maximum MMPDU size by the AP’s use of the GAS Query Response Fragment ID field in the GAS Comeback Response frame; the Query Response Fragment ID shall be set to 1 for the initial fragment and incremented by 1 for each subsequent fragment in a multi-fragment query response. If the Query Response is a multi-fragment response (i.e., contains more than 1 fragment), the STA shall transmit all fragments that belong to the same Query Response until all fragments are exhausted. The STA shall set the More GAS Fragments field of the GAS Query Response Fragment ID to 0 when the transmitted fragment is the final fragment.

A STA shall use the following procedures to deliver a GAS Query Response.

Upon receipt of a GAS Comeback Request frame, an MLME-GAS.indication primitive shall be issued to the STA’s SME. Upon receipt of an MLME-GAS.response primitive, the STA shall transmit a GAS Comeback Response frame to the requesting STA according to the following procedures. If the requesting non-AP STA is in the associated state and in the power-save mode, the AP shall buffer the frame for transmission according to the procedures in 11.2.1; otherwise the STA shall queue the frame for transmission.

a)	If the PostReplyTimer expires before the GAS Query Response is received from the advertisement server then the STA shall buffer for transmission a GAS Comeback Response frame with a status code equal to “Timeout” (see Table 11-4). If the query response is subsequently received from the server, it shall be dropped by the STA.

b)	If the Query Response received from the server is larger than dot11GASQueryResponseLengthLimit, it shall be dropped by the AP. Then the STA shall buffer for transmission a GAS Comeback Response frame with status code set to “Query Response too large”.

c)	If the Query Response is received before the expiry of the PostReplyTimer and its length is less than dot11GASQueryResponseLengthLimit, then the Query Response shall be buffered in one or more GAS Comeback Response frames with a status code set to “success”. The STA transmits one GAS Comeback Response frame in response to each GAS Comeback Request frame. If the Query Response received from the external network is less than or equal to the maximum MMPDU payload size, then the GAS Query Response Fragment ID shall be set to zero and the More GAS Fragments field in the GAS Query Response Fragment ID shall be set to zero. If the Query Response received from the external network is greater than the maximum MMPDU payload size, then the GAS Query Response Fragment ID shall be set to zero if this is the first fragment of the Query Response transmitted, otherwise it shall be incremented by 1; the More GAS Fragments field in the GAS Query Response Fragment ID shall be set to one if there are more fragments of the Query Response to be transmitted, otherwise it shall be set to zero (i.e., this fragment is the last fragment of the Query Response).

d)	If a query response has not been received and the PostReplyTimer has not expired, the STA shall transmit a GAS Comeback Response frame with status equal to “Query response outstanding” (see Table 11-4) and GAS Comeback Delay set to the value in dot11GasComebackDelay for this Advertisement Protocol to indicate when the non-AP STA should comeback to obtain its Query Response.

e)	If an STA receives a GAS Comeback Request frame whose source MAC address and Dialog Token do not match the destination MAC address and Dialog Token respectively of an outstanding GAS Initial Response frame, the STA should transmit a GAS Comeback Response frame with a status code equal to “No request outstanding”.

11.23.3.2.4 Non-AP STA Procedures to retrieve a GAS Query Response

If the GAS query result is not immediately available, a non-AP STA shall transmit a GAS Comeback Request frame including the Dialog Token (drawn from the corresponding GAS Initial Response frame) immediately after the expiry of the GAS Comeback Delay. In response, the STA provides the Query Response in one or more GAS Comeback Response frames with the corresponding Dialog Token.

If a non-AP STA receives a GAS Comeback Response frame with status set to “Query response outstanding”, the non-AP STA shall wait for the GAS Comeback Delay from that frame and upon expiry of the GAS Comeback Delay, transmit another GAS Comeback Request frame. If the non-AP STA’s dot11GASResponseTimer (set in 11.23.3.2.1 step b) expires prior to receiving a GAS Comeback Response frame whose source MAC address and Dialog Token match those in the corresponding GAS Initial Response frame, the STA shall issue an MLME-GAS.confirm primitive with result code set to “timeout” and shall set the Query Response Length to 0.

If a non-AP STA receives a GAS Comeback Response frame with status set to “success” and the More GAS Fragments field in the GAS Query Response Fragment ID set to one, it shall transmit another GAS Comeback Request frame in order to retrieve the next GAS fragment of a multi-fragment query response.

If a non-AP STA receives a GAS Comeback Response frame with status set to “success” and the More GAS Fragments field in the GAS Query Response Fragment ID set to zero, the non-AP STA’s MLME shall determine that all fragments have been received by confirming that all fragment IDs from 0 to the value in the GAS Query Response Fragment ID when the More GAS Fragments field was set to 0 have been received. Upon receipt of the first GAS Comeback Response frame and every GAS Comeback Response frame thereafter, the dot11GASResponseTimer shall be reset. If all of the query response fragments were received before the expiry of the dot11GASResponseTimer, then the MLME shall issue an MLME-GAS.confirm with result code set to “success” along with the query response. If all of the query response fragments were not received before the expiry of the dot11GASResponseTimer, then the MLME shall issue an MLME-GAS.confirm with result code set to “transmission failure” and shall set the Query Response Length to 0.

After a non-AP STA receives the first GAS fragment of a multi-fragment query response, it shall continue retrieving the query response until all GAS fragments are received or until a transmission failure is detected; the non-AP STA shall not commence the retrieval of a another non-native GAS Query Response from the same STA until all GAS fragments are received or until a transmission failure is detected on the first GAS Query Response.

If a non-AP STA receives a GAS Comeback Response with status set to “Timeout” or “Query Response too large”, then the MLME shall issue an MLME-GAS.confirm with result code so indicating and shall set the Query Response Length to 0.

If a non-AP STA receives a GAS Comeback Response with status set to “No request outstanding”, then the MLME shall issue an MLME-GAS.confirm with result code set to “unspecified failure” and shall set the Query Response Length to 0.

11.23.3.2.5 GAS procedures interaction with Multiple BSSID Set

Non-AP STAs in the un-associated state may use GAS procedures to query servers in an external network for information. As described in 11.23.3.2, APs indicate their support for a particular GAS Advertisement Protocol by including an Advertisement protocol element with that Advertisement protocol ID in Beacon and Probe Response frames as described in 7.2.3.1 and 7.2.3.9 respectively. Non-AP STAs receiving Beacon or Probe Response frames from different APs may choose to engage in GAS frame exchange sequences with one or more of these APs. In some deployment scenarios, these APs may be operating as a Multiple BSSID set (as defined in 11.10.11) and may relay the GAS queries to the same logical advertisement server. Depending on the configuration of the IEEE 802.11 access network, the external network and the advertisement server, a query response from the advertisement server may or may not be dependent on the BSSID used in the GAS frame exchange sequence and thus the STA from which the query was relayed. If the GAS Query Response is dependent on the BSSID, a non-AP STA may choose to post queries using GAS procedures to more than one STA and expect possibly different Query Responses. If the Query Response is not dependent on the BSSID, then a non-AP STA may choose to post queries using GAS procedures to only one STA in the Multiple BSSID set (i.e., posting the same query to another member of the Multiple BSSID set would yield the same response).

When a Multiple BSSID (as defined in 11.10.11) set contains two or more members and dot11InterworkingServiceEnabled is true and dot11GASAdvertisementID is not null and a query to the advertisement server corresponding to the value of dot11GASAdvertisementID is not dependent on the BSSID value used in the GAS frame exchange sequence to post the query, then the PAME-BI bit in the Advertisement protocol tuple of the Advertisement protocol element corresponding to the value of dot11GASAdvertisementID shall be set to 1; otherwise this bit shall be set to zero.