IEEE P802.11
Wireless LANs

| TGu Single GAS Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-015 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Necati Canpolat | Intel Corp. | JF3-206 2111 NE 25 th Ave, Hillsboro, OR 97124 | 503-264-8014 | necati.canpolat@intel.com |
| Vivek Gupta | Intel Corp | JF3-206 2111 NE 25 th Ave, Hillsboro, OR 97124 | 408-765-7766 | vivek.g.gupta@intel.com |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | 858-525-3693 | gabor.bajko@nokia.com |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA | 630-363-1389 | dstanley@arubanetworks.com |
| Kaberi Banerjee | Self | 5521 Bigoak Drive, San Jose, CA 95129 | 408-973-1278 | kaberi@pacbell.net |
| Stephen McCann | Research in Motion UK Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-905-629-4746 | mmontemurro@rim.com |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This submission addresses the following comments from the P802.11u Initial Sponsor Ballot: 313, 329, 332, and 334.  TGu has currently two separate ways of query transport protocols i.e., Native GAS and Non-Native GAS Protocol. The Native-GAS is used for local queries and the Non-Native GAS is used to query the information remotely located at an external server.  This dual query scheme requires the non-AP STA to have, at a minimum, knowledge as to what may be categorized as local information, prior to the enquiry. The Non-AP STA need not know where the information is located. The non-AP STA should not be forced to know if the information is local or remotely located. It unnecessarily makes the information query implementation and behaviour complicated. The same information could be locally or remotely stored and retrieved. TGu seems to have made an implicit assumption about the location of the information, which has led to creation of two methods for query transport. Non-Native GAS is indeed a super set of the two GAS transport mechanisms. This submission combines the Native and Non-Native GAS into a single protocol, GAS.

This document is based on IEEE 802.11u-d8.02.

3. Definitions

Change the following text as shown:

3.267 advertisement protocol: IEEE information query protocol and higher layer protocols defined external to 802.11 that can be used for network and service discovery.

3.268 advertisement server: a logical server which provides the information respository for a specific advertisement protocol.  The location of the physical server which instantiates the advertisement server is outside the scope of this specification.

3.273 Information Query Protocol: The query protocol for information retrieval.

Change the following text as shown:

4. Abbreviations and acronyms

Change the abbreviation as following:

IQP 	information query protocol

## 5.4 Overview of the services

Insert the following subclause 5.4.8 after 5.4.7

5.4.8 Interworking with External Networks

The Interworking Service allows non-AP STAs to access services provided by an external network according to the subscription or other characteristics of that external network. An IEEE 802.11 non-AP STA may have a subscription relationship with an external network, e.g., with an SSPN.

An overview of the interworking functions addressed in this standard is provided below:

—	Network Discovery and Selection

•	Discovery of suitable networks through the advertisement of access network type, roaming consortium and venue information, via management frames

•	Selection of a suitable IEEE 802.11 infrastructure using advertisement services (e.g., an IEEE 802.21 Information Server) in the BSS or in an external network reachable via the BSS

•	Selection of an SSPN or External Network with its corresponding IEEE 802.11 infrastructure

—	Emergency Services

•	Emergency Call and Network Alert support at the link level

—	QoS Map distribution

—	SSPN Interface service between the AP and the SSPN

The Generic Advertisement Service (GAS), described in 5.9, can be used by a STA to provide support for the network selection process and as a conduit for communication by a non-AP STA with other information resources in a network before joining the wireless LAN.

The Interworking Service supports Emergency Services (ES) by providing methods for users to access emergency services via the IEEE 802.11 infrastructure, advertising that emergency services are supported (see 11.23.6) and identifying that a traffic stream is used for emergency services.

The Interworking Service provides QoS mapping for SSPNs and other external networks. Since each SSPN or other external network may have its own layer-3 end-to-end packet marking practice (e.g., DSCP usage conventions), a means to re-map the layer-3 service levels to a common over-the-air service level is necessary. The QoS Map service provides STAs a mapping of network-layer QoS packet marking to over-the-air QoS frame marking (i.e., user priority).

The SSPN Interface service supports service provisioning and transfer of user permissions from the SSPN to the AP. The method and protocol by which these permissions are transferred from the SSPN are out of scope of this standard.

## 5.9 Generic Advertisement Service

The Generic Advertisement Service (GAS) provides functionality that enables STAs to discover the availability of information related to desired network services, e.g., information about services such as provided in an IBSS, local access services, available Subscription Service Providers (SSP) and/or SSPNs or other external networks. GAS uses a generic container to advertise network services' information over an IEEE 802.11 network. Public Action frames are used to transport this information.

While the specification of network services information is out of scope of IEEE 802.11, in an Infrastructure BSS there is a need for STAs to query for information on network services provided by SSPNs or other external networks beyond an AP, before they associate to the wireless LAN.

In an IBSS, GAS functionality enables a STA to access the availability and information related to desired services provided by other STAs in the IBSS. Exchange of information using GAS may be performed either prior to joining an IBSS or after joining the IBSS.

There are a number of reasons why providing information to a STA in a pre-associated state is beneficial:

—	It supports more informed decision making about an IEEE 802.11 infrastructure with which to associate. This is generally more efficient than requiring a non-AP STA to associate with an AP before discovering the information and then deciding whether or not to stay associated.

—	It is possible for the non-AP STA to query multiple networks in parallel.

—	The non-AP STA can discover information about APs that are not part of the same administrative group as the AP with which it is associated, supporting the selection of an AP belonging to a different IEEE 802.11 infrastructure that has an appropriate SSP roaming agreement in place.

7.3.1.9 Status Code field

Insert the status codes 59 thru 68 and change the Reserved category code row in Table 7-24 as follows (note the entire table is not shown here):

| Change the text in the following sub-clauses as shown: Table 7-23—Status Codes |  |
| --- | --- |
| Status Code | Meaning |
| 59 | GAS Advertisement Protocol not supported |
| 60 | No outstanding GAS request |
| 61 | GAS Response not received from the advertisement server |
| 62 | STA timed out waiting for GAS Query Response |
| 63 | GAS Response is larger than query response length limit |
| 64 | Advertisement server in the network is not currently reachable |
| 65 | Requested information is not available for this BSSID |
| 66 | Transmission failure |
| 67 | Request refused due to permissions received via SSPN interface |
| 68 | Request refused because AP does not support Emergency Services |
| 69 -59 65535 | Reserved |

7.3.1.33 GAS Query Response Fragment ID

A GAS Query Fragment Response ID is used by the STA to indicate when a GAS Query Response spans multiple MMPDUs. STAs use this field to inform the non-AP STA of the GAS fragment number of the transmitted frames as well as identifying the last GAS fragment of the Query Response. Non-AP STAs use this field to determine if any fragments of the Query Response are missing. The maximum value permitted in the GAS Query Response Fragment ID is 127. The More GAS Fragments field is set to 1 in GAS Query Response fragments of GAS Comeback Response frames that have another GAS fragment of the current query response to follow; otherwise, it is set to 0. The format of GAS Query Response Fragment ID is shown in Figure 7-36r.

|  | B0 | B6 | B7 |
| --- | --- | --- | --- |
|  | GAS Query Response Fragment ID |  | More GAS Fragments |
| Bits: | 7 |  | 1 |
| Figure 7-36r—GAS Query Response Fragment ID |  |  |  |

7.3.2.90 Interworking information element

The Interworking information element contains information about the Interworking Service capabilities of a STA as shown in Figure 7-95o119.

|  | Element ID | Length | Access Network Options | Venue Info (optional) | HESSID (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 0 or 2 | 0 or 6 |
| Figure 7-95o119—Interworking element format |  |  |  |  |  |

The Length is a one-octet field whose value is 1 plus the sum of the lengths of each optional field present in the element.

The format of Access Network Options field is shown in Figure 7-95o120.

| Bits: | B0 - B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- |
|  | Network Type Codes | Internet | ASRA | Reserved | UESA |
| Figure 7-95o120—Access Network Options format |  |  |  |  |  |

A non-AP STA sets Internet, ASRA and UESA fields to 0 when including the Interworking element in the Probe Request frame. A non-AP STA sets the Internet, and ASRA bits to 0 when including the Interworking element in (Re)association Request frames. In (Re)association Request frames, a non-AP STA sets the UESA bit according to the procedures in 11.23.6. The Network Type Codes are shown in Table 7-43bk. The Network Type Codes field is set by the AP to advertise its Network Type to non-AP STAs. A non-AP STA uses this field to indicate the desired Network Type in an active scan. See Annex W.1 for informative text on usage of fields contained within the Interworking element.

| Table 7-43bk—Network Type Codes |  |  |
| --- | --- | --- |
| Network Type Codes | Meaning | Description |
| 0 | Private network | Non-authorized users are not permitted on this network. Examples of this network type are home networks and enterprise networks, which may employ user accounts. Private networks do not necessarily employ encryption. |
| 1 | Private network with guest access | Private network but guest accounts are available. Example of this network type is enterprise network offering access to guest users. |
| 2 | Chargeable public network | The network is accessible to anyone, however, access to the network requires payment. Further information on types of charges may be available through other methods (e.g., 802.21, http/https redirect or DNS redirection). Examples of this network type is a hotspot in a coffee shop offering internet access on a subscription basis or a hotel offering in-room internet access service for a fee. |
| 3 | Free public network | The network is accessible to anyone and no charges apply for the network use. An example of this network type is an airport hotspot or municipal network providing free service. |
| 4 | Personal Device Network | A network of personal devices. An example of this type of network is a camera attaching to a printer, thereby forming a network for the purpose of printing pictures. |
| 5 | Emergency Services Only Network | A network dedicated and limited to accessing emergency services. |
| 6 to 13 | Reserved | Reserved |
| 14 | Test or experimental | The network is used for test or experimental purposes only. |
| 15 | Wildcard | Wildcard network type |

Bit 4 is the Internet field. The AP sets this field to 1 if the network provides connectivity to the Internet; otherwise it is set to 0 indicating that it is unspecified whether the network provides connectivity to the Internet.

Bit 5 is the Additional Step Required for Access (ASRA) field. It is set to 1 by the AP to indicate that the network requires a further step for access. It is set to 0 whenever dot11RSNAEnabled is true. For more information, refer to Network Authentication Type Information in 7.3.4.4.

Bit 7 is the Unauthenticated Emergency Service Accessible (UESA) field. When the AP sets it to 0, this field indicates that no unauthenticated emergency services are reachable through this AP. When set to 1, this field indicates that higher layer unauthenticated emergency services are reachable through this AP. A STA uses the Interworking information element with the UESA bit set to 1 to gain unauthenticated access to a BSS to access emergency services. See 11.23.6.

The Venue Info field is defined in 7.3.1.34.

The HESSID field, which is the identifier for a homogeneous ESS, specifies the value of HESSID, see 11.23.2. A STA uses this field to indicate the desired HESSID in an active scan per 11.1.3. The HESSID field for an AP is set to the value of dot11HESSID.

7.3.2.91 Advertisement Protocol element

The Advertisement Protocol element contains information that identifies a particular advertisement protocol and its corresponding Advertisement Control. The Advertisement Protocol element format is shown in Figure 7-95o121.

|  | Element ID | Length | Advertisement Protocol Tuple # 1 | Advertisement Protocol Tuple # 2 (optional) | ... | Advertisement Protocol Tuple # n (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | variable | variable |  | variable |
| Figure 7-95o121—Advertisement Protocol element format |  |  |  |  |  |  |

The Length is a one-octet field whose value is equal to the sum of the lengths of the Advertisement Protocol Tuple fields.

The format of Advertisement Protocol Tuple is shown in Figure 7-95o122.

|  | B0 - B6 | B7 |  |
| --- | --- | --- | --- |
|  | Query Response Length Limit | PAME-BI | Advertisement Protocol ID |
| Octets: | 1 |  | variable |
| Figure 7-95o122—Advertisement Protocol Tuple format |  |  |  |

The Advertisement Protocol Tuple field is defined as follows:

—	The Query Response Length Limit indicates the maximum number of octets an AP will transmit in the Query Response field contained within one or more GAS Comeback Response frames. The Query Response Length Limit may be set to a value larger than the maximum MMPDU size in which case the Query Response spans multiple MMPDUs. The Query Response Length Limit is encoded as an integer number of 256 octet units. A value of zero is not permitted. A value of 0x7F means the maximum limit enforced is determined by the maximum allowable number of fragments in the GAS Query Response Fragment ID (see 7.3.1.33). The non-AP STA sets the Query Response Length Limit to zero on transmission and the AP ignores it upon reception.

—	Bit 7, the Pre-Association Message Exchange BSSID Independent (PAME-BI) bit, is used by an AP to indicate whether the Advertisement server, which is the non-AP STA’s peer for this advertisement protocol, will return a Query Response which is independent of the BSSID used for the GAS frame exchange. This bit is set to 1 to indicate the Query Response is independent of the BSSID; it is set to zero to indicate that the Query Response may be dependent on the BSSID. See 11.23.3.2.4 for further information. Bit 7 is reserved for non-AP STAs.

—	The Advertisement Protocol ID is a variable length field. When this field contains a vendor specific Advertisement Protocol ID then this field will be structured per the Vendor Specific information element defined in 7.3.2.26, where the Element ID of the Vendor Specific information element of —	7.3.2.26 is the first octet of the field and contains the vendor specific value for Advertisement Protocol ID defined in Table 7-43bl; otherwise its length is one octet and its value is one of the values in Table 7-43bl. When one or more vendor-specific tuples are included in the Advertisement Protocol element, their total length needs to be constrained such that the total length of all the Advertisement Protocol Tuple fields (both vendor specific and otherwise) is less than or equal to 255 octets.

Change the Advertisement Protocol Ids in Table 7-43be.

| Table 7-43be—Advertisement Protocol ID definitions |  |
| --- | --- |
| Name | Value |
| Information Query Protocol | 0 |
| MIH Information Service | 1 |
| MIH Command and Event Services Capability Discovery | 2 |
| Emergency Alert System (EAS) | 3 |
| Location-to-Service Translation Protocol | 4 |
| Reserved | 5-220 |
| Vendor Specific | 221 |
| Reserved | 222-255 |

—	The Information Query Protocol (IQP) supports information retrieval from an information repository on a STA or external network.  IQP is a protocol used by a requesting STA to query another STA (i.e., the receiving STA can respond to queries with or without proxying the query to a server in an external network). See 11.23.3.1.1 for information on IQP procedures.

—	MIH Information Service is a service defined in IEEE 802.21 (see IEEE P802.21-2008) to support information retrieval from an information repository.

—	MIH Command and Event Services capability discovery is a mechanism defined in IEEE 802.21 (see IEEE P802.21-2008) to support discovering capabilities of command service and event service entities in a STA or an external network.

—	The Emergency Alert System (EAS) service allows a network to disseminate emergency alert notifications from an external network to unauthenticated or unassociated or associated non-AP STAs. To provide a standardized alerting system, EAS uses the Common Alerting Protocol (CAP) (see OASIS CAP) carrying EDXL (see OASIS EDXL) formatted messages. Utilizing GAS and EAS Advertisement Protocol ID, CAP and EDXL can operate transparently over the air interface. The structure of the CAP Alert Message is defined in 1.3 of OASIS CAP. The message format itself is defined in 3.2 of OASIS EDXL, which is a special emergency type of XML message. The underlying transport mechanism in IEEE 802.11 networks for CAP is HTTP.

—	Location-to-Service Translation Protocol (LoST) is used by a non-AP STA to access information from PSAP databases, for example a local emergency dial-string. It is also used to determine the location-appropriate PSAP URI for emergency services. The operation and message format is defined in RFC 5222. The underlying transport mechanism for LoST is HTTP.

—	Advertisement Protocol ID 221 is reserved for Vendor Specific advertisement protocols. When the Advertisement Protocol ID is equal to 221, the format of the Advertisement Protocol ID subfield follows the format of the vendor specific information element in 7.3.2.26.

7.3.2.94 Roaming Consortium information element

The Roaming Consortium Information element contains information identifying the roaming consortium and/or SSP whose security credentials can be used to authenticate with the AP transmitting this element; see 11.23.8. The element’s format is shown in Figure 7-95o127.

|  | Element ID | Length | Number of IQP OIs | OI #1 and #2 Lengths | OI #1 (optional) | OI #2 (optional) | OI #3 (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable | variable | variable |
| Figure 7-95o127—Roaming Consortium information element format |  |  |  |  |  |  |  |

The Length is a one-octet field whose value is equal to 2 plus the sum of the number of octets in each OI field present.

The Number of IQP OIs field’s format is a one-octet unsigned integer whose value is the number of additional roaming consortium OIs obtainable via IQP. A value of zero means that no additional OIs will be returned in response to a IQPquery for the Roaming Consortium List. A value of 255 means that 255 or more additional OIs are obtainable via IQP.

The OI #1 and #2 Lengths field format is shown in Figure 7-95o128.

—	The value of the OI #1 Length subfield is the length in octets of the OI #1 field.

—	The value of the OI #2 Length subfield is the length in octets of the OI #2 field. If the OI #2 field is not present, the value of the OI #2 Length subfield is set to zero.

Note—When there are three OIs, the OI #3 Length is calculated by subtracting sum of 2 plus the value of the OI #1 Length subfield plus the value of the OI #2 Length subfield from the value of the Length field.

| Bits: | B0 - B3 | B4 - B7 |
| --- | --- | --- |
|  | OI #1 Length | OI #2 Length |
| Figure 7-95o128—OI Lengths field format |  |  |

The OI field is defined in 7.3.1.31. Each OI identifies a roaming consortium (group of SSPs with inter-SSP roaming agreement) or a single SSP. The value of the OI(s) in this table are equal to the value of the first 3 OIs in the dot11RoamingConsortiumTable. If fewer than 3 values are defined in the dot11RoamingConsortiumTable, then only as many OIs as defined in the table are populated in this element. The values of the OIs in this element are equal to the values of the first OIs, up to 3, from the table.

Change the following sub clauses as shown:

7.3.4 Information Query Protocol elements

Information Query Protocol (IQP) elements are defined to have a common format consisting of a 2-octet Info ID field, a 2-octet length field, and a variable-length element-specific information field. Each element is assigned a unique Info ID as defined in this standard. The IQP element format is shown in Figure 7-95o130. See Annex W.1 for informative text on IQP usage.

|  | Info ID | Length | Information |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |
| Figure 7-95o130—Information Query Protocol element format |  |  |  |

Each IQP element in 7.3.4 is assigned a unique 2-octet Info ID. The set of valid Info IDs are defined in Table 7-43bn. The 2-octet Info ID is encoded following the conventions given in 7.1.1.

The Length field specifies the number of octets in the Information field and is encoded following the conventions given in 7.1.1.

The  IQP elements that may be configured are shown in Table 7-43bn.

| Table 7-43bn—Information Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | IQP Info Element (clause) |
| Reserved | 0-255 | n/a |
| IQP Capability List | 256 | 7.3.4.1 |
| Venue Name information | 257 | 7.3.4.2 |
| Emergency Call Number information | 258 | 7.3.4.3 |
| Network Authentication Type information | 259 | 7.3.4.4 |
| Roaming Consortium List | 260 | 7.3.4.5 |
| IP Address Type Availability information | 261 | 7.3.4.7 |
| NAI Realm List | 262 | 7.3.4.8 |
| 3GPP Cellular Network information | 263 | 7.3.4.9 |
| AP Geo Location | 264 | 7.3.4.10 |
| AP Civic Location | 265 | 7.3.4.11 |
| AP Location Public Identifier URI | 266 | 7.3.4.12 |
| Domain Name List | 267 | 7.3.4.13 |
| Emergency Alert Identifier URI | 268 | 7.3.4.14 |
| TDLS Discovery | 269 | 7.3.4.15 |
| Reserved | 270– 56796 | n/a |
| Information Query Protocol vendor-specific list | 56797 | 7.3.4.6 |
| Reserved | 56798 – 65535 | n/a |

7.3.4.1 IQP Capability List

The IQP Capability List provides a list of information/capabilities that has been configured on a STA or a remote information repository. The IQP Capability List element is returned in response to a GAS Initial Query Request with protocol id value of 0 and Info ID set as specified in Table 7-43bg..

The format of the IQP Capability List is provided in Figure 7-95o131

.

|  | Info ID | Length | Info ID #1 | Info ID #2 (optional) |  | Info ID #n (optional) | IQP Vendor-Specific List #1 (optional) |  | IQP Vendor-Specific List #n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 | 0 or 2 |  | 0 or 2 | variable |  | variable |
| Figure 7-95o131—IQP Capability List format |  |  |  |  |  |  |  |  |  |

The first Info ID is a 2-octet field whose value is drawn from Table 7-43bn corresponding to the IQP Capability List.

The Length is a 2-octet field whose value is equal to 2 times the number of Info IDs following the length, excluding those in IQP Vendor Specific Lists.

Each subsequent Info ID included in the IQP Capability List declares that a subsequent query for that Info ID will return the requested IQP element. Each Info ID returned is one of the Info IDs in Table 7-43bn. The Info ID for IQP Capability List is always included in the IQP Capability List returned in a GAS Query Response. The list does not include any duplicate Info IDs, except possibly the Info ID for the IQP  Vendor-specific list. The Info IDs returned in the IQP Capability List are ordered by increasing Info ID value except for IQP Vendor-specific lists which are always ordered to be at the end.

The IQP Vendor-specific list is defined in 7.3.4.6. The IQP Capability List and IQP Vendor-specific list are structured such that the first 2 octets of IQP Vendor-specific list are interpreted in the IQP Capability List as the Info ID corresponding to the IQP Vendor-specific list. When a IQP Vendor-specific list is present in the IQP Capability List, the IQP Capability List element contains the capabilities of that vendor-specific query protocol.

7.3.4.2 Venue Name information

The Venue Name information provides one or more venue names associated with the BSS. The format of the Venue Name information is shown in Figure 7-95o132. The Venue Name information may be used to provide additional metadata on the BSS. For example, this information may be used to assist a user in selecting the appropriate BSS with which to associate. One or more Venue Name fields may be included in the same or different languages.

|  | Info ID | Length | Venue Info | Venue Name Duple #1 | Venue Name Duple #2 (optional) |  | Venue Name Duple #N (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 | variable | variable |  | variable |
| Figure 7-95o132—Venue Name information format |  |  |  |  |  |  |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the Venue Name information as defined in Figure 7-95o132.

The Length is a 2-octet field whose value is equal to two plus the number of octets in Venue Name Duple fields.

The Venue Info fields is defined in 7.3.1.34.

The format of the Venue Name Duple field is shown in Figure 7-95o133.

|  | Length | Language Code | Venue Name |
| --- | --- | --- | --- |
| Octets: | 1 | 3 | Variable |
| Figure 7-95o133—Venue Name Duple field |  |  |  |

—	The Length is a one octet field whose value is equal to 3 plus the number of octets in the Venue Name field.

—	The Language Code field is an ISO-14962-1997 [B54] encoded string that defines the language used in the Venue Name field. The Language Code field is a two or three character language code selected from ISO-639 [B53]. A two character language code has a zero (“null” in ISO-14962-1997) appended to make it 3 octets in length.

—	The Venue Name field is a UTF-8 formatted field containing the venue’s name. The maximum length of this field is 252 octets. UTF-8 format is defined in RFC 3629.

7.3.4.3 Emergency Call Number information

The Emergency Call Number information provides a list of emergency phone numbers to an emergency responder (such as directed by a PSAP) that is used in a specific geographical area. The format of the Emergency Call Number information is provided in Figure 7-95o134.

|  | Info ID | Length | Emergency Call Number Unit #1 | Emergency Call Number Unit #2 (optional) |  | Emergency Call Number Unit #N (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable | variable |  | variable |
| Figure 7-95o134—Emergency Call Number information format |  |  |  |  |  |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the Emergency Call Number information.

The Length is a 2-octet field whose value is determined by the number and size of the Emergency Call Number Units.

Each Emergency Call Number Unit has the structure shown in Figure 7-95o135.

|  | Length of Emergency Call Number | Emergency Call Number |
| --- | --- | --- |
| Octets: | 1 | variable |
| Figure 7-95o135—Emergency Call Number Unit format |  |  |

The Length of Emergency Call Number is a one octet field whose value is determined by the size of the Emergency Call Number field.

The Emergency Call Number field indicates the dialing digits used to obtain emergency services from the network. This field is encoded using the UTF-8 character set, defined in RFC 3629.

7.3.4.4 Network Authentication Type information

The Network Authentication Type information provides a list of authentication types when ASRA is set to 1. The format of the Network Authentication Type information is shown in Figure 7-95o136.

|  | Info ID | Length | Network Authentication Type Unit #1 | Network Authentication Type Unit #2 (optional) |  | Network Authentication Type Unit #N (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable | variable |  | variable |
| Figure 7-95o136—Network Authentication Type information format |  |  |  |  |  |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the Network Authentication Type information.

The Length is a 2-octet field whose value is determined by the number and size of the Network Authentication Type Units.

Each Network Authentication Type Unit has the structure shown in Figure 7-95o137.

|  | Network Authentication Type Indicator | Re-direct URL Length | Re-direct URL (optional) |
| --- | --- | --- | --- |
| Octets: | 1 | 2 | variable |
| Figure 7-95o137—Network Authentication Type Unit |  |  |  |

The Network Authentication Type Indicator has one of the values shown in Table 7-43bo.

Each Network Authentication Type Indicator defines additional information that may be communicated.

| Table 7-43bo—Network Authentication Type Indicator |  |
| --- | --- |
| Value | Meaning |
| 0 | Acceptance of terms and conditions |
| 1 | On-line enrollment supported |
| 2 | http/https redirection |
| 3 | DNS redirection |
| 4-255 | Reserved |

If the Network Authentication Type Indicator is zero, the network requires the user to accept terms and conditions, the Re-direct URL Length will be set to 0 and the Re-direct URL will not be present.

If the Network Authentication Type Indicator is 1, the network supports on-line enrollment. Higher-layer protocols on the non-AP STA may indicate to the user that accounts may be created. When the Network Authentication Type Indicator is 1, the Re-direct URL Length will be set to 0 and the Re-direct URL will not be present.

If the Network Authentication Type Indicator is 2 the network infrastructure performs http/https redirect.

If the Network Authentication Type Indicator is 3, the network supports DNS redirection. Higher layer software on the non-AP STA will exchange credentials with the network, the Re-direct URL Length will be set to 0 and the Re-direct URL will not be present.

The Re-direct URL Length field is a 2-octet field whose value is the length in octets of the Re-direct URL. The value of the Re-direct URL Length field is set to 0 whenever the Re-direct URL is not present.

If the Network Authentication Type Indicator is 2, a Re-direct URL may optionally be included. If the Network Authentication Type Indicator is other than 2, a Re-direct URL is not included. The URL is formatted in accordance with RFC 3986.

7.3.4.5 Roaming Consortium List

The Roaming Consortium List provides a list of information about the Roaming Consortium and/or SSPs whose networks are accessible via this AP. This list may be returned in response to a GAS Initial Query Request with protocol id value of 0 and Info ID set as specified in Table 7-43bg. see 11.23.3.1.1.1. The format of the Roaming Consortium List is provided in Figure 7-95o138.

|  | Info ID | Length | OI Duple #1 (optional) | OI Duple #2 (optional) |  | OI Duple #N (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable | variable |  | variable |
| Figure 7-95o138—Roaming Consortium List format |  |  |  |  |  |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the Roaming Consortium List.

The Length is a 2-octet field whose value is dependent on the number and size of OIs present in the element.

The format of the OI Duple field is provided in Figure 7-95o139a. There are zero or more OI Duples in this list. OIs contained within the Roaming Consortium element (see7.3.2.94) are also included in this list. The value of the OI subfield(s) in this list are equal to the values of the OI(s) in the dot11RoamingConsortiumTable.

—	The value of the OI Length field is equal to the number of octets in the OI field.

—	The OI field is defined in 7.3.1.31. Each OI identifies a roaming consortium (group of SSPs with inter-SSP roaming agreement) or a single SSP.

|  | OI Length | OI |
| --- | --- | --- |
| Octets: | 1 | variable |
| Figure 7-95o139—OI Duple format |  |  |

.

7.3.4.6 Information Query Protocol vendor specific list

The Information Query Protocol vendor-specific list is used to query for information not defined in this standard within a single defined format, so that reserved Info IDs are not usurped for nonstandard purposes and inter operability is more easily achieved in the presence of nonstandard information. The element is in the format shown in Figure 7-95o140.

|  | Info ID | Length | OI | Vendor Specific Content |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable | variable |
| Figure 7-95o140—Information Query Protocol vendor specific query format |  |  |  |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the Information Query Protocol vendor specific list.

The Length is a 2-octet field whose value is equal to the number of octets in the OI field plus the number of octets in the Vendor-Specific Content field.

The OI field is defined in 7.3.1.31.

The Vendor-Specific Content field is content that has been defined by the entity identified in the OI field.

7.3.4.7 IP Address Type Availability Information

The IP Address Type Availability information provides non-AP STA with the information about the availability of IP address version and type which could be allocated to the non-AP STA after successful association. This information may be returned in response to a GAS Initial Query Request with protocol id value of 0 and Info ID set as specified in Table 7-43bg.. The format of the IP Address Type Availability information is shown in Figure 7-95o141.

|  | Info ID | Length | IP Address |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 |
| Figure 7-95o141—IP Address Type Availability information |  |  |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the IP Address Type Availability information.

The Length is a 2-octet field whose value is 1.

The format of the IP Address field shown in Figure 7-95o142.

| Bits: | B0 - B1 | B2 - B7 |
| --- | --- | --- |
|  | IPv6 Address | IPv4 Address |
| Figure 7-95o142—IP Address field format |  |  |

The IPv6 address field format is shown in Table 7-43bp.

| Table 7-43bp—IPv6 address field values |  |
| --- | --- |
| Address Value | Meaning |
| 0 | Address type not available |
| 1 | Address type available |
| 2 | Availability of the address type not known |
| 3 | Reserved |

The IPv4 address field format is shown in Table 7-43bq.

.

| Table 7-43bq— IPv4 address field values |  |
| --- | --- |
| Address Value | Meaning |
| 0 | Address type not available |
| 1 | Public IPv4 address available |
| 2 | Port-restricted IPv4 address available |
| 3 | Single NATed private IPv4 address available |
| 4 | Double NATed private IPv4 address available |
| 5 | Port-restricted IPv4 address and single NATed IPv4 address available |
| 6 | Port-restricted IPv4 address and double NATed IPv4 address available |
| 7 | Availability of the address type is not known |
| 8 - 63 | Reserved |

7.3.4.8 NAI Realm List

The NAI Realm List provides a list of NAI realms corresponding to SSPs or other entities whose networks or services are accessible via this AP; optionally included for each NAI realm is a list of one or more EAP Method sub-fields, which that NAI realm uses for authentication. The NAI Realm List may be returned in response to a GAS Initial Query Request with protocol id value of 0 and Info ID set as specified in Table 7-43bg.. The format of the NAI Realm List is provided in Figure 7-95o143.

|  | Info ID | Length | Secondary NAI Realm Pages | NAI Realm Count | NAI Realm Data #1 (optional) | NAI Realm Data #2 (optional) | . . . | NAI Realm Data #n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | 1 | variable | variable |  | variable |
| Figure 7-95o143—NAI Realm List format |  |  |  |  |  |  |  |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the NAI Realm List.

The Length field is a 2-octet field whose value is determined by the number and size of the NAI Realm Data fields.

The Secondary NAI Realm Pages is a 1-octet field which specifies the total number of non-null Secondary NAI Realm Lists which can be returned in response to a IQP query for Secondary NAI Realm Lists. If the value of NAI Realm Pages is 1, then only 1 non-null Secondary NAI Realm List can be returned and its Info ID value is the first value in Table 7-43bn for Secondary NAI Realm List. If the value of NAI Realm Pages is 2, then only 2 non-null Secondary NAI Realm Lists can be returned and their Info ID values are the first two values in Table 7-43bn for Secondary NAI Realm List, and so on.

The NAI Realm Count is a one-octet field which specifies the number of NAI realms included in the NAI Realm list.

The format of the NAI Realm Data field is shown in Figure 7-95o144.

|  | NAI Realm Data Field Length | NAI Realm Encoding | NAI Realm Length | NAI Realm | EAP Method Count | EAP Method #1 (optional) | EAP Method #2 (optional) | . . . | EAP Method #n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 1 | 1 | variable | 1 | variable | variable |  | variable |
| Figure 7-95o144—NAI Realm Data field format |  |  |  |  |  |  |  |  |  |

NAI Realm Data Field Length is a 2-octet sub-field whose value is equal to 3 plus the length of the NAI Realm sub-field plus the sum of the lengths of the EAP Method List subfields.

The NAI Realm Encoding is a 1-octet sub-field whose format is shown in Figure 7-95o145.

The NAI Realm Encoding Type sub-field is a 1-bit sub-field. It is set to 0 to indicate that the NAI Realm in the NAI Realm sub-field is formatted in accordance with RFC-4282. It is set to 1 to indicate it is a UTF-8 formatted character string which is not formatted in accordance with RFC-4282.
Note—this encoding is to facilitate roaming consortium or other entities that use non-standard NAI realm formats.

| Bits: | B0 | B1 - B7 |
| --- | --- | --- |
|  | NAI Realm Encoding Type | Reserved |
| Figure 7-95o145—NAI Realm Encoding sub-field format |  |  |

NAI Realm Length sub-field is a 1-octet sub-field whose value is the length of the NAI Realm sub-field.

The NAI Realm sub-field is one or more NAI Realms formatted as defined in the NAI Realm Encoding Type bit of the NAI Realm Encoding subfield. If there is more than one NAI Realm in this sub-field, the NAI Realms are delimited by a semi-colon character (i.e., “;”, which is encoded in UTF-8 format as 0x3B). All the realms included in the NAI Realm sub-field support all the EAP methods identified by the EAP Method sub-fields, if present. The maximum length of this sub-field is 255 octets.The EAP Method Count specifies the number of EAP methods sub-fields for the NAI realm. If the count is zero, there is no EAP method information provided for the NAI realm.

The format of the optional EAP Method sub-field is shown in Figure 7-95o146. Each EAP Method sub-field contains a set of Authentication Parameters associated with the EAP-Method.

|  | Length | EAP Method | Authentication Parameter Count | Authentication Parameter #1 (optional) | Authentication Parameter #2 (optional) | . . . | Authentication Parameter #n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable |  | variable |
| Figure 7-95o146—EAP Method sub-field format |  |  |  |  |  |  |  |

The length of the EAP Method sub-field is a 1-octet sub-field whose value is equal to 2 plus the length of the Authentication Parameter sub-fields.

The EAP method sub-field is a 1-octet sub-field which is set to the EAP Type value as given in IANA EAP Method Type Numbers.

The Authentication Parameter Count indicates how many additional Authentication Parameter sub-fields are specified for the supported EAP Method. If the Authentication Parameters Count sub-field is zero, there are no Authentication Parameters sub-fields present, meaning no additional Authentication Parameters are specified for the EAP Method. If the value of the EAP Method field is 254 indicating an Expanded EAP Type, then the format of the Authentication Parameter sub-fields is shown in Table 7-43bs.

The format of the Authentication Parameter sub-field is shown in Figure 7-95o147.

|  | ID | Length | Authentication Parameter Value |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure 7-95o147—Authentication Parameter sub-field format |  |  |  |

The ID is a 1-octet field which indicates the type of authentication information provided.

The length of the Authentication Parameter sub-field is a 1-octet sub-field whose value is equal to the length in octets of the Authentication Parameter Value field.

The Authentication Parameter Value is a variable length field containing the value of the parameter indicated by the ID.

The ID and its associated formats are specified in Table 7-43br. Each ID indicates a different type of information. Use of multiple Authentication Parameter sub-fields allows all the required authentication parameter requirements to be provided.

| Table 7-43br—Authentication Parameter types |  |  |  |
| --- | --- | --- | --- |
| Authentication Information | ID | Description | Length (octets) |
| Reserved | 0 |  |  |
| Expanded EAP Method | 1 | Expanded EAP Method Subfield | 7 |
| Non-EAP Inner Authentication Type | 2 | Enum (0 - Reserved, 1 - PAP, 2 – CHAP, 3 - MSCHAP, 4 - MSCHAPV2) | 1 |
| Inner Authentication EAP Method Type | 3 | Value drawn from IANA EAP Method Type Numbers | 1 |
| Expanded Inner EAP Method | 4 | Expanded EAP Method Subfield | 7 |
| Credential Type | 5 | Enum (1-SIM, 2-USIM, 3-NFC Secure Element, 4-Hardware Token, 5-Softoken, 6 - Certificate, 7 – username/password, 8-Vendor Specific) | 1 |
| Tunneled EAP Method Credential Type | 6 | Enum (1-SIM, 2-USIM, 3-NFC Secure Element, 4-Hardware Token, 5-Softoken, 6-Certificate, 7-username/password, 8-Anonymous, 9-Vendor Specific) | 1 |
| Reserved | 7 - 220 |  |  |
| Vendor Specific | 221 | variable | variable |
| Reserved | 222 - 255 |  |  |

If the EAP Method type is an Expanded EAP type (the EAP Method value is 254), the Authentication Parameter is used to specify additional information on the EAP method. Table 7-43bs describes the Authentication Parameter format for the Expanded EAP method; values for the Vendor ID and Vendor Type are specified in RFC 3748. The Vendor ID and Vendor Type fields are expressed in big endian byte order.

| Table 7-43bs—Authentication Parameter format for the Expanded EAP Method |  |
| --- | --- |
| Parameters | Length (octets) |
| ID | 1 |
| Length | 1 |
| Vendor ID | 3 |
| Vendor Type | 4 |

The Non-EAP Inner Authentication Type is specified as single enumerated value given in Table 7-43br. This Authentication Information type is used for non-EAP Inner Authentication methods. The possible values are PAP (as specified in RFC 1334), CHAP (as specified in RFC 1994), MSCHAP (as specified in RFC 2433), and MSCHAPv2 (as specified in RFC 2759).

The Inner Authentication EAP Method Type is specified as the EAP number as defined in IANA EAP Method Type Numbers. This Authentication Information type is used when the Inner Authentication method is an EAP method. If the Inner Authentication EAP Method Type is equal to 254 indicating an Expanded EAP Type, then the Expanded EAP Method Authentication Parameter is included.

A Credential Type can be selected by a single enumerated value as shown in Table 7-43br. If the value is equal to the “Vendor Specific” value, then a Vendor-Specific Authentication Parameter is included.

Vendor-Specific Authentication Parameters are specified as shown in Table 7-43bt.

| Table 7-43bt—Vendor-Specific Authentication Parameters |  |
| --- | --- |
| Parameters | Length (octets) |
| ID | 1 |
| Length | 1 |
| OI | variable |
| Authentication Parameter Value | Vendor-specific Content |

The Info ID field is equal to the one of the values in Table 7-43bn corresponding to a Secondary NAI Realm List.

The remainder of the fields in Figure 7-95o148 are defined to be identical to the fields having the same name in 7.3.4.8.

7.3.4.9 3GPP Cellular Network information

The 3GPP Cellular Network information contains cellular information such as network advertisement information e.g., network codes and country codes to assist a 3GPP non-AP STA in selecting an AP to access 3GPP networks. The format of the 3GPP Cellular Network information is shown in Figure 7-95o149.

|  | Info ID | Length | Payload |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |
| Figure 7-95o149—3GPP Cellular Network information format |  |  |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the 3GPP Cellular Network information.

The Length field is a 2-octet field and is equal to the length of the Payload field.

The Payload field is a generic container whose content is defined in Annex A of 3GPP TS 24.234 v8.1.0.

7.3.4.10 AP Geo Location

The AP Geo Location element provides the AP’s location in LCI format, see 7.3.2.22.9. This list element may be returned in response to a GAS Initial Query Request with protocol id value of 0 and Info ID set as specified in Table 7-43bg.. The format of the AP Geo Location element is provided in Figure 7-95o150.

|  | Info ID | Length | Location Configuration Report |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 18 |
| Figure 7-95o150—AP Geo Location format |  |  |  |

The Length field is a 2-octet field and is equal to 18.

The format of the Location Configuration Report is provided in 7.3.2.22.9. There is no Optional Subelements field present in the Location Configuration Report when it is used in the AP Geo Location element. This information is taken from the dot11APLCITable MIB object.

7.3.4.11 AP Civic Location

The AP Civic Location element provides the AP’s location in Civic format. This list element may be returned in response to a GAS Initial Query Request with protocol id value of 0 and Info ID set as specified in Table 7-43bg.. The format of the AP Civic Location element is provided in Figure 7-95o151.

|  | Info ID | Length | Location Civic Report |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |
| Figure 7-95o151—AP Civic Location format |  |  |  |

The Length field is a 2-octet field and is equal to the length of the Location Civic Report.

The format of the Location Civic Report is provided in 7.3.2.21.13. This information is taken from the dot11ApCivicLocation MIB object.

7.3.4.12 AP Location Public Identifier URI

The AP Location Public Identifier URI element provides an indirect reference to where the location information for the AP can be retrieved. This list element may be returned in response to a GAS Initial Query Request with protocol id value of 0 and Info ID set as specified in Table 7-43bg.. The format of the AP Location Public Identifier URI element is provided in Figure 7-95o152.

|  | Info ID | Length | Public Identifier URI |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |
| Figure 7-95o152—AP Civic Location format |  |  |  |

The Info ID field is equal to the value in Table 7-43by corresponding to the AP Location Public Identifier URI.

The Length field is a 2-octet field whose value is the length of the AP Location Public Identifier URI field.

The Public Identifier URI field is defined in 7.3.2.22.13.

7.3.4.13 Domain Name List

The Domain Name List element provides a list of domain names corresponding to SSPs whose networks are accessible via the AP. Domain Names in this element are taken from dot11DomainNameTable. This list element may be returned in response to a GAS Initial Query Request with protocol id value of 0 and Info ID set as specified in Table 7-43bg.. The format of the Domain Name List element is provided in Figure 7-95o153.

|  | Info ID | Length | Domain Name field #1 | Domain Name field #2 (optional) | . . . | Domain Name field #N (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable | variable |  | variable |
| Figure 7-95o153—Domain Name List format |  |  |  |  |  |  |

The Length is a 2-octet field whose value is equal to the number and size of the Domain Name Fields.

The Domain Name field is shown in Figure 7-95o154.

|  | Length | Domain Name |
| --- | --- | --- |
| Octets: | 1 | variable |
| Figure 7-95o154—Domain Name field format |  |  |

The Length subfield is the length in octets of the Domain Name subfield.

The Domain Name subfield is a domain name compliant with the “Preferred Name Syntax” as defined in RFC 1035. The maximum length of this field is 255 octets.

7.3.4.14 Emergency Alert Identifier URI information

The Emergency Alert Identifier URI information provides a URI for EAS message retrieval. The format of the Emergency Alert Identifier URI information is provided in Figure 7-95o155.

|  | Info ID | Length | Emergency Alert Identifier URI |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |
| Figure 7-95o155—Emergency Alert Identifier URI information format |  |  |  |

The Length is a 2 - octet field whose value is equal to the length of the Emergency Alert Identifier URI field.

The Emergency Alert Identifier URI is a variable length field used to indicate the URI at which an EAS message may be retrieved. See 11.23.7. The Emergency Alert Identifier URI is formatted in accordance with RFC 3986.

7.3.4.15 TDLS Discovery information

The TDLS Discovery information contains information that may be used by STAs to discover TDLS capabilities of a peer STA.The format of the TDLS Discovery information is provided in Figure 7-95o156.

|  | Info ID | Length | OI | TDLS Discovery Information |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 3 | 0 - 252 |
| Figure 7-95o156—TDLS Discovery information format |  |  |  |  |

The Length field is set to 12.

The OI field indicates the type of TDLS discovery information.

The TDLS Discovery Information contains capability information that a STA may use to establish a TDLS link. An example of the peer discovery information is described in 11.23.8.

## 7.4 Action frame format details

7.4.7 Public Action details

7.4.7.1 Public Action frames

Change the first paragraph of 7.4.7.1 as follows.

The Public Action frame is defined to allow inter-BSS and AP to un-associated-STA communications and Generic Advertisement Services. The defined Public Action frames are listed in Table 7-57e.

Change Table 7-57e by inserting new 4 rows and change Reserved row Action field value.

| Table 7-57e—Public Action field values |  |
| --- | --- |
| Action field value | Description |
| 10 | GAS Initial Request, see 7.4.7.14 |
| 11 | GAS Initial Response, see 7.4.7.15 |
| 12 | GAS Comeback Request, see 7.4.7.16 |
| 13 | GAS Comeback Response, see 7.4.7.17 |
| 914-255 | Reserved |

7.4.7.14 GAS Initial Request frame format

The GAS Initial Request frame is a Public Action frame. It is transmitted by a requesting STA to request information from another STA. The format of the GAS Initial Request frame body is shown in Table 7-57am.

| Table 7-57am—GAS Initial Request frame body format |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Action |
| 2 | Dialog Token |
| 3 | Advertisement Protocol element |
| 4 | Query Request length |
| 5 | Query Request |

The Category field is set to the value indicating a Public Action frame, as specified in Table 7-24.

The Action field is set to the value specified in Table 7-57e for a GAS Initial Request frame.

The Dialog Token field is defined in 7.3.1.12 and set by the requesting STA.

The Advertisement Protocol element is defined in 7.3.2.91. The Advertisement Protocol element includes exactly one Advertisement Protocol ID.

The Query Request length field is defined in Figure 7-101bc. The value of the Query Request length field is set to the total number of octets in the Query Request field.

|  | B0 | B15 |
| --- | --- | --- |
|  | Query Request length |  |
| Octets: | 2 |  |
| Figure 7-101bc—Query Request length field |  |  |

The Query Request field is defined in Figure 7-101bd. The Query Request field is a generic container whose value is a GAS query which is formatted in accordance with the protocol specified in the Advertisement Protocol element.

|  | Query Request |
| --- | --- |
| Octets: | variable |
| Figure 7-101bd—Query Request field |  |

7.4.7.15 GAS Initial Response frame format

The GAS Initial Response frame is a Public Action frame. It is transmitted by an STA responding to a request from a requesting STA. The format of the GAS Initial Response frame body is shown in Table 7-57an.

| Table 7-57an—GAS Initial Response frame body format |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Action |
| 2 | Dialog Token |
| 3 | Status Code |
| 4 | GAS Comeback Delay |
| 5 | Advertisement Protocol element |
| 6 | Query Response Length |
| 7 | Query Response (optional) |

The Category field is set to the value indicating a Public Action frame, as specified in Table 7-24.

The Action field is set to the value specified in Table 7-57e for a GAS Initial Response frame.

The Dialog Token field is copied from the corresponding GAS Initial Request frame.

The Status Code values are defined in Table 7-23.

The GAS Comeback Delay field specifies the delay time value in TUs. Upon expiry of this delay, the requesting STA should attempt to retrieve the Query Response using a Comeback Request frame. The GAS Comeback Delay field format is provided in Figure 7-101be.

A zero value will be returned by the STA when a Query Response is provided in this frame.

|  | B0 | B15 |
| --- | --- | --- |
|  | GAS Comeback Delay |  |
| Octets: | 2 |  |
| Figure 7-101be—GAS Comeback Delay field |  |  |

The Advertisement Protocol element is defined in 7.3.2.91. The Advertisement Protocol element includes exactly one Advertisement Protocol ID.

The Query Response Length field is defined in Figure 7-101bf. The value of the Query Response Length field is set to the total number of octets in the Query Response field. If the Query Response Length field is set to 0, then there is no Query Response included in this Action frame.

|  | B0 | B15 |
| --- | --- | --- |
|  | Query Response Length |  |
| Octets: | 2 |  |
| Figure 7-101bf—Query Response length field |  |  |

The Query Response field is defined in Figure 7-101bg. The Query Response field is a generic container whose value is the response to a GAS query and is formatted in accordance with the protocol specified in the Advertisement Protocol element.

.

|  | Query Response |
| --- | --- |
| Octets: | variable |
| Figure 7-101bg—Query Response field |  |

7.4.7.16 GAS Comeback Request frame format

The GAS Comeback Request frame is a Public action frame. It is transmitted by a requesting STA to a responding STA. The format of the GAS Comeback Request frame body is shown in Table 7-57ao.

| Table 7-57ao—GAS Comeback Request frame body format |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Action |
| 2 | Dialog Token |

The Category field is set to the value indicating a Public Action frame, as specified in Table 7-24.

The Action field is set to the value specified in Table 7-57e for a GAS Comeback Request frame.

The Dialog Token field is copied from the corresponding GAS Initial Request frame.

7.4.7.17 GAS Comeback Response frame format

The GAS Comeback Response frame is a Public Action frame. It is transmitted by a responding STA to a requesting STA. The format of the GAS Comeback Response frame body is shown in Table 7-57ap.

| Table 7-57ap—GAS Comeback Response frame body format |  |
| --- | --- |
| Order | Information |
| 0 | Category |
| 1 | Action |
| 2 | Dialog Token |
| 3 | Status Code |
| 4 | GAS Query Response Fragment ID |
| 5 | GAS Comeback Delay |
| 6 | Advertisement Protocol element |
| 7 | Query Response Length |
| 8 | Query Response (optional) |

The Category field is set to the value indicating a Public Action frame, as specified in Table 7-24.

The Action field is set to the value specified in Table 7-57e for a GAS Comeback Response frame.

The Dialog Token field is copied from the Dialog Token field of the corresponding GAS Initial Response frame.

The Status Code values are defined in Table 7-23. The same status code value will be present in all fragments of a multi-fragment query response.

The GAS Query Response Fragment ID is defined in 7.3.1.33. If the responding STA has not received a response to the query that it posted on behalf of a requesting STA, then the responding STA sets the GAS Query Response Fragment ID to 0. When there is more than one query response fragment, the responding STA sets the GAS Query Response Fragment ID to 1 for the initial fragment and increments it by 1 for each subsequent fragment in a multi-fragment Query Response. The More GAS Fragments field is set to 0 whenever the final fragment of a query response is being transmitted. A GAS Query Response Fragment ID field having a non-zero Fragment ID and the More GAS Fragments field set to 1 indicates to the requesting STA that another GAS Comeback frame exchange should be performed to continue the retrieval of the query response.

The GAS Comeback Delay field format is provided in Figure 7-101be. A non-zero GAS Comeback Delay value is returned by the responding STA in this frame to indicate that the GAS query being carried out on behalf of the  requesting STA is still in progress.

—	A non-zero value indicates to the requesting STA that another GAS Comeback frame exchange should be performed after expiry of the GAS Comeback Delay timer in order to retrieve the query response.

—	This field is 0 for all GAS Comeback Response frames containing a query response or a fragment of a multi-fragment query response.

The Advertisement Protocol element is defined in 7.3.2.91. The Advertisement Protocol element includes exactly one Advertisement Protocol ID.

The Query Response Length field is defined in Figure 7-101bf. The value of the Query Response Length field is the total number of octets in the Query Response field. If the Query Response Length field is set to 0, then there is no Query Response included in this Action frame.

The Query Response field is defined in Figure 7-101bg. The value of the Query Response field is a generic container dependent on the advertisement protocol specified in the Advertisement Protocol element and the query itself. In a multi-fragment query response, the response to the query posted on behalf of a requestingSTA is fragmented such that each fragment to be transmitted fits within the MMPDU size limitation.

10. Layer management

10.3.70.1 MLME-GAS.request

10.3.70.1.1 Function

This primitive requests the information of a specific advertisement service from another STA and requests the STA to provide the Generic Advertisement Service.

10.3.70.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS.request (
	PeerSTAAddress,
	DialogToken,
	AdvertisementProtocolID,
	Query,
	QueryFailureTimeout
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43bl | This contains an Advertisement Protocol ID (see 7.3.2.91) which may be 802.11 assigned or vendor specified. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |
| QueryFailureTimeout | Integer | > 1 | The time limit, in units of Beacon intervals, after which the GAS query procedure will be terminated. |

10.3.70.1.3 When generated

This primitive is generated by the SME at a STA to request a specific advertisement service from another STA.

10.3.70.1.4 Effect of receipt

The STA operates according to the procedures defined in 11.23.3.

10.3.70.2 MLME-GAS.confirm

10.3.70.2.1 Function

This primitive reports the status code and Query Response to a GAS query of a specific advertisement service from a STA, which may be relaying the query response from a server in an external network.

10.3.70.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS.confirm (
	PeerSTAAddress,
	DialogToken,
	ResultCode,
	
	ResponseInfo 
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| ResultCode | Enumeration | SUCCESS, TIMEOUT, UNSPECIFIED_FAILURE, ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED, QUERY_RESPONSE_TOO_LARGE, SERVER_UNREACHABLE, TRANSMISSION_FAILURE | Indicates the result response to the GAS request from the peer MAC entity. |
| ResponseInfo | String | N/A | Query Response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

The mapping of Status Code received in the GAS Response frame is mapped to the corresponding Result Code in Table 11-4.

10.3.70.2.3 When generated

This primitive is generated by the MLME as a response to the MLME-GAS.request primitive indicating the result of that request.

The primitive is generated when the requesting STA receives a query response in a GAS Initial Response frame or one or more GAS Comeback Response frames.

10.3.70.2.4 Effect of receipt

The STA operates according to the procedures defined in 11.23.3.

10.3.70.3 MLME-GAS.indication

10.3.70.3.1 Function

This primitive reports to the STA’s SME about the GAS Request.

10.3.70.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS.indication (

PeerSTAAddress,

DialogToken,

AdvertisementProtocolID,

Query

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the query message was received. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43bl | This contains an Advertisement Protocol ID (see 7.3.2.91) which may be 802.11 assigned or vendor specified. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

10.3.70.3.3 When generated

This primitive is generated by the MLME as a result of receipt of a GAS request from STA.

10.3.70.3.4 Effect of receipt

The SME is notified of the request from the STA.

The SME operates according to the procedures defined in 11.23.3.

The SME generates an MLME-GAS.response primitive within a dot11GASResponseTimeout.

10.3.70.4 MLME-GAS.response

10.3.70.4.1 Function

This primitive responds to the request for an advertisement service by a specified STA MAC entity.

10.3.70.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS.response (
	PeerSTAAddress,
	DialogToken,
	ResultCode,
	ResponseInfo
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity to which query response information is transmitted. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| ResultCode | Enumeration | SUCCESS, NO_REQUEST_OUTSTANDING, ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED, QUERY_RESPONSE_OUTSTANDING, QUERY_RESPONSE_TOO_LARGE, SERVER_UNREACHABLE, TIMEOUT | Indicates the result response to the GAS-request from the peer MAC entity. See Table 11-4. |
| ResponseInfo | String | N/A | Query Response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

10.3.70.4.3 When generated

This primitive is generated by the MLME at a STA as a result of an MLME-GAS.indication primitive.

10.3.70.4.4 Effect of receipt

This primitive causes the MAC entity at the STA to send a GAS Initial Response frame to the requesting STA and optionally one or more GAS Comeback Response frames.

10.3.71 Protected Dual of Network Discovery and Selection Support

This set of primitives supports the process of Generic Advertisement Services using Protected Dual of Public Action frames.

10.3.71.1 MLME-PDGAS.request

10.3.71.1.1 Function

This primitive requests the information of a specific advertisement service from another STA and requests the STA to provide the Generic Advertisement Service.

10.3.71.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-PDGAS.request (
	PeerSTAAddress,
	DialogToken,
	AdvertisementProtocolID,
	Query,
	QueryFailureTimeout
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43bl | This contains an Advertisement Protocol ID (see 7.3.2.91) which may be 802.11 assigned or vendor specified. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |
| QueryFailureTimeout | Integer | > 1 | The time limit, in units of Beacon intervals, after which the GAS query procedure will be terminated. |

10.3.71.1.3 When generated

This primitive is generated by the SME at a STA to request a specific advertisement service from another STA, which may relay the query to a server in an external network. This primitive is used when Management Frame Protection is negotiated.

10.3.71.1.4 Effect of receipt

The STA operates according to the procedures defined in 11.23.3

10.3.71.2 MLME-PDGAS.confirm

10.3.71.2.1 Function

This primitive reports the status code and query response to a GAS query of a specific advertisement service from a STA.

10.3.71.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-PDGAS.confirm (
	PeerSTAAddress,
	DialogToken,
	ResultCode,
	ResponseInfo 
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS transaction. |
| ResultCode | Enumeration | SUCCESS, TIMEOUT, UNSPECIFIED_FAILURE, ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED, QUERY_RESPONSE_TOO_LARGE, SERVER_UNREACHABLE, TRANSMISSION_FAILURE | Indicates the result response to the GAS request from the peer MAC entity. |
| ResponseInfo | String | N/A | Query Response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

The mapping of Status Code received in the GAS Response frame is mapped to the corresponding Result Code in Table 11-4.

10.3.71.2.3 When generated

This primitive is generated by the MLME as a response to the MLME-GAS.request primitive indicating the result of that request. This primitive is used when Management Frame Protection is negotiated.

The primitive is generated when the requesting STA receives a query response in a GAS Initial Response frame or one or more GAS Comeback Response frames.

10.3.71.2.4 Effect of receipt

The STA operates according to the procedures defined in 11.23.3.

10.3.71.3 MLME-PDGAS.indication

10.3.71.3.1 Function

This primitive reports to the STA’s SME about the GAS Request.

10.3.71.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-PDGAS.indication (
	PeerSTAAddress,
	DialogToken, 
	AdvertisementProtocolID,
	Query

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the query message was received. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43bl | This contains an Advertisement Protocol ID (see 7.3.2.91) which may be 802.11 assigned or vendor specified. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

10.3.71.3.3 When generated

This primitive is generated by the MLME as a result of receipt of a GAS request from STA. This primitive is used when Management Frame Protection is negotiated.

10.3.71.3.4 Effect of receipt

The SME is notified of the request from the STA.

The SME operates according to the procedures defined in 11.23.3.

The SME generates an MLME-PDGAS.response primitive within a dot11GASResponseTimeout.

10.3.71.4 MLME-PDGAS.response

10.3.71.4.1 Function

This primitive responds to the request for an advertisement service by a specified STA MAC entity.

10.3.71.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-PDGAS.response (
	PeerSTAAddress,
	DialogToken,
	ResultCode,
	ResponseInfo
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity to which query response information is transmitted. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| ResultCode | Enumeration | SUCCESS, NO_REQUEST_OUTSTANDING, ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED, QUERY_RESPONSE_OUTSTANDING, QUERY_RESPONSE_TOO_LARGE, SERVER_UNREACHABLE, TIMEOUT | Indicates the result response to the GAS-request from the peer MAC entity. See 11.4 |
| ResponseInfo | String | N/A | Query Response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

10.3.71.4.3 When generated

This primitive is generated by the MLME at a STA as a result of an MLME-GAS.indication primitive. This primitive is used when Management Frame Protection is negotiated.

10.3.71.4.4 Effect of receipt

This primitive causes the MAC entity at the STA to send a GAS Initial Response frame to the requesting STA and optionally one or more GAS Comeback Response frames.

Change the following subclauses as shown:

## 11.23 WLAN Interworking with External Networks Procedures

11.23.1 General

This subclause describes the actions and the procedures that provide interworking capabilities between 802.11 infrastructure and external networks.

11.23.2 Interworking capabilities and information

STAs indicate their support for Interworking Service by setting the dot11InterworkingServiceEnabled MIB variable to true. When dot11InterworkingServiceEnabled is true, STAs include the Interworking element in Beacon and Probe Response frames and non-AP STAs include the Interworking element in Probe Request frames.

When dot11InterworkingServiceEnabled and dot11ExtendedChannelSwitchEnabled are both set to TRUE, the AP may provide its operating channel and regulatory class to an Interworked SSPN using the values from dot11RegulatoryClassesTable MIB entry.

The Interworking element contains signaling for Homogeneous ESSs. The HESSID is a 6-octet MAC address which identifies the homogeneous ESS. The HESSID value shall be identical to one of the BSSIDs in the homogeneous ESS. Thus, it is a globally unique identifier that in conjunction with the SSID, may be used to provide network identification for an SSPN.

NOTE—It is required by this standard that the HESSID field in the Interworking element is administered consistently across all BSSs in a homogeneous ESS.

The Interworking element also provides an Access Network Options  in Beacon and Probe Response frames to assist the non-AP STA with network discovery and selection.

11.23.3 Interworking Procedures: Generic Advertisement Services

This subclause describes the actions and procedures that are used to invoke Generic Advertisement Services (GAS). GAS may be used to enable network selection for STAs where dot11InterworkingServiceEnabled is true. GAS provides transport mechanisms for advertisement services while STAs are in the un-associated state as well as the associated state. This is accomplished via the use of Public Action management frames which are class 1 frames. GAS messages shall be transmitted using individually-addressed Public Action frames.  When Management Frame Protection is negotiated, stations shall use individually addressed Protected Dual of Public Action frames instead of individually addressed Public Action frames.

A GAS message exchange may take place between two STAs; one STA transmits a GAS Query and the other STA transmits the GAS Query Response as described in 11.23.3.2.

The advertisement protocol transported by GAS is one of the query protocols in Table 7-43be.

GAS shall be supported by a STA when dot11InterworkingServiceEnabled is true.  IQP shall be supported by a STA when dot11InterworkingServiceEnabled is true.  Advertisement protocols other than IQP shall be supported when the corresponding dot11GASAdvertisementID is not null.

STAs shall not transmit a GAS Query for any Advertisement Protocol unless that Advertisement Protocol ID is included in the Advertisement Protocol element in a Beacon or Probe response frame. The Advertisement Protocol element specifies the Advertisement Protocols that a STA may use to communicate with advertisement servers, which may be located in a STA or an external network. The Advertisement Protocol identifies the query language used by the advertisement server.

The GAS protocol, which is used to transport Queries and Query Responses, is transparent to the Advertisement Protocol.

11.23.3.1  Information Query Protocol

11.23.3.1.1 Information Query Protocol procedures

Information Query Protocol (IQP) shall be supported by a STA whenever dot11InterworkingServiceEnabled is true.  A STA may use IQP to retrieve information and discover supported services by a peer STA.

A STA accomplishes this by transmitting one or more IQP Info IDs or IQP vendor-specific query elements in the Query Request field in a GAS Initial Request frame. In the Query, Info IDs and vendor-specifc query elements, if present, shall be ordered by increasing  Info ID. The receiving STA responds to the query using a GAS Initial Response frame and optionally one or more GAS Comeback Response frames.   The IQP Query Response is comprised of IQP elements drawn from Table 7-43bn and shall be ordered by increasing Info ID.

Note: I think we should decide on a different path for IQP query in which not all information is available.  Since we now have a single GAS protcol, my thinking is that IQP should handle this (just like other protocols (e.g., MIH) would have to handle it too).  A couple options come to my mind:

- GAS responder just provides Info IDs it has available and doesn’t give any indication on what Info IDs were not available

- If none of the requested Info IDs are available, then GAS Response would be null, but status would be success

- We could define a Status Info ID which provided IQP query status and provided Info IDs (and possibly vendor-specific query elements) corresponding to unavailable information.

IQP frame usage for Infrastructure BSSs and IBSSs shall be in accordance with Table 11-3. Frame usage defines the entities permitted to transmit and receive particular IQP elements. STAs where dot11InterworkingServiceEnabled is true which are capable of operating in an IBSS shall be capable of requesting an IQP Capability List and returning the IQP Capability List in a GAS message exchange; STA support for all other IQP elements is optional. STAs where dot11InterworkingServiceEnabled is true shall be capable of returning the IQP Capability List in an IQP GAS message exchange;  STA’s support for all other IQP elements is optional.

| Table 11-3—Information Query Protocol usage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | BSS |  | IBSS |
| Info Name | IQP Info Element (clause) | AP | Non-AP STA | STA |
| IQP Capability List | 7.3.4.1 | T, R | T, R | T, R |
| Venue Name information | 7.3.4.2 | T | R | ---- |
| Emergency Call Number information | 7.3.4.3 | T | R | ---- |
| Network Authentication Type information | 7.3.4.4 | T | R | ---- |
| Roaming Consortium List | 7.3.4.5 | T | R | ---- |
| Information Query Protocol vendor-specific list | 7.3.4.6 | T, R | T, R | T, R |
| IP Address Type Availability information | 7.3.4.7 | T, R | T, R | T, R |
| NAI Realm List | 7.3.4.8 | T | R | T, R |
| 3GPP Cellular Network information | 7.3.4.9 | T | R | ---- |
| AP Geo Location | 7.3.4.10 | T | R | T, R |
| AP Civic Location | 7.3.4.11 | T | R | T, R |
| Domain Name List | 7.3.4.12 | T | R | ---- |
| Emergency Alert Identifier URI | 7.3.4.13 | T | R | T, R |
| TDLS Discovery | 7.3.4.14 | T | R | T, R |
| Symbols T IQP element may be transmitted by MAC entity R IQP element may be received by MAC entity --- IQP element is neither transmitted nor received by MAC entity |  |  |  |  |

A STA that encounters an unknown or reserved IQP Info ID value in a GAS frame (see Table 7-57aj) received without error shall ignore that IQP Info ID and shall parse any remaining IQP Info IDs.

A STA that encounters an unknown vendor-specific OI field or subfield in a GAS frame (see Table 7-57aj) received without error shall ignore that field or subfield respectively, and shall parse any remaining fields or subfields for additional information with recognizable field or subfield values.

11.23.3.1.1.1 Roaming Consortium List procedures

The Roaming Consortium List, which contains a set of OIs, can be retrieved from an AP by a non-AP STA using the GAS procedures defined in 11.23.3.1.1. The list of OIs included in the Roaming Consortium List shall be those OIs in the dot11RoamingConsortiumTable. An AP shall only include an OI in the dot11RoamingConsortiumTable, if in conjunction with an AS, it is capable of successfully authenticating a non-AP STA having valid security credentials for the SSPN identified by that OI. Methods used by the AP to authenticate the non-AP STA include, but are not limited to, RSNA algorithms and Open System authentication.

Each OI identifies an SSP or group of SSPs (i.e., a roaming consortium). An SSP or group of SSPs can register for and obtain an OI using the procedures defined in [B11a].

A non-AP STA can have a locally stored binding between an OI and a set of security credentials with which it can authenticate to the network identified by the OI, that is, the SSP’s SSPN. The method by which this binding is obtained is outside the scope of this standard. A non-AP STA can select from that list of credentials when authenticating to the BSS.

11.23.3.1.1.2 AP Geo Location procedures

A STA where dot11InterworkingServiceEnabled is true may retrieve an AP's Geo location using GAS procedures in 11.23.3.1.1. A STA in the associated state should retrieve Geo location information from the AP using the procedures in 11.10.8.

11.23.3.1.1.3 AP Civic Location procedures

A STA where dot11InterworkingServiceEnabled is true may retrieve an AP's Civic location using GAS procedures in 11.23.3.1.1. A STA in the associated state should retrieve Civic location information from the AP using the procedures in 11.10.8.

11.23.3.1.1.4 AP Procedures for advertising EAP Method associated with an NAI Realm

When dot11RSNAEnabled is true, NAI realms along with their supported authentication methods may be advertised using the NAI Realm List (see 7.3.4.8). Each realm may be optionally associated with a set of EAP methods. Each EAP method may be optionally associated with a set of Authentication Parameters. The NAI realm information provides a hint on the methods a STA can establish an association in an RSN IEEE 802.1X environment. If the non-AP STA recognizes the NAI realm, it may attempt authentication even if it believes the EAP methods are incorrect.

A non-AP STA having dot11InterworkingServiceEnabled set to TRUE may process the NAI realm list. The selection of the NAI realm the non-AP STA uses for authentication is out of scope of this standard. A non-AP STA requests the NAI Realm list using GAS procedures defined in 11.23.3.1.1.

A non-AP STA having dot11InterworkingServiceEnabled set to true may optionally process the EAP Method list as follows:

—	The EAP Method list provided by the AP shall be in priority order (the most preferred EAP Method is listed first).

—	The credential types help the STA to determine what credentials to use for authentication.

—	The STA should confirm the GAS advertisement after an RSNA is established by performing a GAS query for the NAI Realm List using Protected Dual of Public Action frames.

NOTE—The advertisements should be confirmed after the RSNA is established to avoid downgrade attacks.

The policy which determines whether or not a non-AP STA should attempt authentication and/or association with any particular IEEE 802.11 Access Network is outside the scope of this standard.

11.23.3.1.3

11.23.3.2 GAS Protocol

A non-AP STA obtains GAS advertisement capability information from Beacon or Probe Response frames. The Interworking element indicates support for GAS protocol and IQP protocol. The Advertisement Protocol element indicates the other Advertisement Protocol IDs supported in the BSS or IBSS. A STA transmits a GAS query using a GAS Initial Request frame and the responding STAprovides the query response or information on how to receive the query response in a GAS Initial Response frame. The logical flow of GAS messages is shown in Figure 11-17d.

Figure 11-17d Flow of GAS messages.

11.23.3.2.1 STA Procedures to Transmit a GAS Query

Upon receipt of an MLME-GAS.request primitive, the non-AP STA shall engage in the following procedure to transmit a query:

a)	The non-AP STA sends a GAS query by transmitting a GAS Initial Request frame containing a Dialog Token, an Advertisement Protocol element containing an Advertisement Protocol ID and the Query Request field.

b)	Upon transmission of the GAS Initial Request frame, the STA shall set a timer, referred to as the dot11GASResponseTimer, equal to the dot11GASResponseTimeout MIB object or the QueryFailureTimeout parameter provided in the MLME-GAS.request primitive. If both values are present, the timer shall be set to the lesser of the two values.

c)	If the non-AP STA is not in the associated state, it shall remain in active mode until the receipt of a GAS Initial Response frame with the same Dialog Token as in the GAS Initial Request frame or until the expiry of the timer, whichever occurs first. If the non-AP STA is in the associated state, it may go into power save state until the GAS Initial Response frame is available for receipt or the timer expiry, whichever occurs first.

d)	If the dot11GASResponseTimer expires before a GAS Initial Response frame is received, the  GAS query was not successful and the MLME shall issue an MLME-GAS.confirm primitive indicating “timeout” and shall set the Query Response Length field to 0.

11.23.3.2.2 STA Procedures to Respond to a GAS Initial Request frame

Upon receipt of a GAS Initial Request frame, an MLME-GAS.indication primitive shall be issued to the STA’s SME. Upon receipt of an MLME-GAS.response primitive, the STA shall transmit a GAS Initial Response frame to the requesting non-AP STA according to the following procedures. If the requesting non-AP STA is in the associated state and in the power-save mode, the STA shall buffer the frame for transmission according to the procedures in 11.2.1; otherwise the STA shall queue the frame for transmission.

a)	If the Advertisement Protocol ID in the Advertisement Protocol element does not equal the value contained in any dot11GASAdvertisementID MIB object, then the STA shall not post the query to an advertisement server. The STA shall transmit a directed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code equal to “Advertisement Protocol Not Supported” (see Table 11-4), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame and a Comeback Delay and Query Response Length both set to 0.

b)	If the query request corresponds to an advertisement protocol whose server is currently unreachable, the responding STA shall transmit a directed GAS Initial Response frame to the requesting non-AP STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code equal to “Server Unreachable”, an Advertisement Protocol element containing an Advertisement Protocol ID equal to the Advertisement Protocol ID contained in the GAS Initial Request frame and a Comeback Delay and Query Response Length both set to 0. The method used by the AP to determine the server is unreachable is out of scope of this specification. A STA receiving a status code indicating the server is unreachable should wait at least 1 minute before transmitting any further queries using the same Advertisement Protocol ID to the responding STA.

c)	If the Advertisement Protocol ID in the Advertisement Protocol element equals the value contained in any dot11GASAdvertisementID MIB object, then the STA shall post the query to the advertisement server. The methods and protocols the STA uses to post the query are outside the scope of this specification.

d)	If the query result is immediately available and its length is equal to or less than the maximum MMPDU size, the STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to “success”, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 0, the Query Response and a Query Response Length set to the query response length.

e)	If the query result is immediately available but its length is larger than the maximum MMPDU size, the responding STA shall transmit an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to “success”, an Advertisement Protocol element containingthe Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to 1 TU and a Query Response Length set to 0; this indicates the query response will be transmitted using GAS Comeback Request and Response frames which support GAS fragmentation. and The responding STA shall queue and transmit the query response as described in 11.23.2.3. The requesting STA shall retrieve the query response as described in 11.23.2.4.

f)	If the query result is not immediately available,  the responding STA initializes a timer, referred to as the PostReplyTimer, to the value in dot11GASResponseTimeout MIB object and transmits an individually addressed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code set to “success”, an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame, a GAS Comeback Delay set to the value in dot11GASComebackDelay for this Advertisement Protocol and a Query Response Length set to 0.

| Table 11-4—GAS MLME Primitive’s Encoding of Result Code to Status Code field |  |
| --- | --- |
| StatusCode | ResultCode |
| 59 | ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED |
| 60 | NO_OUTSTSANDING_REQUEST |
| 61 | RESPONSE_NOT_RECEIVED_FROM_SERVER |
| 62 | TIMEOUT |
| 63 | QUERY_RESPONSE_TOO_LARGE |
| 64 | SERVER_UNREACHABLE |
| 65 | REQUESTED_INFO_NOT_AVAILABLE |
| 66 | TRANSMISSION_FAILURE |

11.23.3.2.3 STA Procedures for delivering a GAS Query Response

This clause only applies when the responding STA includes a Comeback Delay having a non-zero value in the Gas Initial Response frame.

After receiving a query response from the advertisement server the STA shall buffer the query response for a minimum of dot11GASResponseBufferingTime after the expiry of the GAS Comeback Delay. If the responding STA does not receive a GAS Comeback Request frame whose source MAC address and Dialog Token match the source MAC address and Dialog Token respectively of the corresponding GAS Initial Response frame within this time, it may drop the query response.

If the query responseis larger than the configured Query Response Length Limit, the responding STA shall discard the response and instead return a status code of “GAS Query Response larger than permitted per configured STA policy” in the GAS Comeback Response frame. This behavior helps to prevent abuses of the medium which may be caused by overly general queries, which evoke a very large query response.

The GAS protocol supports Query Responses whose length is greater than the 802.11 maximum MMPDU size by the STA’s use of the GAS Query Response Fragment ID field in the GAS Comeback Response frame; the Query Response Fragment ID shall be set to 1 for the initial fragment and incremented by 1 for each subsequent fragment in a multi-fragment query response. If the Query Response is a multi-fragment response (i.e., contains more than 1 fragment), the STA shall transmit all fragments that belong to the same Query Response until all fragments are exhausted. The STA shall set the More GAS Fragments field of the GAS Query Response Fragment ID to 0 when the transmitted fragment is the final fragment.

A STA shall use the following procedures to deliver a GAS Query Response.

Upon receipt of a GAS Comeback Request frame, an MLME-GAS.indication primitive shall be issued to the STA’s SME. Upon receipt of an MLME-GAS.response primitive, the STA shall transmit a GAS Comeback Response frame to the requesting STA according to the following procedures. If the requesting STA is in the associated state and in the power-save mode, the responding STA shall buffer the frame for transmission according to the procedures in 11.2.1; otherwise the STA shall queue the frame for transmission.

a)	If the PostReplyTimer expires before the GAS Query Response is received from the advertisement server then the STA shall buffer for transmission a GAS Comeback Response frame with a status code equal to “Timeout” (see Table 11-4). If the query response is subsequently received from the advertisement server, it shall be dropped by the STA.

b)	If the Query Response received from the advertisement server is larger than dot11GASQueryResponseLengthLimit, it shall be dropped by the responding STA. Then the STA shall buffer for transmission a GAS Comeback Response frame with status code set to “Query Response too large”.

c)	If the Query Response is received before the expiry of the PostReplyTimer and its length is less than dot11GASQueryResponseLengthLimit, then the Query Response shall be buffered in one or more GAS Comeback Response frames with status code set to “success”. The STA transmits one GAS Comeback Response frame in response to each GAS Comeback Request frame. If the Query Response received from the advertisement server is less than or equal to the maximum MMPDU payload size, then the GAS Query Response Fragment ID shall be set to zero and the More GAS Fragments field in the GAS Query Response Fragment ID shall be set to zero. If the Query Response received from the advertisement server is greater than the maximum MMPDU payload size, then the GAS Query Response Fragment ID shall be set to zero if this is the first fragment of the Query Response transmitted, otherwise it shall be incremented by 1; the More GAS Fragments field in the GAS Query Response Fragment ID shall be set to one if there are more fragments of the Query Response to be transmitted, otherwise it shall be set to zero (i.e., this fragment is the last fragment of the Query Response).

d)	If a query response has not been received from the advertisement serverand the PostReplyTimer has not expired, the responding STA shall transmit a GAS Comeback Response frame with status equal to “Query response outstanding” (see Table 11-4) and GAS Comeback Delay set to the value in dot11GasComebackDelay for this Advertisement Protocol to indicate when the non-AP STA should comeback to obtain its Query Response.

e)	If a responding STA receives a GAS Comeback Request frame whose source MAC address and Dialog Token do not match the destination MAC address and Dialog Token respectively of an outstanding GAS Initial Response frame, the STA should transmit a GAS Comeback Response frame with a status code equal to “No request outstanding”.

11.23.3.2.4 STA Procedures to retrieve a GAS Query Response

This clause only applies when the responding STA includes a Comeback Delay having a non-zero value in the Gas Initial Response frame.

A requesting STA shall transmit a GAS Comeback Request frame including the Dialog Token (drawn from the corresponding GAS Initial Response frame) immediately after the expiry of the GAS Comeback Delay. In response, the responding STA provides the Query Response in one or more GAS Comeback Response frames with the corresponding Dialog Token.

If a requesting STA receives a GAS Comeback Response frame with status set to “Query response outstanding”, the requesting STA shall wait for the GAS Comeback Delay from that frame and upon expiry of the GAS Comeback Delay, transmit another GAS Comeback Request frame. If the requesting STA’s dot11GASResponseTimer (set in 11.23.3.2.1 step b) expires prior to receiving a GAS Comeback Response frame whose source MAC address and Dialog Token match those in the corresponding GAS Initial Response frame, the STA shall issue an MLME-GAS.confirm primitive with result code set to “timeout” and shall set the Query Response Length to 0.

If a requesting STA receives a GAS Comeback Response frame with status set to “success” and the More GAS Fragments field in the GAS Query Response Fragment ID set to one, it shall transmit another GAS Comeback Request frame in order to retrieve the next GAS fragment of a multi-fragment query response.

If a requesting STA receives a GAS Comeback Response frame with status set to “success” and the More GAS Fragments field in the GAS Query Response Fragment ID set to zero, the requesting STA’s MLME shall determine that all fragments have been received by confirming that all fragment IDs from 0 to the value in the GAS Query Response Fragment ID when the More GAS Fragments field was set to 0 have been received. Upon receipt of the first GAS Comeback Response frame and every GAS Comeback Response frame thereafter, the dot11GASResponseTimer shall be reset. If all of the query response fragments were received before the expiry of the dot11GASResponseTimer, then the MLME shall issue an MLME-GAS.confirm with result code set to “success” along with the query response. If all of the query response fragments were not received before the expiry of the dot11GASResponseTimer, then the MLME shall issue an MLME-GAS.confirm with result code set to “transmission failure” and shall set the Query Response Length to 0.

After a requesting STA receives the first GAS fragment of a multi-fragment query response, it shall continue retrieving the query response until all GAS fragments are received or until a transmission failure is detected; the requesting STA shall not commence the retrieval of a another GAS Query Response from the same STA until all GAS fragments are received or until a transmission failure is detected on the first GAS Query Response.

If a requesting STA receives a GAS Comeback Response with status set to “Timeout” or “Query Response too large”, then the MLME shall issue an MLME-GAS.confirm with result code so indicating and shall set the Query Response Length to 0.

If a requesting STA receives a GAS Comeback Response with status set to “No request outstanding”, then the MLME shall issue an MLME-GAS.confirm with result code set to “unspecified failure” and shall set the Query Response Length to 0.

11.23.3.2.5 GAS procedures interaction with Multiple BSSID Set

Non-AP STAs in the un-associated state may use GAS procedures to query advertisement servers for information. As described in 11.23.3.2, APs indicate their support for a particular GAS Advertisement Protocol by including an Advertisement protocol element with that Advertisement protocol ID in Beacon and Probe Response frames as described in 7.2.3.1 and 7.2.3.9 respectively. Non-AP STAs receiving Beacon or Probe Response frames from different APs may choose to engage in GAS frame exchange sequences with one or more of these APs. In some deployment scenarios, these APs may be operating as a Multiple BSSID set (as defined in 11.10.11) and may relay the GAS queries to the same advertisement server. Depending on the configuration of the IEEE 802.11 access network, the external network and the advertisement server, a query response from the advertisement server may or may not be dependent on the BSSID used in the GAS frame exchange sequence and thus the STA from which the query was relayed. If the GAS Query Response is dependent on the BSSID, a non-AP STA may choose to post queries using GAS procedures to more than one STA and expect possibly different Query Responses. If the Query Response is not dependent on the BSSID, then a non-AP STA may choose to post queries using GAS procedures to only one STA in the Multiple BSSID set (i.e., posting the same query to another member of the Multiple BSSID set would yield the same response).

When a Multiple BSSID (as defined in 11.10.11) set contains two or more members and dot11InterworkingServiceEnabled is true and dot11GASAdvertisementID is not null and a query to the advertisement server corresponding to the value of dot11GASAdvertisementID is not dependent on the BSSID value used in the GAS frame exchange sequence to post the query, then the PAME-BI bit in the Advertisement protocol tuple of the Advertisement protocol element corresponding to the value of dot11GASAdvertisementID shall be set to 1; otherwise this bit shall be set to zero.

11.23.4 Interworking Procedures: IEEE 802.21 MIH Support

The IEEE 802.21 MIH (Media Independent Handover) standard supports handovers across heterogeneous networks. STAs with dot11InterworkingServiceEnabled set to TRUE and having the dot11GasAdvertisementId MIB object set to MIH Information Service (see Table 7-43bl) shall support the transmission and reception of MIIS queries for STAs in all states. STAs with dot11InterworkingServiceEnabled set to TRUE and having a dot11GasAdvertisementId MIB object set to MIH Command and Event Services Capability Discovery (see Table 7-43bl) shall provide support for MICS/MIES capability discovery for non-AP STAs in all states.

Additionally, support for MIIS query and MICS/MIES capability discovery to non-AP STA's in the associated state is provided by the STA forwarding IP datagrams destined for the MIH PoS to the MIIS server.

A non-AP STA discovers support for these services by receiving Beacon or Probe Response frames with an Advertisement Protocol element having Advertisement Protocol ID(s) for MIH Information Service and/or MIES/MICS capability discovery.

Non-AP STAs in the un-authenticated or un-associated or associated states can use GAS procedures to discover MIH Command and Event Services Capability as specified in Table 7-43bl.

11.23.7 Interworking Procedures: Emergency Alert System (EAS) Support

The Emergency Alert System (EAS) provides alerts, typically issued by authorities. The Interworking Procedures EAS support enables the alerts to be transmitted upon request from APs to non-AP STAs. Subsequent to advertisement in Beacon and Probe Response frames, a non-AP STA uses GAS queries to retrieve an EAS message from the network according to the following procedures.

When dot11EASEnabled is true, EAS operation shall be supported. When EAS operation is not supported, dot11EASEnabled shall be set to FALSE.

When the IEEE 802.11 infrastructure is informed of the availability of an EAS message (the mechanism by which is out of scope of this standard), an AP with dot11EASEnabled set to TRUE shall advertise the availability of the EAS message by including an Emergency Alert Identifier element (see 7.3.2.95) for that message in its Beacon and Probe Response frames. The AP shall include one instance of an Emergency Alert Identifier element in its Beacon and Probe Response frames for each active EAS Message. The Emergency Alert Identifier element provides an Alert Identifier Hash value, a unique indicator of the EAS Message of the alert to the non-AP STA. The Alert Identifier Hash value allows the non-AP STA to determine whether this is a new alert.

NOTE—The same value of hash will be computed by each AP in an ESS and by each AP in different ESSs. Thus a non-AP STA, which can download emergency alert messages when in a pre-associated state, can unambiguously determine that it has already downloaded the message, avoiding unnecessary duplicates.

When an EAS Message has expired (the mechanism by which is out of scope of this standard), an AP with dot11EASEnabled set to TRUE shall remove the corresponding instance of an Emergency Alert Identifier element from its Beacon and Probe Response frames.

The Alert Identifier Hash in the Emergency Alert Identifier element shall be computed using HMAC-SHA1-64 hash algorithm as shown in 7.3.2.95.

Upon receiving an Emergency Alert Identifier element for an EAS Message which has not already been retrieved from the network, a non-AP STA having dot11EASEnabled set to TRUE shall retrieve the EASN message from the AP either:

—	using the GAS procedures, indicating to the AP the hash of the message it wants to retrieve, or

—	retrieve the Emergency Alert Server URI (see 7.3.4.15) using an IQP query according to the procedures in 11.23.3.1.1. then the STA shall form the EAS Message URI by concatenating the Emergency Alert Server URI with the hexadecimal numerals of the Alert Identifier Hash converted to UTF-8 encoded characters and the “.xml” file extension.

Example: If the Emergency Alert Server URI is http://eas.server.org, the Alert Identifier Hash is 0x1234567890abcdef, then the EAS Message URI is http://eas.server.org/1234567890abcdef.xml.

A non-AP STA in the un-associated state having dot11EASEnabled set to TRUE shall retrieve the EAS message using GAS procedures defined in 11.23.3.2 with Advertisement Protocol ID set to the value for EAS (see Table 7-43bl). A non-AP STA in the associated state where dot11EASEnabled is set to true shall retrieve the EAS message using either GAS procedures defined in 11.23.3.2 with Advertisement Protocol ID set to the value for EAS (see Table 7-43bl) or HTTP using Internet Protocols (the latter being preferred).

11.23.10 Support for TDLS Discovery

The TDLS Discovery element (7.3.4.16) can be used in an IQP query to discover TDLS capabilities (e.g. another TDLS capable STA) using public action frames. An example of TDLS discovery using IQP is given in Figure 7-101bg1. The discovery can work as follows:

1)	STA1 determines the MAC address of STA2.

2)	STA1 sends an IQP request to STA2. The IQP request includes a TDLS Discovery element.

3)	STA2 receives the IQP request frame and includes the TDLS Discovery element in the IQP response frame to STA1.

Figure 7-101bg1—Example TDLS capability discovery using GAS

A.4.22 Interworking (IW) with External Networks extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Interworking with External Networks capabilities supported? |  |  |  |
| IW1 | Interworking capabilities and Information | 7.3.2.90, 11.23.2 | CF19:M | Yes, No, N/A |
| IW1.1 | Interworking information element | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.2 | Network Type | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.3 | 802.11 Venue Type | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.4 | HESSID | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.5 | Interworking Action frame | 7.4.7a | IW1:M | Yes, No, N/A |
| IW2 | Generic Advertisement Services | 11.23.3 | CF19:M | Yes, No, N/A |
| IW2.1 | Advertisement Protocol element | 7.3.2.91 | IW2:M | Yes, No, N/A |
| IW2.2 | GAS Protocol | 11.23.3.2 | IW2:M | Yes, No, N/A |
| IW2.2.1 | Information Query Protocol | 7.3.4, 7.3.4.6 | IW2.2:M | Yes, No, N/A |
| IW2.2.2 | IQP Capability List | 7.3.4.1 | IW2.2.1:M | Yes, No, N/A |
| IW2.2.3 | Venue Name information | 7.3.4.2 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.4 | Emergency Call Number information | 7.3.4.3 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.5 | Network Authentication Type information | 7.3.4.4 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.6 | Roaming Consortium List | 7.3.4.5 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.7 | IP Address Type Availability information | 7.3.4.7 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.8 | NAI Realm List | 7.3.4.8 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.9 | 3GPP Cellular Network information | 7.3.4.9 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.10 | AP Geos Location | 7.3.4.10 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.11 | AP Civic Location | 7.3.4.11 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.12 | Domain Name List | 7.3.4.12 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.13 | Emergency Alert URI | 7.3.4.13 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.14 | Information Query Protocol vendor-specific list | 7.3.4.6 | IW2.2.1:O | Yes, No, N/A |
| IW2.3 | GAS Protocol | 11.23.3.2 | IW2:O | Yes, No, N/A |
| IW2.3.1 | MIH IS | 7.3.2.91 | IW2.3:O | Yes, No, N/A |
| IW2.3.2 | MIH Event and Command Serivices Discovery | 7.3.2.91 | IW2.3:O | Yes, No, N/A |
| IW2.3.3 | Emergency Alert System (EAS) | 7.3.2.91, 7.3.2.95 | IW2.3:O | Yes, No, N/A |
| IW2.3.4 | Location-to-Service Translation Protocol | 7.3.2.91 | IW2.3:O | Yes, No, N/A |
| IW2.3.5 | Advertisement Protocol ID, Vendor Specific | 7.3.2.91 | IW2.3:O | Yes, No, N/A |
| IW2.4 | GAS Initial Request frame | 7.4.7.14 | IW2:M | Yes, No, N/A |
| IW2.5 | GAS Initial Response frame | 7.4.7.15 | IW2:M | Yes, No, N/A |
| IW2.6 | GAS Comeback Request frame | 7.4.7.16 | IW2:M | Yes, No, N/A |
| IW2.7 | GAS Comeback Response frame | 7.4.7.17 | IW2:M | Yes, No, N/A |
| IW3 | QoS Mapping from External Networks | 11.23.9 9.9.3.1, 9.9.3.2 | CF19:O | Yes, No, N/A |
| IW3.1 | QoS Map Set element | 7.3.2.93 | IW3:M | Yes, No, N/A |
| IW3.2 | Transport of QoS Map Set | 11.23.9 | IW3:M | Yes, No, N/A |
| IW3.3 | QoS Map Configure | 7.4.2.5 | IW3:M | Yes, No, N/A |
| IW4 | MIH Support | 11B, 11.23.4 | CF19:O | Yes, No, N/A |
| IW4.1 | MAC State Generic Convergence Function Support | 11B | IW4:M | Yes, No, N/A |
| IW4.2 | Informational events | 11B.3 | IW4:M | Yes, No, N/A |
| IW4.3 | ESS status reporting | 11B.4.1 | IW4:M | Yes, No, N/A |
| IW4.4 | Network configuration | 11B.4.2 | IW4:M | Yes, No, N/A |
| IW4.5 | Network events | 11B.4.3 | IW4:M | Yes, No, N/A |
| IW4.6 | Network command interface | 11B.4.4 | IW4:M | Yes, No, N/A |
| IW4.7 | Mobility management | 11B.5.1 | IW4:M | Yes, No, N/A |
| IW4.8 | Network configuration | 11B.4.2 | IW4:M | Yes, No, N/A |
| IW5 | Extended channel switch enabled | 7.3.2.58, 11.1.3 | (CF15 AND DSE9):M | Yes, No, N/A |
| IW6 | Expedited Bandwidth Request | 7.3.2.92 | CF19:O | Yes, No, N/A |
| IW7 | SSPN Interface | 11.23.5 | CF19:O | Yes, No, N/A |

Annex D

(normative) 

ASN.1 encoding of the MAC and PHY MIB

Change the dot11StationConfigEntry list in Annex D by inserting the shown entries:

-- **********************************************************************

-- * dot11DomainName TABLE

-- **********************************************************************

dot11DomainNameTable   OBJECT-TYPE

SYNTAX                SEQUENCE OF Dot11DomainNameEntry

MAX-ACCESS            not-accessible

STATUS                current

DESCRIPTION

This is a table of Domain Names which form the Domain Name List in Information Query Protocol. The Domain Name List may be transmitted to a non-AP STA in a GAS Response. Each table entry corresponds to a single Domain Name.

::= { dot11imt 6 }

-- **********************************************************************

-- * dot11RoamingConsortium TABLE

-- **********************************************************************

dot11RoamingConsortiumTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RoamingConsortiumEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“This is a Table of OIs which are to be transmitted in an IQP Roaming Consortium List. Each table entry corresponds to a roaming consortium or single SSP. The first 3 entries in this table are transmitted in Beacon and Probe Response frames.”

::= { dot11imt 5 }

-- **********************************************************************

-- * dot11GASAdvertisement TABLE

-- **********************************************************************

dot11GASQueryResponseLengthLimit OBJECT-TYPE

SYNTAX INTEGER (1..127)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object indicates the maximum number of octets an STA will transmit in one or more Query Response fields contained within GAS Comeback Response  frame(s). A value of 127 means the maximum limit enforced is  contained by the maximum allowable number of fragments in the GAS Query  Fragment Response ID”

::= { dot11GASAdvertisementEntry 5}

Insert the following annex to the proper sequence of annexes:

Annex W

(informative) 

Interworking with External Networks

W.1.1 Airport

A business traveler arrives for the first time into an airport having a WLAN. The user wants to download email onto their laptop utilizing the airport’s hotspot, a chargeable network. Once associated, the user needs to connect via VPN connection back to their company’s servers to access email and information from the customer database.

1)	The laptop’s non-AP STA performs an active scan by transmitting a Probe Request frame containing the wildcard SSID and an Interworking element with Network Type subfield set to “Chargeable Public Network”. In response, it receives Probe Response frames from several of the airport’s APs, in the immediate neighborhood, for the SSID “Narita Hotspot”.

2)	The Probe Response received by the laptop indicated the following capabilities:

a)	Extended capabilities element indicates: AP provides Interworking Service.

b)	Interworking element indicates: venue group = 1 (Assembly) and 802.11 Venue Type = 3 (passenger terminal), Internet = 1 (Internet access available), ASRA = 1 (there is an additional step required for network access).

c)	Advertisement Protocol element indicating AP supports GAS for IEEE 802.21-IS.

d)	Roaming Consortium element present containing an OI for “Hotspot Roaming International”.

e)	There is no RSN element present in the received beacon frame.

3)	Since the laptop’s SME does not recognize the Roaming Consortium OI, it invokes the GAS protocol to query the network’s IEEE 802.21-IS. The IEEE 802.21-IS’s response indicates the roaming partners for “Narita Hotspot” and the laptop has security credentials for one of them.

4)	Since the AP indicated ASRA = 1, the SME again invokes the GAS protocol to retrieve the Network Authentication Type information. The response indicates that https redirection is in use and provides the Re-direct URL of “hotspot.narita.co.jp”. Note that this is helpful since some networks use conditional re-direction—that is, access to a walled garden is provided for free, but a subscription fee is required to access the Internet.

5)	Since the Laptop’s SME now knows it should be able to successfully authenticate with the network, the STA associates to the AP.

6)	The following operations are then carried out by higher layers operating within the laptop:

a)	The laptop’s SME autonomously launches an http client providing to it the URL of hotspot.narita.co.jp which provides the proper security credentials to the network, thereby successfully authenticating it to the network.

b)	The VPN client is autonomously launched, establishing a secure session to user’s corporate network. Then the user launches the email application to download email and other required information.

W.1.2 Shopping

A shopper visits a shopping mall and wants to use a smartphone to discover items on sale. In this mall, the mall’s IT department is providing WLAN facilities for all the stores in the mall, so there is only one SSID for shoppers (i.e., there is not a different SSID for each store in the mall). The user arrives at the mall and taps an icon on the screen to put the smartphone in “shopping mode”. The smartphone’s shopping application causes the non-AP STA to carry out the following steps:

1)	The smartphone’s non-AP STA performs an active scan by transmitting a Probe Request frame containing the wildcard SSID and an Interworking element with Network Type subfield set to “Free Public Network”. In response, it receives Probe Response frames from several of the mall’s APs, but only one SSID is provided which is “Silicon Valley Mall”. The mall’s APs did not transmit Probe Responses for the SSIDs “Engineering”, “Deliveries” and “Janitorial” since their Network Type is “Private network”.

2)	The Probe Response received by the smartphone indicated the following capabilities:

a)	Extended capabilities element indicates: AP provides Interworking Service.

b)	Interworking element indicates: venue group = 6 (mercantile) and 802.11 Venue Type = 4 (shopping mall), Internet = 0 (unspecified).

c)	RSN element indicates: IEEE 802.1X authentication.

3)	Since the AP indicated Interworking service is available, the smartphone’s non-AP STA uses the MLME-GAS.request primitive to invoke GAS to request the IQP Capabilities List (see 7.3.4.1). In the IQP Capabilities List, the AP has indicated support for Venue Name and Domain Name List. Subsequent to receipt of the IQP Capabilities List, the non-AP STA invokes the MLME-GAS.request primitive to retrieve the other two lists.

4)	Next, the non-AP STA’s supplicant searches the received Domain Name list to determine whether it has any stored credentials for these domains. If so:

a)	The smartphone autonomously associates to the “Silicon Valley Mall Shopping” SSID and displays the information shown below:

i)	Venue Name: Silicon Valley Mall, 1234 Main Street, Rownhams, CA 98765-1234

ii)	SSID: Silicon Valley Mall

iii)	802.11 Venue type: Shopping Mall

b)	The supplicant autonomously provides the security credentials for the selected domain.

5)	Higher-layer protocols then download discount coupons being offered for items on sale.

W.1.3 Sales Meeting

A sales person travels across town to a meeting at ACME manufacturing. While there, the sales person needs to send email to get a document from engineering. On a laptop, the user requests the WLAN via the laptop’s UI to search for guest networks. The laptop performs the following steps.

1)	The laptop’s non-AP STA performs an active scan by transmitting a Probe Request frame containing the wildcard SSID and an Interworking element with Network Type subfield set to “Private Network with Guest Access”. In response, it receives Probe Response frames from several of ACME Manufacturing’s APs, but only one SSID is provided which is “Guest”. ACME Manufacturing’s APs did not transmit Probe Responses for the SSIDs “Engineering” and “Finance” since their Network Type is “Private network”.

2)	The Probe Response received by the laptop indicated the following capabilities:

a)	Extended capabilities element indicates: AP provides Interworking Service

b)	Interworking element indicates: Internet is available, venue group = 2 (Business) and 802.11 Venue Type = 8 (Research and Development Facility).

c)	RSN element indicates: IEEE 802.1X authentication with CCMP pairwise and group cipher suites.

3)	Since the AP indicated Interworking service is available, the laptop’s non-AP STA uses the MLME-GAS.request primitive to invoke GAS to request the IQP Capabilities List (see 7.3.4.1). In the IQP Capabilities List, the AP has indicated support for Venue Name. Upon receipt of the IQP Capabilities List, the non-AP STA again invokes the MLME-GAS.request primitive to retrieve the Venue Name.

4)	The laptop’s UI displays the following information, and automatically associates to the network:

a)	SSID: Guest (Type: Private network with Guest access)

b)	Venue Name: ACME Manufacturing, 1234 Main Street, Rownhams, CA 98765-1234

c)	802.11 Venue Type: Research and Development Facility

d)	Internet is available

5)	Upon prompt, the user enters the username and password supplied by their point of contact from ACME Manufacturing and is then able to send and receive email.

W.1.4 Museum

A visitor enters a Museum which is advertising virtual docent service (audio tracks describing each of the major exhibits). The visitor taps an icon on a smartphone, requesting it to search for free networks. The smartphone then carries out the following:

1)	The smartphone’s non-AP STA performs an active scan by transmitting a Probe Request frame containing the wildcard SSID and an Interworking element with Network Type subfield set to “Free Public Network”. In response, it receives Probe Response frames from several of the muse1)	ums APs, but only one SSID is provided which is “Visitors”. The museum’s APs did not transmit Probe Responses for the SSID “Maintenance” since its Network Type is “Private network”.

2)	The Probe Response received by the smartphone indicated the following capabilities:

a)	Extended capabilities element indicates: AP provides Interworking Service

b)	Interworking element indicates: venue group = 1 (assembly), 802.11 Venue Type = 9 (museum), and ASRA = 0 (no additional steps are required for access)

3)	Since the AP indicated Interworking service is available, the smartphone’s non-AP STA uses the MLME-GAS.request primitive to invoke GAS to request the IQP Capabilities List (see 7.3.4.1). In the IQP Capabilities List, the AP has indicated support for Venue Name. Upon receipt of the IQP Capabilities List, the non-AP STA again invokes the MLME-GAS.request primitive to retrieve the Venue Name.

4)	The smartphone’s UI displays the following information, asking the user whether or not they wish to connect to the network:

a)	Venue Name: Museum of Modern Art (MOMA)

b)	SSID: Visitors

c)	802.11 Venue Type: Museum

d)	No authentication required

5)	The user taps the “Connect” icon on the smartphone’s display. Note that the smartphone’s non-AP STA knows that the network uses open system authentication since there is no RSN element present in the beacon and ASRA = 0.

W.1.6 Emergency alert

A traveller has enabled display of Emergency Alerts on their wireless device (non-AP STA), by appropriate setting of the higher-layer emergency alert application on their device. The traveller arrives at a new destination and turns on the device. The device, when switched on, will perform a search and then associate with an AP (access point) that the traveller has a subscription to, or with an open AP (if the traveller has enabled the device to do that). During the steps leading up to association, the device, when it becomes aware of an Emergency Alert, will obtain and display it. The emergency alert will likely be obtained from the AP that has the service the traveller has subscribed to, but the device may also obtain the emergency alert from other APs, if an AP that the traveller has a subscription for is not available.

1)	The traveller’s non-AP STA performs an active scan by transmitting a Probe Request frame containing the wildcard SSID. If it is associated with an AP, it checks the beacon.

2)	In response, it receives Probe Response frames from one or more APs which it checks  (or, alternately, checks the beacon) for the subscription and also whether the Emergency Alert information element indicates that there are emergency alert message(s).

3)	If there are one or more Emergency Alert messages to be downloaded, the Emergency Alert Identifier element provides an Alert Identifier Hash value.

3)	The device uses the hash of each of the available emergency message(s) to determine whether that message has already been received, or whether it is a new message that must be downloaded.

4)	If there are one or more Emergency Alert messages to be downloaded, the device forms the EAS Message URI by concatenating the Emergency Alert Server URI with the hexadecimal numerals of the Alert Identifier Hash, using the specified method.

5)	Should the device be un-associated with an AP when it determines that a new Emergency Alert is available, it retrieves the EAS message using either GAS proceedures with Advertisement Protocol ID set to the value for EAS. If the device is associated with the AP, it retrieves the EAS message using either GAS proceedures with Advertisement Protocol ID set to the value for EAS or, preferrably, HTTP using Internet Protocols.

Note—there are 10 IDs assigned for the Secondary NAI Realm List and each Secondary NAI Realm List having a different Info ID provides different information. This allows an AP to provide additional NAI Realm information as needed by a particular 802.11 access network.