IEEE P802.11
Wireless LANs

| Emergency service Support |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Rene Purnadi | RIM | 5000 Riverside Drive Irving, TX 75039, USA | +1 972 373 1739 | rpurnadi@rim.com |
| Jarkko Kneckt | Nokia | Rakentajainrinne 6 02330 Espoo, Finland |  | Jarkko.Kneckt@nokia.com |
| Stephen McCann | Research in Motion UK Ltd | 200 Bath Road, Slough, Berks, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |
| Ashish Shukla | Marvell | 1st Floor, Muttha Tower, Don Bosco Marg, Yerwada, Pune, India. | +91-20-40130016 | ashishs@marvell.com |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4018 | KazuyukiA.Sakoda@jp.sony.com |
| Henry Qu | Honeywell | 19979 Stevens Creek Boulevard, Cuppertino, CA 95014 | +1 408-343-3736 | Huyu.Qu@honeywell.com |

Abstract

This submission suggests to place initial hooks to support emergency service over mesh network.

It also suggests resolutions to CIDs 2474, 2475, 2476, 2477, 2482, 2486.

| CID | Comment / Explanation | Recommended Change | Resolution Code | Resolution Notes |
| --- | --- | --- | --- | --- |
| 2474 | Considering of supporting emergency call over mesh network, the matching Mesh ID may be an obstacle to allow an MSTA that do not have a matching Mesh-ID to initiate an emergency call. The MSTA should have a default emergency call profile | add emergency call profile beside the other profile(s) corresponding to Mesh ID(s). I will provide solution through submission | Reject | Currently one mesh STA is capable to operate in single mesh profile at a time. |
| 2475 | The mesh network should be capable to support emergency call. If the mesh network is enabled to support emergency call, the Mesh configuration element should contain the emergency call support capability (similar to TG-U's ESC and UESA). | Add information element similar to TG-U's ESC and UESA in the Mesh configuration element. I will provide solution through submission | Counter | The information elements for the beacon frame will be copied from the 802.11u. Thus, they are automatically added, if the conditions as specified in 802.11u are met. |
| 2476 | the mesh peering open should have an indication that the peering is for an emergency call so that regardless of the Mesh ID, the emergency call profile is the one that is active. The receiving MSTA should accept the request even though the Accepting Mesh Peering bits is set to 0 (e.g. the receiving MSTA may need to drop the other peering to accommodate the emergency peering) | if peering is for an emergency call, add an emergency call indication in the peering open frame. I will provide solution through submission | accept | accept. Submission required (see 802.11-10-0269R1) |
| 2477 | The emergency call may not need authentication. In case emergency call need no authentication, an emergency call indication is needed to bypass the authentication during peering | if peering is for an emergency call, add an emergency call indication in the peering open frame. I will provide solution through submission | accept. | accept. Submission required (see 802.11-10-0269R1) |
| 2482 | If the portal is able to support emergency call (e.g. can connect to PSAP or Public Safety Answering Point), it should indicate so | add TG-U's like element (ESC and UESA) in the PANN element. I will provide solution through submission | Counter | Accept in principle. The PANN element adds information elements to provide emergency signaling information. |
| 2486 | If the root is able to support emergency call (e.g. belongs to PSAP) it should include either TG-U's like element ESC or UESA or an indication that it supports emergency call | add ESC and UESA or emergency indication in the RANN element. I will provide solution through submission | Counter | Accept in principle. The RANN element adds information elements to provide emergency signaling information. |

Add this section after 5.2.13.5.11

5.2.13.5.12 Emergency service support over mesh network

An authenticated or an unauthenticated emergency service should be supported over mesh network. The Beacon and Probe Response should inform whether a mesh STA supports an emergency service. When a mesh portal or a root mesh STA supports an emergency service, it should also inform this capability in the portal announcement or in the root announcement.

This information helps a mesh STA to select which mesh STA to have peering with. When a mesh STA starts mesh peering, it indicates whether the peering is for emergency service or for a normal peering. A peering for emergency service will be put in the highest priority and may proceed without authentication.

Modify Table 7-26 as follow

| - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| Information element | Element ID | Total length of element in octets including the Type and Length octets | Extensible |
| … | … | … | … |
| Mesh Peering Management Error: Reference source not found |  | 89, 101, or 123 | Yes |
| Portal Announcement (PANN) Error: Reference source not found |  | 178 | Yes |
| Root Announcement (RANN) RANN element |  | 234 | Yes |
| … | … | … | … |

Modify section7.3.2.89 as follow (TG-u 8.01D):

7.3.2.89 Interworking information element

The Interworking information element contains information about the Interworking Service capabilities of a STA as shown in Figure 7-95o113.

|  | Element ID | Length | Access Network Options | Venue Info (optional) | HESSID (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 0 or 2 | 0 or 6 |
| Figure 7-95o113—Interworking element format |  |  |  |  |  |

The Length is a one-octet field whose value is 1 plus the length of each optional field present in the element.

The format of Access Network Options field is shown in Figure 7-95o114.

| Bits: | B0 - B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- |
|  | Network Type | Internet | ASRA | ESC | UESA |
| Figure 7-95o114—Access Network Options format |  |  |  |  |  |

A non-AP STA sets Internet, ASRA, ESC and UESA fields to 0 when including the Interworking element in the Probe Request frame. A non-AP STA sets the Internet, ASRA, and ESC bits to 0 when including the Interworking element in (Re)association request frames. In (Re)association request frames, a non-AP STA sets the UESA bit according to the procedures in 11.23.5. A mesh STA sets Internet to 0 or 1 depending whether it is connected to a mesh portal and whether the mesh portal is connected to internet or not, and ASRA fields to 0 when including the Interworking element in Beacon frame and Probe Response frame. The Network Type Codes are shown in Table 7-43bb. The Network Type field is set by the AP to advertise its Network Type to non-AP STAs. A non-AP STA uses this field to indicate the desired Network Type in an active scan. See Annex W.1 for informative text on usage of fields contained within the Interworking element.

Table 7-43bb—Network Type codes

| Network Type Codes | Meaning | Description |
| --- | --- | --- |
| 0 | Private network | Non-authorized users are not permitted on this network. Examples of this network type are home networks and enterprise networks, which may employ user accounts. Private networks do not necessarily employ encryption. |
| 1 | Private network with guest access | Private network but guest accounts are available. Example of this network type is enterprise network offering access to guest users. |
| 2 | Chargeable public network | The network is accessible to anyone, however, access to the network requires payment. Further information on types of charges may be available through other methods (e.g., 802.21, http/https redirect or DNS redirection). Examples of this network type is a hotspot in a coffee shop offering internet access on a subscription basis or a hotel offering in-room internet access service for a fee. |
| 3 | Free public network | The network is accessible to anyone and no charges apply for the network use. An example of this network type is an airport hotspot or municipal network providing free service. |
| 4 | Personal Device Network | A network of personal devices. An example of this type of network is a camera attaching to a printer, thereby forming a network for the purpose of printing pictures. |
| 5 to 13 | Reserved | Reserved |
| 14 | Test or experimental | The network is used for test or experimental purposes only. |
| 15 | Wildcard | Wildcard network type |

Bit 4 is the Internet field. The AP sets this field to 1 if the network provides connectivity to the Internet; otherwise it is set to 0 indicating that it is unspecified whether the network provides connectivity to the Internet.

Bit 5 is the Additional Step Required for Access (ASRA) field. It is set to 1 by the AP to indicate that the network requires a further step for access. It is set to 0 whenever dot11RSNAEnabled is true. For more information, refer to Network Authentication Type Information in 7.3.4.4. The non-AP STAs set this bit to 0 in Probe Request frames.

Bit 6 is the Emergency Services Capability (ESC) field. It is set to 1 by the AP or mesh STA to indicate that higher layer Emergency Services are available at the AP or mesh STA. When ESC field is set to 0, the Emergency Services are not supported, see 11.23.5. The non-AP STAs set this bit to 0 in Probe Request frames.

Bit 7 is the Unauthenticated Emergency Service Accessible (UESA) field. When the AP or mesh STA sets it to 0, this field indicates that no unauthenticated emergency services are reachable through a BSS using this SSID or MBSS using the Mesh-ID of the MBSS. When set to 1, this field indicates that higher layer unauthenticated emergency services are reachable through a BSS using this SSID or MBSS using this Mesh-ID. A STA uses the Interworking information element with the UESA bit set to 1 to gain unauthenticated access to a BSS to access emergency services. See 11.23.5 together with Annex W.4.2 and Annex W.4.4. A non-AP STA sets the UESA field to 0 in Probe Request frames.

The Venue Info field is a 2-octet field. It contains Venue Group and Venue Type fields. The format of Venue Info field is shown in Figure 7-95o115.

|  | Venue Group | Venue Type |
| --- | --- | --- |
| Octets: | 1 | 1 |
| Figure 7-95o115—Venue Info format |  |  |

The Venue Group and Venue Type fields are both one octet values selected from Table 7-43bc and Table 7-43bd respectively. The entries in Table 7-43bc and Table 7-43bd are drawn from the International Building Code’s Use and Occupancy Classifications [B52].

Modify section7.3.2.99 as follow:

- Mesh Peering Management element

The Mesh Peering Management element is transmitted by a mesh STA to manage a mesh peering with a peer mesh STA. The format of the Mesh Peering Management element is shown in Figure s20 (Mesh Peering Management element).

| Element ID | Length | Mesh Peering Protocol Identifier | Local Link ID | Peer Link ID (conditional) | Emergency service Identifier | Reason Code (conditional) | Chosen PMK (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 4 | 2 | 2 | 1 | 2 | 16 |
| - Mesh Peering Management element |  |  |  |  |  |  |  |

…………

| - Mesh Peering Protocol Identifier valuesinput.doc |  |  |
| --- | --- | --- |
| OUI | Value | Meaning |
| 00-0F-AC | 0 | Mesh Peering Management Protocol |
| 00-0F-AC | 1 | Authenticated Mesh Peering Exchange Protocol |
| 00-0F-AC | 2-255 | Reserved |
| Vendor OUI | 0-255 | Vendor Specific |

The Local Link ID is the unsigned integer value generated by the local mesh STA to identify the mesh peering instance. This field is present for all three types of Mesh Peering Management elements.

The Peer Link ID is the unsigned integer value generated by the peer mesh STA to identify the mesh peering instance. This field is not present for the Mesh Peering Open frame, is present for the Mesh Peering Confirm frame, and is optionally present for the Mesh Peering Close frame.

The format of the Emergency service Identifier is shown in figure s-21a and contains the following bit

| B0 | B1 B7 |
| --- | --- |
| EI | Reserved |
| Bits: 1 | 7 |

- Emergency service Indicator (EI) bit (bit 0) defines when a mesh STA initiates or responds a mesh peering for an emergency service. If EI=1, the mesh peering is intended for an emergency service. If EI=0 the peering is intended for a normal peering.

The Reason Code field enumerates reasons for sending a Mesh Peering Close. It is present for the Mesh Peering Close frame and is not present for Mesh Peering Open or Mesh Peering Confirm frames. The reason code is defined in Error: Reference source not found.

Modify section 7.3.2.107 as follows

- PANN element

The Portal Announcement (PANN) element is used for announcing in the MBSS the presence of a mesh STA collocated with a portal.

The PANN element is transmitted in a Beacon or a Mesh Interworking Action frame (see 7.4.17 (Mesh Interworking Action frame details)).

The format of the PANN element is shown in PANN element.

| Element ID | Length |  | Flags |  | Hop count |  | Time to Live |  | Mesh Portal Address |  | Emergency service Information |  | PANN Sequence Number |  | Interval |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 6 |  | 1 |  | 4 |  | 2 |  |
| - PANN element |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given inElement IDsinput.doc for this element. The length is set to 156.

The Flags field is reserved for future use.

The Hop Count field is coded as an unsigned integer. When the mesh STA collocated with a portal sends the PANN primitive, it sets the hop count to 0. The Hop Count field indicates the number of hops the PANN primitive has traversed.

The Time to Live field is coded as an unsigned integer and indicates the maximum number of hops allowed for this element.

The Mesh Portal Address is represented as a 48-bit MAC address and is set to the MAC address of the mesh STA that is collocated with the portal.

The Emergency service Information field consists of:

B0-B5: reserved

B6: ESC (Emergency Service Capability) is set to 1 if emergency service is supported, 0 otherwise

B7: UESA (Un-authenticated Emergency Service Accessible) is set to 1 if an unauthenticated emergency service is allowed, 0 otherwise

The PANN Sequence Number field is coded as an unsigned integer and is set to a PANN Sequence Number specific for the originating mesh STA collocated with the portal.

The Interval field is coded as an unsigned integer and is set to the number of seconds between the periodic transmissions of Portal Announcements by the mesh STA collocated with the portal.

Detailed usage of the PANN element is described in Error: Reference source not found

Modify section 7.3.2.108 as follows

- RANN element

The Root Announcement (RANN) element is used for announcing the presence of a mesh STA configured as root mesh STA with dot11MeshHWMProotMode set to rann (4). RANN elements are sent out periodically by such a root mesh STA.

The RANN element is transmitted in a Beacon or a Mesh Path Selection Action frame (see 7.4.16 (Mesh Path Selection Action frame details)).

The format of the RANN element is shown in RANN element.

| Element ID | Length | Flags | Hop count | Time to Live | Root mesh STA Address | Emergency service Information | HWMP Sequence Number | Interval | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 1 | 4 | 4 | 4 |
| - RANN element |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element. The length is set to 212.

The format of the Flags field is shown in RANN Flags field format.

| B0 | B1 B7 |
| --- | --- |
| Portal Role | Reserved |
| Bits: 1 | 7 |
| - RANN Flags field format |  |

The Flags field is set as follows.

Bit 0: Portal Role (0 = non-portal, 1 = portal).

Bit 1-7: Reserved.

The Hop Count field is coded as an unsigned integer and indicates the number of hops from the originating root mesh STA to the mesh STA transmitting this element.

The Time to Live field is coded as an unsigned integer and indicates the remaining number of times the RANN will be propagated.

The Root mesh STA Address field is represented as a 48-bit MAC address and is set to the Root mesh STA MAC address.

The Emergency service Information field consists of:

B0-B5: Reserved

B6: ESC (Emergency Service Capability) is set to 1 if emergency service is supported, 0 otherwise

B7: UESA (Un-authenticated Emergency Service Accessible) is set to 1 if an unauthenticated emergency service is allowed, 0 otherwise

The HWMP Sequence Number is coded as an unsigned integer and is set to the HWMP sequence number specific to the root mesh STA.

The Interval field is coded as an unsigned integer and is set to the number of TUs between the periodic transmissions of Root Announcements.

The Metric field is set to the cumulative metric from the originating root mesh STA to the mesh STA transmitting the announcement. Detailed usage of the RANN element is described in Error: Reference source not found.

Modify 8.1.6 as shown below (11u 8.01D):

8. Security

8.1.6 Emergency Service establishment in an RSN

An AP that supports RSNAs and supports interworking Emergency Services supports both RSNAs and Emergency Services associations (see 11.3.2.1) simultaneously.

A mesh STA that supports both RSNA and interworking Emergency Services shall support both RSNA and Emergency Services associations simultaneously. In an emergency service operation, a mesh STA with RSNA enabled and the UESA bit set to 1 shall transmit group addressed frames without encryption as individually addressed frames to mesh STAs with Emergency Services association.

Modify 11.23.6 as follows (11u 8.01D):

11.23.6 Interworking Procedures: Emergency Services Support

Emergency Service support provides STAs with the ability to contact authorities, in an emergency situation. The following procedures allow the STA to determine whether emergency services are supported by the AP or mesh STA, and whether un-authenticated emergency service access is allowed.

In an AP or mesh STA, when dot11ESNetwork is set to TRUE, emergency service operation shall be supported. When emergency operation is not supported, dot11ESNetwork shall be set to FALSE.

When the AP or mesh STA is located in a regulatory domain that requires location capabilities, the ESC field shall only be set to 1 if location capability is enabled on the AP or mesh STA. Location capability is enabled when the Civic Location or Geo Location field in the Extended Capabilities Element is set to 1 in a Beacon or probe response frame.

In addition, in an AP the ESC field shall only be set to 1 if both of the following are true (see Annex W.4.2 for further information):

—dot11QosOptionImplemented is true

—dot11EBREnabled is true.

change the 5th paragraph 11C.1.6 as shown (based on 802.11-10-0236R3)

The ESC and the UESA in the Interworking element and tThe Mesh Formation Info field in the Mesh Configuration element isare available to assist mesh STAs in determining which neighbor mesh STAs to establish mesh peerings with. The details of the usage of this information are beyond the scope of this standard.

Add the following section at the end of section 11C.1.6 (based on 802.11-10-0236R3)

11C.1.6.1 Inclusion of Interworking element in the mesh Beacon and Probe Response

The Interworking information element (7.3.2.89) in Beacon and Probe Response indicates whether a candidate peer mesh STA supports an emergency service. If the received Beacon and Probe Response from the candidate peer mesh STA indicates that emergency service is supported, a mesh STA that requires emergency service and has not established peering with the candidate peer mesh STA may initiate mesh peering for emergency service, regardless of the Accepting Additional Mesh Peerings subfield value.

Modify 11C.2.1 as follows:

- MBSS peering management framework

- Overview

The MBSS peering management framework supports all functions to establish, management, and tear down peerings between mesh STAs. When a mesh STA is establishing a mesh peering, it indicates whether the peering is a mesh peering for an emergency service or for a normal mesh peering. In an MBSS Wwhen dot11MeshSecurityActivated is enabled in an MBSS, all mesh STAs shall manage mesh peering and mesh TKSA except when the mesh peering is for an emergency service that requires no authentication.

MBSS peering management functions shall be invoked after a candidate peering mesh STA is discovered via Candidate peer mesh STA discovery procedure in Error: Reference source not found.

One of the following protocols shall be invoked to establish the mesh peering with the candidate peer mesh STA:

- The Mesh Peering Management (MPM) protocol that establishes and manages the mesh peering between candidate peer mesh STAs. When dot11MeshSecurityActivated is not enabled for the MBSS, the mesh STAs shall execute the MPM protocol to manage their mesh peerings with peer mesh STAs. See Error: Reference source not found for MPM protocol details.

- The Authenticated Mesh Peering Exchange (AMPE) protocol that establishes and manages mesh peering and mesh TKSA between candidate peer mesh STAs. When dot11MeshSecurityActivated is enabled for MBSS, the mesh STAs shall execute the AMPE protocol to manage their mesh peerings and the corresponding mesh TKSAs with peer mesh STAs. See Error: Reference source not found for AMPE handshake details.

- Authenticated emergency service (candidate peer mesh STA’s ESC bit set to 1 and UESA bit set to 0) or unauthenticated emergency service (candidate peer mesh STA’s ESC bit set to 1 and UESA bit set to 1) is supported over mesh network. If a mesh STA establishes the authenticated mesh peering for emergency service, the mesh STAs shall set the Emergency service Indicator (EI) in mesh peering management element in mesh peering management frame to 1 and execute the AMPE protocol to manage their mesh peerings and the corresponding mesh TKSAs with peer mesh STAs. If a mesh STA establishes the unauthenticated mesh peering for emergency service, the mesh STAs shall set the Emergency service Indicator (EI) in mesh peering management element in mesh peering management frame to 1 and execute the MPM protocol to manage their mesh peerings with peer mesh STAs regardless the value sets in the dot11MeshSecurityActivated.

Note: explain the implication of this operation. i.e., as emergency service is a sort of urgent requirement to the user, mesh peering can be established without authentication. (see 11C.3.1.1)

change Figure s49 in section 11C.2.1

Figure s49 - Logical flowchart of protocol interaction in Mesh Peering Management framework

Add section 11C.3.1.1 at the end of section 11C.3.1

11C.3.1.1 Unauthenticated mesh peering for emergency service

The unauthenticated mesh peering for emergency service is supported when the candidate peer mesh STA sets the ESC to 1 and the UESA to 1 in the Beacon and Probe Response. If a mesh STA establishes the unauthenticated mesh peering for emergency service, the mesh STAs should set the Emergency service Indicator (EI) to 1 and execute the MPM protocol to manage their mesh peerings with peer mesh STAs.

Modify 8th paragraph of 11C.3.2 as follows:

11C.3.2 Processing Mesh Peering Management frames

11C.3.2.1 Overview

change the 5th paragraph 11C.3.2.1 as shown

The mesh STA shall process the configuration parameters conveyed in the Mesh Peering Open and Mesh Peering Confirm frames in a mesh peering instance. Mesh STA shall verify that the parameters reported by the candidate peer mesh STA in the Mesh Peering Confirm frame match the parameter values the mesh STA has agreed to use for this mesh peering instance. In particular, the mesh STA shall verify the following fields or subfields. This verification is needed to satisfy the consistency property, i.e., to guarantee that mesh STAs agree on the configuration before establishing a mesh peering.

The configuration includes the following elements:

- Mesh ID element

- Fields in Mesh Configuration element

- Active Path Selection Protocol Identifier field

- Active Path Selection Metric Identifier field

- Congestion Control Mode Identifier field

- Synchronization Protocol Identifier field

- Authentication Protocol Identifier field

- Emergency service Information

- Mesh Capability field, including the following subfields

- Accepting Additional Mesh Peerings: in an emergency service the Accepting Additional Mesh Peering is ignored

- MCCA Enabled

- Supported rates element, Extended Supported Rates element, and HT Operation element if present

Mesh STAs shall verify that the same Authentication Protocol is used. If the agreed authentication protocol is SAE, the mesh STA shall verify that Mesh Peering Protocol Identifier in MPM element is set to “Authenticated Mesh Peering Exchange”, AMPE element and MIC element are present in the frame, and RSN element is present if it is a Mesh Peering Open frame or Mesh Peering Confirm frame. If the agreed authentication protocol is not SAE, the AMPE element and MIC element shall be ignored. The RSN element may be processed as may be the requirement by the agreed authentication protocol.

Add this paragraph after 8th paragraph in 11C.3.2.1 as shown

In an authenticated mesh peering for emergency service, mesh STAs shall verify that the same Authentication Protocol is used. If the agreed authentication protocol is SAE, the mesh STA shall verify that Mesh Peering Protocol Identifier in MPM element is set to “Authenticated Mesh Peering Exchange”, AMPE element and MIC element are present in the frame, and RSN element is present if it is a Mesh Peering Open frame or Mesh Peering Confirm frame. If the agreed authentication protocol is not SAE, the AMPE element and MIC element shall be ignored. The RSN element may be processed as may be the requirement by the agreed authentication protocol.

In an unauthenticated mesh peering for emergency service, the MPM protocol is used to manage their mesh peerings with peer mesh STAs regardless the value sets in the dot11MeshSecurityActivated.

change the 9th paragraph 11C.3.2.1 as shown

Mesh STAs shall verify that the candidate peer mesh STA sets the Accepting Additional Mesh Peerings subfield to 1. In an emergency service, the Accepting Additional Mesh Peerings is ignored.

- Mesh Peering Open frame contents

change the 1st  paragraph 11C.3.2.2.1 as shown

The Mesh Peering Open frame shall contain:

- In the Mesh Peering Management element, the Mesh Peering Protocol Identifier shall be set to 00-0F-AC:0 “Mesh Peering Management Protocol”.

- Mesh ID, which shall be set to the Mesh ID supported by the mesh STA’s mesh profile.

- Mesh Configuration

- Active Path Selection Protocol ID field shall be set to the value supported by the mesh STA’s mesh profile

- Active Path Selection Metric ID field shall be set to the value supported by the mesh STA’s mesh profile

- Congestion Control Mode ID shall be set to the value supported by the mesh STA’s mesh profile.

- Synchronization Protocol ID shall be set to the value supported by the mesh STA’s mesh profile.

- Authentication Protocol ID shall be set to the value supported by the mesh STA’s mesh profile.

- Mesh Capability field

- Emergency service Indicator bit in Emergency service Identifier field indicates whether the mesh peering is for an emergency or a non-emergency peering

- Accepting Additional Mesh Peerings field is set to 1. Accepting Additional Mesh Peering is ignored in an emergency service

- MCCA Enabled field is set according to the mesh STA’s configuration

- Mesh Peering Management element shall contain the Local Link ID field. The Local Link ID field shall be set to the identifier generated to identify the current mesh peering instance.

Other elements as defined in 7.4.14.2 (Mesh Peering Open frame details) may be present and set to the value according to mesh STA’s parameter configuration.

Add section 11C.4.1.1 at the end of section 11C.4.1

11C.4.1.1 Authenticated mesh peering for emergency service

The authenticated mesh peering for emergency service is supported when the candidate peer mesh STA sets the ESC to 1 and the UESA to 0 in the Beacon and Probe Response. If a mesh STA establishes the authenticated mesh peering for emergency service, the mesh STAs should set the Emergency service Indicator (EI) to 1 and execute the AMPE protocol to manage their mesh peerings and the corresponding mesh TKSAs with peer mesh STAs.

Add this paragraph at the end of Section 11C.8.2.1

PANN includes the Emergency service Information field to indicate whether the mesh portal supports an emergency service. If an intermediary mesh STA cannot support an emergency service for any reason, it shall change the ESC bit from 1 to 0 before propagating the PANN. The UESA bit indicates whether an emergency service requires or does not require authentication and it is ignored when ESC is 0

Modify table s38 as shown

| - Content of a PANN element in Case A |  |
| --- | --- |
| Field | Value |
| ID | Value given in Element IDsinput.doc for the PANN element |
| Length | 156 |
| Flags | Not used |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for the Portal Announcement |
| Mesh Portal Address | Mesh STA MAC address |
| Emergency service Identifier | Bit 0-5: Reserved Bit 6 (ESC): 1 if emergency service is supported, 0 otherwise Bit 7 (UESA): 1 if unauthenticated emergency service is supported, 0 otherwise. Reserved if ESC is set to 0 |
| PANN Sequence Number | A sequence number specific for the mesh STA collocated with the portal. The PANN sequence number shall be incremented by the mesh STA collocated with the portal before each transmission. |
| Interval | dot11MeshPortalAnnouncementInterval |

Modify table s39 as shown

| - Content of a PANN element in Case B |  |
| --- | --- |
| Field | Value |
| ID | Value given in Element IDsinput.doc for the PANN element |
| Length | 156 |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Mesh Portal Address | As received |
| Emergency service Identifier | As received, but the mesh STA may change the ESC bit from 1 to 0 to indicate that emergency service, for any reason, is not supported through this mesh STA although the mesh portal is able to support an emergency service |
| PANN Sequence Number | As received |
| Interval | As received |

- Proactive RANN mechanism

Add this paragraph shown after the last paragraph

RANN includes the Emergency service Information field to indicate whether the root mesh STA supports an emergency service. If an intermediary mesh STA cannot support an emergency service for any reason, it shall change the ESC bit from 1 to 0 before propagating the RANN. The UESA bit indicates whether an emergency service requires or does not require authentication and it is ignored when ESC is 0

- Conditions for generating and sending a RANN

Modify Table s62 as follows:

| - Contents of a RANN element in Case Ainput.doc |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 (Element IDs) for the RANN element |
| Length | 212 |
| Flags | Bit 0: Portal Role (0 = non-portal, 1 = portal) Bit 1 – 7: Reserved |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for this element |
| Root Mesh STA Address | MAC address of the root mesh STA |
| Emergency service Identifier | Bit 0-5: Reserved Bit 6 (ESC): 1 if emergency service is supported, 0 otherwise Bit 7 (UESA): 1 if unauthenticated emergency service is supported, o otherwise. Reserved if ESC is set to 0 |
| HWMP Sequence Number | Last used HWMP sequence number of the root mesh STA + 1 |
| Interval | dot11MeshHWMPrannInterval |
| Metric | Initial value of active path selection metric |

Modify Table s63 as follows:

| - Contents of a RANN element in Case Binput.doc |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 (Element IDs) for the RANN element |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Root Mesh STA Address | As received |
| Emergency service Identifier | As received, but the mesh STA may change the ESC bit from 1 to 0 to indicate that emergency service, for any reason, is not supported through this mesh STA although the root mesh STA is able to support an emergency service |
| HWMP Sequence Number | As received |
| Interval | dot11MeshHWMPrannInterval |
| Metric | As receivedown link metric toward the transmitting mesh STA |

Modify Table A.4.23.1 as follows:

- General Mesh Support

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| … | … | … | … | ... |
| *MP1.4.1 | Processing of Mesh Peering Open frame | 11C.3.2.2 Mesh Peering Open frames | MP1.4:M | Yes No N/A  |
| *MP1.4.2 | Processing of Mesh Peering Confirm frame | 11C.3.2.3 Mesh Peering Confirm frames | MP1.4:M | Yes No N/A  |
| *MP1.4.3 | Processing of Mesh Peering Close frame | 11C.3.2.4 Mesh Peering Close frames | MP1.4:M | Yes No N/A  |
| MP1.4.4 | MPM Finite State Machine | 11C.3.3 Mesh Peering Management Finite State Machine | MP1.4:M | Yes No N/A  |
| *MP1.4.5 | Processing of Mesh Peering Open frame for emergency service | 11C.3.2.2 Mesh Peering Open frames | MP1.4:M | Yes No N/A  |
| *MP1.4.6 | Processing of Mesh Peering Confirm frame for emergency service | 11C.3.2.3 Mesh Peering Confirm frames | MP1.4:M | Yes No N/A  |
| *MP1.4.7 | Processing of Mesh Peering Close frame for emergency service | 11C.3.2.4 Mesh Peering Close frames | MP1.4:M | Yes No N/A  |
| MP1.5 | Authenticated Mesh Peering Exchange | 11C.4 Authenticated Mesh Peering Exchange | MP1:O | Yes No N/A  |
| … | … | ... | … | … |