IEEE P802.11
Wireless LANs

| Resolving a few PLM Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Networks | 1322 Crossman ave, Sunnyvale, CA | +1 408 227 4500 | dharkins at arubanetworks dot com |
| Kazuyuki Sakoda | Sony Coporation | Kitashinagawa, Shinagawa-ku Tokyo 141-0001 Japan | +81 3 5448 4018 | Kazuyukia.Sakoda(at)jp.sony.com |

Abstract

This submission addresses CIDs 2317, 2318, 2604, 2661, 2664, 2669, 2671, 2673, 2780, 2781 and 2782.

Modify section 7.3.2.114 as indicated:

- Authenticated Mesh Peering Exchange element

The Authenticated Mesh Peering Exchange element includes information needed to perform the authentication sequence during an Authenticated Mesh Peering Exchange. This element is shown in Authenticated Mesh Peering Exchange element.

| Element ID | Length | Selected Pairwise Cipher Suite | Local Nonce | Peer Nonce | Key Replay Counter | GTKdata |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 4 | 32 | 32 | 8 | variable |
| - Authenticated Mesh Peering Exchange element |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this element.

The Length field is variable and indicates the number of octets in the information field (fields following the Element ID and Length fields).

The Selected Pairwise Cipher Suite field contains a pairwise cipher suite selector, as defined in 7.3.2.25.1, indicating a cipher suite to be used to secure the link.

The Local Nonce field contains a nonce value chosen by the mesh STA that is sending the element. It is encoded following the conventions from 7.1.1.

The Peer Nonce field contains a nonce value that was chosen by the peer mesh STA or candidate peer mesh STA to which the element is being sent. It is encoded following the conventions from 7.1.1.

The Key Replay Counter field is optional. It is only used for the Mesh Group Key Inform frame (see 11C.5.3) and the Mesh Group Key Acknowledge frame (see 11C.5.4). It is represented as an unsigned binary number.

The GTKdata field is optional. When present, it contains the bit string of {GTK || Key RSC || GTKExpirationTime} as the GTK data material. When present, the GTKdata field is protected by the exchange in which it is contained (see 11C.4 (Authenticated Mesh Peering Exchange)). The Key RSC denotes the last frame sequence number sent using the GTK and is specified in Table 8-4 of 8.5.2. GTKExpirationTime denotes the key lifetime of the GTK in seconds and the format is specified in Figure 8-30 of 8.5.2.

Modify section 11C.2.1 as indicated :

- General

The Mesh Peering Management framework supports all functions to establish, manage, and tear down peerings between mesh STAs. When dot11MeshSecurityActivated is true, a mesh STA shall manage Mesh peering and Mesh TKSA for each peer mesh STA.

Mesh STAs shall not transmit frames other than the ones used for candidate peer mesh STA discovery, mesh peering management, and SAE to a neighboring mesh STA until a mesh peering has been established with the mesh STA.

MBSS peering management functions shall be invoked after a candidate peering mesh STA is discovered via Candidate peer mesh STA discovery procedure in Error: Reference source not found.

One of the following protocols shall be invoked to establish the mesh peering with the candidate peer mesh STA:

- The Mesh Peering Management (MPM) protocol that establishes and manages the mesh peering between candidate peer mesh STAs. When dot11MeshSecurityActivated is false, the mesh STA shall execute the MPM protocol to manage its mesh peerings with peer mesh STAs. See Error: Reference source not found for MPM protocol details.

- The Authenticated Mesh Peering Exchange (AMPE) protocol that establishes and manages mesh peering and Mesh TKSA between candidate peer mesh STAs. When dot11MeshSecurityActivated is true, the mesh STA shall execute the AMPE protocol to manage its mesh peerings and the corresponding Mesh TKSAs with peer mesh STAs. See Error: Reference source not found for AMPE handshake details.

The Authenticated Mesh Peering Exchange is inclusive of the Mesh Peering Management protocol but differs in that it has additional requirements on creation and processing of frames.

A mesh STA shall use a Mesh Peering Instance Controller (Error: Reference source not found) to manage all mesh peering instances established or in the process of establishment or teardown with its peer mesh STAs and candidate peer mesh STAs. The mesh peering instance is represented by the state machine (see table s35 for MPM state machine and table s36 for AMPE state machine) and all associated information and mesh peering state used to process and manage Peer Link Management Frames.

Error: Reference source not found demonstrates the logical flow of protocol interactions in the peering management framework.

Replace figure s50 with the following:

N

N

Y

Y

N

Y

Y

N

N

Y

Candidate Peer Discovery

Security

Enabled?

Shared PMK

Exists?

AMPE

Succeeds?

SAE Authentication

Succeeds?

MPM

Succeeds?

Mesh Peering

Mesh Peering Security Association

The AMPE protocol requires the existence of a shared Mesh PMK security association (Mesh PMKSA) established between the two candidate mesh peer STAs. If via the discovery procedure, the mesh STA identifies an existing valid Mesh PMKSA it shares with the candidate peer mesh STA, the mesh STA shall initiate AMPE protocol directly to establish mesh peering and Mesh TKSA with the candidate peer mesh STA. If AMPE execution fails, the mesh STA shall use the mesh peering instance controller to handle the failure. The mesh STA shall continue with the candidate peer mesh STA discovery procedure.

If the shared Mesh PMKSA is not identified, the mesh STA shall execute an authentication protocol to mutually authenticate with the candidate peer mesh STA.

Mesh STAs shall support SAE authentication (see 8.2A (Authentication using a pre-shared secret) using a pre-shared secret with the candidate peer mesh STA.

The authentication protocol shall satisfy the following requirements:

- Using authentication credential(s) as required by the authentication protocol to achieve mutual authentication of the two mesh STAs

- Produce mutually shared Mesh PMKSA

- Achieve secrecy of the shared Mesh PMK

Upon successful completion of the authentication protocol, the mesh STA shall initiate AMPE protocol to establish mesh peering and Mesh TKSA with the candidate peer mesh STA using the newly established Mesh PMKSA. Upon failure of authentication, the mesh STA shall terminate the mesh peering establishment procedure with current candidate peer mesh STA, and the mesh STA shall continue with the candidate peer mesh STA discovery procedure.

Note—If a vendor specific authentication protocol is enabled, it must satisfy the same requirements as the mandatory authentication protocol in order to ensure correct interaction with AMPE handshake.

The successful completion of AMPE establishes the mesh peering and Mesh TKSA with the peer mesh STA, and the mesh TK and MGTKs are installed. Then data traffic is allowed to flow on the mesh peering and protection on the data traffic is enabled. If the 802.1X ports are attached to 802.11 SME, the 802.1X uncontrolled port shall be closed to enable data traffic. Upon failure of AMPE, the mesh STA shall terminate the mesh peering establishment procedure with the current candidate peer mesh STA, and the mesh STA shall continue with the candidate peer mesh STA discovery procedure.

The candidate peer mesh STA discovery procedure may receive useful information from execution outcome from SAE, MPM, and AMPE to make candidate peer mesh STA discovery more effective.

- If SAE execution fails, depending on the reason of failure from SAE, the mesh STA may or may not discover the same candidate peer mesh again through the new candidate peer mesh STA procedure.

- If AMPE execution fails and the reason was the failure of mutual authentication using the shared mesh MPKSA, the mesh STA may discover the same candidate peer mesh STA in order to execute SAE authentication to establish a new Mesh PMKSA with the candidate peer mesh STA.

- If AMPE execution fails and the reason was the failure to reach agreement on some mesh peering related parameters, the mesh STA may discover the same candidate peer mesh STA in order to execute AMPE handshake again with a different parameter set for mesh peering negotiation.

- If AMPE execution fails due to retry failure or other internal reasons, the mesh STA may choose not to discover the same candidate peer mesh STA, but try to establish mesh peerings with other new candidate peer mesh STAs.

Details of decision making on what protocol to invoke with the same or different candidate peer mesh STA upon failure of SAE, MPM, or AMPE execution are beyond the scope of this specification.

After successful establishment of mesh peering and Mesh TKSA, the mesh STA may initiate Mesh Group Key Handshake (see Error: Reference source not found) to update its MGTK to all of its peer mesh STAs.

Move section 11C.2.2.4 to 11C3.2 and modify as indicated:

11C3.2 Pre-processing Mesh Peering Management frames

Each mesh peering instance, for both MPM FSMs and AMPE FSMs, shall be identified by a set of information called the mesh peering instance identifier, which includes the localLinkID, peerLinkID, localMAC, and peerMAC. In addition, the AMPE FSMs are further identified by the PMKName from the Mesh Peering Management element.

localMAC is the MAC address of the mesh STA that is being used with this mesh peering instance. peerMAC is the MAC address of the peer mesh STA or the candidate peer mesh STA. localLinkID is an integer generated by the mesh STA. peerLinkID is an integer generated by the peer mesh STA or the candidate peer mesh STA.The localLinkID shall be unique among all existing link identifiers used by the mesh STA for its current mesh peering management finite state machines. The mesh STA selects the localLinkID to provide high assurance that the same number has not been used to identify a recent mesh peering management finite state machine. The peerLinkID shall be supplied by the peer mesh STA or candidate peer mesh STA in Mesh Peering Open and Confirm frames. The mesh peering management finite state machine identifiers are transmitted via Mesh Peering Management frames.

The mesh peering instance controller on a mesh STA shall pre-process incoming Mesh Peering Management frames and shall either discard the frame or pass it to the corresponding active mesh peering instance finite state machine for further processing.

If the Mesh Peering Protocol Identifier field in the Mesh Peering Management element indicates “Mesh Peering Management Protocol”, the Authenticated Mesh Peering element and MIC element, if present in the frame, shall be ignored.

If the Mesh Peering Protocol Identifier field in the Mesh Peering Management element indicates “Authenticated Mesh Peering Exchange” and the Authenticated Mesh Peering Exchange element or MIC element is not included in the frame, the frame shall be silently discarded..

If the frame contains a group address in TA or RA, it shall be silently discarded.

The mesh peering instance controller then locates an active FSM, either an MPM FSM if the Mesh Peering Protocol Identifier indicates “Mesh Peering Management Protocol” or an AMPE FSM if the Mesh Peering Protocol indicates “Authenticated Mesh Peering Exchange.” A match is determined by checking the contents of the Mesh Peering Management Element with each peering instance. A match shall be found if the following conditions hold:

- If t

- he sender’s MAC address is the same as the peerMAC of the mesh peering instance, and;

- The receiver’s MAC address is the same as the localMAC of the mesh peering instance, and;

- The value of Local Link ID field is the same as the peerLinkID of the mesh peering instance, and;

- Either the Peer Link ID field does not exist, or the Peer Link ID field is the same as the localLinkID of the mesh peering instance.

If no match is found but there exists a mesh peering instance for which the receiver’s MAC address is the same as the localMAC and the Peer Link ID is the same as the localLinkID, the mesh peering instance shall be deemed a match and the Local Link ID field shall be copied to the peerLinkID of the mesh peering instance.

If the incoming Mesh Peering Management frame is for AMPE the received frame must be further processed as follows:.

- If the chosen PMK from the received frame is different than the PMKName that identifies the valid Mesh PMKSA established with the candidate peer mesh STA the frame shall be dropped.

- If the peerNonce in the mesh peering instance exists and is different than the Local Nonce field of the received frame, the frame shall be dropped.

- If the peerNonce in the mesh peering instance does not exist, the Local Nonce field in the received frame shall be copied to the peerNonce field.

- If the Peer Nonce field in the received frame exists and is different than the localNonce field the frame shall be dropped.

The mesh peering instance controller shall treat the mismatched, or dropped, incoming Mesh Peering Open frame as a request to establish a new mesh peering, or a new mesh peering and a Mesh TKSA if the frame is for AMPE. A mismatched or dropped incoming Mesh Peering Confirm frame or Mesh Peering Close frame shall be silently discarded.

When the mesh STA has established a Mesh PMKSA with the candidate peer mesh STA, the mesh peering instance controller shall silently discard the Mesh Peering Open frame in the following two conditions:

- The Mesh Peering Open frame supports Mesh Peering Management protocol and the active authentication is SAE, or

- The Mesh Peering Open frame supports AMPE but the mesh STA does not support the Mesh PMKSA as identified by PMKName in the Chosen PMK field in Authenticated Mesh Peering Exchange element.

If the Mesh Peering Open frame is not discarded the mesh peering instance controller may generate a new protocol finite state machine and actively reject or accept the mesh peering open request. A unique local link ID shall be generated for the mesh peering instance. If the mesh peering instance is to be established by Authenticated Mesh Peering Exchange, a random local nonce shall be generated for identifying the mesh peering instance as well.

The mesh peering open request may be rejected due to an internal reason. If the mesh peering open request is rejected, the REQ_RJCT event shall be passed to the newly generated protocol finite state machine to actively reject the mesh peering open request.

NOTE— Example internal reasons to reject new mesh peering request could be the mesh STA has reached its capacity to set up more mesh peering, the mesh STA is configured to reject mesh peering request from another specific peer mesh STA.

When the mesh peering open request is accepted, the received Mesh Peering Open frame shall be passed to the newly generated mesh peering instance finite state machine to trigger further actions.

Modify 11C3.1 as indicated

- General

The Mesh Peering Management protocol is used to establish, maintain, and close mesh peerings between mesh STAs when security is not required.

A mesh STA shall assign a unique AID to every peer mesh STA during the mesh peering establishment procedure. AID is used to encode TIM element in the Beacon frame (see 7.3.2.6 (TIM element)). AID 0 (zero) is reserved to indicate the presence of buffered groupcast MSDUs and MMPDUs.

The mesh STA shall start the mesh peering management protocol in either of the following two cases. In the first case, the mesh STA passively listen to incoming Mesh Peering Open requests from a candidate peer mesh STA. In the second case, the mesh STA actively creates a mesh peering instance to establish a mesh peering with a candidate peer mesh STA.

A mesh peering instance ends when the mesh peering is closed. The mesh peering close can be caused by the mesh STA due to events outside the scope of this specification. The response to these events is outside the scope of this standard.

The MPM protocol uses Mesh Peering Open frames, Mesh Peering Confirm frames, and Mesh Peering Close frames to establish, manage, and tear down a mesh peering.

A Mesh Peering Open frame requests that a mesh peering instance be established between the Mesh Peering Open sender and the receiver. Mesh Peering Open frames propose mesh parameters for the mesh peering instance. The mesh STA processes the received parameters. If it agrees with the parameters, the mesh STA sends a Mesh Peering Confirm frame in response to the Mesh Peering Open frame. If the mesh STA disagrees on the parameters or there is a failure due to other reasons, the mesh STA closes the mesh peering by sending a Mesh Peering Close frame.

The protocol succeeds in establishing a mesh peering when the following requirements are satisfied: 1) both mesh STAs have sent and received (and correctly processed) a Mesh Peering Open frame regarding this mesh peering; 2) both mesh STAs have sent and received (and correctly processed) a corresponding Mesh Peering Confirm frame regarding this mesh peering.

Modify existing section 11C3.3(Mesh Peering Management Finite State Machine) to 11C3.4 and 11C3.2 (Processing Mesh Peering Management frames) to 11C3.3

11C3.3 Processing Mesh Peering Management frames

Modify new section 11C.3.3.1 as indicated

11C3.3.1 Overview

Mesh Peering Management frames are Self Protected Action frames. When Mesh Peering Management frames are used for MPM, protection on these frames is not enabled (see 7.4.14 (Self Protected Action frame details)).

The construction of Mesh Peering Management frames is specified in Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found.

Mesh Peering Management frames accepted by the Mesh Peering Instance Controller shall be passed to the corresponding mesh peering management state machine for further processing as defined in this subclause.

Modify new section 11C3.4 as indicated

11C3.4 Mesh Peering Management Finite State Machine (MPM FSM)

11C3.4.1 General

Each mesh peering instance, including its states and resources, are managed by a mesh peering management finite state machine (MPM FSM). The MPM FSM uses MLME primitives to control the mesh STA to send and receive Mesh Peering Management frames.

Modify new section 11C3.4.3 as indicated

11C3.4.3 Events and Actions

The finite state machine uses three types of events: events for state machine controlling external events generated by frame processing, and events associated with internal timers.

The state machine controlling events are:

- CNCL(localLinkID, peerMAC, ReasonCode) — Used to instruct the mesh peering instance to cancel the mesh peering with the peer mesh STA. localLinkID identifies the MPM FSM for the corresponding mesh peering instance. peerMAC is the MAC address of the peer mesh entity. ReasonCode is used to inform the reason to cancel the mesh peering instance.

- ACTOPN(peerMAC, localLinkID) — The SME uses this event to create a new mesh peering instance to actively initiate the mesh peering establishment with the candidate peer mesh STA whose MAC address is peerMAC. localLinkID identifies the MPM FSM.

The events generated by frame processing are

- OPN_ACPT — PeeringOpen_Accept(peerMAC, peerLinkID) event indicates that a Mesh Peering Open frame meeting the correctness criteria of Error: Reference source not found has been received from peerMAC for the mesh peering instance identified by peerLinkID.

- OPN_RJCT — PeeringOpen_Reject(peerMAC, peerLinkID, Configuration, ReasonCode) event indicates that a Mesh Peering Open frame from peerMAC for the mesh peering instance identified by peerLinkID is rejected due to incomplete or erroneous configuration or other internal reasons. The ReasonCode can be set to MESH-CONFIGURATION-POLICY-VIOLATION.

- CNF_ACPT — PeeringConfirm_Accept(peerMAC, localLinkID, peerLinkID) event indicates that a Mesh Peering Confirm frame meeting the correctness criteria of Error: Reference source not found has been received from peerMAC for the mesh peering instance identified by localLinkID and peerLinkID.

- CNF_RJCT — PeeringConfirm_Reject(peerMAC, localLinkID, peerLinkID, ReasonCode) event indicates that a Mesh Peering Confirm frame from peerMAC for the mesh peering instance identified by localLinkID and peerLinkID is rejected due to incomplete or erroneous configuration. The ReasonCode is set to MESH-CONFIGURATION-POLICY-VIOLATION.

- CLS_ACPT — PeeringClose_Accept(peerMAC, localLinkID, peerLinkID, reasonCode) event indicates that a Mesh Peering Close frame meeting the correctness criteria of Error: Reference source not found has been received from peerMAC for the mesh peering instance identified by localLinkID and peerLinkID. The reasonCode specifies the reason that caused the generation of the Mesh Peering Close frame.

- REQ_RJCT – PeeringRequest_Reject(peerMAC, peerLinkID, reasonCode) event indicates a special incidence that the mesh STA rejects the incoming Mesh Peering Open frame requesting to set up a new mesh peering due to internal reason. The incoming request is identified by the peerMAC and peerLinkID as received from the Mesh Peering Open frame. The reasonCode is set to PEER_LINK_CLOSED.

Three types of timers are used by the finite state machine.

The retryTimer triggers a re-send of the Mesh Peering Open frame when a Mesh Peering Confirm frame was not received as a response.

The confirmTimer signals that a link establishment attempt should be aborted because a Mesh Peering Confirm frame responding to a Mesh Peering Open frame was never received. The holdingTimer signals that its mesh peering instance may be completely closed and facilitates graceful shutdown.

The internal events are as follows. The term Timeout(localLinkID, item) represents a timeout identified locally by item, for the mesh peering instance identified by localLinkID.

- TOR1— This event refers to Timeout(localLinkID, retryTimer) and the dot11MeshMaxRetries has not been reached. The Mesh Peering Open frame shall be resent.

- TOR2—This event refers to Timeout(localLinkID, retryTimer) and the dot11MeshMaxRetries has been reached. The mesh peering instance shall be closed when TOR2 occurs.

- TOC— The Timeout(localLinkID, confirmTimer) event. The confirmTimer aborts a mesh peering establishment attempt if a Mesh Peering Open frame never arrives after receiving the Mesh Peering Confirm frame. TOC event occurs, the mesh peering instance shall be closed.

- TOH— The Timeout(localLinkID, holdingTimer) event. The holdingTimer allows a grace period for closing the mesh peering instance; it is necessary to avoid deadlocks and livelocks that arise due to interactions between mesh peering establishment and termination. When TOH occurs, the mesh peering instance shall be closed completely and the finite state machine shall transition to IDLE state.

The finite state machine may take an action triggered by an event. It uses two types of actions: sending a Mesh Peering Management frame and handling a timer.

Actions related to sending a Mesh Peering Management frame:

- sndOPN — The sendOpen(peerMAC, localLinkID, Configuration) is the action that the mesh STA takes to send a Mesh Peering Open frame to the candidate peer mesh STA, whose MAC address is peerMAC. The frame shall carry localLinkID and the supported Mesh Configuration, as specified in Configuration.

- The MLME-MeshPeeringManagement.request primitive shall be invoked to send the frame to the peer mesh entity.

- sndCNF — The sendConfirm(peerMAC, localLinkID, peerLinkID, Configuration) is the action that the mesh STA takes to send a Mesh Peering Confirm frame to the candidate peer mesh STA, whose MAC address is peerMAC. The frame shall carry localLinkID, peerLinkID, and the supported Mesh Configuration, as specified in Configuration.

- The MLME-MeshPeeringManagement.request primitive shall be invoked to send the frame to the peer mesh entity.

- sndCLS — The sendClose(peerMAC, localLinkID, peerLinkID, reasonCode) is the action that the mesh STA takes to send a Mesh Peering Close frame to the peer mesh STA or candidate peer mesh STA, whose MAC address is peerMAC. The MLME-MeshPeeringManagement.request primitive shall be invoked to send the frame to the peer mesh entity.

The actions on handling timers are setTimer(localLinkID, item, value) and clearTimer(localLinkID, item).

- The setTimer(localLinkID, item, timeout) action sets the timeout value specified by timeout to the timer specified by item. This action only sets the timer for one time for the mesh peering instance identified by localLinkID. When the timeout time has passed, the timer expires and the event Timeout(localLinkID, item) is triggered, after which the timer is no longer in effect.

The corresponding actions are denoted as setR, setC, setH, for timer retryTimer, confirmTimer, holdingTimer respectively.

Before setting the retryTimer, the finite state machine shall apply the following backoff algorithm to compute the updated timeout value:

timeout = return timeout + (getRandom mod timeout),

where getRandom routine generates a random value. The initial value of timeout shall be set to dot11MeshRetryTimeout. This function statistically increases the length of time for each Mesh Peering Open retry by 50%. The backoff was inserted into the design to recover from a “gold rush”, which could happen if several already-linked mesh STAs simultaneously detected a new mesh STA trying to enter the MBSS.

Modify new 11C.3.4.5 as indicated

11C3.4.5 IDLE state

IDLE is a quiescent state the finite state machine enters prior to establishing a new mesh peering.

Modify section 11C.4.5.1 as indicated

- General

The Authenticated Mesh Peering Exchange is inclusive of the Mesh Peering Management protocol. AMPE frames have additional processing and construction requirements on top of those for MPE frames.

The Mesh Peering Management frames shall be generated with additional information using RSN element and AMPE element to support Authenticated Mesh Peering Exchange. Upon receiving a Mesh Peering Management frame when Authenticated Mesh Peering Exchange is enabled, the mesh STA shall perform Authenticated Mesh Peering Exchange specific processing operations followed by the basic Mesh Peering Management protocol frame processing as specified in Error: Reference source not found

References: