IEEE P802.11
Wireless LANs

| TGp D10.0 SB01 Comment Resolutions for CIDS 2004 and 2018 to 2020 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Carl Kain | Noblis (US DOT) | 3160 Fairview Park Drive, South Falls Church, VA 22042 | 703-610-1788 | ckain@noblis.org |
| Jeremy Landt | Transcore | 8600 Jefferson St. NE, Albuquerque, NM 87113 | 505-856-8015 | jerry.landt@transcore.com |
| Alastair Malarky | Mark IV Industries | 6030 Ambler Drive, Mississaugua, Ontario, Canada L4W 2P1 | 905-624-3020 | amalarky@ivhs.com |
| Justin McNew | Kapsch TrafficCom ¦ TechnoCom Mobility Solutions | 2035 Corte del Nogal, Suite 105, Carlsbad, CA 92011 | 760-438-5115 x 175 | justin.mcnew@kapsch.net |
| George Vlantis | STMicroelectronics | 2525 Augustine Drive, Santa Clara, CA 95054 | 408-451-8109 (w) 408-893-9357 (c) | George.Vlantis@st.com |

Abstract

This submission addresses four comments that were submitted during the 1st recirculation Sponsor Ballot (SB01) of the 802.11p amendment Draft 10.0(1).

Comment resolutions to CIDs 2004, 2018, 2019, and 2020, plus recommended changes relative to Draft 10.0 (if applicable) are provided for consideration by the Comment Resolution Committee.

|  | “Sponsor Ballot 01” Comment Resolution |
| --- | --- |

- Comment:  [From 11-10-0239r0-sb1-tgp-comment-resolutions-master.xls(2) ]

| C ID | Name | Cat | Pg | Subclause | Line | Comment | Must Sat | Proposed Change | Recommended Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2004 | Ecclesine, Peter | Technical | 20 | 18.3.10.2 | 14 | Both in 17.3.10.2 and 17.3.10.3 should qualify the use of dot11ACRType with dot11OCBEnabled true, to stay in scope of the 11p PAR. | Yes | Qualify the optional enhanced optional ACR and non-adjacent channel rejection with dot11OEBEnabled true two places in clause 17. | Agree in Principle. The optional enhanced performance specifications for ACR and non-adjacent channel rejection are device performance characteristics based on hardware implementation and not a behavior. The dot11ACRType element in the MIB should be a read-only status attribute written by the PHY management entity and remain independent of the value of operating band or dot11OCBEnabled’s state. The read-only attribute will equal 2 if the PHY satisfies the optional specification. Additionally, the MIB has some editorial issues. The PICS OF5.2.1 and OF5.3.1 are correctly consistent and not constrained by CF2.1 or CF17. Therefore: (1) In Annex D, within the definition of the “Dot11PHYOFDMEntry” sequence, on page 27, line 31, change the underlined “dot11StationClass” to the underlined “dot11STATransmitPowerClass”. (2) In Annex D, within the definition of the “dot11StationClass” element, on page 27, line 37, change the name of the element from “dot11StationClass” to “dot11STATransmitPowerClass”. (3) In Annex D, within the “dot11StationClass” element, on page 27, line 38, change the SYNTAX statement from “SYNTAX INTEGER (1..4)” to “SYNTAX INTEGER { Class A(1), Class B(2), Class C(3), Class D(4) }”. (4) In Annex D, within the “dot11ACRType” element, on page 27, line 47, change the SYNTAX statement from “SYNTAX INTEGER (1..2)” to “SYNTAX INTEGER { Table 17-13(1), Table 17-13a(2) }”. (5) In Annex D, within the definition of the dot11ACRType MIB element, on page 27, line 48: Change the value of MAX-ACCESS from “read-write” to “read-only”. (6) In Annex D, within the definition of the “dot11ACRType” element, on page 27, line 51: Within the quotes following “DESCRIPTION”, insert a new paragraph where the first line is “This is a status variable.” and the second line is “It is written by the PHY.” (7) In Annex D, within the definition of the “dot11ACRType” element, on page 27, line 53: After the last line of the “DESCRIPTION” quotation, append the line “DEFVAL { 1 }”. (See Clause 2 of Doc. #11-10/0285r0 for an illustration of the consequences of these changes.) |
| 2018 | Worstell, Harry | General | 18 | 11.20 | 40 | With the introduction of dot11OCBEnabled, there is a huge coexistance problem with legacy devices in the bands commonly used for infrastructure networks. If an 11p-capable device operating in the 2.4 or 5 GHz band were to have dot11OCBEnabled set true, and operate outside the context of the existing infrastructure BSSs within range, it would have serious performance implications on the existing BSSs. | Yes | Either specify the coexistance procedures that shall be followed before a STA uses the procedures in 11.20, or disallow it. The preferred solution is to insert a paragraph in J.2 (before the J.2.1 subheading) "Unless otherwise stated, dot11OCBEnabled shall be set false." | Agree in Principal. The default value for the MIB variable dot11OCBEnabled is stated in the DESCRIPTION of the MIB element in Annex D, page 27, line 15. However, there is a deficiency that the statement “DEFVAL { false }” which assigns the default value is omitted. (See DEFVAL examples in 802.11ma-2007 and in 802.11mb D2.01.) Therefore: In Annex D, page 27, line 16, append a new line after the DESCRIPTION, the line: “DEFVAL { false }’ |
| 2019 | Worstell, Harry | Technical | 33 | J.2 | 9 | For the 5.9 GHz band the standard states that dot11OCBEnabled shall be set true, but it is unspecified for other bands. | Yes | Insert a paragraph in J.2 (before the J.2.1 subheading) "Unless otherwise stated, dot11OCBEnabled shall be set false." | Agree in Principal. The default value for the MIB variable dot11OCBEnabled is stated in the DESCRIPTION of the MIB element in Annex D, page 27, line 15. However, there is a deficiency that the statement “DEFVAL { false }” which assigns the default value is omitted. (See DEFVAL examples in 802.11ma-2007 and in 802.11mb D2.01.) Therefore: In Annex D, page 27, line 16, append a new line after the DESCRIPTION, the line: “DEFVAL { false }’ |
| 2020 | Worstell, Harry | Technical | 19 | 11.20 | 8 | "When operating in a band for which dot11OCBEnabled is permitted to be false" but there is no normative text that states whether that MIB variable is permitted to be false. | Yes | delete the cited phrase, leaving "A STA for which dot11OCBEnabled is true shall use information from the CF Parameter Set element of all received Beacon frames" | Agree The behavior of a STA receiving a beacon frame containing the CF Parameter Set element must be specified unambiguously and normatively, regardless of the operating band and the state of dot11OCBEnabled. The text should read: “A STA shall use information from the CF Parameter Set element of all received Beacon frames, without regard for the BSSID, to update its NAV as specified in 9.3.2.2.” Therefore: Change the first sentence of the last paragraph of subclause 11.20, on page 19, line 8 as follows: (1) Delete the clause “When operating in a band for which dot11OCBEnabled is permitted to be false,” (2) Capitalize the word “A” before the word “STA”. (3) Delete the clause “for which dot11OCBEnabled is true”. |

- Illustration of the consequences of the editorial instructions for CID 2004 when applied to Annex D, page 27 of TGp D10.0:

The consequence of step (1) of the “Recommended Resolution” for CID 2004 is shown as it affects “dot11PhyOFDMEntry”:

Dot11PhyOFDMEntry ::=

SEQUENCE { dot11CurrentFrequency               INTEGER,

dot11TIThreshold                    Integer32,

dot11FrequencyBandsSupported        INTEGER,

dot11ChannelStartingFactor          Integer32,

dot11FiveMHzOperationImplemented    TruthValue,

dot11TenMHzOperationImplemented     TruthValue,

dot11TwentyMHzOperationImplemented  TruthValue,

dot11PhyOFDMChannelWidth            INTEGER,

dot11STATransmitPowerClass          INTEGER,

dot11ACRType                        INTEGER }

The consequence of steps (2) and (3) of the “Recommended Resolution” for CID 2004 are shown as they affect “dot11StationClass”:

dot11STATransmitPowerClass OBJECT-TYPE

SYNTAX INTEGER { Class A(1), Class B(2), Class C(3), Class D(4) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“The station transmit power class: Class A=1, Class B=2,

Class C=3, Class D=4.”

::=  { dot11PhyOFDMEntry 5 }

The consequence of steps (4) through (7) of the “Recommended Resolution” for CID 2004 are shown as they affect “dot11ACRType”:

dot11ACRType OBJECT-TYPE

SYNTAX INTEGER { Table 17-13(1), Table 17-13a(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a status variable.

It is written by the PHY.

The Adjacent and Nonadjacent Channel Rejection performance:

When this attribute = 1 the levels in Table 17-13 apply;

When this attribute = 2 the levels in Table 17-13a apply.”

DEFVAL { 1 }

::=  { dot11PhyOFDMEntry 6 }

- Recommended Resolution(s) of the Comments

See the right column of the table above for the resolutions of the individual comments.

- Motion:

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the recommended changes to P802.11p D10.0 noted above and instruct the editor to make these changes to the latest draft of P802.11p.

- Motion by: ____________________Date: __________

- Second:  _____________________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

References:

(1)  IEEE 802.11 Working Group of the IEEE 802 Committee, “IEEE P802.11pTM/D10.0, Draft Standard for Information Technology — Telecommunications and information exchange between systems — Local and metropolitan area networks — Specific requirements, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, Amendment 7: Wireless Access in Vehicular Environments”.

(2)  Lee Armstrong, IEEE Doc. #802.11-10-0239/r0, “SB1 TGp Comment Resolutions Master” spreadsheet.