IEEE P802.11
Wireless LANs

| TGae Areas of Technical Proposals |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Santosh Pandey | Cisco | 170 West Tasman Drive, San Jose, CA | +1 408 853 5990 | sanpande@cisco.com |

### Overview

This document identifies areas where the TGae group need to consider developing technical proposals. The goals are as follows:

- Agree on areas we need technical proposals and their rough content

- Agree on order in which we develop those proposals (if sequencing is required)

# Areas of Technical Proposals

|  | Area | Details | Requirements # |
| --- | --- | --- | --- |
| 1 | Frame priority mechanism | - Define frame prioritation - Group/Individual prioritation - Capabiltiy to change priority with time/session - Depend on network? BSS, IBSS, not-BSS(11p), Mesh or home/ enterprise - How does HCCA/EDCA affect the TGae solution? - Consider management frame inside data frame? - How does TGae address Vendor-specific Action frames, Public Action, or Protected Dual Action frames? - For request/response management frames, should the response be set to the same priority as the request? - Need to consider how to prioritize frames associated with Emergency Services. | 4, 5, 7, 8, 10, 11, 13, 15, 16, 19, 20, 25, 26, Issues |
| 2 | Frame priority management | - Define policy with frame prioritization - Can management frames be aggregated? - Should TGae provide a means to aggregate prioritized management frames? - Should be easy to exchange during capability exchange - Does it define change in priority of frames based on STA state/activity, e.g. power save, voice call? Change with time/session - Priority range? Allowed lowest-highest priority - Capability to extend with future 802.11 changes - Should be easy to exchange between STAs - The priority policy should be managed on a task basis. - Some management frames should remain at a fixed priority (i.e. cannot be addressed by a policy) - How are IEEE 802.11n management frames addressed for fixed priority? - Should the management prioritization policy be secured for both data confidentiality and integrity. | Issues |
| 3 | 11ae capability exchange | - Advertise 11ae capability - Individual/Group frame prioritization exchange or policy enchange - If a group is always going to be a set priority, should that priority be exchanged - Define setting priorities post-association/pre-association for all 11ae capable clients - Define change in priorities post-association/pre-association - Security of policy/capability exchange - Does the priority information need to be communicated in the header of the TGae frame. (with 11w, each frame requires a sequence number for CCMP to do replay detection). - Should the management prioritization policy be communicated in a unicast or group-addressed manner. - Should the management prioritization policy be secured for both data confidentiality and integrity. - A non-AP STA should not be able to request a management frame prioritization policy. - How policy is determined when the STA transitions from one BSS to another BSS. How policy is determined when a STA joins a new BSS. - What happens if there is a gap between when the STA associates and when the STA determines the policy? If there is a gap, can the AP query the STA for the management frame prioritization policy? - Should a TGae-capable STA apply a new management frame policy when it has been received? | 1, 2, 3, 6, 9, 10, 12, 14, 15, 17, 18, 20, 21, 22, 23, 24 |
| 4 | Enforcing policy | - How to check to see if the STA is observing 11ae prioritization ? - Any way to preventing misbehaving STA from disrupting network - Should the policy enforcing depend on the frames and STA state, e.g. power save, voice call - What does the AP or non-AP STA do when it receives a frame that does not meet the management frame prioritization policy? | 17, 19, 20, 21, 22, 24 |

The requirements are mapped from doc.: IEEE 802.11-10/0188r0