IEEE P802.11
Wireless LANs

| TDLS Discovery Clause 10 Primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-02-99 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG |  |  |  |
| Menzo Wentink | Qualcomm | Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

### Overview

This submission proposes Clause 10 primitives for TDLS discovery.

Insert the following subclauses to Clause 10, and renumber accordingly.

### 10.3.44 TDLS Discovery

The following MLME primitives support the signaling of TDLS Discovery.

#### 10.3.44.1 MLME-TDLSDISCOVERY.request

##### 10.3.44.1.1 Function

This primitive requests that a TDLS Discovery Request frame be sent through the AP path.

##### 10.3.44.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSDISCOVERY.request(

DestinationAddress,

TDLSDiscoveryRequest,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DestinationAddress | MAC Address | Any valid MAC Address | Specifies the DA to which a TDLS Discovery Request frame must be transmitted. |
| TDLSDiscoveryRequest | Sequence of octets | As defined in TDLS Discovery Request frame | Specifies the proposed service parameters for the TDLS Discovery Request frame. |

##### 10.3.44.1.3 When Generated

This primitive is generated by the SME to request that a TDLS Discovery Request frame be sent through the AP.

##### 10.3.44.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TDLS Discovery Request frame. The STA then attempts to transmit this frame.

#### 10.3.44.2 MLME-TDLSDISCOVERY.confirm

##### 10.3.44.2.1 Function

This primitive is generated when a valid TDLS Discovery Response frame is received.

##### 10.3.44.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSDISCOVERY.confirm(

TDLSPeerSTAAddress,

TDLSDiscoveryResponse

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSPeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the MAC address of the TDLS peer STA from which a TDLS Discovery Response frame was received. |
| TDLSDiscoveryResponse | Sequence of octets | As defined in TDLS Discovery Response frame | Specifies the service parameters contained in the received TDLS Discovery Response frame. |

##### 10.3.44.2.3 When Generated

This primitive is generated when a valid TDLS Discovery Response frame is received.

##### 10.3.44.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the MLME-TDLSDISCOVERY.confirm primitive and may use the reported data.

#### 10.3.44.3 MLME-TDLSDISCOVERY.indication

##### 10.3.44.3.1 Function

This primitive indicates that a TDLS Discovery Request frame was received from a TDLS peer STA.

##### 10.3.44.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSDISCOVERY.indication(

TDLSPeerSTAAddress,

TDLSDiscoveryRequest

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSPeerSTAAddress | MACAddress | Any valid individual MAC Address | Specifies the MAC address of the TDLS peer STA from which a TDLS Discovery Request frame was received. |
| TDLSDiscoveryRequest | Sequence of octets | As defined in TDLS Discovery Request frame | Specifies the proposed service parameters of the TDLS Discovery Request frame. |

##### 10.3.44.3.3 When Generated

This primitive is generated by the MLME when a valid TDLS Discovery Request frame is received.

##### 10.3.44.3.4 Effect of Receipt

On receipt of this primitive, the SME should operate according to the procedure in 11.19.

#### 10.3.44.4 MLME-TDLSDISCOVERY.response

##### 10.3.44.4.1 Function

This primitive requests that a TDLS Discovery Response frame be sent directly to the TDLS peer STA from which a TDLS Discovery Request frame was received.

##### 10.3.44.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSDISCOVERY.response(

TDLSPeerSTAAddress,

TDLSDiscoveryResponse

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSPeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the MAC address of the TDLS peer STA to which a TDLS Discovery Response frame must be transmitted. |
| TDLSDiscoveryResponse | Sequence of octets | As defined in TDLS Discovery Response frame | Specifies the proposed service parameters for the TDLS Discovery Response frame. |

##### 10.3.44.4.3 When Generated

This primitive is generated by the SME to request that a TDLS Discovery Response frame be sent to the TDLS peer STA STA from which a TDLS Discovery Request frame was received.

##### 10.3.44.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TDLS Discovery Response frame. The STA then attempts to transmit this frame to the TDLS peer STA.