IEEE P802.11
Wireless LANs

| TDLS Discovery Clause 10 Primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-02-99 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG |  |  |  |
| Menzo Wentink | Qualcomm | Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

### Overview

This submission proposes Clause 10 primitives for TDLS discovery, adds the corresponding PICS item and cleans up one information element from the TDLS Discovery Request and Response.

In Clause 7.4.11.11, delete element 16 (TDLS Discovery Element).

In Clause 7.4.11.12, delete element 16 (TDLS Discovery Element).

Add the following PICS item to A.4.21.

### A.4.21 Tunneled Direct Link Setup extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TDLS1 | Tunneled Direct Link Setup | 7.4.11, 11.19 | CF2&CF17:M | Yes □ No □ N/A □ |
| TDLS1.1 | TDLS Setup | 7.3.2.61, 7.4.11.1, 7.4.11.2, 7.4.11.3, 11.19.4 | CF2&CF17:M | Yes □ No □ N/A □ |
| TDLS1.2 | TDLS Teardown | 7.3.2.61, 7.4.11.4, 11.19.5 | CF2&CF17:M | Yes □ No □ N/A □ |
| TDLS1.3 | TDLS Peer Key Handshake | 8.5.9 | CF2&CF17:M | Yes □ No □ N/A □ |
| TDLS1.4 | TDLS Peer PSM | 7.3.2.61, 7.3.2.62, 7.4.11.8, 7.4.11.9 11.2.1.13 | CF2&CF17:O | Yes □ No □ N/A □ |
| TDLS1.5 | TDLS Peer U-APSD | 7.3.2.61, 7.3.2.64, 7.3.2.65, 7.4.11.5, 7.4.11.12 11.2.1.14 | CF2&CF17:O | Yes □ No □ N/A □ |
| TDLS1.6 | TDLS Channel Switching | 7.3.2.61, 7.3.2.63, 7.4.11.6, 7.4.11.7, 11.19.6 | CF2&CF8& CF11&CF17:O | Yes □ No □ N/A □ |
| TDLS1.7 | TDLS Discovery | 7.3.2.61, 7.4.11.11, 7.4.11.12, 11.19.3 | CF2&CF8& CF11&CF17:O | Yes □ No □ N/A □ |

Insert the following subclauses to Clause 10, and renumber accordingly.

### 10.3.44 TDLS Discovery

The following MLME primitives support the signaling of TDLS Discovery.

#### 10.3.44.1 MLME-TDLSDISCOVERY.request

##### 10.3.44.1.1 Function

This primitive requests that a TDLS Discovery Request frame be sent through the AP path.

##### 10.3.44.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSDISCOVERY.request(

DestinationAddress,

TDLSDiscoveryRequest,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DestinationAddress | MAC Address | Any valid MAC Address | Specifies the DA to which a TDLS Discovery Request frame must be transmitted. |
| TDLSDiscoveryRequest | Sequence of octets | As defined in TDLS Discovery Request frame | Specifies the proposed service parameters for the TDLS Discovery Request frame. |

##### 10.3.44.1.3 When Generated

This primitive is generated by the SME to request that a TDLS Discovery Request frame be sent through the AP.

##### 10.3.44.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TDLS Discovery Request frame. The STA then attempts to transmit this frame.

#### 10.3.44.2 MLME-TDLSDISCOVERY.confirm

##### 10.3.44.2.1 Function

This primitive is generated when a valid TDLS Discovery Response frame is received.

##### 10.3.44.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSDISCOVERY.confirm(

TDLSPeerSTAAddress,

TDLSDiscoveryResponse

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSPeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the MAC address of the TDLS peer STA from which a TDLS Discovery Response frame was received. |
| TDLSDiscoveryResponse | Sequence of octets | As defined in TDLS Discovery Response frame | Specifies the service parameters contained in the received TDLS Discovery Response frame. |

##### 10.3.44.2.3 When Generated

This primitive is generated when a valid TDLS Discovery Response frame is received.

##### 10.3.44.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the MLME-TDLSDISCOVERY.confirm primitive and may use the reported data.

#### 10.3.44.3 MLME-TDLSDISCOVERY.indication

##### 10.3.44.3.1 Function

This primitive indicates that a TDLS Discovery Request frame was received from a TDLS peer STA.

##### 10.3.44.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSDISCOVERY.indication(

TDLSPeerSTAAddress,

TDLSDiscoveryRequest

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSPeerSTAAddress | MACAddress | Any valid individual MAC Address | Specifies the MAC address of the TDLS peer STA from which a TDLS Discovery Request frame was received. |
| TDLSDiscoveryRequest | Sequence of octets | As defined in TDLS Discovery Request frame | Specifies the proposed service parameters of the TDLS Discovery Request frame. |

##### 10.3.44.3.3 When Generated

This primitive is generated by the MLME when a valid TDLS Discovery Request frame is received.

##### 10.3.44.3.4 Effect of Receipt

On receipt of this primitive, the SME should operate according to the procedure in 11.19.

#### 10.3.44.4 MLME-TDLSDISCOVERY.response

##### 10.3.44.4.1 Function

This primitive requests that a TDLS Discovery Response frame be sent directly to the TDLS peer STA from which a TDLS Discovery Request frame was received.

##### 10.3.44.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSDISCOVERY.response(

TDLSPeerSTAAddress,

TDLSDiscoveryResponse

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSPeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the MAC address of the TDLS peer STA to which a TDLS Discovery Response frame must be transmitted. |
| TDLSDiscoveryResponse | Sequence of octets | As defined in TDLS Discovery Response frame | Specifies the proposed service parameters for the TDLS Discovery Response frame. |

##### 10.3.44.4.3 When Generated

This primitive is generated by the SME to request that a TDLS Discovery Response frame be sent to the TDLS peer STA STA from which a TDLS Discovery Request frame was received.

##### 10.3.44.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TDLS Discovery Response frame. The STA then attempts to transmit this frame to the TDLS peer STA.