IEEE P802.11
Wireless LANs

| RFI Path Error fixes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6, 80200 München, Germany | +49 * 89 *636 * 49926 | bahr et siemens dod com |

Abstract

This submission is a proposal for normative text to resolve RFI comments related to Path Errors.

Resolved Comments of TGs Letter Ballot LB 159:

Accept: CID 2211

Counter: CIDs 2210, 2212, 2236, 2237

Reject: CID 2744

Resolution notes from 11-10-0369-00

All changes based on IEEE 802.11s draft standard D4.01. In some places, the numbering has to be adapted to the changes.

Instruction to Editor: Change Table 7-22 “Reason codes” in clause 7.3.1.7 “Reason Code field” of the IEEE 802.11s draft standard as indicated by the winword change marks below:

- Reason Code field

Insert the following rows into Reason codesinput.doc and change the last row (Reserved) as shown.

| - Reason codesinput.doc |  |
| --- | --- |
| Reason code | Meaning |
|  | “MESH-PEERING-CANCELLED”. SME cancels the mesh peering instance with the reason other than reaching the maximum number of peer mesh STAs |
|  | “MESH-MAX-PEERS”. The mesh STA has reached the supported maximum number of peer mesh STAs |
|  | “MESH-CONFIGURATION-POLICY-VIOLATION”. The received information violates the Mesh Configuration policy configured in the mesh STA profile |
|  | “MESH-CLOSE-RCVD”. The mesh STA has received a Mesh Peering Close message requesting to close the mesh peering. |
|  | “MESH-MAX-RETRIES”. The mesh STA has re-sent dot11MeshMaxRetries Mesh Peering Open messages, without receiving a Mesh Peering Confirm message. |
|  | “MESH-CONFIRM-TIMEOUT”. The confirmTimer for the mesh peering instance times out. |
|  | “MESH-INVALID-GTK”. The mesh STA fails to unwrap the GTK or the values in the wrapped contents do not match |
|  | “MESH-INCONSISTENT-PARAMETERS”. The mesh STA receives inconsistent information about the mesh parameters between Mesh Peering Management frames |
|  | “MESH-INVALID-SECURITY-CAPABILITY”. The mesh STA fails the Authenticated Mesh Peering Exchange because due to failure in selecting either the pairwise ciphersuite or group ciphersuite |
|  | “MESH-PATH-ERROR-UNSPECIFIED”. The mesh STA sends PERR with no specific reason for the unreachable destination contained in the PERR. |
|  | “MESH-PATH-ERROR-NO-FORWARDING-INFORMATION”. The mesh STA does not have forwarding information for this destination. |
|  | “MESH-PATH-ERROR-DESTINATION-UNREACHABLE”. The mesh STA determines that the link to the next hop of an active path in its forwarding information is no longer usable. |
|  | “MAC-ADDRESS-ALREADY-EXISTS-IN-MBSS”. The Deauthentication frame was sent because the MAC address of the STA already exists in the mesh BSS. See Error: Reference source not found. |
| +1 40-65535 | Reserved |

- The Mesh reason codes need to be allocated before sponsor ballot by ANA.

Instruction to Editor: Change Table 7-26 “Element IDs” in clause 7.3.2 “Information elements” of the IEEE 802.11s draft standard as indicated by the winword change marks below:

- Information elements

Insert the following rows (ignoring the header row and footer note) in Element IDsinput.doc—Element IDs in the correct position to preserve ordering by the “Element ID” column and update the “Reserved” range of codes appropriately.

| - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| Information element | Element ID | Total length of element in octets including the Type and Length octets | Extensible |
| Mesh Configuration Error: Reference source not found |  | 10 | Yes |
| Mesh ID Error: Reference source not found |  | 2 to 34 |  |
| Mesh Link Metric Report Error: Reference source not found |  | 3 to 257 |  |
| Congestion Notification Error: Reference source not found |  | 10 | Yes |
| Mesh Peering Management Error: Reference source not found |  | 8, 10, or 12 | Yes |
| Mesh Channel Switch Announcement Error: Reference source not found |  | 9 | Yes |
| Mesh Awake Window Error: Reference source not found |  | 4 | Yes |
| Beacon Timing Error: Reference source not found |  | 7 to 257 |  |
| MCCAOP Setup Request Error: Reference source not found |  | 7 | Yes |
| MCCAOP Setup Reply Error: Reference source not found |  | 4 or 8 | Yes |
| MCCAOP Advertisements Error: Reference source not found |  | 6 to 257 | Yes |
| MCCAOP Reservation Teardown Error: Reference source not found |  | 3 or 9 | Yes |
| Portal Announcement (PANN) Error: Reference source not found |  | 17 | Yes |
| Root Announcement (RANN) Error: Reference source not found |  | 23 | Yes |
| Path Request (PREQ) Error: Reference source not found |  | 39 to 254 |  |
| Path Reply (PREP) Error: Reference source not found |  | 33 or 39 |  |
| Path Error (PERR) PERR element |  | 17 to 251 |  |
| Proxy Update (PXU) Error: Reference source not found |  | 17 to 257 |  |
| Proxy Update Confirmation (PXUC) Error: Reference source not found |  | 10 | Yes |
| Authenticated Mesh Peering Exchange Error: Reference source not found |  | 86 to 257 |  |
| MIC Error: Reference source not found |  | 18 |  |

- Element ID for these elements needs to be allocated before sponsor ballot by ANA.

Instruction to Editor: Change clause 7.3.2.111 “PERR element” of the IEEE 802.11s draft standard as indicated by the winword change marks below:

- PERR element

The Path Error (PERR) element is used for announcing an unreachable destination.

The format of the PERR element is shown in Path Error element.

| Element ID | Length | TTL | Number of Destinations N | Flags #1 | Destination Address #1 | HWMP Sequence Number #1 | Destination External Address #1 | Reason Code #1 | ... |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 | 0 or 6 | 2 |  |  |
| - Path Error element |  |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element. The length is variable and set to (2 + 13 * Number of Destinations) or to (2 + 19 * Number of Destinations) octets.

The TTL (Time To Live) field is coded as an unsigned integer and indicates the remaining number of hops allowed for this element.

The Number of Destinations N field is coded as an unsigned integer and indicates the number of announced destinations in PERR (Destination Address, HWMP Sequence Number, and Reason Code). The maximum value of N is 19.

The Flags field is shown in PERR Flags field format.

| B0 | B1 | B2 B5 | B6 | B7 |
| --- | --- | --- | --- | --- |
| Reserved | RC | Reserved | AE | Reserved |
| Bits:1 | 1 | 4 | 1 | 1 |
| - PERR Flags field format |  |  |  |  |

Bit 0: Reserved (set to 0).

Bit 1 (RC):The RC subfield indicates the validity of the Reason Code field. It is set to 1 if the Reason Code field is valid, and set to 0 otherwise. When the RC subfield is 0, the Reason Code field is reserved.

Bit 2-5: Reserved (set to 0).

Bit 6 (AE): Address Extension (1 = destination external address is present, 0 = otherwise)

Bit 7: Reserved (set to 0).

The Destination Address field is represented as a 48-bit MAC address and indicates the detected unreachable destination MAC address.

The HWMP Sequence Number field is coded as an unsigned integer and indicates the HWMP sequence number of invalidated destination. The HWMP Sequence Number field is reserved when the USN subfield is 1.

The Destination External Address field is set to the external address, on behalf of which the PERR is sent. This field is present only if Bit 6 (AE) in Flags = 1 and is represented as a 48 bit MAC address.

The Reason Code field specifies the reason for sending a PERR element. The Reason Code is defined in Error: Reference source not found.

The detailed usage of the PERR element is described in 11C.10.8 (Path Error (PERR)).

Instruction to Editor: Change clause 11C.10.4 “Addressing of Mesh Path Selection Action frame” of the IEEE 802.11s draft standard as indicated by the winword change marks below:

- Addressing of Mesh Path Selection Action frame

All HWMP elements are sent in a Mesh Path Selection Action frame (see 7.4.16 (Mesh Path Selection Action frame details)). The RANN element may also be sent in a Beacon frame. “Cases” refer to the different conditions that trigger the transmission of a Mesh Path Selection Action frame. PREQ cases are specified in Error: Reference source not found. PREP conditions are specified in Error: Reference source not found. PERR cases are specified in Conditions for generating and sending a PERR. RANN cases are specified in Error: Reference source not found.

- PREQ Case A, Case B (if previous PREQ received was group addressed), Case C, and Case E (if previous PREQ received was group addressed):

- Address 1: Group address

- Address 2: Address of the mesh STA sending the PREQ

- Address 3: Same as Address 2

- PREQ Case B (if previous PREQ received was individually addressed), Case E (if previous PREQ received was individually addressed):

- Address 1: Next hop MAC address to the Mesh STA identified as the Target MAC address in the PREQ element

- Address 2: Address of the mesh STA sending the PREQ

- Address 3: Same as Address 2

- PREQ Case D:

- Address 1: Address 2 of the frame containing the RANN element that triggered the PREQ

- Address 2: Address of the mesh STA sending the PREQ

- Address 3: Same as Address 2

- PREP Case A, Case C, Case D:

- Address 1: Address of the next hop to the Originator Mesh STA Address in the PREQ that triggered the PREP

- Address 2: Address of the mesh STA sending the PREP

- Address 3: Same as Address 2

- PREP Case B:

- Address 1: Address of the next hop to the Originator Mesh STA Address in the PREP that triggered the PREP

- Address 2: Address of the mesh STA sending the PREP

- Address 3: Same as Address 2

NOTE—Address 1 can only be identical to the “Originator Mesh STA Address” in the PREP element, if Address 2 is the last hop to the originator.

- PERR Case A (individually addressed):

- Address 1: Address of each one of the precursors for which the active forwarding information has been invalidated  (see Case A, Conditions for generating and sending a PERR)

- Address 2: Address of the mesh STA sending the PERR

- Address 3: Same as Address 2

- PERR Case B (individually addressed):

- Address 1: Address of the transmitter of the frame that triggered the PERR (see Case B, Conditions for generating and sending a PERR)

- Address 2: Address of the mesh STA sending the PERR

- Address 3: Same as Address 2

- PERR Case C (individually addressed):

- Address 1: Address of each one of the neighbor peer mesh STAs

- Address 2: Address of the mesh STA sending the PERR

- Address 3: Same as Address 2

- PERR Case D (individually addressed):

- Address 1: Address of each one of the precursors for which the active forwarding information has been invalidated (see Effect of receipt)

- Address 2: Address of the mesh STA sending the PERR

- Address 3: Same as Address 2

- PERR all cases (group addressed):

- Address 1: group address

- Address 2: Address of the mesh STA sending the PERR

- Address 3: Same as Address 2

- RANN:

- Address 1: group address

- Address 2: Address of the mesh STA sending the RANN

- Address 3: Same as Address 2

Multiple HWMP elements may be sent in the same Mesh Path Selection Action frame if they share the same intended Address 1.

Instruction to Editor: Change clause 11C.10.8.3 “Conditions for generating and sending a PERR” of the IEEE 802.11s draft standard as indicated by the winword change marks below:

- Conditions for generating and sending a PERR

A mesh STA shall send out a PERR element in a Mesh Path Selection frame, as defined in 7.4.16.2 (

HWMP Mesh Path Selection frame format), in the following cases:

Case A: Original transmission (next hop is unusable)

The mesh STA has not sent a PERR element less than dot11MeshHWMPperrMinInterval TUs ago and the following applies:

- The mesh STA determines that the link to the next hop of an active path in its forwarding information is no longer usable.

NOTE—The detection might be triggered by the fact that a mesh STA is unable to forward an MSDU/MMPDU to a next hop mesh STA.

The HWMP sequence number in the forwarding information of all unreachable destinations announced in this PERR is incremented by 1. The forwarding information for each unreachable destination announced in this PERR is invalidated.

The contents of a PERR element in Case A shall be as shown in Contents of a PERR element in Case Ainput.doc.

| - Contents of a PERR element in Case Ainput.doc |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 (Element IDs) for the PERR element |
| Length | 2 + N * 13 |
| TTL | The maximum number of hops the element can be propagated before being discarded. |
| Number of Destinations | Number of announced unreachable destinations in the PERR. |
| Flags #1 | Bit 0: Reserved Bit 1(RC): 1 Bit 2-5: Reserved Bit 6 (AE): 0 Bit 7: Reserved |
| Destination Address #1 | MAC address of unreachable destination #1 |
| HWMP Sequence Number #1 | HWMP sequence number for Destination Address #1 from the forwarding information after above increment. |
| Reason Code #1 | “MESH-PATH-ERROR-DESTINATION-UNREACHABLE” (see 7.3.1.7 (Reason Code field)). |
| ... | ... |

Case B: Original transmission (missing forwarding information)

The mesh STA has not sent a PERR element less than dot11MeshHWMPperrMinInterval TUs ago and one of the following applies:

- The mesh STA receives an individually addressed frame with a destination address not matching its own MAC address for which it has no forwarding information.

- The mesh STA receives an individually addressed frame with a destination address not matching its own MAC address and dot11MeshForwarding is false.

The contents of a PERR element in Case B shall be as shown in Contents of a PERR element in Case Ainput.doc.

| - Contents of a PERR element in Case Binput.doc |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 (Element IDs) for the PERR element |
| Length | 2 + N * 13 |
| TTL | The maximum number of hops the element can be propagated before being discarded. |
| Number of Destinations | Number of announced destinations with missing forwarding information in the PERR. |
| Flags #1 | Bit 0: Reserved Bit 1(RC): 1 Bit 2-5: Reserved Bit 6 (AE): 0 Bit 7: Reserved |
| Destination Address #1 | MAC address of destination with missing forwarding information #1. This is Address 3 of the received individually addressed frame. |
| HWMP Sequence Number #1 | reserved (0) |
| Reason Code #1 | “MESH-PATH-ERROR-NO-FORWARDING-INFORMATION” (see 7.3.1.7 (Reason Code field)). |
| ... | ... |

Case C: Original transmission (proxy information is unusable)

The mesh STA has not sent a PERR element less than dot11MeshHWMPperrMinInterval TUs ago and the following applies:

- The mesh STA is a proxy mesh STA and determines that an active proxy information where the mesh STA is the proxy mesh STA is no longer usable.

The contents of a PERR element in Case B shall be as shown in Contents of a PERR element in Case Ainput.doc.

| - Contents of a PERR element in Case Cinput.doc |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 (Element IDs) for the PERR element |
| Length | 2 + N * 19 |
| TTL | The maximum number of hops the element can be propagated before being discarded. |
| Number of Destinations | Number of announced unreachable external destinations in the PERR. |
| Flags #1 | Bit 0: Reserved Bit 1(RC): 0 Bit 2-5: Reserved Bit 6 (AE): 1 Bit 7: Reserved |
| Destination Address #1 | MAC address of proxy mesh STA #1 with unusable active proxy information |
| HWMP Sequence Number #1 | Last used HWMP sequence number for Destination Address #1. |
| Destination External Address #1 | External MAC address of the active proxy information that is not longer usable and for which the mesh STA is the proxy mesh STA. |
| Reason Code #1 | Reserved |
| ... | ... |

Case D: PERR propagation

The mesh STA has not sent a PERR element less than dot11MeshHWMPperrMinInterval TUs ago and all of the following applies:

- The mesh STA received a PERR from a neighbor peer mesh STA.

- A destination in the PERR is the same as one of the destinations in the active forwarding information of the mesh STA where the next hop is the transmitter of the received PERR, and the forwarding information or the proxy information has been invalidated according to conditions in Effect of receipt case b), c) or d).

- dot11MeshForwarding is true

- The TTL field in the received PERR element is greater than or equal to 1.

The contents of a PERR element in Case D shall be as shown in Contents of a PERR element in Case Dinput.doc.

| - Contents of a PERR element in Case Dinput.doc |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 7-26 (Element IDs) for the PERR element |
| Length | if AE flag = 0: 2 + N * 13 if AE flag = 1: 2 + N * 19 |
| TTL | TTL in received PERR element – 1 |
| Number of Destinations | 1 number of destinations in the PERR  received value received number of destinations less the number of received destinations for which the transmitter of the PERR is not the next hop |
| Flags #1 | As received |
| Destination Address #1 | MAC address of unreachable destination #1, as received |
| HWMP Sequence Number #1 | If Flags #1 bit 1 (RC) = 1 and Reason Code j#1 = “MESH-PATH-ERROR-NO-FORWARDING-INFORMATION” and received value = 0, then HWMP sequence number for Destination Address #1 from the forwarding information. Otherwise, as received |
| Destination External Address #1 | As received. This field is only present if Bit 6 of the Flags #1 field (AE flag) is 1. |
| Reason Code #1 | As received |
| ... | ... |

Instruction to Editor: Change clause 11C.10.8.4.2 “Acceptance criteria” of the IEEE 802.11s draft standard as indicated by the winword change marks below:

- Acceptance criteria

The PERR shall be accepted (and shall be processed as described in Effect of receipt) if the following applies:

- The mesh STA that receives the PERR has forwarding information stored where

- the destination is contained in the list of unreachable destinations of the PERR and

- the next hop is the transmitter of the received PERR

Otherwise, the PERR element shall be discarded.

Instruction to Editor: Change clause 11C.10.8.4.3 “Effect of receipt” of the IEEE 802.11s draft standard as indicated by the winword change marks below:

- Effect of receipt

The following applies only to a PERR element that was accepted according to the acceptance criteria in Acceptance criteria.

- The mesh STA creates a list of unreachable destinations consisting of those destinations from the recieved PERR for which the next hop in the local active forwarding information is the transmitter of the PERR.

- If the RC bit is 1 and the Reason Code is “MESH-PATH-ERROR-NO-FORWARDING-INFORMATION” and the HWMP Sequence Number is 0, the receiving mesh STA increments the HWMP sequence number in the forwarding information of the listed unreachable destination by 1 and the forwarding information.

- If b) did not apply and the received HWMP sequence number for a listed unreachable destination is higher than the current HWMP sequence number in the forwarding information for that destination, the receiving mesh STA shall consider that destination unreachable and shall set the HWMP sequence number in the forwarding information to the HWMP sequence number received in the PERR and shall invalidate the forwarding information associated with this unreachable destination.

- If the AE flag for a destination is 1, the receiving mesh STA shall consider the Destination External Address unreachable and shall invalidate the proxy information associated with this unreachable external destination (proxy mesh STA is the Destination Address of the PERR, external MAC address is the Destination External Address of the PERR).

- A PERR element is propagated according to the conditions defined in Conditions for generating and sending a PERR Case B “PERR propagation”.