IEEE P802.11
Wireless LANs

| PHY Simulations and Methodology |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Abu-Surra, Shadi | Samsung |  |  | sasurra@sta.samsung.com |
| Ban, Koichiro | Toshiba |  |  | koichiro.ban@toshiba.co.jp |
| Banerjea, Raja | Marvell |  |  | rajab@marvell.com |
| Basson, Gal | Wilocity |  |  | gal.basson@wilocity.com |
| Blanksby, Andrew | Broadcom |  |  | andrew.blanksby@broadcom.com |
| Borges, Daniel | Apple |  |  | drborges@apple.com |
| Borison, David | Ralink |  |  | david_borison@ralinktech.com |
| Chang, Kapseok | ETRI |  |  | kschang@etri.re.kr |
| Chu, Liwen | STMicroelectronics |  |  | Liwen.chu@st.com |
| Chung, Hyun Kyu | ETRI |  |  | hkchung@etri.re.kr |
| Coffey, Sean | Realtek |  |  | coffey@realtek.com |
| Cordeiro, Carlos | Intel |  |  | Carlos.Cordeiro@intel.com |
| Dorsey, John | Apple |  |  | jdorsey@apple.com |
| Elboim, Yaron | Wilocity |  |  | yaron.elboim@wilocity.com |
| Fischer, Matthew | Broadcom |  |  | mfischer@broadcom.com |
| Giraud, Claude | NXP |  |  | claude.giraud@nxp.com |
| Golan, Ziv | Wilocity |  |  | Ziv.golan@wilocity.com |
| Gong, Michelle | Intel |  |  | Michelle.x.gong@intel.com |
| Grieve, David | Agilent |  |  | david_grieve@agilent.com |
| Grodzinsky, Mark | Wilocity |  |  | Mark.grodzinsky@wilocity.com |
| Hansen, Christopher | Broadcom |  |  | chansen@broadcom.com |
| Hart, Brian | Cisco |  |  | brianh@cisco.com |
| Hassan, Amer | Microsoft |  |  | amerh@microsoft.com |
| Hong, Seung Eun | ETRI |  |  | iptvguru@etri.re.kr |
| Hosoya, Kenichi | NEC |  |  | k-hosoya@ce.jp.nec.com |
| Hosur, Srinath | Texas Instruments |  |  | hosur@ti.com |
| Hsu, Alvin | MediaTek |  |  | alvin.hsu@mediatek.com |
| Hsu, Julan | Samsung |  |  | Julan.hsu@samsung.com |
| Hung, Kun-Chien | MediaTek |  |  | kc.hung@mediatek.com |
| Jain, Avinash | Qualcomm |  |  | avinashj@qualcomm.com |
| Jauh, Alan | MediaTek |  |  | alan.jauh@mediatek.com |
| Jeon, Paul | LGE |  |  | bjjeon@lge.com |
| Jin, Sunggeun | ETRI |  |  | sgjin@etri.re.kr |
| Jones, VK | Qualcomm |  |  | vkjones@qualcomm.com |
| Joseph, Stacy | Beam Networks |  |  | stacy@beamnetworks.com |
| Jun, Haeyoung | Samsung |  |  | Haeyoung.jun@samsung.com |
| Kaaja, Harald | Nokia |  |  | harald.kaaja@nokia.com |
| Kafle, Padam | Nokia |  |  | padam.kafle@nokia.com |
| Kakani, Naveen | Nokia |  |  | naveen.kakani@nokia.com |
| Kasher, Assaf | Intel |  |  | Assaf.kasher@intel.com |
| Kasslin, Mika | Nokia |  |  | mika.kasslin@nokia.com |
| Kim, Hodong | Samsung |  |  | hodong0803.kim@samsung.com |
| Kim, Yongsun | ETRI |  |  | doori@etri.re.kr |
| Kreifeldt, Rick | Harman International |  |  | rick.kreifeldt@harman.com |
| Kwon, Edwin | Samsung |  |  | cy.kwon@samsung.com |
| Kwon, Hyoungjin | ETRI |  |  | kwonjin@etri.re.kr |
| Kwon, Hyukchoon | Samsung |  |  | hyukchoon.kwon@samsung.com |
| Laine, Tuomas | Nokia |  |  | tuomas.laine@nokia.com |
| Lakkis, Ismail | Tensorcom |  |  | ilakkis@tensorcom.com |
| Lee, Hoosung | ETRI |  |  | hslee@etri.re.kr |
| Lee, Keith | AMD |  |  | keith.lee@amd.com |
| Lee, Wooyong | ETRI |  |  | wylee@etri.re.kr |
| Liu, Yong | Marvell |  |  | yongliu@marvell.com |
| Lou, Hui-Ling | Marvell |  |  | hlou@marvell.com |
| Majkowski, Jakub | Nokia |  |  | jakub.majkowski@nokia.com |
| Marin, Janne | Nokia |  |  | janne.marin@nokia.com |
| Maruhashi, Kenichi | NEC |  |  | k-maruhashi@bl.jp.nec.com |
| Matsumoto, Taisuke | Panasonic |  |  | matsumoto.taisuke@jp.panasonic.com |
| Meerson, Yury | Wilocity |  |  | Yury.meerson@wilocity.com |
| Mese, Murat | Broadcom |  |  | mesem@broadcom.com |
| Montag, Bruce | Dell |  |  | bruce_montag@dell.com |
| Myles, Andrew | Cisco |  |  | amyles@cisco.com |
| Nandagopalan, Saishankar | Broadcom |  |  | nsai@broadcom.com |
| Ngo, Chiu | Samsung |  |  | Chiu.ngo@samsung.com |
| Nikula, Eero | Nokia |  |  | eero.nikula@nokia.com |
| Park, DS | Samsung |  |  | dspark@samsung.com |
| Park, Minyoung | Intel |  |  | Minyoung.park@intel.com |
| Pi, Zhouyue | Samsung |  |  | zpi@sta.samsung.com |
| Ponnampalam, Vish | MediaTek |  |  | vish.ponnampalam@mediatek.com |
| Prasad, Narayan | NEC |  |  | prasad@nec-labs.com |
| Prat, Gideon | Intel |  |  | Gideon.prat@intel.com |
| Ramachandran, Kishore | NEC |  |  | kishore@nec-labs.com |
| Raymond, Yu Zhan | Panasonic |  |  | Raymond.Yuz@sg.panasonic.com |
| Ronkin, Roee | Wilocity |  |  | Roee.ronkin@wilocity.com |
| Rozen, Ohad | Wilocity |  |  | Ohad.rozen@wilocity.com |
| Sachdev, Devang | NVIDIA |  |  | dsachdev@nvidia.com |
| Sadri, Ali | Intel |  |  | Ali.S.Sadri@intel.com |
| Sampath, Hemanth | Qualcomm |  |  | hsampath@qualcomm.com |
| Sanderovich, Amichai | Wilocity |  |  | Amichai.sanderovich@wilocity.com |
| Sankaran, Sundar | Atheros |  |  | Sundar.Sankaran@Atheros.com |
| Scarpa, Vincenzo | STMicroelectronics |  |  | vincenzo.scarpa@st.com |
| Seok, Yongho | LGE |  |  | yongho.seok@lge.com |
| Shao, Huai-Rong | Samsung |  |  | hr.shao@samsung.com |
| Shen, Ba-Zhong | Broadcom |  |  | bzshen@broadcom.com |
| Sim, Michael | Panasonic |  |  | Michael.Simhc@sg.panasonic.com |
| Singh, Harkirat | Samsung |  |  | har.singh@sisa.samsung.com |
| Soffer, Menashe | Intel |  |  | Menashe.soffer@intel.com |
| Song, Seungho | SK Telecom |  |  | shsong@sktelecom.com |
| Sorin, Simha | Wilocity |  |  | Simha.sorin@wilocity.com |
| Smith, Matt | Atheros |  |  | matt.smith@atheros.com |
| Stacey, Robert | Intel |  |  | Robert.stacey@intel.com |
| Sutskover, Ilan | Intel |  |  | Ilan.sutskover@intel.com |
| Taghavi, Hossain | Qualcomm |  |  | mtaghavi@qualcomm.com |
| Takahashi, Kazuaki | Panasonic |  |  | takahashi.kazu@jp.panasonic.com |
| Trachewsky, Jason | Self |  |  | jtrachewsky@gmail.com |
| Trainin, Solomon | Intel |  |  | Solomon.trainin@intel.com |
| Usuki, Naoshi | Panasonic |  |  | usuki.naoshi@jp.panasonic.com |
| Varshney, Prabodh | Nokia |  |  | prabodh.varshney@nokia.com |
| Vertenten, Bart | NXP |  |  | bart.vertenten@nxp.com |
| Vlantis, George | STMicroelectronics |  |  | george.vlantis@st.com |
| Wang, Chao-Chun | MediaTek |  |  | chaochun.wang@mediatek.com |
| Wang, Homber | TMC |  |  | homber@emcite.com |
| Wang, James | MediaTek |  |  | james.wang@mediatek.com |
| Yee, James | MediaTek |  |  | james.yee@mediatek.com |
| Yucek, Tevfik | Atheros |  |  | Tevfik.Yucek@Atheros.com |
| Yong, Su Khiong | Marvell |  |  | skyong@marvell.com |
| Zhang, Hongyuan | Marvell |  |  | hongyuan@marvell.com |

Abstract

# This document is part and is in support of the complete proposal described in 802.11-10/432r0 (slides) and 802.11-10/433r0 (text).

# This document contains physical layer (PHY) simulation results for the complete proposal.

# Simulation Conditions

Except as noted below, the following general conditions were employed for the PHY simulations.

## Packet Lengths

8192 bytes for data frames and 256 bytes for control frames (MCS 0 Control PHY only) were employed.

## Channel Models

- Additive White Gaussian Noise (AWGN) model

- 802.11ad Channel Models

  - Channel 1: Conference room, omni TX, omni RX, LOS

  - Channel 2: Conference room, omni TX, directional RX, NLOS

  - Channel 3: Conference room, directional TX, directional RX, NLOS

  - Channel 4: Living room, omni TX, omni RX, LOS

  - Channel 5: Living room, omni TX, directional RX, NLOS

  - Channel 6: Living room, directional TX, directional RX, NLOS

## Hardware Impairments

### Phase Noise

The phase noise in the simulation follows 802.11-09/0296r16.  An example plot of the phase noise from the simulation is given below.

### PA Non-linearity

The power amplifier (PA) non-linearity model from 802.11-09/0296r16 was employed.  The proper back off from saturation was selected to meet the both the TX spectral mask and TX EVM requirements of the proposal.

The following PA output backoffs (with respect to PA saturation power) were used for the packet error rate (PER) simulations:

- MCS 1-5 (SCM/BPSK) 		5.2 dB

- MCS 6-9 (SCM/QPSK) 		6.7 dB

- MCS 10 (SCM/16QAM) 	6.8 dB

- MCS 11-12 (SCM/16-QAM) 	7.0 dB

- MCS 13-14 (OFDM/SQPSK)	10.0 dB

- MCS 15-17 (OFDM/QPSK)	10.0 dB

- MCS 18-21 (OFDM/16-QAM)	12.0 dB

- MCS 22-24 (OFDM/64-QAM)	14.0 dB

### Frequency offset

Carrier frequency and symbol timing were offset by -13.675ppm at the receiver

# Control PHY Simulations

From the complete proposal, 802.11-10/0433r0, section 21.4.

## Simulation Conditions

Frame size 256 bytes.

## Simulation Results

# OFDM MCS Simulations

## OFDM Modulation and Coding Sets

From the complete proposal, 802.11-10/0433r0, section 21.5.

| MCS index | Modulation | Code Rate | NBPSC | NCBPS | NDBPS | Data Rate (Mbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | SQPSK | 1/2 | 1 | 336 | 168 | 693.00 |
| 14 | SQPSK | 5/8 | 1 | 336 | 210 | 866.25 |
| 15 | QPSK | 1/2 | 2 | 672 | 336 | 1386.00 |
| 16 | QPSK | 5/8 | 2 | 672 | 420 | 1732.50 |
| 17 | QPSK | 3/4 | 2 | 672 | 504 | 2079.00 |
| 18 | 16-QAM | 1/2 | 4 | 1344 | 672 | 2772.00 |
| 19 | 16-QAM | 5/8 | 4 | 1344 | 840 | 3465.00 |
| 20 | 16-QAM | 3/4 | 4 | 1344 | 1008 | 4158.00 |
| 21 | 16-QAM | 13/16 | 4 | 1344 | 1092 | 4504.50 |
| 22 | 64-QAM | 5/8 | 6 | 2016 | 1260 | 5197.50 |
| 23 | 64-QAM | 3/4 | 6 | 2016 | 1512 | 6237.00 |
| 24 | 64-QAM | 13/16 | 6 | 2016 | 1638 | 6756.75 |

## Simulation Results

### OFDM MCSs 13-24 on AWGN

### OFDM  MCSs 13-24 on Channel Model #1 - Conference room, omni TX, omni RX, LOS

### OFDM  MCSs 13-24 on Channel Model #2 - Conference room, omni TX, directional RX, NLOS

### OFDM  MCSs 13-24 on Channel Model #3 - Conference room, directional TX, directional RX, NLOS

### OFDM  MCSs 13-24 on Channel Model #4 - Living room, omni TX, omni RX, LOS

### OFDM  MCSs 13-24 on Channel Model #5 - Living room, omni TX, directional RX, LOS

### OFDM  MCSs 13-24 on Channel Model #6 - Living room, directional TX, directional RX, LOS

# Single Carrier MCS Simulations

## Modulation and Coding Sets

From the complete proposal, 802.11-10/0433r0, section 21.6.

## Power Amplifier Backoff Justification

Examples of the transmit mask and EVM are given below to justify the choice of power amplifier backoff employed in the single carrier simulations,

### BSPK Spectral Mask Results

### QPSK Spectral Mask Results

### 16-QAM Spectral Mask Results

### QPSK EVM Results

### 16-QAM EVM Results

## Simulation Results

### SC MCSs 1-12 AWGN

### SC MCSs 1-12 Channel Model #1

### SC MCSs 1-12 Channel Model #2

### SC MCSs 1-12 Channel Model #3

### SC MCSs 1-12 Channel Model #4

### SC MCSs 1-12 Channel Model #5

### SC MCSs 1-12 Channel Model #6

# Low Power Single Carrier MCS

## Simulation Conditions

- AWGN

  - Uncoded Packet length = 4K octets

  - Stop after finding 250 frames in error

- Multipath

  - RMS delay spread ~ 1 ns (maximum delay spread = 5 ns)

  - Uncoded Packet length = 4K octets

  - Stop after finding 250 frames in error

- No non-linear impairments

## AWGN Simulation Results

## Multipath Simulation Results