IEEE P802.11
Wireless LANs

| 11af draft amendment proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yohannes Alemseged | NICT | 3-4 Hikarionooka, 239-0847 Yokosuka, Japan | +81-46- | yohannes@nict.go.jp |
| Ha Nguyen Tran |  |  |  |  |
| Chen Sun |  |  |  |  |
| Zhou Lan |  |  |  |  |
| Sum Chean |  |  |  |  |
| Hiroshi Harada |  |  |  |  |

Abstract

This document presents the proposed text highlighted in yellow corresponding to the proposal in 10-261r2 and the updated proposal in 10-261r3 to be incorporated in the baseline document D 0.01.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

This amendment’s baseline is IEEE Std 802.11™–2007, as amended by

- Amendment 1 802.11k-2008

- Amendment 2 802.11r-2008

- Amendment 3 802.11y-2008

- Amendment 4 802.11w-2009

- Amendment 5 802.11n-2009

- Draft P802.11p_D10.0

- Draft P802.11s_D4.01

- Draft P802.11u_D8.03

- Draft P802.11v_D9.0

- Draft P802.11z_D7.0

- Draft P802.11aa_D0.04

3. Definitions

EDITORIAL NOTE—The subclause numbering of definitions is of the form “3.af<x>” where <x> is an increasing number. The 802.11 technical editor will assign numbers when merging this list into the baseline document.

Insert new definition 3.af1 retaining the alphabetic ordering:

3.af1 TVWS: Television White Space - TV channels where regulatory authorities permit operation by 802.11 STAs.

4. Abbreviations and acronyms

Insert the following new acronym into Clause 4, while maintaining alphabetic ordering:

TVWS			TV White Space

7. Frame formats

7.3 MAC frame formats

7.3.2 Information elements

7.3.2.22 Measurement Report element

7.3.2.22.1 Basic Report

Replace Figure 7-66 by the following figure

New Figure 7-66 Map field format

Insert the following paragraph into 7.3.2.22.1 after the Radar bit description

- TV signal bit, which shall be set to 1 when TV broadcast signal, was detected operating in the channel during the measurement period. The algorithm to detect TV broadcast signal shall satisfy regulatory requirements and is outside the scope of this standard. Otherwise, the TV signal bit shall be set to 0.

17. Orthogonal frequency division multiplexing (OFDM) PHY specification

17.3 OFDM PLCP sublayer

17.3.8 PMD operating specifications (general)

17.3.8.3 Operating channel frequencies

17.3.8.3.2 Channel numbering

Change the first sentence of 17.3.8.3.2 as shown:

Channel numbering in TVWS bands is given in regulation, otherwise channel center frequencies are defined at every integral multiple of 5 MHz above Channel starting frequency.

18. High Rate direct sequence spread spectrum (HR/DSSS) PHY

Specification

18.4 High Rate PMD sublayer

18.4.6 PMD operating specifications, general

18.4.6.2 Numbering of operating channels

Change the first sentence of 18.4.6.2 as shown:

The channel numbering in TVWS bands is given in regulation, otherwise channel center frequencies and CHNL_ID numbers shall be as shown in Table 18-9.

20. High Throughput (HT) PHY specification

20.3 HT PLCP sublayer

20.3.15 Channel numbering and channelization

Change the first paragraph as shown:

The STA may operate in the 5 GHz band and/or 2.4 GHz band. When using 20 MHz channels, it uses

channels defined in 17.3.8.3 (5 GHz band) or 18.4.6 (2.4 GHz band). When using 40 MHz channels, it can operate in the channels defined in 20.3.15.1 (Channel allocation in the 2.4 GHz Band), and 20.3.15.2

(Channel allocation in the 5 GHz band) and 20.3.15.2a (Channel allocation in TVWS bands).

Insert new clause 20.3.15.2a as shown:

20.3.15.2a Channel allocation in TVWS bands

Channel center frequencies are defined in regulations that define channel numbers for operation in TVWS bands.

Insert rows for Regulatory classes <ANA> through <ANA+9>, and change the Reserved row in Table J.1 as follows (note that the entire table is not shown here):

EDITORIAL NOTE—<ANA> will be replaced with a number assigned by the 802.11 Assigned Numbers Authority once that assignment has been made.

Table J.1—Regulatory classes in the United States

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- |
| <ANA> | Center on center | 40 | — | — | 3, 4, 5, 10, 11, 12, 13, <ANA I.3> |
| <ANA+1> | Center on center | 40 | — | — | 3, 4, 5, 10, 11, 12, 14, <ANA I.3> |
| <ANA+2> | Center on center | 20 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+3> | Center on center | 10 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+4> | Center on center | 5 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+5> | Center on lower bound | 40 | — | — | 3, 4, 5, 10, 11, 12, 13, <ANA I.3> |
| <ANA+6> | Center on lower bound | 40 | — | — | 3, 4, 5, 10, 11, 12, 14, <ANA I.3> |
| <ANA+7> | Center on lower bound | 20 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+8> | Center on lower bound | 10 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
| <ANA+9> | Center on lower bound | 5 | — | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |

NOTE—Channel starting frequency “Center on lower bound” means the center frequency of the 802.11 primary modulation is on the lowest frequency of the TV channel, and “Center on center” means the center frequency of the 802.11 primary modulation is on the center frequency of the TV channel.