IEEE P802.11
Wireless LANs

| Text for Regulatory/Operating Classes Specifications Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Tuncer Baykas | NICT | 3-4 Hikarino-oka, Yokosuka, Japan |  | tuncerbaykas@nict.go.jp |
| M. Azizur Rahman | NICT | 3-4 Hikarino-oka, Yokosuka, Japan |  | aziz @nict.go.jp |
| Junyi Wang | NICT | 3-4 Hikarino-oka, Yokosuka, Japan |  | junyi.wang @nict.go.jp |
| Zhou Lan | NICT | 3-4 Hikarino-oka, Yokosuka, Japan |  | lan@nict.go.jp |
| Hiroshi Harada | NICT | 3-4 Hikarino-oka, Yokosuka, Japan |  | harada@nict.go.jp |

Abstract

This document presents the proposed text corresponding to the proposal in IEEE 802.11-10/257r1 and its updated version IEEE 802.11-10/257r2, to be incorperated in the base line document in D0.01.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

Annex J (normative) Country elements and regulatory classes

J.1 Country information and regulatory classes

Insert rows for Regulatory classes <ANA> through <ANA+4>, and change the Reserved row in Table J.1 as follows (note that the entire table is not shown here):

EDITORIAL NOTE—<ANA> will be replaced with a number assigned by the 802.11 Assigned Numbers Authority once that assignment has been made.

Table J.1—Regulatory classes in the United States

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- |
| <ANA> | 0.050 | 40 | 29,93,99,105,117,123 | — | 3, 4, 5, 10, 11, 12, 13, <ANA I.3> |
|  | 0.051 |  | 88,94,100,106,118,124 |  |  |
|  | 0.052 |  | 89,95,101,107,119,125 |  |  |
|  | 0.053 |  | 90,96,102,120, |  |  |
|  | 0.054 |  | 91,97,103,115,121 |  |  |
| <ANA+1> | 0.050 | 40 | 29,93,99,105,117,123 | — | 3, 4, 5, 10, 11, 12, 14, <ANA I.3> |
|  | 0.051 |  | 88,94,100,106,118,124 |  |  |
|  | 0.052 |  | 89,95,101,107,119,125 |  |  |
|  | 0.053 |  | 90,96,102,120, |  |  |
|  | 0.054 |  | 91,97,103,115,121 |  |  |
| <ANA+2> | 0.050 | 20 | 90,96,102,108, 120,126 | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
|  | 0.051 |  | 27,91,97,103,109,115,121,127 |  |  |
|  | 0.052 |  | 28,86,92,98,104,116,122 |  |  |
|  | 0.053 |  | 29,87,93,99,105,117,123 |  |  |
|  | 0.054 |  | 30,88,94,100,106,118,124 |  |  |
| <ANA+3> | 0.050 | 10 | 26,32,90,96,102,108,114,120, 126 | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
|  | 0.051 |  | 27,85,91,97,103,109,115,121, 127 |  |  |
|  | 0.052 |  | 6,28,86,92,98,104,110,116,122,128 |  |  |
|  | 0.053 |  | 29,87,93,99,105,117,123 |  |  |
|  | 0.054 |  | 30,88,94,100,106,118,124 |  |  |
| <ANA+4> | 0.050 | 5 | 7,29,87,93,99,105,111,117,123,129 | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
|  | 0.051 |  | 30,88,94,100,106,118,124 |  |  |
|  | 0.052 |  | 1,25,31,89,95,101,107,113,119,125 |  |  |
|  | 0.053 |  | 26,32,84,90,96,102,108,114, 120,126 |  |  |
|  | 0.054 |  | 5,27,85,91,97,103,109,115,121,127 |  |  |

Insert rows for Regulatory classes <ANA> through <ANA+4>, and change the Reserved row in Table J.2 as follows (note that the entire table is not shown here):

EDITORIAL NOTE—<ANA> will be replaced with a number assigned by the 802.11 Assigned Numbers Authority once that assignment has been made.

Table J.2—Regulatory classes in Europe

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- |
| <ANA> | 0.050 | 40 | 88,96,120,128,136,144 | — | 3, 4, 5, 10, 11, 12, 13, <ANA I.3> |
|  | 0.051 |  | 91,123,131,139 |  |  |
|  | 0.052 |  | 94,118,126,124,142 |  |  |
|  | 0.053 |  | 89,121,129,127 |  |  |
|  | 0.054 |  | 92,116,124,132,140 |  |  |
| <ANA+1> | 0.050 | 40 | 88,96,120,128,136,144 | — | 3, 4, 5, 10, 11, 12, 14, <ANA I.3> |
|  | 0.051 |  | 91,123,131,139 |  |  |
|  | 0.052 |  | 94,118,126,124,142 |  |  |
|  | 0.053 |  | 89,121,129,127 |  |  |
|  | 0.054 |  | 92,116,124,132,140 |  |  |
| <ANA+2> | 0.050 | 20 | 88,96,120,128,136,144 | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
|  | 0.051 |  | 91,115,123,131,139 |  |  |
|  | 0.052 |  | 86,94,118,126,134,142 |  |  |
|  | 0.053 |  | 89,97,121,129,137,145 |  |  |
|  | 0.054 |  | 92,116,124,132,140 |  |  |
| <ANA+3> | 0.050 | 10 | 92,116,124,132,140 | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
|  | 0.051 |  | 87,95,119,127,135,143 |  |  |
|  | 0.052 |  | 90,98,114,122,130,138,146 |  |  |
|  | 0.053 |  | 85,93,117,125,133,141 |  |  |
|  | 0.054 |  | 88,96,120,128,136,144 |  |  |
| <ANA+4> | 0.050 | 5 | 88,96,120,128,136,144 | — | 3, 4, 5, 10, 11, 12, <ANA I.3> |
|  | 0.051 |  | 91,99,115,123,131,139,147 |  |  |
|  | 0.052 |  | 86,94,118,126,134,142 |  |  |
|  | 0.053 |  | 89,97,113,121,129,137,145 |  |  |
|  | 0.054 |  | 84,92,116,124,132,140 |  |  |

Insert rows for Regulatory classes <ANA> through <ANA+4>, and change the Reserved row in Table J.3 as follows (note that the entire table is not shown here):

EDITORIAL NOTE—<ANA> will be replaced with a number assigned by the 802.11 Assigned Numbers Authority once that assignment has been made.