IEEE P802.11
Wireless LANs

| TGae Teleconference Minutes Apr-May 2010 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-04-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | Research In Motion | 4701 Tahoe Blvd. Mississauga, ON. L4W 0B4 | 905-629-4746 Ext. 4999 | mmontemurro@rim.com |

Thursday April 8, 2010

11:00am

### Attendees:

Michael Montemurro,

Allan Thompson,

Santosh Pandey,

Kaberi Bannerjee,

Henry Ptasinski.

- Call to order

- Review of IEEE Intellectual Property Policy

- No essential patent.

- Discussion on Frame Formats for Prioritized Managmeent Frames

  - The feeling was that we could use an “implicit” advertisement approach.

  - Management

  - Requirement for explicit advertisement of priority:

    - Explicit priority is used for data frames to communicate the priority beyond the link between two STA’s.

    - Management frames do not have to travel beyond the link between 2 STA’s.

    - Need to determine whether the priority is regenerated at each hop in a mesh network.

    - IEEE 802.11u frames are not propagated beyond the MAC. There is no specification for how IEEE 802.11u frames may be forwarded beyond the MAC.

    - Need to determine whether we need to propagate priority beyond the link. The consensus is that we do not have to.

    - The MAC’s are provisioned with the priority policy, even though it might not be explicitly communicated.

  - CCMP protection

    - CCMP includes the priority value in the QoS header in the Nonce.

    - The consensus is that the QoS header does not need to be included in the Nonce calculation (although it is today for data frames).

    - The nonce construction could then be set the QoS header to zero in the Nonce.

    - Receiver replay detection: A sliding window approach would have to be adopted for management frame prioritization.

    - IPSEC uses a sliding window approach to replay detection.

    - If management frames are received out of order, what does the receiver do? Likely the MAC would not reorder management frames. The reordering should not be important for management frames.

  - MAC Sequence Counter space

    - The space is defined as per-RA, per-TA, per-TID.

    - Management frames have their sequence counter own space.

    - We would have to adopt a mechanism for duplicate detection. Are there issues with doing this on current implementations?

    - This has implications on BlockACK. BlockACK implements a sliding window, but it will require more investigation.

    - ACTION: Matt Fischer. Investigate how we could use implicit signalling for duplicate detection and recommend an approach that would need to be documented in TGae.

    - We need to respect current implementations.

  - There doesn’t seem to be serious showstoppers with implicit priority signalling for management frame prioritization.

    - ACTION: Henry Ptasinski. Put together some normative text to cover the areas that would need to change to to support implicit transmission.

  - ACTION: Mike. Talk to Adrian about getting a MS Word version of the latest draft of 11mb.

  - We could consider no aggregation for management frames.

  - ACTION: Matt Fischer.  A-MSDU and A-MPDU require frames to use the same TID.  Make a recommendation on how to address A-MSDU and A-MPDU.

  - Management frames encapsulated in data frames would follow data frame transmission rules.

  - We came to the conclusion that enforcing prioritization policy was not possible because it is not measurable.

  - Admission Control discussion:

    - Management frames are currently considered out-of-scope of Admission Control. Changing this might require explicit prioritiy mechanisms.

    - Management frames are not subject t

  - APSD discussion:

    - APSD behaviour should not be affected by TGae.

- The next call on April 22, where we will discussion Management Frame Priority policy.

- Adjourn until the next teleconference call on April 22.