IEEE P802.11
Wireless LANs

| TDLS Discovery Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-905-629-4746 | mmontemurro@rim.com |
| Menzo Wentink | Qualcomm | Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

### Abstract

This submission proposes possible TDLS discovery information, that could be used to leverage layer 2 or layer 3 TDLS mechanisms.

### 2. Normative References

Add the following reference:

W3C Recommendation, Extensible Markup Language (XML) 1.0 (Fifth Edition), http://www.w3.org/TR/REC-xml/.

### 4. Abbreviations and Acronyms

Add the following acronym:

SSDP		Simple Service Discovery Protocol

URL		Universal Resource Locator

XML		Extensible Markup Language

### 7.3.2 Information elements

In table 7-26, insert a new information element, and renumber the reserved values accordingly:

Table 7-26—Element IDs

| Information element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| TDLS Discovery (see 7.3.2.95a) | <ANA> | 2-252 | Yes |

Add a new subclause 7.2.3.95a as follows:

7.3.2.95a TDLS Discovery

The TDLS Discovery element contains information that may be used by STAs to discover TDLS capabilities of a peer STA.

|  | Element ID | Length | TDLS Discovery Information |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 0-250 |

Figure 7-95o33 - TDLS Discovery

The Length field is set to the length of the TDLS Discovery Information field.

The TDLS Discovery Information field contains capability information that a STA may use to establish a TDLS link, in Extensible Markup Language (XML) format. An example of the peer discovery information is described in Annex Y.

7.4.11.12 TDLS Discovery Response frame format

Add the following information element to the TDLS Discovery Response frame, as the final element:

| 16 | TDLS Discovery | Optional. The TDLS Discovery element is specified in 7.3.2.95a. |
| --- | --- | --- |

Add the following Annex

# Annex Y (Informative) TDLS Discovery

Y.1 TDLS discovery information

TDLS discovery information allows TDLS peer STAs to discover their capabilities. An example of TDLS discovery information expressed in the XML format could be as follows:

<TDLS discovery>

<mode> tdls </mode>

<wireless info>

<mac address> 00:01:02:03:04:05 </mac address>

<bssid> 00:0a:0b:0c:0d:0e </bssid>

</wireless info>

< network info>

< DHCP> No</DHCP>

< IP address>192.168.2.1 </IP address>

< Subnet> 255.255.255.0 </subnet>

<Hostname> myphone </Hostname>

</network info>

</TDLS discovery>

The minimum required information for TDLS discovery is contained in “wireless info”.  The “mac address” field contains the MAC address of the transmitting STA and the “bssid” identifies the BSS in which the STA is a member. The “network info” could optionally be used to aid the peer devices in establishing a TDLS link.

Y.2 TDLS Discovery by Higher Layers

The discovery of TDLS capable STAs can also be accomplished by leveraging well-defined higher layer protocols. Since TDLS peer STAs are members of the same BSS and most likely on the same IP network, there are at least two IP based multicast protocols that are well suited for this purpose, namely:

- Bonjour (http://www.apple.com/support/bonjour/)

- UPnP (http://www.upnp.org)

For the Bonjour multicast advertisement protocol, a STA could advertise a Bonjour record with a Service Type (http://www.dns-sd.org/ServiceTypes.html) that identifies TDLS capabilities. The TDLS Bonjour record, at a minimum, should include whether the STA is TDLS capable and the BSSID of the AP to which the STA is associated.  The TDLS Bonjour record may contain other information that is useful and relevant for TDLS discovery and setup.

For the UPnP protocol, a STA could advertise its presence to other STAs on the network by using the Simple Service Discovery Protocol (SSDP). A new STA could use SSDP to discover other STAs that supports UPnP on the network. To support TDLS discovery, the information that is exchanged between STAs, at a minimum, should contain whether the STA is TDLS capable and the BSSID of the AP to which the STA is associated, in addition to basic information about the devices and their services and a description Universal Resource Locator (URL), which can be used to gather additional information about a STA.