IEEE P802.11
Wireless LANs

| TDLS Discovery Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-08-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-905-629-4746 | mmontemurro@rim.com |
| Menzo Wentink | Qualcomm | Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

### Abstract

This submission proposes aTDLS discovery information element using ANQP.  It also re-alligns the IEEE 802.11u and IEEE 802.11z amendments by correcting cross references in 11u.

It addresses sponsor ballot comments 3001, 3004, 3010, 3012 and 3016.

It is based on IEEE 802.11u  D11.0 and IEEE 802.11z  D12.0

### 2. Normative References

Add the following reference:

W3C Recommendation, Extensible Markup Language (XML) 1.0 (Fifth Edition), http://www.w3.org/TR/REC-xml/.

### 4. Abbreviations and Acronyms

Add the following acronym:

XML		Extensible Markup Language

Modify the following clause:

7.3.4.16 TDLS Discovery information

The TDLS Discovery information contains information that a STA uses to discover TDLS capabilities of a peer STA. The format of the TDLS Discovery information is provided in Figure 7-95o157.

|  | Info ID | Length | Discovery information |  |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |  |
| Figure 7-95o157—TDLS Discovery information format |  |  |  |  |

The Length field is a 2-octet field whose value is the length of the Discovery information field.

The Discovery Information field contains capability information that a STA uses to establish a TDLS link, in Extensible Markup Language (XML) format. An example of the peer discovery information is described in Annex Y.1. An example of the use of peer discovery information is described in 11.23.3.2.6.

Add the following Annex

# Annex Y (Informative) TDLS Discovery

Y.1 Discovery information

The discovery information, expressed in the XML format, allows TDLS peer STAs to discover their capabilities.

An example of such discovery information is as follows:

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001?XMLSchema"

xmlns="tdlsdiscovery:base:1.0">

<Discovery information>

<mode> tdls </mode>

<wireless info>

<mac address> 00:01:02:03:04:05 </mac address>

<bssid> 00:0a:0b:0c:0d:0e </bssid>

</wireless info>

< network info>

< DHCP> No</DHCP>

< IP address>192.168.2.1 </IP address>

< Subnet> 255.255.255.0 </subnet>

<Hostname> myphone </hostname>

</network info>

<location info>

<location civic report>civic address</location civic report>

</location info>

</Discovery information>

The minimum required information for discovery is contained in “wireless info”.  The “mac address” field contains the MAC address of the transmitting STA and the “bssid” identifies the BSS in which the STA is a member. The “network info” and “location info” could optionally be used to aid the peer devices in establishing a TDLS link.

Y.2 TDLS Discovery by Higher Layers

The discovery of TDLS capable STAs can also be accomplished by leveraging well-defined higher layer protocols. Since TDLS peer STAs are members of the same BSS and most likely on the same IP network, there are at least two IP based multicast protocols that are well suited for this purpose, namely:

- Bonjour (http://www.apple.com/support/bonjour/)

- UPnP (http://www.upnp.org)

For the Bonjour multicast advertisement protocol, a STA advertises a Bonjour record with a Service Type (http://www.dns-sd.org/ServiceTypes.html) that identifies TDLS capabilities. The TDLS Bonjour record, at a minimum, should include whether the STA is TDLS capable and the BSSID of the AP to which the STA is associated.  The TDLS Bonjour record may contain other information that is useful and relevant for TDLS discovery and setup.

For the UPnP protocol, a STA advertises its presence to other STAs on the network by using the Simple Service Discovery Protocol (SSDP). A new STA uses SSDP to discover other STAs that support UPnP in the network. To support TDLS discovery, the information that is exchanged between STAs, at minimum, should include whether the STA is TDLS capable and the BSSID of the AP to which the STA is associated. In addition other information about STAs, their services and associated URLs, can also be provided.