IEEE P802.11
Wireless LANs

| TDLS Discovery Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-08-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-905-629-4746 | mmontemurro@rim.com |
| Menzo Wentink | Qualcomm | Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

### Abstract

This submission deletes the TDLS discovery information element and removes associated cross references in 11u.

It addresses sponsor ballot comments 3001, 3004, 3010, 3012 and 3016.

It is based on IEEE 802.11u  D11.0

Modify Table 7-43bn in the following clause:

7.3.4 Access Network Query Protocol elements

| Table 7-43bn—Access Network Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | ANQP Info Element (clause) |
| Reserved | 0-255 | n/a |
| ANQP Query list | 256 | 7.3.4.1 |
| ANQP Capability list | 257 | 7.3.4.2 |
| Venue Name information | 25 | 7.3.4.3 |
| Emergency Call Number information | 259 | 7.3.4.4 |
| Network Authentication Type information | 260 | 7.3.4.5 |
| Roaming Consortium list | 261 | 7.3.4.6 |
| IP Address Type Availability information | 262 | 7.3.4.8 |
| NAI Realm list | 263 | 7.3.4.9 |
| 3GPP Cellular Network information | 264 | 7.3.4.10 |
| AP Geospatial Location | 265 | 7.3.4.11 |
| AP Civic Location | 266 | 7.3.4.12 |
| AP Location Public Identifier URI | 267 | 7.3.4.13 |
| Domain Name list | 268 | 7.3.4.14 |
| Emergency Alert Identifier URI | 269 | 7.3.4.15 |
| Emergency NAI | 270 | 7.3.4.16 |
| Reserved | 271– 56796 | n/a |
| Access Network Query Protocol vendor-specific list | 56797 | 7.3.4.7 |
| Reserved | 56798 – 65535 | n/a |

Delete the following clause and renumber successive clauses:

7.3.4.16 TDLS Discovery information

The TDLS Discovery information contains information that a STA uses to discover TDLS capabilities of a peer STA. The format of the TDLS Discovery information is provided in Figure 7-95o157.

|  | Info ID |  | Length | OI | TDLS Discovery information |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 |  | 2 | variable | variable |  |
|  |  | Figure 7-95o157—TDLS Discovery information format |  |  |  |  |

The Length field is set to the sum of OI field length and TDLS Discovery information field length.

The OI field is defined in 7.3.1.31 and indicates the type of TDLS Discovery information.

The TDLS Discovery Information contains capability information that a STA may use to establish a TDLS link. The format is described in 7.3.2.65b. An example of the peer discovery information is described in 11.23.3.2.6.

Modify Table 11-4 in the following clause:

11.23.3.2 Access Network Query Protocol procedures

| Table 11-4—Access Network Query Protocol usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BSS |  | IBSS |
| Info Name | ANQP Info Element (clause) | Element Type | AP | Non-AP STA | STA |
| ANQP Query list | 7.3.4.1 | Q | T, R | T, R | T, R |
| ANQP Capability list | 7.3.4.2 | Q | T, R | T, R | T, R |
| Venue Name information | 7.3.4.3 | S | T | R | ---- |
| Emergency Call Number information | 7.3.4.4 | S | T | R | ---- |
| Network Authentication Type information | 7.3.4.5 | S | T | R | ---- |
| Roaming Consortium list | 7.3.4.6 | S | T | R | ---- |
| Access Network Query Protocol vendor-specific list | 7.3.4.7 | Q, S | T, R | T, R | T, R |
| IP Address Type Availability information | 7.3.4.8 | S | T, R | T, R | T, R |
| NAI Realm list | 7.3.4.9 | S | T | R | T, R |
| 3GPP Cellular Network information | 7.3.4.10 | S | T | R | ---- |
| AP Geospatial Location | 7.3.4.11 | S | T | R | T, R |
| AP Civic Location | 7.3.4.12 | S | T | R | T, R |
| The AP Location Public Identifier URI | 7.3.4.13 | S | T | R | T, R |
| Domain Name list | 7.3.4.14 | S | T | R | ---- |
| Emergency Alert Identifier URI | 7.3.4.15 | S | T | R | T, R |
| Emergency NAI | 7.3.4.16 | S | T | R | ---- |
| Symbols Q element is an ANQP Query S element is an ANQP Response T ANQP element may be transmitted by MAC entity R ANQP element may be received by MAC entity --- ANQP element is neither transmitted nor received by MAC entity |  |  |  |  |  |

Delete the following clause:

11.23.3.2.6 TDLS Discovery procedures

The TDLS Discovery element (7.3.4.16) can be used in a ANQP query to discover TDLS capabilities (e.g. another TDLS capable STA) using public action frames. An example of TDLS discovery using ANQP is given in Figure 11-17h. The discovery shall work as follows:

1)	STA1 determines the MAC address of STA2.

2)	STA1 sends a ANQP request to STA2. The ANQP request includes a TDLS Discovery element and transmits the frame as a GAS query.

3)	STA2 receives the ANQP request frame and includes the TDLS Discovery element in the ANQP response frame to STA1.

Figure 11-17h—Example TDLS capability discovery using ANQP