IEEE P802.11
Wireless LANs

| MCCA text for LB 161 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dee Denteneer | Philips | HTC 37; Eindhoven; The Netherlands | +31 40 27 49743 | Dee.denteneer@philips.com |

Abstract

This document provides text changes that intend to resolve the MCCA related CIDs for LB 159 identified in the accompanying spreadsheet 10/158r1 by the labels “MCCA-1”, “MCCA-2”, and “MCCA-3” in Column X.

Abstract

This document provides text changes that resolve the MCCA related CIDs for LB 161 identified in the accompanying spreadsheet 10/501r2 by the label “X”, “Y”, or “Z” in Column X.

- Definitions specific to IEEE 802.11

Insert the following new definitions:

The subclause numbering of definit

- Information elements

Insert the following rows (ignoring the header row and footer note) in Element IDsinput.doc—Element IDs in the correct position to preserve ordering by the “Element ID” column and update the “Reserved” range of codes appropriately.

| - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| Information element | Element ID | Total length of element in octets including the Type and Length octets | Extensible |
| Mesh Configuration Error: Reference source not found |  | 10 | Yes |
| Mesh ID Error: Reference source not found |  | 2 to 34 |  |
| Mesh Link Metric Report Error: Reference source not found |  | 3 to 257 |  |
| Congestion Notification Error: Reference source not found |  | 10 | Yes |
| Mesh Peering Management Error: Reference source not found |  | 5, 7, 9, 21, 23, or 25 | Yes |
| Mesh Channel Switch Parameters Error: Reference source not found |  | 8 | Yes |
| Mesh Awake Window Error: Reference source not found |  | 4 | Yes |
| Beacon Timing Error: Reference source not found |  | 3 to 253 |  |
| MCCAOP Setup Request Error: Reference source not found |  | 7 | Yes |
| MCCAOP Setup Reply Error: Reference source not found |  | 4 or 8 | Yes |
| MCCAOP Advertisements Error: Reference source not found |  | 6 to 257 | Yes |
| MCCAOP Teardown Error: Reference source not found |  | 3 or 9 | Yes |
| Gate Announcement (GANN) Error: Reference source not found |  | 17 | Yes |
| Root Announcement (RANN) Error: Reference source not found |  | 23 | Yes |
| Path Request (PREQ) Error: Reference source not found |  | 39 to 254 |  |
| Path Reply (PREP) Error: Reference source not found |  | 33 or 39 |  |
| Path Error (PERR) Error: Reference source not found |  | 17 to 251 |  |
| Proxy Update (PXU) Error: Reference source not found |  | 17 to 257 |  |
| Proxy Update Confirmation (PXUC) Error: Reference source not found |  | 9 | Yes |
| Authenticated Mesh Peering Exchange Error: Reference source not found |  | 86 to 257 |  |
| MIC Error: Reference source not found |  | 18 |  |

- Mesh Capability

The Mesh Capability field comprises a set of values indicating whether a mesh STA is a possible candidate for mesh peering establishment. The details of the Mesh Capability field are shown in .

| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Accepting Additional Mesh Peerings | MCCA Supported | MCCA Enabled | Forwarding | MBCA Enabled | TBTT Adjusting | Mesh Power Save Level | Reserved |
| Bits: 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| - Mesh Capability field |  |  |  |  |  |  |  |

The Accepting Additional Mesh Peerings subfield is set to 1 if the mesh STA is willing to establish additional mesh peerings with other mesh STAs and set to 0 otherwise (i.e., the Accepting Additional Mesh Peerings subfield is set to the value of dot11MeshAcceptingAdditionalPeerings).

The MCCA Supported subfield is set to 1 if the mesh STA implements MCCA and set to 0 otherwise (i.e., the MCCA Supported subfield is set to the value of dot11MCCAImplemented).

The MCCA Enabled subfield is set to 1 if the mesh STA is using the MCCA CF and set to 0 otherwise (i.e., the MCCA Enabled subfield is set to the value of dot11MCCAActivated).

The Forwarding subfield is set to 1 if the mesh STA forwards MSDUs and set to 0 otherwise (i.e., the Forwarding subfield is set to the value of dot11MeshForwarding).

The MBCA Enabled subfield is set to 1 if the mesh STA activates MBCA, and is set to 0 otherwise (i.e., the MBCA Enabled subfield is set to the value of dot11MBCAActivated). (See 11C.12.4 (Mesh Beacon Collision Avoidance (MBCA)))

The TBTT Adjusting subfield is set to 1 while the TBTT adjusting procedure is on going to notify that the mesh STA’s TBTT is shifting intentionally, and is set to 0 otherwise. (See 11C.12.4.4.3 (Adjustment procedure))

The Mesh Power Save Level field and the Power Management field in the Frame Control field indicate the non-peer mesh power mode of the mesh STA. The Power Management subfield set to 1 and the Mesh Power Save Level field set to 0 indicate that the mesh STA is operating in light sleep mode for non-peer mesh STAs. The Power Management field set to 1 and the Mesh Power Save Level set to 1 indicate that mesh STA is operating in deep sleep mode for non-peer mesh STAs. The Mesh Power Save Level field is reserved, if the Power Management subfield is set to 0.

MCCAOP Setup Request element

- General

The MCCAOP Setup Request element is used to make an MCCAOP reservation. This element is transmitted in individually addressed MCCA Setup Request frames or in group addressed MCCAOP Setup Request frames. The mesh STA transmitting the MCCA Setup Request element is the MCCAOP owner of the MCCAOPs that will be scheduled with this reservation setup request. The receivers of the MCCAOP Setup Request are the MCCAOP responders. The format of the element is as shown in MCCAOP Setup Request element.

| Element ID | Length | MCCAOP Reservation ID | MCCAOP Reservation |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 4 |
| - MCCAOP Setup Request element |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element.

The Length field is set to 5 octets.

The MCCAOP Reservation ID field is an eight bit unsigned integer that represents the ID for the MCCAOP reservation. It is determined by the MCCAOP owner. When used in combination with the MAC address of the MCCAOP owner, the MCCAOP Reservation ID uniquely identifies the MCCAOP reservation.

If this MCCAOP Setup Request is for an individually addressed transmission, the MCCAOP Reservation ID is between 0 and 127 and the MCCAOP Setup Request element is transmitted in an individually addressed frame to the intended responder. If this MCCAOP Setup Request is for a group addressed transmission, the MCCAOP Reservation ID is between 128 and 254 and the MCCAOP Setup Request element is transmitted in individually addressed frames to each of the intended responders or in a group addressed frame. The value 255 is not used to identify a single MCCAOP reservation.

The MCCAOP Reservation field is described inError: Reference source not found.

- MCCAOP Reservation field

The MCCAOP Reservation field is a 4 octet field specifying a schedule for frame transmissions called MCCAOPs. The MCCAOP Reservation field consists of 3 subfields and its format is illustrated in MCCAOP Reservation field.

| MCCAOP Duration | MCCAOP Periodicity | MCCAOP Offset |
| --- | --- | --- |
| Octets: 1 | 1 | 2 |
| - MCCAOP Reservation field |  |  |

The MCCAOP Duration field is one octet in length and contains an unsigned integer. It specifies the duration of the MCCAOPs in multiples of 32 µs.

The MCCAOP Periodicity field is one octet in length and contains a positive integer. It specifies the number of MCCAOPs scheduled in each DTIM interval.

The MCCAOP Offset field is two octets in length and contains an unsigned integer. It specifies the beginning of the first MCCAOP in each DTIM interval. The value is specified in multiples of 32 µs. The sum of MCCAOP Offset plus MCCAOP Duration is constrained to be smaller than the duration of the DTIM interval divided by MCCAOP Periodicity.

- MCCAOP Setup Reply element

The MCCAOP Setup Reply element is used to reply to an MCCAOP Setup Request. This element is transmitted in individually addressed MCCA Setup Reply frames. The mesh STA transmitting the MCCA Setup Reply element is the MCCAOP responder of the MCCAOPs scheduled in this reservation setup. The receiver of the MCCAOP Setup Reply is the MCCAOP owner. The format of the element is as shown in Error: Reference source not found.

| Element ID | Length | MCCAOP Reservation ID | MCCA Reply Code | MCCAOP Reservation |
| --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 4 |
| - MCCAOP Setup Reply element |  |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element.

The Length field is set to 2 or 6 octets.

The MCCAOP Reservation ID field is an eight bit unsigned integer that represents the ID for the requested series of MCCAOPs. It is determined by the MCCAOP owner and copied from the MCCAOP Setup Request element. When used in combination with the MAC address of the MCCAOP owner, the MCCAOP Reservation ID uniquely identifies the MCCAOP reservation. If this MCCAOP Setup Request is for an individually addressed transmission, the MCCAOP Reservation ID is between 0 and 127. If this MCCAOP Setup Request is for a group addressed transmission, the MCCAOP Reservation ID is between 128 to 254. The value 255 is reserved and is not used to identify a single MCCAOP reservation.

The MCCA Reply Code field is a one octet field that contains the reply code used in an MCCAOP Setup Reply element. The reply codes are defined in MCCA Reply codesinput.doc

| - MCCA Reply codesinput.doc |  |
| --- | --- |
| MCCA Reply code | Meaning |
| 0 | Accept |
| 1 | Reject: MCCAOP reservation conflict |
| 2 | Reject: MAF limit exceeded |
| 3 | Reject: MCCA Track Limit exceeded |
| Other | Reserved |

The MCCAOP Reservation field includes an alternative to the MCCAOP reservation specified in the MCCAOP Setup Request message. Its format is described in Error: Reference source not found. When the MCCA Reply Code is 1, the MCCAOP Reservation field may be present. When the MCCA Reply Code is set to other values, the MCCAOP Reservation field is not present.

- MCCAOP Advertisements element

- General

The MCCAOP Advertisements element is used by a mesh STA to advertise MCCAOP reservations to its neighbors. This element can be carried in selected Beacon frames with a chosen frequency. This element is also transmitted in an MCCAOP Advertisements frame. The format of the element is as shown in MCCAOP Advertisements element.

| Element ID | Length | MCCA Information | TX-RX Times Report | Broadcast Times Report | Interfering Times Report |
| --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 4 | variable | variable | variable |
| - MCCAOP Advertisements element |  |  |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element. The Length field is set to 4 to 255 octets.

The MCCA Information field is 3 octets in length. It is described in MCCA Information field.

The TX-RX Times Report field is a variable length field that contains an MCCAOP Reservation Report field as described in MCCAOP Reservation Report field. This field is only present when the TX-RX Report Present subfield of the MCCAOP Information field is equal to 1. Through the TX-RX Times Report field, a mesh STA advertises the MCCAOPs in which it is involved as an MCCAOP owner or an MCCAOP responder and in which individually addressed frames are transmitted. This report optionally includes transmission times and reception times of a STA collocated with the mesh STA, for example Beacon and HCCA times.

The Broadcast Times Report field is a variable length field that contains an MCCAOP Reservation Report field as described in MCCAOP Reservation Report field. This field is only present when the Broadcast Report Present subfield of the MCCAOP Information field is equal to 1. Through the Broadcast Times Report field, a mesh STA advertises the MCCAOPs in which it is involved as an MCCAOP owner or an MCCAOP responder and in which group addressed frames are transmitted.

The Interfering Times Report field is a variable length field that contains an MCCAOP Reservation Report field as described in MCCAOP Reservation Report field. This field is only present when the Interfering Report Present subfield of the MCCAOP Information field is equal to 1. Through the Interfering Times Report field, a mesh STA reports the TX-RX andsbroadcast times of its peer mesh STAs and optionally its neighboring non-peer mesh STAs, excluding its own TX-RX and Broadcast times.

- MCCA Information field

The MCCA Information field is 3 octets in length and is used to provide information on the MCCAOP reservations. The field consists of seven subfields and its format is illustrated in .

| B0 B7 | B8 B15 | B16 | B17 | B18 | B19 | B20 | B21 B23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCCA Access Fraction | MCCA Access Fraction Limit | Accept Reservations | TX-RX Report Present | Broadcast Report Present | Interfering Report Present | Last Advertisement | Advertisement Identifier |
| Bits: 8 | 8 | 1 | 1 | 1 | 1 | 1 | 3 |
| - MCCA Information field |  |  |  |  |  |  |  |

The MCCA Access Fraction subfield is an eight bit unsigned integer. The MCCA Access Fraction subfield is set to the current value of MCCA Access Fraction at the mesh STA rounded down (floor) to the nearest multiple of (1/255).

The MCCA Access Fraction Limit subfield is an eight bit unsigned integer. The MCCA Access Fraction Limit subfield is set to the maximum MCCA access fraction allowed at the mesh STA rounded down (floor) to the nearest multiple of (1/255).

The Accept Reservations subfield is one bit in length. It is set to 1 if the mesh STA can accept and track additional reservations.

The TX-RX Report Present subfield is one bit in length. It is set to 1 if a TX-RX Report field is present in the MCCAOP Advertisement element and set to 0 if no TX-RX Report field is present.

The Broadcast Report Present subfield is one bit in length. It is set to 1 if a Broadcast Report field is present in the MCCAOP Advertisement element and set to 0 if no Broadcast Report field is present.

The Interfering Report Present subfield is one bit in length. It is set to 1 if an Interfering Times Report field is present in the MCCAOP Advertisement element and set to 0 if no Interfering Times Report field is present.

The Last Advertisement subfield is one bit in length. It is set to 0 if the Advertisement element element is the last one of a series of  Advertisment elements and it is set to 1 if the Advertisement element is not the last one of a series of advertisement elements.

The Advertisement Identifier subfield is a three bits unsigned integer. It identifies the MCCAOP Advertisement carried in the MCCAOP Advertisement element, except when the Advertisement Identifier subfield equals 7 and the Last Advertisement subfield equals 1.

- MCCAOP Reservation Report field

The MCCAOP Reservation Report field is of variable length and is used to report a number of MCCAOP reservations. The field consists of a variable number of subfields and its format is illustrated in MCCAOP Reservation Report field.

| Number of Reported MCCAOP Reservations | MCCAOP Reservation 1 | ... | MCCAOP Reservation n |
| --- | --- | --- | --- |
| Octets: 1 | 4 |  | 4 |
| - MCCAOP Reservation Report field |  |  |  |

The Number of Reported MCCAOP Reservations is a field of one octet with an unsigned integer that specifies the number, n, of MCCAOP reservations reported in this field.

The MCCAOP Reservation 1 through MCCAOP Reservation n fields specify the MCCAOP reservations reported. Each field is of length 4 octets and contains an MCCAOP Reservation field, whose format is shown in MCCAOP Reservation field in Error: Reference source not found.

- MCCAOP Teardown element

The MCCAOP Teardown element is used to announce the teardown of an MCCAOP reservation. The MCCAOP Teardown element is transmitted in individually addressed MCCA Teardown frames or in group addressed MCCA Teardown frames. Its format is shown in MCCAOP Teardown element.

| Element ID | Length | MCCAOP Reservation ID | MCCAOP Owner |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 6 |
| - MCCAOP Teardown element |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element.

The Length is variable and set to 1 or 7 octets.

An MCCAOP Teardown element is transmitted by either the MCCAOP owner or the MCCAOP responder of a MCCAOP reservation to tear down the MCCAOP reservation.

The MCCAOP Reservation ID field is an eight bit unsigned integer that represents the ID for the MCCAOP reservation.

The MCCAOP Owner field is an optional field. It is six octets long and indicates the 48-bit MAC address of the MCCAOP owner. This field is only included if the element is transmitted by the MCCAOP responder.

- Action frame format details

- Mesh Resource Coordination Action frame details

- General

Action frame formats for management of Mesh Resource Coordination are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Mesh Resource Coordination Action field valuesinput.doc.

| - Mesh Resource Coordination Action field valuesinput.doc |  |
| --- | --- |
| Action field value | Description |
|  | Congestion Control Notification |
|  | MCCA Setup Request |
|  | MCCA Setup Reply |
|  | MCCAOP Advertisement Request |
|  | MCCAOP Advertisements |
|  | MCCA Teardown |
|  | TBTT Adjustment Request |
|  | TBTT Adjustment Response |
| 8-255 | Reserved |

- Congestion Control Notification frame format

The Congestion Control Notification frame is used to indicate its congestion status to its neighbor peer mesh STA(s). This frame is transmitted using individual addresses or group addresses. The format of the Congestion Control Notification frame body is shown in Congestion Control Notification frame bodyinput.doc.

| - Congestion Control Notification frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Congestion Notification element |  |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc representing Congestion Control Notification.

The Congestion Notification element field is set as described in Error: Reference source not found.

- MCCA Setup Request frame format

The MCCA Setup Request frame is used to set up an MCCAOP reservation. It is transmitted by a mesh STA with dot11MCCAActivated equal to true to one or more neighbor peer mesh STA with dot11MCCAActivated equal to true. This frame is transmitted using individual addresses or group addresses. The format of the MCCA Setup Request frame body is shown in MCCA Setup Request frame bodyinput.doc.

| - MCCA Setup Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Setup Request element |  |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc representing MCCA Setup Request.

The MCCA Setup Request element is described in Error: Reference source not found.

- MCCA Setup Reply frame format

The MCCA Setup Reply frame is used to reply to an MCCAOP Setup Request. It is transmitted by a mesh STA with dot11MCCAActivated equal to true to a neighbor peer mesh STA with dot11MCCAActivated equal to true. This frame is transmitted using individual addresses. The format of the MCCA Setup Reply frame body is shown in MCCA Setup Reply frame bodyinput.doc.

| - MCCA Setup Reply frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Setup Reply element |  |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc representing MCCA Setup Reply.

The MCCA Setup Reply element is described in Error: Reference source not found.

- MCCAOP Advertisement Request frame format

The MCCAOP Advertisement Request frame is transmitted by a mesh STA with dot11MCCAActivated equal to true to a neighbor peer mesh STAs with dot11MCCAActivated equal to true in order to request MCCA advertisements from a neighbor peer mesh STA. This frame is transmitted using individual addresses. The format of the MCCAOP Advertisement Request frame body is shown in MCCAOP Advertisement Request frame bodyinput.doc.

| - MCCAOP Advertisement Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc representing MCCAOP Advertisement Request.

- MCCAOP Advertisements frame format

The MCCAOP Advertisements frame is transmitted by a mesh STA with dot11MCCAActivated equal to true to one or more neighbor peer mesh STAs with dot11MCCAActivated equal to true. This frame is transmitted using group addresses or individual addresses. The format of the MCCAOP Advertisements frame body is shown in MCCAOP Advertisements frame bodyinput.doc.

| - MCCAOP Advertisements frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Advertisements element |  |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc representing MCCAOP Advertisements.

The MCCAOP Advertisements element is described in Error: Reference source not found.

- MCCA Teardown frame format

The MCCA Teardown frame is transmitted by a mesh STA with dot11MCCAActivated equal to true to one or more neighbor peer mesh STAs with dot11MCCAActivated equal to true. This frame is transmitted using group addresses or individual addresses. The format of the MCCATeardown frame body is shown in MCCA Teardown frame bodyinput.doc.

| - MCCA Teardown frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCAOP Teardown element |  |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc representing MCCP Teardown.

The MCCAOP Teardown element is described in Error: Reference source not found.

- TBTT Adjustment Request frame format

The TBTT Adjustment Request frame is used to request a particular neighbor peer mesh STA to adjust its TBTT. This frame is transmitted using individual addresses. The format of the TBTT Adjustment Request frame body is shown in TBTT Adjustment Request frame bodyinput.doc.

| - TBTT Adjustment Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
| - to (N_Info+2) | Beacon Timing element | Repeated N_Info times (N_Info is a number of beacon timing information tuples as described in 11C.12.4.2.3 (Transmitter’s procedure). |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc representing TBTT Adjustment Request.

The Beacon Timing element is set as described in Error: Reference source not found. When the number of beacon timing information exceeds 50, multiple Beacon Timing elements are present. The elements are present in the order of Report Number field value in the Report Control field.

- TBTT Adjustment Response frame format

The TBTT Adjustment Response frame is used to respond to a TBTT adjustment request. This frame is transmitted using individual addresses. The format of the TBTT Adjustment Response frame body is shown in TBTT Adjustment Response frame bodyinput.doc.

| - TBTT Adjustment Response frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Status Code |  |
| - to (N_Info+2) | Beacon Timing element (optional) | Repeated N_Info times (N_Info is a number of beacon timing information tuples as described in 11C.12.4.2.3 (Transmitter’s procedure). |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc representing TBTT Adjustment Response.

The Beacon Timing element, defined in Error: Reference source not found is present only if the Status Code is set to Error: Reference source not found (i.e., when the request is not successful due to the neighbor constraint). When the number of beacon timing information exceeds 50, multiple Beacon Timing elements are present. The elements are present in the order of Report Number field value in the Report Control field.

- MAC architecture

Insert the following text to the end of 9.1:

Due to the distributed nature of the MBSS, only the MCF is present in a mesh STA.

Replace MAC architecture with the following figure:

- The changes comprise the addition of MCF component over DCF;

- MAC architecture

- Hybrid coordination function (HCF)

- HCF contention-based channel access (EDCA)

Change the second paragraph in 9.1.3.1 as shown:

- The following text is based on the amendment by TGp.

For each AC, an enhanced variant of the DCF, called an enhanced distributed channel access function (EDCAF), contends for TXOPs using a set of EDCA parameters. When communicating data frames outside the context of a BSS (dot11OCBEnabled is true), the EDCA parameters are the corresponding default values or are as set by the SME in the MIB attribute table dot11EDCATable (except for TXOP limit values, which shall be set to zero for each AC). When communicating within a BSS, the EDCA parameters used are from the EDCA Parameter Set element, or from the default values for the parameters when no EDCA Parameter Set element is received from the AP of the BSS with which the STA is associated or when the STA is a mesh STA. The parameters used by the EDCAF to control its operation are defined by MIB attribute table dot11QAPEDCATable at the AP and by MIB attribute table dot11EDCATable at the non-AP STA.

Change the second to last paragraph in 9.1.3.1 as shown:

The management frames shall be sent using the access category AC_VO without being restricted by admission control procedures. A QoS STA shall also send management frames using the access category AC_VO before associating with any BSS or before establishing mesh peerings with MBSS, even if there is no QoS facility available in that BSS. BlockAckReq and BlockAck control frames shall be sent using the same QoS parameters as the corresponding QoS data frames. PS-Poll control frames shall be sent using the access category AC_BE to reduce the likelihood of collision following a Beacon frame. For the purpose of determining the proper AC for an RTS frame, the RTS frame shall inherit the UP of the data or management frame(s) that are included in the frame exchange sequence where the RTS is the first frame.

Insert the following new subclause immediately after the 9.1.3:

- Mesh coordination function (MCF)

The mesh facility includes an additional coordination function called MCF that is usable only in an MBSS. Mesh STAs shall implement the MCF only. MCF has both a contention-based channel access and contention free channel access mechanism. The contention based mechanism is EDCA and the contention free mechanism is called the MCF coordinated channel access (MCCA) that optimizes frame exchanges in the MBSS. MCF uses the default values for PTKSA, GTKSA and STKSA Replay Counters. The operation rules of the EDCA are defined in HCF contention-based channel access (EDCA). The operation rules of the MCCA are defined in MCF coordinated channel access (MCCA).

- Multirate support

- clause numbering based on TGn adding 9.6.0c.

Insert the following new subclause after the 9.6.0c.

- Basic Rate Set and Basic MCS Set for mesh STA

Because a mesh STA does not establish a mesh peering with a mesh STA using a different BSSBasicRateSet, all mesh STAs in an MBSS use the same BSSBasicRateSet. See 11C.4.3.1 (Overview).

Mesh STAs should adopt the mandatory PHY rates as the default BSSBasicRateSet to reduce the risk that a candidate peer mesh STA utilizes a different BSSBasicRateSet. If the mesh STA is also an HT STA, it should adopt the MCSs of mandatory MCSs as the default BSSBasicMCSSet.

Once the mesh STA establishes a mesh peering with a mesh STA, it shall change neither the BSSBasicRateSet nor the BSSBasicMCSSet parameters.

- HCF

- HCF contention-based channel access (EDCA)

- EDCA TXOPs

Change the fifth paragraph in 9.9.1.2 as shown:

It should be noted, that when transmitting multiple frames in a TXOP using acknowledgment mechanisms other than Normal Ack, a protective mechanism should be used (such as RTS/CTS or the protection mechanism described in 9.13). A QoS AP may send broadcast/multicast group addressed frames without using any protection mechanism. In a QoS IBSSs or in MBSSs, or when dot11OCBEnabled is true, broadcast/multicast group addressed frames shall be sent one at a time, and backoff shall be performed after the transmission of each of the broadcast/multicast group addressed frames.

- Truncation of TXOP

Change the fourth paragraph in 9.9.1.7 as follows:

A STA shall interpret the reception of a CF-End frame as a NAV reset, i.e., it resets its NAV timer to zero at

the end of the PPDU containing this frame. After receiving a CF-End frame with a matching BSSID, an AP

may respond by transmitting a CF-End frame after SIFS.

Insert the following new subclause after 9.9:

- MCF

- General

Under MCF, the basic unit of allocation of the right to transmit onto the WM is the TXOP. Each TXOP is defined by a starting time and a defined maximum length.

Under MCF, there are two types of TXOPs: EDCA TXOPs and MCCA TXOPs. The EDCA TXOP is obtained by a mesh STA winning an instance of EDCA contention (see HCF contention-based channel access (EDCA)). The MCCA TXOP is obtained by a mesh STA gaining control of the WM during an MCCAOP. The MCCAOP is an advance reservation of a time interval for its transmission by means of an MCCA Opportunity Reservation (see MCF coordinated channel access (MCCA)). Neither EDCA TXOPs nor MCCA TXOPs shall exceed dot11MaxDwellTime (if using an FH PHY).

- MCF contention-based channel access

In an MBSS, EDCA implements the contention-based channel access. HCF contention-based channel access (EDCA) defines EDCA.

- MCF coordinated channel access (MCCA)

- General

MCF coordinated channel access (MCCA) is an optional access method that allows mesh STAs that implement MCCA to access the WM at selected times with lower contention than would otherwise be possible. This standard does not require all mesh STAs to use MCCA. MCCA can be used by a subset of mesh STAs in an MBSS. However, MCCAOP reservations can only be set up among mesh STAs that have dot11MCCAActivated true and operate on the same channel. The performance of MCCA may be impacted by devices that do not respect MCCAOP reservations.

MCCA enabled mesh STAs use management frames to make an advance reservation for transmissions. The mesh STA transmitting an MCCAOP Setup Request frame to initiate an advance reservation is the MCCAOP owner of the MCCAOP reservation defined in this setup. The receivers of the MCCAOP Setup Request are the MCCAOP responders. The MCCAOP owner and the MCCAOP responders advertise this advance reservation to their neighbors via MCCAOP Advertisements. The MCCA enabled neighbor mesh STAs that could cause interference to transmissions during these reserved time periods, or that would experience interference from them, shall not initiate a transmission during these reserved time periods. During its MCCAOP, the MCCAOP owning mesh STA obtains a TXOP by winning an instance of EDCA contention. Because of its advance reservation, the MCCAOP owning mesh STA experiences no competition from other MCCA enabled neighbor mesh STAs. At the start of an MCCAOP, the EDCAF of the MCCAOP owner replaces the AIFSN, CWmin, and CWmax value of its dot11EDCATable with MCCA access parameters.

In order to use MCCA, a mesh STA maintains synchronization with its neighboring mesh STAs. Mesh STAs that use MCCA shall use a DTIM interval with a duration of

2^n 100 TU

with n a non-negative integer less than or equal to 5. Additionally, a mesh STA shall track the reservations of its neighboring mesh STAs.

NOTE— In this clause, a frame with the RA field set to an individually address is referred to as an individually addressed frame. The DA field of the individually addressed frame in this context may not be set to an individual address, if the frame uses 4 address or 6 address format. Similarly, frames with the RA field set to a group address are referred to as group addressed frames, in this subclause.

- MCCA operation process

As a consequence of an MLME-ACTIVATEMCCA.request a mesh STA shall set the MCCA Enabled subfield of the Mesh Capability field in the Mesh Configuration element to 1 in Beacon and Probe Response frames it transmits.

It shall not initiate or accept MCCAOP Setup Requests for dot11MCCAScanDuration TUs after the receipt of the MLME-ACTIVATEMCCA.request.

During this period the mesh STA learns its neighborhood MCCAOP times by receiving Beacon, Probe Response, or MCCAOP Advertisements frames of neighboring mesh STAs with the MCCA Enabled sub-field of the Mesh Capability field in the Mesh Configuration element equal to 1.

After this period the mesh STA may initiate and accept MCCAOP Setup Requests as per MCCAOP setup procedure.

- MCCAOP reservations

An MCCAOP reservation specifies a schedule for frame transmissions. The time periods scheduled for frame transmissions in the reservation are called MCCAOPs. The schedule is set up between an MCCAOP owner and one (for individually addressed frames) or more (for group addressed frames) MCCAOP responders. MCCAOPs are setup by means of the procedure defined in MCCAOP setup procedure. Once annMCCAOP reservation is set up,

- access to the channel by MCCA enabled mesh STAs is governed by the procedures in Access during MCCAOPs.

- the MCCAOP reservation is advertised according to the procedures in MCCAOP Advertisements .

The schedule is defined by means of the MCCAOP Reservation field defined in Error: Reference source not found. AnnMCCAOP reservation schedules a series of MCCAOPs with a common duration given in the MCCAOP Duration subfield of the MCCAOP Reservation field. This series is started after the first DTIM Beacon following the successful completion of the MCCAOP setup procedure and terminated when thenMCCAOP reservation is torn down.

The reservation defines a regular schedule of MCCAOPs in the DTIM interval of the MCCAOP owner. The number of MCCAOPs in the DTIM interval is given by the value of the MCCAOP Periodicity subfield of the MCCAOP Reservation field. The MCCAOP Offset subfield specifies the offset of the first scheduled MCCAOP of the transmission schedule relative to the beginning of the DTIM interval of the MCCAOP owner. The following MCCAOPs are separated by a time interval with a duration equal to the length of the DTIM period divided by the value in the MCCAOP Periodicity subfield. This schedule is repeated in all DTIM intervals until the reservation is torn down.

An example of an MCCAOP reservation schedule is shown in Example MCCAOP reservation with MCCAOP Periodicity equal to 2. In this particular example, the MCCAOP Periodicity equals two, so that there are two MCCAOPs in each DTIM interval. As further illustrated in the figure, the MCCAOP Offset value indicates the beginning of the first MCCAOP in each DTIM interval.

- Example MCCAOP reservation with MCCAOP Periodicity equal to 2

If a mesh STA adjusts its TBTT, e.g., in response to a TBTT Adjustment Request, it shall adjust the MCCAOP reservations accordingly by modifying the MCCAOP Offset of each MCCAOP reservation.

The set of MCCAOP reservations in which a mesh STA is involved as an MCCAOP owner or an MCCAOP responder and that are used for individually addressed transmissions are referred to as the TX-RX times of this mesh STA. The set of MCCAOP reservations in which a mesh STA is involved as an MCCAOP owner or an MCCAOP responder and that are used for group addressed transmissions are referred to as the Broadcast times of this mesh STA.

An MCCAOP reservation is identified by an MCCAOP Reservation ID. The MCCAOP owner shall select an MCCAOP Reservation ID that is unique among all of its MCCAOP reservations. The MCCAOP Reservation ID and MCCAOP owner’s MAC address uniquely identify the MCCAOP reservation in the mesh BSS. The MCCAOP Reservation ID is an 8-bit unsigned integer. If this MCCAOP Setup Request is for an individually addressed transmission, the MCCAOP Reservation ID is between 0 and 127. If this MCCAOP Setup Request is for a group addressed transmission, the MCCAOP Reservation ID is between 128 and 254. The value 255 is not used to identify a specific MCCAOP reservation but is reserved for usage in MCCAOP Teardown elements as described Clause 9.9a.3.9.

- Neighborhood MCCAOP times at a mesh STA

The MCCAOP reservations of a mesh STA and its neighbors define a set of MCCAOPs that are already being used in the mesh neighborhood of a mesh STA.d. These times are referred to as neighborhood MCCAOP times for the mesh STA.tThus, neighborhood MCCAOP times at a mesh STA include all MCCAOPs for which the mesh STA or one of its neighbors, including neighbors from other MBSSs, is either transmitter or receiver.

A mesh STA shall track at least dot11MaxTrackStates MCCAOP reservations, including its own. The value for Max-Track is advertised in the MCCA Information field of the MCCAOP Advertisements. These tracked MCCAOP reservations are advertised as described in MCCAOP Advertisements . Access during MCCAOPs specifies how to access the medium during the tracked neighborhood MCCAOestimes.

- Interfering times for a mesh STA

Through the Interfering Times Report in its MCCAOP advertisements, a mesh STA reports MCCAOP reservations that its neighbors have advertised in their TX-RX Times Reports and in their Broadcast Times Reports and in which the mesh STA is not involved itself. These times shall not be used for a new MCCAOP with the reporting mesh STA as they may experience interference from the transmissions scheduled to take place during the reported MCCAOPs or may cause interference to them.

The interfering times are directly derived from the TX-RX Times Reports and Broadcast Times Reports of the neighbor mesh STAs. The Interfering Times Report reflects the latest TX-RX Times Reports and Broadcast Times Reports from the neighbor peer mesh STAs.

- MCCA access fraction (MAF)

The MCCA Access Fraction at a mesh STA is the ratio of the time reserved for MCCAOPs in the DTIM interval of this mesh STA  to the duration of the DTIM interval. This parameter is reported in the MCCA Information field in the MCCAOP Advertisements element. The maximum value for the MAF that is allowed at a mesh STA is specified by dot11MAFlimit. The dot11MAFlimit is copied into the MCCA Access Fraction Limit subfield of the MCCA Information field in the MCCAOP Advertisements element described in Clause 7.3.2.106.

The MAF and the MAF Limit may be used to limit the use of MCCA in the mesh neighborhood of a mesh STA, as specified in MCCAOP setup procedure. Before attempting to set up annMCCAOP reservation with a peer mesh STA, a mesh STA is required to ensure that the newnMCCAOP reservation does not cause the MAF of itself or of any of its peer neighbor mesh STAs to exceed the MAF Limit. An MCCAOP Setup Request shall be refused by the intended MCCAOP responder if the MAF limit of its own neighbors is exceeded due to the new setup.

- MCCAOP setup procedure

The setup of an MCCAOP reservation is initiated by the MCCAOP owner, and is accepted, countered, or rejected by the MCCAOP responder. The setup procedure for an MCCAOP reservation is as follows:

- The MCCAOP owner builds a map of neighborhood MCCAOP times in the DTIM interval after hearing Advertisements from all of its neighbor mesh STAs with the MCCA Enabled subfield of the Mesh Capability field in the Mesh Configuration element equal to 1. If no advertisement was heard from a neighbor mesh STA in the last dot11MCCAAdvertPeriodMax DTIM intervals or if the Partial Information bit was equal to 1 in the MCCA Information field of the most recent advertisement, the mesh STA may request an MCCAOP Advertisement from the neighbor mesh STA.

- The MCCAOP owner determines the MCCAOP reservation. The MCCAOP parameters shall be chosen in such a way that they satisfy the following conditions:

- The reservation does not overlap with the neighborhood MCCAOP times of the MCCAOP owner

- The reservation does not overlap with the interfering times of the intended MCCAOP responders

- The reservation does not overlap with Beacon transmission times of itself or one of its neighbor mesh STAs.

- The reservation does not overlap with known Beacon transmission times or HCCA times of neighbor APs.

- The reservation does not cause the MAF limit to be exceeded for either itself or its neighbor mesh STAs.

- The Accept Reservations subfield of the MCCAOP Information field equals 1 in the most recent MCCAOP Advertisements element received from all MCCAOP responders.

- If the conditions in b) are satisfied and if the MCCAOP reservation is intended for individually addressed frames, the MCCAOP owner transmits an MCCAOP Setup Request element to the intended MCCAOP responder with chosen MCCAOP parameters.

- If the conditions in b) are satisfied and if the MCCAOP reservation is intended for group addressed frames, the MCCAOP owner transmits an MCCAOP Setup Request to the intended MCCAOP responders with chosen MCCAOP parameters.

- The MCCAOP responder verifies the following conditions:

- The reservation does not overlap with its neighborhood MCCAOP times excluding those owned by the intended MCCAOP owner.

- The reservation does not overlap with Beacon transmission times of itself or one of its neighbor mesh STAs.

- The reservation does not overlap with known Beacon transmission times or HCCA times of neighbor APs

- The reservation does not cause the MAF limit to be exceeded for itself or its peer mesh STAs.

- The number of reservations in its neighborhood MCCAOP times does not exceed dot11MCCAMaxTrackStates.

- If the conditions in e) are satisfied, the responder shall send an MCCAOP Setup Reply frame to the MCCAOP owner with the MCCA Reply Code field in MCCAOP Setup Reply element equal to 0: Accept, as defined in Table s9.

- If the not all of the conditions in e) are satisfied and the MCCAOP request is intended for individually addressed transmissions, the responder shall send an MCCA Setup Reply frame to the MCCAOP owner which is constructed as follows:

- If one of the conditions in e1-e3 is not satisfied and both conditions e4 and e5 are satisfied, the responder may calculate an alternative MCCAOP reservation and include it in the MCCAOP Reservation field of the MCCAOP Setup Reply element. It sets the MCCA Reply Code field of the MCCAOP Setup Reply element to 1: Reject: MCCAOP Reservation conflict, as defined in Table s9.

- If the condition in e4 is not satisfied, it sets the MCCA Reply Code field of the MCCAOP Setup Reply element to 2: Reject: MAF Limit exceeded, as defined in Table s9.

- If the condition in e4 is satisfied and the condition in e5 is not satisfied, it sets the MCCA Reply Code field of the MCCAOP Setup Reply element to 3: Reject: MCCAOP Track Limit exceeded, as defined in Table s9.

- If not all of the conditions in e) are satisfied and the MCCAOP request is intended for group addressed transmissions, the responder sends an MCCA Setup Reply frame to the MCCAOP owner with the MCCA Reply Code field in MCCAOP Setup Reply element equal to rejection of the MCCAOP Setup Request.

- If the conditions in e) are satisfied and the  and the MCCAOP request is intended for for group addressed transmissions, the responder shall include the reservation into its MCCAOP Advertisements only after the MCCAOP Advertisements from the owner of the broadcast MCCAOP is received.

- If the MCCAOP owner receives an MCCA Setup Reply frame with MCCA Reply Code equal to Accept, the MCCAOP reservation is established. Otherwise, the mesh STA may repeat the MCCAOP setup procedure using a modified MCCAOP Setup Request. If an alternative MCCAOP reservation is included in the MCCAOP Setup Reply, the mesh STA may consider this alternative in its modified MCCAOP Setup Request.

- MCCAOP Advertisements

A mesh STA with dot11MCCAActivated equal to true shall track up to dot11MCCAMaxTrackStates Reservations including its own reservations. If the number of reservations in its neighborhood MCCAOP times is less than dot11MCCAMaxTrackStates, the mesh STA can track and accept additional reservations. In this case, it shall set the Accept Reservations field in the MCCAOP Information field to 1 in the MCCAOP Advertisements element it transmits. If the number of reservations in its neighborhood MCCAOP times equals dot11MCCAMaxTrackStates, the mesh STA cannot track and accept additional reservations. In this case, it shall set the Accept Reservations field in the MCCAOP information field to 0 in the MCCAOP Advertisements element it transmits.

The mesh STA shall advertise the tracked reservations in the TX-RX, Broadcast, and Interfering Times using the MCCAOP Advertisements element.

If a mesh STA reports all tracked reservations in one MCCAOP Advertisement element, its shall set the Last Advertisment subfield of the MCCA Information field in the MCCAOP Advertisement element to 0 and the Advertisement Identifier subfield of the MCCA Information field in the MCCAOP Advertisement to 0. If the mesh STA reports the tracked reservations in a number MCCAOP Advertisements elements, it shall act as follows. If the number of MCCAOP Advertisment elements exceeds 8, it shall set the the Last Advertisment subfield of the MCCA Information field in the MCCAOP Advertisement element to 1 and the Advertisement Identifier subfield of the MCCA Information field in the MCCAOP Advertisement element to 7.  If the number of MCCAOP Advertisement elements used is between 1 and 8, it shall number these MCCAOP Advertisement elements consecutively, and set the Last Advertisement subfield to 0 for the MCCAOP Advertisement element with the highest value for the Advertisement Identifier subfield, and set the Last Advertisement subfield to 1 for the other MCCAOP Advertisement elements.

The mesh STA shall advertise at least one MCCAOP Advertisement element in every dot11MCCAAdvertPeriodMax DTIM intervals. It shall advertise the MCCAOP Advertisement elements that have changed since they were last advertised at least once in every dot11MCCAAdvertPeriodMax DTIM intervals. If a mesh STA receives an Advertisement Request frame it shall advertise all MCCAOP Advertisement elements.

The MCCAOP advertisements shall be transmitted in Beacon frames, Probe Response frames or MCCA Advertisements frames. The advertised times may include:

- TX-RX Times Report:

- All MCCAOP times for individually addressed frames for which the mesh STA is the MCCAOP owner or the MCCAOP responder.

- Broadcast Times Report:

- All MCCAOP times for group addressed frames for which the mesh STA is the MCCAOP owner or the MCCAOP responder.

- Optionally, known Target Beacon Transmission Times for which the mesh STA is either the transmitter or the receiver.

- Optionally, other transmission or reception times of a STA that is collocated with the reporting mesh STA, for example, Beacon or HCCA times of a collocated AP.

- Interfering Times Report:

- TX-RX and broadcast times reported by neighborpeer mesh STAs of the mesh STA excluding those times for which the mesh STA is either the MCCAOP owner or the MCCAOP responder.

- Optionally, TX-RX and broadcast times reported by neighbor non-peer mesh STAs of the mesh STA.

Each of the reports shall consist of a number of reported MCCAOP reservations, as described in Error: Reference source not found. If a mesh STA adjusts its TBTT, e.g., in response to a TBTT Adjustment Request, it shall adjust the Reports accordingly by modifying the MCCAOP Offset of each of the reported MCCAOP reservations. If a mesh STA adjusts its timing offset value with respect to a neighbor mesh STA, as specified in 11C.12.2.2 (Neighbor Offset Protocol), it shall adjust the Reports accordingly by modifying the MCCAOP Offset of each of the reported MCCAOP reservations for which this neighbor mesh STA is the owner.

- MCCAOP teardown

9.9.a.3.9.1 Conditions that trigger an MCCAOP teardown

The MCCAOP owner and the MCCAOP responder may initiate a teardown of an MCCAOP reservation, e.g. when the reservation is no longer needed. A mesh STA shall act as follows to resolve conflicts between MCCAOP reservations in its neighborhood MCCAOP times. If the conflict is caused by overlapping reservations from its TX-RX times and broadcast times, it shall select one of these reservations and initiate a teardown for it. If the conflict is caused by an overlap between a reservation from its TX-RX times or broadcast times, and another reservation from its interfering times, it shall act as follows. It creates a first unsigned integer by inverting the bit order of its MAC address and a second unsigned integer by inverting the bit order of the lowest of the known MAC addresses of the owner and responder(s) of the reservation in the interfering times. If the first unsigned integer is smaller than the second unsigned integer, it shall initiate a teardown of the reservation in its TX-RX or broadcast times. Otherwise, it may initiate a teardown of the reservation in its TX-RX or broadcast times.

9.9.a.3.9.2 MCCAOP teardown procedure

The teardown is initiated by transmitting an MCCA Teardown frame. The MCCAOP Reservation ID field in the MCCAOP Teardown element is set to the MCCAOP Reservation ID of the reservation that it is to be torn down. In case the tear down is initiated by an MCCAOP responder, the MCCAOP Owner field of the MCCAOP Teardown element is set to the MAC address of the MCCAOP owner.

The transmitter of the MCCATeardown frame deletes the reservation after the MCCA Teardown frame has been successfully transmitted. The receiver of the MCCATeardown acts as follows. In case the MCCAOP Reservation ID field corresponds to a reservation for individually addressed transmissions, it deletes the reservation. If the reservation is for group addressed transmissions for which it is the MCCAOP owner, it deletes the reservation if there are no other MCCAOP responders for this reservation.

There are also other conditions that trigger the MCCAOP owner and responder to delete a reservation. An MCCAOP owner shall  delete a reservation for an individually addressed transmission when it has not received an acknowledgement for any frame transmission in the MCCAOPs corresponding to the reservation for greater than dot11MCCAOPtimeout time. An MCCAOP responder shall delete a reservation for individually addressed transmission or group addressed transmissions when it has not received a frame transmission in any of the MCCAOPs corresponding to the reservation for greater than dot11MCCAOPtimeout time.

The MCCAOP owner acts as follows when deleting a reservation

- it stops execuringthe access procedure described in Access by MCCAOP owners at the start of the MCCAOPs corresponding to the reservation that wasndeleted.

- In case the reservation was for individually addressed frames, it stop advertising the MCCAOP reservation in its TX-RX Times Report

- In case the reservation was for group addressed frames, it stop advertising the MCCAOP reservation in its Broadcast Times Report

- The MCCAOP responder acts as follows when deleting a reservation

- it stops execuring the procedure described in 9.9a.3.10.2 during the MCCAOPs corresponding to the reservation that was deleted

- In case the reservation was for individually addressed frames, it stop advertising the MCCAOP reservation in its TX-RX Times Report

- In case the reservation was for group addressed frames, it stop advertising the MCCAOP reservation in its Broadcast Times Report

- Access during MCCAOPs

- Access by MCCAOP owners

At the start of the MCCAOP, the EDCAF of the MCCAOP owner shall set AIFSN[AC] equal to dot11MCCAAIFSN, CWmax[AC] equal to dot11MCCACWmax and CW[AC] equal to dot11MCCACWmin for all ACs. The values of the retry counters are copied to temporary storage and reset to 0. The TXOP limit shall specify a duration value equal to the MCCAOP Duration.

During the MCCAOP the EDCAFs of the ACs operates as specified in HCF contention-based channel access (EDCA), with the following modifications.

During the MCCAOP the EDCAF of each AC shall consider only those frame whose RA matches the MAC address of the MCCA responder.

In cases where the access to the medium is delayed the TXOP limit value shall specify a duration to end no later than the MCCAOP start time plus the MCCAOP Duration.

As specified in Clause 9.9a.3.10.2, neighboring STAs shall not access the WM during an MCCAOP, until they receive a frame from either the MCCAOP owner or the MCCAOP responder, With the exception of truncation of an MCCA TXOP by means of an CF-End standard  EDCA TXOP rules apply for the remainder of the MCCAOP. For HT mesh STAs, these include the reverse direction protocol as specified in Clause 9.15

The MCCAOP owner may adjust the duration of an MCCAOP by setting the Duration/ID field in the frames it transmits. In particular, If an MCCAOP owner has no data to transmit in an MCCAOP corresponding to an MCCAOP reservation that is intended for individually addressed frames, it shall  transmit an individually addressed QoS Null frame during the MCCAOP to end the MCCAOP. If an MCCAOP owner has no data to transmit in an MCCAOP reservation that is intended for group addressed frames, it may transmit a group addressed QoS Null frame during the MCCAOP to end the MCCAOP.

At the end of the MCCAOP, the parameters used by the EDCAF of the MCCAOP owner shall be set to the MIB attribute table dot11EDCATable. The retry counters shall be restored to the values as present at the start of the MCCAOP.

- Access during an MCCAOP by mesh STAs that are not the MCCAOP owner

The MAC of a mesh STA with dot11MCCAActivated is true shall provide a Reservation Allocation Vector (RAV) mechanism to indicate a busy medium from the start of an MCCAOP corresponding to a reservation in its interfering times until the receipt of a frame transmitted by either the MCCAOP owner or the MCCAOP responder. The RAV mechanism is provided in addition to the PHY and virtual CS mechanisms described in 9.2.1. It is different from the virtual CS mechanism in two aspects. Firstly, a mesh STA can be neighbor to multiple ongoing MCCAOPs corresponding to different reservations and the regular NAV setting and updating rules do not suffice to prevent interference during these reservations. Secondly, the virtual CS mechanism is set immediately upon receipt of a frame, whereas the RAV mechanism is based on reservation frames received at some earlier time instant. When either the CS function provided by the PHY, the virtual CS function provided by the MAC via the NAV, or the RAV mechanism indicate a busy medium during an MCCAOP for which the mesh STA is neither the MCCAOP owner nor the MCCAOP responder, the medium shall be considered busy, otherwise it shall be considered idle.

The RAV mechanism maintains an index of future MCCAOPs based on the reservation information that is available in the interfering times of a mesh STA. At the start of each MCCAOP corresponding to a reservation in the interfering times, a RAV is set to indicate a busy medium for the duration of the MCCAOP given in the MCCAOP Duration field of the MCCAOP reservation. The RAV may be thought of as a counter, corresponding to an MCCAOP corresponding to a reservation in the interfering times. The RAV counts down to zero at a uniform rate. When the counter is zero, the RAV indication is that the medium is idle; when nonzero, the indication is busy.

The mesh STA clears the RAV timer upon receipt of a frame from either the MCCAOP owner or responder. The RAV for an MCCAOP is not cleared upon receipt of a frame originating from stations that are not the MCCAOP owner or responder. Since the NAV is set upon receipt of frames with a Duration/ID field, the MCCAOP owner and responder adjust the reservation period of an MCCAOP to their actual traffic needs by the Duration/ID field in the transmitted frame and obtain protection of the frame transmission via the NAV setting.

The RAV mechanism can be represented by a number of counters, where each counter corresponds to one MCCAOP. The number of counters needed at any instant is equal to the number of MCCAOPs at this instant corresponding to reservations in the interfering times of the mesh STA.

- Protection mechanisms

- clause numbering based on TGn adding 9.13.3

- Protection mechanisms for transmissions of HT PPDUs

Insert the following new subclause after 9.13.3.4:

- Protection rules for HT mesh STA in an MBSS

The HT protection field in a mesh STA may be set to no protection mode only if:

- All STAs detected in the primary or the secondary channel are HT STAs, and

- All mesh STA members of this MBSS that are one-hop neighbors of the transmitting mesh STA are either:

- 20/40 MHz HT mesh STAs in a 20/40 MHz MBSS, or

- 20 MHz HT mesh STAs in a 20 MHz MBSS.

The HT protection field in a mesh STA may be set to non-member protection mode only if:

- A non-HT STA is detected in either the primary or the secondary channel or in both the primary and secondary channels, that is not known by the transmitting mesh STA to be a member of this MBSS, and

- All mesh STA members of this MBSS that are one-hop neighbors of the transmitting mesh STA are HT mesh STAs.

The HT protection field in a mesh STA may be set to 20 MHz protection mode only if:

- All STAs detected in the primary and all STAs detected in the secondary channel are HT STAs and All mesh STA members of this MBSS that are one-hop neighbors of the transmitting mesh STA are HT mesh STAs, and

- The MBSS is a 20/40 MHz MBSS, and

- There is at least one 20 MHz HT mesh STA that is one-hop neighbor of the transmitting mesh STA.

The HT protection field in a mesh STA is set to non-HT mixed mode otherwise.

If two peer HT mesh STAs report the same protection mode in HT protection field, the protection mechanisms of the related mode shall be used to protect the transmission between the peer HT mesh STAs.

If an HT mesh STA and its peer HT mesh STA report different protection modes in HT protection field, the following rules shall be used:

- If an HT mesh STA or its peer HT mesh STA reports non-HT mixed mode, the protection mechanisms of non-HT mixed mode shall be used to protect the transmission between the peer HT mesh STAs.

- If an HT mesh STA or its peer HT mesh STA reports non-member protection mode and non-HT mixed mode is not reported by any of these HT mesh STAs, the protection mechanisms of non-member protection mode shall be used to protect the transmission between the peer HT mesh STAs

- If an HT mesh STA or its peer HT mesh STA reports 20 MHz protection mode and neither non-HT mixed mode nor non-member protection mode is reported by any of these HT mesh STAs, the protection mechanisms of 20 MHz protection mode shall be used to protect the transmission between the peer HT mesh STA.

If at least one HT peer mesh STA reports Non Greenfield HT STAs Present, the protection rules related to Non Greenfield HT STAs Present should also be applied to the communication between HT peer mesh STAs.

- Layer management

- Activate MCCA

The following primitives describe how a mesh entity changes its mesh power mode for a mesh peering.

- MLME-ACTIVATEMCCA.request

- Function

This primitive requests that the MAC entity activates MCCA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-ACTIVATEMCCA.request(

MCCAScanDuration,

MAFLimit,

MCCAAdvertPeriodMax,

MCCAMaxTrackStates,

MCCACWmin,

MCCACWmax,

MCCAAIFSN

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MCCAScanDuration | Integer | 0-65535 | Specifies the duration in TUs that the mesh STA shall not initiate or accept MCCAOP Setup Requests. |
| MAFLimit | Integer | 0-255 | Specifies the maximum MCCA access fraction allowed at the mesh STA. This number is always a multiple of (1/255) of the DTIM Interval. |
| MCCAAdvertPeriodMax | Integer | 0-255 | Specifies the maximum interval that a mesh STA with dot11MCCAActivated equal to true waits for an MCCAOP Advertisements update. It is expressed in number of DTIM intervals. |
| MCCAMaxTrackStates | Integer | >=83 | Specifies the total number of MCCAOP reservations that the MAC entity must be able to track. |
| MCCACWmin | Integer | 0-15 | Specifies the value of the minimum size of the contention that the MAC entity uses for channel access during an MCCAOP. |
| MCCACWmax | Integer | 0-63 | Specifies the value of the maximum size of the contention that the MAC entity uses for channel access during an MCCAOP. |
| MCCAAIFSN | Integer | 0-15 | Specifies the value of the AIFSN that the MAC entity uses for channel access during an MCCAOP. |

- When Generated

This primitive is generated by the SME to start the use of MCCA.

- Effect of receipt

This primitive sets dot11MCCAEnabled to true and initializes the MCCA parameters.

- MLME-ACTIVATEMCCA.confirm

- Function

This primitive reports the result of an MLME-MCCAACTIVATE.request.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-ACTIVATEMCCA.confirm(

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS | Indicates the result of the MLME-ACTIVATEMCCA.request. |

- When Generated

This primitive is generated as a result of an MLME-ACTIVATEMCCA.request.

- Effect of Receipt

The SME is notified of the results of the request to start the use of MCCA.

- MBSS Congestion Control

The following primitives describe how a mesh STA transmits congestion control notification to a specified peer MAC entity.

- MLME-MBSSCongestionControl.request

- Function

This primitive requests that a Congestion Control Notification frame be sent to the specified mesh STA.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME- MBSSCongestionControl.request(

peerMAC,

CongestionNotification,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity to which the Congestion Control Notification is sent. |
| CongestionNotification | Congestion Notification element | As defined in Error: Reference source not found | A set of local congestion information generated by the mesh STA. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the SME to request that a Congestion Control Notification frame to be sent to a specified peer entity.

- Effect of receipt

On receipt of this primitive, the MLME constructs a Congestion Control Notification frame. This frame is then scheduled for transmission. The MLME subsequently issues an MLME-MBSSCongestionControl.confirm that reflects the results of this request.

- MLME-MBSSCongestionControl.confirm

- Function

This primitive reports the results of a Congestion Control Notification request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME- MBSSCongestionControl.confirm(

ResultCode

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Result Code | Enumeration | SUCCESS, INVALID_PARAMETERS, NOT_SUPPORTED | Indicates the result of the MLME-MBSSCongestionControl request. |

- When generated

This primitive is generated by the MLME as a result of an MLME-MBSSCongestionControl.request primitive to indicate the result of that request.

- Effect of receipt

The SME is notified of the results of the Congestion Control Notification request.

- MLME-MBSSCongestionControl.indication

- Function

This primitive indicates that a specific peer MAC entity is notifying the local congestion.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME- MBSSCongestionControl.indication(

peerMAC,

CongestionNotification,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the Congestion Control Notification was received. |
| CongestionNotification | Congestion Notification element | As defined in Error: Reference source not found | A set of local congestion information contained in the received Congestion Control Notification frame. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the MLME as a result of the receipt of a Congestion Control Notification frame from a specific peer MAC entity.

- Effect of receipt

The SME is notified of the results of the receipt of the Congestion Control Notification from the specified peer MAC entity. The mesh STA received this primitive subsequently activates the local rate control as described in11C.11 (Intra-mesh congestion control).

- Adjustment procedure

When a mesh STA adjusts its TBTT, it suspends its TSF for a period of time. Specifically, a mesh STA shall operate the following procedure when it adjusts its TBTT.

- Prior to and during the TBTT adjustment procedure, the mesh STA shall set the TBTT Adjusting field in the Mesh Configuration element to 1, in order to announce that it is adjusting its TBTT to avoid beacon collision.

- The mesh STA shall look for the alternative TBTT that does not cause beacon collision. When the mesh STA is in light sleep mode or in deep sleep mode, it shall stay awake state at least for its beacon period and assure that the alternative TBTT does not cause beacon collision.

- The mesh STA shall suspend its TSF timer for a period of time, no longer than half of the Group Delivery Idle Time (defined in Error: Reference source not found) within a single beacon period, to slow its TSF.

- The mesh STA shall adjust the neighbor STA’s TBTT information (see Error: Reference source not found), which are to be contained in the Beacon Timing element, accordingly by subtracting the delay amount.

- The mesh STA shall adjust the MCCAOP reservations accordingly by modifying the MCCAOP Offset of each MCCAOP reservation, when it activates MCCA. (See 9.9a.3.3 (MCCAOP reservations))

- The mesh STA may repeat suspending its TSF timer over multiple beacon periods until its TBTT is set to the timing that does not cause beacon collision.

- Upon completion of the TBTT adjustment, the mesh STA shall change the TBTT Adjusting field in the Mesh Configuration element to 0.

- Upon completion of the TBTT adjustment, the mesh STA shall change the TBTT Adjusting field in the Mesh Configuration element to 0, and update the Report Status subfield in the Report Control field in the Beacon Timing element as described in Error: Reference source not found.

- MCCA use by power saving mesh STA

When a mesh STA operates in MCCA enabled mode and establishes MCCAOPs, the mesh STA shall be in active mode or light sleep mode towards neighbor peer mesh STAs it has established MCCAOPs with.

A scheduled peer service period begins at the MCCAOP start time, if the MCCAOP responder operates in light or deep sleep mode for the MCCAOP owner. The MCCAOP owner is the transmitter during the scheduled peer service period. The MCCAOP responder is the receiver during the scheduled peer service period. Scheduled peer service periods are not used, if the MCCAOP responder operates in active mode for the MCCAOP owner.

The scheduled peer service period continues until it is successfully terminated by acknowledged QoS Null or Mesh Data frame with set EOSP bit from transmitter to receiver as described in Error: Reference source not found.

- (normative) Protocol Implementation Conformance Statement (PICS) proforma

- Abbreviations and special symbols

- General abbreviations for Item and Supported columns

Insert the following to the end of A.2.2:

MP	Mesh Protocol Capability

HWM	HWMP Routing Protocol Capability

- PICS proforma - IEEE Std 802.11, 2007

- IUT configuration

Change the following row of the table in A.4.3 as shown:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF2 | Independent station (not neither an AP nor a mesh STA) | 5.2 | O.1 | Yes No  |

Insert the following row after the “*CF2” row of the table in A.4.3:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF2a | Mesh Station | 5.2.13 (Mesh BSS: The 802.11 wireless mesh network) | O.1 | Yes No  |

- MAC protocol

- MAC protocol capabilities

Insert the following rows to end of table in A.4.4.1:

- numbering of PC39 based on TGn ending with PC37, TGp using PC38

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| *PC39 | Mesh Basic Service Set | Error: Reference source not found | CF2a:M | Yes No  |
| *PC40 | Simultaneous Authentication of Equals | 8.2a (Authentication using a password) | CF2a:M | Yes No  |

- MAC frames

Insert the following rows after the “FT25” row of the table in A.4.4.2:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| FT26 | Mesh Data frames | 7.1.3.1 (Frame Control field), 7.1.3.6.3 (Mesh Control field) | MP2:M | Yes No N/A  |
| FT27 | Mesh Peering Open frame | 7.4.14.2 ( Mesh Peering Open frame format) | MP1.4.1:M | Yes No N/A  |
| FT28 | Mesh Peering Confirm frame | 7.4.14.3 (Mesh Peering Confirm frame format) | MP1.4.2:M | Yes No N/A  |
| FT29 | Mesh Peering Close frame | 7.4.14.4 (Mesh Peering Close frame format) | MP1.4.3:M | Yes No N/A  |
| FT30 | Mesh Link Metric Request frame | 7.4.15.2 (Mesh Link Metric Request frame format) | MP11.1:M | Yes No N/A  |
| FT31 | Mesh Link Metric Report frame | 7.4.15.3 (Mesh Link Metric Report frame format) | MP11.2:M | Yes No N/A  |
| FT32 | Mesh Path Selection frame | 7.4.16.2 ( HWMP Mesh Path Selection frame format) | HWM1:M | Yes No N/A  |
| FT33 | Gate Announcement frame | 7.4.17.2 ( Gate Announcement frame format), Error: Reference source not found | MP13:M | Yes No N/A  |
| FT34 | Congestion Control Notification frame | 7.4.18.2 (Congestion Control Notification frame format), Error: Reference source not found | MP6.2:M | Yes No N/A  |
| FT35 | MCCA Setup Request frame | 7.4.18.3 (MCCA Setup Request frame format) | MP5:M | Yes No N/A  |
| FT36 | MCCA Setup Reply frame | 7.4.18.4 (MCCA Setup Reply frame format) | MP5:M | Yes No N/A  |
| FT37 | MCCAOP Advertisement Request frame | 7.4.18.5 (MCCAOP Advertisement Request frame format) | MP5:M | Yes No N/A  |
| FT38 | MCCAOP Advertisements frame | 7.4.18.6 (MCCAOP Advertisements frame format) | MP5:M | Yes No N/A  |
| FT39 | MCCA Teardown frame | 7.4.18.7 (MCCA Teardown frame format) | MP5:M | Yes No N/A  |
| FT40 | TBTT Adjustment Request frame | 7.4.18.8 (TBTT Adjustment Request frame format) | MP4.2.3:M | Yes No N/A  |
| FT41 | TBTT Adjustment Response frame | 7.4.18.9 (TBTT Adjustment Response frame format) | MP4.2.3:M | Yes No N/A  |
| FT42 | Proxy Update frame | 7.4b.2.2 (Proxy Update frame format) | MP12:M | Yes No N/A  |
| FT43 | Proxy Update Confirmation frame | 7.4b.2.3 (Proxy Update Confirmation frame format ) | MP12:M | Yes No N/A  |

Insert the following rows after the “FR25” row of the table in A.4.4.2:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| FR26 | Mesh Data frames | 7.1.3.1 (Frame Control field), 7.1.3.6.3 (Mesh Control field) | MP2:M | Yes No N/A  |
| FR27 | Mesh Peering Open frame | 7.4.14.2 ( Mesh Peering Open frame format) | MP1.4.1:M | Yes No N/A  |
| FR28 | Mesh Peering Confirm frame | 7.4.14.3 (Mesh Peering Confirm frame format) | MP1.4.2:M | Yes No N/A  |
| FR29 | Mesh Peering Close frame | 7.4.14.4 (Mesh Peering Close frame format) | MP1.4.3:M | Yes No N/A  |
| FR30 | Mesh Link Metric Request frame | 7.4.15.2 (Mesh Link Metric Request frame format) | MP11.1:M | Yes No N/A  |
| FR31 | Mesh Link Metric Report frame | 7.4.15.3 (Mesh Link Metric Report frame format) | MP11.2:M | Yes No N/A  |
| FR32 | Mesh Path Selection frame | 7.4.16.2 ( HWMP Mesh Path Selection frame format) | HWM1:M | Yes No N/A  |
| FR33 | Gate Announcement frame | 7.4.17.2 ( Gate Announcement frame format), Error: Reference source not found | MP13:M | Yes No N/A  |
| FR34 | Congestion Control Notification frame | 7.4.18.2 (Congestion Control Notification frame format), Error: Reference source not found | MP6.2:M | Yes No N/A  |
| FR35 | MCCA Setup Request frame | 7.4.18.3 (MCCA Setup Request frame format) | MP5:M | Yes No N/A  |
| FR36 | MCCA Setup Reply frame | 7.4.18.4 (MCCA Setup Reply frame format) | MP5:M | Yes No N/A  |
| FR37 | MCCAOP Advertisement Request frame | 7.4.18.5 (MCCAOP Advertisement Request frame format) | MP5:M | Yes No N/A  |
| FR38 | MCCAOP Advertisements frame | 7.4.18.6 (MCCAOP Advertisements frame format) | MP5:M | Yes No N/A  |
| FR39 | MCCA Teardown frame | 7.4.18.7 (MCCA Teardown frame format) | MP5:M | Yes No N/A  |
| FR40 | TBTT Adjustment Request frame | 7.4.18.8 (TBTT Adjustment Request frame format) | MP4.2.3:M | Yes No N/A  |
| FR41 | TBTT Adjustment Response frame | 7.4.18.9 (TBTT Adjustment Response frame format) | MP4.2.3:M | Yes No N/A  |
| FR42 | Proxy Update frame | 7.4b.2.2 (Proxy Update frame format) | MP12:M | Yes No N/A  |
| FR43 | Proxy Update Confirmation frame | 7.4b.2.3 (Proxy Update Confirmation frame format ) | MP12:M | Yes No N/A  |

- MAC addressing functions

Insert the following new rows to end of the table in A.4.4.4:

- numbering of AD6 based on TGp ending with AD5

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| AD6 | 4 Address Format | 7.1.2 (General frame format), 7.1.3.1 (Frame Control field), 7.1.3.6.3 (Mesh Control field) | MP1:M | Yes No N/A  |
| AD7 | 6 Address Format | 7.1.2 (General frame format), 7.1.3.1 (Frame Control field), 7.1.3.6.3 (Mesh Control field) | MP1:M | Yes No N/A  |

- QoS base functionality

Change the “QB” row of the table in A.4.14 as shown:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| QB5 | Automatic power-save delivery (APSD) | 7.4.2, 11.2.1 | (CF1 AND CF12):O (CF2 AND CF12):O | Yes No N/A  |

- QoS enhanced distributed channel access (EDCA)

Change the “QD7” row of the table in A.4.15 as shown:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| QD7 | Power management in an infrastructure BSS or in an IBSS | 11.2 | (CF1 AND CF12):O (CF2 AND CF12):O | Yes No N/A  |

- QoS hybrid coordination function (HCF) controlled channel access (HCCA)

Change the table in A.4.16 as shown:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| QP1 | Traffic specification (TSPEC) and associated frame formats | 7.4.2 | (CF1 AND CF12):M (CF2 AND CF12):M | Yes No N/A  |
| QP2 | HCCA rules | 9.1.3.2, 9.9.2, 9.9.2.1-9.9.2.3 | (CF1 AND CF12):M (CF2 AND CF12):M | Yes No N/A  |
| QP3 | HCCA schedule generation and management | 9.9.3 | (CF1 AND CF12):M | Yes No N/A  |
| QP4 | HCF frame exchange sequence | 9.9.1, 9.3.2 | (CF1 AND CF12):M (CF2 AND CF12):M | Yes No N/A  |
| QP5 | Traffic stream (TS) management | 11.4 | (CF1 AND CF12):M (CF2 AND CF12):M | Yes No N/A  |
| QP6 | Minimum TSPEC parameter set | 9.9.3 | (CF1 AND CF12):M (CF2 AND CF12):M | Yes No N/A  |
| QP7 | Power management in an infrastructure BSS | 11.2.1.4, 11.2.1.5, 11.2.1.6, 11.2.1.7, 11.2.1.8, 11.2.1.9, 11.2.1.10 | (CF1 AND CF12):M (CF2 AND CF12):M | Yes No N/A  |

- numbering of following subclause based on TGu ending with A.4.22. A.4.23 is the first unused subclause.

Insert the following new subclause after the A.4.22:

- Mesh Protocol Capabilities

- General Mesh Support

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| *MP1 | Support of Mesh Capability | 5.2.13 (Mesh BSS: The 802.11 wireless mesh network) | PC39:M | Yes No N/A  |
| MP1.1 | Announcement of mesh profile and supplemental information for the mesh discovery | Error: Reference source not found, Error: Reference source not found | MP1:M | Yes No N/A  |
| MP1.2 | Candidate peer mesh STA determination | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP1.3 | Becoming a member of a mesh BSS | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP1.4 | Mesh Peering Management | Error: Reference source not found | MP1:M | Yes No N/A  |
| *MP1.4.1 | Processing of Mesh Peering Open frame | Error: Reference source not found | MP1.4:M | Yes No N/A  |
| *MP1.4.2 | Processing of Mesh Peering Confirm frame | Error: Reference source not found | MP1.4:M | Yes No N/A  |
| *MP1.4.3 | Processing of Mesh Peering Close frame | Error: Reference source not found | MP1.4:M | Yes No N/A  |
| MP1.4.4 | MPM Finite State Machine | Error: Reference source not found | MP1.4:M | Yes No N/A  |
| MP1.5 | Authenticated Mesh Peering Exchange | Error: Reference source not found | MP1:O | Yes No N/A  |
| MP1.5.1 | Protected Mesh Peering Management frame processing | Error: Reference source not found | MP1.5:M | Yes No N/A  |
| MP1.5.2 | AMPE Finite State Machine | Error: Reference source not found | MP1.5:M | Yes No N/A  |
| MP1.5.3 | MGTK distribution | Error: Reference source not found | MP1.5:M | Yes No N/A  |
| *MP2 | Mesh Control field | 7.1.3.6.3 (Mesh Control field) | MP1:M | Yes No N/A  |
| MP2.1 | Address Extension recognition | 7.1.3.6.3 (Mesh Control field) | MP2:M | Yes No N/A  |
| MP2.2 | Decrement of Mesh frame TTL | 7.1.3.6.3 (Mesh Control field) | MP2:M | Yes No N/A  |
| MP3 | Mesh STA beaconing | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP4 | Mesh STA synchronization | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP4.1 | Neighbor Offset Protocol | Error: Reference source not found | MP4:M | Yes No N/A  |
| MP4.2 | Mesh Beacon Collision Avoidance | Error: Reference source not found | MP4:O | Yes No N/A  |
| MP4.2.1 | Beacon timing advertisement | Error: Reference source not found | MP4.2:M | Yes No N/A  |
| MP4.2.2 | TBTT selection | Error: Reference source not found | MP4.2:M | Yes No N/A  |
| *MP4.2.3 | TBTT adjustment | Error: Reference source not found | MP4.2:M | Yes No N/A  |
| MP4.2.4 | Frame transmission across reported TBTT | Error: Reference source not found | MP4.2:O | Yes No N/A  |
| MP4.2.5 | Delayed beacon transmission | Error: Reference source not found | MP4.2:O | Yes No N/A  |
| *MP5 | MCCA | 9.9a.3 (MCF coordinated channel access (MCCA)) | MP1:O | Yes No N/A  |
| MP5.1 | MCCAOP Advertisement | 9.9a.3.8 (MCCAOP Advertisements) | MP5:M | Yes No N/A  |
| MP5.2 | Neighbor MCCAOP Recognition | 9.9a.3.4 (Neighborhood MCCAOP times at a mesh STA) - 9.9a.3.6 (MCCA access fraction (MAF)) | MP5:M | Yes No N/A  |
| MP5.3 | MCCAOP Setup | 9.9a.3.7 (MCCAOP setup procedure) | MP5:M | Yes No N/A  |
| MP5.4 | Access during MCCAOPs | 9.9a.3.10 (Access during MCCAOPs) | MP5:M | Yes No N/A  |
| MP6 | Intra Mesh Congestion Control | Error: Reference source not found | MP1:O | Yes No N/A  |
| MP6.1 | Local congestion monitoring and detection | Error: Reference source not found | MP6:M | Yes No N/A  |
| *MP6.2 | Congestion Control signaling | Error: Reference source not found | MP6:M | Yes No N/A  |
| MP6.3 | Local rate control | Error: Reference source not found | MP6:M | Yes No N/A  |
| MP7 | MBSS channel selection | 11.9.7 (Selecting and advertising a new channel), 11.9a.3 (Selecting and advertising a new channel and/or regulatory class) | MP1:M | Yes No N/A  |
| MP8 | Mesh power save operation (operation in light or deep sleep mode) | Error: Reference source not found | MP1:O | Yes No N/A  |
| MP8.1 | Mesh power modes | Error: Reference source not found, Error: Reference source not found | MP8:M | Yes No N/A  |
| MP8.2 | STA power state transitions | Error: Reference source not found | MP8:M | Yes No N/A  |
| MP8.3 | Mesh Awake Window and frame transmission | Error: Reference source not found | MP8:M | Yes No N/A  |
| MP9 | Mesh power save support | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP9.1 | TIM transmission | Error: Reference source not found | MP9:M | Yes No N/A  |
| MP9.2 | Link specific mesh power modes | Error: Reference source not found | MP9:M | Yes No N/A  |
| MP9.3 | Non-peer power modes | Error: Reference source not found | MP9:M | Yes No N/A  |
| MP9.4 | Frame transmission to mesh STA in light sleep mode | Error: Reference source not found, Error: Reference source not found | MP9:M | Yes No N/A  |
| MP9.5 | Frame transmission to mesh STA in deep sleep mode | Error: Reference source not found, Error: Reference source not found | MP9:M | Yes No N/A  |
| MP10 | Airtime link metric computation | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP11 | Link metric reporting | Error: Reference source not found | MP1:M | Yes No N/A  |
| *MP11.1 | Link metric request | Error: Reference source not found | MP11:M | Yes No N/A  |
| *MP11.2 | Link metric report | Error: Reference source not found | MP11:M | Yes No N/A  |
| *MP12 | Proxy operation | Error: Reference source not found | MP1:O | Yes No N/A  |
| *MP13 | Gate announcement | Error: Reference source not found | MP1:O | Yes No N/A  |
| MP14 | MSDU/MMPDU forwarding | Error: Reference source not found | MP1:O | Yes No N/A  |
| MP14.1 | Individually addressed MSDU forwarding | Error: Reference source not found | MP14:M | Yes No N/A  |
| MP14.2 | Group addressed MSDU forwarding | Error: Reference source not found | MP14:M | Yes No N/A  |
| MP14.3 | MMPDU forwarding | Error: Reference source not found | MP14:M | Yes No N/A  |

- HWMP Routing Protocol Capabilities

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| *HWM1 | Hybrid Wireless Mesh Protocol | Error: Reference source not found | MP1:M | Yes No N/A  |
| HWM1.1 | On-demand path selection | Error: Reference source not found | HWM1:M | Yes No N/A  |
| HWM1.2 | Proactive tree building | Error: Reference source not found | HWM1:M | Yes No N/A  |
| HWM1.3 | Forwarding | Error: Reference source not found | HWM1:O | Yes No N/A  |

- (normative) ASN.1 encoding of the MAC and PHY MIB

Insert the following objects to the Major sections:

-- dot11MeshStationConfigTable 						::= { dot11smt 18 }

-- dot11MeshHWMPConfigTable 						::= { dot11smt 19 }

Change the “Dot11StationConfigEntry” of the “dotStationConfigTable” as follows:

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID 								MacAddress,

dot11MediumOccupancyLimit 								INTEGER,

dot11CFPollable 								TruthValue,

dot11CFPeriod								INTEGER,

dot11CFPMaxDuration 								INTEGER,

dot11AuthenticationResponseTimeOut 								Unsigned32,

dot11PrivacyOptionImplemented 								TruthValue,

dot11PowerManagementMode 								INTEGER,

dot11DesiredSSID								OCTET STRING,

dot11DesiredBSSType 								INTEGER,

dot11OperationalRateSet 								OCTET STRING,

dot11BeaconPeriod 								INTEGER,

dot11DTIMPeriod 								INTEGER,

dot11AssociationResponseTimeOut 								Unsigned32,

dot11DisassociateReason 								INTEGER,

dot11DisassociateStation 								MacAddress,

dot11DeauthenticateReason 								INTEGER,

dot11DeauthenticateStation 								MacAddress,

dot11AuthenticateFailStatus 								INTEGER,

dot11AuthenticateFailStation 								MacAddress,

dot11MultiDomainCapabilityImplemented 								TruthValue,

dot11MultiDomainCapabilityEnabled 								TruthValue,

dot11CountryString 								OCTET STRING,

dot11SpectrumManagementImplemented 								TruthValue,

dot11SpectrumManagementRequired 								TruthValue,

dot11RSNAOptionImplemented 								TruthValue,

dot11RSNAPreauthenticationImplemented 								TruthValue,

dot11RegulatoryClassesImplemented	 							TruthValue,

dot11RegulatoryClassesRequired 								TruthValue,

dot11QosOptionImplemented 								TruthValue,

dot11ImmediateBlockAckOptionImplemented 								TruthValue,

dot11DelayedBlockAckOptionImplemented 								TruthValue,

dot11DirectOptionImplemented 								TruthValue,

dot11APSDOptionImplemented 								TruthValue,

dot11QAckOptionImplemented 								TruthValue,

dot11QBSSLoadOptionImplemented 								TruthValue,

dot11QueueRequestOptionImplemented 								TruthValue,

dot11TXOPRequestOptionImplemented 								TruthValue,

dot11MoreDataAckOptionImplemented 								TruthValue,

dot11AssociatedinNQBSS 								TruthValue,

dot11DLSAllowdInQBSS 								TruthValue,

dot11DLSAllowed 								TruthValue,

dot11AssociateStation 								MacAddress,

dot11AssociateID 								INTEGER,

dot11AssociateFailStation 								MacAddress,

dot11AssociateFailStatus 								INTEGER,

dot11ReassociateStation 								MacAddress,

dot11ReassociateID 								INTEGER,

dot11ReassociateFailStation 								MacAddress,

dot11ReassociateFailStatus 								INTEGER,

dot11RadioMeasurementCapable 								TruthValue,

dot11RadioMeasurementEnabled 								TruthValue,

dot11RRMMeasurementProbeDelay 								INTEGER,

dot11RRMMeasurementPilotPeriod 								INTEGER,

dot11RRMLinkMeasurementEnabled 								TruthValue,

dot11RRMNeighborReportEnabled 								TruthValue,

dot11RRMParallelMeasurementsEnabled								TruthValue,

dot11RRMRepeatedMeasurementsEnabled								TruthValue,

dot11RRMBeaconPassiveMeasurementEnabled								TruthValue,

dot11RRMBeaconActiveMeasurementEnabled								TruthValue,

dot11RRMBeaconTableMeasurementEnabled 								TruthValue,

dot11RRMBeaconMeasurementReportingConditionsEnabled								TruthValue,

dot11RRMFrameMeasurementEnabled 								TruthValue,

dot11RRMChannelLoadMeasurementEnabled 								TruthValue,

dot11RRMNoiseHistogramMeasurementEnabled 								TruthValue

dot11RRMStatisticsMeasaurementEnabled 								TruthValue,

dot11RRMLCIMeasurementEnabled 								TruthValue,

dot11RRMLCIAzimuthEnabled 								TruthValue,

dot11RRMTransmitStreamCategoryMeasurementEnabled								TruthValue,

dot11RRMTriggeredTransmitStreamCategoryMeasurementEnabled TruthValue,

dot11RRMAPChannelReportEnabled 								TruthValue,

dot11RRMMIBEnabled 								TruthValue,

dot11RRMMaxMeasurementDuration 								Unsigned32,

dot11RRMNonOperatingChannelMaxMeasurementDuration								Unsigned32,

dot11RRMMeasurementPilotTransmissionInformationEnabled TruthValue,

dot11RRMMeasurementPilotCapability 								Unsigned32,

dot11RRMNeighborReportTSFOffsetEnabled 								TruthValue,

dot11RRMRCPIMeasurementEnabled 								TruthValue,

dot11RRMRSNIMeasurementEnabled 								TruthValue,

dot11RRMBSSAverageAccessDelayEnabled 								TruthValue,

dot11RRMBSSAvailableAdmissionCapacityEnabled 								TruthValue,

dot11RRMAntennaInformationEnabled 								TruthValue,

dot11FastBSSTransitionImplemented 								TruthValue,

dot11LCIDSEImplemented 								TruthValue,

dot11LCIDSERequired 								TruthValue,

dot11DSERequired 								TruthValue,

dot11ExtendedChannelSwitchEnabled 								TruthValue,

dot11RSNAProtectedManagementFramesEnabled 								TruthValue,

dot11RSNAUnprotectedManagementFramesAllowed 								TruthValue,

dot11AssociationPingResponseTimeout 								Unsigned32,

dot11AssociationMaximumPingAttempts 								INTEGER,

dot11HighThroughputOptionImplemented 								TruthValue,

dot11RSNAPBACRequired 								TruthValue},

dot11MeshActivated								TruthValue }

Insert “dot11MeshActivated” to the end of “dotStationConfigTable”:

dot11MeshActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This is a control variable.

It is written by an external management entity.

This attribute shall specify whether or not mesh services are supported by a station.”

::= { dot11StationConfigEntry 94 }

Change the dot11OperationEntity as shown:

dot11OperationEntry ::=

SEQUENCE {dot11MACAddress 							MacAddress,

dot11RTSThreshold 						INTEGER,

dot11ShortRetryLimit 						INTEGER,

dot11LongRetryLimit 						INTEGER,

dot11FragmentationThreshold 						INTEGER,

dot11MaxTransmitMSDULifetime 						Unsigned32,

dot11MaxReceiveLifetime 						Unsigned32,

dot11ManufacturerID 						DisplayString,

dot11ProductID 						DisplayString,

dot11CAPLimit 						INTEGER,

dot11HCCWmin 						INTEGER,

dot11HCCWmax 						INTEGER,

dot11HCCAIFSN 						INTEGER,

dot11ADDBAResponseTimeout 						INTEGER,

dot11ADDTSResponseTimeout 						INTEGER,

dot11ChannelUtilizationBeaconInterval 						INTEGER,

dot11ScheduleTimeout 						INTEGER,

dot11DLSResponseTimeout 						INTEGER,

dot11QAPMissingAckRetryLimit 						INTEGER,

dot11EDCAAveragingPeriod 						INTEGER, }

dot11SAERetransPeriod						INTEGER,

dot11SAEAntiCloggingThreshold						INTEGER,

dot11SAESync						INTEGER }

Insert the following to the end of dot11OperationEntry Table in Annex D:

dot11SAERetransPeriod OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when establishing or becoming a member of a BSS.

This object specifies the initial retry timeout, in millisecond units, used by the SAE authentication and key establishment protocol."

DEFVAL { 40 }

::= { dot11OperationEntry 21}

dot11SAEAntiCloggingThreshold OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This object specifies the maximum number of SAE protocol instances allowed to simultaneously be in either Commit or Confirmed state."

DEFVAL { 5 }

::= { dot11OperationEntry 22}

dot11SAESync OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This object specifies the maximum number of synchronization errors that are allowed to happen prior to disassociation of the offending SAE peer."

DEFVAL { 5 }

::= { dot11OperationEntry 23}

Change the “dot11RSNAConfigNumberOfPTKSAReplayCounters” in Annex D as follows:

dot11RSNAConfigNumberOfPTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Specifies the number of PTKSA replay counters per association:

0 –> 1 replay counters,

1 -> 2 replay counters,

2 –> 4 replay counters,

3 –> 16 replay counters"

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 18 }

Change the “dot11RSNAConfigNumberOfGTKSAReplayCounters” in Annex D as follows:

dot11RSNAConfigNumberOfGTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Specifies the number of GTKSA replay counters per association:

0 –> 1 replay counter,

1 –> 2 replay counters,

2 –> 4 replay counters,

3 –> 16 replay counters"

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 29 }

Change the “dot11RSNAConfigNumberOfSTKSAReplayCounters” in Annex D as follows:

dot11RSNAConfigNumberOfSTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Specifies the number of STKSA replay counters per association:

0 -> 1 replay counter,

1 -> 2 replay counters,

2 -> 4 replay counters,

3 -> 16 replay counters"

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 37 }

Insert the following dot11 MeshSTATable to the end of Annex D:

**********************************************************************

* dot11MeshSTAConfig TABLE

**********************************************************************

dot11MeshStationConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MeshStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Mesh Station Configuration attributes. In tabular form to allow for multiple instances on an agent."

dot11MeshStationConfigEntry  OBJECT-TYPE

SYNTAX Dot11MeshStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MeshStationConfigTable. It is possible for there to be multiple IEEE 802.11 interfaces on one agent, each with its unique MAC address. The relationship between an IEEE 802.11 interface and an interface in the context of the Internet-standard MIB is one-to-one. As such, the value of an ifIndex object instance can be directly used to identify corresponding instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an ifEntry. Interface tables in this MIB module are indexed by ifIndex."

INDEX { ifIndex }

::= { dot11MeshStationConfigTable 1 }

Dot11MeshStationConfigEntry ::=

SEQUENCE {

dot11MeshID							OCTET STRING,

dot11MeshNumberOfPeerings							INTEGER,

dot11MeshAcceptingAdditionalPeerings							TruthValue,

dot11MeshConnectedToMeshGate							TruthValue,

dot11MeshSecurityActivated							TruthValue,

dot11MeshActiveAuthenticationProtocol							INTEGER,

dot11MeshActivePeeringProtocol							INTEGER,

dot11MeshConfigGroupUpdateCount							INTEGER,

dot11MeshActivePathSelectionProtocol							INTEGER,

dot11MeshActivePathSelectionMetric							INTEGER,

dot11MeshForwarding							TruthValue,

dot11MeshMaxRetries							INTEGER,

dot11MeshRetryTimeout							INTEGER,

dot11MeshTTL							INTEGER,

dot11MeshGateAnnouncementProtocol 							TruthValue,

dot11MeshGateAnnouncementInterval							INTEGER,

dot11MeshConfirmTimeout							INTEGER,

dot11MeshHoldingTimeout							INTEGER,

dot11MeshActiveCongestionControlMode							INTEGER,

dot11MeshActiveSynchronizationProtocol							INTEGER,

dot11MeshNbrOffsetMaxNeighbor							INTEGER,

dot11MBCAActivated							TruthValue,

dot11MeshBeaconTimingReportInterval							INTEGER,

dot11MeshBeaconTimingReportMaxNum							INTEGER,

dot11MeshDelayedBeaconTxInterval							INTEGER,

dot11MeshDelayedBeaconTxMaxDelay							INTEGER,

dot11MeshDelayedBeaconTxMinDelay							INTEGER,

dot11MeshAverageBeaconFrameDuration							INTEGER,

dot11MeshSTAMissingAckRetryLimit							INTEGER,

dot11MCCAImplemented 							TruthValue,

dot11MCCAActivated 							TruthValue,

dot11MAFlimit 							INTEGER,

dot11MCCAScanDuration							INTEGER,

dot11MCCAAdvertPeriodMax							INTEGER,

dot11MCCAMaxTrackStates OBJECT-TYPE							INTEGER,

dot11MCCAOPtimeout							INTEGER,

dot11MCCACWmin							INTEGER,

dot11MCCACWmax							INTEGER,

dot11MCCAAIFSN							INTEGER

}

dot11MeshID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request.

This attribute reflects the Mesh ID configured in this entity."

dot11MeshNumberOfPeerings OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object shall indicate the number of mesh peering currently maintained by the STA. This value is reflected in the Number of Peerings subfield in the Mesh Formation Info field in the Mesh Configuration element."

dot11MeshAcceptingAdditionalPeerings OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object shall specify whether or not the station is willing to accept additional peerings. This value is reflected in the Accepting Additional Mesh Peerings subfield in the Mesh Capability field in the Mesh Configuration element."

dot11MeshConnectedToMeshGate OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This object shall specify whether or not the station has a mesh path to a mesh STA collocated with a mesh gate. This value is reflected in the Connected to Mesh Gate subfield in the Mesh Formation Info field in the Mesh Configuration element."

dot11MeshSecurityActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify whether or not the station is security enabled."

dot11MeshActiveAuthenticationProtocol OBJECT-TYPE

SYNTAX INTEGER { Null (0), SAE (1), VendorSpecific (255) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request.

This attribute shall specify the active authentication protocol."

DEFVAL { Null }

dot11MeshActivePeeringProtocol OBJECT-TYPE

SYNTAX INTEGER { MPM (0), AMPE (1), VendorSpecific (255) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request.

This attribute shall specify the active mesh peering protocol."

DEFVAL { MPM }

dot11MeshConfigGroupUpdateCount OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

The number of times Mesh Group Key Inform frame in Mesh Group Key Handshake will be retried per Mesh Group Key Handshake attempt."

DEFVAL { 3 }

dot11MeshActivePathSelectionProtocol OBJECT-TYPE

SYNTAX INTEGER { HWMP (0), VendorSpecific (255) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request.

This attribute shall specify the active path selection protocol."

DEFVAL { HWMP }

dot11MeshActivePathSelectionMetric OBJECT-TYPE

SYNTAX INTEGER { AirtimeLinkMetric (0), VendorSpecific (255) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request.

This attribute shall specify the active path selection metric."

DEFVAL { AirtimeLinkMetric }

dot11MeshForwarding OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the ability of a mesh STA to forward MSDUs."

DEFVAL { true }

dot11MeshMaxRetries OBJECT-TYPE

SYNTAX INTEGER

(0..16)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute specifies the maximum number of Mesh Peering Open retries that can be sent to establish a new mesh peering instance in a mesh BSS."

DEFVAL { 2 }

dot11MeshRetryTimeout OBJECT-TYPE

SYNTAX INTEGER

(1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute specifies the initial retry timeout, in millisecond units, used by the Mesh Peering Open message."

DEFVAL { 40 }

dot11MeshTTL OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the value of Mesh TTL field set at a source mesh STA."

DEFVAL { 31 }

dot11MeshGateAnnouncementProtocol OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify whether or not the mesh STA, which is collocated with a mesh gate, is using the Gate Announcement protocol."

DEFVAL { false }

dot11MeshGateAnnouncementInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

The Gate Announcement Interval is the number of seconds between the transmission of two Gate Announcements."

DEFVAL { 10 }

dot11MeshConfirmTimeout OBJECT-TYPE

SYNTAX INTEGER

(1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute specifies the initial retry timeout, in millisecond units, used by the Mesh Peering Open message."

DEFVAL { 40 }

dot11MeshHoldingTimeout OBJECT-TYPE

SYNTAX INTEGER

(1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute specifies the confirm timeout, in millisecond units, used by the mesh peering management to close a mesh peering."

DEFVAL { 40 }

dot11MeshActiveCongestionControlMode OBJECT-TYPE

SYNTAX INTEGER {

Null (0),

CongestionControlSignaling (1),

VendorSpecific (255) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request.

This attribute shall specify the active congestion control protocol."

DEFVAL { Null }

dot11MeshActiveSynchronizationProtocol OBJECT-TYPE

SYNTAX INTEGER { NeighborOffsetProtocol (0), VendorSpecific (255) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request.

This attribute shall specify the active synchronization protocol."

DEFVAL { NeighborOffsetProtocol }

dot11MeshNbrOffsetMaxNeighbor OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the maximum number of neighbor STAs with which the mesh STA maintains synchronization using the neighbor offset protocol."

DEFVAL { 16 }

dot11MBCAActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request.

This attribute shall specify whether or not the station activates Mesh Beacon Collision Avoidance mechanisms."

DEFVAL { false }

dot11MeshBeaconTimingReportInterval OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify when the Beacon Timing element is present in Beacon frames. Beacon Timing element is present when the DTIM Count value in the Beacon frame is zero or equal to an integer multiple of the set value."

DEFVAL { 4 }

dot11MeshBeaconTimingReportMaxNum OBJECT-TYPE

SYNTAX INTEGER (0..50)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the maximum number of Beacon Timing Information field contained in a Beacon Timing element in the transmitting Beacon frames."

DEFVAL { 16 }

dot11MeshDelayedBeaconTxInterval OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the interval of the delayed beacon transmission, for MBCA purpose. The value 0 indicates that the delayed beacon transmission is disabled."

DEFVAL { 0 }

dot11MeshDelayedBeaconTxMaxDelay OBJECT-TYPE

SYNTAX INTEGER (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the maximum delay time from a TBTT for the delayed beacon transmission, for MBCA purpose. The value is expressed in units of microseconds."

DEFVAL { 0 }

dot11MeshDelayedBeaconTxMinDelay OBJECT-TYPE

SYNTAX INTEGER (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the minimum delay time from a TBTT for the delayed beacon transmission, for MBCA purpose. The value is expressed in units of microseconds."

DEFVAL { 0 }

dot11MeshAverageBeaconFrameDuration OBJECT-TYPE

SYNTAX INTEGER (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the expected averaged duration of last 16 Beacon frames received by the neighboring mesh STAs. The value is expressed in units of microseconds."

dot11MeshSTAMissingAckRetryLimit OBJECT-TYPE

SYNTAX INTEGER (0..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute indicates the number of times the mesh STA may retry a frame for which it does not receive an ACK for a STA in power-save mode after the mesh STA does not receive an ACK to a directed MPDU sent with the EOSP set to 1."

dot11MCCAImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute shall specify whether or not the MCCA is implemented in this station."

dot11MCCAActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify whether or not the station is MCCA enabled."

DEFVAL { false }

dot11MAFlimit OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the maximum MCCA Access Fraction allowed at the mesh STA. This number expresses a multiple of (1/255)."

DEFVAL { 128 }

dot11MCCAScanDuration OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Specifies the duration in TUs after the activation of MCCA that the mesh STA shall not initiate or accept MCCAOP Setup Requests."

DEFVAL { 1000 }

dot11MCCAAdvertPeriodMax OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the maximum interval that a mesh STA with dot11MCCAActivated equal to true waits for an MCCAOP Advertisements update. It is expressed in number of DTIM intervals."

DEFVAL { 1 }

dot11MCCAMaxTrackStates OBJECT-TYPE

SYNTAX INTEGER (83..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the total number of MCCAOP reservations that the MAC entity must be able to track."

DEFVAL { 83 }

dot11MCCAOPtimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute specifies the timeout value for an MCCAOP teardown. It is expressed in TU."

DEFVAL { 10000 }

dot11MCCACWmin OBJECT-TYPE

SYNTAX INTEGER (0..aCWmin)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the value of the minimum size of the window that shall be used by the mesh STA during the MCCAOP for which it is the MCCAOP owner for generating a random number for the backoff."

DEFVAL { 0 }

dot11MCCACWmax OBJECT-TYPE

SYNTAX INTEGER (0..aCWmax)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the value of the maximum size of the window that shall be used by the mesh STA during the MCCAOP for which it is the MCCAOP owner for generating a random number for the backoff. The value of this attribute shall be such that it could always be expressed in the form of 2X - 1, where X is an integer."

DEFVAL { 31 }

dot11MCCAAIFSN OBJECT-TYPE

SYNTAX INTEGER (0..15)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the number of slots, after a SIFS duration, that the mesh STA shall sense the medium idle either before transmitting or executing a backoff during an MCCAOP for which it is the MCCAOP owner."

DEFVAL { 1 }

**********************************************************************

* End of dot11MeshSTAConfig TABLE

**********************************************************************

**********************************************************************

* dot11MeshHWMPConfig TABLE

**********************************************************************

dot11MeshHWMPConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MeshHWMPConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Mesh Station HWMP Configuration attributes. In tabular form to allow for multiple instances on an agent."

dot11MeshHWMPConfigEntry  OBJECT-TYPE

SYNTAX Dot11MeshHWMPConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MeshHWMPConfigTable. It is possible for there to be multiple IEEE 802.11 interfaces on one agent, each with its unique MAC address. The relationship between an IEEE 802.11 interface and an interface in the context of the Internet-standard MIB is one-to-one. As such, the value of an ifIndex object instance can be directly used to identify corresponding instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an ifEntry. Interface tables in this MIB module are indexed by ifIndex."

INDEX { ifIndex }

::= { dot11MeshStationConfigTable 1 }

Dot11MeshHWMPConfigEntry ::=

SEQUENCE {

dot11MeshHWMPmaxPREQretries							INTEGER,

dot11MeshHWMPnetDiameter							INTEGER,

dot11MeshHWMPnetDiameterTraversalTime							INGETER,

dot11MeshHWMPpreqMinInterval							INTEGER,

dot11MeshHWMPperrMinInterval							INTEGER,

dot11MeshHWMPactivePathToRootTimeout							INTEGER,

dot11MeshHWMPactivePathTimeout							INTEGER,

dot11MeshHWMProotMode							INTEGER,

dot11MeshHWMProotInterval							INTEGER,

dot11MeshHWMPrannInterval							INTEGER,

dot11MeshHWMPtargetOnly							INTEGER,

dot11MeshHWMPmaintenanceInterval							INTEGER,

dot11MeshHWMPconfirmationInterval							INTEGER

}

dot11MeshHWMPmaxPREQretries OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the number of Action frames containing a PREQ that an originator mesh STA can send to a particular path target for a specific path discovery."

DEFVAL { 3 }

::= { dot11MeshHWMPConfigEntry 1}

dot11MeshHWMPnetDiameter OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the estimate of the maximum number of hops that it takes for an HWMP element to propagate across the mesh BSS."

DEFVAL { 31 }

::= { dot11MeshHWMPConfigEntry 2}

dot11MeshHWMPnetDiameterTraversalTime OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the estimate of the interval of time (in TUs) that it takes for an HWMP element to propagate across the mesh BSS."

DEFVAL { 500 }

::= { dot11MeshHWMPConfigEntry 3}

dot11MeshHWMPpreqMinInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a PREQ element."

DEFVAL { 100 }

::= { dot11MeshHWMPConfigEntry 4}

dot11MeshHWMPperrMinInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a PERR element."

DEFVAL { 100 }

::= { dot11MeshHWMPConfigEntry 5}

dot11MeshHWMPactivePathToRootTimeout OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the time (in TUs) for which mesh STAs receiving a proactive PREQ shall consider the forwarding information to the root mesh STA to be valid; it must be greater than dot11MeshHWMProotInterval."

DEFVAL { 5000 }

::= { dot11MeshHWMPConfigEntry 6}

dot11MeshHWMPactivePathTimeout OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the time (in TUs) for which mesh STAs receiving a PREQ to individual target(s) shall consider the forwarding information to be valid."

DEFVAL { 5000 }

::= { dot11MeshHWMPConfigEntry 7}

dot11MeshHWMProotMode OBJECT-TYPE

SYNTAX INTEGER {

noRoot(0),

proactivePREQnoPREP(2),

proactivePREQwithPREP(3),

rann(4) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute controls the configuration of a mesh STA as root mesh STA. A mesh STA is configured as a root mesh STA if dot11MeshHWMProotMode is set to 2, 3 or 4. Different values correspond to different modes of the root mesh STA. The mesh STA is not a root mesh STA when the attribute is set to 0."

DEFVAL { noRoot }

::= { dot11MeshHWMPConfigEntry 8}

dot11MeshHWMProotInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the minimum interval of time (in TUs) during which a root mesh STA can send only one Action frame containing a proactive PREQ element."

DEFVAL { 2000 }

::= { dot11MeshHWMPConfigEntry 9}

dot11MeshHWMPrannInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a RANN element."

DEFVAL { 1000 }

::= { dot11MeshHWMPConfigEntry 10}

dot11MeshHWMPtargetOnly OBJECT-TYPE

SYNTAX INTEGER { intermediateMSTA(0), targetOnly(1) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute, when set to intermediateMSTA (0), shall allow intermediate mesh STAs to respond with a PREP to a PREQ if they have valid forwarding information to the requested target. When set to targetOnly (1), only the target MSTA is allowed to respond with a PREP to a PREQ."

DEFVAL { targetOnly }

::= { dot11MeshHWMPConfigEntry 11}

dot11MeshHWMPmaintenanceInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a PREQ element for path maintenance."

DEFVAL { 2000 }

::= { dot11MeshHWMPConfigEntry 12}

dot11MeshHWMPconfirmationInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a PREQ element for root path confirmation."

DEFVAL { 2000 }

::= { dot11MeshHWMPConfigEntry 13}