IEEE P802.11
Wireless LANs

| Channel Power Limit Map |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., MS SJ-14-4, San Jose, CA 95134-1706 | +1-408-527-0815 | petere@cisco.com |

Abstract

Submission for candidate P802.11af draft text. A more general mechanism should be provided in 802.11 as an alternative to the existing frequency and power capability elements that assume contiguous channels with the same power limit for each channel. In TVWS we have TV channel bandwidths defined in regulation and RLAN channel bandwidths, and they are not likely the same.

A Channel Power Limit Map is specified for operation in a particular band under control of a database. The possibility of multiple maps for multiple clients (fixed and portable) is provided.

Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor:  Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.   As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

Discussion:

OFCOM Digital Dividend consultations and discussions in ECO SE43 raise the possibility that a geolocation database may provide to a master station a list of permissible channels and transmit power limits for each channel, based on geolocation of the master station. The geolocation database control concept is suitable for all shared licensed bands, not just TV bands, so a more general mechanism should be provided in 802.11 as an alternative to the existing frequency and power elements that assume contiguous channels with the same power limit for each channel. In TVWS we have TV channel bandwidths defined in regulation and RLAN channel bandwidths, and they are not likely the same. A Channel Power Limit Map is specified for operation in a particular band under control of a database. The possibility of multiple maps for multiple clients (fixed and portable) is provided.

The 802.11-2007 standard provides several means for reporting and controlling transmitter power, and several means for reporting and controlling channel utilization. Approved amendment IEEE 802.11y-2008 provides DSE Power Constraint and Extended Channel Switching. The 11af baseline lacks a single means to signal both permissible channels and transmitter power limits for each permissible channel in a band or in another band.

See 7.3.2.14 Extended Supported Rates and 7.3.2.19 Supported Rates for elements that don’t really scale. See 7.3.2.27 Extended Capabilities element and note it does not include DFS/TPC or any band information.

We use Supported Regulatory Classes to exchange multiband capabilities in Probes and (re)Association handshakes. A Supported Regulatory Class conveys support for the procedures specified in Annex I Behavior limits that are related to that class.

Proposed Resolution:

Accept technical comment xxxx based on this discussion and editorial instructions in 10/0514r0.

## Editing instructions:

TGaf Editor: Include the editing instructions given in submission 802.11-10/210r8 if “Operating classes” are not defined in the current 802.11af baseline.

TGaf Editor: Remove any draft text changes derived from submission 802.11-10/472

TGaf Editor: Apply the remaining text changes in this submission to a draft amendment that contains “Operating classes” in its baseline or in the draft text:

3 Definitions

TGaf Editor: In 3, change text as shown:

Insert new definitions 3.af1 and 3.af2 retaining the alphabetic ordering:

3.af1 channel power limit map: A list of channel number, transmit power limit tuples for a subband.

3.af21 TVWS: Television White Space - TV channels where regulatory authorities permit operation by 802.11 STAs.

7 Frame formats

7.3 Management frame body components

7.3.2 Information elements

7.3.2.9 Country information element

TGaf Editor: In 7.3.2.9, insert text as shown:

Replace Figure 7-46 with this figure:

|  | Element ID | Length | Country String | Country field triplets | Pad (if needed) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 | n x 3 | 0 or 1 |

Figure 7-46—Country information element format

Change second paragraph of 7.3.2.9 as shown:

The element ID for this element is set to the value for Country, specified in Table 7-26 (Element IDs). The length of the element is variable, as the element may contain more than one triplet comprising the First Channel Number, Number of Channels, and Maximum Transmit Power Level fields and referred to as subband triplets. Alternatively, where dot11OperatingClassesRequired is true and the first octet of a Country field triplet First Channel Number/Operating Extension Identifier octet has a positive integer value of 201 or greater, then that triplet comprises the Operating Extension Identifier, Operating Class, and Coverage Class fields. Together they are referred to as an operating triplet. Alternatively, where dot11OperatingClassesRequired is true and the first octet of a Country field triplet has a positive integer value of 202, then that triplet comprises the Channel Power Identifier, Channel Number, and Maximum Transmit Power Level fields. Together they are referred to as a channel-power triplet. The minimum length of the element is 8 octets. The format of the Country field triplets is shown in Table 7-26baf1.

Table 7-26baf1—Country field triplets

| Name | 1st octet | 2nd octet | 3rd octet |
| --- | --- | --- | --- |
| Channel-power triplet | Channel power identifier (202) | Channel number | Maximum transmit power level |
| Operating triplet | Operating extension identifier (201) | Operating class | Coverage class |
| Subband triplet | First channel number (1-200) | Number of channels | Maximum transmit power level |

Change first sentence of fourth paragraph of 7.3.2.9 as shown:

The First Channel Number/Operating Extension Identifier/Channel Power Identifier field is 1 octet in length.

Insert paragraph before last pargraph of 7.3.2.9:

The Channel Power Identifier field has a positive integer value of 202. The channel number field of a channel-power triplet contains a positive integer value that indicates an operating channel for this operating class. The maximum transmit power level field of a channel-power triplet indicates the maximum power, in dBm, allowed to be transmitted on the operating channel number of the channel-power triplet. The channel number and maximum transmit power level values are referred to as channel power limit tuples. Where dot11OperatingClassesRequired and dot11LCIDSERequired are true, consecutive channel-power triplets following an operating triplet have monotonically increasing channel number fields. Where dot11OperatingClassesRequired and dot11LCIDSERequired are true, in an operating class any maximum transmit power level in a channel-power triplet takes prescedence over any subband maximum transmit power level for the channel number in the channel-power triplet.

7.4 Action frame format details

7.4.7 Public Action details

7.4.7.1 Public Action frames

TGaf Editor: In 7.4.7.1, insert text as shown:

Insert the following row (ignoring the header row) in Table 7-57e Public Action field values in the correct position to preserve ordering by the "Action field value" column.

Table 7-57e—Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Channel Power Limit Map Announcement |

TGaf Editor: Before clause 10, insert text as shown:

Insert the following new clause after the last subclause of 7.4.7:

7.4.7.af1 Channel Power Limit Map Announcement frame format

The Channel Power Limit Map Announcement frame is a Public Action frame requesting that a dependent STA use the values in the announcement as the current regulatory limits of the named channels. It is transmitted by an enabling STA. The format of the Channel Power Limit Map Announcement frame is shown in Figure 7-57vaf1.

|  |  |  |  |  |  |  | These two fields are repeated, as determined by the Length field |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Category | Action | RequesterSTA Address | ResponderSTA Address | Length | Band Number | Channel Number | Maximum Transmit Power Level |  |
| Octets: | 1 | 1 | 6 | 6 | 1 | 1 | 1 | 1 |  |

Figure 7-57vaf1 Channel Power Limit Map Announcement frame format

The Category field shall be set to the value for public action defined in Table 7-24.

The Action field shall be set to indicte a Channel Power Limit Map Announcement frame, as defined in Table 7-57e.

The RequesterSTAAddress field is the MAC address of the requesting STA that grants enablement. The length of the RequesterSTAAddress field is 6 octets.

The ResponderSTAAddress field is the MAC address of the responding STA that operates based on the enablement. The length of the ResponderSTAAddress field is 6 octets.

The Length field indicates the length of the remaining frame fields in octets, and the value is variable. The minimum value of the length field is 1.

The Band Number field indicates the lowest operating class value for which the channel-power map announcement applies. The length of the Band Number field is 1 octet.

The Channel Number field is a positive integer value that indicates an operating channel for the operating class indicated by the Band number. The length of the Channel Number field is 1 octet.

The Maximum Transmit Power Level field indicates the maximum power, in dBm, allowed to be transmitted on the preceding Channel Number. The length of the Maximum Transmit Power Level field is 1 octet.

The Channel Number and Maximum Transmit Power Level pair values are referred to as channel power limit tuples.

7.4.9a Protected Dual of Public Action frames

7.4.9a.1 Protected Dual of Public Action details

Insert the following row (ignoring the header row) in Table 7-57m Protected Dual of Public Action field values in the correct position to preserve ordering by the "Action field value" column.

Table 7-57m—Protected Dual of Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Protected Channel Power Limit Map Announcement |

Insert the following paragraph at the end of 7.4.9a:

7.4.9a.7af1 Protected Channel Power Limit Map Announcement frame format

The Protected Channel Power Limit Map Announcement frame format is the same as the Channel Power Limit Map Announcement frame format (see 7.4.7.af1). It is used instead of the Channel Power Limit Map Announcement frame when Management Frame Protection is negotiated.

TGaf Editor: Before clause 10 in the current draft, insert text as shown:

9 MAC sublayer functional descripion

- 9.8 Operation across regulatory domains

9.8.3 Operation with operating classes

Change text of last dashed item of 9.8.3 as shown:

- When dot11OperatingClassesRequired is true, or where operating classes domain information is present and the STA parsing a Country element finds a First Channel Number or Operating Extension IdentifierClass field or Channel Power Identifier field with a reserved value, the STA shall ignore the remainder of the Country element and shall parse any remaining management frame body for additional elements.

- When dot11OperatingClassesRequired and dot11LCIDSERequired are true, and a Channel Power Identifier is found, for operation in the class given in the previous operating triplet, maximum transmit power level in a channel power limit tuple shall take prescedence over any subband maximum transmit power level for the channel number.

- When dot11OperatingClassesRequired and dot11LCIDSERequired are true, and a Channel Power Limit Map Announcement frame is received, for operation in the operating classes indicated by the Band field for this regulatory domain, channel transmit limit tuples replace any previous values.

Annex A (normative) Protocol Implementation Conformance Statement (PICS) proforma

A.4 PICS proforma—IEEE Std 802.11-<year>

A.4.3 IUT configuration

TGaf Editor: Insert the following row (ignoring the header row) at end of A.4.af as follows:

### A.4.af TVWS functions

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| WS4 | Channel Power Limit Map Announcement | 9.8.3, Annex J.2.4 | CFaf:M | Yes ℃ No ℃ N/A ℃ |

Annex J (normative) Country elements and regulatory classes

J.2 Band-specific operating requirements

J.2.4 TVWS band in the United States

TGaf Editor: Insert text at end of J.2.4 before the last sentence and Table J.5 as follows:

An enabling STA can issue a channel power limit map announcement to any of its dependent STAs in order to inform them of the current regulatory limits. A dependent AP or DFS owner with dot11DSERequired set to TRUE and receiving a channel power limit map announcement from its enabling STA shall replace all existing channel transmit limit tuples as indicated by the Band field for this regulatory domain.