IEEE P802.11
Wireless LANs

| Clause 17 changes for sub 5 MHz channel widths |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dave Halasz | Aclara | 30400 Solon Road Solon, OH 44139 | +1 440-528-7477 | dhalasz@aclara.com |

Abstract

This document contains modifications for the use of 2.5 MHz channels widths (1/8 clock).

17.1 Introduction

Insert the following paragraph at the end of the clause 17.1:

The OFDM system also provides a “eighth-clocked” operation using 2.5 MHz channel spacing with data communication capabilities of .75, 1.1, 1.5, 2.25, 3, 4.5, 6, and 6.7 Mbit/s. The support of transmitting and receiving at data rates of .75, 1.5, and 3 Mbit/s is mandatory when using 2.5 MHz channel spacing. The eighth-clocked operation multiplies symbol times and clear channel assessment (CCA) times by a factor of eight when using 2.5 MHz channel spacing. The regulatory requirements and information regarding use of this OFDM system are in Annex I and Annex J.

17.2.2 TXVECTOR parameters

Change the following row in Table 17-1 in clause 17.2.2:

Table 17-1—TXVECTOR parameters

| Parameter | Associate primitive | Value |
| --- | --- | --- |
| DATATRATE | PHY-TXSTART.request (TXVECTOR) | 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s for 20 MHz channel spacing (Support of 6, 12, and 24 data rates is mandatory.) |
|  |  | 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s for 10 MHz channel spacing (Support of 3, 6, and 12 data rates is mandatory.) |
|  |  | 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mbit/s for 5 MHz channel spacing (Support of 1.5, 3, and 6 Mbit/s is mandatory) |
|  |  | .75, 1.1, 1.5, 2.25, 3, 4.5, 6, and 6.7 Mbit/s for 2.5 MHz channel spacing (Support of .75, 1.5, and 3 Mbit/s is mandatory) |

17.2.2.2 TXVECTOR DATARATE

Insert in clause 17.2.2.2 as shown:

The DATARATE parameter describes the bit rate at which the PLCP shall transmit the PSDU. Its value can be any of the rates defined in Table 97. Data rates of 6, 12, and 24 Mbit/s shall be supported for 20 MHz channel spacing, data rates of 3, 6, and 12 Mbit/s shall be supported for 10 MHz channel spacing, and data rates of 1.5, 3, and 6 Mbit/s shall be supported for 5 MHz channel spacing, and data rates of .75, 1.5, and 3 Mbit/s shall be supported for 2.5 MHz channel spacing; other rates may also be supported.

17.2.3 RXVECTOR parameters

Change the following row in Table 17-2 in clause 17.2.3:

Table 17-2—RXVECTOR parameters

| Parameter | Associate primitive | Value |
| --- | --- | --- |
| DATARATE | PHY-RXSTART.request (RXVECTOR) | 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s for 20 MHz channel spacing (Support of 6, 12, and 24 data rates is mandatory.) |
|  |  | 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s for 10 MHz channel spacing (Support of 3, 6, and 12 data rates is mandatory.) |
|  |  | 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mbit/s for 5 MHz channel spacing (Support of 1.5, 3, and 6 Mbit/s is mandatory) |
|  |  | .75, 1.1, 1.5, 2.25, 3, 4.5, 6, and 6.7 Mbit/s for 2.5 MHz channel spacing (Support of .75, 1.5, and 3 Mbit/s is mandatory) |

17.2.3.3 DATARATE

Insert in clause 17.2.3.3 as shown:

DATARATE shall represent the data rate at which the current PPDU was received. The allowed values of the DATARATE are 6, 9, 12, 18, 24, 36, 48, or 54 Mbit/s for 20 MHz channel spacing,; 3, 4.5, 6, 9, 12, 18, 24, or 27 Mbit/s for 10 MHz channel spacing; and 1.5, 2.25, 3, 4.5, 6, 9, 12, or 13.5 Mbit/s for 5 MHz channel spacing; and  .75, 1.1, 1.5, 2.25, 3, 4.5, 6, and 6.7 Mbit/s for 2.5 MHz channel spacing.

17.3.2.2 Modulation-dependent parameters

Change Table 17-3 in clause 17.3.2.2 as shown:

Table 17-3—Modulation-dependent parameters

| Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Data rate (Mbit/s) (20 MHz channel spacing) | Data rate (Mbit/s) (10 MHz channel spacing) | Data rate (Mbit/s) (5 MHz channel spacing) | Data rate (Mbit/s) (2.5 MHz channel spacing) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BPSK | 1/2 | 1 | 48 | 24 | 6 | 3 | 1.5 | .75 |
| BPSK | 3/4 | 1 | 48 | 36 | 9 | 4.5 | 2.25 | 1.1 |
| QPSK | 1/2 | 2 | 96 | 48 | 12 | 6 | 3 | 1.5 |
| QPSK | 3/4 | 2 | 96 | 72 | 18 | 9 | 4.5 | 2.25 |
| 16-QAM | 1/2 | 4 | 192 | 96 | 24 | 12 | 6 | 3 |
| 16-QAM | 3/4 | 4 | 192 | 144 | 36 | 18 | 9 | 4.5 |
| 64-QAM | 2/3 | 6 | 288 | 192 | 48 | 24 | 12 | 6 |
| 64-QAM | 3/4 | 6 | 288 | 216 | 54 | 27 | 13.5 | 6.7 |

17.3.2.3 Timing related parameters

Change Table 17-4 in clause 17.3.2.3 as shown:

Table 17-4—Timing-related parameters

| Parameter | Value (20 MHz channel spacing) | Value (10 MHz channel spacing) | Value (5 MHz channel spacing) | Value (2.5 MHz channel spacing) |
| --- | --- | --- | --- | --- |
| NSD: Number of data subcarriers | 48 | 48 | 48 | 48 |
| NSP: Number of pilot subcarriers | 4 | 4 | 4 | 4 |
| NST: Number of subcarriers, total | 52 (NSD+ NSP) | 52 (NSD+ NSP) | 52 (NSD+ NSP) | 52 (NSD+ NSP) |
| ΔF: Subcarrier frequency spacing | 0.3125 MHz (=20 MHz/64) | 0.15625 MHz (= 10 MHz/64) | 0.078125 MHz (= 5 MHz/64) | 0.0390625 MHz (= 2.5 MHz/64) |
| TFFT: Inverse Fast Fourier Transform (IFFT) / Fast Fourier Transform (FFT) period | 3.2 μs (1/ΔF) | 6.4 µs (1/ΔF) | 12.8 µs (1/ΔF) | 25.6 µs (1/ΔF) |
| TPREAMBLE: PLCP preamble duration | 16 μs (TSHORT + TLONG) | 32 µs (TSHORT+ TLONG) | 64 µs (TSHORT+ TLONG) | 128 µs (TSHORT+ TLONG) |
| TSIGNAL: Duration of the SIGNAL BPSK-OFDM symbol | 4.0 μs (TGI+ TFFT) | 8.0 µs (TGI+ TFFT) | 16.0 µs (TGI+ TFFT) | 32.0 µs (TGI+ TFFT) |
| TGI: GI duration | 0.8 μs (TFFT/4) | 1.6 µs (TFFT/4) | 3.2 µs (TFFT/4) | 6.4 µs (TFFT/4) |
| TGI2: Training symbol GI duration | 1.6 μs (TFFT/2) | 3.2 µs (TFFT/2) | 6.4 µs (TFFT/2) | 12.8 µs (TFFT/2) |
| TSYM: Symbol interval | 4 μs (TGI+ TFFT) | 8 µs (TGI+ TFFT) | 16 µs (TGI+ TFFT) | 32 µs (TGI+ TFFT) |
| TSHORT: Short training sequence duration | 8 μs (10 × TFFT /4) | 16 µs (10 × TFFT/4) | 32 µs (10 × TFFT/4) | 64 µs (10 × TFFT/4) |
| TLONG: Long training sequence duration | 8 μs (TGI2+ 2 × TFFT) | 16 µs (TGI2+ 2 × TFFT) | 32 µs (TGI2+ 2 × TFFT) | 64 µs (TGI2+ 2 × TFFT) |

17.3.2.4 Mathematical conventions in the signal descriptions

Change the third paragraph of clause 17.3.2.4 as shown:

The subframes of which Equation (7) are composed are described in 17.3.3, 17.3.4, and 17.3.5.9. The time offsets tSUBFRAME determine the starting time of the corresponding subframe; tSIGNAL is equal to 16 µs for

20 MHz channel spacing, 32 µs for 10 MHz channel spacing, and 64 µs for 5 MHz channel spacing, and 128 µs for 2.5 MHz channel spacing, and tDATA is equal to 20 µs for 20 MHz channel spacing, 40 µs for 10 MHz channel spacing, and 80 µs for 5 MHz channel spacing, and 160 µs for 2.5 MHz channel spacing.

17.3.3 PLCP preamble (SYNC)

Change the first paragraph of clause 17.3.3 as shown:

The PLCP Preamble field is used for synchronization. It consists of 10 short symbols and two long symbols that are shown in Figure 17-4 and described in this subclause. The timings described in this subclause and shown in Figure 17-4 are for 20 MHz channel spacing, are doubled for half-clocked (i.e., 10 MHz) channel spacing, and are quadrupled for quarter-clocked (i.e., 5 MHz) channel spacing, and are multiplied by 8 for eighth-clocked (i.e., 2.5 MHz) channel spacing.

17.3.4.1 RATE field

Change Table 17.5 in clause 17.3.4.1 as shown:

Table 17-5—Contents of the SIGNAL field

| R1–R4 | Rate (Mbit/s) (20 MHz channel spacing) | Rate (Mbit/s) (10 MHz channel spacing) | Rate (Mbit/s) (5 MHz channel spacing) | Rate (Mbit/s) (2.5 MHz channel spacing) |
| --- | --- | --- | --- | --- |
| 1101 | 6 | 3 | 1.5 | .75 |
| 1111 | 9 | 4.5 | 2.25 | 1.1 |
| 0101 | 12 | 6 | 3 | 1.5 |
| 0111 | 18 | 9 | 4.5 | 2.25 |
| 1001 | 24 | 12 | 6 | 3 |
| 1011 | 36 | 18 | 9 | 4.5 |
| 0001 | 48 | 24 | 12 | 6 |
| 0011 | 54 | 27 | 13.5 | 6.7 |

17.3.8.1 Outline description

Change Table 17-11 in clause 17.3.8.1 as shown:

Table 17-11—Major parameters of the OFDM PHY

| Information data rate | 6, 9, 12, 18, 24, 36, 48 and 54 Mbit/s (6, 12 and 24 Mbit/s are mandatory) (20 MHz channel spacing) | 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s (3, 6, and 12 Mbit/s are mandatory) (10 MHz channel spacing) | 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mb/s (1.5, 3, and 6 Mbit/s are mandatory) (5 MHz channel spacing) | .75, 1.1, 1.5, 2.25, 3, 4.5, 6, and 6.7 Mbit/s (.75, 1.5, and 3 Mbit/s are mandatory) (2.5 MHz channel spacing) |
| --- | --- | --- | --- | --- |
| Modulation | BPSK OFDM QPSK OFDM 16-QAM OFDM 64-QAM OFDM | BPSK OFDM QPSK OFDM 16-QAM OFDM 64-QAM OFDM | BPSK OFDM QPSK OFDM 16-QAM OFDM 64-QAM OFDM | BPSK OFDM QPSK OFDM 16-QAM OFDM 64-QAM OFDM |
| Error correcting code | K = 7 (64 states) convolutional code | K = 7 (64 states) convolutional code | K = 7 (64 states) convolutional code | K = 7 (64 states) convolutional code |
| Coding rate | 1/2, 2/3, 3/4 | 1/2, 2/3, 3/4 | 1/2, 2/3, 3/4 | 1/2, 2/3, 3/4 |
| Number of subcarriers | 52 | 52 | 52 | 52 |
| OFDM symbol duration | 4.0 µs | 8.0 µs | 16.0 µs | 32.0 µs |
| GI | 0.8 µsa (TGI) | 1.6 µs (TGI) | 3.2 µs (TGI) | 6.4 µs (TGI) |
| Occupied bandwidth | 16.6 MHz | 8.3 MHz | 4.15 MHz | 2.075 MHz |

17.3.8.6 Slot time

Change the first paragraph of clause 17.3.8.6 as shown:

The slot time for the OFDM PHY shall be 9 µs for 20 MHz channel spacing,  shall be 13 µs for 10 MHz channel spacing, and shall be 21 µs for 5 MHz channel spacing, and shall be 37 µs for 2.5 MHz channel spacing.

17.3.9.2 Transmit spectrum mask

Create Figure 17-12d by copying Figure 17-12c and changing the Frequency axis entries to -3.75, -2.5, -1.375, -1.125, 1.125, 1.375, 2.5 and 3.75. Also change the label to, “Figure 17-12d—Transmit spectrum mask for 2.5 MHz transmission”.

17.3.9.4 Transmit center frequency tolerance

Change clause 17.3.9.4 as shown:

The transmitted center frequency tolerance shall be ±20 ppm maximum for 20 MHz and 10 MHz channels, and shall be ±10 ppm maximum for 5 MHz and 2.5 MHz channels. The transmit center frequency and the symbol clock frequency shall be derived from the same reference oscillator.

17.3.9.5 Symbol clock frequency tolerance

Change clause 17.3.9.5 as shown:

The symbol clock frequency tolerance shall be ±20 ppm maximum for 20 MHz and 10 MHz channels, and shall be ±10 ppm maximum for 5 MHz and 2.5 MHz channels. The transmit center frequency and the symbol clock frequency shall be derived from the same reference oscillator.

17.3.10.1 Receiver minimum input sensitivity

Change Table 17-13 in clause 17.3.10.1 as shown:

Table 17-13—Receiver performance requirements

| Modulation | Coding rate (R) | Adjacent channel rejection (dB) | Alternate adjacent channel rejection (dB) | Minimum sensitivity (dBm) (20 MHz channel spacing) | Minimum sensitivity (dBm) (10 MHz channel spacing) | Minimum sensitivity (dBm) (5 MHz channel spacing | Minimum sensitivity (dBm) (2.5 MHz channel spacing |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BPSK | 1/2 | 16 | 32 | –82 | –85 | -88 | -91 |
| BPSK | 3/4 | 15 | 31 | –81 | –84 | -87 | -90 |
| QPSK | 1/2 | 13 | 29 | –79 | –82 | -85 | -88 |
| QPSK | 3/4 | 11 | 27 | –77 | –80 | -83 | -86 |
| 16-QAM | 1/2 | 8 | 24 | –74 | –77 | -80 | -83 |
| 16-QAM | 3/4 | 4 | 20 | –70 | –73 | -77 | -80 |
| 64-QAM | 2/3 | 0 | 16 | –66 | –69 | -72 | -75 |
| 64-QAM | 3/4 | –1 | 15 | –65 | –68 | -71 | -74 |

17.3.10.5 CCA sensitivity

Change clause 17.3.10.5 as shown:

The start of a valid OFDM transmission at a receive level equal to or greater than the minimum modulation

and coding rate sensitivity (–82 dBm for 20 MHz channel spacing,  –85 dBm for 10 MHz channel spacing, and -88 dBm for 5 MHz channel spacing, and -91 dBm for 2.5 MHz channel spacing) shall cause CCA to indicate busy with a probability > 90% within 4 µs for 20 MHz channel spacing, 8 µs for 10 MHz channel spacing, and 16 µs for 5 MHz channel spacing, and 32 µs for 2.5 MHz channel spacing. If the preamble portion was missed, the receiver shall hold the CS signal busy for any signal 20 dB above the minimum modulation and coding rate sensitivity (–62 dBm for 20 MHz channel spacing, –65 dBm for 10 MHz channel spacing, and –68 dBm for 5 MHz channel spacing, and –71 dBm for 2.5 MHz channel spacing).

NOTE—CCA detect time is based on finding the short sequences in the preamble, so when TSYM  doubles, so does CCA detect time.

17.4.2 OFDM PHY MIB

Change the following rows in Table 17-14 in clause 17.4.2:

Table 17-14—MIB attribute default values/ranges

| Managed object | Default value/range | Operational semantics |
| --- | --- | --- |
| dot11SupportedDataRatesTxTable |  |  |
| dot11 Supported data rates Tx value | 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s for 20 MHz channel spacing (Mandatory rates: 6, 12, and 24) 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s for 10 MHz channel spacing (Mandatory rates: 3, 6, and 12) 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mb/s for 5 MHz channel spacing (Mandatory rates: 1.5, 3, and 6) .75, 1.1, 1.5, 2.25, 3, 4.5, 6, and 6.7 Mbit/s for 2.5 MHz channel spacing (Support of .75, 1.5, and 3 Mbit/s is mandatory) | Static |
| dot11SupportedDataRatesRxTable |  |  |
| dot11 Supported data rates Rx value | 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s for 20 MHz channel spacing (Mandatory rates: 6, 12, and 24) 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s for 10 MHz channel spacing (Mandatory rates: 3, 6, and 12) 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mb/s for 5 MHz channel spacing (Mandatory rates: 1.5, 3, and 6) .75, 1.1, 1.5, 2.25, 3, 4.5, 6, and 6.7 Mbit/s for 2.5 MHz channel spacing (Support of .75, 1.5, and 3 Mbit/s is mandatory) | Static |

17.4.4 OFDM PHY characteristics

Change Table 111 in clause 17.4.4 as shown:

Table 111—OFDM PHY characteristics

| Characteristics | Value (20 MHz channel spacing) | Value (10 MHz channel spacing) | Value (5 MHz channel spacing) | Value (2.5 MHz channel spacing) |
| --- | --- | --- | --- | --- |
| aSlotTime | 9 µs | 13 µs | 21 µs | 37 µs |
| aSIFSTime | 16 µs | 32 µs | 64 µs | 128 µs |
| aCCATime | < 4 µs | < 8 µs | <16 µs | <32 µs |
| aPHY-RX-START-Delay | 24 µs | 48 µs | 96 µs | 192 µs |
| aRxTxTurnaroundTime | < 2 µs | < 2 µs | <2 µs | <2 µs |
| aTxPLCPDelay | Implementation dependent | Implementation dependent | Implementation dependent | Implementation dependent |
| aRxPLCPDelay | Implementation dependent | Implementation dependent | Implementation dependent | Implementation dependent |
| aRxTxSwitchTime | << 1 µs | << 1 µs | << 1 µs | << 1 µs |
| aTxRampOnTime | Implementation dependent | Implementation dependent | Implementation dependent | Implementation dependent |
| aTxRampOffTime | Implementation dependent | Implementation dependent | Implementation dependent | Implementation dependent |
| aTxRFDelay | Implementation dependent | Implementation dependent | Implementation dependent | Implementation dependent |
| aRxRFDelay | Implementation dependent | Implementation dependent | Implementation dependent | Implementation dependent |
| aAirPropagationTime | << 1 µs | << 1 µs | << 1 µs | << 1 µs |
| aMACProcessingDelay | < 2 µs | < 2 µs | < 2 µs | < 2 µs |
| aPreambleLength | 16 µs | 32 µs | 64 µs | 128 µs |
| aPLCPHeaderLength | 4 µs | 8 µs | 16 µs | 32 µs |
| aMPDUMaxLength | 4095 | 4095 | 4095 | 4095 |
| aCWmin | 15 | 15 | 15 | 15 |
| aCWmax | 1023 | 1023 | 1023 | 1023 |

17.5.4.3 PMD_SAP service primitive parameters

Change Table 17-18 in clause 17.5.4.3 as shown:

Table 17-18—List of parameters for the PMD primitives

| Parameter | Associate primitive | Value (20 MHz channel spacing) | Value (10 MHz channel spacing) | Value (5 MHz channel spacing) | Value (2.5 MHz channel spacing) |
| --- | --- | --- | --- | --- | --- |
| TXD_UNIT | PMD_DATA.request | One(1), Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value |
| RXD_UNIT | PMD_DATA.indicate | One(1), Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value |
| TXPWR_LEVEL | PMD_TXPWRLVL.request | 1–8 (max of 8 levels) | 1–8 (max of 8 levels) | 1–8 (max of 8 levels) | 1–8 (max of 8 levels) |
| RATE | PMD_RATE.request | 12 Mbit/s (for BPSK) 24 Mbit/s (for QPSK) 48 Mbit/s (for 16-QAM) 72 Mbit/s (for 64-QAM) | 6 Mbit/s (for BPSK) 12 Mbit/s (for QPSK) 24 Mbit/s (for 16-QAM) 36 Mbit/s (for 64-QAM) | 3 Mbit/s (for BPSK) 6 Mbit/s (for QPSK) 12 Mbit/s (for 16-QAM) 18 Mbit/s (for 64-QAM) | 1.5 Mbit/s (for BPSK) 3 Mbit/s (for QPSK) 6 Mbit/s (for 16-QAM) 9 Mbit/s (for 64-QAM) |
| RSSI | PMD_RSSI.indicate | 0–8 bits of RSSI | 0–8 bits of RSSI | 0–8 bits of RSSI | 0–8 bits of RSSI |