IEEE P802.11
Wireless LANs

| General frame format comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4018 | KazuyukiA.Sakoda(at)jp.sony.com |

Abstract

This document provides suggested resolutions to CID3193, 3152, 3266, 3211, 3237, and 3188.

## Summary of the intention of this document

- Reorganize the action frame categorization.  CID3193

- Relocate Multihop Action frame from “subtype=Multihop Action” to “subtype=Action, Category=Multihop Action”  CID3152, 3266, 3211

- Split 11C.7 into 2 parts and relocate the latter suclauses to subclause under clause 9.  CID3237

- Clean up the description in 11C.7.5.

- Relocate some part of 11C.7.5 to a new subclause under Caluse 9.

- Clean up 7.2.2.2 to remove confusion.  CID3188

## Suggested changes to the draft spec

Apply the following changes.
Corresponding changes to D5.0 are indicated in the following text with “Track Changes” on, to clarify the direction to the editor. Please update the part indicated by the “Track Changes” only.

- Mesh path selection and forwarding

Mesh path selection enables path discovery over multiple instances of the wireless medium within a mesh BSS. The overview of the Mesh path selection framework is described in 11C.7 (Mesh path selection framework). Hybrid Wireless Mesh Protocol (HWMP) is defined as the default path selection protocol for the mesh BSS. HWMP provides both proactive path selection and reactive path selection. Details of HWMP are described in 11C.10 (Hybrid Wireless Mesh Protocol (HWMP)). Link metrics are used to determine a candidate path to the destination of the mesh path. The default link metric (the airtime link metric) is defined in 11C.9 (Airtime link metric).

Once the mesh path of a particular pair of the source STA and the destination STA is found through the mesh path selection procedure, mesh STAs propagate the data by the forwarding function. The details of the forwarding function are described in 9.22 (Mesh forwarding framework).

As a result of the mesh path selection and forwarding, MSDUs can be transmitted among all the mesh STAs in a mesh BSS, even if the mesh STAs are not neighbor STAs of each other. Error: Reference source not found depicts the MSDU transfer within a mesh BSS.

- MAC frame formats

- General frame format

Change the first paragraph of 7.1.2 as follows:

- following text is based on the amendment by TGn

The MAC frame format comprises a set of fields that occur in a fixed order in all frames. Figure 7-1 depicts the general MAC frame format. The first three fields (Frame Control, Duration/ID, and Address 1) and the last field (FCS) in Figure 7-1 constitute the minimal frame format and are present in all frames, including reserved types and subtypes. The fields Address 2, Address 3, Sequence Control, Address 4, QoS Control, HT Control, and Frame Body are present only in certain frame types and subtypes. Each field is defined in Error: Reference source not found. The format of each of the individual subtypes of each frame type is defined in Error: Reference source not found. The components of management frame bodies are defined in Error: Reference source not found. The formats of management frames of subtype Action are defined inin Multihop Action (4-address Action) frame format details.

Valid type and subtype combinations as shown:

- Mesh Control field

The Mesh Control field is present in the unfragmented Mesh Data frame, in the first fragment of the Mesh Data frame, and in the management frame of subtype Action, Category Multihop Action (Multihop Action frame) transmitted by a mesh STA. When the Mesh Control field is present in the Mesh Data frame, the Mesh Control field is inserted as a header of the frame body data and placed as follows:

- When the frame body contains other than A-MSDU and the frame is not encrypted, the Mesh Control field is placed in the first octets of the frame body.

- When the frame body contains other than A-MSDU and the frame is encrypted, the Mesh Control field is placed in the first octets of the encrypted data portion.

- When the frame body contains A-MSDU, the Mesh Control field is placed in the Aggregate MSDU subframe header as shown in Error: Reference source not found.

When the Mesh Control field is present in the Multihop Action frame, the Mesh Control field is placed as specified in 7.4.16 (Multihop Action frame details).

The Mesh Control field is a variable length (from 6 to 18 octet, details are shown in Valid values for the Address Extension Modeinput.doc) field that includes:

- a one octet Mesh Flags field to control Mesh Control processing

- a one octet Mesh Time to Live field for use in multi-hop forwarding to aid in limiting the effect of transitory path selection loops

- a four octet Mesh Sequence Number to suppress duplicates

- and in some cases a 6 or 12 octet Mesh Address Extension field containing extended addresses enabling up to a total of 6 addresses in Mesh Data

The structure of the Mesh Control field is defined in Error: Reference source not found.

| Mesh Flags | Mesh Time To Live (Mesh TTL) | Mesh Sequence Number | Mesh Address Extension |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 4 | 0, 6, or 12 |
| - Mesh Control field |  |  |  |

The Mesh Flags field is 1 octet in length and contains the Address Extension Mode subfield. The structure of the Mesh Flags field is shown in Error: Reference source not found.

| B0 B1 | B2 B7 |
| --- | --- |
| Address Extension Mode | Reserved |
| Bits: 2 | 6 |
| - Mesh Flags field |  |

The Address Extension Mode subfield indicates the contents of the Mesh Address Extension field. Valid values for the Address Extension Modeinput.doc defines valid values for the Address Extension Mode and describes the corresponding contents of the Mesh Address Extension field (see 11C.7.5 (Frame addressing and data forwarding in an MBSS), for the usage of the optional addresses contained in the Mesh Address Extension field). If the Address Extension Mode is 00, the Mesh Address Extension field is not present. For all other values, the Mesh Address Extension field is present following the Mesh Sequence Number field.

| - Valid values for the Address Extension Modeinput.doc |  |  |  |
| --- | --- | --- | --- |
| Address Extension Mode value (binary) | Address Extension Mode description | Mesh Address Extension field length (octets) | Applicable frame types |
| 00 | No Mesh Address Extension field | 0 | Data |
| 01 | Mesh Address Extension field contains Address 4 | 6 | Management (Multihop Action), Data (proxied, group addressed) |
| 10 | Mesh Address Extension field contains Address 5 and Address 6 | 12 | Data (proxied, individually addressed) |

The Mesh Time to Live (Mesh TTL) field is 1 octet in length and contains an unsigned integer corresponding to the remaining number of times the MSDU/MMPDU can be forwarded. 11C.7.5.2 (Addressing of individually addressed frames and data forwarding) and 11C.7.5.3 (Addressing of group addressed frames and data forwarding) describe how Mesh TTL is used in both individually and group addressed frames.

The Mesh Sequence Number field is 4 octets in length and contains an unsigned integer sequence number counter value. Source mesh STAs assign mesh sequence numbers from a single modulo- 4 294 967 296 counter, starting at 0 and incrementing by 1 for each MSDU or MMPDU that is transmitted with a Mesh Control field. Usage of the Mesh Sequence Number is described in 11C.7.5.5 (Detection of duplicate MSDUs/MMPDUs).

NOTE—It is believed that a 32 bit sequence number is sufficient as the rollover would occur after a period of 5 days assuming a source continuously transmitting at a rate of 104 frames per second.

The Mesh Address Extension field, shown in , is 6 o, 18 octets in length and is present only when the Address Extension Mode subfield of the Mesh Flags field is a non-zero value. The Mesh Address Extension field providee additional address fields for mesh address extension as defined in Valid values for the Address Extension Modeinput.doc. The interpretation of the extended Address fields is described in 11C.7.5.1 (Overview).

| Address 4 | Address 5 | Address 6 |
| --- | --- | --- |
| Octets: 6 | 6 | 6 |
| - Mesh Address Extension field |  |  |

The Address 4 subfield is present in Management frames of subtype Action, Category Multihop Action or in proxied group addressed Mesh Data frames to carry a fourth address (which is not included as a part of the MAC header for these frames). The Address 4 subfield is not present in the Mesh Address Extension field of individually addressed Mesh Data frames (because a fourth address is included as part of the MAC header for these frames).

The Address 5 subfield and Address 6 subfield are present to transport the addresses of source and destination end station of End-to-End 802 communication in cases where either (or both) of the end stations are not mesh STAs at the beginning or end of a single mesh path. (See Figure s54 (Example addressing for a Mesh Data frame).)

NOTE— This is useful, for example, when the end stations of IEEE 802 communication are non-mesh, external STAs that communicate over a mesh via proxy mesh STAs.

Details on the usage of these optional address fields are given in 11C.7.5 (Frame addressing and data forwarding in an MBSS).

- Aggregate MSDU format (A-MSDU)

Insert the following at the end of 7.2.2.2:

When Mesh Data frames are aggregated, the Aggregate MSDU subframe header includes Mesh DA, Mesh SA, Length, and Mesh Control. The A-MSDU subframe structure for Mesh Data is defined in Error: Reference source not found. If the A-MSDU subframe is sent as a result of the MA-UNITDATA.request primitivs, the Mesh DA and Mesh SA fields of the A-MSDU subframe header contain the addresses passed in the MA-UNITDATA.requesn primitive.oOtherwise the Mesh DA and Mesh SA fields contain the addresses determined in 11C.7.5 (Frame addressing and data forwarding in an MBSS).

- Management frames

Change the first to the sixth paragraphs of 7.2.3 and Management frame format as shown:

- following text and figure are based on the amendment by TGn

The frame format for a management frame is independent of frame subtype and is defined in Management frame format

| Octets: 2 | 2 | 6 | 6 | 6 | 2 | 4 | 0-2312 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Duration | Address 1 (DA) | Address 2 (SA) | Address 3 BSSID | Sequence Control | HT Control | Frame Body | FCS |
| MAC Header |  |  |  |  |  |  |  |  |
| - Management frame format |  |  |  |  |  |  |  |  |

A STA uses the contents of the Address 1 (DA) field to perform the address matching for receive decisions. In the case where the Address 1 (DA) field contains a group address and the frame subtype is other than Beacon or the frame subtype Action, Category Multihop Action, the Address 3 field BSSID also is validated to ensure that the broadcast or multicast originated from a STA in the BSS of which the receiving STA is a member or from a mesh STA to which mesh peering is maintained. More details of addressing and forwarding of group addressed frame in MBSS are defined in Clause 11C.7.5.3 (Addressing of group addressed frames and data forwarding). When the Address 1 (DA) field contains a group address and the frame subtype is either Probe Request or Action with Category Public, a wildcard BSSID value matches all receiving STA’s BSSIDs. If the frame subtype is Beacon, other address matching rules apply, as specified in 11.1.2.3. Frames of subtype Probe Request with a group address in the Address 1 field are additionally processed as described in 11.1.3.2.1. If the frame subtype is Action, the Category is Public, and the Action is 20/40 BSS Coexistence Management, then additional address matching rules for receive decisions apply as specified in 11.14 and 11.16.

The address fields for all management frame do not vary by frame subtype except Multihop Action frame are as follows:

- The Address 1 field of the management frame is set to the RA (=DA) and is determined as the destination of the frame.

- The Address 2 field of the management frame is set to the TA (=SA) and is determined as the address of the STA transmitting the frame.

- The Address 3 field BSSID of the management frame as follows:

- In management frames of subtype Probe Request, the Address 3 field is set to the BSSID. tThe BSSID is either a specific BSSID as described  in item c) below or the wildcard BSSID as defined in the procedures specified in 11.1.3.

- In management frames of subtype Action, Category Public, the Address 3 field is set to the BSSID. tThe BSSID value is set according to 11.18.

- Otherwise:

- If the STA is an AP or is associated with an AP, the Address 3 field is set to the BSSID. tThe BSSID is the address currently in use by the STA contained in the AP.

- If the STA is a member of an IBSS, the Address 3 field is set to the BSSID. tThe BSSID is the BSSID of the IBSS.

- If the STA is a mesh STA, the Address 3 field is set to Address 2.

The DA field is the destination of the frame.

The SA field is the address of the STA transmitting the frame.

The address fields for the Multihop Action frame are as follows:

- The Address 1 field is set to the RA and is determined as the address of the receiver of the frame.

- The Address 2 field is set to the TA and is determined as the address of the transmitter of the frame.

- The Address 3 field is set to the DA and is determined as the address of the destination mesh STA of the frame.

- numbering of following subclauses based on TGp ending with 7.2.3.14. 7.2.3.15 is the first unused subclause.

- Action field

Insert the following new rows into Category valuesinput.doc, and change the Reserved row as shown.

| - Category valuesinput.doc |  |  |  |
| --- | --- | --- | --- |
| Code | Meaning | See subclause | Robust |
|  | Mesh Action | 7.4.15 Mesh Action frame detailsMesh Action frame details | Yes |
|  |  | Mesh Path Selection Action frame details |  |
|  |  | Mesh Interworking Action frame details |  |
|  |  | Mesh Resource Coordination Action frame details |  |
|  | Multihop Action | 7.4.16 Multihop Action frame detailsError: Reference source not found | Yes |
| < +1> 11-1245 | Reserved | — | — |
|  | Self Protected | Self Protected Action frame details | No |

- GANN element

The Gate Announcement (GANN) element is used for announcing the presence of a mesh STA collocated with a mesh gate in the MBSS.

The GANN element is transmitted in a Beacon or a Gate Announcement frame (see Mesh Interworking Action frame details).

- RANN element

The Root Announcement (RANN) element is used for announcing the presence of a mesh STA configured as root mesh STA with dot11MeshHWMProotMode set to rann (4). RANN elements are sent out periodically by such a root mesh STA.

The RANN element is transmitted in a Beacon or a HWMP Mesh Path Selection frame (see Mesh Path Selection Action frame details).

- Action frame format details

- Self Protected Action frame details

- Self Protected Action fields

- Mesh Action frame details

- Mesh Action fields

Several Action frame formats are defined for mesh BSS operation.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Mesh Action field valuesinput.doc.

| - Mesh Action field valuesinput.doc |  |
| --- | --- |
| Action field value | Description |
|  | Mesh Link Metric Request |
|  | Mesh Link Metric Report |
| 2-63 | Reserved |
| 64 | HWMP Mesh Path Selection |
| 65-127 | Reserved |
| 128 | Gate Announcement |
| 129-191 | Reserved |
| 192 | Congestion Control Notification |
| 193 | MCCA Setup Request |
| 194 | MCCA Setup Reply |
| 195 | MCCAOP Advertisement Request |
| 196 | MCCAOP Advertisements |
| 197 | MCCAOP Reservation Teardown |
| 198 | TBTT Adjustment Request |
| 199 | TBTT Adjustment Response |
| 200-255 | Reserved |

- Mesh Link Metric Request frame format

The Mesh Link Metric Request frame is transmitted by a mesh STA to a neighbor peer mesh STA in a mesh to request metric information. This frame is transmitted in an individually addressed frame. The format of the Mesh Link Metric Request frame body is shown in Mesh Link Metric Request frame bodyinput.doc.

| - Mesh Link Metric Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Link Metric.

The Action field is set to the value in Table s16 (Mesh Action field values)Mesh Action field valuesinput.doc representing Mesh Link Metric Request.

- Mesh Link Metric Report frame format

The Mesh Link Metric Report frame is transmitted by a mesh STA to a neighbor peer mesh STA in a mesh to advertise metric information. This frame is transmitted in an individually addressed frame. The format of the Mesh Link Metric Report frame body is shown in Mesh Link Metric Report frame bodyinput.doc.

| - Mesh Link Metric Report frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Mesh Link Metric Report element |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Link Metric.

The Action field is set to the value in Table s16 (Mesh Action field values) Mesh Action field valuesinput.doc representing Mesh Link Metric Report.

- HWMP Mesh Path Selection frame format

The HWMP Mesh Path Selection frame is transmitted by a mesh STA to establish, update or delete paths to other mesh STAs using the HWMP protocol defined in 11C.10 (Hybrid Wireless Mesh Protocol (HWMP)). The format of the HWMP Mesh Path Selection frame body is shown in HWMP Mesh Path Selection frame bodyinput.doc.

Note — HWMP Mesh Path Selection frame contains one or more of elements indicated in HWMP Mesh Path Selection frame bodyinput.doc.

| - HWMP Mesh Path Selection frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Root Announcement element (optional) |  |
|  | Path Request element (optional) |  |
|  | Path Reply element (optional) |  |
|  | Path Error element (optional) |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Path Selection.

The Action field is set to the value in Table s16 (Mesh Action field values) Mesh Path Selection Action field valuesinput.doc representing HWMP Mesh Path Selection.

The Root Announcement element is set as described in RANN element.

The Path Request element is set as described in Error: Reference source not found.

The Path Reply element is set as described in Error: Reference source not found.

The Path Error element is set as described in Error: Reference source not found.

- Gate Announcement frame format

The Gate Announcement is transmitted by a mesh STA collocated with a mesh gate to announce its presence in the MBSS. This frame is transmitted using group addresses. The format of the Gate Announcement frame body is shown in Gate Announcement frame bodyinput.doc.

| - Gate Announcement frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Gate Announcement element |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Interworking.

The Action field is set to the value in Table s16 (Mesh Action field values) Mesh Interworking Action field valuesinput.doc representing Gate Announcement.

The Gate Announcement element is set as described in Error: Reference source not found.

- Congestion Control Notification frame format

The Congestion Control Notification frame is used to indicate its congestion status to its neighbor peer mesh STA(s). This frame is transmitted using individual addresses or group addresses. The format of the Congestion Control Notification frame body is shown in Congestion Control Notification frame bodyinput.doc.

| - Congestion Control Notification frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Congestion Notification element |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Resource Coordination.

The Action field is set to the value in Table s16 (Mesh Action field values) Mesh Resource Coordination Action field valuesinput.doc representing Congestion Control Notification.

The Congestion Notification element field is set as described in Error: Reference source not found.

- MCCA Setup Request frame format

The MCCA Setup Request frame is used to set up an MCCAOP Reservation. It is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE to one or more neighbor peer mesh STA with dot11MCCAActivated equal to TRUE. This frame is transmitted using individual addresses or group addresses. The format of the MCCA Setup Request frame body is shown in MCCA Setup Request frame bodyinput.doc.

| - MCCA Setup Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Setup Request element |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Resource Coordination.

The Action field is set to the value in Table s16 (Mesh Action field values) Mesh Resource Coordination Action field valuesinput.doc representing MCCA Setup Request.

The MCCA Setup Request element is described in Error: Reference source not found.

- MCCA Setup Reply frame format

The MCCA Setup Reply frame is used to reply to an MCCAOP Setup Request. It is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE to a neighbor peer mesh STA with dot11MCCAActivated equal to TRUE. This frame is transmitted using individual addresses. The format of the MCCA Setup Reply frame body is shown in MCCA Setup Reply frame bodyinput.doc.

| - MCCA Setup Reply frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Setup Reply element |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Resource Coordination.

The Action field is set to the value in  Table s16 (Mesh Action field values) representing MCCA Setup Reply.

The MCCA Setup Reply element is described in Error: Reference source not found.

- MCCAOP Advertisement Request frame format

The MCCAOP Advertisement Request frame is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE in order to request MCCA advertisements from a neighbor peer mesh STA. This frame is transmitted using individual addresses. The format of the MCCAOP Advertisement Request frame body is shown in MCCAOP Advertisement Request frame bodyinput.doc.

| - MCCAOP Advertisement Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc Table s16 (Mesh Action field values)  representing MCCAOP Advertisement Request.

- MCCAOP Advertisements frame format

The MCCAOP Advertisements frame is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE to one or more neighbor peer mesh STAs with dot11MCCAActivated equal to TRUE. This frame is transmitted using group addresses or individual addresses. The format of the MCCAOP Advertisements frame body is shown in MCCAOP Advertisements frame bodyinput.doc.

| - MCCAOP Advertisements frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Advertisements element |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Resource Coordination.

The Action field is set to the value in  Table s16 (Mesh Action field values) representing MCCAOP Advertisements.

The MCCAOP Advertisements element is described in Error: Reference source not found.

- MCCAOP Reservation Teardown frame format

The MCCAOP Reservation Teardown frame is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE to one or more neighbor peer mesh STAs with dot11MCCAActivated equal to TRUE. This frame is transmitted using group addresses or individual addresses. The format of the MCCAOP Reservation Teardown frame body is shown in MCCAOP Reservation Teardown frame bodyinput.doc.

| - MCCAOP Reservation Teardown frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Reservation Teardown element |  |

The Category field is set to the value in Category valuesinput.doc for category Mesh Resource Coordination.

The Action field is set to the value in  Table s16 (Mesh Action field values) representing MCCAOP Reservation Teardown.

The MCCAOP Reservation Teardown element is described in Error: Reference source not found.

- TBTT Adjustment Request frame format

The TBTT Adjustment Request frame is used to request a particular neighbor peer mesh STA to adjust its TBTT. This frame is transmitted using individual addresses. The format of the TBTT Adjustment Request frame body is shown in TBTT Adjustment Request frame bodyinput.doc.

| - TBTT Adjustment Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
| - to (N_Info+2) | Beacon Timing element | Repeated N_Info times (N_Info is a number of beacon timing information tuples as described in 11C.12.4.2.3 (Transmitter’s procedure). |

The Category field is set to the value in Category valuesinput.doc for category Mesh Resource Coordination.

The Action field is set to the value in  Table s16 (Mesh Action field values) representing TBTT Adjustment Request.

The Beacon Timing element is set as described in Error: Reference source not found. When the number of beacon timing information exceeds 50, multiple Beacon Timing elements are present. The elements are present in the order of Report Number field value in the Report Control field.

- TBTT Adjustment Response frame format

The TBTT Adjustment Response frame is used to respond to a TBTT adjustment request. This frame is transmitted using individual addresses. The format of the TBTT Adjustment Response frame body is shown in TBTT Adjustment Response frame bodyinput.doc.

| - TBTT Adjustment Response frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Status Code |  |
| - to (N_Info+2) | Beacon Timing element (optional) | Repeated N_Info times (N_Info is a number of beacon timing information tuples as described in 11C.12.4.2.3 (Transmitter’s procedure). |

The Category field is set to the value in Category valuesinput.doc for category Mesh Resource Coordination.

The Action field is set to the value in  Table s16 (Mesh Action field values) representing TBTT Adjustment Response.

The Beacon Timing element, defined in Error: Reference source not found is present only if the Status Code is set to Error: Reference source not found (i.e., when the request is not successful due to the neighbor constraint). When the number of beacon timing information exceeds 50, multiple Beacon Timing elements are present. The elements are present in the order of Report Number field value in the Report Control field.

7.4.16 Multihop Action frame details

7.4.16.1 Multihop Action fields

Several Multihop Action frame formats are defined for for mesh BSS operation. An Action Value field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame formats are defined in  Multihop Action field valuesinput.doc.

| - Multihop Action field valuesinput.doc |  |
| --- | --- |
| Action field value | Description |
| 0 | Proxy Update |
| 1 | Proxy Update Confirmation |
| 2-255 | Reserved |

7.4.16.2 Proxy Update frame format

The Proxy Update frame is used to inform the update of proxy information and is transmitted using individual addresses. The format of the Proxy Update frame body is shown in Proxy Update frame bodyinput.doc.

| - Proxy Update frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Mesh Control |  |
|  | Proxy Update element |  |

The Mesh Control field is set as defined in Error: Reference source not found.

The Category field is one octet and is set to the value in Category valuesinput.doc for category Mesh Proxy Forwarding.

The Action field is set to the value in  Multihop Action field valuesinput.doc representing Proxy Update.

The Mesh Control field is set as defined in Error: Reference source not found.

The Proxy Update element is described in Error: Reference source not found.

7.4.16.3 Proxy Update Confirmation frame format

The Proxy Update Confirmation frame is used to inform that the corresponding Proxy Update element has been properly received, and is transmitted using individual addresses. The format of the Proxy Update Confirmation frame body is shown in Proxy Update Confirmation frame bodyinput.doc.

| - Proxy Update Confirmation frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Mesh Control |  |
|  | Proxy Update Confirmation element |  |

The Category field is one octet and is set to the value in Category valuesinput.doc for category Mesh Proxy Forwarding.

The Action field is set to the value in  Multihop Action field valuesinput.doc representing Proxy Update Confirmation.

The Mesh Control field is set as defined in Error: Reference source not found.

The Proxy Update Confirmation element is described in Error: Reference source not found.

- Mesh path selection framework

- General

The term “mesh path selection” is used to describe selection of multi-hop paths between mesh STAs at the link layer.

Instruction to the editor:

Relocate 11C.7.5 and 11C.7.6 to a new subclause under clause 9, 9.22 (Mesh forwarding framework), as indicated below.

Insert the following new subclause after 9.21.

- following clause numbering is based on TGxx ending with 9.21.

  - Mesh forwarding framework

    - General

The term “mesh forwarding” is used to describe forwarding of MSDUs and MMPDUs across paths determined by the mesh path selection between mesh STAs at the link layer.

9.22.2 Frame addressing and data forwarding in an MBSS

9.22.2.1 Overview

Mesh Data frames and Multihop Action frames enable multihop MSDU and MMPDU forwarding in an MBSS using the Mesh Control field described in 7.1.3.6.3 (Mesh Control field). In this subclause, addressing of the Mesh Data and Multihop Action frames and MSDU/MMPDU forwarding behavior are described.

Valid address field usage for Mesh Data and Multihop Action framesinput.doc shows the valid combinations of address fields in Mesh Data frames and Multihop Action frames along with the corresponding value of the Address Extension Mode field.

NOTE— ToDS and FromDS fields are located in the Frame Control field (see 7.1.3.1.3 (To DS and From DS fields)). Address Extension Mode subfield is located in the Mesh Flags field in the Mesh Control field (see 7.1.3.6.3 (Mesh Control field)). Address 1, Address 2, and Address 3 fields are located in the MAC header (see 7.1.2 (General frame format)). Address 4 is located in the MAC header if both ToDS and FromDS fields are 1, otherwise Address 4 is located in the Mesh Address Extension field of the Mesh Control field ((see 7.1.2 (General frame format) and 7.1.3.6.3 (Mesh Control field)). Address 5 and Address 6 are located in the Mesh Control field if it is present (see 7.1.3.6.3 (Mesh Control field)).

| - Valid address field usage for Mesh Data and Multihop Action framesinput.doc |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supported frames | ToDS FromDS field | Address Extension Mode value (binary) | Address 1 | Address 2 | Address 3 | Address 4 | Address 5 | Address 6 |
| Mesh Data (individually addressed) | 11 | 00 | RA | TA | DA = Mesh DA | SA = Mesh SA | Not Present | Not Present |
| Mesh Data (group addressed) | 01 | 00 | DA | TA | SA = Mesh SA | Not Present | Not Present | Not Present |
| Mesh Data (proxied, individually addressed) | 11 | 10 | RA | TA | Mesh DA | Mesh SA | DA | SA |
| Mesh Data (proxied, group addressed) | 01 | 01 | DA | TA | Mesh SA | SA | Not Present | Not Present |
| Multihop Action |  |  |  |  |  |  |  |  |
| Multihop Action | 00 | 01 | RA | TA | DA = Mesh DA | SA = Mesh SA | Not Present | Not Present |

In individually addressed Mesh Data and Multihop Action frames, Address 1 and Address 2 correspond to the mesh STA receiver address (RA) and the mesh STA transmitter address (TA) for a particular mesh link. Address 3 and Address 4 correspond to the destination and source end stations of a mesh path. The Address Extension Mode indicates the presence of optional address extension fields including Address 5 and Address 6 in the Mesh Control field that correspond to the end-to-end destination address (DA) and source address (SA) of external STAs that communicate over the mesh BSS via proxy mesh STAs.

NOTE— Individually addressed Mesh Data frames only use mesh STA addresses in fields Address 1, Address 2, Address 3 and Address 4. This allows intermediate mesh STAs to forward Mesh Data frames without necessarily having any knowledge of the source and destination addresses, which may be external addresses. Thus, proxy information only needs to be maintained wherever a mesh STA is a mesh gate or a mesh STA is collocated with AP.

The term source mesh STA refers to the first mesh STA on a mesh path. A source mesh STA may be a mesh STA that is the initial source of an MSDU/MMPDU or a mesh STA that receives an MSDU/MMPDU from a STA outside the mesh BSS and translates and forwards the MSDU/MMPDU on the mesh path. The address of the source mesh STA is referred to as the Mesh SA.

The term destination mesh STA refers to the final mesh STA on a mesh path. A destination mesh STA may be a mesh STA that is the final destination of an MSDU/MMPDU or a mesh STA that receives an MSDU/MMPDU from a mesh path and translates and forwards the MSDU/MMPDU on another mesh path or to a STA outside of the mesh BSS. The address of the destination mesh STA is referred to as the Mesh DA.

In group addressed Mesh Data frames, Address 1 and Address 2 correspond to the group address and the mesh STA transmitter address (TA). Address 3 corresponds to the mesh source of the group addressed Mesh Data frame. The Address Extension Mode indicates the presence of an optional address extension field Address 4 in the Mesh Control field that corresponds to the source address (SA) of external STAs that communicate over the mesh BSS via proxy mesh STAs.

NOTE— The reason for not using four-address MAC header format for group addressed traffic is to avoid interactions with existing implementations. Earlier versions of this standard defined the four-address MAC header format (previously called WDS format) without defining procedures for its use. As a result there is a large number of deployed devices that use the four-address frame format in ways that would affect and be affected by mesh traffic if four-address group addressed frames were to be used.

Example addressing for a Mesh Data frame

illustrates the addressing of a Mesh Data frame that contains an MSDU transmitted and forwarded on a mesh path from a mesh STA collocated with a portal (STA 1) to a mesh STA collocated with an AP (STA 2) where the source is a STA outside of the mesh BSS (STA 33) that is reachable via the portal and the destination is an 802.11 STA associated with the AP (STA 22).

- Example addressing for a Mesh Data frame

Details on how these address mappings work in forwarding processing are described in 9.22.2.2 Addressing of individually addressed Mesh Data frames and data forwarding and 9.22.2.3 Addressing of group addressed Mesh Data frames and data forwarding.

9.22.2.2 Addressing of individually addressed Mesh Data frames and data forwarding

9.22.2.2.1 At source mesh STAs

MSDUs sent by a mesh STA (as a consequence of a MA-UNITDATA.request) and destined to another mesh STA in the MBSS shall be transmitted using the 4-address format frame (with Address Extension Mode set to 00), where the four address fields are set as follows (see Valid address field usage for Mesh Data and Multihop Action framesinput.doc, “Mesh Data (individually addressed)1):

- Address 1: The address of the next-hop mesh STA (toward the destination mesh STA according to the forwarding information — see Error: Reference source not found)

- Address 2: The address of the transmitter mesh STA

- Address 3: The address of the destination mesh STA

- Address 4: The address of the source mesh STA

MSDUs not sent by a mesh STA or not destined to another mesh STA in the MBSS shall be transmitted using the 6-address format frame (with Address Extension Mode set to 10), where the mesh address extension field in the Mesh Control field carries the addresses of the end stations, as specified in Valid address field usage for Mesh Data and Multihop Action framesinput.doc (“Mesh Data (proxied, individually addressed)). The 6 addresses are defined as follows:

- Address 1: The address of the next-hop mesh STA (toward the destination mesh STA according to forwarding information — see Error: Reference source not found)

- Address 2: The address of the transmitter mesh STA

- Address 3: The address of the destination mesh STA at the end of the mesh path

- Address 4: The address of the source mesh STA at the beginning of the mesh path

- Address 5: The address of the destination end station (may be the same as Address 3 if the destination is the mesh STA at the end of the mesh path)

- Address 6: The address of the source end station (may be the same as Address 4 if the source is the mesh STA at the beginning of the mesh path)

The Mesh TTL field in the Mesh Control field shall be set to the value of dot11MeshTTL. The MSDUs are forwarded multiple hops, limited by the Mesh TTL value.

The source mesh STA shall set the Mesh Sequence Number field in the Mesh Control field to a value from a modulo- 4 294 967 296 counter that is incremented by 1 for each new MSDU transmitted with a Mesh Control field.

9.22.2.2.2

At intermediate and destination mesh STAs

On receipt of an individually addressed Mesh Data frame), a mesh STA shall perform the following.

- The mesh STA shall decipher the frame and check it for authenticity. If it is not from a peer mesh STA, the frame shall be silently discarded.

- The mesh STA shall check to see whether the Address 3 field is known; if it is an unknown address, the mesh STA may perform any of the following three actions:

- silently discard the frame

- trigger a path discovery procedure depending on the path selection protocol that is currently active in the mesh BSS

- inform the Mesh STA in Address 2 that the destination is unreachable depending on the path selection protocol that is currently active in the mesh BSS.
(see Error: Reference source not found Case A)

- If Address 2 is not one of the precursors for this destination mesh STA (see Error: Reference source not found), the frame shall be discarded.

- If the frame is not discarded and one or more MSDUs are collected by the frame reception, the mesh STA may detect duplicate MSDUs referring the Mesh Sequence Number in the corresponding Mesh Control field and discard them (see 9.22.2.5 Detection of duplicate MSDUs/MMPDUs).

If Address 3 does not match the mesh STA’s own address, but is a known destination MAC addresses in the forwarding information (i.e., the mesh STA is an intermediate mesh STA — see Error: Reference source not found), the following actions are taken:

- The lifetime of the forwarding information to the destination (Address 3) is set to its initial value.

- The lifetime of the forwarding information to the source (Address 4) is set to its initial value.

- The lifetime of the precursor list entry for the precursor to the destination (Address 2) is set to the maximum of the initial value and the current value.

- The lifetime of the precursor list entry for the precursor to the source (next hop to the destination) is set to the maximum of the initial value and the current value.

- If one or more MSDUs are collected, the Mesh TTL in the corresponding Mesh Control field is decremented by 1. If zero has been reached, the MSDU shall be discarded.

- If the MSDU has not been discarded, the mesh STA shall forward the MSDU via a frame setting the Address 1 field to the MAC address of the next hop mesh STA as determined from the forwarding information (see Error: Reference source not found) and the Address 2 field to its own MAC address and queue the frame for transmission.

If Address 3 matches the mesh STA’s own MAC address and one or more MSDUs are collected from the frame, the mesh STA shall check the Address Extension Mode field in each Mesh Control field and take the following actions based on its value:

- If the Address Extension Mode is 00, the MA-UNITDATA.indication primitive is passed from the MAC sublayer entity to the LLC sublayer entity or entities.

- If the Address Extension Mode is 10:

- If the mesh STA is the final destination of the MSDU (Address 5 = Address 3), the MA-UNITDATA.indication primitive is passed from the MAC sublayer entity to the LLC sublayer entity or entities.

- If the current mesh STA is a proxy mesh STA for external STAs, the mesh STA shall first check whether or not the end-to-end destination address (Address 5) is one of the external addresses in its proxy information (see Error: Reference source not found). If Address 5 is equal to one of the external addresses, the mesh STA shall forward the MSDU to the final destination (see Error: Reference source not found case b).

Note that during the forwarding process at intermediate mesh STAs, the content of the MSDU is not changed.

9.22.2.3 Addressing of group addressed Mesh Data frames and data forwarding

9.22.2.3.1 At source mesh STAs

MSDUs sent by a mesh STA (as a consequence of a MA-UNITDATA.request) and destined to a group of STAs shall be transmitted using a 3-address group addressed Mesh Data frame (with Address Extension Mode set to 00) as described in this subclause. (see Valid address field usage for Mesh Data and Multihop Action framesinput.doc ,“Mesh Data (group addressed)” as well.) An implementation may circumvent the unreliability of group addressed transmissions by using multipl  individually addressed Mesh Data frames, which are individually acknowledged. In such case, the frame may be converted to individually addressed frames and transmitted as individually addressed frame to each peer mesh STAs as described in 9.22.2.2.1 At source mesh STAs, by setting the Address 3 field to the group address. The circumstances for choosing this method are outside the scope of the standard.

For 3-address group addressed frames, the address fields are set as follows:

- Address 1: The group address

- Address 2: The address of the transmitter mesh STA

- Address 3: The address of the source mesh STA

The source mesh STA shall also set the ToDS bit to zero and the FromDS bit to one in the Frame Control field.

If the MSDU is forwarded on behalf of an external STA and Address 1 (RA/DA) is a group address, the source mesh STA, which is a proxy mesh STA, shall set the Address Extension Mode to 01 and Address 4 to the address of the external STA as specified in Valid address field usage for Mesh Data and Multihop Action framesinput.doc, “Mesh Data (proxied, group addressed)”.

The source mesh STA shall set the Mesh TTL field in the Mesh Control to dot11MeshTTL in order to control the hop count. The MSDUs are forwarded multiple hops, limited by the Mesh TTL value. For example, if the Mesh TTL field is 1, MSDUs are delivered to immediate neighbors only.

The source mesh STA shall set the Mesh Sequence Number field in the Mesh Control field to a value from a modulo- 4 294 967 296 counter that is incremented by 1 for each new MSDU transmitted with a Mesh Control field.

Procedures that enhance the reliability or efficiency of group addressed transmissions are outside the scope of this standard.

9.22.2.3.2 At intermediate and destination mesh STAs

On receipt of a group addressed Mesh Data frame  with Address 1 (DA) equal to the group address, or on receipt of a individually addressed Mesh Data frame with Address 3  Mesh SA) equal to the group address, a mesh STA shall perform the following:

- The mesh STA shall decipher the frame and check it for authenticity. If it is not from a peer mesh STA, the frame shall be silently discarded.

- If the frame is not discarded and one or more MSDUs are collected by the frame reception, the mesh STA may detect duplicate MSDUs referring the Mesh Sequence Number in the corresponding Mesh Control field and discard them (see 9.22.2.5 Detection of duplicate MSDUs/MMPDUs).

- The mesh STA decrements the Mesh TTL in the Mesh Control field. If the Mesh TTL value has reached zero, the corresponding MSDU shall not be forwarded to other mesh STAs. If the Mesh TTL value has not reached zero and if the dot1MeshForwarding is 1, the mesh STA shall forward the MSDU via a frame setting Address 2 to its own MAC address.

When the SA and the Mesh SA are not identical, the MSDU shall be forwarded by using 4-address group addressed Mesh Data frame (with Address.Extension set to 01). Otherwise, the MSDU shall be forwarded by using a 3-address group addressed Mesh Data frame (with Address.Extension set to 00). An implementation may circumvent the unreliability of group addressed transmissions by using multiple individually addressed Mesh Data frames, which are individually acknowledged. In such case, the frame may be converted to individually addressed frames and transmitted as an individually addressed frame to each peer mesh STAs as described in At intermediate and destination mesh STAs, by setting the address 3 field to the group address. The circumstances for choosing this method and the ability to determine all the addresses of the neighbor peer mesh STAs are beyond the scope of the standard.

If the frame has not been discarded and one or more valid MSDUs are collected from the frame, the MA-UNITDATA.indication primitive is passed from the MAC sublayer entity to the LLC sublayer entity or entities.

9.22.2.4 Addressing of Management frames and MMPDU forwarding

9.22.2.4.1 General

All MMPDUs except MMPDU transmitted using Multihop Action frames are transmitted over only one hop to peer mesh STAs.

Note that in several cases, the reception and processing of an Action frame leads to the transmission of a new Action frame of the same type that may include an identical or a modified version of the contents from the elements of the received Action frame.

9.22.2.4.2 MMPDU forwarding using Multihop Action frames

MMPDUs transmitted using Multihop Action frames (see 7.2.3.15 (Multihop Action frame format)) are forwarded according to the procedures in 9.22.2.2 Addressing of individually addressed Mesh Data frames and data forwarding. The frame shall be transmitted using 4-address format frame (with Address Extension Mode set to 01), where the four address fields are set as shown in Valid address field usage for Mesh Data and Multihop Action framesinput.doc, “Multihop Action”. A mesh STA transmitting a Multihop Action frame shall set Address 1 field to the address of the next-hop mesh STA (toward the destination mesh STA according to the forwarding information — see Error: Reference source not found).

The source mesh STA shall set the Mesh TTL field in the Mesh Control field to the value of dot11MeshTTL, and set the Mesh Sequence Number field in the Mesh Control field to a value from a modulo- 4 294 967 296 counter that is incremented by 1 for each new MMPDU transmitted using a Multihop Action frame.

At intermediate and destination mesh STAs, on receipt of Multihop Action frame, the address matching, the forwarding information update, and the Mesh TTL decrement are performed as described in 11C.7.5.2.2 (At intermediate and destination mesh STAs), and the MMPDU is forwarded according to the forwarding information.

At intermediate mesh STAs, frame fields following the Mesh Control field are treated as MSDU in 11C.7.5.2.2 (At intermediate and destination mesh STAs) and are not required to be examined.

If the Address 3 in the received Multihop Acion frame matches the mesh STA’s own MAC address, the mesh STA (destination mesh STA) shall process the content of the MMPDU accordingly.

.

9.22.2.5 Detection of duplicate MSDUs/MMPDUs

A mesh STA may receive the same MSDU or MMPDU from different neighbor peer mesh STAs more than once.

Duplicate MSDU/MMPDU filtering is facilitated through the inclusion of a Mesh Sequence Number field in the Mesh Control field in Mesh Data frames and Multihop Action frames as specified in 7.1.3.6.3 (Mesh Control field).

The receiving mesh STA shall keep a cache of recently received <Mesh SA, Mesh Sequence Number> tuples. The Mesh Source Address (Mesh SA) is contained in Address 4 for individually addressed Mesh Data frames and Multihop Action frames. The Mesh Source Address (Mesh SA) is contained in Address 3 for group addressed Mesh Data frames.

A mesh STA shall reject an MSDU/MMPDU with a Mesh Control field as a duplicate if it matches an <Mesh SA, Mesh Sequence Number> tuple of an entry in the cache.

9.22.2.6  Mesh STAs that do not forward

A mesh STA that has dot11MeshForwarding equal to false does not forward either MSDUs, or MMPDUs of type Multihop Action. The circumstances in which a mesh STA may be allowed to become a non forwarding entity and the authority to set dot11MeshForwarding to false are beyond the scope of this standard.

A mesh STA that does not forward is a special case of a mesh STA. Such mechanism depends on whether the path selection protocol provides a mechanism to allow mesh STAs not to participate in forwarding. The HWMP path selection protocol provides such a mechanism, see Error: Reference source not found.

9.22.3 Frame forwarding to non-MBSS STAs

To send or forward a frame, a mesh STA first follows the data forwarding procedures defined in Insert the following new subclause after 9.21.. If the mesh STA is not able to determine an intra-MBSS path to the destination MAC address of the proxy MSTA for an external destination MAC address, the mesh STA shall assume that the destination is outside the MBSS and shall forward the frame to one or more mesh gates (see Error: Reference source not found). If there is no mesh gate available, the mesh STA shall silently discard the frame.

Discovery of mesh gates by mesh STAs is performed using propagated elements, such as a GANN (Error: Reference source not found). Other methods specific to the HWMP path selection protocol are also available, such as the proactive PREQ (Error: Reference source not found) or the proactive RANN (Error: Reference source not found), when the Gate Role flag field in these HWMP elements is 1.

Instruction to the editor:

Additionally, change the references:

Mesh Path Selection Action frame to HWMP Mesh Path Selection frame

11C.7 (Mesh path selection and forwarding framework) to 9.22 (Forwarding framework)

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D5.0, April 2010.