IEEE P802.11
Wireless LANs

| First Re-circulation Sponsor Ballot – EBR Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This submission addresses the following comments from the P802.11u First Re-circulation Sponsor Ballot: CID 1075.

This document is based on IEEE 802.11u-d9.0.

7	Frame formats

7.4	Action frame format details

7.4.2	QoS Action frame details

7.4.2.2 ADDTS Response frame format

Change Table 7-47 by inserting a new row.

| Table 7-47—ADDTS Response frame body |  |
| --- | --- |
| Order | Information |
| 1 | Category |
| 2 | Action |
| 3 | Dialog token |
| 4 | Status Code |
| 5 | TS Delay |
| 6 | TSPEC |
| 7 – n | TCLAS (optional) |
| n + 1 | TCLAS processing (optional) |
| n + 2 | Schedule |
| n + 3 | Expedited Bandwidth Request (optional) |

Change the next to last paragraph in 7.4.2.2 as follows:

The Dialog Token, TS Delay, TSPEC, TCLAS, TCLAS Processing and Expedited Bandwidth Request fields in this frame are contained in an MLME-ADDTS.response primitive that causes the frame to be sent. The TS Delay information element is present in an ADDTS Response frame only if the status code is set to 47.