IEEE P802.11
Wireless LANs

| Resolving Miscellaneous Security Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Networks | 1322 Crossman ave, Sunnyvale, CA | +1 408 227 4500 | dharkins at arubanetworks dot com |

Abstract

This document resolves CIDs 3059, 3154, 3162, 3194, 3221, 3236

Modify 3.s19 as indicated

- password: A shared, secret, and potentially low-entropy, word, phrase, code, or key used as a credential for authentication purposes. The method of distribution of a password to the units in the system is outside the scope of this standard.

Modify 7.3.2.96.7 as indicated

- Mesh Peering Protocol Identifier

The Mesh Peering Protocol Identifier field indicates the type of mesh peering protocol that is currently used to establish mesh peerings. Error: Reference source not found provides the mesh peering protocol identifier values defined by this standard.

- Pre-RSNA security methods

Open System Authentication and Open System Deauthentication shall not be used between mesh STAs.

Modify 8.4.1.1.1 as indicated

- PMKSA

PTKSA. PMKSAs are cached for up to their lifetimes. The PMKSA consists of the following elements:

- PMKID, as defined in 8.5.1.2. The PMKID identifies the security association.

- Authenticator’s or peer’s MAC address.

- PMK.

- Lifetime, as defined in 8.5.1.2.

- AKMP.

All authorization parameters specified by the AS or local configuration. This can include parameters such as the STA’s authorized SSID.

Modify section 8.5.1.5.3 as indicated

- PMK-R0

- If the AKM negotiated is 00:0F-AC:3 then XXKey shall be the second 256 bits of the MSK (which is derived from the IEEE 802.1X authentication), i.e. XXKey = L(MSK, 256, 256). If the AKM negotiated is 00-0F-AC:4, then XXKey shall be the PSK. If the AKM negotiated is 00-0F-AC:Error: Reference source not found, then XXKey shall be the MPMK generated as the result of SAE authentication.

Modify the titles of the following sections from 11C.5 as indicated

- Security capabilities selection

- Instance Pairwise Cipher Suite selection

- Group Cipher Suite selection

- Construction and processing of AES-SIV-protected Mesh Peering Management frames

- MGTK distribution

- Processing Mesh Peering Confirm action frames

- Processing Mesh Peering Close action frames

- Additional events and actions to Mesh Peering Management FSM

References: