IEEE P802.11
Wireless LANs

| Clause 7 changes for the measurement report considering adjacent channel separation requirement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Jae-Hyung Song | LG Electronics | San 4-1, Bongcheon7-Dong, Kwanak-Gu Seoul, Korea | +82-2-2102-0177 | jaehyung.song@lge.com |

Abstract

Submission for the P802.11af draft, which contains modifications to the measurement report, taking the FCC’s adjacent channel separation requirement into account.

Introduction

Section 15.709(a)(2) of FCC-08-260A1 specifies following:

“For personal/portable TVBDs, the maximum conducted output power over the TV channel of operation shall not exceed 100 milliwatts; except that for personal/portable TVBDs that do not meet the adjacent channel separation requirements in Section 15.712(a), the maximum conducted output power shall not exceed 40 milliwatts.”

While the adjacent channel separation is required when the primary signal in the adjacent channel is a TV signal (including signals from Digital television station, digital and analog Class A TV station, low power TV station, TV translator and TV booster stations), when the primary signal is other than TV (i.e., wireless microphone), the adjacent channel separation is not required.

The current measurement report defined in clause 7.3.2.2.1, only signals whether a primary signal is detected or not.  Whether the detected primary signal requires the adjacent channel separation requirement is not signalled at all.

Hence, this submission suggests following modifications.

7.3 Management frame body components

7.3.2 Information elements

7.3.2.22 Measurement report element

7.3.2.22.1 Basic report

Replace Figure 7-66 by the following figure:

|  | BSS | Orthogonal frequency division multiplexing (OFDM) preamble | Unidentified Signal | Primary service signal/Radar | Unmeasured | Adjacent channel separation required | Reserved (0) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: | 0 | 1 | 2 | 3 | 4 | 5 | 5-7 6-7 |

Insert the description of the Adjacent channel separation required bit as shown:

- Adjacent channel separation required bit, which shall be set to 1 when the measured signal is identified to be a primary service signal/radar that is entitled to protection, which requires the adjacent channel separation requirement to its adjacent channels, was detected operating in the channel during the measurement period.  Otherwise, the Adjacent channel separation required bit shall be set to 0.  When the Primary service signal/Radar bit is set to 0, the Adjacent channel separation required bit is deprecated.