IEEE P802.11
Wireless LANs

| Access to Emergency Services without credentials |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 525 3693 | gabor.bajko@nokia.com |

Introduction

This proposal addresses comment ID

This proposal is based on Draft P802.11u_D9.0

- Make the following changes to  figure 7-95o120 in section 7.3.2.90:

| Bits: | B0 - B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- |
|  | Access Network Type | Internet | ASRA | UESA | EAPA |
| Figure 7-95o120—Access Network Options format |  |  |  |  |  |

Bit 7 is the “Emergency Access without Peer Authentication” bit. When the AP sets this bit to 0, it indicates that no association for Emergency Service purposes is allowed without peer authentication. When this bit is set to 1, it indicates that Emergency Services are available without requiring peer-authentication to get access to the network.

- Make the following changes to  table 7-43bn in section 7.3.2.91:

Table 7-43bn—Access Network Query Protocol info ID definitions

| Table 7-43bn—Access Network Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | ANQP Info Element (clause) |
| Reserved | 0-255 | n/a |
| ANQP Capability list | 256 | 7.3.4.1 |
| Venue Name information | 257 | 7.3.4.2 |
| Emergency Call Number information | 258 | 7.3.4.3 |
| Network Authentication Type information | 259 | 7.3.4.4 |
| Roaming Consortium list | 260 | 7.3.4.5 |
| IP Address Type Availability information | 261 | 7.3.4.7 |
| NAI Realm list | 262 | 7.3.4.8 |
| 3GPP Cellular Network information | 263 | 7.3.4.9 |
| AP Geo Location | 264 | 7.3.4.10 |
| AP Civic Location | 265 | 7.3.4.11 |
| AP Location Public Identifier URI | 266 | 7.3.4.12 |
| Domain Name list | 267 | 7.3.4.13 |
| Emergency Alert Identifier URI | 268 | 7.3.4.14 |
| TDLS Discovery | 269 | 7.3.4.15 |
| Emergency NAI | 270 | 7.3.4.16 |
| Reserved | 271– 56796 | n/a |
| Access Network Query Protocol vendor-specific list | 56797 | 7.3.4.6 |
| Reserved | 56798 – 65535 | n/a |

Add the following sections to the draft specification:

7.3.4.16 Emergency NAI

The Emergency NAI element contains an emergency string which can be used by a STA as its identity to indicate emergency access request.

|  | Info ID | Length | Emergency NAI string |  |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |  |

The Length field indicates the length of the Emergency NAI string.

The Emergency NAI field contains a string which can be used by STAs as its identity to indicate network access request without peer authentication.

11.23.3.2.6 Emergency NAI procedures

A STA which does not have valid credentials to connect to a network but it wants to connect to the network for Emergency Service purpose, it shall check the value of the EAPA bit. When the EAPA bit is set to 1, Emergency Services are reachable through the AP without requiring peer authentication. The STA shall use the ANQP protocol to download the Emergency NAI string from the AP and use it to indicate its intention to access the network without peer authentication by using it as its identity in the authentication process, as described in RFC5216.