IEEE P802.11
Wireless LANs

| Access to Emergency Services without credentials |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 323 Fairchild Dr. Mountain View, CA, 94043 | +1 858 525 3693 | gabor.bajko@nokia.com |

Introduction

This proposal addresses comment ID 1001

This proposal is based on Draft P802.11u_D9.0

Insert the following new references into 2 maintaining the ordering in the base spec:

- IETF RFC 5216, The EAP-TLS Authentication Protocol, D. Simon, B. Aboba, R. Hurst March 2008.

- Make the following changes to  table 7-43bn in section 7.3.2.91:

Table 7-43bn—Access Network Query Protocol info ID definitions

| Table 7-43bn—Access Network Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | ANQP Info Element (clause) |
| Reserved | 0-255 | n/a |
| ANQP Capability list | 256 | 7.3.4.1 |
| Venue Name information | 257 | 7.3.4.2 |
| Emergency Call Number information | 258 | 7.3.4.3 |
| Network Authentication Type information | 259 | 7.3.4.4 |
| Roaming Consortium list | 260 | 7.3.4.5 |
| IP Address Type Availability information | 261 | 7.3.4.7 |
| NAI Realm list | 262 | 7.3.4.8 |
| 3GPP Cellular Network information | 263 | 7.3.4.9 |
| AP Geo Location | 264 | 7.3.4.10 |
| AP Civic Location | 265 | 7.3.4.11 |
| AP Location Public Identifier URI | 266 | 7.3.4.12 |
| Domain Name list | 267 | 7.3.4.13 |
| Emergency Alert Identifier URI | 268 | 7.3.4.14 |
| TDLS Discovery | 269 | 7.3.4.15 |
| Emergency NAI | 270 | 7.3.4.16 |
| Reserved | 271– 56796 | n/a |
| Access Network Query Protocol vendor-specific list | 56797 | 7.3.4.6 |
| Reserved | 56798 – 65535 | n/a |

Insert  the following section after section 7.3.4.15 in the draft specification:

7.3.4.16 Emergency NAI

The Emergency NAI element contains an emergency string which can be used by a STA as its identity to indicate emergency access request.

|  | Info ID | Length | Emergency NAI string |  |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |  |

The Info ID field is equal to the value in Table 7-43bn corresponding to the Emergency NAI information.

The Length field indicates the length of the Emergency NAI string.

The Emergency NAI field contains a UTF-8 string formatted in accordance with RFC4282.

Insert  the following section before section 11.23.4 in the draft specification:

11.23.3.2.6 Emergency NAI procedures

A dot11InterworkingServiceEnabled STA which does not have valid credentials to authenticate to a network can use the Emergency NAI string as its EAP identity. 
The Emergency NAI string can be retrieved from the AP using the ANQP procedures in 11.23.3.2. 

The STA uses The Emergency NAI to indicate its intention to access the network without peer authentication by using the Emergency NAI as its identity in the authentication process, as described in RFC5216.

- Make the following changes to  table 11-4 in section 11.23.3.2:

| Table 11-4—Access Network Query Protocol usage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | BSS |  | IBSS |
| Info Name | ANQP Info Element (clause) | AP | Non-AP STA | STA |
| ANQP Capability list | 7.3.4.1 | T, R | T, R | T, R |
| Venue Name information | 7.3.4.2 | T | R | ---- |
| Emergency Call Number information | 7.3.4.3 | T | R | ---- |
| Network Authentication Type information | 7.3.4.4 | T | R | ---- |
| Roaming Consortium list | 7.3.4.5 | T | R | ---- |
| Access Network Query Protocol vendor-specific list | 7.3.4.6 | T, R | T, R | T, R |
| IP Address Type Availability information | 7.3.4.7 | T, R | T, R | T, R |
| NAI Realm list | 7.3.4.8 | T | R | T, R |
| 3GPP Cellular Network information | 7.3.4.9 | T | R | ---- |
| AP Geo Location | 7.3.4.10 | T | R | T, R |
| AP Civic Location | 7.3.4.11 | T | R | T, R |
| The AP Location Public Identifier URI | 7.3.4.12 | T | R | T, R |
| Domain Name list | 7.3.4.13 | T | R | ---- |
| Emergency Alert Identifier URI | 7.3.4.14 | T | R | T, R |
| TDLS Discovery | 7.3.4.15 | T | R | T, R |
| Emergency NAI | 7.3.4.16 | T | R | ---- |
| Symbols T ANQP element may be transmitted by MAC entity R ANQP element may be received by MAC entity --- ANQP element is neither transmitted nor received by MAC entity |  |  |  |  |

Note to the Editor: Document 11-10-654r1 there was a column added to table 11-4. The value for element type for Emergency NAI is “S”

- Make the following changes to  Table 7-43br:

| Table 7-43br—Authentication Parameter types |  |  |  |
| --- | --- | --- | --- |
| Authentication Information | ID | Description | Length (octets) |
| Reserved | 0 |  |  |
| Expanded EAP Method | 1 | Expanded EAP Method Subfield | 7 |
| Non-EAP Inner Authentication Type | 2 | Enum (0 - Reserved, 1 - PAP, 2 – CHAP, 3 - MSCHAP, 4 - MSCHAPV2) | 1 |
| Inner Authentication EAP Method Type | 3 | Value drawn from IANA EAP Method Type Numbers | 1 |
| Expanded Inner EAP Method | 4 | Expanded EAP Method Subfield | 7 |
| Credential Type | 5 | Enum (1-SIM, 2-USIM, 3-NFC Secure Element, 4-Hardware Token, 5-Softoken, 6 - Certificate, 7 – username/password, 8- none*, 9-Vendor Specific) *none means server-side authentication only | 1 |
| Tunneled EAP Method Credential Type | 6 | Enum (1-SIM, 2-USIM, 3-NFC Secure Element, 4-Hardware Token, 5-Softoken, 6-Certificate, 7-username/password, 8-Anonymous, 9-Vendor Specific) | 1 |
| Reserved | 7 - 220 |  |  |
| Vendor Specific | 221 | Variable | variable |
| Reserved | 222 - 255 |  |  |

- Make the following changes to  Table A.4.22:

A.4.22 Interworking (IW) with External Networks extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Interworking with External Networks capabilities supported? |  |  |  |
| IW1 | Interworking capabilities and Information | 7.3.2.90, 11.23.2 | CF19:M | Yes, No, N/A |
| IW1.1 | Interworking information element | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.2 | Access Network Type | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.3 | Venue Type | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.4 | HESSID | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.5 | Interworking Action frame | 7.4.7a | IW1:M | Yes, No, N/A |
| IW2 | Generic Advertisement Services | 11.23.3 | CF19:M | Yes, No, N/A |
| IW2.1 | Advertisement Protocol element | 7.3.2.91 | IW2:M | Yes, No, N/A |
| IW2.2 | GAS Protocol | 11.23.3.1 | IW2:M | Yes, No, N/A |
| IW2.2.1 | Access Network Query Protocol | 7.3.4 | IW2.2:M | Yes, No, N/A |
| IW2.2.2 | ANQP Capability list | 7.3.4.1 | IW2.2.1:M | Yes, No, N/A |
| IW2.2.3 | Venue Name information | 7.3.4.2 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.4 | Emergency Call Number information | 7.3.4.3 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.5 | Network Authentication Type information | 7.3.4.4 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.6 | Roaming Consortium list | 7.3.4.5 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.7 | IP Address Type Availability information | 7.3.4.7 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.8 | NAI Realm list | 7.3.4.8 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.9 | 3GPP Cellular Network information | 7.3.4.9 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.10 | AP Geos Location | 7.3.4.10 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.11 | AP Civic Location | 7.3.4.11 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.12 | AP Location Public Identifier URI | 7.3.4.12 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.13 | Domain Name list | 7.3.4.13 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.14 | Emergency Alert URI | 7.3.4.14 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.15 | Emergency NAI | 7.3.4.16 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.16 | ANQP vendor-specific list | 7.3.4.6 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.17 | MIH IS | 11.23.4 | IW2:O | Yes, No, N/A |
| IW2.2.18 | MIH Event and Command Serivices Discovery | 11.23.4 | IW2.3:O | Yes, No, N/A |
| IW2.2.18 | Emergency Alert System (EAS) | 7.3.2.91, 7.3.2.95 | IW2.3:O | Yes, No, N/A |
| IW2.2.19 | Location-to-Service Translation Protocol | 7.3.2.91 | IW2.3:O | Yes, No, N/A |
| IW2.2.20 | Advertisement Protocol ID, Vendor Specific | 7.3.2.91 | IW2.3:O | Yes, No, N/A |