IEEE P802.11
Wireless LANs

| First Re-circulation Sponsor Ballot – TDLS Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

Abstract

This submission addresses the following comments from the P802.11u First Re-circulation Sponsor Ballot: CID 1089.

In resolving CID 1089, it was realized that TDLS Discovery requires the requesting STA to send Query Information.  Previously, only a 2-octet Info ID or a ANQP vendor-specific list could be used in an ANQP Query.  However, if a TDLS Discovery frame is included in an ANQP Query containing Info IDs, any Info ID following the TLDS Discovery frame might not be parsable by the responding STA.  Therefore, this submission adds the ANQP Query list as a container for Info IDs forming an ANQP query.  All STAs are required to support the ANQP Query list.

This document is based on IEEE 802.11u-d9.0 and 11-10-620r2.

7	Frame formats

7.3	Management frame body components

7.3.4	Access Network Query Protocol elements

Insert a new row in the table, renumbering subsequent rows as shown below:

| Table 7-43bn—Access Network Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | ANQP Info Element (clause) |
| Reserved | 0-255 | n/a |
| ANQP Query list | 256 | 7.3.4.1 |
| ANQP Capability list | 257 | 7.3.4.2 |
| Venue Name information | 258 | 7.3.4.3 |
| Emergency Call Number information | 259 | 7.3.4.4 |
| Network Authentication Type information | 260 | 7.3.4.5 |
| Roaming Consortium list | 261 | 7.3.4.6 |
| IP Address Type Availability information | 262 | 7.3.4.8 |
| NAI Realm list | 263 | 7.3.4.9 |
| 3GPP Cellular Network information | 264 | 7.3.4.10 |
| AP Geo Location | 265 | 7.3.4.11 |
| AP Civic Location | 266 | 7.3.4.12 |
| AP Location Public Identifier URI | 267 | 7.3.4.13 |
| Domain Name list | 268 | 7.3.4.14 |
| Emergency Alert Identifier URI | 269 | 7.3.4.15 |
| TDLS Discovery | 270 | 7.3.4.16 |
| Reserved | 271– 56796 | n/a |
| Access Network Query Protocol vendor-specific list | 56797 | 7.3.4.7 |
| Reserved | 56798 – 65535 | n/a |

Insert the following new clause, renumbering subsequent clauses

7.3.4.1	ANQP Query list

The ANQP Query list provides a list of identifiers of ANQP element for which the requesting STA is querying in an ANQP Query.  The ANQP Query list element is included in a GAS Query Request.

The format of the ANQP Capability list is provided in Figure 7-95o130a.

|  | Info ID | Length | ANQP Query ID #1 | ANQP Query ID #2 (optional) | … | ANQP Query ID#n (optional) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 | 0 or 2 |  | 0 or 2 |  |
| Figure 7-95o130a—ANQP Query list format |  |  |  |  |  |  |  |

The Info ID is a 2-octet field whose value is drawn from Table 7-43bn corresponding to the ANQP Query list.

The Length is a 2-octet field whose value is equal to 2 times the number of ANQP Query ID fields.

Each ANQP Query ID field value is an Info ID drawn from Table 7-43bn.  Including an Info ID in the ANQP Query list declares that the STA performing the ANQP Query is requesting the ANQP element corresponding to that Info ID be returned in the ANQP Query Response.  The Info IDs included in the ANQP Query list are ordered by monotonocally increasing Info ID value.

11 MLME

11.23 WLAN Interworking with External Networks Procedures

11.23.4	Interworking Procedures: IEEE 802.21 MIH Support

11.23.3.2 Access Network Query Protocol procedures

A STA may use ANQP to retrieve information as defined in Table 7-43bn from a peer STA.

A STA accomplishes this by transmitting one or more ANQP elements having an element type of Q in Table 11-4 in the Query Request field in a GAS Initial Request frame. ANQP elements shall be ordered by non-decreasing Info ID. The ANQP Query Response is comprised of ANQP elements drawn from Table 7-43bn having an element type of S in Table 11-4 and shall be ordered by non-decreasing Info ID. If information is not available for a particular ANQP element, then a query for that element will return that element with all optional fields not present.

ANQP frame usage for Infrastructure BSSs and IBSSs shall be in accordance with Table 11-3. Frame usage defines the entities permitted to transmit and receive particular ANQP elements. When dot11InterworkingServiceEnabled is true, STAs shall be capable of using the ANQP Query list to request the ANQP Capability list and returning the ANQP Capability list in an ANQP GAS message exchange; support for all other ANQP elements is optional.

| Table 11-4—Access Network Query Protocol usage |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BSS |  |  | IBSS |
| Info Name | ANQP Info Element (clause) | Element Type | AP | Non-AP STA |  | STA |
| ANQP Query list | 7.3.4.1 | Q | T, R | T, R |  | T, R |
| ANQP Capability list | 7.3.4.2 | S | T, R | T, R |  | T, R |
| Venue Name information | 7.3.4.3 | S | T | R |  | ---- |
| Emergency Call Number information | 7.3.4.4 | S | T | R |  | ---- |
| Network Authentication Type information | 7.3.4.5 | S | T | R |  | ---- |
| Roaming Consortium list | 7.3.4.6 | S | T | R |  | ---- |
| Access Network Query Protocol vendor-specific list | 7.3.4.7 | Q, S | T, R | T, R |  | T, R |
| IP Address Type Availability information | 7.3.4.8 | S | T, R | T, R |  | T, R |
| NAI Realm list | 7.3.4.9 | S | T | R |  | T, R |
| 3GPP Cellular Network information | 7.3.4.10 | S | T | R |  | ---- |
| AP Geo Location | 7.3.4.11 | S | T | R |  | T, R |
| AP Civic Location | 7.3.4.12 | S | T | R |  | T, R |
| The AP Location Public Identifier URI | 7.3.4.13 | S | T | R |  | T, R |
| Domain Name list | 7.3.4.14 | S | T | R |  | ---- |
| Emergency Alert Identifier URI | 7.3.4.15 | S | T | R |  | T, R |
| TDLS Discovery | 7.3.4.16 | Q, S | T | R |  | T, R |
| Symbols Q element is an ANQP Query S element is an ANQP Response T ANQP element may be transmitted by MAC entity R ANQP element may be received by MAC entity --- ANQP element is neither transmitted nor received by MAC entity |  |  |  |  |  |  |

Insert the following new clause, renumbering subsequent clauses

11.23.3.2.1 ANQP Query list procedures

The ANQP Query list is used by a requesting STA to perform an ANQP Query using the procedures defined in 11.23.3.2.  The requesting STA shall only include Info IDs in the ANQP Query list which have the sole element type of S as shown in Table 11-4.  Info IDs which have an element type of Q shall not be included in the NQP Query list (e.g., the Info ID for ANQP Vendor Specific list shall not be included).

A responding STA that encounters an unknown or reserved ANQP Info ID value in an ANQP Query list received without error shall ignore that ANQP Info ID and shall parse any remaining ANQP Info IDs.

Change the following Figure

11.23.3.2.5 TDLS Discovery procedures

Figure 11-17h—Example TDLS capability discovery using ANQP

Insert A.4.21 after A.4.21 as following:

A.4.22 Interworking (IW) with External Networks extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Interworking with External Networks capabilities supported? |  |  |  |
| IW1 | Interworking capabilities and Information | 7.3.2.90, 11.23.2 | CF19:M | Yes, No, N/A |
| IW1.1 | Interworking information element | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.2 | Access Network Type | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.3 | Venue Type | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.4 | HESSID | 7.3.2.90 | IW1:M | Yes, No, N/A |
| IW1.5 | Interworking Action frame | 7.4.7a | IW1:M | Yes, No, N/A |
| IW2 | Generic Advertisement Services | 11.23.3 | CF19:M | Yes, No, N/A |
| IW2.1 | Advertisement Protocol element | 7.3.2.91 | IW2:M | Yes, No, N/A |
| IW2.2 | GAS Protocol | 11.23.3.1 | IW2:M | Yes, No, N/A |
| IW2.2.1 | Access Network Query Protocol | 7.3.4 | IW2.2:M | Yes, No, N/A |
| IW2.2.2 | ANQP Query list | 7.3.4.1 | IW2.2.1:M | Yes, No, N/A |
| IW2.2.3 | ANQP Capability list | 7.3.4.2 | IW2.2.1:M | Yes, No, N/A |
| IW2.2.4 | Venue Name information | 7.3.4.3 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.5 | Emergency Call Number information | 7.3.4.4 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.6 | Network Authentication Type information | 7.3.4.5 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.7 | Roaming Consortium list | 7.3.4.6 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.8 | IP Address Type Availability information | 7.3.4.8 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.9 | NAI Realm list | 7.3.4.9 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.10 | 3GPP Cellular Network information | 7.3.4.10 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.11 | AP Geos Location | 7.3.4.11 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.12 | AP Civic Location | 7.3.4.12 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.13 | AP Location Public Identifier URI | 7.3.4.13 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.14 | Domain Name list | 7.3.4.14 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.15 | Emergency Alert URI | 7.3.4.15 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.16 | ANQP vendor-specific list | 7.3.4.7 | IW2.2.1:O | Yes, No, N/A |
| IW2.2.17 | MIH IS | 11.23.4 | IW2:O | Yes, No, N/A |
| IW2.2.18 | MIH Event and Command Serivices Discovery | 11.23.4 | IW2.3:O | Yes, No, N/A |
| IW2.2.19 | Emergency Alert System (EAS) | 7.3.2.91, 7.3.2.95 | IW2.3:O | Yes, No, N/A |
| IW2.2.20 | Location-to-Service Translation Protocol | 7.3.2.91 | IW2.3:O | Yes, No, N/A |
| IW2.2.21 | Advertisement Protocol ID, Vendor Specific | 7.3.2.91 | IW2.3:O | Yes, No, N/A |
| IW2.3 | GAS Initial Request frame | 7.4.7.14 | IW2:M | Yes, No, N/A |
| IW2.4 | GAS Initial Response frame | 7.4.7.15 | IW2:M | Yes, No, N/A |
| IW2.5 | GAS Comeback Request frame | 7.4.7.16 | IW2:M | Yes, No, N/A |
| IW2.6 | GAS Comeback Response frame | 7.4.7.17 | IW2:M | Yes, No, N/A |
| IW3 | QoS Mapping from External Networks | 11.23.9 9.9.3.1, 9.9.3.2 | CF19:O | Yes, No, N/A |
| IW3.1 | QoS Map Set element | 7.3.2.93 | IW3:M | Yes, No, N/A |
| IW3.2 | Transport of QoS Map Set | 11.23.9 | IW3:M | Yes, No, N/A |
| IW3.3 | QoS Map Configure | 7.4.2.5 | IW3:M | Yes, No, N/A |
| IW4 | MIH Support | 11B, 11.23.4 | CF19:O | Yes, No, N/A |
| IW4.1 | MAC State Generic Convergence Function Support | 11B | IW4:M | Yes, No, N/A |
| IW4.2 | Informational events | 11B.3 | IW4:M | Yes, No, N/A |
| IW4.3 | ESS status reporting | 11B.4.1 | IW4:M | Yes, No, N/A |
| IW4.4 | Network configuration | 11B.4.2 | IW4:M | Yes, No, N/A |
| IW4.5 | Network events | 11B.4.3 | IW4:M | Yes, No, N/A |
| IW4.6 | Network command interface | 11B.4.4 | IW4:M | Yes, No, N/A |
| IW4.7 | Mobility management | 11B.5.1 | IW4:M | Yes, No, N/A |
| IW4.8 | Network configuration | 11B.4.2 | IW4:M | Yes, No, N/A |
| IW5 | Extended channel switch enabled | 7.3.2.58, 11.1.3 | (CF15 AND DSE9):M | Yes, No, N/A |
| IW6 | Expedited Bandwidth Request | 7.3.2.92 | CF19:O | Yes, No, N/A |
| IW7 | SSPN Interface | 11.23.5 | CF19:O | Yes, No, N/A |