IEEE P802.11
Wireless LANs

| MCCA MLME text for LB 161 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dee Denteneer | Philips | HTC 37; Eindhoven; The Netherlands | +31 40 27 49743 | Dee.denteneer@philips.com |

Abstract

This document provides text changes that intend to resolve the MCCA related CIDs for LB 159 identified in the accompanying spreadsheet 10/158r1 by the labels “MCCA-1”, “MCCA-2”, and “MCCA-3” in Column X.

Abstract

This document provides text changes that resolve the MCCA related CIDs for LB 161 identified in the accompanying spreadsheet 10/501r3 by the label “MLME” in Column X.

- Definitions specific to IEEE 802.11

Insert the following new definitions:

The subclause numbering of definit

- Layer management

- MCCA management interface

The following primitives describe how a mesh entity changes its mesh power mode for a mesh peering.

- MLME-ACTIVATEMCCA.request

- Function

This primitive requests that the MAC entity activates MCCA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-ACTIVATEMCCA.request(

MCCAScanDuration,

MAFLimit,

MCCAAdvertPeriodMax,

MCCAMaxTrackStates,

MCCACWmin,

MCCACWmax,

MCCAAIFSN

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MCCAScanDuration | Integer | 0-65535 | Specifies the duration in TUs that the mesh STA shall not initiate or accept MCCAOP Setup Requests. |
| MAFLimit | Integer | 0-255 | Specifies the maximum MCCA access fraction allowed at the mesh STA. This number is always a multiple of (1/255) of the DTIM Interval. |
| MCCAAdvertPeriodMax | Integer | 0-255 | Specifies the maximum interval that a mesh STA with dot11MCCAActivated equal to true waits for an MCCAOP Advertisements update. It is expressed in number of DTIM intervals. |
| MCCAMaxTrackStates | Integer | >= dot11MCCAMinTrackStates | Specifies the total number of MCCAOP reservations that the MAC entity must be able to track. |
| MCCACWmin | Integer | 0-15 | Specifies the value of the minimum size of the contention that the MAC entity uses for channel access during an MCCAOP. |
| MCCACWmax | Integer | 0-63 | Specifies the value of the maximum size of the contention that the MAC entity uses for channel access during an MCCAOP. |
| MCCAAIFSN | Integer | 0-15 | Specifies the value of the AIFSN that the MAC entity uses for channel access during an MCCAOP. |

- When Generated

This primitive is generated by the SME to start the use of MCCA.

- Effect of receipt

This primitive sets dot11MCCAEnabled to true and initializes the MCCA parameters. The MLME subsequently issues an MLME-ACTIVATEMCCA.confirm

primitive that reflects the results.

- MLME-ACTIVATEMCCA.confirm

- Function

This primitive reports the result of an MLME-MCCAACTIVATE.request.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-ACTIVATEMCCA.confirm(

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS,MCCA_NOT_IMPLEMENTED, INVALID_MAF_LIMIT,INVALID_ MCCAAdvertPeriodMax,INVALID_MAXTRACKSTATES,INVALID_MCCACWMIN,INVALID_MCCACWMAX,INVALID_MCCAAIFSN | Indicates the result of the MLME-ACTIVATEMCCA.request. |

- When Generated

This primitive is generated as a result of an MLME-ACTIVATEMCCA.request.

- Effect of Receipt

The SME is notified of the results of the request to start the use of MCCA.

Note to the editor: renumber the following clauses as appropriate

      - MLME-MCCASETUP.request

- Function

This primitive requests that the MAC entity sets up an MCCA reservation.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MCCASETUP.request(

MCCAOPDuration,

MCCAOP Periodicity,

MCCAOP Responder

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MCCAOP Duration | Integer | 0-65535 | Specifies the MCCAOP Duration of the needed MCCAOPs in multiples of 32 μs |
| MCCAOP Periodicity | Integer | 0-255 | Specifies the MCCAOP Periodicity of the needed MCCAOPs |
| MCCAOP Responder | MAC address | Any valid individual MAC address | Specifies the MAC address of the intended MCCAOP responder |

- When Generated

This primitive is generated by the SME to start an MCCA setup procedure.

- Effect of receipt

This primitive causes the selection of an appropriate offset and the transmission of the MCCA Setup Request provided that the conditions for the transmission are met. The MLME subsequently issues an MLME-MCCASETUP.confirm primitive that reflects the results.

- MLME-MCCASETUP.confirm

- Function

This primitive is generated by the MLME as a result of an MLME-MCCASETUP.request primitive to establish an MCCAOP reservation with a specified peer MAC entity.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-ACTIVATEMCCA.confirm(MCCAOP Parameters,

MCCAOPID,

MCCAOP responder,

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MCCAOP Parameters | MCCAOP Reservation | See Clause 7.3.2.104.2 | The MCCAOP reservation parameters |
| MCCAOPID | Integer | 0-255 | MCCAOPID of MCCA reservation |
| MCCAOP responder | MAC address | Any valid individual MAC address | Specifies the MAC address of the intended MCCAOP responder |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, MCCAOP_Reservation_Conflict, MAF_Limit_Exceeded, MCCA_Track_Limit_Exceeded, , | Indicates the result of the MLME-MCCASETUP.request. |

- When Generated

This primitive is generated as a result of an MLME-MCCASETUP.request to establish an MCCAOP reservation with a peer mesh STA.

- Effect of Receipt

The SME is notified of the results of the MCCA setup procedure.

- MLME-MCCASETUP.indication

- Function

This primitive reports the establishment of an MCCAOP reservation with a specified peer MAC entity.

.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MCCASETUP.indication(

MCCAOP Parameters,

MCCAOPID,

MCCAOP owner

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MCCAOP Parameters | MCCAOP Reservation | See Clause 7.3.2.104.2 | The MCCAOP reservation parameters |
| MCCAOPID | Integer | 0-255 | MCCAOPID of MCCA reservation |
| MCCAOP owner | MAC address | Any valid individual MAC address | Specifies the MAC address of the intended MCCAOP responder |

- When Generated

This primitive is generated by the MLME as result of the establishment of an MCCAOP reserveation with a specific peer MAC entity that resulted from an MCCA setup procedure that was initiated by that specific peer MAC entity.

- Effect of Receipt

The SME is notified of the results of the request to start an MCCA setup procedure.

- MLME-MCCATEARDOWN.request

- Function

This primitive requests that the MAC entity tears down an MCCA reservation.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MCCATEARDOWN.request(

MCCAOPID,

Peer MAC,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MCCAOPID | Integer | 0-255 | Specifies the MCCAOPID of the reservation to be torn down. |
| Peer MAC | MAC address | Any valid individual MAC address | Specifies the MAC address of the peer MAC for the MCCA reservation |

- When Generated

This primitive is generated by the SME to start an MCCA TEARDOWN procedure.

- Effect of receipt

This primitive causes the teardown MCCAOP reservation indicated by means of the MCCAOPID. The MLME subsequently issues an MLME-MCCASETUP.confirm primitive that reflects the results.

- MLME-MCCATEARDOWN.confirm

- Function

This primitive reports the result of an MLME-MCCATEARDOWN.request.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-ACTIVATEMCCA.confirm(

MCCAOPID,

Peer MAC

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MCCAOPID | Integer | 0-255 | Specifies the MCCAOPID of the reservation to be torn down. |
| Peer MAC | MAC address | Any valid individual MAC address | Specifies the MAC address of the peer MAC for the MCCA reservation. |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS,INVALID_PEER_MAC,, INVALID_MCCAOPID | Indicates the result of the MLME-MCCATEARDOWN.request. |

- When Generated

This primitive is generated as a result of an MLME-MCCATEARDOWN.request.

- Effect of Receipt

The SME is notified of the results of the request to start an MCCA TEARDOWN procedure.

- MLME-MCCATEARDOWN.indication

- Function

This primitive reports that an MCCAOP reservation with a specified peer MAC entity has been torn down.

.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MCCATEARDOWN.indication(

MCCAOPID,

Peer MAC

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MCCAOPID | Integer | 0-255 | Specifies the MCCAOPID of the reservation to be torn down. |
| Peer MAC | MAC address | Any valid individual MAC address | Specifies the MAC address of the peer MAC for the MCCA reservation. |

- When Generated

This primitive is generated by the MLME as result of the tear down of an MCCAOP reserveation with a specific peer MAC entity that resulted from an MCCA TEARDOWN procedure that was initiated by that specific peer MAC entity.

- Effect of Receipt

The SME is notified of the results of the request to start an MCCA TEARDOWN procedure.