IEEE P802.11
Wireless LANs

| Normative Text for DSE Measurement Reporting in TV White Spaces |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Eunsun Kim | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1860 | esun.kim@lge.com |
| Yongho Seok | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1947 | yongho.seok@lge.com |

Abstract

This submission extends the DSE Measurement Report in order to more accurately indicate the primary service signals and increase the utilization in the TV White Spaces.

The document is based on IEEE 802.11af-D0.03.

Introduction

This document is intended to resolve comment number 148.

Measurement Request IE and Measurement Report IE do not specify the regulatory class. When an AP operating in 20MHz channel receives Measurement Report IE with Primary Service Signal bit set to 1, it does not know which channel is occupied by a primary user. Even though only one DTV channel among 4 consecutive channels is occupied by a primary user, other unoccupied three channels will not be utilized because the AP does not know which channel is used by a primary user.

If an AP is a dependent STA and its associated dependent STA performs a measurement report, then the general measurement report could not reach to the enabling STA of the dependent AP. It just reaches to its associated (dependent) AP. However, the measurement result should be reported to the enabling STA in order that the enabling STA can decide to perform extended channel switch. A dependent AP cannot decide to switch new operating channel and/or operating class (11.9a.3.1).

Therefore, it is proposed to use DSE Measurement Request frame and DSE Measurement Report frame, in which the operating class and channel number are already specified for the measurement and they can possibly transmitted to the enabling STA.

Informative

When an AP requested a measurement report on CH 3 in Figure af2, which is 10MHz and its center frequency is on the lowe boundary of the two TV channels and a STA reports a measurement indicating primary service signal is detected, the AP does not know on which of two TV channels a primary service signal is detected, TV CH (n+2) or TV CH (n+3).

In case a primary user signal is detected on TV CH (n+2), the transmission power of adjacent channels is listed in the second column in Table af2, which is determined by the regulation. In case a primary signal is detected on TV CH (n+3), the transmission power of adjacent channels is listed in the third column in Table af2, which is determined by the regulation. However, the actual transmission power of adjacent channels is limited by the AND operation of the second and third columns in the table, which is listed in the fourth column, since it is just reported that a primary service signal is detected on CH 3, which will reduce the utility in TVWS. Considering the larger channel bandwidth that occupies more TV channels, the utility will be decreased drastically.

Figure af2. 10MHz channelization and Measurement Report Channel in TVWS

Table af2. Transmission power limit

|  | Primary User Detected on TV CH (n+2) | Primary User Detected on TV CH (n+3) | Primary User Signal Reported on CH 3 |
| --- | --- | --- | --- |
| Transmission Power Limit on CH 1 | 40 mW | 100 mW | 40 mW |
| Transmission Power Limit on CH 2 | - | 40 mW | - |
| Transmission Power Limit on CH 3 | - | - | - |
| Transmission Power Limit on CH 4 | 40 mW | - | - |
| Transmission Power Limit on CH 5 | 100 mW | 40 mW | 40 mW |

Note: “-“ means the transmission on that Channel is not allowed by the regulation

Proposed Resolution:

Accept technical comment number 148 based on this discussion and editorial instructions in 10/0xxxr0.

TGaf editor: Modify the following clauses:

- 7.4.7.8 DSE Measurement Report frame format

TGaf editor: Replace Figure 7-101h6 by the following figure:

|  | Category | Action Value | Requester STA Address | Responder STA Address | Length | Operating Class | Channel Number | Measurement Report Mode | Measurement Start Time | Measurement Duration | Reported DSE LCI fields | Primary Service Signal Map |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 6 | 2 | 1 | 1 | 1 | 8 | 2 | n x 26 | variable |

- Figure 7-101h6—DSE Measurement Report frame body format

TGaf editor: Insert the following at the end of clause 7.4.7.8 as the following:

Primary Service Signal Map field indicates which TV channels where the measurement report is applied are occupied by the primary service signals. It is defined in Figure af1.

|  |  | One or More TV Channel Numbers |
| --- | --- | --- |
|  | Number of TV Channels | TV Channel Number |
| Octets: | 1 | 1 |

- Figure af1— Primary Service Signal Map field format

Number of Channels subfield defines the number of Channel Number subfields that are contained in the Primary Service Signal Map field with maximum value of 8

Channel Number subfield indicates the TV channel numbers on which a primary service signal is detected.

Note—Primary service signal is a signal that requires protection by regulation such as TV broadcast signal, PLMR/CMR signal, offshore radiotelephone signal, low power auxiliary service signal including microphone signal, and other signals that require protection from harmful interference. The algorithm to detect the primary service signal/radar shall satisfy regulatory requirements and is outside the scope of this standard.