IEEE P802.11
Wireless LANs

| Normative text for enablement frame |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5062 | sun@nict.go.jp |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | lan@nict.go.jp |
| Yohannes D. Alemseged | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | yohannes@nict.go.jp |
| Ha Nguyen Tran | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | haguen@nict.go.jp |
| Junyi Wang | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | Junyi.wang@nict.go.jp |
| Chin Sean Sum | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | sum@nict.go.jp |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | harada@nict.go.jp |

Abstract

This document is a normative text proposal for IEEE 802.11af/D0.03. The DSE procedure in IEEE 802.11y allows enablement for one channel at a time. However, a dependent STA should be able to seek enablement for multiple channels at the same time. Further, the enabling STA should be able to grant enablement for multiple channels at the same time. This allows for more flexible operation on TVWS channels for IEEE 802.11 products. The modification given in this contribution makes enablement for multiple channels possible.

- 7.4.7.3 DSE Enablement frame format

TGaf editor: Add new frame elements in Figure 7-101h1—DSE Enablement frame body format:

TGaf editor: Insert the following new clauses after the last subclause of 7.4.7.3:

The Channel Number field of enablement request indicates the channel numbers for which dependent STA requests. The Channel Number filed of the enablement response indicates the channel granted by the enabling STA. The dependent STA can select a set of channels from the list of available channels and the enabling STA can grants a channel. The channel number is defined within a regulatory class as shown in Annex J.