IEEE P802.11
Wireless LANs

| Normative Text for Network Channel Enablement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5062 | sun@nict.go.jp |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | lan@nict.go.jp |
| Chin Sean Sum | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | sum@nict.go.jp |
| Yohannes D. Alemseged | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | yohannes@nict.go.jp |
| Ha Nguyen Tran | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | haguen@nict.go.jp |
| Junyi Wang | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | Junyi.wang@nict.go.jp |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | harada@nict.go.jp |

Abstract

This document is a normative text proposal for IEEE 802.11af/D0.03. In order to operate in TV bands under FCC regulation, a dependent STA, which has no access to a TV bands database, shall operate on network channel under the control of an enabling STA (cf. FCC-08-260), which has access to the database. The proposed Public Action frame, Network Channel enablement, is used by STAs to enable controlling on the usage of network channels in a regulatory class. The frame enables a dependent STA to seek control for one or multiple channels at the same time. Further, the enabling STA should be able to set up control for one or multiple channels at the same time. This allows for more flexible operation on network channels for IEEE 802.11 products in TV bands.

NOTE—IEEE 802.11 terms differ from FCC 47CFR [B8], Section 15.703 Definitions in many particulars, but generally an enabling STA is a Master Mode station, and each dependent STA has only one enabling STA at any time. Generally, dependent STAs are Client stations. (cf. 802.11af/D0.02)

- 7.4.7 Public Action details

- 7.4.7.1 Public Action frames

Change Table 7-57e as shown:

Table 7-57e—Public Action field values

| Action field value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | DSE enablement |
| 2 | DSE deenablement |
| 3 | DSE Registered Location Announcement |
| 4 | Extended Channel Switch Announcement |
| 5 | DSE measurement request |
| 6 | DSE measurement report |
| 7 | Measurement Pilot |
| 8 | DSE power constraint |
| 9 | Network Channel Enablement |
| 910-255 | Reserved |

TGaf editor: Insert the following new clauses after the last subclause of 7.4.7:

- 7.4.7.af2 Network Channel Enablement frame format

The Network Channel Enablement frame is a Public Action frame. It is transmitted by a STA and a dependent STA as part as part of the procedure to enable control of a dependent STA by an enabling STA for a particular 802.11 channel in TV bands under a regulatory class. The format of the Network Channel Enablement frame body is shown in Figure 7.4.7af3

| Category | Action Value | Requester STA address | Responder STA Address | Length | Network Channel Enablement Identifier | Operating Class | Channel Number | Maximum Transmit Power | … | Operating Class | Channel Number | Maximum Transmit Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Octets: | 1 | 6 | 6 | 2 | 1 | 1 | 1 | 1 | … | 1 | 1 | 1 |

Figure 7.4.7af3—Network Channel Enablement frame body format

The Category field is set to the value for public action defined in Table 7-24.

The Action Value field is set to indicate a Network Channel Enablement frame, as defined in Table 7-57e.

The RequesterSTAAddress field is the MAC address of the requesting STA that initiates the Network Channel Enablement process. The length of the RequesterSTAAddress field is 6 octets.

The ResponderSTAAddress field is the MAC address of the responding STA that grants TWVS Channel Enablement indicating. The length of the ResponderSTAAddress field is 6 octets.

The Operating Class field indicates the channel set for which the measurement request applies. The Operating Class and Channel Number fields together specify the channel frequency and channel bandwidth for which the channel enablement applies. Valid values for the Operating Class field are shown in Annex J.

The Length field indicates the length of the remaining frame fields in octets, and the value is variable. The minimum value of the Length field is 13.

The Channel Number field of enablement request indicates the channel numbers for which dependent STA requests. The Channel Number filed of the enablement response indicates the channel granted by the enabling STA. The dependent STA can select a set of channels from the list of available channels and the enabling STA can grants a channel. The channel number is defined within a regulatory class as shown in Annex J.

The Maximum Transmit filed gives the maximum allowable transmit power in dBm for TV bands operation.

10. Layer Management

10.3 MLME SAP Interface

TGaf Editor: Insert a new subclause at the end of clause 10.3 as follows:

10.3.af2 Network channel enablement management

The following MLME primitives support the signaling of network channel enablement management.

10.3.af2.1 MLME-NETWORKCHANNELENABLEMENT.request

10.3.af2.1.1 Function

This primitive requests that a (Protected) Network Channel Enablement public action frame be sent by a STA.

10.3.af2.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-NETWORKCHANNELENABLEMENT.request (

NetworkChannelEnablment

Protected,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| NetworkChannelEnablement | A set of information subfields | As described in 7.4.7.af2 | As described in 7.4.7.af2 |
| Protected | Boolean | true, false | Specifies whether the request is sent using a Robust Management frame. If true, the request is sent using the Protected Channel Power Management Announcement frame. If false, the request is sent using the Channel Power Management Announcement frame. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 (Vendor Specific element) | Zero or more elements. |

10.3.af2.1.3 When generated

This primitive is generated by the STA management entity (SME) to request that a (Protected) Channel Enablement public action frame be sent to a STA that is associated to the enabling STA.

10.3.35.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs and transmits a (Protected) Channel Enablement public action frame.

10.3.af2.2 MLME-NETWORKCHANNELENABLEMENT.indication

10.3.af2.2.1 Function

This primitive reports the result of a request to network channel enablement.

10.3.af2.2.2 Semantics of the service primitive

The primitive parameters as follows:

MLME- NETWORKCHANNELENABLEMENT.confirm (

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, UNSPECIFIED_FAILURE | Reports the result of a network channel enablement request |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 (Vendor Specific element) | Zero or more elements. |

10.3.af2.2.3 When generated

This primitive is generated by the MLME when a network channel enablement request completes.  Possible unspecified failure causes include an inability to schedule a network channel enablement public action frame.

10.3.af2.2.4 Effect of receipt

The SME is notified of the results of the network channel enablement procedure.

10.3.af2.3 MLME-NETWORKCHANNELENABLEMENT.conform

10.3.af2.3.1  Function

This primitive indicates that a (Protected) Channel Enablement public action frame was received from an STA.

10.3.af2.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-NETWORKCHANNELPOWERMANAGEMENT.confirm (

PeerMACAddress,

ChannelPowerManagementAnnouncement

Protection,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMACAddress | MACAddress | Any valid individual MAC address | The address of the peer MAC entity from which the Channel Enablement public action frame was received. |
| ChannelPowerManagementAnnouncement | A set of information subfields | As described in 7.4.7.af2 | As described in 7.4.7.af2 |
| Protected | Boolean | true, false | Specifies whether the request was received using a Robust Management frame. If true, the request was received using the Protected Channel Enablement public action frame. If false, the request was received using the Channel Enablement public action frame. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 (Vendor Specific element) | Zero or more elements. |

10.3.af2.3.3 When generated

This primitive is generated by the MLME when a valid (Protected) Channel Enablement public action frame is received.

10.3.af2.3.4 Effect of receipt

On receipt of this primitive, the SME decides whether to accept the switch request.

10.3.af2.4 MLME-CHANNELPOWERMANAGEMENT.response

10.3.af2.4.1  Function

This primitive is used to schedule an accepted network channel enablement.

10.3.af2.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-NETWORKCHANNELENABLEMENT.confirm (

NetworkChannelEnablment

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| NetworkChannelEnablment | A set of information subfields | As described in 7.4.7.af2 | As described in 7.4.7.af2 |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 (Vendor Specific element) | Zero or more elements. |

10.3.af2.4.3 When generated

This primitive is generated by the SME to schedule an accepted network channel enablement request.

10.3.af2.4.4 Effect of receipt

On receipt of this primitive, the MLME schedules the network channel enablement.

11. MLME

Insert the following new subclause as 11.11.6, as shown:

11.11.6 Network Channel Enablement

A dependent STA shall operate on network channels under the control of an enabling STA. The dependent STA may request enablement on multiple network channels and the enabling STA may grant permission of multiple network channels to the dependent STA by using the Network Channel Enablement public action frame (cf. 7.4.7af2 Network Channel Enablement frame format).