IEEE P802.11
Wireless LANs

| Proposal to Improve TDLS TPK Handshake |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ashish Shukla | Marvell | 1st Floor, Muttha Tower, Don Bosco Marg, Yerwada, Pune, India | +91-20-4013 0016 | ashishs@marvell.com |
| Henry Ptasinski | Broadcom Corporation | 150 Mathilda Place, Sunnyvale, CA, 94086 USA | +1-408-543-3316 | henryp@broadcom.com |
| Menzo Wentink | Qualcomm | Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

### Abstract

This submission proposes changes in TDLS TPK Handshake to allow TDLS STA not to respond to the received incorrectly formed TPK Messages sent by a STA thereby preventing possibility of potential Denial of Service attack. This submission also suggests destroying TPKSA when TDLS Setup Confirm is not received for some reason.

All the changes are based on 802.11z draft 9.0.

Delete rows containing Status code 70 and Status Code 71 (inform ANA that Status Codes 70, 71 are free) from Table 7-23 as shown here:

#### 7.3.1.9 Status Code field

Insert six new status codes into Table 7-23, and update the reserved values accordingly:

Table 7-23—Status codes

| Status code | Meaning |
| --- | --- |
| 2 | TDLS wakeup schedule rejected but alternative schedule provided |
| 3 | TDLS wakeup schedule rejected |
| 5 | Security disabled |
| 6 | Unacceptable lifetime |
| 7 | Not in same BSS |
| 72 | Invalid contents of RSNIE |

Apply the changes as shown here:

##### 8.5.9.3.3 TPK Handshake Message 2

If the TDLS responder STA validates the TPK Handshake Message 1 for this TDLS instance, the TDLS responder STA may respond with TPK Handshake Message 2. To do so, the TDLS responder STA shall add an RSN IE, FTIE, and Timeout Interval IE to its TDLS Setup Response frame. The IEs shall be formatted as follows:

- The RSN IE shall include the following:

- Include a pairwise cipher suite from one of those presented in RSN IE of message 1 of this sequence in the Pairwise Cipher Suite List, and set the Pairwise Cipher Suite count to 1.

- The version number shall be the minimum of the maximum version supported by the TDLS responder STA and the version number received in the RSNIE of Message 1.

  - All other RSN IE fields shall be same as those received in Message 1.

- The Timeout Interval IE shall be the same as that received in the TPK Handshake message 1.

- The FTIE shall include the following:

  - ANonce shall be set to a value chosen randomly by the TDLS responder STA, following the recommendations of 8.5.7.

  - SNonce shall be same as that received in message 1 of this sequence

  - The MIC shall be calculated on the concatenation, in the following order, of:

    - TDLS initiator STA MAC address (6 octets)

    - TDLS responder STA MAC address (6 octets)

    - Transaction Sequence number (1 octet) which shall be set to the value 2

    - Link Identifier IE

    - RSN IE

    - Timeout Interval IE

    - FTIE, with the MIC field of the FTIE set to 0.

The MIC shall be calculated using the TPK-KCK and the AES-128-CMAC algorithm. The output of the AES-128-CMAC shall be 128 bits.

  - All other fields shall be set to 0.

The TDLS responder STA sends Message 2 to the TDLS initiator STA.  The TDLS initiator STA shall process Message 2 as follows:

- If the TDLS initiator STA Address and TDLS responder STA Address of the Link Identifier element do not match those for an outstanding TDLS Setup Request, the TDLS initiator STA shall silently discard the received TDLS Setup Response frame.

- If the SNonce field of the FTIE does not match that of an outstanding request to the TDLS responder STA, then the TDLS initiator STA shall silently discard the received TDLS Setup Response frame.

- Otherwise, the TDLS initiator STA shall compute the TPK and then validate the MIC in the FTIE as specified in MIC calculation procedure for TPK Handshake Message 2. If invalid, the TDLS initiator STA shall discard the message.

- If the version of the RSN IE is zero or is greater than the version of the RSN IE sent in Message 1, then the TDLS initiator STA shall reject the response with status code 44 (“Unsupported RSN information element version). Otherwise, the TDLS initiator STA shall:

- If the contents of the RSNIE, with the exception of the Pairwise cipher suite count and Pairwise cipher suite list are not the same as those sent by the TDLS initiator STA in Message 1 of this sequence, then the TDLS initiator STA shall reject the response with status code 72 (“Invalid contents of RSNIE”);

- If the Pairwise Cipher Suite count is other than 1, then the TDLS initiator STA shall reject the response with status code 42 (“Invalid pairwise cipher”);

- If the selected pairwise cipher suite was not included in the Initiator’s request, then the TDLS initiator STA shall reject the TDLS Setup Response with status code 42 (“Invalid pairwise cipher”);

- If the Timeout Interval IE is not the same as that sent in Message 1, the TDLS initiator STA shall reject the TDLS Setup Response with status code 6 (“Unacceptable lifetime”);

- If the BSSID in the Link Identifier element is different from the one sent in Message 1, then the TDLS initiator STA shall reject the response with status code 7 (“Not in same BSS”).

If the TDLS initiator STA validates TDLS Message 2, the TDLS initiator STA shall create a TPKSA and respond with Message 3 as defined in 11.21.2. The TDLS initiator STA uses the MLME-SETKEYS.request primitive to configure the Temporal Key into its STA.

##### 8.5.9.3.4 TPK Handshake Message 3

If the TDLS initiator STA responds to Message 2 for this TDLS instance, the TDLS initiator STA shall add an RSN IE, FTIE, and Timeout Interval IE to its TDLS Setup Confirm frame. The IEs shall be formatted as follows:

- The RSNIE shall be the same as the RSNIE received in Message 2.

- The Timeout Interval IE shall be the same as that received in the TPK Handshake message 1.

- With the exception of the MIC field, the contents of the FTIE shall be the same as the FTIE received in Message 2.

- The MIC shall be calculated on the concatenation, in the following order, of:

- TDLS initiator STA MAC address (6 octets)

- TDLS responder STA MAC address (6 octets)

- Transaction Sequence number (1 octet) which shall be set to the value 3

- Link Identifier IE

- RSN IE

- Timeout Interval IE

- FTIE, with the MIC field of the FTIE set to 0.

The MIC shall be calculated using the TPK-KCK and the AES-128-CMAC algorithm. The output of the AES-128-CMAC shall be 128 bits.

- All other fields shall be set to 0.

The TDLS initiator STA sends Message 3 to the TDLS responder STA.  The TDLS responder STA shall process Message 3 as follows:

- If the Source and Destination Addresses of the Link Identifier IE do not match those for an outstanding TDLS Setup Request, the TDLS responder STA shall discard the message.

- If the ANonce and SNonce fields of the FTIE do not match that of an outstanding request to the TDLS initiator STA, then the TDLS responder STA shall discard the message.

- Otherwise, the TDLS responder STA shall validate the MIC in the FTIE as specified in the MIC calculation procedure for TPK Handshake Message 3. If invalid, the TDLS responder STA shall discard the message.

- If any of the following checks fail, then the TDLS responder STA shall discard the message, the TDLS responder STA shall abandon the TPK Handshake identified by the <ANonce, SNonce> combination, and delete existing TPK Handshake Key state for this sequence.

  - Contents of RSNIE are not the same as what were sent by the TDLS responder STA in Message 2

  - The Timeout Interval IE is not the same as that sent in Message 2

  - The BSSID from the Link Identifier element is not the same as that sent in Message 2

On successful processing of Message 3, the TPK Handshake is considered successful, and the TDLS peer STA shall use the MLME-SETKEYS.request primitive to configure the Temporal Key into its STA.

The TPKSA shall be deleted by the TDLS responder STA if it does not receive a valid TPK Handshake Message 3 from the TDLS Initiator STA within dot11TDLSResponseTimeout.