IEEE P802.11
Wireless LANs

| TVWS Enablement Draft Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-08-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Padam Kafle | Nokia | 6021 Connection Drive, Irving, TX, 75039 | +1-214-673-6232 | Padam.kafle@nokia.com |
| Mika Kasslin | Nokia | Itämerenkatu 11-13, 00180 Helsinki, Finland | +358-50-4836294 | Mika.kasslin@nokia.com |
| Naveen Kakani | Nokia | 6021 Connection Drive, Irving, TX, 75039 | +1 214 763 1041 | Naveen.Kakani@nokia.com |
| Eunsun Kim | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1860 | esun.kim@lge.com |
| Yongho Seok | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1947 | yongho.seok@lge.com |

Abstract

This submission extends the network initiation procedure in TVWS band to include channel availability query for beaconing stations before they can transmit in TVWS channels. The submission also extends the DSE enablement procedure to include the device identity verification.

Submission addresses WG Technical Review of D0.02 comments 68, 69, 70 and 73.

This document is based on IEEE 802.11af-D0.05.

Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor:  Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.   As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| 68 | Padam Kafle |  | J.2.4 | 21 | 31-33 | T | In the sentence "An enabling STA may act as a dependent STA in a BSS that operates under the control of another enabling STA -permitting an IT department to operate as the TV bands database interface for all the Master Mode stations IT manages.", the last part does not look right. When any station acts as a dependent STA, it can not be considered any more as master mode station or enabling STA, it should be referred as client mode station or device. |  | Clarify or edit the sentence as "A STA may act as a dependent STA in a BSS that operates under the control of an enabling STA outside of the BSS-permitting an IT department to operate as the TV bands database interface for all dependent stations (or access points) IT manages." |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69 |  | Padam Kafle | J.2.4 | 22 | 1-3 | T | When an enabling signal is received from a dependent AP, what information inside the DSE Registered Location element is set based on the information received from its enabling device, and what fields are due to its own are not clear. Also, clear description on how the enablement procedure between a dependent STA to the enabling device is supposed to be done in this scenario is not covered in current 11.11.4 and 11.11.5 clause (of 802.11y). It is important to add the necessary settings and steps in 802.11af spec, as it is different than enablement of a simple dependent STA receiving enabling signal directly from its master. | as per the comment |  |
| 70 |  | Padam Kafle | J.2.4 | 22 | 1-3 | T | The "dependent AP" term only covers infrastructure BSS, I suggest to also include the IBSS case. In existing clause 11.11, the steps and content of signaling between such dependent AP and its enabling device is not specified completely. It is difficult to understand how the DSE service area of the enabling device (which could be a server in the IT department) is bounded when the enabling signal can be sent my multiple dependent APs. How the enabler can assert that the dependent AP is within its service area, or the coverage it creates for client stations remains within the coordinates specified for the enabler STA. The DSE enablement process between the dependent AP and its enabler should be extended, and should be clearly specified in the standard. | Clarify, as per the comment. |  |
| 73 |  | Padam Kafle | J.2.4 | 21 | 26-31 | T | When a dependent STA is connected to its enabler or master device via cellular or other non-WLAN media, how it is supposed to receive an enabling signal is not clear. It would be good to specify same general steps applicable to WLAN as well as non-WLAN connectivity. The procedure that needs to be used when using non-WLAN media are not clearly specified. | Need clarification. |  |
| 14 |  | 21.32 | J.2.4 | 21 | 32 | T | Dependent STAs must provide its enabling STA with its FCC ID since the FCC wants to be able to issue a "no channels available" to a specific device or model of device. The inability to identify the single problematic device would result in a "no channels available" shutting down the entire .11 network. | Each dependent STA during the enablement process shall provide its FCC ID to the enabling STA using a MAC message. |  |
| 57 |  | 21.33 | J.2.4 | 21 | 33 | T | Dependent STAs must provide its enabling STA with its FCC ID since the FCC wants to be able to issue a "no channels available" to a specific device or model of device. The inability to identify the single problematic device by not supporting this would result in a "no channels available" shutting down the entire .11 network. | Each dependent STA during the enablement process shall provide its FCC ID to the enabling STA using [MAC message to be defined]. |  |

Discussion:

Please refer to slide sets on 10/1056r2 and 10/1226r0 for general background of TVWS enablement requirements, open issues and proposed terms and concept. The proposed draft text in this document provides resolution to comments 69 and 70, and also addresses concerns of comments 68 and 73 to some extent. Comment 68 has been also addressed by 10/0737r3.

Based on the approved text from 11-10/0737r3, Registered Location Query Protocol was introduced for exchanging the enablement request and responses by means of GAS public action frames. Hence, the TVWS enablement procedure is specified by using DSE public action frames (802.11y) and also as an element of the RLQP protocol to be used with GAS public action frames (802.11u).

While looking into the need of the DSE enablement procedure for a dependent STA which intends to initiate its own network (i.e., it will transmit its own beacon frames, which may also require to include enabling signal element for assisting enablement of other dependent STAs in a TVWS channel) as asked by technical comments 69 and 70, the signalling exchange between a beaconing STA to the RLS can be considered as Channel Availability Query process instead of using the enablement term based on the recent new FCC ruling (10-174). The earlier assumption after introduction of RLQP protocol for enablement signalling from 11-10/0737r3 was that the dependent AP’s location are already known to the enabler (or a registered location server) and because of such pre-established relationship, the enabler authorizes the enablement of such beaconing dependent APs without any need of exchanging location and other parameters. Another assumption was that the dependent STA and the enabler exchanges other information required to authorize devices by means of higher layers.

However, with the recent FCC ruling, the Mode II personal/portable device must have their own geolocation capability to determine their geographic position, which is the information to be used on obtaining the list of available channel list. Another need is to provide the device identification information as required by FCC rules.

Technical comment 70 wanted to provide details in clause 11.11 (which corresponds to MLME clause 10.12 in REVmb 5.0) that covers the steps and content of signalling between dependent AP and its enabling device. This submission provides more detailed description to address these concerns; especially the procedures for Channel Availability Query are added in 10.12.6.

In addition to address the concerns of comments discussed above, the proposed text also introduces general terms and device types in relation to enablement and channel availability query in TVWS band. The enablement and channel availability query process are described in terms of 2 possible interfaces: one between a non-beaconing dependent STA to the Enabling STA and one between a beaconing STA and RLS.

For the comments 14 and 57, the comments asked that dependent STAs should provide the FCC ID to its enabling STA during the enablement process. Therefore FCC ID during the enablement is proposed. Also the FCC ID should be provided in the Channel Availability Query element. It means that after the initial enablement, a STA provides its FCC ID, and the enabling STA or dependent AP provides the available channel list. Therefore, FCC ID field is proposed in the Channel Avialility Query frame. The proposed resolutions for these comments are also addressed in another text: 1040r4.

Proposed Resolution:

Agree in principle with technical comments 69, 70 and 73 based on the discussion and editorial instructions in this document. Ccomment 68 has been accepted by 10/0737r3.

## Editing instructions:

3. Definitions

TGaf Editor: Insert text on page l line 48 as shown:

Personal/portable station: A STA that transmits and/or receives frames from a location that may change.

6. Layer Management

6.3 MLME SAP Interface

TGaf Editor: Insert a new subclause at the end of clause 6.3 as follows:

6.3.af4  Channel Availability Query

The following MLME primitives support the signaling of Channel Availability Query process for the channel query requests and responses.

6.3.af4.1 MLME-CHANNELAVAILABILITYQUERY.request

6.3.af4.1.1 Function

This primitive requests that a (Protected) Channel Availability Query public action frame be sent by a STA to a specified peer MAC entity.

6.3.af4.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CHANNELAVAILABILITYQUERY.request (

ChannelAvailabilityQuery,

ChannelQueryTimeLimit,

Protected,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ChannelAvailabilityQuery | A set of information subfields | As described in 8.5.8.af4 | As described in 8.5.8.af4 |
| ChannelQueryTime- Limit | Integer | >= 1 | Specifies a time limit (in TU) after which the channel query process will be terminated. |
| Protected | Boolean | true, false | Specifies whether the request is sent using a Robust Management frame. If true, the request is sent using the Protected Channel Availability Query frame. If false, the request is sent using the Channel Availability Query frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af4.1.3 When generated

This primitive is generated by the STA management entity (SME) to request channel query procedure with a specified peer MAC entity. During the channel query procedure, the SME can generate additional MLME-CHANNELAVAILABILITYQUERY.request primitives.

6.3.af4.1.4 Effect of receipt

This primitive initiates a channel query procedure. The MLME subsequently issues a MLME-CHANNELAVAILABILITYQUERY.confirm primitive that reflects the results.

6.3.af4.2 MLME-CHANNELAVAILABILITYQUERY.confirm

6.3.af4.2.1  Function

This primitive reports the results of a channel query attempt with a specified peer MAC entity.

6.3.af4.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CHANNELAVAILABILITYQUERY.confirm (

ChannelAvailabilityQuery,

Protection,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ChannelAvailabilityQuery | A set of information subfields | As described in 8.5.8.af4 | As described in 8.5.8.af4 |
| Protected | Boolean | true, false | Specifies whether the response was received using a Robust Management frame. If true, the response was received using the Protected Channel Availability Query public action frame. If false, the response was received using the Channel Availability Query public action frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af4.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-CHANNELAVAILABILITYQUERY.request primitive for channel query with a specified peer MAC entity.

6.3.af4.2.4 Effect of receipt

The SME is notified of the results of the channel query procedure.

6.3.af4.3 MLME-CHANNELAVAILABILITYQUERY.indication

6.3.af4.3.1 Function

This primitive indicates receipt of a request from a specific peer MAC entity to establish a channel query relationship with the STA processing this primitive.

6.3.af4.3.2 Semantics of the service primitive

The primitive parameters as follows:

MLME- CHANNELAVAILABILITYQUERY.indication (

ChannelAvailabilityQuery,

Protection,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ChannelAvailabilityQuery | A set of information subfields | As described in 8.5.8.af4 | As described in 8.5.8.af4 |
| Protected | Boolean | true, false | Specifies whether the request was received using a Robust Management frame. If true, the request was received using the Protected Channel Availability Query public action frame. If false, the request was received using the Channel Availability Query public action frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af4.3.3 When generated

This primitive is generated by the MLME as a result of the receipt of a channel query request from a specific peer MAC entity.

6.3.af4.3.4 Effect of receipt

The SME is notified of the receipt of this channel query request.

6.3.af4.4 MLME-CHANNELAVAILABILITYQUERY.response

6.3.af4.4.1  Function

This primitive is used to send a response to a specified peer MAC entity that requested channel query with the STA that issued this primitive.

6.3.af4.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- CHANNELAVAILABILITYQUERY.response (

ChannelAvailabilityQuery,

Protection,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ChannelAvailabilityQuery | A set of information subfields | As described in 8.5.8.af4 | As described in 8.5.8.af4 |
| Protected | Boolean | true, false | Specifies whether the response is sent using a Robust Management frame. If true, the response is sent using the Protected Channel Availability Query public action frame. If false, the response is sent using the Channel Availability Query public action frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af4.4.3 When generated

This primitive is generated by the SME of a STA as a response to an MLME-CHANNELAVAILABILITYQUERY.indication primitive.

6.3.af4.4.4 Effect of receipt

This primitive initiates transmission of a response to the specific peer MAC entity that requested channel query.

8. Frame formats

8.4 Management frame body components

8.4.5 Registered Location Query Protocol elements

TGaf Editor: Change the Table 8-35af1, as shown:

Table 8-35af1 – Registered Location Query Protocol info ID definitions

| Info Name | Info ID | RLQP Info element (clause) |
| --- | --- | --- |
| Reserved | 0 | N/A |
| DSE Enablement | 1 | 8.4.5.1 |
| Channel Power Management | 2 | 8.4.5.2 |
| Reserved Channel Availability Query | 3 – 220 3 | N/A 8.4.5.3 |
| Reserved | 4 – 220 | N/A |
| Vendor Specific | 221 | 8.4.2.28 |
| Reserved | 222 – 255 | N/A |

8.4.5.1 DSE Enablement element

TGaf Editor: Change the Figure 8-35af2 (DSE Enablement frame body format), as shown:

|  | Info ID | Length | RequesterSTA Address | ResponderSTA Address | Reason Result Code | Enablement Identifier |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 6 | 6 | 1 | 2 |

|  | Device Identification Info | White Space Map |
| --- | --- | --- |
| Octets: | Variable | Variable |

Figure 8-35af2—DSE Enablement frame body format

TGaf Editor: Insert the following text in line 23, page 23:

Device Identification Info field has a format as specified in 8.4.5.3.

TGaf Editor: Insert the following new subclause after 8.4.5.2 in line 62 of page 23, as shown:

8.4.5.3 Channel Availability Query element

The Channel Availability Query element of RLQP is used to exchange the requests and responses for the Channel Availability Query process using the GAS protocol. The Channel Availability Query element is included in a GAS Query Request or returned in a response to a GAS Query Request.

The element is in the format shown in Figure 8-45af4 (Channel Availability Query element format).

|  | Info ID | Length | RequesterSTA Address | ResponderSTA Address | Reason Result Code | Channel Query Info |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 6 | 6 | 1 | 2 |

|  | STA LCI | Device Identification Info | WSM element body fields |
| --- | --- | --- | --- |
| Octets: | 18 | Variable | Variable |

Figure 8-45af4 Channel Availability Query element format

The Info ID field is set to the value for Channel Availability Query defined in Table 8-35af1.

The Length field indicates the length of the remaining element fields in octets, and the value is variable.  The minimum value of the Length field is 15.

The RequesterSTAAddress field is the MAC address of the requesting STA that initiates the channel query process. The length of the RequesterSTAAddress field is 6 octets.

The ResponderSTAAddress field is the MAC address of the responding STA that responds to the channel query requesting STA. The length of the ResponderSTAAddress field is 6 octets.

The Reason Result Code field is used to indicate the reason that a Channel Availability Query was generated. The length of the Reason Result Code field is 1 octet. The reason result codes that have been allocated are shown in Table 8-45af2 (Reason Result Code field values).

Table 8-45af2—Reason Result Code field values

| Reason Result Code field value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Channel Availability List requested |
| 2 | Reserved |
| 3 | Success with the Available Channel List result |
| 4 | Request declined |
| 5 | Request not successful because of Device ID verification failure |
| 6 | Request not successful as one or more parameters have invalid values |
| 7 | Handshake timeout |
| 8-255 | Reserved |

The Channel Query Info field is used to indicate the type of Channel Query request and associated parameters. The format of the Channel Query Info field is shown in Figure 8-45af5.

|  | B0 B1 | B2 B4 | B5 | B6 B7 |
| --- | --- | --- | --- | --- |
|  | STA Type | Location Type | Device ID Present | Reserved |
| Bits: | 2 | 3 | 1 | 2 |

- Figure 8-45af5—Channel Query Info field format

STA Type subfield is 2 bits in length set to an integer that specifies the type of station’s mode of operation, and is defined in Table 8-45af3 (STA Type subfield).

Table 8-45af3— STA Type subfield

| STA Type value | Description |
| --- | --- |
| 0 | Personal/portable AP STA |
| 1 | Personal/portable non-AP STA |
| 2 | Fixed STA |
| 3 | Reserved |

Location Type subfield is 3 bits in length that specifies the type of location format provided for the STA LCI field contained in the Available Channel Query element. When Location Type value is set to 0, STA LCI is not provided.

Table 8-45af4— Location Type subfield

| Location Type value | Description |
| --- | --- |
| 0 | Location information is not provided |
| 1 | Location in LCI format |
| 2-6 | Reserved |
| 7 | Vendor-specific location information |

Device ID Present subfield set to 1 indicates that the Device Identification Info field is present in the Channel Availibility Query element; otherwise it is set to 0 to indicate that the Device Identification field is not provided.

The format of the STA LCI field is provided by the first 128 bits of DSE Registered Location element in 8.4.2.54, in which the last 5 bits (B123 – B127) remains reserved. The STA LCI contains the location of the STA requesting the Channel Availability Query, which is taken from the dot11STALCITable MIB object. When the STA Type subfield of the Channel Query Request Info field is not set to Fixed STA, the altitude information (Altitude Type, Altitude Resolution, Altitude Fraction and Altitude Integer subfields) of the STA LCI field remains unused.

The Device Identification Info field is used to indicate the identification of the station requesting the channel availability query. The format of the Device Identification Info field is as shown in Figure 8.45af6. The details of the type, length and value subfields are country-specific, which are specified in Annex E.

| Device Identification Type | Length | Device Identification Value |
| --- | --- | --- |
| Octets: 1 | 1 | Variable |

- Figure 8.45af6— Device Identification Info field format.

The remaining fields are as described in the White Space Map element (see 8.4.2.af1).

TGaf Editor: Change the Table 8-131 in page 24 as shown:

Table 8-131 Public Action field values

| Action field value | Description |
| --- | --- |
| 9(724r3) | Network Channel Enablement |
| <ANA>(472r1) | White Space Map Announcement |
| <ANA>(767r1) | Channel Power Management Announcement |
| <ANA> | Channel Availability Query |
| <ANA>-255 | Reserved |

TGaf Editor: Insert the following text in line 32, page 24 below the Table 8-131:

8.5.8.4 DSE Enablement frame format

Change the Table 8-134(Reason Result Code field values) as shown:

Table 8-134—Reason Result Code field values

| Reason Result Code field value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Reserved |
| 2 | Enablement requested |
| 3 | Success |
| 4 | Request declined |
| 5 | Request not successful as one or more parameters have invalid values |
| 6 | Enablement denied because the enabling STA is unable to handle additional dependent STAs |
| 7 | Handshake timeout |
| 8 | Enablement requested with device ID information for verification |
| 9 | Enablement denied because of Device ID verification failure |
| 8 10-255 | Reserved |

TGaf Editor: In 8.5.8, insert the following text in page 26, line 28:

- 8.5.8.af4 Channel Availability Query frame format

The Channel Availability Query frame is an Action frame. It is transmitted by a STA as part of channel query. The format of the Channel Availability Query frame Action field is shown in Figure 8.58af4 (Channel Availability Query Action field format).

|  | Category | Public Action | RequesterSTA Address | ResponderSTA Address | Reason Result Code | Length |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 6 | 1 | 1 |

|  | Channel Query Info | STA LCI | Device Identification Info | WSM element body fields |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 18 | Variable | Variable |

- Figure 8-58af4—Channel Availability Query Action field format

The Category field is set to the value for public action defined in Table 8-36 (Category values).

The Public Action field is set to indicate a Channel Availability Query frame, as defined in Table 8-131 (Public Action field values).

The Length field indicates the length of the remaining frame fields in octets, and the value is variable. The minimum value of the Length field is 2.

The remaining fields are as described in the Channel Availability Query element (see 8.4.5.3) and White Space

Map element (see 8.4.2.af1).

TGaf Editor: In page 26, change the Table 8-143 (Protected Dual of Public Action field values) as shown:

Table 8-143 – Protected Dual of Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Protected Channel Power Management Announcement |
| <ANA> | Protected Channel Availability Query |

TGaf Editor: In page 26, insert the following text at the end of 8.5.11:

Insert the following clause at the end of 8.5.11.7, before 8.5.11.8, as shown:

8.5.11.7af2 Protected Channel Availability Query frame format

The Protected Channel Availability Query frame format is the same as the Channel Availability Query frame format (see 8.5.8.af4).  It is used instead of the Channel Availability Query frame when Management Frame Protection is negotiated.

10 MLME

10.12 DSE procedures

10.12.1 General

TGaf Editor: at the end of clause 10.12.1, insert text as shown:

10.12.2.2 Enablement requester STA

Insert the following text after the last paragraph of 10.12.2.2

When RLQP is employed by an Enablement requester STA, it may use GAS public action frame with RLQP protocol containing a DSE Enablement element as described in 10.12.6, instead of the procedure described in this subclause.

10.12.2.3 Enablement responder STA

Insert the following text after the last paragraph of 10.12.2.3

The Enablement requester STA may send the enablement request using GAS public action frame, in which case the enablement responder STA shall use the RLQP protocol containing a DSE Enablement element as described in 10.12.6.

10.12.6 DSE Registered Location Query Protocol procedures

10.12.6.1 General

TGaf Editor: In page 33, move the existing text in 10.12.6 to a new subclause 10.12.6.1 General and Insert the following text at the end of subclause 10.12.6:

10.12.6.2 Channel Availability Query Procedures

Channel Availability Query procedure using RLQP utilizes the registered location server (RLS) for obtaining the list of available channels.

NOTE 1- The RLS is a functional entity not necessarily capable to transmit or receive WLAN signals, but it is connected to the station that advertises RLQP by any connectivity means, wireless or wireline.

A Fixed STA or a personal/portable AP STA should perform the Channel Availability Query process before it can select a channel to begin its transmissions.

The Channel Query requesting STA accomplishes this by transmitting a RLQP element with Info ID for Channel Availability Query in the Query Request field in a GAS Initial Request frame. The responding STA address in such frames is set to the address of the STA that advertises support for RLQP, which is the RLQP responding STA that forwards the query request to the RLQP advertisement server. The details of the procedures for Channel Availability Query requesting STA and Channel Availablity Query Responding STA are provided in the subclauses 10.12.6.2.1 and 10.12.6.2.2.

10.12.6.2.1 Channel Availability Query Requesting STA

A STA employing RLQP may use GAS protocol for its Channel Query process with a RLS. Upon receipt of the MLME-GAS.request primitive for sending a GAS query request to a peer station for the purpose of Channel Availability Query, the STA performs the procedures as described in 10.23.3.1.1. The channel availability query requesting STA transmits a RLQP element with Info ID for Channel Availability Query in the Query Request field in a GAS Initial Request frame. The specific information items in the Query Request field of the GAS Initial Request frame are generated based on the requirements of a station, as follows.

  - Reason result code = 1

  - STA identity assertion (in RequesterSTAAddress and the Device ID field)

  - STA location assertion (in STA LCI field)

  - Channel Query Info field and other optional fields are used to indicate the type of channel query request and other related parameters as described in 8.4.5.3.

A STA may provide a vendor specific location information and device identification when the parameters to establish identity and location mapping of the requesting station to the RLS are based on any vendor-specific ways.

A personal/portable AP STA shall provide their location information to the RLS in order to request channel availability query. A personal/portable STA should also provide its device identification to the RLS in the Device ID field in a format indicated in the Device ID Type subfield of the Channel Availability Query Request Info field, which is subject to the regulation in its region of operation.

10.12.6.2.2 Channel Query Responding STA

A STA employing RLQP may use GAS protocol for its Channel Query process with a RLS. Upon receipt of the MLME-GAS.response primitive for sending a GAS query response to a peer station for the purpose of Channel Availability Query, the STA performs the procedures as described in 10.23.3.1.2. The channel availability query responding STA transmits a RLQP element with Info ID for Channel Availability Query in the Query Response field in a GAS Initial Response frame or one or more GAS Comeback Response frames. The specific information items in the Query Response field of the GAS Initial Response frame are generated based on the requirements of a station, as follows.

  - STA identity assertion (in RequesterSTAAddress)

  - The result of the channel availability query as defined in 8.4.5.3.

  - WSM element

NOTE - The RLS is a functional entity which is accessible from a STA that had transmitted Advertisement Protocol element containing the Protocol ID for RLQP. For the Channel Availability Query process, RLS provides the Query Response with the list of available channel for the location provided by the requesting STA. In some regulatory domains, the RLS may perform access to external databases to derive its response for channel query request and the associated information it shall provide in its Query Response.

TGaf editor: Insert the following new row in B.4.af:

### B.4.af TVWS functions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| *WS1 | Fixed STA TVWS Operation | 10.12.3, Annex D, Annex E.2.4 | CFaf:O | Yes, No. N/A |
| *WS2 | Master STA TVWS Operation | 10.12.3, 10.12.4, Annex D, Annex E.2.4 | CFaf:O | Yes, No. N/A |
| *WS3 | Client STA TVWS Operation | 10.12.5, Annex D, Annex E.2.4 | CFaf:O | Yes, No. N/A |
| WS3.1 | Dependent STA TVWS Behavior | 10.12.5, Annex D, Annex E.2.4 | WS3:M | Yes, No. N/A |
| WS4 | White Space Map Announcement | 8.4.2.af1, 8.5.8.af1, 10.af2.2 | CFaf: M | Yes, No. N/A |
| WS5 | Multi-band Operation | 8.4.2.af2, 10.af2.3 | CFaf:O | Yes, No. N/A |
| WS6 | Channel Power Management Announcement | 8.4.2.af1, 8.5.8.af2, 10.af1, Annex E.2.4 | CFaf:M | Yes, No. N/A |
| WS7 | Channel Availability Query | 8.4.5.3, 8.5.8.af4, 8.5.11.7af2, 10.12.6.2, | CFaf:M | Yes, No. N/A |

Annex C

(normative)

ASN.1 encoding of the MAC and PHY MIB

TGaf Editor: Insert the following text at the beginning of the Annex C in page 73, line 9, as shown:

C.3 MIB Detail

Insert the following entry at the end of the “dot11smt OBJECT IDENTIFIER ::= { ieee802dot11 1 }” of “Major Sections” in Annex C as follows:

-- dot11STALCITable ::= { dot11smt <ANA> }

TGaf Editor: Insert the following entries at the end of dot11StationConfigEntry sequence List:

dot11WSSTAType 		INTEGER,

TGaf Editor: Insert the following elements after dot11ChannelPowerManagementActivated in Annex C as shown:

dot11WSSTAType OBJECT-TYPE

SYNTAX INTEGER {personal/portable AP STA (1), personal/portable non-AP STA(2), Fixed STA (3), reserved (4)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

This attribute specifies the type of station mode for the purpose of channel availability query requirements. This value is used to set the desired channel availability query request parameters in using GAS frames carrying channel availability query element of RLQP. When set to personal/portable AP STA, the station indicates that it operates as a personal/portable AP STA after channel availability query. When set to personal/portable non-AP STA, the station indicates that it operates as a personal/portable non-AP station after channel availability query. When set to fixed STA, the station indicates that it operates as a fixed STA after channel availability query. When set to any other values, its use is reserved, and remains unspecified."

::= { dot11WSSTATypeEntry <ANA>}

Insert the following dot11STALCITable elements in Annex C:

-- ********************************************************************

-- * dot11STALCI TABLE

-- ********************************************************************

dot11STALCITable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11STALCIEntry

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This table represents the geolocation of the STA as specified in clause       10.12.6.2.”

::= { dot11imt <ANA> }

dot11STALCIEntry OBJECT-TYPE

SYNTAX Dot11STALCIEntry

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“STA’s location information in Geospatial coordinates”

INDEX {dot11STALCIDIndex }

::= { dot11STALCITable 1 }

Dot11STALCIEntry ::=

SEQUENCE {

dot11STALCIIndex 				Unsigned32, dot11STALCILatitudeResolution 		Unsigned32,

dot11STALCILatitudeInteger 			Integer32, dot11STALCILatitudeFraction 		Integer32, dot11STALCILongitudeResolution 		Unsigned32, dot11STALCILongitudeInteger 		Integer32,

dot11STALCILongitudeFraction 		Integer32, dot11STALCIAltitudeType 			INTEGER, dot11STALCIAltitudeResolution 		Unsigned32, dot11STALCIAltitudeInteger 			Integer32, dot11STALCIAltitudeFraction 		Integer32,

dot11STALCIDatum 				INTEGER }

dot11STALCIIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for STA LCI elements in dot11STALCITable, greater than 0."

::= { dot11STALCIEntry 1 }

dot11STALCILatitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Latitude resolution is 6 bits indicating the number of valid bits in the fixed-point value of Latitude. This field is derived from IETF RFC 3825, and is accessed big-endian.”

::= { dot11STALCIEntry 2 }

dot11STALCILatitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Latitude is a twos-complement 34 bit fixed point value consisting of 9 bits of integer and 25 bits of fraction. This field contains the 9 bits of integer portion of Latitude. This field is derived from RFC-3825, and is accessed big-endian.”

::= { dot11STALCIEntry 3}

dot11STALCILatitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Latitude is a twos-complement 34 bit fixed point value consisting of 9 bits of integer and 25 bits of fraction. This field contains the 25 bits of fraction portion of Latitude. This field is derived from RFC-3825, and is accessed big-endian.”

::= { dot11STALCIEntry 4}

dot11STALCILongitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Longitude resolution is 6 bits indicating the number of valid bits in the fixed-point value of Longitude. This field is derived from RFC-3825, and is accessed big-endian.”

::= { dot11STALCIEntry 5}

dot11STALCILongitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Longitude is a twos-complement 34 bit fixed point value consisting of 9 bits of integer and 25 bits of fraction. This field contains the 9 bits of integer portion of Longitude. This field is derived from RFC-3825, and is accessed big-endian.”

::= { dot11STALCIEntry 6}

dot11STALCILongitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Longitude is a 2’s complement 34 bit fixed point value consisting of 9 bits of integer and 25 bits of fraction. This field contains the 25 bits of fraction portion of Longitude. This field is derived from IETF RFC 3825, and is accessed big-endian.”

::= { dot11STALCIEntry 7}

dot11STALCIAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2),

hagm (3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Altitude Type is four bits encoding the type of altitude. Codes defined are: meters in 2s-complement fixed-point 22-bit integer part with 8-bit fraction floors: in 2s-complement fixed-point 22-bit integer part with 8-bit fraction hagm: Height Above Ground in meters, in 2s-complement fixed-point 22-bit integer part with 8-bit fraction. This field is derived from IETF RFC 3825, and is accessed big-endian.”

::= { dot11STALCIEntry 8}

dot11STALCIAltitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Altitude resolution is 6 bits indicating the number of valid bits in the altitude. This field is derived from IETF RFC 3825, and is accessed big-endian.”

::= { dot11STALCIEntry 9}

dot11STALCIAltitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-2097151..2097151)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Altitude is a 30 bit value defined by the Altitude type field. The field is encoded as a 2s-complement fixed-point 22-bit integer Part with 8-bit fraction. This field contains the fixed-point Part of Altitude. This field is derived from IETF RFC 3825, and is accessed big-endian.”

::= { dot11STALCIEntry 10}

dot11STALCIAltitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-127..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Altitude is a 30 bit value defined by the Altitude type field. The field is encoded as a 2s-complement fixed-point 22-bit integer Part with 8-bit fraction. This field is derived from IETF RFC 3825, and is accessed big-endian.”

::= { dot11STALCIEntry 11 }

dot11STALCIDatum OBJECT-TYPE

SYNTAX INTEGER (0..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable. It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation. Datum is an 8-bit value encoding the horizontal and vertical references used for the coordinates given in this LCI. IETF RFC 3825 defines the values of Datum. Type 1 is WGS-84, the coordinate system used by GPS. Type 2 is NAD83 with NAVD88 vertical reference. Type 3 is NAD83 with Mean Lower Low Water vertical datum. All other types are reserved. This field is derived from IETF RFC 3825, and is accessed big-endian.”

::= { dot11STALCIEntry 12 }

-- **********************************************************************

-- * End of dot11STALCI TABLE

-- **********************************************************************

Annex E (normative) Country elements and operating classes

E.2 Band-specific operating requirements

E.2.4 TVWS band in the United States (54 MHz – 698 MHz)

E.2.4.1 General

TGaf Editor: Change following text in the second paragraph E.2.4.1:

In TVWS, additional requirements are amended to the definitions, which are described in clause 3, especially for the definition of enabling station (STA) and dependent station (STA). Enabling STAs and dependent APs in TVWS determines the available TV channels at its location using its own geographic location identification and TV bands database access capabilities. Dependent STAs in TVWS receives available TV channel list from the enabling STA or the dependent AP of that enabling STA that enables its operation.

TGaf Editor: Change second sentence of fourth paragraph as shown:

Regulations specify that:

- A registered STA "must" not operate as an enabling STA until the database provides TVWS channels to attempt operation on. After receiving a list of TVWS channels from the database, a registered STA may act as a "Master" STA, and control the operation of dependent STAs.

- Dependent non-AP STAs and dependent APs are “Client” STAs,

- “Client” STAs are not registered, and they “must” operate under the control of an enabling STA.

TGaf Editor: Insert the following subclause at the end of E.2.4:

E.2.4.af1 Device Identification Info Description

Device Identification Info field (see 8.4.5.3) has the following Type, Length and Value subfields, as defined in the Table E-24af2 (Device Identification Info Field Details).

Table E-24af2— Device Identification Info field in the USA

| Device Identification Type | Lengh | Device Identification Value |
| --- | --- | --- |
| 0 | 14 | FCC ID (Device Identification contains 14 octet FCC ID) |
| 1 | 18 | FCC ID and Device Serial Number (Device Identification contains 14 octet FCC ID and 4 octet device serial number) |
| 2-220 | N/A | Reserved |
| 221 | Variable | Vendor specific Device Identification |
| 222-255 | N/A | Reserved |