IEEE P802.11
Wireless LANs

| D0.1 PHY comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation |  |  | assaf.kasher@intel.com |

Abstract

This document proposes resoltions to comments on Draft 0.1 of TGad classified as PHY commnets.

# PHY related Comments

| 101 | 3.195 | 24 | 18 | TR | Definition of antenna should include antenna diversity with a pair of single element antennas, only one of which is active at any time. | "a single element antenna, a pair of single element antennas, only one of which is active at any time, " or . . . |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution: Reject

Discussion:  The definition of antenna is directed at the beamforming protocol.  A single pair of single element antennas, one of which is active at any time is consideted to be two antennas.

| 379 | 303 | 18 | ER | Phrase "transmission completion of the PLCP header the local MAC entity" is incorrect. | Change to "… PLCP header of/in the local MAC entity" |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

TGad Editor: change P303L18 as follows:

transmission completion of the PLCP header to the local MAC entity

| 490 | 305 | 28 | ER | "a index" | change to "an index". |
| --- | --- | --- | --- | --- | --- |
| 491 | 306 | 8 | ER | "followed by another PPDY with ….." | change to "followed by another PPDU with …..". |

Proposed Resolution: Accept

| 492 | 306 | 13 | ER | TRN-T-Packet indicates…..TRN-R subfields…... | change to "TRN-T-Packet indicates…..TRN-T subfields…...". |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

| 390 | 306 | Row 6 | ER | In Table 64, the unit for TRN-LEN is not mentioned. Although this is explained later in clause 21.8.2.2.2, it is still helpful to give the unit in the table, or at lease provide a link to the corresponding clause. | In the table, mention "A value of N indicates that the AGC has 4N subfields and that the TRN-R/T field has 5N subfields." and include a link to 21.8.2.2.2, similar to what is done in Tables 75 and 78. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

| 494 | 308 | 11 | ER | unclear words "CP" and "SC" | change to "Control PHY (CP), Single Carrier (SC)". |
| --- | --- | --- | --- | --- | --- |
| 495 | 308 | 11 | ER | Grammatical error, "The section below describes the common requirement from all 4 PHYs:" | change to "The section below describes the common requirments for all 4 PHYs:". |

Proposed resolution: Counter

TGad Editor: Add Make the following change to P308L11

Change all 4 PHYs to all 4 mmWave PHYs

TGad Editor: Add SC and CP to the list of acronyms.

| 391 | 309 | 26 | ER | The Note is unclear. I think it refers to over-the-air measurements but that is not how it is written. The sentences also have some grammar errors. | Clarify the wording of this note. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Counter

TGad Editor: Change the paragaph starting P309L26

NOTE – For RF power measurements performed over the air, the input level shall be corrected to compensate for the antenna gain in the implementation. The gain of the antenna is the maximum estimated gain by the manufacturer. In the case of the phased-array antenna, the gain of the phased-array antenna is the maximum sum of estimated element gain minus 3 dB implementation loss.

| 46 | 309 | 2-3 | TR | The measurement bandwidth must be provided for this requirement | Specify the measurement bandwidth |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Counter

Discussion: The first requirement (-23dB) is independent of the measurement bandwidth, since it referred to “the overalltransmitted power”. For the second requirement (OFDM) RBW should be defined.

TGad Editor: change P309L2-3 as follows:

The transmitter center frequency leakage shall not exceed –23 dB relative to the overall transmitted power, or, equivalently, +2.5 dB relative to the average power of a subcarrier, measured over a subcarrier spacing bandwidth (in OFDM).

| 393 | 310 | 14 | ER | Second column of table 67 is called OFDM, but is also contains single carrier values. Table overlaps with table 66. | Rename column to ''value" and fix the overlap. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Accept

| 392 | 310 | Table 67 | ER | Row marked with "TDFT: IDFT/DFT period" in fact only corresponds to OFDM PHY mode. | Replace the row description to "TDFT: OFDM IDFT/DFT period" |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:  Accept

| 488 | 310 |  | ER | The tables overlap with each other | Align the tables properly. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:  Accept

| 2 | 313 | 1 | ER | Figure 133 is only valid for SC Packets not for general case. | Since there is no reference to Figure 133, delete it. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Counter

TGad Editor: Add “an example for SC” to the caption.  Add a reference to figure 133 after “two parts” in P313L29

| 394 | 313 | 9 | TR | It is not mentioned whether the use of windowing is required or what the value of the rolloff time TR should be. | Add that the choice of the rolloff time TR is up to the implementer and that it may be set to 0. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

TGad Editor: Add the following text after P313L22:

The choice of windowing function is implementation dependent, as long as transmit EVM and transmit mask requirements are met.

| 496 | 313 | 26 | TR | "The preamble …….. indication of modulation (SC or OFDM)",please elaborate more on the indication. Since the format of the preamble is common to both OFDM packet and SC packet, how does the preamble indicate the difference? | suggest explaination. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Reject

Discussion: The way the modulation type is indicated is discussed in the paragraph beginning at P314L15.

| 47 | 313 | 9-12 | TR | Is it not best to specify the maximum extent of the overlap zone ? | Specify the maximum value of TR |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

See resolution to CID394

| 441 | 314 | 6 | TR | Number of Ga128 repetition was 15 in the presentation (10/440r2). | Please confirm it. |
| --- | --- | --- | --- | --- | --- |
| 48 | 314 | 11 | ER | As specified in the formula, Ga128 is repeated 15 times | Change n=0,1,...,14 to n=0,1,...,13 and also adapt next line |

Proposed Resolution: Reject

The text describes 14 repetitions of Ga128 plus one repetition of –Ga128, which is coherent with the forumula.

| 1 | 314 | 16 | ER | First paragraph is confusing. For example Gv128 is defined but not Gu128. Indictaing that the last 128 samples of Gu512(n) are 18 equal to the last 128 samples used in the Short Training field is unnecessary. | Change the text as: "The Channel Estimation field is used for channel estimation, as well as indication which modulation is 16 going to be used for the packet. Concatenation of two complementary Golay sequences, Gu512(n) and Gv512(n) , followed by -Gb(128.) The Gu512 and Gv512 Golay sequences are defined as, ..., .... where Ga128 and Gb128 are as defined in 21.9. When the data field of the packet is modulated using single carrier, the Gu512 and Gv512 fields are 28 concatenated in the order illustrated in Figure 134. When the data field of the packet is modulated 29 using OFDM, the Gu512 and Gv512 fields are concatenated in the order illustrated in Figure 135. " Use -Gb128 instead of Gv128 in Figure 133, 134, 135 |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

Discussion: The choice of using Gv128, is there to emphasize it used as a cylic postfix.  However, Gu128 has not been defined.

TGad editor: change the paragraph starting at P314L16 as follows:

The Channel Estimation field is used for channel estimation, as well as indication which modulation is going to be used for the packet. The Channel Estimation field is composed of a concatenation of two complementary Golay sequences, Gu512(n) and Gv512(n) where the last 128 samples of Gu512(n) and Gv512(n) are equal to the last 128 samples used in the Short Training field. They are followed by a 128 samples sequence Gv128(n) equal to the first 128 samples of both Gv512(n) and Gu512(n).

TGad editor: in figure 135, replace Gu128 with Gv128

| 395 | 314 | 1-2 | ER | While the caption of Fig. 133 is general, it only shows the preamble for SC PHY, and not for OFDM. In OFDM the position of Gu512 and Gv512 are swapped and Gv128 is replaced by Gu128. | Either add a new figure for OFDM, or change the caption to "The SC preamble" |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

See resolution to CID2

| 396 | 315 | 1 | ER | In Fig. 134, the labels "Short Training Field" and "Channel Estimation Field" are missing. | Add these labels underneath the diagram. |
| --- | --- | --- | --- | --- | --- |
| 499 | 315 | Figure 134 | ER | Miss text illustration for "Short Training Field" and "Channel Estimation Field" | Add the texts |
| 502 | 315 | 18 | ER | "wave form" | change to "waveform" |

Proposed Resolution: Accept

| 500 | 314 | 7 | TR | The SFD field should be specified clearly | Specify SFD filed and value |
| --- | --- | --- | --- | --- | --- |
| 501 | 314 | 7 | TR | The SFD length is short which requires relatively high SNR for frame synchronization. | Change the length of SFD |

Proposed Resolution: Reject

Discussion: the CEF doubles as an SFD, there is no need for the overhead associated with an additional SFD.

| 397 | 315 | 1-4 | ER | Discrepancy between Figs. 133, 134, 135: In Figs. 134, 135, the CEF does not include the last 128 chips corresponding to Gu128/Gv128, but in Fig. 133 CEF does include Gv128. The same discrepancy exists in the text of 21.3.6.2 and the equation describing the CEF waveform. | Throughout 21.3.6 (and the whole spec), use the same convention as to whether include Gv128/Gu128 as part of the CEF or leaving it out of this field. The former would be preferable. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

TGad editor: Change figures 134 and 135 so that the CEF covers the last 128 samples.

.

| 398 | 315 | 1-4 | TR | There is no point for defining two terminologies Gu128 and Gv128, while both refer to an identical sequence (-Gb128). In fact Gu128 is not mentioned in the body of 21.3.6.2, causing an inconsistency. | Eliminate Gu128, and only use the term Gv128 to indicate the last 128 chips of both the SC and OFDM preambles. Alternatively, eliminate both Gu128 and Gv128, and simply use the term -Gb128. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

See resolution to 1

| 49 | 315 | 7-9 | ER | What is the meaning of the superscript R ? | Indicate the meaning of R or remove it |
| --- | --- | --- | --- | --- | --- |

Proposed Reslution: Counter

TGad Editor: Remove the R superscript

| 504 | 317 | 13, Table | TR | The table does not give the LDPC code matrix but the value of i which indicates the cyclic-permutation of original identitiy matrix by shifting i columns to the right. | Rename the Table and provide clear explanations |
| --- | --- | --- | --- | --- | --- |
| 505 | 317 | 17, Table | TR | The table does not give the LDPC code matrix but the value of i which indicates the cyclic-permutation of original identitiy matrix by shifting i columns to the right. | Rename the Table and provide clear explanations |
| 503 | 317 | 9, Table 6 | TR | The table does not give the LDPC code matrix but the value of i which indicates the cyclic-permutation of original identitiy matrix by shifting i columns to the right. | Rename the Table and provide clear explanations |
| 506 | 318 | 2, Table 7 | TR | The table does not give the LDPC code matrix but the value of i which indicates the cyclic-permutation of original identitiy matrix by shifting i columns to the right. | Rename the Table and provide clear explanations |

Proposed Resolution: Counter

TGad editor: in each of the captions to tables 69, 70, 71 and 72 add the following:

Each non blank element i in the table is the cylic permutation matrix Piof size Z×Z.  Blank entries represent the zero matrix of size Z×Z.

| 347 | 318 | 17 | TR | The transmitter selects values for bit x1 through x7 of the scrambler. The selection process is not defined. | Seed value is set of all zeros when PHY is initialized and incremented in a rollover counter for each frame that is sent by the PHY. |
| --- | --- | --- | --- | --- | --- |
| 507 | 318 | 18 | ER | Not clear | Clarify |

Proposed resolution: counter

TGad editor: change the following text in P318L17 as follows:

The values selected for bits x1 through x7 should be set to pseudo-random non-zero state.

| 462 | 318 | 28 | ER | It shall be clearly stated that the control PHY is just the MCS 0 mentioned in previous clauses | make a clear statement in the introduction of control PHY. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

TGad Editor: Add the following text at the end of P318L28:

Control PHY is transmitted when the RXVECTOR indicates MCS 0.

| 498 | 319 | 6 | TR | The STF consists of 38 repetitions of Gb128 (for frame detection), and a single -Gb128 sequence (for synchronization). Is this single -Gb128 good enough for synchronization? | suggest to increase the number of repetition of -Gb128 for synchronization purpose. |
| --- | --- | --- | --- | --- | --- |
| 4 | 319 | 23 | TR | Only two sequences in STF for synchronization is too short. | Use V512 for synchronization. |
| 508 | 319 | 23 | TR | The SFD field should be specified clearly | Specify SFD filed and value |
| 509 | 319 | 23 | TR | The SFD length is short which requires relatively high SNR for frame synchronization. | Change the length of SFD |

Proposed Resolution: Reject

There is no SFD.  These two sequences are used as cyclic prefix for the channel estimation.  The CE field is used also for channel estimation and timing.

| 50 | 319 | 26 | ER | As specified in the formula, -Gb128 and-Ga128 is repeated twice | Correct value of n in formula |
| --- | --- | --- | --- | --- | --- |
| 399 | 319 | 26 | TR | Lengths are inconsistent with the figure and the paragraph above. Since there are 38 repetitions of Gb128, in the first line we should have n= 0,1,…,38x128-1 Similarly, other limits should change accordingly. | Change the limits to n= 0,1,…,38x128-1, n= 38x128,…,39x128-1, n= 39x128,…,40x128-1, respectively. |

Proposed Resolution:  Counter

TGad Editor: Change the formula in P319L26 to the following:

| 400 | 320 | Table 73 | TR | Where does the scrambling start? From the bit 5 of the PHY header? Is the PHY header scrambled, as well? | Clarify in the text where scrambling starts for all PHY modes. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

TGad Editor: Change the text in 21.4.3.3.1 (p321L6);

The operation of the scrambler is defined in Error: Reference source not found. Bits x1, x2, x3, x4 of the scrambler shift register are initialized using the bits in the scrambler initialization bits from the header, bits x5, x6, x7 are set to 1.  The header is scrambled starting from bit 5.  The scrambling of the data field continues the scrambling of the header with no reset.

| 401 | 321 | 11 | TR | "an effective LDPC code rate of ½" This is the case only if L_DPCW=LCWD=168. Otherwise, the code rate is less than ½. | Replace the phrase with "an effective LDPC code rate less than or equal to ½" |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Accept

| 344 | 321 | 26 | ER | Did not mention the first five bits of the LDPFCW should not be scrambled (1 reserved bits + 4 scrambler initialization bits) | LDPFCW are scrambled as described in 21.3.9, starting from the sixth bit. |
| --- | --- | --- | --- | --- | --- |

Propsed Resolution: Counter

See resolution to CID400

| 61 | 322 | 17 | ER | I think, there is no symbol corresponding to equality in the equation. | Insert '=' between 'rDATA(nTc)' and '(Ga…)' in the equation. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Accept

| 510 | 322 | 22 | TR | Performance requirements are not given | Provide the performance requirements |
| --- | --- | --- | --- | --- | --- |
| 511 | 322 | 25 | TR | Transmitter performance requirements are not given | Provide the performance requirements |

Proposed Resolution: Counter

TGad editor: Replace the text in P322L22 as follows:

Performance requirements of the Control PHY (CP) are defined in subclauses 21.4.4.1 through 21.4.4.2.

TGad Editor: Replace the text P322L25 as follows:

Transmitter performance requirements of the Control PHY are defined in 21.4.4.1.1.

| 402 | 323 | 6 | TR | EVM equation should have a + instead of - between the squared I/Q error contributions | change - to + |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Accept

| 497 | 324 | 11 | TR | There is no PLCP Header defined in Fig 139. | change the section title to "Header". |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Accept

| 62 | 324 | 18 | ER | In Table 75, the second sentence in the Description of 'Training Length' contains an editorial error. | Replace 'define' with 'defined'. |
| --- | --- | --- | --- | --- | --- |
| 63 | 326 | 9 | ER | Step (5) contains an editorial error. | Remove 'to' between 'continuation' and 'of'. |

Proposed Resolution: Accept

| 463 | 325 | 10 | ER | the "below" in this sentence is very confusing, it is below in the sense of MCS index or in the sense of positions in the table | directly mention MCS index |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

TGad Editor: Change the text in P325L10

MCS 13-17 are mandatory

| 67 | 326 | 23 | TR | The scrambling of the data field continues the scrambling of the header with no reset as defined in 21.5.3.2.1. However, what makes me confused is whether the scrambling of the data field continues that in step (1) or that in step (5), defined in 21.5.3.1.3. In step (4), the shift register is initialized to all ones, followed by the continuation of the scrambling of step (4) in step (5). If the scrambling of the data field continues the scrambling in step (5), it is not affected by the scrambling seed in step (1), which is not efficient in my opinion. That is because when frame retransmission occurs, it is preferable to change the scrambling seed of the data field in terms of data randomization. | Clearly define the start value of scrambler for data field in 21.5.3.2.1, that is, whether the start value comes from step (1) or step (5) (see 21.5.3.1.3). If it is resonable to define the former, then as a change, we recommend the following: The scrambling in step (4) can continues that in step (1). By doing this, the scrambling of the data field can be varying with that of the header field in step (1). |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

Discussion: The intention is that during the encoding of the header a one time pad sequence is used.  It is generated from the same shift register used for the scrambler, however, besides that, it is unrelated.

TGad editor: in P326L5, replace “the one time” with “a one time”

TGad editor: in P326L8, update the text as follows:

- A sequence c3 is generated as (q1,q2,…,qLH,p1,p2,…p160) XORed with the continuation of the one-time pad sequence, generated in step Error: Reference source not found.

| 64 | 326 | 30 | ER | There is an editorial error. | Remove 'is' between 'R' and 'equal'. |
| --- | --- | --- | --- | --- | --- |
| 52 | 331 | 6 | ER | also applies to QPSK ? | replace SQPSK by QPSK or SQPSK |
| 53 | 331 | 6 | ER | also applies to QPSK ? | replace SQPSK by QPSK or SQPSK |
| 489 | 331 | 24 | ER | repeated “number of” | remove one of it. |

Proposed Resolution: Accept

| 54 | 331 | 33-34 | ER | What is the added value of the sentence "The pilot sequence P is created by creating a sequence of zeros corresponding to tones –NSR to NSR" ? | Remove the sentence |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

This sentence is needed to define P which is used in the formula in P332L10.

To avoid confusion the minus (-) before the first NSR should not be allowed to break afterwards (e.g. insert a non-breaking hyphen: Ctrl+Shift+_).

| 66 | 332 | 10,11 | TR | In the Equation of Line 10, DM(k),n can't technically define the value corresponding to each k. For example, in case of k=-1, there is no definition of M(-1). Additionally, dM(k) is an editorial error in the definition of Dk,n in Line 11 | Replace 'DM(k),n' with 'Dk,n' in Line 10. Replace 'dM(k)' with 'dM(k),n' in Line 11. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Accept

| 403 | 333 | 25 | TR | EVM equation should have a + instead of - between the squared I/Q error contributions | change - to + |
| --- | --- | --- | --- | --- | --- |
| 442 | 333 | 25 | TR | Minus "-" in the brackets could be plus "+". | check it. |

Proposed Resolution: Accept

| 55 | 333 | 25 | ER | What is the meaning of P0 | Indicate the meaning of P0 |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Accept

TGad Editor: Add the following after P333L33

P0   is the average power of the constellation