IEEE P802.11
Wireless LANs

| M-MCCA-MLME text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dee Denteneer | Philips | HTC 34; 5656 AE Eindhoven, The Netherlands |  | Dee.denteneer@philips.com |

Abstract

This submission implements the resolutions for the comments from LB 165 with CIDs 4132 and 4133.

- MLME-MCCASETUP.request

- Function

This primitive requests that the MAC entity sets up an MCCA reservation.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MCCASETUP.request(

MCCAOP Duration,

MCCAOP Periodicity,

MCCAOP Responder,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MCCAOP Duration | Integer | 0-65535 | Specifies the MCCAOP Duration of the needed MCCAOPs in multiples of 32 μs. |
| MCCAOP Periodicity | Integer | 0-255 | Specifies the MCCAOP Periodicity of the needed MCCAOPs. |
| MCCAOP Responder | MAC address | Any valid individual MAC address | Specifies the MAC address of the intended MCCAOP responder. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When generated

This primitive is generated by the SME to start an MCCA setup procedure.

- Effect of receipt

This primitive causes the selection of an appropriate offset and the transmission of the MCCA Setup Request provided that the conditions for the transmission are met. The MLME subsequently issues an MLME-MCCASETUP.confirm primitive that reflects the results.

- MLME-MCCASETUP.confirm

- Function

This primitive is generated by the MLME as a result of an MLME-MCCASETUP.request primitive to establish an MCCAOP reservation with a specified peer MAC entity.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MCCASETUP.confirm(

MCCAOP Parameters,

MCCAOPID,

MCCAOP responder,

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MCCAOP Parameters | MCCAOP Reservation | See Error: Reference source not found | The MCCAOP reservation parameters |
| MCCAOPID | Integer | 0-255 | MCCAOPID of MCCA reservation |
| MCCAOP Responder | MAC address | Any valid individual MAC address | Specifies the MAC address of the intended MCCAOP responder. |
| Result Code | Enumeration | SUCCESS, INVALID_PARAMETERS, MCCAOP_RESERVATION_CONFLICT, MAF_LIMIT_EXCEEDED, MCCA_TRACK_LIMIT_EXCEEDED | Indicates the result of the MLME-MCCASETUP.request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When generated

This primitive is generated by the MLME as a result of an MLME-MCCASETUP.request to establish an MCCAOP reservation with a peer mesh STA.

- Effect of receipt

The SME is notified of the results of the MCCA setup procedure.

- MLME-MCCASETUP.indication

- Function

This primitive reports the establishment of an MCCAOP reservation with a specified peer MAC entity.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MCCASETUP.indication(

MCCAOP Parameters,

MCCAOPID,

MCCAOP owner,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MCCAOP Parameters | MCCAOP Reservation | See Error: Reference source not found | The MCCAOP reservation parameters |
| MCCAOPID | Integer | 0-255 | MCCAOPID of MCCA reservation |
| MCCAOP owner | MAC address | Any valid individual MAC address | Specifies the MAC address of the MCCAOP owner. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When generated

This primitive is generated by the MLME as result of the establishment of an MCCAOP reservation with a specific peer MAC entity that resulted from an MCCA setup procedure that was initiated by that specific peer MAC entity.

- Effect of receipt

The SME is notified of the results of the request to start an MCCA setup procedure.

Note to the editor insert the following subclause as 10.3.78.6, and renumber the following clauses accordingly

- MLME-MCCASETUP.response

- Function

This primitive is used to send a response to a specific peer MAC entity that requested that the MAC entity sets up an MCCA reservation.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MCCASETUP.response(

MCCAOP Parameters,

MCCAOPID,

MCCAOP responder,

ResultCode,

VendorSpecificInfo

)

| MCCAOP Parameters | MCCAOP Reservation | See Error: Reference source not found | The MCCAOP reservation parameters |
| --- | --- | --- | --- |
| MCCAOPID | Integer | 0-255 | MCCAOPID of MCCA reservation |
| MCCAOP Responder | MAC address | Any valid individual MAC address | Specifies the MAC address of the intended MCCAOP responder. |
| Result Code | Enumeration | SUCCESS, INVALID_PARAMETERS, MCCAOP_RESERVATION_CONFLICT, MAF_LIMIT_EXCEEDED, MCCA_TRACK_LIMIT_EXCEEDED | Indicates the result of the MLME-MCCASETUP.request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When generated

This primitive is generated by the SME of a SRA as a response to an MLME-MCCASETUP.indication procedure.

- Effect of receipt

This primitive initiates transmission of a response to the specific peer MAC entity that requested the set up of an MCCAOP reservation.

Note to the editor: insert following clause as Clause 10.3.78.7, and renumber following clauses accordingly

- MLME-MCCAADVERTISEMENT.request

- Function

This primitive requests that the MAC entity sends out and MCCA Advertisments Request to the the peer MAC entity.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MCCAADVERTISEMENT.request(

PeerMAC,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC | MAC address | Any valid individual MAC address | Specifies the MAC address of the peer MAC that will send the Advertisment |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When generated

This primitive is generated by the SME request an Advertisment from a specified peer MAC entity.

- Effect of receipt

This primitive causes the transmission of an MCCA Advertisment Request frame to the specified peer MAC entity. The MLME subsequently issues an MLME-MCCAADVERTISEMENT.confirm primitive that reflects the results.

- MLME-MCCAADVERTISEMENT.confirm

- Function

This primitive reports the result of an MLME-MCCAADVERTISEMENT.request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MCCAADVERTISEMENT.confirm(

MCCAOP Advertisements,

Peer MAC,

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MCCAOP Advertisements | MCCAOP Advertisements | See Error: Reference source not found | One or more MCCAOP Advertisments elements |
| Peer MAC | MAC address | Any valid individual MAC address | Specifies the MAC address of the transmitter of the MCCAOP Advertisment |
| Result Code | Enumeration | SUCCESS, INVALID_Advertisment, Advertisment Timeout | Indicates the result of the MLME-MCCAADVERTISEMENT.request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When generated

This primitive is generated by the MLME as a result of an MLME-MCCAADVERTISEMENT.request.

- Effect of receipt

The SME is notified of the results of the MCCA Advertisment Request frame.

- MLME-MCCAADVERTISEMENT.indication

- Function

This primitive reports that an Advertisement Request has been received from a specified peer MAC entity.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MCCAADVERTISEMENT.indication(,

Peer MAC,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC | MAC address | Any valid individual MAC address | Specifies the MAC address of the transmitter of the MCCAOP Request |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When generated

This primitive is generated by the MLME upon receipt of an MCCA Advertisement Request frame from the specified  peer MAC entity.

- Effect of receipt

The SME is notified of the results of the request to advertise the MCCAOP reservations.

- MLME-MCCAADVERTISEMENT.response

- Function

This primitive is used to cause the transmission of an MCCAOP Advertisement frame to the specified peer MAC.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MCCAADVERTISEMENT.response (,

MCCAOP Advertisements,

Peer MAC,

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MCCAOP Advertisements | MCCAOP Advertisements | See Error: Reference source not found | One or more MCCAOP Advertisments elements |
| Peer MAC | MAC address | Any valid individual MAC address | Specifies the MAC address of the transmitter of the MCCAOP Advertisment |
| Result Code | Enumeration | SUCCESS, REFUSED | Indicates the result of the MLME-MCCAADVERTISEMENT.response |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When generated

This primitive is generated by the SME of a STA as a response to an MLME-MCCAADVERTISEMENT.indication procedure.

- Effect of receipt

This primitive initiates transmission of a response to the specified peer MAC entity that requested advertisement of the MCCAOP resrevations.