IEEE P802.11
Wireless LANs

| LB 165 PM comment resolution text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jarkko Kneckt | Nokia Corporation | Rakentajainrinne 6, 02330 Espoo Finland |  | Jarkko.Kneckt@nokia.com |

Abstract

The submission provides normative text for comment resolutions provided in 11-10-848r1.

- Definitions

Add the following new definition and set it to correct alphabetical location

peer trigger frame: A Mesh Data or QoS Null frame that initiates a mesh peer service period.

mesh peer service period (MPSP): A mesh peer service period is a contiguous period of time during which one or more individually addressed frames are transmitted between two peer mesh STAs with at least one of those mesh STAs operating in power save mode. A mesh peer service period is directional and may contain one or more TXOPs. A mesh STA may have multiple mesh peer service periods ongoing in parallel. No more than one mesh peer service period may be set up in each direction with each peer mesh STA.

- Abbreviations and acronyms

Modify the RSPI field as shown.

RSPI 	Receiver Service Period Initiated

- EOSP (end of service period) subfield

Insert the following paragraph to the end of EOSP (end of service period) subfield:

The mesh STA uses the EOSP subfield to indicate the end of the current mesh peer service period (MPSP) in which it operates as the owner. The mesh STA sets the EOSP subfield to 1 in its transmission and retransmissions of the MPSP’s final frame to end a MPSP, and sets it to 0 otherwise.

- Power Management field

Change Error: Reference source not found as follows:

The Power Management field is 1 bit in length and is used to indicate the power management mode of a

STA.

In an infrastructure BSS or in an IBSS the following applies: The value of this field remains constant in each frame from a particular STA within a frame exchange sequence (see Annex S).The value indicates the mode in which the station will be after the successful completion of the frame exchange sequence. A value of 1 indicates that the STA will be in PS mode. A value of 0 indicates that the STA will be in active mode. This field is always set to 0 in frames transmitted by an AP.

In an MBSS the following applies: The value of this field together with the Mesh Power Save Level subfield in the QoS Control field indicates the mesh power mode in which the mesh STA will be after the completion of the frame exchange sequence. A value of 0 in a group addressed frame indicates that the mesh STA will be in active mode. For non-peer mesh STAs, a value of 1 in a group addressed frame indicates that the mesh STA will be in light or in deep sleep mode. For peer mesh STAs a value of 1 in group addressed frame indicates that the mesh STA will be in deep sleep, light sleep or active mode. For the peer mesh STAs the link specific mesh power mode rules are determined by the Power Management field in the individually addressed frames exchanged between these peers, as described in 11C.13.4 (Mesh power mode indications and transitions). In an individually addressed frame, a value of 0 indicates that the mesh STA will be in active mode and a value of 1 indicates that the mesh STA will be in light or deep sleep mode toward the mesh peering after the successful completion of the frame exchange sequence.

- Mesh Power Save Level subfield

The Power Management field and Mesh Power Save Level subfield in the Frame Control field indicate the mesh power mode of the mesh STA. The Power Management field set to 1 and the Mesh Power Save Level subfield set to 0 indicate that the mesh STA is operating in light sleep mode (see 11C.13.9.3 (Operation in light sleep mode)). The Power Management field set to 1 and the mesh Power Save Level subfield set to 1 indicate that the mesh STA is operating in deep sleep mode (see 11C.13.9.4 (Operation in deep sleep mode)). The Mesh Power Save Level subfield is reserved if the Power Management field is 0.

- Receiver Service Period Initiated (RSPI) subfield

The Receiver Service Period Initiated (RSPI) subfield is one bit in length. The subfield is set to 0 to indicate that the mesh peer service period for the receiver of the peer trigger frame is not initiated and set to 1 to indicate that the mesh peer service period is initiated. The use of RSPI subfield in the peer trigger frames is described in 11C.13.10.2 (Initiation of a mesh peer service period).

- Power save in a mesh BSS

- General

A mesh STA shall have the capability to buffer frames and track the mesh power mode of each peer mesh STA. A mesh STA shall use mesh peer service periods for individually addressed frame transmissions to neighboring peer mesh STAs in light or deep sleep mode. A mesh STA transmits a group addressed frame after the DTIM Beacon when any of its peer mesh STAs are in light or deep sleep mode. These capabilities are referred as support for power save.

Operation in light or in deep sleep mode is optional.

NOTE— In this clause, a frame with the RA field set to an individual address is referred to as an individually addressed frame. The DA field of the individually addressed frame in this context might not be set to an individual address, if the frame uses 4 address or 6 address format. Similarly, frames with the RA field set to a group address are referred to as group addressed frames, in this subclause.

- Link specific mesh power modes

A mesh STA is in one of two different power states as defined in 11.2.1.1.

The manner in which a mesh STA transitions between power states is determined by the combination of mesh peering specific mesh power modes. A mesh STA shall be in Awake state if the mesh STA has a mesh peer service period ongoing with any peer mesh STA or if the mesh STA operates in active mode toward any peer mesh STA.

A mesh peering is always associated with two mesh STAs. Both mesh STAs have their own mesh power mode for the mesh peering. A mesh STA defines a mesh power mode in which it operates for the mesh peering and maintains a state about the mesh power mode of the peer mesh STA within the mesh peering. The mesh power modes of mesh peerings are independent and a mesh STA may operate in different mesh power modes for each mesh peering. A mesh STA maintains a mesh power mode for non-peer mesh STAs that is described in Non-peer mesh power modes. An example illustration of the use of mesh power modes is shown in Error: Reference source not found.

Three mesh power modes are defined: active mode, light sleep mode, and deep sleep mode. The used mesh power mode shall be indicated by the Power Management field and Mesh Power Save Level field. The difference between modes is as follows

- Active mode: The mesh STA shall be in Awake state all the time.

- Light sleep mode: The mesh STA alternates between Awake and Doze state as specified in 11c.13.9.3 (operation in light sleep mode). The mesh STA shall listen to all the Beacon frames from its peer mesh STA.

- Deep sleep mode: The mesh STA alternates between Awake and Doze states, as specified in clause 11c.13.9.4 (operation in deep sleep mode). The mesh STA may choose not to listen to the Beacons from its peer mesh STA.

A mesh power mode is defined as the combination of the Power Management field indicated in the Frame Control field and the Mesh Power Save Level as shown in the Mesh power mode definitioninput.doc.

| - Mesh power mode definitioninput.doc |  |  |  |
| --- | --- | --- | --- |
| Activity level | Mesh power mode | Power Management field | Mesh Power Save Level field |
| Highest Lowest | Active mode | 0 | Reserved |
|  | Light sleep mode | 1 | 0 |
|  | Deep sleep mode | 1 | 1 |

Instructions for editor. Change the caption to read: An example of the mesh power modes indications. (The black arrows between mesh STAs indicate mesh peering).

Change the text to read: “mesh STA X operates in active|light sleep| deep sleep mode toward the peer mesh STA Y”where X and Y are A,B or C and mesh power mode as specified in the figure.

- Non-peer mesh power modes

The non-peer mesh power mode indicates the mesh power mode of the mesh STA toward the non-peer mesh STAs. The non-peer mesh power mode is indicated by the Power Management field in the Frame Control field in the Mesh Capability field in Beacon and Probe Response frames. The non-peer mesh power mode is also indicated with the Power Management field in the Frame Control field in group addressed Mesh Data frames.

The non-peer mesh STAs may send Probe Request and Mesh Peering Open Request frames, only during the Awake Window of the mesh STA that has power management field set to 1.

- Mesh power mode indications and transitions

- General

When a mesh STA is operating in active mode for a link, the mesh STA shall set the Power Management bit to 0 in all transmitted individually addressed Mesh Data or QoS Null frames on this link.

When a mesh STA is operating in light sleep mode for a link, the mesh STA shall set the Power Management bit to 1 and the Mesh Power Save Level field to 0 in all transmitted individually addressed Mesh Data or QoS Null frames on this link.

When a mesh STA is operating in deep sleep mode for a link, the mesh STA shall set the Power Management bit to 1 and the Mesh Power Save Level field to 1 in all transmitted individually addressed Mesh Data or QoS Null frames on this link.

The algorithm to trigger the change of mesh power mode is beyond the scope of this standard.

- Transition to a higher activity level

A mesh STA may use group addressed or individually addressed Mesh Data or QoS Null frames to change its mesh power mode to a higher activity level, for example; from deep sleep to light sleep or to active mode; or from light sleep to active mode.

Individually addressed frames may be used to temporarily raise the activity level of the mesh STA for a mesh peering. This may be useful in cases when a link temporarily requires efficient data transmission with the peer mesh STA and the mesh STA desires to be able to transit back to lower activity level without performing the mesh power mode transition signaling with all peer mesh STAs.

- Transition to a lower activity level

A mesh STA shall use acknowledged individually addressed Mesh Data or QoS Null frames to change its mesh power mode for a link to a lower activity level, for example; from active mode to light or deep sleep mode; or from light sleep to deep sleep mode.

If a mesh STA is operating in deep sleep mode for a link or the mesh STA intends to change the mesh power mode of at least one link to deep sleep mode, the mesh STA shall set the Power Management bit to 1 and Mesh Power Save Level field to 1 for all transmitted group addressed frames.

If a mesh STA is not operating in deep sleep mode for any link and the mesh STA is operating in light sleep mode for a link or the mesh STA intends to change its operation mode for at least one link to light sleep mode, the mesh STA shall set the Power Management bit to 1 and Mesh Power Save Level field to 0 in all transmitted group addressed frames.

- TIM transmissions

- in an MBSS

The TIM element identifies the peer mesh STAs operating in light or deep sleep mode for which traffic is pending and buffered in the reporting mesh STA. This information is coded in a partial virtual bitmap using AIDs. In addition, the TIM contains an indication whether group addressed traffic is pending. The mesh STA shall identify those peer mesh STAs for which it is prepared to deliver buffered MSDUs by setting bits in the partial virtual bitmap of the TIM that correspond to the appropriate AIDs.

- TIM types

There are two different TIM types: TIM and DTIM. A mesh STA shall transmit a TIM with every Beacon frame. Every DTIMPeriod, a TIM of type DTIM is transmitted with a Beacon frame. After a DTIM the mesh STA shall send the buffered group addressed MSDUs and MMPDUs, before transmitting any individually addressed frames. The More Data field of each group addressed frame shall be set to indicate the presence of further buffered group addressed MSDUs and MMPDUs. The mesh STA sets the More Data field to 0 in the last transmitted group addressed frame following the transmission of the DTIM Beacon.

If CCA is IDLE for the duration of PHY specific Group Delivery Idle Time when a mesh STA expects to receive a group addressed frame, the receiving mesh STA may assume that no more frames destined to group addresses will be transmitted and return to Doze state. The Group Delivery Idle Time is identical to the TXOP Limit for AC_VI specified by the default EDCA Parameter Set shown in Table 7-37.

- Mesh Awake Window

A mesh STA in light or deep sleep mode shall be in Awake state during its own Mesh Awake Window. An Mesh Awake Window is present after the Beacon and Probe Response frames containing an Mesh Awake Window element. A mesh STA shall include a Mesh Awake Window element in its DTIM Beacon frames and may include a Mesh Awake Window element in its TIM Beacon and Probe Response frames. Mesh STA in light or deep sleep mode shall include a Mesh Awake Window element in its Beacon frame if the Beacon frame indicates buffered traffic for at least one peer mesh STA. The start of the Mesh Awake Window is measured from the end of the Beacon or Probe Response transmission. The duration of the Mesh Awake Window period is defined by the value of the Mesh Awake Window field in the Mesh Awake Window element.

If the mesh STA that has its Mesh Awake Window transmits frames destined to group addresses during its Mesh Awake Window, the duration of Mesh Awake Window is extended by an additional PostAwakeDuration. The PostAwakeDuration follows the group address frame, and the mesh STA that has its Mesh Awake Window shall stay in awake state until it has transmitted all of its group addressed frames and the PostAwakeDuration has expired. The PostAwakeDuration is equal to duration of the Mesh Awake Window.

The peer mesh STAs may send a peer trigger frame during the Mesh Awake Window of the light or deep sleep mode mesh STA in order to initiate a mesh peer service period. A successfully transmitted peer trigger frame initiates a mesh peer service period as described in 11C.13.10.

The first Mesh Data or QoS Null frame that a peer mesh STA transmits during the Mesh Awake Window is a peer trigger frame.

Non-peer mesh STAs may send Probe Request and Mesh Peering Open frames to the light or deep sleep mode mesh STA during its Mesh Awake Window. The light or deep sleep mode mesh STA shall operate in Awake state at least until the completion of the mesh peering management procedure once it is initiated (see Error: Reference source not found and Error: Reference source not found), until the completion of the Authentication procedure once it is initiated (see 8.2a (Authentication using a password)), or the transmission of the Probe Response frame upon the reception of Probe Request frame.

- Power Save Support

As described in 11C.13.2 (Link specific mesh power modes) a mesh STA indicates its mesh power mode for each mesh peering and obtains the mesh power modes of its peer mesh STAs.

A mesh STA shall only transmit frames to mesh STAs operating in a light or deep sleep mode if the conditions defined in 11C.13.9.3, 11C.3.9.4 and 11C.13.10 are met and otherwise the mesh STA shall buffer frames.

As described in 11C.13.5 (TIM transmissions in an MBSS) a mesh STA can indicate the presence of buffered traffic in TIM elements for all peer mesh STAs in light or deep sleep mode. As described in 11C.13.6 (TIM types), a mesh STA transmits its group addressed frames after its DTIM Beacon if any of its peer mesh STA is in light or deep sleep mode.

As described in 11C.13.10 mesh  peer service periods are used for frame transmissions towards a mesh STA that operates in light or deep sleep mode. Mesh Peer service periods are not used in frame exchanges towards active mode mesh STAs.

A mesh STA may initiate a mesh peer service period with a peer mesh STA that operates in deep or light sleep mode by transmitting a peer trigger frame during the Mesh Awake Window of the peer mesh STA.

- Operation in link specific mesh power mode

- General

Detailed operations of mesh STA in each mesh power mode are described in the following subclauses. Mesh power management operation depicts example power state transitions of mesh STAs, when three mesh STAs are operating in the mesh power modes shown in the Error: Reference source not found.

- Mesh power management operation

- Operation in active mode

A mesh STA operating in active mode shall be in Awake state. Mesh peer service periods are not used in frame exchanges toward active mode mesh STAs.

Active mode mesh STA may receive peer trigger frames from a peer mesh STA in light or deep sleep mode at all times when they do not have a mesh peer service period ongoing.

- Operation in light sleep mode

A mesh STA operating in light sleep mode shall enter the Awake state prior to every TBTT that matches the Beacon timing of its own and peer mesh STA. The mesh STA shall remain in Awake state after its Beacon transmission for the duration of the Mesh Awake Window and the duration of its group addressed frame transmissions, whichever occurs later. The mesh STA may return to the Doze state after the Beacon reception from a peer mesh STA, to which it indicated to operate in light sleep mode, if the peer mesh STA did not indicate buffered individually addressed or group addressed frames. If an indication of buffered frames is received, the light sleep mode mesh STA shall send a peer trigger frame with RSPI field set to 1 to initiate a mesh peer service period with the beaconing mesh STA.

If a light sleep mode mesh STA enters the Awake state in order to listen to DTIM Beacon and group addressed transmissions from its peer mesh STA, the mesh STA shall remain Awake until the More Data field of a received group addressed MSDU is set to 0.

If a mesh STA receives a peer trigger frame initiating a mesh peer service period from a peer mesh STA in light sleep for the link, the mesh STA shall remain in Awake state until the mesh peer service period is terminated as defined in 11C.13.10.4. The mesh STA may return to Doze state after its Mesh Awake Window if no peer trigger frame is received during the Mesh Awake Window.

A mesh STA in light or deep sleep mode may receive peer trigger frames during its Mesh Awake Window as described in Mesh Awake Window.

- Operation in deep sleep mode

A mesh STA operating in deep sleep mode shall enter the Awake state prior to every TBTT of its own and shall remain in Awake state after the Beacon transmission for the duration of the Mesh Awake Window and the duration of its group addressed frame transmissions, whichever occurs later. A mesh STA operating in deep sleep mode might not receive Beacon frames from its peer mesh STAs as long as it maintains synchronization with peer mesh STAs. The logic how the mesh STAs in deep sleep mode maintain synchronization among neighbors is beyond the scope of this standard. Guidance for the synchronization maintenance by the mesh STA in deep sleep mode is given in Annex Error: Reference source not found.

A mesh STA in light or deep sleep mode may receive peer trigger frames during its Mesh Awake Window as described in Mesh Awake Window.

- Conditions for Doze State

A mesh STA may enter Doze state if all following conditions are fulfilled:

- the mesh STA operates in light or deep sleep mode for all of its mesh peerings, as described in Operation in light sleep mode or Operation in deep sleep mode

- the mesh STA has no mesh peer service period ongoing, as described in 11C.13.10

- the Mesh Awake Window indicated by the mesh STA has expired, as described in Mesh Awake Window

- the mesh STA has terminated its group addressed frames delivery sequence after its DTIM Beacon, as described in TIM types

Guidance for using the power save in mesh BSS and default parameter values are given in Annex Power Save parameters selection.

- Mesh peer service periods

- General

Mesh peer service periods are used for frame exchange in a link in which at least one of the mesh STAs operates in light or deep sleep mode. A mesh peer service period is a contiguous period of time during which one or more individually addressed frames are transmitted between two peer mesh STAs. A mesh peer service period is directional and may contain one or more TXOPs. One mesh STA operates as the owner in the mesh peer service period. It obtains TXOPs in order to transmit Data frames or Management frames to the recipient in the mesh peer service period. Furthermore, the mesh STA initiates the termination of the mesh peer service period. The other mesh STA operates as the recipient in the mesh peer service period and does not obtain TXOPs for transmitting Data frames or Management frames to the owner of the mesh peer service period. A mesh STA may have multiple mesh peer service periods ongoing in parallel. At most one mesh peer service period can be set up in each direction with each peer mesh STA. A mesh peer service period is initiated by a peer trigger frame. An example mesh peer service period between two mesh STAs in light or deep sleep mode is shown in Figure s57. The numbering in the left-hand-side describes the phase of the operation. 1 indicates the Initiation phase, 2 indicates the data transmission phase and 3 indicates the termination phase of the mesh peer service period.

Instructions to editor:Please change to figure s57the peer service period to be mesh peer service period.

- Mesh peer service period

- Initiation of a mesh peer service period

A Mesh Data frame or a QoS Null frame that requires acknowledgement may be used as a peer trigger frame. The RSPI and the EOSP subfields in the QoS Control field control the initiation of a mesh peer service period. Table s62 lists how mesh peer service periods shall be initiated with different combinations of RSPI and EOSP field values.

Mesh peer service periods are not used in frame exchange toward active mode mesh STAs.

| - Mesh peer service period triggering with RSPI and EOSP field combinations in peer trigger frameinput.doc |  |  |
| --- | --- | --- |
| RSPI | EOSP | Mesh peer service period triggering |
| 0 | 0 | One mesh peer service period is initiated. The transmitter of the trigger frame is the owner in the mesh peer service period. |
| 0 | 1 | No mesh peer service period is initiated. |
| 1 | 0 | Two mesh peer service periods are initiated. Both mesh STAs are owners in a mesh peer service period. |
| 1 | 1 | One mesh peer service period is initiated. The receiver of the trigger frame is the owner in the mesh peer service period. |

The mesh peer service period may be initiated in the following cases:

- The mesh STA in light or deep sleep mode receives a peer trigger frame during its Mesh Awake Window as described in Mesh Awake Window

- The mesh STA in active mode receives a peer trigger frame from the peer mesh STA in light or deep sleep mode as described in Operation in active mode

- The mesh STA receives a peer trigger frame from the peer mesh STA in light sleep mode as described in Operation in light sleep mode

In addition, when a mesh STA enables MCCA while in a light or deep sleep mode, a scheduled service period begins at the each MCCAOP start time as described in MCCA use by power saving mesh STA. A mesh STA in a light or deep sleep mode shall enter the Awake state prior to the start time of scheduled service period.

- Operation during a mesh peer service period

During the mesh peer service period, the transmitter of the mesh peer service period and peer mesh STA shall operate in Awake state. The mesh peer service period may contain one or more TXOPs.

Reverse Direction Grant (RDG) shall not be used when the receiver of the TXOP operates in light or deep sleep mode and is not an owner in the mesh peer service period toward the TXOP holder.

- Termination of a mesh peer service period

The mesh peer service period is terminated after a successfully acknowledged QoS Null or Mesh Data frame with the EOSP bit set to 1 from the owner of the mesh peer service period.

If the mesh STA does not receive an acknowledgement to a frame that requires an acknowledgement and sent with the EOSP subfield set to 1, the mesh STA shall retransmit that frame at least once within the same mesh peer service period — subject to applicable retry or lifetime limit. The maximum number of retransmissions within the same mesh peer service period is the lesser of the Max Retry Limit and the MIB attribute dot11MeshSTAMissingAckRetryLimit.

NOTE—If an Ack to the retransmission of this last frame in the same mesh peer service period is not received, the mesh STA can use the next mesh peer service period to further retransmit that frame subject to the applicable retry or lifetime limit.

- MCCA use by power saving mesh STA

When a mesh STA operates in MCCA enabled mode and establishes MCCAOPs, the mesh STA shall be in active mode or light sleep mode towards neighbor peer mesh STAs it has established MCCAOPs with.

A scheduled mesh peer service period begins at the MCCAOP start time, if the MCCAOP responder operates in light or deep sleep mode for the MCCAOP owner. The MCCAOP owner is the owner of the scheduled mesh peer service period. The MCCAOP responder is the recipient of the scheduled mesh peer service period. Scheduled mesh peer service periods are not used, if the MCCAOP responder operates in active mode for the MCCAOP owner.

The scheduled mesh peer service period continues until it is successfully terminated by the acknowledged QoS Null or Mesh Data frame with the EOSP bit set to 1 from the owner of the mesh peer service period to the recipient of the mesh peer service period as described in 11C.13.10.

- General Mesh Support

Modify the following PICS as shown:

| MP9.4 | Frame transmission to mesh STA in light sleep mode | Power Save Support, Mesh peer service periods | MP9:M | Yes No N/A  |
| --- | --- | --- | --- | --- |
| MP9.5 | Frame transmission to mesh STA in deep sleep mode | Power Save Support, Mesh peer service periods | MP9:M | Yes No N/A  |

- Power Save parameters selection

- General

Power save mechanisms enable mesh STA operation in Doze state. In Doze state a mesh STA may set the transceivers off. The power save mechanism may adjust the mesh power mode of a mesh STA for various reasons that are beyond the scope of the standard including but not limited to traffic load or scanning and network maintenance needs.

The Annex specifies recommendations how to select a mesh power mode for the mesh STA based on traffic load, recommendations how to do scanning in mesh BSSs that may contain mesh STAs in light or deep sleep mode and recommended default values for power save related parameters.

- Selecting the mesh power mode based on traffic load

A mesh STA may adjust its mesh power mode based on the traffic load or the QoS requirements of the forwarded traffic. If a mesh STA has high traffic load or a mesh STA is congested, the mesh STA should operate in active mode to reduce delays or overhead that may be created by the operation in light or deep sleep mode.

A mesh STA may consider operating in active mode for all peer mesh STAs, even if only a single link is congested. When mesh STA operates in active mode, the mesh STA may receive frames at any time and mesh peer service periods may be triggered on demand, as in case of an AP that receives a trigger frame at times controlled by a non-AP STA.

If traffic load is moderate or low and best effort data is transmitted, a mesh STA may consider operating in light sleep mode for all or some mesh peerings. The mesh STA operating in light sleep mode for a mesh peering receives beacons from the peer mesh STA with periodic indication of buffered traffic.

If traffic load is low or no traffic is transmitted, a mesh STA may consider operating in deep sleep mode for a mesh peering. The mesh STA does not need to wake-up to receive a beacon from the peer mesh STA relative to which it operates in deep sleep mode.

The mesh STA may use deep sleep mode to control the number of times it enters the Awake state to receive Beacon frame from a peer mesh STA. If the mesh STA operates in deep sleep mode for all of its mesh peering, the mesh STA needs only to operate in Awake state during its own beacon transmission and Mesh Awake Window.

The mesh STA that forwards real time traffic (AC3 or AC2) with EDCA should operate in active mode. A mesh STA forwarding real time traffic with EDCA may operate in light or deep sleep mode, if the mesh STA is capable to initiate mesh peer service periods frequently and the other forwarding peer mesh STAs operate in active mode. Poor handling of the mesh power modes may result to delays and inappropriate QoS of the forwarded MSDUs.

- Scanning of mesh BSSs

A mesh BSS may have mesh STAs in deep sleep, light sleep and active mode. With active scanning only devices in Awake state at the transmission time of Probe Request frame may be found. However, with passive scanning also mesh STAs in light or deep sleep mode can be found.

Since mesh STAs in light or deep sleep mode may transmit Beacons at long intervals, a mesh STA seeking for candidate mesh STAs for a new mesh peerings should do passive scanning relatively for a longer time compared to passive scanning in BSS infrastructure mode. Mesh STAs in light or deep sleep mode with long DTIM interval might not be discovered with short scanning durations. Mesh STAs in light or deep sleep mode may use a short DTIM interval, if they intend to establish new mesh peerings.

- Default Parameters

The following are the recommended default values for power save related parameters for mesh STAs:

| - Default parameters for mesh STAs in light or deep sleep mode |  |  |
| --- | --- | --- |
|  | For moderate power save | For aggressive power save |
| Beacon Period | 200 TU | 800 TU |
| DTIM Period | 4 | 1 |
| Mesh Awake Window | 10 TU | 10 TU |

A mesh STA that is eager to conserve power and likely to operate in deep sleep mode with most of the links should utilize the value from aggressive power save parameters.

Although mesh STAs may utilize individual parameters regardless of the parameters used by neighbor mesh STAs or peer mesh STAs, each implementer should recognize balance between the power save efficiency and delay in the service initiation.

- MSDU forwarding in an MBSS containing mesh STAs in light or deep sleep mode

The battery powered mesh STAs should avoid forwarding MSDUs, to avoid power consumption and possible additional delays and inefficiencies that power save mechanisms may cause. If a light or deep sleep mode mesh STA forwards MSDUs, it should select its mesh power mode based on the traffic load and the traffic type as described in Selecting the mesh power mode based on traffic load.

The mesh STAs that desire to operate in light or deep sleep mode may select the light or deep sleep mode:

- mesh STAs that are “mains powered” may apply light or deep sleep mode if they do not have MSDUs to forward

- mesh STAs, that are “battery powered” and desire to minimize the power consumption, may freely use all mesh power modes

Mesh STAs in light or deep sleep mode may degrade the link metric value. The use of worse metric values reduces the probability of a light or deep sleep mode mesh STA being used for MSDU forwarding. If the mesh STA will operate in active mode for the link in forwarding path it should apply the link metric value without degradation.

References: