IEEE P802.11
Wireless LANs

| Proposed Text for Resolution of RFI-FWD Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd, Suite 400, Lake Mary, FL 32746 USA | +1-407-562-4050 | guenael dod strutt et motorola dod com |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6 80200 München, Germany |  | bahr et siemens dod com |

Abstract

This document proposes changes to the normative text of IEEE 802.11s D6.0 in order to resolve commens of letter ballot LB 165 related to clause 11C.8.3.

Resolved Comments:

CID 4145, 4146, 4147, 4148, 4149, 4249 with resolution code “Counter” and resolution nodes as given in 11-10/897r0

Instruction to editor: Change text of clause 9.22 Mesh forwarding framework as indicated by the WinWord change marks.

9.22 Mesh forwarding framework

9.22.1 General

The term “mesh forwarding” is used to describe forwarding of MSDUs and MMPDUs across paths determined by the mesh path selection between mesh STAs at the link layer.

9.22.2 Frame addressing and forwarding in an MBSS

9.22.2.1 Frame addressing

Mesh Data frames and Multihop Action frames enable multihop MSDU and MMPDU forwarding in an MBSS using the Mesh Control field described in 7.1.3.6.3 (Mesh Control field). In this subclause, addressing of the Mesh Data and Multihop Action frames and MSDU/MMPDU forwarding behavior are described.

Table s31 (Valid address field usage for Mesh Data and Multihop Action frames) shows the valid combinations of address fields in Mesh Data frames and Multihop Action frames along with the corresponding value of the Address Extension Mode field.

NOTE— ToDS and FromDS fields are located in the Frame Control field (see 7.1.3.1.3 (To DS and From DS fields)). Address Extension Mode subfield is located in the Mesh Flags field in the Mesh Control field (see 7.1.3.6.3 (Mesh Control field)). Address 1, Address 2, and Address 3 fields are located in the MAC header (see 7.1.2 (General frame format)). Address 4 is located in the MAC header if both ToDS and FromDS fields are 1, otherwise Address 4 is located in the Mesh Address Extension field of the Mesh Control field ((see 7.1.2 (General frame format) and 7.1.3.6.3 (Mesh Control field)). Address 5 and Address 6 are located in the Mesh Control field if it is present (see 7.1.3.6.3 (Mesh Control field)).

| - Valid address field usage for Mesh Data and Multihop Action frames |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supported frames | ToDS FromDS field | Address Extension Mode value (binary) | Address 1 | Address 2 | Address 3 | Address 4 | Address 5 | Address 6 |
| Mesh Data (individually addressed) | 11 | 00 | RA | TA | DA = Mesh DA | SA = Mesh SA | Not Present | Not Present |
| Mesh Data (group addressed) | 01 | 00 | DA | TA | SA = Mesh SA | Not Present | Not Present | Not Present |
| Mesh Data (proxied, individually addressed) | 11 | 10 | RA | TA | Mesh DA | Mesh SA | DA | SA |
| Mesh Data (proxied, group addressed) | 01 | 01 | DA | TA | Mesh SA | SA | Not Present | Not Present |
| Multihop Action | 00 | 01 | RA | TA | DA = Mesh DA | SA = Mesh SA | Not Present | Not Present |

In individually addressed Mesh Data and Multihop Action frames, Address 1 and Address 2 correspond to the mesh STA receiver address (RA) and the mesh STA transmitter address (TA) for a particular mesh link. Address 3 and Address 4 correspond to the destination and source end stations of a mesh path. The Address Extension Mode indicates the presence of optional address extension fields including Address 5 and Address 6 in the Mesh Control field that correspond to the end-to-end destination address (DA) and source address (SA) of external STAs that communicate over the mesh BSS via proxy mesh gates.

NOTE— Individually addressed Mesh Data frames only use mesh STA addresses in fields Address 1, Address 2, Address 3 and Address 4. This allows intermediate mesh STAs to forward Mesh Data frames without necessarily having any knowledge of the source and destination addresses, which may be external addresses. Thus, proxy information only needs to be maintained by a mesh gate or a mesh STA that is collocated with one or more APs.

The term source mesh STA refers to the first mesh STA on a mesh path. A source mesh STA may be a mesh STA that is the initial source of an MSDU/MMPDU or a mesh STA that receives an MSDU/MMPDU from a STA outside the mesh BSS and translates and forwards the MSDU/MMPDU on the mesh path. The address of the source mesh STA is referred to as the Mesh SA.

The term destination mesh STA refers to the final mesh STA on a mesh path. A destination mesh STA may be a mesh STA that is the final destination of an MSDU/MMPDU or a mesh STA that receives an MSDU/MMPDU from a mesh path and translates and forwards the MSDU/MMPDU on another mesh path or to a STA outside of the mesh BSS. The address of the destination mesh STA is referred to as the Mesh DA.

In group addressed Mesh Data frames, Address 1 and Address 2 correspond to the group address and the mesh STA transmitter address (TA). Address 3 corresponds to the mesh source of the group addressed Mesh Data frame. The Address Extension Mode indicates the presence of an optional address extension field Address 4 in the Mesh Control field that corresponds to the source address (SA) of external STAs that communicate over the mesh BSS via proxy mesh gates.

NOTE— The reason for not using four-address MAC header format for group addressed traffic is to avoid interactions with existing implementations. Earlier versions of this standard defined the four-address MAC header format (previously called WDS format) without defining procedures for its use. As a result there is a large number of deployed devices that use the four-address frame format in ways that would affect and be affected by mesh traffic if four-address group addressed frames were to be used.

Figure s50 (Example addressing for a Mesh Data frame) illustrates the addressing of a Mesh Data frame that contains an MSDU transmitted and forwarded on a mesh path from a mesh STA collocated with a portal (STA 1) to a mesh STA collocated with an AP (STA 2) where the source is a STA outside of the mesh BSS (STA 33) that is reachable via the portal and the destination is an 802.11 STA associated with the AP (STA 22).

- Example addressing for a Mesh Data frame

Details on how these address mappings work in forwarding processing are described in 9.22.2.2 (Addressing of individually addressed Mesh Data frames and data forwarding) and 9.22.2.3 (Addressing of group addressed Mesh Data frames and data forwarding).

9.22.2.2 Addressing of individually addressed Mesh Data frames and data forwarding

9.22.2.2.1 At source mesh STAs (individually addressed)

MSDUs sent by a mesh STA (as a consequence of a MA-UNITDATA.request with an individual destination address) and destined to another mesh STA in the MBSS shall be transmitted using the 4-address format frame (with Address Extension Mode set to 00), where the four address fields are set as follows (see Table s31 (Valid address field usage for Mesh Data and Multihop Action frames), “Mesh Data (individually addressed)”):

- Address 1: The address of the next-hop mesh STA (toward the destination mesh STA according to the forwarding information — see 11C.10.8.4 (Forwarding information))

- Address 2: The address of the transmitter mesh STA

- Address 3: The address of the destination mesh STA

- Address 4: The address of the source mesh STA

MSDUs that are either

- sent by a mesh STA (as a consequence of a MA-UNITDATA.request with an individual destination address) and destined to an address that is different from the mesh STA at the end of a mesh path, or

- received from the DS with an individual destination address

shall be transmitted using the proxied individually addressed Mesh Data format frame (with Address Extension Mode set to 10), where the mesh address extension field in the Mesh Control field carries the addresses of the end stations, as specified in Table s31 (Valid address field usage for Mesh Data and Multihop Action frames) “Mesh Data (proxied, individually addressed)”. The 6 addresses are defined as follows:

- Address 1: The address of the next-hop mesh STA (toward the destination mesh STA according to forwarding information — see 11C.10.8.4 (Forwarding information))

- Address 2: The address of the transmitter mesh STA

- Address 3: The address of the destination mesh STA at the end of the mesh path

- Address 4: The address of the source mesh STA at the beginning of the mesh path

- Address 5: The address of the destination end station (may be the same as Address 3 if the destination is the mesh STA at the end of the mesh path)

- Address 6: The address of the source end station (may be the same as Address 4 if the source is the mesh STA at the beginning of the mesh path)

Note: the destination address is distinct from the mesh STA at the end of the mesh path in two cases: 1) when the destination is an external address and 2), when the destination is a mesh STA distinct from the mesh STA at the end of the path.  The latter case can occur if a source mesh STA does not have a mesh path to the destination mesh STA; and the source mesh STA sends the MSDU to another mesh STA that is likely to have a path to the destination mesh STA (this other mesh STA may be a root, as described in 11C.10 Hybrid Wireless Mesh Protocol).

The Mesh TTL field in the Mesh Control field shall be set to the value of dot11MeshTTL.The MSDUs are forwarded multiple hops, limited by the Mesh TTL value.

The source mesh STA shall set the Mesh Sequence Number field in the Mesh Control field to a value from a modulo- 4 294 967 296 counter that is incremented by 1 for each new MSDU transmitted with a Mesh Control field and for each new MMPDU transmitted using a Multihop Action frame.

9.22.2.2.2 At intermediate and destination mesh STAs (individually addressed)

On receipt of an individually addressed Mesh Data frame, a mesh STA shall perform the following.

- The mesh STA shall decipher the frame and check it for authenticity. If it is not from a peer mesh STA, the frame shall be silently discarded.

- The mesh STA shall check to see whether the Address 3 field is known; if it is an unknown address, the mesh STA may perform any of the following three actions:

- silently discard the frame

- trigger a path discovery procedure depending on the path selection protocol that is currently active in the mesh BSS

- inform the Mesh STA in Address 2 that the destination is unreachable depending on the path selection protocol that is currently active in the mesh BSS.
(see 11C.10.11.3 (Conditions for generating and sending a PERR) Case A)

- If Address 2 is not one of the precursors for this destination mesh STA (see 11C.10.8.4 (Forwarding information)), the frame shall be discarded.

- If the frame is not discarded and one or more MSDUs are collected by the frame reception, the mesh STA may detect duplicate MSDUs referring the Mesh Sequence Number in the corresponding Mesh Control field and discard them (see 9.22.2.5 (Detection of duplicate MSDUs/MMPDUs)).

If either of the following conditions is met:

- Address 3 does not match the mesh STA’s own address, but is a known destination MAC addresses in the forwarding information or proxy information, or

- Address 3 matches the mesh STA’s own address, the Address Extension mode is 10 and Address 5 is a known destination MAC addresses in the forwarding information or proxy information,

then the following actions are taken:

- The lifetime of the forwarding information to the destination (Address 3) is set to its initial value.

- The lifetime of the forwarding information to the source (Address 4) is set to its initial value.

- The lifetime of the precursor list entry for the precursor to the destination (Address 2) is set to the maximum of the initial value and the current value.

- The lifetime of the precursor list entry for the precursor to the source (next hop to the destination) is set to the maximum of the initial value and the current value.

- If one or more MSDUs are collected, the Mesh TTL in the corresponding Mesh Control field is decremented by 1. If zero has been reached, the MSDU shall be discarded.

- If the MSDU has not been discarded, the mesh STA shall forward the MSDU via a frame setting the Address 1 field to the MAC address of the next hop mesh STA as determined from the forwarding information (see 11C.10.8.4 (Forwarding information)) and the Address 2 field to its own MAC address and queue the frame for transmission.

If Address 3 matches the mesh STA’s own MAC address and one or more MSDUs are collected from the frame, the mesh STA shall check the Address Extension Mode field in each Mesh Control field and take the following actions based on its value:

- If the Address Extension Mode is 00, the MA-UNITDATA.indication primitive is passed from the MAC sublayer entity to the LLC sublayer entity or entities.

- If the Address Extension Mode is 10:

- If Address 5 is equal to Address 3, the mesh STA is the final destination of the MSDU, and the MA-UNITDATA.indication primitive is passed from the MAC sublayer entity to the LLC sublayer entity or entities.

- If Address 5 is not equal to Address 3 and Address 5 is not a known destination MAC addresses in the forwarding information or proxy information, the MSDU is forwarded to the DS according to 11C.8.3.2 (Handling of MSDUs received from the MBSS) cases a) or b) or d)).

Note that during the forwarding process at intermediate mesh STAs, the content of the MSDU is not changed.

9.22.2.3 Addressing of group addressed Mesh Data frames and data forwarding

9.22.2.3.1 At source mesh STAs (group addressed)

MSDUs sent by a mesh STA (as a consequence of a MA-UNITDATA.request with a group destination address) shall be transmitted using a group addressed Mesh Data frame (with Address Extension Mode set to 00). (see Table s31 (Valid address field usage for Mesh Data and Multihop Action frames), “Mesh Data (group addressed)”) An implementation may circumvent the unreliability of group addressed transmissions by using multiple individually addressed Mesh Data frames, which are individually acknowledged. In such case, the frame may be converted to individually addressed frames and transmitted as individually addressed frame to each peer mesh STAs as described in 9.22.2.2.1 (At source mesh STAs (individually addressed)), by setting the Address 3 field to the group address. The circumstances for choosing this method are outside the scope of the standard.

In group addressed Mesh Data frames, the address fields are set as follows:

- Address 1: The group address

- Address 2: The address of the transmitter mesh STA

- Address 3: The address of the source mesh STA

If the MSDU is received from the DS and Address 1 (RA/DA) is a group address, the source mesh STA, which is a proxy mesh gate, shall transmit the MSDU using a proxied group addressed Mesh Data frame (with Address Extension Mode to 01) (see Table s31 (Valid address field usage for Mesh Data and Multihop Action frames), “Mesh Data (proxied, group addressed)”).

In proxied group addressed Mesh Data frames, the address fields are set as follows:

- Address 1: The group address

- Address 2: The address of the transmitter mesh STA

- Address 3: The address of the source mesh STA

- Address 4: The address of the external STA

The source mesh STA shall set the Mesh TTL field in the Mesh Control to dot11MeshTTL in order to control the hop count. The MSDUs are forwarded multiple hops, limited by the Mesh TTL value. For example, if the Mesh TTL field is 1, MSDUs are delivered to immediate neighbors only.

The source mesh STA shall set the Mesh Sequence Number field in the Mesh Control field to a value from a modulo- 4 294 967 296 counter that is incremented by 1 for each new MSDU transmitted with a Mesh Control field and for each new MMPDU transmitted using a Multihop Action frame.

Procedures that enhance the reliability or efficiency of group addressed transmissions are outside the scope of this standard.

9.22.2.3.2 At recipient mesh STAs (group addressed)

On receipt of a group addressed Mesh Data frame with Address 1 (DA) equal to the group address, or on receipt of an individually addressed Mesh Data frame with Address 3 (Mesh DA) equal to the group address, a mesh STA shall perform the following:

- The mesh STA shall decipher the frame and check it for authenticity. If it is not from a peer mesh STA, the frame shall be silently discarded.

- If the frame is not discarded and one or more MSDUs are collected by the frame reception, the mesh STA may detect duplicate MSDUs referring the Mesh Sequence Number in the corresponding Mesh Control field and discard them (see 9.22.2.5 (Detection of duplicate MSDUs/MMPDUs)).

- The mesh STA decrements the Mesh TTL in the Mesh Control field. If the Mesh TTL value has reached zero, the corresponding MSDU shall not be forwarded to other mesh STAs. If the Mesh TTL value has not reached zero and if the dot1MeshForwarding is 1, the mesh STA shall forward the MSDU via a frame setting Address 2 to its own MAC address.

When the SA and the Mesh SA are not identical (the source address is therefore an external address), the MSDU shall be forwarded by using a proxied group addressed Mesh Data frame (with Address.Extension set to 01). Otherwise, the MSDU shall be forwarded by using a group addressed Mesh Data frame (with Address.Extension set to 00). An implementation may circumvent the unreliability of group addressed transmissions by using multiple individually addressed Mesh Data frames, which are individually acknowledged. In such case, the frame may be converted to individually addressed frames and transmitted as an individually addressed frame to each peer mesh STAs as described in 9.22.2.2.2 (At intermediate and destination mesh STAs (individually addressed)), by setting the address 3 field to the group address. The circumstances for choosing this method and the ability to determine all the addresses of the neighbor peer mesh STAs are beyond the scope of the standard.

If the frame has not been discarded and one or more valid MSDUs are collected from the frame, the MA-UNITDATA.indication primitive is passed from the MAC sublayer entity to the LLC sublayer entity or entities.

9.22.2.4 Addressing of Management frames and MMPDU forwarding

9.22.2.4.1 General

All MMPDUs except MMPDU transmitted using Multihop Action frames are transmitted over only one hop to peer mesh STAs.

Note that in several cases, the reception and processing of an Action frame leads to the transmission of a new Action frame of the same type that may include an identical or a modified version of the contents from the elements of the received Action frame.

A mesh STA may convert a group addressed management frame to individually addressed frames and transmit as an individually addressed frame to each peer mesh STAs, if the frame is intended to be delivered to its peer mesh STAs only. The circumstances for choosing this method are outside the scope of the standard.

9.22.2.4.2 MMPDU forwarding using Multihop Action frames

MMPDUs transmitted using Multihop Action frames (see 7.4.16 (Multihop Action frame details)) are forwarded according to the procedures in 9.22.2.2 (Addressing of individually addressed Mesh Data frames and data forwarding). The frame shall be transmitted using 4-address format frame (with Address Extension Mode set to 01), where the four address fields are set as shown in Table s31 (Valid address field usage for Mesh Data and Multihop Action frames), “Multihop Action”. A mesh STA transmitting a Multihop Action frame shall set Address 1 field to the address of the next-hop mesh STA (toward the destination mesh STA according to the forwarding information — see 11C.10.8.4 (Forwarding information)).

The source mesh STA shall set the Mesh TTL field in the Mesh Control field to the value of dot11MeshTTL, and set the Mesh Sequence Number field in the Mesh Control field to a value from a modulo- 4 294 967 296 counter that is incremented by 1 for each new MSDU transmitted with a Mesh Control field and for each new MMPDU transmitted using a Multihop Action frame.

At intermediate and destination mesh STAs, on receipt of Multihop Action frame, the address matching, the forwarding information update, and the Mesh TTL decrement are performed as described in 9.22.2.2.2 (At intermediate and destination mesh STAs (individually addressed)), and the MMPDU is forwarded according to the forwarding information.

At intermediate mesh STAs, frame fields following the Mesh Control field are treated as MSDU in 9.22.2.2.2 (At intermediate and destination mesh STAs (individually addressed)) and are not required to be examined.

If the Address 3 in the received Multihop Action frame matches the mesh STA’s own MAC address, the mesh STA (destination mesh STA) shall process the content of the MMPDU accordingly.

9.22.2.5 Detection of duplicate MSDUs/MMPDUs

A mesh STA may receive the same MSDU or MMPDU from different neighbor peer mesh STAs more than once.

Duplicate MSDU/MMPDU filtering is facilitated through the inclusion of a Mesh Sequence Number field in the Mesh Control field in Mesh Data frames and Multihop Action frames as specified in 7.1.3.6.3 (Mesh Control field).

The receiving mesh STA shall keep a cache of recently received <Mesh SA, Mesh Sequence Number> tuples. The Mesh Source Address (Mesh SA) is contained in Address 4 for individually addressed Mesh Data frames and Multihop Action frames. The Mesh Source Address (Mesh SA) is contained in Address 3 for group addressed Mesh Data frames.

A mesh STA shall reject an MSDU/MMPDU with a Mesh Control field as a duplicate if it matches an <Mesh SA, Mesh Sequence Number> tuple of an entry in the cache.

9.22.2.6 Mesh STAs that do not forward

A mesh STA that has dot11MeshForwarding equal to false does not forward either MSDUs, or MMPDUs of type Multihop Action. The circumstances in which a mesh STA may be allowed to become a non forwarding entity and the authority to set dot11MeshForwarding to false are beyond the scope of this standard.

A mesh STA that does not forward is a special case of a mesh STA. Such mechanism depends on whether the path selection protocol provides a mechanism to allow mesh STAs not to participate in forwarding. The HWMP path selection protocol provides such a mechanism, see 11C.10 (Hybrid Wireless Mesh Protocol (HWMP)).

9.22.3 Frame forwarding to non-MBSS STAs

To send or forward a frame, a mesh STA first follows the data forwarding procedures defined in 9.22.2 (Frame addressing and forwarding in an MBSS). If the mesh STA is not able to forward the frame to the proxy mesh gate for an external destination MAC address, the mesh STA shall assume that the destination is outside the MBSS and shall forward the frame to known mesh gates in the MBSS as an individually addressed frame according to the procedures for frame addressing and data forwarding of individually addressed frames at source mesh STAs in an MBSS (9.22.2.2.1 (At source mesh STAs (individually addressed))).. If there is no mesh gate available, the mesh STA shall silently discard the frame.

Discovery of mesh gates by mesh STAs is performed using propagated elements, such as a GANN (11C.8.2 (Gate announcement protocol)). Other methods specific to the HWMP path selection protocol are also available, such as the proactive PREQ (11C.10.4.2 (Proactive PREQ mechanism)) or the proactive RANN (11C.10.4.3 (Proactive RANN mechanism)), when the Gate Role flag field in these HWMP elements is 1.

Instruction to editor: Change text of clause 11C.8.3 Data forwarding behavior of a proxy mesh gate as indicated by the WinWord change marks.

11C.8.3 Data forwarding behavior of a proxy mesh gate

11C.8.3.1 General

Forwarding of MSDUs from the DS into the MBSS by a proxy mesh gate follows the procedures given in 9.22.2 (Frame addressing and forwarding in an MBSS).

Forwarding of MSDUs from the MBSS into the DS by a proxy mesh gate follows the procedures that apply for the specific collocated network.

A proxy mesh gate can learn the addresses of the other proxy mesh gates in the MBSS and of external addresses proxied by them through the receipt of path selection messages and messages carrying proxy information (for example, see 7.3.2.113 (PXU element)).

11C.8.3.2 Handling of MSDUs received from the MBSS

An MSDU destined to a STA outside the MBSS, sent from a mesh STA in the MBSS and received by the proxy mesh gate has one of the following possible destinations:

- An external address proxied by the proxy mesh gate: The proxy mesh gate forwards the MSDU to the external address through the DS.

- An external address that is proxied by another proxy mesh gate in the MBSS: The MSDU is forwarded to the proxy mesh gate that proxies the external address through the MBSS. The procedures for frame addressing and data forwarding at intermediate mesh STAs in an MBSS apply (9.22.2.2.2 (At intermediate and destination mesh STAs (individually addressed))).

- A group address: The mesh gate forwards the MSDU into the MBSS according to the procedures for frame addressing and data forwarding of group addressed frames at recipient mesh STAs in an MBSS (9.22.2.3.2 (At recipient mesh STAs (group addressed))). Furthermore, the mesh gate forwards the MSDU to the DS using a group addressed frame.

- An address unknown to the proxy mesh gate: The mesh gate forwards the MSDU to the DS. Furthermore, the mesh gate forwards the MSDU according to the procedures for frame forwarding to non-MBSS STAs (see 9.22.3 (Frame forwarding to non-MBSS STAs)).

11C.8.3.3 Handling of MSDUs that enter the MBSS

An MSDU received by a proxy mesh gate from the DS has one of the following possible destinations:

- A mesh STA address that the mesh gate knows to be inside the MBSS: The mesh gate forwards the MSDU according to the procedures for frame addressing and data forwarding at source mesh STAs in an MBSS (9.22.2.2.1 (At source mesh STAs (individually addressed))). Address 3 is the address of the destination mesh STA.

- An external address that is proxied by another proxy mesh gate in the MBSS: The mesh gate forwards the MSDU according to the procedures for frame addressing and data forwarding at source mesh STAs in an MBSS (9.22.2.2.1 (At source mesh STAs (individually addressed))). Address 3 is the address of the proxy mesh gate that proxies the external destination address.

- A group address: The mesh gate forwards the MSDU according to the procedures for frame addressing and data forwarding of group addressed frames at source mesh STAs in an MBSS (9.22.2.3.1 (At source mesh STAs (group addressed))).

- An address that is unknown to the mesh gate: The mesh gate forwards the MSDU to other known mesh gates in the MBSS as an individually addressed frame according to the procedures for frame addressing and data forwarding of individually addressed frames at source mesh STAs in an MBSS (9.22.2.2.1 (At source mesh STAs (individually addressed))). Address 3 is the address of the other known mesh gates in the MBSS.

Note, that the procedure to determine that an address is unknown depends on the active path selection protocol. It may require an attempt to establish a path to the destination (see 11C.7 (Mesh path selection framework)).