IEEE P802.11
Wireless LANs

| Gate Announcement protocol (the return) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Lake Mary, FL 32746 USA | +1-407-562-4050 | Guenael.Strutt@motorola.com |

Abstract

The Gate Announcement protocol needed a serious scrub, and this document is designed to resolve comments 4143, 4144, 4156, 4234 and 4248

| 4143 | In LB161, CID3242 reads "There are 3 methods for the mesh gates discovery. Sounds like too much options." And the resolution notes reads:"The options are there in order to decrease the network load substantially. Both the GANN mechanism and the proactive tree mechanisms use the same frame propagation procedure - flooding the whole network. Especially the last two options (the Gate Role bit set in proactive PREQs / RANNs) are there for this purpose. If the Gate Role bit is set, the GANN does not need to be sent." I believe this guideline in the resolution notes should be mentioned in the specification (at least as an informative note). | Clarified by stating “The gate announcement protocol can be avoided when using one of the HWMP proactive path selection methods with the Gate Role field set to 1” |
| --- | --- | --- |
| 4144 | "The GANN element may be transmitted in Beacon frames. If this is the case, a mesh STA shall include the GANN element into a Beacon frame at least once every dot11MeshGateAnnouncementInterval." Many information are still missing for the GANN in Beacon frames. 1. what happens if dot11MeshGateAnnouncementInterval and beacon interval have mismatch. 2. when GANN is transmitted via beacon frame, does GANN frame need to be sent out separately? If not, why do we need GANN frame? 3. how is the GANN seq number is incremented when the element is transmitted over beacon frame? 4. how does the intermediate STAs treat the GANN propagation when GANN element is received thru beacon frames? Do they propagate the element using beacon frame as well? If so, the element propagation requires larger latency as the beacon is transmitted only at beacon interval. Is it acceptable? | OK, it is no longer in the beacon frame |
| 4156 | It is not clear how mesh STA use RANN with Gate Roll = 1 and GANN element. Should mesh STA use either of these options? How they can be determined. | The text now says that the GANN can be avoided altogether when using the RANN |
| 4234 | The configuration of the Gate Protocol and the inclusion of GANNs into beacons are broken. | No beacons anymore |
| 4248 | The Gate Announcement protocol is a nice mechanism, but with the introduction of the term mesh gate and the replacement of Portal by Gate in the announcement protocol, the scope of the announcement protocol changed (it got wider). It is questionable, that this scope has been the original one as the announcement protocol has been designed. | All checked |

- Interworking

- Overview of interworking in a mesh BSS

An MBSS functions like an IEEE 802 LAN segment that is compatible with IEEE 802.1D. The MBSS appears as a single access domain.

An MBSS may contain two or more mesh gates. In this case, the MBSS has more than one “port” (in the sense of IEEE Std 802.1D-2004, for example) through which it accesses the DS. Accordingly, broadcast loops may occur. Therefore, mesh gates should implement a loop preventing protocol in the DS.

NOTE— In the DS, a typical implementation may use the Rapid Spanning Tree Protocol (RTSP) as specified in IEEE 802.1D-2004. With RTSP, the resulting active DS topology forms a tree. Then, even if multiple mesh gates connect with the same DS, the MBSS will access the DS through a single mesh gate only.

Mesh gates are discovered by mesh STAs using the

gate announcement protocol (see Gate announcement protocol). The gate announcement protocol is not needed when using one of the HWMP proactive path selection methods with the Gate Role field equal to 1:

- The proactive PREQ mechanism (see Error: Reference source not found), with the Gate Role field equal to 1 (see Conditions for generating and sending a PREQ)

- The proactive RANN mechanism (see Error: Reference source not found), with the Gate Role field equal to 1 (see Conditions for generating and sending a RANN)

Mesh gates propagate gate announcement elements (or RANNs and PANNs with the Gate Role field equal to 1 if using such mechanisms) only if dot11MeshGateAnnouncementProtocol is equal to 1.  The decision to announce the presence of a mesh gate is beyond the scope of the standard.

- Gate announcement protocol

- General

This subclause describes the function, generation and processing of the Gate Announcement (GANN) element.

- Function

The Gate Announcement (GANN) element, described in 7.3.2.108 (GANN element), is used to announce the presence of a mesh gate in the mesh BSS. Gate Announcements allow mesh STAs to discover a mesh gate and, if necessary, build a path towards it.

- Conditions for generating and sending a GANN

The GANN element is transmitted in Mesh Interworking frames.

Note to Editor: remove GANN element from beacon frame

A mesh STA shall send a GANN frame in the following cases:

Case A: Original transmission

All of the following applies:

- The mesh STA is a mesh gate

- dot11MeshGateAnnouncementProtocol is true

- At every dot11MeshGateAnnouncementInterval

The content of a GANN element in Case A shall be as shown in Content of a GANN element in Case A.

| - Content of a GANN element in Case A |  |
| --- | --- |
| Field | Value |
| Element ID | Value given in Table 7-26 (Element IDs) for the GANN element |
| Length | 15 |
| Flags | Not used |
| Hop Count | 0 |
| Element TTL | Maximum number of hops allowed for the Gate Announcement |
| Mesh Gate Address | Mesh STA MAC address |
| GANN Sequence Number | A sequence number specific for the mesh gate. The GANN sequence number shall be incremented by the mesh gate before each transmission. |
| Interval | dot11MeshGateAnnouncementInterval |

Case B: Propagation

All of the following applies:

- The mesh STA has received and accepted a Gate Announcement

- The decremented Element TTL of the Gate Announcement is equal to or greater than 1

- dot11MeshForwarding is true

The content of a GANN element in Case B shall be as shown in Content of a GANN element in Case B.

| - Content of a GANN element in Case B |  |
| --- | --- |
| Field | Value |
| Element ID | Value given in Table 7-26 (Element IDs) for the GANN element |
| Length | 15 |
| Flags | As received |
| Hop Count | As received + 1 |
| Element TTL | As received – 1 |
| Mesh Gate Address | As received |
| GANN Sequence Number | As received |
| Interval | As received |

- GANN processing

- General

A received Gate Announcement is subject to certain acceptance criteria. Processing depends on the contents of the Gate Announcement and the information available at the receiving mesh STA.

- Acceptance criteria

The Gate Announcement element shall not be accepted (and shall not be processed as described in Effect of receipt) if the GANN Sequence Number of the Gate Announcement is equal or lower than the GANN Sequence Number of the most recently accepted Gate Announcement with the same Mesh Gate Address.

- Effect of receipt

The following applies only to a Gate Announcement element that was accepted according to the acceptance criteria in Acceptance criteria. The receiving mesh STA shall transmit a Gate Announcement as defined in Conditions for generating and sending a GANN, Case B.

The Mesh Gate Address field of the GANN contains the address of the mesh gate, and may be stored for the purpose of determining paths to the mesh gates. Paths to mesh gates allow mesh STAs to forward MSDUs to addresses for which no path could be determined (see 9.22.3 (Frame forwarding to non-MBSS STAs)).

- Conditions for generating and sending a PREQ

A mesh STA shall send a PREQ element in an HWMP Mesh Path Selection frame, as defined in 7.4.15.4 (

HWMP Mesh Path Selection frame format), in the following cases:

Case A: Path Discovery (Original Transmission)

All of the following applies:

- The mesh STA needs to establish an on-demand path to one or more targets for which there is no ongoing path discovery initiated by this mesh STA.

- The mesh STA has not sent a PREQ element for the target mesh STAs less than dot11MeshHWMPpreqMinInterval TUs ago. If this is the case, the transmission of the PREQ has to be postponed until this condition becomes true.

- The mesh STA has not made more than (dot11MeshHWMPmaxPREQretries – 1) repeated attempts at path discovery towards the target of the PREQ.

The content of a PREQ element in Case A shall be as shown in Contents of a PREQ in Case Cinput.doc.

| - Content of a PREQ element in Case Ainput.doc |  |  |
| --- | --- | --- |
| Field |  | Value |
| Element ID |  | Value given in Table 7-26 (Element IDs) for the PREQ element |
| Length |  | 26 + N*11 (if Bit 6 in Flags (AE) = 0) 32 + N*11 (if Bit 6 in Flags (AE) = 1) |
| Flags |  | Bit 0: dot11MeshGateAnnouncementProtocol Bit 1: 0 (group addressed) Bit 2: 0 (no proactive PREP applicable) Bit 3 – 5: Reserved Bit 6: Address Extension (AE) (1 – if external address present, 0 – otherwise) Bit 7: Reserved |
| Hop Count |  | 0 |
| Element TTL |  | Maximum number of hops allowed for this element, e.g., dot11MeshHWMPnetDiameter. |
| Path Discovery ID |  | New unique Path Discovery ID, for instance, previous Path Discovery ID + 1 |
| Originator Mesh STA Address |  | MAC address of the path originator |
| Originator HWMP Sequence Number |  | Previous Originator HWMP SN + 1. See Error: Reference source not found |
| Originator External Address |  | Present only if Bit 6 in Flags = 1. This value is set to the external address, which is the source address of the MSDU (from outside the mesh BSS) that triggered the path discovery at the originator. |
| Lifetime |  | The time for which mesh STAs receiving the PREQ consider the forwarding information to be valid, e.g., dot11MeshHWMPactivePathTimeout. |
| Metric |  | Initial value of active path selection metric |
| Target Count |  | N (N>=1) |
| Per Target | Per Target Flags | Bit 0 (TO): dot11MeshHWMPtargetOnly Bit 1: Reserved Bit 2 (USN): 0 if forwarding information for Target Address with valid HWMP sequence number exists, 1 otherwise. Bit 3-7: Reserved |
|  | Target Address | MAC address of requested target |
|  | Target HWMP Sequence Number | If Per Target Flags Bit 2 (USN) is 0, the latest HWMP sequence number stored by the originator mesh STA for the target mesh STA from the forwarding information (seeError: Reference source not found). Otherwise, reserved. |

Case B: Path Maintenance (Original Transmission) (optional)

All of the following applies:

- the mesh STA has a path to a given target mesh STA that is not a root mesh STA

- the last PREQ to this target was sent dot11MeshHWMPmaintenanceInterval TUs (or more) ago.

The content of a PREQ in Case B shall be as shown in Content of a PREQ element in Case Binput.doc.

| - Content of a PREQ element in Case Binput.doc |  |  |
| --- | --- | --- |
| Field |  | Value |
| Element ID |  | Value given in Table 7-26 (Element IDs) for the PREQ element |
| Length |  | 26 + N*11 |
| Flags |  | Bit 0: dot11MeshGateAnnouncementProtocol Bit 1: 0 (group addressed) Bit 2: 0 (no proactive PREP applicable) Bit 3 – 5: Reserved Bit 6: 0 (no address extension) Bit 7: Reserved |
| Hop Count |  | 0 |
| Element TTL |  | Maximum number of hops allowed for this element, e.g., dot11MeshHWMPnetDiameter |
| Path Discovery ID |  | New unique Path Discovery ID, for instance, previous Path Discovery ID + 1 |
| Originator Mesh STA Address |  | MAC address of the originator of the PREQ |
| Originator HWMP Sequence Number |  | Originator HWMP SN + 1. See Error: Reference source not found |
| Originator External Address |  | Field not present in this element |
| Lifetime |  | The time for which mesh STAs receiving the PREQ consider the forwarding information to be valid, e.g., dot11MeshHWMPactivePathTimeout. |
| Metric |  | Initial value of active path selection metric |
| Target Count |  | N (N>=1) |
| Per Target | Per Target Flags | Bit 0 (TO): 1 Bit 1: Reserved Bit 2 (USN): 0 if forwarding information for Target Address with valid HWMP sequence number exists, 1 otherwise. Bit 3-7: Reserved |
|  | Target Address | MAC Address of target mesh STA |
|  | Target HWMP Sequence Number | If Per Target Flags Bit 2 (USN) is 0, the latest HWMP sequence number for this target known to the originator mesh STA. |

Case C: Proactive PREQ (original transmission)

All of the following applies:

- The root mesh STA is configured as root mesh STA using proactive PREQs (dot11MeshHWMProotMode = 2 OR dot11MeshHWMProotMode = 3).

- The root mesh STA sent its previous proactive PREQ dot11MeshHWMProotInterval TUs ago.

The contents of a PREQ in Case C shall be as shown in Contents of a PREQ in Case Cinput.doc.

| - Contents of a PREQ in Case Cinput.doc |  |  |
| --- | --- | --- |
| Field |  | Value |
| Element ID |  | Value given in Table 7-26 (Element IDs) for the PREQ element |
| Length |  | 37 |
| Flags |  | Bit 0: dot11MeshGateAnnouncementProtocol (mesh gate role) Bit 1: 0 (group addressed) Bit 2: As needed (proactive PREP) Bit 3 – 5: Reserved Bit 6: 0 (no address extension) Bit 7: Reserved |
| Hop Count |  | 0 |
| Element TTL |  | Maximum number of hops allowed for this element, e.g., dot11MeshHWMPnetDiameter. |
| Path Discovery ID |  | New unique Path Discovery ID, for instance, previous Path Discovery ID + 1 |
| Originator Mesh STA Address |  | MAC address of the root mesh STA |
| Originator HWMP Sequence Number |  | Originator HWMP SN + 1. See Error: Reference source not found. |
| Originator External Address |  | Field not present in this element |
| Lifetime |  | dot11MeshHWMPactivePathToRootTimeout |
| Metric |  | Initial value of active path selection metric |
| Target Count |  | 1 |
| Per Target | Per Target Flags | Bit 0 (TO): 1 Bit 1: Reserved Bit 2 (USN): 1 Bit 3-7: Reserved |
|  | Target Address | Broadcast address |
|  | Target HWMP Sequence Number | 0 |

Case D: Root Path Confirmation (Original Transmission)

One of the following applies:

- The mesh STA has received a RANN and the metric (RANN metricmetric to the transmitter of the RANN) is better than the metric to the root in the current forwarding information.

- The mesh STA has a path to a root mesh STA and the last PREQ to the root mesh STA was sent dot11MeshHWMPconfirmationInterval TUs (or more) ago.

The content of a PREQ element in Case D shall be as shown in Content of a PREQ element in Case Dinput.doc.

| - Content of a PREQ element in Case Dinput.doc |  |  |
| --- | --- | --- |
| Field |  | Value |
| Element ID |  | Value given in Table 7-26 (Element IDs) for the PREQ element |
| Length |  | As required |
| Flags |  | Bit 0: dot11MeshGateAnnouncementProtocol Bit 1: 1 (individually addressed) Bit 2: 0 (no proactive PREP applicable) Bit 3 – 5: Reserved Bit 6: 0 (no address extension) Bit 7: Reserved |
| Hop Count |  | 0 |
| Element TTL |  | Maximum number of hops allowed for this element, e.g., dot11MeshHWMPnetDiameter |
| Path Discovery ID |  | Not used |
| Originator Mesh STA Address |  | MAC address of the originator mesh STA |
| Originator HWMP Sequence Number |  | Originator HWMP SN + 1. See Error: Reference source not found. |
| Originator External Address |  | Field not present in this element |
| Lifetime |  | The time for which mesh STAs receiving the PREQ consider the forwarding information to be valid, e.g., dot11MeshHWMPactivePathToRootTimeout. |
| Metric |  | Initial value of active path selection metric |
| Target Count |  | 1 |
| Per Target | Per Target Flags | Bit 0 (TO): 1 Bit 1: Reserved Bit 2 (USN): 0 Bit 3-7: Reserved |
|  | Target Address | Root mesh STA MAC Address |
|  | Target HWMP Sequence Number | The latest HWMP sequence number for this target known to the originator mesh STA |

- Conditions for generating and sending a RANN

A mesh STA sends out a RANN element in an HWMP Mesh Path Selection frame, as defined in 7.4.15.4 (

HWMP Mesh Path Selection frame format), in the following cases:

Case A: Original transmission

All of the following conditions apply:

- The mesh STA is configured as a root mesh STA using the proactive RANN mechanism (dot11MeshHWMProotMode = 4).

- The root mesh STA sent its previous RANN dot11MeshHWMPrannInterval TUs ago

The contents of a RANN element in Case A shall be as shown in Contents of a RANN element in Case Ainput.doc.

| - Contents of a RANN element in Case Ainput.doc |  |
| --- | --- |
| Field | Value |
| Element ID | Value given in Table 7-26 (Element IDs) for the RANN element |
| Length | 21 |
| Flags | Bit 0: dot11MeshGateAnnouncementProtocol Bit 1 – 7: Reserved |
| Hop Count | 0 |
| Element TTL | Maximum number of hops allowed for this element |
| Root Mesh STA Address | MAC address of the root mesh STA |
| HWMP Sequence Number | Last used HWMP sequence number of the root mesh STA + 1 |
| Interval | dot11MeshHWMPrannInterval |
| Metric | Initial value of active path selection metric |