IEEE P802.11
Wireless LANs

| GAS transport via Data Frames |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Henry Ptasinski | Broadcom Corporation | 150 Mathilda Place, Sunnyvale, CA | +1-408-543-3316 | henryp@broadcom.com |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-905-629-4746 | mmontemurro@rim.com |

### Overview

This submission proposes a mechanism for transporting GAS exchanges via DATA frame encapsulation.

10.3.70.1 MLME-GAS.request

10.3.70.1.2 Semantics of the service primitive

Change the text and table as shown:

The primitive parameters are as follows:

MLME-GAS.request (

PeerSTAAddress,

DialogToken,

AdvertisementProtocolID,

Query,

QueryFailureTimeout,

TranportType

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43bl | This contains an Advertisement Protocol ID (see 7.3.2.91) which may be 802.11 assigned or vendor specified. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |
| QueryFailureTimeout | Integer | >1 | The time limit, in units of Beacon intervals, after which the GAS Query procedure will be terminated. |
| TransportType | Enumeration | ACTION, DATA | Transport used for sending the request: ACTION frame or DATA frame using encapsulation defined in Annex U. |

10.3.70.3 MLME-GAS.indication

10.3.70.3.2 Semantics of the service primitive

Change the text and table as shown:

The primitive parameters are as follows:

MLME-GAS.indication (

PeerSTAAddress,

DialogToken,

AdvertisementProtocolID,

Query,

TranportType

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43bl | This contains an Advertisement Protocol ID (see 7.3.2.91) which may be 802.11 assigned or vendor specified. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |
| TransportType | Enumeration | ACTION, DATA | Transport used for sending the request: ACTION frame or DATA frame using encapsulation defined in Annex U. |

10.3.70.4 MLME-GAS.response

10.3.70.4.2 Semantics of the service primitive

Change the text and table as shown:

The primitive parameters are as follows:

MLME-GAS.response (

PeerSTAAddress,

DialogToken,

ResultCode,

ResponseInfo,

TranportType

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 - 255 | The dialog token to identify the GAS transaction. |
| ResultCode | Enumeration | SUCCESS, NO_OUTSTANDING_REQUEST, ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED, QUERY_RESPONSE_OUTSTANDING, QUERY_RESPONSE_TOO_LARGE, SERVER_UNREACHABLE, TIMEOUT | Indicates the result response to the GAS-request from the peer MAC entity. See Table 11-3. |
| ResponseInfo | String | N/A | Query Response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |
| TransportType | Enumeration | ACTION, DATA | Transport used for sending the request: ACTION frame or DATA frame using encapsulation defined in Annex U. |

11.23.3 Interworking Procedures: Generic Advertisement Services

Insert a new clause after the existing subclauses:

11.23.3.1.4 GAS Transport Selection

An unassociated STA shall only use ACTION as the GAS TransportType.  An associated STA may use either ACTION or DATA as the GAS TransportType.  A STA responding to a GAS request shall use the same transport as was used for the received GAS request.

Annex U Usage of Ethertype 89-0d

U.1 Ethertype 89-0d frame body

Add a new row to table U-1 with the following parameters:

Protocol Name:  ANQP

Payload Type:	<ANA>

Subclause:	11.23.3.2

and adjust the reserved payload types as appropriate.