IEEE P802.11
Wireless LANs

| DSE Status Query/Response |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Jae-Hyung Song | LG Electronics | San 4-1, Bongcheon7-Dong, Kwanak-Gu Seoul, Korea | +82-2-2102-0177 | jaehyung.song@lge.com |
| Hang-Seok Oh | LG Electronics | San 4-1, Bongcheon7-Dong, Kwanak-Gu Seoul, Korea | +82-2-2102-0224 | hs.oh@lge.com |

Abstract

Submission for candidate P802.11af draft text.  In TVWS, the actual operation channel of a dependent STA may be different from the channel that was used for the communication with the enabling STA (or dependent AP or DFS owner), which grants the enablement for the operation in a TVWS channel.

Thus, a mechanism should be provided in 802.11 to allow a dependent STA to be queried for a response that indicates which operation channel(s) that the dependent STA operates on.

A DSE Status Query/Response is specified for such query and response.

This submission addresses TGaf Technical Review of D0.02 comment 47.

Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are auctioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| 47 | Jung Yee | J.2.4 | 21 | 14 | T | Lack of feedback from dependent STA to enabling STA on channel status. | The dependent STA should send a notification message on the same channel that is used for enabling request. By using the channel for enabling request, we try to avoid creating interference in the operating TVWS channel. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

While the IEEE Std 802.11y-2008 defined a master/client operation with parts that can be applied to operation in TVWS, there are some significant differences between operation in 3650 – 3700 MHz operation in the U.S. and operation in TVWS.

In TVWS, since the availability of a WLAN channel is changed dynamically, the channel on which a dependent STA operates may change dynamically.  Furthermore, the IEEE P802.11af draft D0.03 defines the enablement that can be done through a link that is not a TVWS channel.  Assuming that the enablement was performed through a link other than a TVWS channel, there is no way for an enabling STA to identify the channel status of its dependent STAs.

Thus, a mechanism should be provided in 802.11 to query a dependent STA’s channel status.

To provide a function to accommodate the necessity described above, a DSE Status Query/Response is specified to allow an enabling STA to query the channel status of its dependent STAs as a public action frame.

Proposed Resolution:

Agree in principle with the technical comment 47 based on the discussion and editorial instructions in this document.

Note: This document’s baselines are P802.11af-D0.05.

8. Frame Formats

8.5 Action frame format details

8.5.8 Public Action details

8.5.8.1 Public Action frames

TGaf Editor: Insert the following row (ignoring the header row) in Table 8-131 Public Action field values in the correct position to preserve ordering by the “Action field value” column.

Table 8-131 – Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | DSE Status Query/Response |

TGaf Editor: Before clause 10, insert text as shown:

Insert the following new clause after the last subclause of 8.5.8:

8.5.8.af4 DSE Status Query/Response frame format

The DSE Status Query/Response frame is a Public Action frame that is either transmitted by an enabling STA to query its dependent STA for its operations channel(s) status or transmitted by a dependent STA as a response to the query on the operation channel(s).  The format of the DSE Status Query/Response frame body is shown in Figure 8-47af4 (DSE Status Query/Response frame body format).

|  |  |  |  |  |  | These three fields are repeated, as determined by the Length field. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Category | Action Value | Length | DSE Status Query/Response Type | Enablement Identifier | Operating Class (Optional) |  | Channel Number (Optional) | Constrained Maximum Transmit Power (Optional) |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 |

Figure 8-47af4 – DSE Status Query/Response frame body format

The Category field is set to the value for public action defined in Table 8-36 (Category values).

The Action Value field is set to indicate a DSE Status Query/Response frame, as defined in Table 8-131 (Public Action field values).

The DSE Status Query/Response Type field indicates whether the DSE Query/Response element is used to query a dependent STA or is used as a response from a dependent STA.  The DSE Status Query/Response Type field also indicates whether the DSE Query/Response element contains the Operating Class, Channel Number, Constrained Maximum Transmit Power triplet.  A STA sets the DSE Status Query/Response Type field on transmission as specified in Table 8-35af2.

Table 8-35af2 – DSE Status Query/Response Type values

| DSE Status Query/Response Type value | Description |
| --- | --- |
| 0 | The DSE Status Query/Response element is transmitted to query a dependent STA. The DSE Status Query/Response element does not contain the Operating Class, Channel Number, Constrained Maximum Transmit Power triplet. |
| 1 | The DSE Status Query/Response element is transmitted as a response from a dependent STA. The DSE Status Query/Response element contains the Operating Class, Channel Number, Constrained Maximum Transmit Power triplet. |
| 2 – 255 | Reserved. |

The remaining fields are as described in the Channel Power Management Announcement element (see 8.4.2.af5 (Channel Power Management Announcement element)) and are current operating channel(s).

8.5.11 Protected Dual of Public Action frames

8.5.11.1 Protected Dual of Public Action details

Insert the following row (ignoring the header row) in Table 8-143 Protected Dual of Public Action field values in the correct position to preserve ordering by the “Action field value” column.

Table 8-143 – Protected Dual of Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Protected DSE Status Query/Response |

Insert the following clause at the end of 8.5.11.7, before 8.5.11.8, as shown:

8.5.11.7af1 Protected DSE Status Query/Response frame format

The Protected DSE Status Query/Response frame format is the same as the DSE Status Query/Response frame format (see 8.5.8.af4).  It is used instead of the DSE Status Query/Response frame when Management Frame Protection is negotiated.

10. MLME

10.12 DSE procedures

10.12.3 Registered STA operation

TGaf Editor: Change the last sentence of 10.12.3, as shown:

A registered STA that is not an enabling STA may operate as an AP in an infrastructure BSS and relay Public Action frames (specifically, DSE Enablement, DSE Deenablement, DSE Measurement Request, DSE Measurement Report, DSE Power Constraint, DSE Status Query/Response) from a dependent STA to its enabling STA. Note that the enabling signal is not a Public Action frame, and is not relayed (see 10.12.4).

10.12.4 Enabling STA operation with DSE

TGaf Editor: Insert following text at the end of 10.12.4, as shown:

Insert the following paragraph at the end of the second paragraph of 10.12.4, as shown:

An enabling STA, a dependent AP, or a DFS owner may issue a DSE Status Query to any of its dependent STAs with the DSE Status Query/Response Type field value of the DSE Status Query/Response of the DSE Status Query/Response Public Action Frame, set to 0, in order to receive a DSE Status Response that identifies the operation channel status of the queried dependent STA.

10.12.5 Dependent STA operation with DSE

TGaf Editor: At the end of 10.12.5, insert following text, as shown:

Insert the following dashed item at the end of 10.12.5, as shown:

- A dependent STA shall return a DSE Status Response in response to a DSE Status Query, if the request is received from the AP with which it is associated or the enabling STA, with which it last attempted enablement, with a DSE Status Query/Response Public Action Frame, with the DSE Status Query/Response Type field value set to 1, with the Operating Class and the Channel Number fields identifying the channel(s) that the dependent STA is currently operating on, and with the Constrained Maximum Transmit Power field(s) identifying the maximum transmit power on the STA’s operation channel(s).

Annex B (normative) Protocol Implementation Conformance Statement (PICS) proforma

B.4 PICS proforma – IEEE Std 802.11-<year>

B.4.af TVWS functions

TGaf Editor: Insert the following row (ignoring the header row) at the end of B.4.af, as shown:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| WS7 | DSE Status Query/Response | 8.4.5.3, 8.5.8.af4, 10.11.4, 10.11.5, Annex E.2.4 | CFaf:M | Yes ℃ No ℃ N/A ℃ |

Annex E (normative) Country elements and operating classes

E.2 Band specific operating requirements

E.2.4 TVWS band in the United States (54 MHz to 688 MHz)

TGaf Editor: Insert the following text before the text provided by 447r1:

In TVWS band in the U.S., in case of DSE Status Query/Response, a STA shall utilize a DSE Status Query/Response public Action frame.