IEEE P802.11
Wireless LANs

| Moving SCS request element in to action frame |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley at nds dot com |

Abstract

CID#766 in LB164 says “AFAICT, the SCS response element is only used within a SCS Response action frame.  Furthermore, the information element has only 1 subfield which is not an information element.  So why not just define the SCS Response action frame to have these fields and not burn a value from the element ID space?” with a proposed resolution of “Delete the SCS Response information element and move the field and their descriptions to the SCS Request action frame clause.”

This document proposes modifications to P802.11aa D1.01 to address this comment, by removing the SCS Request and SCS Response elements and creating a new SCS Descriptor element.

Instructions to the TGaa editor are marked in purple italics.

Changes to the draft are marked in blue italics. underline indicates an addition and strikethrough indicates deletion.

Rename “SCS Request” to “SCS Descriptor” in table 7-26 in P802.11aa D1.01.

Change section 7.3.2.aa92 in P802.11aa D1.01 as indicated:

7.3.2.aa92 SCS Request Descriptor element

The SCS Request Descriptor element defines information about the stream that is being classified using the procedures defined in 11.21.2. The format of the SCS Request Descriptor element is shown in Figure 7-aa11 7-aa12.

|  | Element ID | Length | Request Type | SCS Descriptor List |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure 7-aa11—SCS Request element format |  |  |  |  |

The Element ID field is set to the SCS Request Descriptor value in Table 7-26.

The value of the Length field is set to 1+n, where n indicates the total length of the SCS Descriptor List field

The element is variable in length. The length of the element is indicated by the Length field.

The SCSID field is set to a non-zero value chosen by the non-AP STA identifying the SCS stream specified in this SCS Descriptor element.

The Request Type field is set to a number to identify the type of SCS request. The Request Types are shown in Table 7-aa7.

| Table 7-aa7—Request Type definitions |  |
| --- | --- |
| Name | Usage Mode |
| Add | 0 |
| Remove | 1 |
| Change | 2 |
| Reserved | 3-255 |

The SCS Descriptor List field contains one or more SCS Descriptors. The format of the SCS Descriptor is defined in Figure 7-aa12.

Change Figure 7-aa12 in P802.11aa D1.01 as indicated by adding two new octets and moving the length field.

|  |  |  |  |  |  | zero or more TCLAS Elements |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | SCSID | Length Request Type | Intra-Access Category Priority Element (optional) | TCLAS Elements (optional) | TCLAS Processing Element (optional) |  |
| Octets: | 1 | 1 | 1 | 1 | 0 or 3 | variable | 0 or 3 |  |
| Figure 7-aa12 —SCS Descriptor element |  |  |  |  |  |  |  |  |

The SCSID field is set to a non-zero value chosen by the non-AP STA identifying the SCS stream specified in the SCS Descriptor List field.

The Length field is set to the total length of the Intra-Access Category Priority element, TCLAS elements and TCLAS Processing element present in the SDS descriptor.

The Intra-Access Category Priority element is present when Request Type is set to “Add” or “Change” and is defined in 7.3.2.90aa.

The TCLAS Elements field contains zero or more TCLAS information elements to specify how incoming MSDUs are classified as part of this SCS stream, as defined in 7.3.2.31. One or more TCLAS information elements are present when Request Type is set to “Add” or “Change” and no TCLAS information elements are present when Request Type is set to “Remove”.

The TCLAS Processing Element field is present when more than one TCLAS elements are present and defines how multiple TCLAS information elements are to be processed, as defined in 7.3.2.33.

The SCS Request Descriptor element is included in SCS Request frames, as described in 7.4.2.5aa. The use of the SCS Request Descriptor element and SCS Request frames is described in 11.21.2.

Remove section “7.3.2.aa93 SCS Response element” from P802.11aa D1.01

Change section 7.4.aa13.1 in P802.11aa D1.01 as indicated:

7.4.aa13.1 SCS Request frame format

SCS Request frames are used to request the creation, modification or deletion of a stream classification using the procedures defined in 11.21.2.

The frame body of the SCS Request frame contains the information shown in Figure 7-aa21

|  | Category | Action | Dialog Token | SCS Request element Descriptor List | Optional Subelements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable |
| Figure 7-aa21—SCS Request frame body format |  |  |  |  |  |

The Category field is set to <ANA> (representing Robust AV Streaming).

The Action field is set to the value specified in Table 7-aa6 for an SCS Request frame.

The Dialog Token field is set to a nonzero value that is unique among the SCS Request frames sent to the AP for which a corresponding SCS Response frame has not been received.

The SCS Request element is defined in 7.3.2.aa92 (#454)

The SCS Descriptor List field contains one or more SCS Descriptor elements, as defined in 7.3.2.aa92.

The Optional Subelements field format contains zero or more Subelements, each consisting of a 1-octet Subelement ID field, a 1-octet Length field, and a variable length Data field, as shown in Figure 7-aa?. Any optional subelements are ordered by non-decreasing Subelement ID.

The Subelement ID field values for the defined optional subelements are shown in Table 7-aa?. A Yes in the Extensible column of a subelement listed in Table 7-aa? indicates that the Length of the subelement might be extended in future revisions or amendments of this standard. When the Extensible column of an element is set to Subelements, then the subelement might be extended in future revisions or amendments of this standard by defining additional subelements within the subelement. See 9.14.2.

| Table 7-aa?—Optional Subelement IDs for SCS Request |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 244 |  |
| 222-255 | Reserved |  |  |

Change section 7.4.aa13.2 in P802.11aa D1.01 as indicated:

7.4.aa13.2 SCS Response frame format

The SCS Response frame is sent in response to an SCS Request frame using the procedures defined in 11.21.2. The frame body of an SCS Response frame contains the information shown in Figure 7-aa22.

|  | Category | Action | Dialog Token | SCS Response Status List |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure 7-aa22—SCS Response frame body format |  |  |  |  |

The Category field is set to <ANA> (representing Robust AV Streaming).

The Action field is set to the value specified in Table 7-aa6 for an SCS Response frame.

The Dialog Token field is set to the nonzero value of the corresponding SCS Request frame. If the SCS Report frame is being transmitted other than in response to an SCS Request frame, then the Dialog token is set to 0

The SCS Response field is defined in 7.3.2.87.

The SCS Status List field contains one or more SCS Status duples. The format of the SCS Status duple is defined in Figure 7-aa14.

|  | SCSID | Status |
| --- | --- | --- |
| Octets: | 1 | 1 2 |
| Figure 7-aa14—SCS Status duple format |  |  |

The SCSID field is set to the value of the SCSID field in the SCS Descriptor received in the SCS Request frame.

The Status field indicates the status of the requested SCSID, as indicated in Table 7-23.

References: