IEEE P802.11
Wireless LANs

| Proposed resolution CID14 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: YYYY-MM-DD |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley at nds dot com |

Abstract

CID14 in LB164 states:

MIB items "dot11QLoadReportEnabled, dot11CF_Pollable and dot11CF-Pollable" don't exist. I think this was intended to say "Its an HC AP"

CIDs 352, 496, 652, 757, 797 and 997 also highlight this issue.

This document proposes text changes to resolve these comments.

Instructions to the TGaa editor are marked in purple italics.

Changes to the draft are marked in blue underline to indicate an addition and blue strikethrough indicates deletion.

7.2.3 Management frames

7.2.3.1 Beacon frame format

Modify Table 7-8 in P802.11aa as indicated:

Change Table 7-8 by adding orders 49 and 50 information fields as shown below

Table 7-8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 49 | QLoad Report | The QLoad Report element is present every dot11QLoadReportIntervalDTIM DTIMs (#51) if dot11QLoadReportActivated (#18) is true (#696) |
| 50 | HCCA TXOP Advertisement | The HCCA TXOP Advertisement element is present if both dot11QLoadReportActivated (#18) is true and an HC is operating in this BSS dot11CF_Pollable are true and dot11CF-Pollable is false |

11.aa24.2 HCCA TXOP Advertisement

Modify first paragraph of 11.aa24.2 in P802.11aa as indicated:

Overlapping An HCCA APs for which dot11RobustAVStreamingImplemented(#29) is true shall advertise coordinate (#564) their TXOP schedules using the HCCA TXOP Advertisement element and HCCA TXOP Advertisement frames.  The HCCA TXOP Advertisement element is included in the Beacon frame of an HC when dot11QLoadReportActivated (#18) is true and dot11CF_Pollable are true and dot11CF-Pollable is false.  An HCCA AP shall advertise the Duration, Service Interval (SI) and Start Times for each TXOP reservation(#354) in the HCCA TXOP Advertisement element as described in 7.3.2.96.  When an HCCA AP that is overlapping with another HC AP, i.e. the Overlap field in the QLoad Report is greater than 0 (#700), it shall examine the TXOP Reservation field(s), if present, in received HCCA TXOP Advertisements of all its directly overlapping HCs APs before accepting a TSPEC request that(#688) has the Access Policy subfield of the TSPEC element set to HCCA or HEMM.  The HC AP shall then schedule its new TXOP such that it does not interfere with any existing TXOP in overlapping HCs APs.

References: