IEEE P802.11
Wireless LANs

| D0.1 comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-08-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Saishankar Nandagopalan | Broadcom |  |  | nsai@broadcom.com |

Abstract

This document proposes resolutions to CIDs 321 and 477 on D0.1.

Discussion: WinCapacity initially represents the buffer size but during the course of transmission as outlined in different sections 9.26.3.2, 9.26.3.3, 9.26.3.4 and 9.26.4 it may change its value since the receiver reordering buffer may be forced to hold on MSDUs/AMSDUs/MPDUs/AMPDUs ready for in order delivery due to deferred reception at the next MAC process. This behavior imposes handling of dynamic capacity.

Resolution: Reject

Discussion: Subclause 11.3 is the subclause in 802.11 spec that defines which frames are allowed to be sent in which state and in which band. On this basis:

- The mmWave Beacon is not listed in 11.3.2, and hence it cannot be used in the OBand

- The mmWave Beacon is listed in 11.3.3, and hence it can be used in the DBand

In other words, it is already clear from 11.3 that the mmWave Beacon is defined only for the DBand (mmWave PHY). Also, if other PHYs want to reuse 11 or reuse the mmWave Beacon, they can always do that. There is nothing in the 802.11 spec, or D0.1, that precludes other TGs from reusing existing frames, frame types, etc.

Resolution: Reject