IEEE P802.11
Wireless LANs

| CID 487 Clarification to 9.4,9.23.7.1, 11.2.3 and 7.2.1.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-08-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sai Nandagopalan | Broadcom | 16340 W Bernardo Drive, San Diego, 92127, CA, USA | 858-521-5192 | nsai@broadcom.com |

Abstract

Some clarifications required per CID 487 on clause 9.4, clause 9.23.7.1, clause 11.2.3 and 7.2.1.11

CID 487 says “The spec needs clarification in several subclauses. It is much easier to list them in separate documents”. This document proposes clarifying the following subclauses 7.2.1.11, 9.4, 9.23.7.1 and 11.2.3.

To the editor.  Please change the following clause 7.2.1.11 as follows.(Page 37, lines 20 to 23)

When sent in response to a Poll frame, the Duration field is set to the value of the Duration field contained in the Poll frame, minus the value of the Response Offset field contained in the Poll frame multiplied by its unit as specified in 7.2.1.10, minus SIFS, minus the time taken to transmit the SPR frame. When not sent in response to a Poll frame and transmitted within an SP or a TXOP allocation, the Duration field is set to the time left in the allocation excluding the SPR transmission time. In all other cases the Duration field is set to zero.

To the editor.  Please change the following clause 9.23.7.1 as follows.(Page 173, lines 28 to 29 and page 174, lines 1 to 3)

A STA that receives a Poll frame shall respond to the PCP/AP with a single directional unicast SPR 28 frame at the time offset from the end of the Poll frame indicated in the Response Offset field within the Poll frame. The Duration/ID field within the SPR frame shall be set to the value of the Duration/ID 1 field contained in the received Poll frame, minus the value of the Response Offset field contained in 2 the received Poll frame multiplied by its unit (7.2.1.10), minus the time taken to transmit the SPR frame minus SIFS.

To the editor.  Please change the following clause 9.4 as follows. (Page 154, lines 7 to 8.)

MAC that supports the mmWave PHY shall complete the transmission of a fragmented MSDU before starting transmission of the another MSDU with the same TID/TSID of the fragmented MSDU.

To the editor.  Please change the following clause 11.2.3 as follows. (Page 242, lines 4 to 8.)

If a PCP/AP sets the BSS Type field within a transmitted mmWave Beacon frame to PBSS or the PCP/AP includes a Non-transmitted BSSID Capability element in a transmitted mmWave Beacon, Announce, or Probe Response frames and the BSS Type field within the Non-transmitted BSSID Capability element is set to PBSS, then the PCP/AP shall follow the PCP power management rules described in 11.2.3.2 if the PCP/AP chooses to employ power management.