IEEE P802.11
Wireless LANs

| CID 487 Clarification to 8.4.7 and 7.3.2.91 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-08-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| SK Yong | Marvell | 5488 Marvell Lane, Santa Clara, CA 95054 | 408-222-8487 | skyong@marvell.com |
| Yong Liu | Marvell | 5488 Marvell Lane, Santa Clara, CA 95054 | 408-222-8412 | yliu@marvell.com |

Abstract

Some clarifications required per CID 487 on Clause 7.3.2.91 and 8.4.7.

Clarification 1: The assumption about PBSS mSTA is the same as that for IBSS STA. And each STA may do both roles as EAP Peer and EAP Anthenticator. So a clarification is needed in the spec to clearly indicate this.

Proposed Resolution: Accept

Replace all "IBSS" with "IBSS/PBSS"  throughout 5.2.3.2

- Clarification 2: The PSKID generation (see 8.4.7 P137L19) is new and should use SHA-256 as in 11r and 11w.

- Proposed Resolution: Accept

- 1) Change PSKID definition in P137L20 to: PSKID = Truncate-128(HMAC-SHA-256(PSK, SSID))
2) Add a sentence in section 7.3.2.25.2 under Table 7-34—AKM suite selectors: “An mSTA operating in the DBand shall not use the AKM suite selector values 00-0F-AC:1 and 00-0F-AC:2."

- Clarification 3: It is not clear how to set AIF in request frames (see 7.3.2.91, P62L2)

- Proposed Resolution: Accept

Add the following text to clarify:

- “The AID field is reserved in Association Request, Reassociation Request, and Probe Request frames”