IEEE P802.11
Wireless LANs

| MIB Updates |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1332 Crossman Ave Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains proposed MIB changes identified in the MIB compilation and additional review.

Revision 2 includes changes agreed on the Sept 2, 2010 con call.

Revision 3 includes additional identified changes and MIB compliation input.

Revision 4, 5 includes additional identified changes and MIB compilation input, and on the Sept 9th con call.

Revision 6 includes additional identified changes, see items 13, Q17 and Q21-23.

(1) P324L22, P378L33 Align the tab spacing on the "MAX-ACCESS" line, also align tabs P321L22, P321L56-59

(2)Table 7-37c in Draft 7 became Table 7-43r, Change from Table 7-37c to Table 7-43r on p58 l64, p310 l26, p324 l42, p372 l4, p375 l20, p377 l50, p380 l24

(3)P67L54, change from "7.3.2.65" to "7.3.2.61"

(4) P324L10, P325L39, P327L57, change from "read-create" to "read-only" and delete P328L1 Defval

(5)P252L37, change from 7.3.2.62.2 to 7.3.2.67.2, P325L43, P363L11, P374L3 change from 7.3.2.63.2 to 7.3.2.68.2

(6)P326L46, change from "8.4.3" to "8.5.1.5"

(7)P323L46, change from “dot11WirelessMGTEventRSNElement” to “dot11WirelessMGTEventRSNARSNElement”

(8)P327L56, shows “OCTET STRING” while “P323L52 shows “DisplayString” – should these match?

Yes, change P323L52 to “OCTET STRING”

(9)P321L15, change from “dot11LocationConfigIndex” to “dot11LocationServicesIndex”

(10)P321L7, change from “dot11LLocation” to “dot11Location”

(11)P311L59,62,63, move the dot11MgmtOptionMulticastDiagnosticsActivated TruthValue, and dot11MgmtOptionLocationTrackingImplemented TruthValue, and

dot11MgmtOptionLocationTrackingActivated TruthValue, rows to follow P312L7, immediately after dot11MgmtOptionSSIDListActivated TruthValue to match the

order in the listed items. Note, number entries in the listed items are already correct.

(12) P310L31, change from “OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(9))” to

“OBJECT-TYPE

SYNTAX  TruthValue”

P 308L30, Change from “dot11RRM3rdPartyMeasurementActivated OCTET STRING” to “dot11RRM3rdPartyMeasurementActivated TruthValue”

(13) P308L23, insert a new entry

“dot11BssMaxIdlePeriodOptions” immediately following the” dot11BssMaxIdlePeriod” entry

And insert the following definition at P308L58, renumbering subsequent entries, skipping 11u assigned values.

dot11BssMaxIdlePeriodOptions OBJECT-TYPE

SYNTAX OCTET-STRING (SIZE (1))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute indicates the options associated with the BSS Max Idle capability. This value is transmitted in the Association Response and Reassociation Reponse frames.”

::= { dot11StationConfigEntry 108}

Also, P308L56, change from “Reassociation” to “Reassociation Response”

(14) P308L25, insert 2 new entries

“dot11TIMBroadcastHighRateTIMRate” and “dot11TIMBroadcastLowRateTIMRate” immediately following the “dot11TIMBroadcastOffset” entry, renumbering subsequent entries.

dot11TIMBroadcastHighRateTIMRate OBJECT-TYPE

SYNTAX INTEGER (0...65535)

UNITS "0.5mbps"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute indicates the rate used to transmit the high data rate

TIM frame, in units of 0.5 Mb/s. A value of 0 indicates that the high rate TIM frame is not   transmitted.”

DEFVAL { 0 }

::= { dot11StationConfigEntry 111}

dot11TIMBroadcastLowRateTIMRate OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "0.5mbps"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute indicates the rate used to transmit the low data rate

TIM frame, in units of 0.5 Mb/s. A value of 0 indicates that the low rate TIM frame is not transmitted.”

DEFVAL { 0 }

::= { dot11StationConfigEntry 112}

(15) P320L54, P320L61, P321L18, change from “read-write” to “not-accessible”

P321L26, P321L41, P321L51, P321L64, P322L6, P322L15, P322L23, P322L33, P32246, P32257, change from “read-write” to “read-create”

P323L1 – Change from “Table 7-43v” to “Table 7-43w”

P94L43, reference is wrong, change from “7.4.7.10” to “7.4.7.12”

(16) P321L7, immediately following the MAC address entry, insert a new entry

dot11LLocationServicesLocationIndicationParamsIndicationMulticastAddress MacAddress

and insert the following entry in the list below, renumbering as needed

dot11LocationServicesLocationIndicationParamsIndicationMulticastAddress  OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute is the destination address to which the Location Track Notification frames are sent in a non-IBSS network. See 7.3.2.71.2 and 11.22.4.1"

::= { dot11LocationServicesEntry 3 }

P321L15, immediately BEFORE the Location Status entry, insert 3 new entries

dot11LLocationServicesLocationIndicationBroadcastDataRate unsigned32

dot11LLocationServicesLocationIndicationOptionsUsed OCTET STRING

dot11LLocationServicesLocationIndicationIndicationParameters OCTET STRING

dot11LocationServicesLocationIndicationBroadcastDataRate  OBJECT-TYPE

SYNTAX unsigned32 (0..4294967295)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute specifies the target data rate at which the STA transmits Location Track Notification Frames. The Broadcast Target Data Rate field format is specified by the Rate Identification

field defined in 7.3.1.32. A value of 0 indicates the STA transmits Location Track Notification frames at

a rate chosen by the STA transmitting the Location Track Notification frames."

DEFVAL { 0 }

::= { dot11LocationServicesEntry 14 }

dot11LocationServicesLocationIndicationOptionsUsed  OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the location configuration options used for transmitting Location Track Notification Frames.

::= { dot11LocationServicesEntry 15 }

dot11LocationServicesLocationIndicationIndicationParameters  OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the location Indication Parameters used for transmitting Location Track Notification Frames.

::= { dot11LocationServicesEntry 15 }

(17) P327L42, change from “SYNTAX INTEGER (0..65535)” to ” SYNTAX INTEGER (0.. 16777215)”

P323L52, insert a new entry immediately following PeerConnectionTime as follows, inserting the corresponding definition on P327L54, renumbering as required.

dot11WirelessMGTEventPeerPeerStatus INTEGER,

and

dot11WirelessMGTEventPeerPeerStatus OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the Peer link connection status as indicated in Table 7-43y. See 7.3.2.68.4."

::= { dot11WirelessMGTEventEntry 27 }

Also, P327L57, change from “read-create” to “read-only”

(18) P323L23, change from “SYNTAX dot11WirelessMGTEventEntry” to “SYNTAX Dot11WirelessMGTEventEntry

P323L30, change from “dot11WirelessMGTEventEntry ::=” to “Dot11WirelessMGTEventEntry ::=”

(19) P323L54 From

dot11WirelessMGTEventIndex OBJECT-TYPE
SYNTAX Unsigned32
MAX-ACCESS read-only
STATUS current
DESCRIPTION
"This attribute contains the Index for EVENT Report elements in
dot11WirelessMGTEventTable,greater than 0."
::= { dot11WirelessMGTEventEntry 1 }

to

dot11WirelessMGTEventIndex OBJECT-TYPE
SYNTAX Unsigned32 (1..4294967295)
MAX-ACCESS read-only
STATUS current
DESCRIPTION
"This attribute contains an auxiliary index into the dot11WirelessMGTEventTable."
::= { dot11WirelessMGTEventEntry 1 }

and P321L15 from

dot11LocationConfigIndex OBJECT-TYPE
SYNTAX Unsigned32
MAX-ACCESS read-write
STATUS current
DESCRIPTION
"This attribute is the Index for LocationServices Report elements in
dot11LocationServicesTable,
greater than 0."
::= { dot11LocationServicesEntry 1 }

To

dot11LocationServicesIndex OBJECT-TYPE
SYNTAX Unsigned32 (1..4294967295)
MAX-ACCESS read-write
STATUS current
DESCRIPTION
"This attribute contains an auxiliary index into the dot11LocationServicesTable."
::= { dot11LocationServicesEntry 1 }

(20) P322L44, change from

dot11LocationServicesLocationIndicationParamsEssDetectionInterval INTEGER,

to

dot11LocationServicesLocationIndicationParamsEssDetectionInterval OBJECT-TYPE,

(21) P325L37, change from

previousTransition Failed(15),

to

previousTransitionFailed(15)

(22) P330L2 – Validate that 52 is open/valid value

(Q1)P355L16, delete the "dot11WNMRqstChanNumber" from the list (appears to be duplicate of the one 2 lines down, and is not included in the list of entries below)

(Q2)P395L32 and P397L19 – first location has "CenterFreq" as INTEGER, second as Integer32, need to make consistent. Change P395L32 from "INTEGER" to Integer32.

(Q3)P381L27, insert a new entry: dot11WNMDiagMfrInfoRprtCertificateID Type OCTET STRING}, inserting a new definition at the end of the list of entries:

dot11WNMDiagMfrInfoRprtCertificateID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..251))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the Certificate ID for the

reported Manufacturer Information STA Diagnostic."

::= { dot11WNMDiagMfrInfoReportEntry 13 }

P399L49, immediately following the entry for dot11WNMDiagMfrInfoRprtDeviceType , insert

dot11WNMDiagMfrInfoRprtCertificateID

(Q4) Should the Annex Q entries have Default values? Some do, some don’t:

P357L55, P358L13, P352L52, P359L18, etc. Read-only variables have  no default values. Read-write/Read-create may have a default value but need not have a default value. Delete the default values in all read-only entries.

(Q5) P355L55 – Has a reference listed; however, this is not done uniformly, except for other references to Annex J – ok to leave? Propose to delete all the REFERENCE entries in Annex Q, P355L54-55, P361L10-11, P378L16-17, P389L41-42, P390L30-31,

(Q6) P347L58 - Delete this second instance of dot11RRMRqstSTAStatTrigQoSDiscardCntThresh OBJECT-TYPE (entry 69), renumbering subsequent entries.

(Q7) P368L46 Delete the “dot11WNMLocationCivicRprtContent ; there is no corresponding definition, and the content is in the subsequent entry.

(Q8) P370L23, change from

::= { dot11WNMMulticastDiagnosticReportEntry 4 }

to

::= { dot11WNMLocationIdentifierReportEntry 4 }

(Q9) P370L36 Renumber the entry from 4 to 5

From
::= { dot11WNMLocationIdentifierReportEntry 4 }

To

::= { dot11WNMLocationIdentifierReportEntry 5 }

(Q10) Reflect changes to Event Time Value and Time Error

P371L7
Change from

dot11WNMEventTransitRprtTimeValue

to

dot11WNMEventTransitRprtUTCOffset

P371L63:  Change from:

dot11WNMEventTransitRprtTimeValue OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(9))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the TimeAdvertisement Time Value as defined in the Time

Advertisement IE Time Value field when the Time Capabilities field is set to 2. The

format is defined in Table 7-37c and is included in the Time Advertisement element in

Beacon and Probe Response frames."

::= { dot11WNMEventTransitReportEntry 6 }

dot11WNMEventTransitRprtTimeError OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the Time Error value as defined in the Time

Advertisement IE Time Error field when the Time Capabilities field is set to 2. This

field is included in the Time Advertisement element in Beacon and Probe Response

frames."

DEFVAL { 0 }

::= { dot11WNMEventTransitReportEntry 7 }

To

dot11WNMEventTransitRprtUTCOffset OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(10))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute indicates the UTC Offset Time Value optionally included in the Event Report."

::= { dot11WNMEventTransitReportEntry 6 }

dot11WNMEventTransitRprtTimeError OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute contains the value of the Event Time Error field optionally included in the Event Report."

::= { dot11WNMEventTransitReportEntry 7 }

P374L27

dot11WNMEventRsnaRprtTimeValue to dot11WNMEventRsnaRprtUTCOffset

P375L14

From

dot11WNMEventRsnaRprtTimeValue OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(9))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the TimeAdvertisement Time Value as defined in the Time

Advertisement IE Time Value field when the Time Capabilities field is set to 2. The

format is defined in Table 7-37c and is included in the Time Advertisement element in

Beacon and Probe Response frames."

::= { dot11WNMEventRsnaReportEntry 6 }

dot11WNMEventRsnaRprtTimeError OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the Time Error value as defined in the Time

Advertisement IE Time Error field when the Time Capabilities field is set to 2. This

field is included in the Time Advertisement element in Beacon and Probe Response

frames."

DEFVAL { 0 }

::= { dot11WNMEventRsnaReportEntry 7 }

To

dot11WNMEventRsnaRprtUTCOffset OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(10))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute indicates the UTC Offset Time Value optionally included in the Event Report."

::= { dot11WNMEventRsnaReportEntry 6 }

dot11WNMEventRsnaRprtTimeError OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute contains the value of the Event Time Error field optionally included in the Event Report."

::= { dot11WNMEventRsnaReportEntry 7 }

P376L56, from

dot11WNMEventTransitRprtEventTSF TSFType,

dot11WNMEventTransitRprtTimeValue OCTET STRING,

dot11WNMEventTransitRprtTimeError

to

dot11WNMPeerRprtEventTSF TSFType,

dot11WNMPeerRprtUTCOffset OCTET STRING,

dot11WNMPeertRprtTimeError

P377L44,

From

dot11WNMEventPeerRprtTimeValue OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(9))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the TimeAdvertisement Time Value as defined in the Time

Advertisement IE Time Value field when the Time Capabilities field is set to 2. The

format is defined in Table 7-37c and is included in the Time Advertisement element in

Beacon and Probe Response frames."

::= { dot11WNMEventPeerReportEntry 6 }

dot11WNMEventPeerRprtTimeError OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the Time Error value as defined in the Time

Advertisement IE Time Error field when the Time Capabilities field is set to 2. This

field is included in the Time Advertisement element in Beacon and Probe Response

frames."

DEFVAL { 0 }

::= { dot11WNMEventPeerReportEntry 7 }

To

dot11WNMEventPeerRprtUTCOffset OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(10))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the UTC Offset Time Value optionally included in the Event Report."

::= { dot11WNMEventPeerReportEntry 6 }

dot11WNMEventPeerRprtTimeError OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute contains the value of the Event Time Error field optionally included in the Event Report."

::= { dot11WNMEventPeerReportEntry 7 }

P379L35, change from

dot11WNMEventWNMLogRprtTimeValue to

dot11WNMEventWNMLogRprtUTCOffset

P380L18 change from

dot11WNMEventWNMLogRprtTimeValue OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(9))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the TimeAdvertisement Time Value as defined in the Time

Advertisement IE Time Value field when the Time Capabilities field is set to 2. The

format is defined in Table 7-37c and is included in the Time Advertisement element in

Beacon and Probe Response frames."

::= { dot11WNMEventWNMLogReportEntry 6 }

dot11WNMEventWNMLogRprtTimeError OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the Time Error value as defined in the Time

Advertisement IE Time Error field when the Time Capabilities field is set to 2. This

field is included in the Time Advertisement element in Beacon and Probe Response

frames."

DEFVAL { 0 }

::= { dot11WNMEventWNMLogReportEntry 7 }

To

dot11WNMEventWNMLogRprtUTCOffset OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(10))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute indicates the UTC Offset Time Value optionally included in the Event Report."

::= { dot11WNMEventWNMLogReportEntry 6 }

dot11WNMEventWNMLogRprtTimeError OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute contains the value of the Event Time Error field optionally included in the Event Report."

::= { dot11WNMEventWNMLogReportEntry 7 }

P398L64, change from dot11WNMEventTransitRprtTimeValue to dot11WNMEventTransitRprtUTCOffset

P399L13, change from dot11WNMEventRsnaRprtTimeValue, to dot11WNMEventRsnaRprtUTCOffset

P399L25, change from dot11WNMEventPeerRprtTimeValue, to dot11WNMEventPeerRprtUTCOffset

P399L36, change from dot11WNMEventWNMLogRprtTimeValue, to dot11WNMEventWNMLogRprtUTCOffset

Also, P310L15, delete “DEFVAL {0}”

(Q11) P341L33, change "wireless" to wireless (delete quotes)

(Q12) P362L39, change from “nu,mber” to “number”, from “1s: to “is” and 7.3.2.70.3 to 7.3.2.71.3:

(Q13) P362L29, P362L40, P392L34, P392L43, change from “70” to “71” in “7.3.2.70.2” and  “7.3.2.70.3”

(Q14) P343L46, insert a new entry immediately following dot11RRMRqstSTAStatTrigTimeout:

dot11RRMRqstSTAStatTrigCondition

P343L52, insert a new entry immediately following dot11RRMRqstSTAStatTrigSTARetryCntThresh:

dot11RRMRqstSTAStatTrigQoSTrigCondition

P343L59, insert a new entry immediately following dot11RRMRqstSTAStatTrigQoSDiscardCntThresh:

dot11RRMRqstSTAStatTrigRsnaTrigCondition

and insert the corresponding definitions in the list below, renumbering as required:

P344L50

dot11RRMRqstSTAStatTrigCondition OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(2))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the trigger values used when requesting triggered STA Statistics

reporting. The format of the STA Counter Trigger Condition field is shown in Figure 7-62h2."

::= { dot11RRMRequestEntry 55 }

P346L22

dot11RRMRqstSTAStatTrigQoSTrigCondition OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(2))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the trigger values used when requesting triggered QoS STA Statistics

reporting. The format of the STA Counter Trigger Condition field is shown in Figure 7-62h4."

::= { dot11RRMRequestEntry 63 }

P348L6

dot11RRMRqstSTAStatTrigRsnaTrigCondition OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(2))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the trigger values used when requesting triggered RSNA STA Statistics

reporting. The format of the STA Counter Trigger Condition field is shown in Figure 7-62h6."

::= { dot11RRMRequestEntry 72 }

(Q15) P355L12, P359L26, P398L8, Change from “dot11WNMDiag8021xAuthRprtEapVendorId” to “dot11WNMDiag8021xAuthRprtEapVendorID”

(Q16) P393L20, change from “::= { dot11LocationServicesEntry 8 }” to “    ::= { dot11WNMLocConfigReportEntry 8 }

(Q17) P355L26, insert a new entry “dot11WNMRqstLocConfigOptions OCTET STRING “ immediately following “dot11WNMRqstLocConfigBcastRate” and insert the following entry in the table below,

Renumbering subsequent entries as required:

P362L53, insert the following entry:

dot11WNMRqstLocConfigOptions OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the location track indication options used, see 7.3.2.71.9."

::= { dot11WNMRequestEntry 46 }

And P398L22, insert a list item “dot11WNMRqstLocConfigOptions OCTET STRING,” immediately following dot11WNMRqstLocConfigBcastRate INTEGER,

P391L62, Insert the following entry in the Dot11WNMLocConfigReportEntry, immediately following
dot11WNMLocConfigRprtLocIndBcastRate, of

dot11WNMLocConfigRprtLocIndOptions OCTET STRING

inserting the following entry in the list below:

dot11WNMLocConfigRprtLocIndOptions OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (0..255))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the location track indication options used, see 7.3.2.71.9."

::= { dot11WNMLocConfigReportEntry 7 }, renumbering subsequent entries

Insert a new entry on P400L25:

dot11WNMLocConfigRprtLocIndOptions OCTET STRING immediately following dot11WNMLocConfigRprtLocIndBcastRate,

(Q18) P353L18, change from “SYNTAX INTEGER (1..255)” to “SYNTAX INTEGER (0..255)

(Q19) P353L11, change from “dot11RRMNeighborReportBSSTerminationTSF OCTET STRING,” to “dot11RRMNeighborReportBSSTerminationTSF TSFType,”

P353L33, change from “SYNTAX OCTET STRING (SIZE (6))” to “SYNTAX TSFType”

(Q20) P393L1, change from VendorSpecific(8) to vendorSpecific(8)

(Q21) Change report entry list items (excluding index items) from read-create/read-write to read-only P366L50, P366L60, P367L4, P367L12, P367L62, P369L12, P370L26, P372L19, P372L27, P372L36, P372L63, P373L6, P375L35, P375L43, P375L52, P376L2, P376L10, P377L65, P378L10, P378L22, P378L31, P378L42, P378L60, P380L40, P382L4, P382L14, P382L25, P382L26, P382L47, P382L58, P385L12, P385L21, P385L35, P385L42, P385L54, P385L63, P386L7, P386L16, P384L25, P386L42, P386L50, P387L7, P388L35, P388L47, P388L56, P390L15, P390L23, P390L35, P390L44, P390L53, P390L62, P391L14, P391L22, P392L30, P392L38, P392L50, P393L3, P393L23, P394L35, P394L43, P394L52, P396L48, P396L61, P397L6, P397L21, P396L31

Change request entry list items P358L2, P358L35 from “read-only” to “read-create”

(Q22) Unsigned Int32 has incorrect max value, D14.0 P354L9, Change from

dot11WNMRequestNextIndex OBJECT-TYPE
SYNTAX Unsigned32(0..65535 )

to

dot11WNMRequestNextIndex OBJECT-TYPE
SYNTAX Unsigned32(0.. 4294967295 )

(Q23) Tables that are defined as read-create with an index value that is not based on an interface or other existing index, need a corresponding MIB attribute that allows an external mgmt entity to get the next available index. Some tables in Annex Q have it. But the location table in Annex D is missing it.

Insert BEFORE dot11LocationServicesTable on P321:

dot11LocationServicesNextIndex OBJECT-TYPE
SYNTAX Unsigned32(0.. 4294967295)
MAX-ACCESS read-only
STATUS current
DESCRIPTION
"Identifies a hint for the next value of dot11LocationServicesIndex to be used in a
row creation attempt for dot11LocationServicesTable. If no new rows can be created
for some reason, such as memory, processing requirements, etc, the SME
shall set this attribute to 0. It shall update this attribute to a proper
value other than 0 as soon as it is capable of receiving new measurement
requests. The nextIndex is not necessarily sequential nor monotonically
increasing."
::= { dot11smt 19 }

This replaces the table at dot11smt 19. Renumber all subsequent dot11smt mib attributes, including dot11LocationServicesTable. Note – skip any 11u values, as 11u has finished.

Notes: Items 7,8,9,10,11, 18, 20, 21, 22, Q1, Q2, Q6, Q8, Q9, Q11, Q15 and Q16, Q20 include changes identified in the MIB compilation. Item Q10 includes additional changes related to the D14.0 SB MIB-1 category comments.

References: