IEEE P802.11
Wireless LANs

| Comment Discussion on map and enablement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Eunsun Kim | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1860 | esun.kim@lge.com |
| Yongho Seok | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1947 | yongho.seok@lge.com |

Abstract

This document suggests proposed resolution on some of Map and Enablement comments. The proposed resolution details are aligned with P802.11af-D0.05.

This document is submitted to resolve some of Map and Enablement comments.

The spread sheet with proposed status and resolutions on Map comments is 10/0789r0.

The spread sheet with proposed status and resolutions on Enablement comments is 10/0713r1.

Comments discussed in this document are as follows:

Map Comments: 10, 32, 53, 59, 60, 61, 89, 98, 102, 121

Enablement Comments: 26, 33, 41, 42, 44, 64, 65, 69, 111, 127, 130

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| 10 | Charles Einoff | 11.af1.2 | 8 | 50 | T | It is more correct to specify a legally protected service rather than distinquishing between primary services and other legitimate services. | Change the text to read: "They shallmay update WSMs when they receive a measurement report in which a legally protectedprimary service signal is measured on a channel, which is indicated as available from a TV bands database system. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 53 | Winston Caldwell | 11.af1.2 | 8 | 50 | T | It would be more precise to specify that the WSMs are updated when a legally protected service signal is detected. | They mayshall update WSMs when they receive a measurement report in which a primary legally protected service signal is measured on a channel, which is indicated as available from a TV bands database system. |

Discussion:

Comments 10 and 53 say that the update of WSM from a measurement result should be mandatory. However, in FCC 47 C.F.R., Section 15.711 (c) (3),  it is said that "A TVBD is required to check for TV signals for a minimum time interval of 30 seconds. If a TV signal is detected on a channel indicated as available for use by the database system, the device will provide a notice of that detection to the operator of the device and a means for the operator to optionally remove the channel from the device’s list of available channels." Update of the Map from the measurement is optional requirement.

Disagree - No change

| 26 | Kaberi Banerjee | J.2.4 | 21 | 54 | T | No STA shall use Channel Switch Announcement; normative text in Appendix allowed ? |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Extended Channel Switch Announcement is allowed, not Channel switch announcement

Disagree - No change

| 32 | Kaberi Banerjee | 11.af1 |  |  | T | Is it possible to have none of the TVWS channels available; if so what is the function and structure of the WSM IE |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 98 | Alex Reznik | 7.3.2.af1 | 4 | 1-37 | T | It is not clear whether the option n=0 is allowed. If it is allowed, I would presume it is meant to inform others STAs that the database was checked at so-and-so time and no channels were available. | Clearly state whether n =0 is allowed. Define how to interpret WSM map in that case. |
| 102 | Alex Reznik | 11.af1.2 | 8 | 48-65 | T | As may be another place to define the meaning of a map with no available channels listed, if needed | Define the meaning of WSM witj no available channels listed. |

Discussion:

Comments 32, 98 and 102 ask for a clarification and addition of description for WSM when there is no available channel. However, if there is no available channel in TV White Space, then enablement procedure is not performed and no WSM is transmitted.

Disagree - No change

| 33 | Kaberi Banerjee |  | 11.af1.1 |  | 8 |  | 27 |  | T |  | For the TVWS band, in an Infrastructure BSS, is the enabling STA also an AP ? If not how does it broadcast the DSE Registered Location Element in a Beacon |  | Clarify the enabling STA role wrt the infrastructure BSS. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 127 | Runga Reddy | 11.af1.1 |  | 8 |  | 23 |  | T |  | When would STAs have independent access to the Internet as indicated on lines 23-24 pg 8? While there is no explicit prohibition on ad-hoc/mesh networks in the R&O, there is no clear direction either. To avoid confusion, TVWS access in 802.11 should be limited to the infrastructure mode, e.g. access through the AP (mode I portable). |  | Modify the text on lines 23-24 pg 8 as follows: "Any STA orOnly the AP may act as an enabling station if ti has its own geographic location identification and database access capabilities." |  |

Discussion:

An enabling STA can be an 802.11 WLAN STA, logical entity e.g. a network server. An enabling STA is not necessarily an AP STA. Submission 802.11-10/737 defines an 802.11u Registered Location Query Protocol (RLQP) and enabling signal. It adds definitions and behavior, answers comment 33 and 127. In a BSS, the AP can be dependent STA and its enabling STA can be a non-AP STA .

Disagree - No change

| 41 | Yohannes, Zhou Lan | J.2.4 | 22 | 31-32 | T | The sentence "The STA detects signals in the frequency bands that are indicated as available channels indicated by the eneabling STA" seems grammatically erronous | replace it with "The STA detects signals in the frequency bands that are indicated as available channels by the enabling STA " |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

In Line 44-45 Page 90, replace "The STA detects signals in the frequency bands that are indicated as available channels indicated by the eneabling STA" with "The STA detects signals in the frequency bands that are indicated as available channels by the enabling STA"

Agree: Replace the sentence as “The STA detects signals in the frequency bands that are indicated as available channels by the enabling STA” in Line 44-45 Page 90.

| 42 | Yohannes, Zhou Lan | J.2.4 | 22 | 35-36 | T | The sentence "If the enablement is successful and no signals are detected during channel availability check, the state of the depedent STA changes from Becoming enabled to Enabled" is redundant. | Remove the sentence |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The sentence does not look like redundant, but the last sentence in the previous paragraph is not well aligned. Because this sentence seems redundant, so it would be better to remove that sentence.

In Line 45 Page 90, remove the sentence "If no signals are detected, the dependent STA becomes enabled"

Agree in Principle: Remove a sentence “If no signals are detected, the dependent STA becomes enabled” in line 45, page 90.

| 44 | Yohannes, Zhou Lan | 11.af1.1 | 8 | 30-32 | T | A STA shouldn’t necessarily be limitted to scanning only the channels available in the list. A case of intialization is not addressed. | During initialization, a recommended channel list is not available. The STA doesn’t know at which channel the DSE is anticipated. A text addressing on how to select the channels to start scanning is required. Detail proposal will be given in XXX |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 111 | Chen Sun | 11.af1.1 | 8 | 31-32 | T | The STA can perform scanning before the list of available channels is received. There shouldn't be any limitation on the channels that the dependent STA can scan before the list of available channels is received. | Rewrite as "The channel availability check can also be done in parallel to listening for enabling signals as an implementer's option if the list of available channels is a known a-priori. When a dependent STA receives a beacon frame that has a WSM IE, then it should only scan the available channels identified within the WSM IE." |

Discussion:

Comment 44 and 111 ask for clarification on the corresponding sentences. WSM only recommends the channels for scanning, but if a STA does not have WSM. Submission 802.11-10/790 is approved and regarding sentence has changed as, “Dependent STAs receiving WSM should scan for existing BSSs on the available channels identified within received WSM element. STA shall operate on the channels indicated in the WSM. When STAs having WSM receive an updated WSM from its AP or enabling STA, the STAs shall move channels for operation if they are operating on the channels which became unavailable by the updated WSM.”

Disagree – No change.

| 59 | Padam Kafle | 7.3.2.af1 | 4 | 1 | T | The white space map element does not need all the fields currently listed. The main information required is the channel list containing TV channels available for TVWS devices. It appears that the available list of channels in a regulatory class can already be provided by using existing "AP Channel Report' IE from 802.11k. Another possibility could be to use "Country" IE. Instead of introducing new IE for similar information, reusing existing mechanisms is recommended. | Clarify and consider reusing the AP channel report or country IE as per the as per the submission 11-10-0554r0 from current 802.11 specification. It is only required to use one regulatory class for 5 MHz channel and 'center on center', with list of available legal channel numbers (refer to Doc: 11-10-0554r0). |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

AP channel report or country IE cannot fully support the functionality of WSM. They also need to be modified. They only just can indicate the subset or super set of available channels, and they cannot indication power level per channel.

Disagree - No change

| 60 | Padam Kafle | 7.3.2.af1 | 4 | 25-28 | T | The TV bands access database access time is not necessary, or at least not explained in D0.02 why this information is required. When an access point receives its WSM information from its database access, it is already a valid informaition for certain period (1 day as per the current FCC rules). After that period, AP should not even transmit outdated information. However, until it is valid, it makes no sense to know which information was obtained when. If the intent is to resolve similar information from multiple APs, then for a client STA, the more important issue is which information could be valid for its location rather than which is latest, or what time stamp the WSM had. | Remove the database access time field or clarify what advantages it can provide (also refer to comments in Doc: 11-10-0554r0) |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Comment 60 asks for removal of TV Band Database access time in a White Space Map. Submission 802.11-10/790r2 is approved and it resolved this comment by replacing the TV Bands Database Access Time to Map ID.

Agree - No change

| 61 | Padam Kafle | 7.3.2.af1 | 4 | 34-37 | T | The power constraint limits for a channel are known as per FCC rules (e.g, if adjacent channel is unavailable due to primary signal, it is allowed with low power). It is not clear why one need to broadcast such information for all channels. If the intent of these limits are for Probe Request frames, such frame can be transmitted only after DSE enablement, during which, the power constraint limit of a particular channel of operation will already be provided from the enabling STA (DSE power constraint frame). Another way an AP will announce its power limit is by using "power constraint" element in beacon frame. | Remove the field. If the intent is to have it in the spec based on the expectation that European regulations require the information of different power levels for TVWS channels for a client device, it can be added when such regulation will actually materialize |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Transmission power level is necessary. The transmission power level differs according to the primary service signal type in the adjacent channels; it is not automatically attained from the available channel list.

Disagree - No change

| 64 | Padam Kafle | 11.af1.1 | 8 | 27-43 | T | The text here defines an enabling signal as a beacon frame from an enabling STA that contain WSM IE. Elsewhere in 11.11 from 802.11y, it appears that the enabling signal can only be any frame containing "DSE Registered Location" element with RegLoc DSE bit set to 1. Is the intent is to use either one of these two options as enabling signal? | Clarify |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 65 | Padam Kafle | A.4.af | 11 | 34-41 | T | Considering a WLAN BSS where an AP provides enabling signal to non-AP STAs (dependent or client STAs), and already they work under control of AP, there are no additional regulatory requirements that these client STAs need to do DSE enablement procedure to operate in TVWS band. | clarify |

Discussion:

Comment 64, 65 ask for a clarification on the enabling signal and enablement procedure. Submission 802.11-10/790 is approved and it is modified corresponding text in it regarding the enabling signal. In addition, submission 802.11-10/737 defines an 802.11u Registered Location Query Protocol (RLQP) and enabling signal. It adds definitions and behavior, answers comment 64 and 65. Any dependent STA who wants to operate in TVWS bands has to perform an enablement procedure, even if it is under control of an AP in other ISM bands. All the operational behaviours are described in the current draft.

Disagree – no change

| 89 | Peter Ecclesine | 7.3.2.af1 | 4 | 19 | T | The White Space Map should be organized based on Largest Maximum Transmit Power Level, list of available channels, next largest Max Power, next list of available channels, . . . Lowest Maximum Transmit Power Level, list of available channels at the lowest limit, so that all STAs can more easily operate over multiple TV channels with the same transmit power limit. | Commenter will provide draft text submission with information element and action frames (similar to submission 514r0 Channel Power Limit Map) with channels ranked by power limit to be applied to D0.02 for the next draft. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Comment 89 asks for that the White Space Map should be organized based on Largest Maximum Transmit Power Level, list of available channels, next largest Max Power, next list of available channels, . . . Lowest Maximum Transmit Power Level, list of available channels at the lowest limit, so that all STAs can more easily operate over multiple TV channels with the same transmit power limit. However, this kind of organization may increase the WSM overhead. When the transmission power levels of the multiple consecutive TV channels are different, then the transmission power of RLAN that spans multiple TV channels is constrained by the minimum of the power among the multiple TV channels.

Disagree.

| 121 | Runga Reddy | 7.3.2.af1 | 4 | 34 | T | What range does the power constraint parameter fall in? | Define the range of values that the power constraint parameter falls in. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The name of power constraint field in the WSM is already changed to Maximum Power Level. No need to define the range of the power constraint parameter.

Disagree - No Change

| 130 | Runga Reddy | J.2.4 | 23 | 1 | T | What other STAs other than the AP, should be allowed to be "enabled"? To avoid confusion, APs only should be controlling access because they are connected to the internet. This requires APs themselves to be connected to the Internet. | Modify Figure J2.1 so that if instructed by the TVWS band database to shutdown, the enabled AP instructs STAs to either shutdown themselves or provide for another AP to service them. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Enablement is independent of bands and channels of operation. When a dependent AP loses enablement, it can no longer transmit, and the Beacons cease.

Disagree - No change

Proposed Resolution:

Agree technical comments 41, 60 based on this discussion.

Agree in principle technical comment 42 based on this discussion.

Disagree technical comments 10, 26, 32, 33, 42, 44, 53, 59, 61, 64, 65, 89, 98, 102, 111, 121, 127 and 130 based on this discussion.

TGaf editor: In Line 44-45 Page 90, replace "The STA detects signals in the frequency bands that are indicated as available channels indicated by the enabling STA" with "The STA detects signals in the frequency bands that are indicated as available channels by the enabling STA"

TGaf editor: In Line 45 Page 90, remove the sentence "If no signals are detected, the dependent STA becomes enabled"